# Biotic and Abiotic Dehalogenation of Halogenated Methanes: Trichlorofluoromethane, Dichlorodifluoromethane, and Tetrachloromethane.

# A Dissertation Presented for the

# Doctor of Philosophy

# Degree in Microbiology

# The University of Tennessee, Knoxville

# Briana M. McDowell

# May 2021

Copyright © 2021 by Briana M. McDowell.

All rights reserved.

# Abstract

Trichlorofluoromethane (CFC-11), dichlorofluoromethane (CFC-12), and tetrachloromethane (CT) are fully halogenated methanes that were produced as refrigerants in the early part of the 1900’s, and later used in many industrial processes. They are ozone depleting agents and common groundwater contaminants. They are volatile chemicals that are moderately soluble in water. Due to their volatility when they are released to the environment they are predominantly found in the atmosphere, though they also dissolve into the groundwater. In anerobic environments they can undergo dehalogenation reactions with a number of redox active compounds. This dissertation presents results from two treatability studies from sites contaminated with CFC-11, CFC-12, and CT. Additionally, the effect of pH on the dehalogenation of CFC-11, CFC-12, and CT is examined, and a sulfidogenic enrichment culture grown in the presence of CT is characterized. Results from the first treatability study indicate that the addition of reactive iron species (i.e., zero valent iron or ferrous sulfide) combined with the bioaugmentation culture KB-1 Plus and lactate can facilitate the transformation of CT into non-halogenated end products. The most effect remediation strategy for CT observed during the treatability study for the second contaminated site was the addition of zero valent iron, this facilitated the transformation of CT to chloroform (CF). CF is a non-desirable end product, and additional remediation efforts are recommended for the second contaminated site. A shift in the type of transformation products formed during the reduction of CFC-11, CFC-12, or CT by super-nucleophilic cobalamin was observed as pH increased. Mackinawite and vivianite were identified as the two precipitate phases formed in the presence of the sulfidogenic enrichment culture. It is likely that the formation of vivianite occurs via precipitation with the phosphate present in the medium, and that mackinawite forms via precipitation with hydrogen sulfide produced by the sulfidogenic bacteria present in the enrichment. Additionally, a greater decline in CT was observed in microcosms that contained active enrichment culture than in heat killed controls, suggesting that the consortium aids in the degradation of CT, probably via the formation of  mackinawite, as it is a reactive iron species.

# Table of Contents

Introduction	1

Chapter 1	2

Transformatioin of Trichlorofluoromethane, Dichlorodifluoromethane, and Tetrachloromethane by Biotic and Abiotic Processes: A Review	2

Abstract	3

Production, use, and occurrence in the environment	3

Toxicology	4

Physical and chemical properties	4

Fate and longevity in environmental systems	4

Degradation pathways	6

Hydrogenolysis	6

Radical-radical coupling	6

Free radical addition to alkenes	6

Combination reactions	7

Carbene hydrolysis	7

Surface associated carbene reduction	7

Transformation of CFC-11, CFC-12, and CT	7

Organometallic cofactors	8

Extracellular electron shuttles	8

Siderophores	12

Conclusions and Future Work	12

Appendix	12

CHAPTER II	17

High-resolution site charaterization and Laboratory-Scale Treatability Study enabling aquifer remediation at an Industrial Site.	17

Abstract	18

Introduction	18

Methods	20

Membrane Interface Probe-Hydraulic Profiling Tool (MiHPT) Logging	20

Aquifer Material Sampling	20

Groundwater Sampling	20

Borehole Abandonment	21

Volatile Organic Compounds (VOC) analysis	22

Total Iron Measurements	22

Quantitative Polymerase Chain Reaction (qPCR)	22

Microcosm Setup and Monitoring	22

Standard Curve Preparation of Gas Chromatograph Equipped with a Flame Ionization Detector (GC-FID) and Gas Chromatograph Equipped with a Micro-Electron Capture Detector (GC-µECD)	25

Data Management	25

Results	26

MiHPT Overview	26

Borehole A1 (BHA1)—MiHPT Data	26

Borehole A2 (BHA2)—MiHPT Data	28

Borehole A3 (BHA3)—MiHPT Data	30

Borehole A4 (BHA4)—MiHPT Data	32

Sediment and Groundwater VOC Data	34

Dissolved Iron Data	37

Overview of Treatability Study Microcosm Performance	39

Lactate Fermentation and Microcosm Performance: Borehole A1 at 12-m depth (BHA1-12 m)	42

PCE (BHA1-12 m)	44

1,1-DCE (BHA1-12 m)	46

CF (BHA1-12 m)	48

CT (BHA1-12 m)	50

CFC-11 (BHA1-12 m)	53

CFC-12 (BHA1-12 m)	55

Lactate Fermentation and Microcosm Performance: Borehole A3 at 7-m Depth (BHA3-7 m)	57

PCE (BHA3-7 m)	59

1,1-DCE (BHA3-7 m)	61

CF (BHA3-7 m)	63

CT (BHA3-7 m)	65

CFC-11 (BHA3-7 m)	67

CFC-12 (BHA3-7 m)	69

Lactate Fermentation and Microcosm Performance: Borehole A3 at 15-m Depth (BHA3-15 m)	71

PCE (BHA3-15 m)	73

1,1-DCE (BHA3-15 m)	75

CF (BHA3-15 m)	77

CT (BHA3-15 m)	79

CFC-11 (BHA3-15 m)	81

CFC-12 (BHA3-15 m)	83

Lactate Fermentation and Microcosm Performance: Borehole A3 at 24-m Depth (BHA3-24 m)	85

PCE (BHA3-24 m)	87

1,1-DCE (BHA3-24 m)	89

CF (BHA3-24 m)	91

CT (BHA3-24 m)	93

CFC-11 (BHA3-24 m)	95

CFC-12 (BHA3-24 m)	97

Lactate Fermentation and Microcosm Performance: Borehole A4 at 10-m Depth (BHA4-10 m)	99

PCE (BHA4-10 m)	101

1,1-DCE (BHA4-10 m)	103

CF (BHA4-10 m)	105

CT (BHA4-10 m)	107

CFC-11 (BHA4-10 m)	109

CFC-12 (BHA4-10 m)	111

qPCR Data from Sterivex Filters.	113

Discussion	115

Conclusions	117

Appendix	118

Libraries and R code used to make Figure 7 and Figure 8	139

CHAPTER III	140

A Laboratory-Scale Treatability Study of Biostimulation, Bioaugmentation, and Amendments with Reactive Iron Species for tetrachloroethene, Tetrachloromethane, and trichloromethane	140

Degradation at a contaminated site.	140

Abstract	141

Introduction	141

Methodology	143

Sample Collection	143

Sample processing and microcosm setup	143

Bioaugmentation	147

Abiotic degradation	147

Microcosm monitoring	147

Data analysis	147

Additional amendments	148

DNA isolation	148

Quantitative polymerase chain reaction (qPCR)	148

Results	149

Detection and quantification of known dechlorinating bacteria with qPCR	149

Microcosm performance	151

PCE transformation	152

Lactate fermentation in PCE-containing microcosms	156

CT transformation	158

Lactate fermentation and formation in CT-containing microcosms	160

CF transformation	162

Lactate fermentation in CF-containing microcosms	165

Microcosms testing abiotic degradation	167

Discussion	169

Appendix	170

CHAPTER IV	181

Effect of the potential of hydrogen on the Biomimetic Dehalogenation of trichlorofluoromethane, dichlorodifluoromethane, and carbon tetrachloride by Super-Nucleophilic vitamin B12.	181

Abstract	182

Introduction	182

Materials and Methods	183

Chemicals	183

Cyanocobalamin-catalyzed dehalogenation	183

Analytical methods.	184

Results	185

Transformation product experiment.	185

Discussion	191

Conclusions	192

Appendix	193

CHAPTER V	198

Characterization of Sulfidogenic Enrichment Culture Grown in the Presence of Carbon Tetrachloride.	198

Abstract	199

Introduction	199

Methods	200

Chemicals	200

Transformation of carbon tetrachloride in enrichment cultures	200

Analytical techniques	201

Statistical analysis of chlorinated VOC data.	201

Microbial community analysis via 16S rRNA gene amplicon sequence analysis.	201

Precipitate characterization	202

Results	203

Chlorinated VOC	203

Microbial community analysis. In order to identify members of the Elkhart enrichment culture	206

Precipitate characterization	208

Discussion	214

Conclusions	217

Appendix	218

Conclusions	224

References	226

VITA	242

# List of Tables

Table 1. Transformation products and reaction pathways of dehalogenation of CFC-11, CFC-12, and CT by inorganic electron shuttles.	10

Table 2. Physical and chemical properties of CFC-11, CFC-12, and CT.	12

Table 3. Amounts of chlorinated VOCs associated with aquifer materials collected in four different boreholes Area A.	35

Table 4. Groundwater Volatile Organic Compound Data.	36

Table 5. Dissolved ferrous iron in groundwater samples.	38

Table 6. Borehole A1-12 m lactate fermentation performance	43

Table 7. Borehole A3-7 m lactate fermentation performance.	58

Table 8. Borehole A3-15 m lactate fermentation performance.	72

Table 9. Borehole A3-24 m lactate fermentation performance	86

Table 10. Borehole A4-10 m lactate fermentation performance.	100

Table 11. qPCR data from Sterivex filters collected at site A.	114

Table 12. Sampling depth intervals for aquifer material collected in Area A.	118

Table 13. Chlorinated compounds added to microcosms.	119

Table 14. Physical properties of target contaminates and projected transformation products.	120

Table 15. Detailed instrument settings used for the analysis of headspace samples with gas chromatography - FID.	123

Table 16. Detailed instrument settings used for the analysis of headspace samples with gas chromatography - µecd.	124

Table 17. Chlorinated compounds of concern and expected degradation products.	125

Table 18. Microcosms established with aquifer core materials to assess target contaminant degradation.	128

Table 19. Preparation of pipettable composite slurries for microcosm setup.	145

Table 20. Detection of total bacterial, Dhc, and Dhb 16S rRNA genes with specific qPCR assays in groundwater samples collected from the contaminated site.	150

Table 21. Consumption of lactate and formation of acetate and propionate in lactate-amended microcosms containing PCE.	157

Table 22. Consumption of lactate and formation of acetate and propionate in lactate-amended microcosms containing CT.	161

Table 23. Consumption of lactate and formation of acetate and propionate in lactate-amended microcosms containing CF.	166

Table 24. Contaminant concentrations measured in the groundwater at the contaminated site.	170

Table 25. Groundwater parameters determined in the monitoring well located in the center of the four sampling locations (data provided by Parsons).	172

Table 26. Volumes of groundwater filtered through each Sterivex cartridge.	173

Table 27. cVOCs added to microcosms.	176

Table 28. Microcosms established to assess target contaminant degradation.	179

Table 29. CFC-11 pH variation experiment performance at 24 hours.	187

Table 30. CFC-12 pH variation experiment performance at 24 hours.	188

Table 31. CT pH variation experiment performance at 24 hours.	189

Table 32. Performance of reactions with super-nucleophilic B12 and transformation products at 24 hours.	190

Table 33. Detailed instrument settings used for the analysis of headspace samples with GC – FID set up with the DB-624 column.	193

Table 34. Detailed instrument settings used for the analysis of headspace samples with GC – FID PLOTQ column.	194

Table 35. Physical and Chemical Properties	197

Table 36. Description of precipitate formation in microcosms.	209

Table 37. Amendment schemes for Elkhart microcosms.	218

Table 38. Detailed instrument settings used for the analysis of headspace samples with gas chromatography.	219

Table 39. Amendment Schemes for X-ray diffraction microcosms.	220

Table 40. X-ray diffraction collection parameters	221

Table 41. Pairwise comparisons for the effect of treatment on carbon tetrachloride.	222

# List of Figures

Figure 1. Trichlorofluoromethane degradation pathways	13

Figure 2. Dichlorodifluoromethane transformation pathways part 1	14

Figure 3. Dichlorodifluoromethane transformation pathways part 2	15

Figure 4. Tetrachloromethane degradation pathways	16

Figure 5. Membrane interface probe-hydraulic profiling tool data from Borehole 1.	27

Figure 6. Membrane interface probe-hydraulic profiling tool data from Borehole 2.	29

Figure 7. Membrane interface probe-hydraulic profiling tool data from Borehole 3.	31

Figure 8. Membrane interface probe-hydraulic profiling tool data from Borehole 4.	33

Figure 9. Performance of monitored natural attenuation, heat kill, biostimulation, and bioaugmentation microcosms.	40

Figure 10. Performance of microcosms amended with reactive iron species, and combined bioaugmentation with reactive iron species.	41

Figure 11. Borehole A1-12 m microcosm performance (PCE).	45

Figure 12. Borehole A1-12 m microcosm performance (1,1-DCE).	47

Figure 13. Borehole A1-12 m microcosm performance (CF).	49

Figure 14. Microcosm setup to test the activity of KB-1 Plus on CT in the absence of aquifer material.	51

Figure 15. Borehole A1-12 m microcosm performance (CT).	52

Figure 16. Borehole A1-12 m microcosm performance (CFC-11).	54

Figure 17. Borehole A1-12 m microcosm performance (CFC-12).	56

Figure 18. Borehole A3-7 m microcosm performance (PCE).	60

Figure 19. Borehole A3-7 m microcosm performance (1,1-DCE).	62

Figure 20. Borehole A3-7 m microcosm performance (CF).	64

Figure 21. Borehole A3-7 m microcosm performance (CT).	66

Figure 22. Borehole A3-7 m microcosm performance (CFC-11).	68

Figure 23. Borehole A3-7 m microcosm performance (CFC-12).	70

Figure 24. Borehole A3-15 m microcosm performance (PCE).	74

Figure 25. Borehole A3-15 m microcosm performance (1,1-DCE).	76

Figure 26. Borehole A3-15 m microcosm performance (CF).	78

Figure 27. Borehole A3-15 m microcosm performance (CT).	80

Figure 28. Borehole A3-15 m microcosm performance (CFC-11).	82

Figure 29. Borehole A3-15 m microcosm performance (CFC-12).	84

Figure 30. Borehole A3-24 m microcosm performance (PCE).	88

Figure 31. Borehole A3-24 m microcosm performance (1,1-DCE.)	90

Figure 32. Borehole A3-24 m microcosm performance (CF).	92

Figure 33. Borehole A3-24 m microcosm performance (CT).	94

Figure 34. Borehole A3-24 m microcosm performance (CFC-11).	96

Figure 35. Borehole A3-24 m microcosm performance (CFC-12).	98

Figure 36. Borehole A4-10 m microcosm performance (PCE).	102

Figure 37. Borehole A4-10 m microcosm performance (1,1-DCE).	104

Figure 38. Borehole A4-10 m microcosm performance (CF).	106

Figure 39. Borehole A4-10 m microcosm performance (CT).	108

Figure 40. Borehole A4-10 m microcosm performance (CFC-11).	110

Figure 41. Borehole A4-10 m microcosm performance (CFC-12).	112

Figure 42. Aquifer material extraction from stainless steel cores.	121

Figure 43. Microcosm setup inside an anoxic chamber.	122

Figure 44. Standard curves for target contaminants and degradation products (GC-FID).	126

Figure 45. Standard curves for target contaminants and degradation products (GC-ECD).	127

Figure 46. Amounts of PCE and transformation products over time.	153

Figure 47. Changes in the amount of PCE, cDCE, and ethene in µmol per bottle over time.	155

Figure 48. Amounts of CT in the microcosms over time.	159

Figure 49. Amounts of CF and transformation products over time.	163

Figure 50. Amounts of CF and transformation products measured in bioaugmentation microcosms #20a–b, 24 a–b, and 26 a–b over time.	164

Figure 51. Abiotic transformation of CT by ZVI in microcosms established with upper aquifer material, lower aquifer material, and a control containing no aquifer material.	168

Figure 52. Schematic showing the temporary well locations surrounding the monitoring well at the contaminated site.	171

Figure 53. Aquifer material cores that were received prior to separating the cores into lower (bottom) and upper (top) cores.	174

Figure 54. Transfer of aquifer material slurry to sterile and N2/CO2-flushed 160-mL glass serum bottles.	175

Figure 55. Dispensing of synthetic, defined reduced anoxic mineral salt medium (RAMM) to the glass serum bottles containing the composite slurry.	177

Figure 56. Microcosms established with aquifer slurries and composite groundwater or RAMM.	178

Figure 57. Reactions with super-nucleophilic vitamin B12 and Ti(III)citrate.	186

Figure 58. Standards run on GC-FID with a PLOTQ column.	195

Figure 59. Standards run on GC-FID with a DB-624 column.	196

Figure 60. Chlorinated volatile organic compound concentrations over time in carbon-tetrachloride-containing microcosms.	204

Figure 61. One-way analysis of variance of the effect of treatment on carbon tetrachloride. Each purple bar indicates the average decline in µmol of CT per treatment. The error bars show the standard error of each treatment. The p-value for each comparison is shown above the brackets which indicate the two treatments being compared.	205

Figure 62. Maximum likelihood tree built from predicted Elkhart operational taxonomic unit.	207

Figure 63. Diffraction patterns of the FeCl2, Na2S, and NaSO4 amended microcosms.	210

Figure 64. Diffraction patterns of the FeCl2 and Na2S amended microcosms.	211

Figure 65. Diffraction patterns of the FeCl2 and Na2SO4 amended microcosms.	212

Figure 66. Diffraction patterns of the FeCl2 amended microcosms.	213

# Introduction

This dissertation is focused on the degradation of the halogenated methanes trichlorofluoromethane (CFC-11), dichlorodifluoromethane (CFC-12), and tetrachloromethane (CT). Chapter 1 presents a literature review, which encompasses the production, occurrence and use of  CFC-11, CFC-12, and CT as well as toxicological profiles, physical and chemical properties, and a discussion of the processes that contribute to the dehalogenation of CFC-11, CFC-12, and CT. Chapter 2 describes a high-resolution site characterization and treatability study for a site contaminated by CT, chloroform (CF), tetrachloroethene (PCE), 1,1-dichloroethene, CFC-11, and CFC-12, and aims to identify contaminant distribution in the vadose and saturated zone at the site as well as identify remediation strategies that may be effective. Chapter 3 details a treatability study performed for a site contaminated with CT, CF, and PCE and aims to determine effective remediation strategies for the site. Chapter 4 uses 16S rRNA amplicon sequencing to identify the community structure of an enrichment culture grown in the presence of CT and utilizes x-ray diffraction to characterize the precipitates formed in the enrichment cultures. Additionally, changes in CT concentration over time are monitored by gas chromatography (GC) with a flame ionization detector (FID). Chapter 5 focuses on the dehalogenation of CFC-11 and CFC-12 by super-nucleophilic vitamin B12. Additionally, a summary of the findings from each chapter can be found in the conclusions section.

# Chapter 1

# Transformatioin of Trichlorofluoromethane, Dichlorodifluoromethane, and Tetrachloromethane by Biotic and Abiotic Processes: A Review

## Abstract

Trichlorofluoromethane (CFC-11), dichlorodifluoromethane (CFC-12), and tetrachloromethane (CT) are ozone depleting agents and groundwater contaminants found the world over. The first few sections of this review discuss the global production occurrence and use, toxicology, physical properties, and fate in environmental systems of CFC-11, CFC-12, and CT. Then there is a detailed discussion of the possible degradation pathways for CFC-11, CFC-12, and CT. Degradation of CFC-11, CFC-12, and CT is favorable in anerobic environments, and can occur via hydrogenolytic, radical-radical coupling, combinatory, hydrolysis, or surface associated carbene reduction pathways. The latter sections of this review discuss the redox active compounds that facilitate the degradation of CFC-11, CFC-12, and CT, and explores possible avenues of future research.

### Production, use, and occurrence in the environment

Trichlorofluoromethane (CFC-11) and dichlorodifluoromethane (CFC-12) are anthropogenic chlorofluorocarbons (CFCs) that were developed in the 1930s for use in refrigeration.1 Their use was later expanded to foam blowing agents, aerosol propellants, insulation, solvents and as building blocks in the production of fluorinated polymers.2 By 1995, estimated global accumulative production for CFC-11 and CFC-12 were 8.6 million and 11.3 million metric tons respectively.3 The alternative fluorocarbons environmental acceptability study (AFEAS) estimated that 87.2% of the CFC-11 and 95% of the CFC-12 produced had been released into the environment as of 1995. Unfortunately, CFC-11, CFC-12, and carbon tetrachloride (CT) are ozone depleting agents and greenhouse gasses.4,5 Once these chemicals reach the upper atmosphere they undergo photochemical decomposition and produce chlorine radicals.5 These chlorine radicals then drive the ozone destruction cycle.6 Due to their ozone destruction potential the production of CFC-11, CFC-12, and CT has been phased out under the Montreal protocol.7

CT is a toxic chlorinated solvent, predicted carcinogen, and an ozone-depleting agent.8 It was widely used during the twentieth century as a dry-cleaning agent, industrial degreaser, grain fumigant, fire extinguishing agent, and in the synthesis of other chemicals most prominently for the production of CFCs.9 Production of CT peaked in 1974 with production volumes reaching 1200 million pounds per year.10 Production volumes declined during the latter part of the twentieth century due to regulations. Under the Montreal Protocol, production of CT as an emissive chemical has been completely banned since 2010.11 CT still produced as an intermediate for the synthesis of chloromethanes and tetrachloroethene, and production volumes as of 2018 were estimated to be between 33 – 55 million pounds per year. CT is released into the environment via evaporation, dumping and spills.8 It can be found in sediment, groundwater and the atmosphere. Global atmospheric emissions for 2014 were estimated to be around 77 million pounds due to emissions from current production, legacy sites, and inadvertent formation and loss during the production of chlorine (Cl2) gas.11 As of 2019, there were 301 active Superfund sites containing CT in the United States and many others abroad.12 CT is ranked 6th on the EPA’s national priorities list for hazardous substances and has an MCL of 0.005 mg/L in drinking water.

This review summarizes the abiotic and biotic mechanisms of CFC-11, CFC-12 and CT degradation, and identifies research needs to advance the remediation of CFC-11, CFC-12, and CT.

### Toxicology

The NIH cancer institute performed a bioassay for carcinogenicity for CFC-11, but was unable to determine if CFC-11 is carcinogenic to rats because too many rats died in the early stages of the study.13 However, a yearlong study of Swiss mice found that exposure to CFC-11 was not correlated with a significant increase in tumor formation.14 Aviado and colleagues reported that inhalation of CFC-11 had toxic effects on the respiratory and circulatory systems of mice, rats, dogs, and monkeys.15 Increased cardiac sensitivity to adrenaline and noradrenaline is the primary health risk from exposure to CFC-11 and other halocarbons.16 Increased cardiac sensitivity to adrenaline and noradrenaline can cause arrhythmia and death.

CFC-12 is considered non-carcinogenic, though there has been little research on the topic.17 In one study, CFC-12 exposure was found not to effect tumorigenesis in Sprague-Dawley rats or Swiss mice.18 Short term effects on the central nervous system were observed in guinea pigs exposed at high concentrations for an hour or more.19 Similarly to CFC-11 the primary health risk from exposure to CFC-12 is increased risk of cardia arrythmia due to cardiac sensitivities to adrenaline and noradrenaline, though higher concentrations of CFC-12 appear to be require to produce cardiac sensitivities and cardiac arrhythmias. 17

The primary health concern from exposure to CT is hepatotoxicity, while hepatic cancer is thought to occur secondarily.20–26 Chronic exposure to CT leads to sever cirrhosis, liver tumors, and chronic respiratory disease.27–29 The hepatotoxicity and carcinogenicity of CT is thought to occur via interactions between cellular macromolecules and the chlorine and trichloromethyl radicals that form during the degradation of CT.30–35 The effects of CT exposure on human and animal health were thoroughly reviewed by the International Program on Chemical Safety (IPCS).26

### Physical and chemical properties

CFC-11, CFC-12, and CT are volatile chemicals that are moderately soluble in water, and have densities greater than water Table 2. When these chemicals reach saturation in water they form dense non aqueous phase liquids. The octanol−water partitioning coefficients of CFC-11, CFC-12, and CT suggest that adsorption onto organic particles is feasible Table 2.36,37

### Fate and longevity in environmental systems

Due to their volatility, CFC-11, CFC-12, and CT are predominantly found in the atmosphere.1 Atmospheric concentrations are globally consistent, though elevated concentrations have been measured over large industrial cities like New York, Heidelberg, and Taipei.38–42 Atmospheric concentrations as of 2020 for CFC-11, CFC-12, and CT are roughly 224, 499, and 77 parts per trillion (ppt) respectively.43 Researchers predict that atmospheric concentrations of CFC-11, CFC-12, and CT will slowly decline for the next hundred years or so. This prediction is based on the calculated lifespans of these chemicals in the atmosphere and the expectation of decreased emissions due to the limited amount of production currently occurring.4,5,44

Once CFC-11, CFC-12, and CT reach the stratosphere they undergo photolytic dissociation.7 The process of photolytic dissociation produces a two odd electron species (free radicals).5 Photolytic dissociation of CFC-11 produces CFCl2 and a chlorine radical (Cl), CFC-12 produces CF2Cl and Cl, and CT produces CCl3 and Cl. The carbon containing free radicals can generate additional one atom chlorine radicals in future reactions. CFC-11 has the potential to generate three chlorine radicals, while CFC-12 and CT have the potential to generate two and four chlorine radicals respectively. Chlorine radicals destroy ozone (O3) in a chain reaction where Cl reacts with O3 to produce ClO and O2. The radical is regenerated when ClO reacts with O to generate

Cl and O2. Free radical reactions continue until they react with another free radical (i.e., OH, Cl, or NO).

An alternative environmental fate for CFC-11, CFC-12, and CT is retention in the subsurface, where they can be stored in the unsaturated or saturated zones. In the unsaturated zone CFC-11, CFC-12 and CT concentrations are driven by barometric pumping and diffusion.1 In porous unsaturated soils barometric pumping of gasses only effects the top few meters of the unsaturated zone.45,46 Barometric pumping as a mechanism of subsurface gas transport is much more effective in fractured rock, where it can  transport gasses hundreds of meters over a few months.47,48 Diffusion is the dominant mechanism of gas transport in unsaturated porous sediments.45 Research suggests that CFC-11 and CFC-12 concentrations decline as depth increases.49 Field studies indicate that CFC-11 and CFC-12 concentrations are between 7 – 10 % of atmospheric concentrations at depths of 43.9 m and 57.5 m in porous unsaturated sediments.49,50

Sorption and desorption of CFC-11, CFC-12, and CT may play a role in the transportation and transformation of these chemicals in the subsurface.51 Sorption of CFC-11 and CFC-12 to dried limestone, Ottawa sands, Yolo sandy loams, and dry soils has been reported.51–53 Russel and collogues reported that CFC-11 and CFC-12 desorbed when soils were wetted, but desorption from the other matrices was not explored.54 Adsorption of CT to dry soils increases as the surface area of the soil increases or the percent of organic carbon in the soil increases.55,56 Adsorption of CT dry soil is a physical process that can be completely reversed.56,57 More information regarding the sorption and desorption rates of CFC-11, CFC-12, and CT on a variety of matrices could provide a better understanding of the transport and transformation of these chemicals in the subsurface.

Introduction of CFC-11, CFC-12, and CT into the saturated zone can occur via gasses dissolving into the groundwater from the atmosphere, industrial solvent spills, and contaminants leaching from landfills.58–63 CFC-11 and CFC-12 are used as groundwater tracers of young groundwater (i.e., ~100 years old).58 They are considered good groundwater tracers, because they were released and monitored globally, their rates of production and solubility are known, and they are considered stable in the groundwater.1,58,64,65 However, CFC-11 and CFC-12 cannot be used as groundwater tracers if their concentration in the groundwater exceed the values possible by equilibrium with modern air (i.e., if the groundwater was contaminated by a solvent spill or landfill leachate).1,59–61,63,66–68 Transformation of CFC-11, CFC-12, or CT has not been observed in aerobic groundwater.1,7,68,69 However, transformation of CFC-11, CFC-12 and CT has been observed under reducing conditions.

### Degradation pathways

Transformation of CFC-11, CFC-12, or CT can occur via several pathways. Each transformation pathway consists of sequential chemical reactions that produce a variety of transformation products.69,70 Some transformation products are more favorable than others depending on their toxicity, carcinogenicity, or recalcitrance. Redox conditions and the presence of reactants determine which transformation path occurs. Transformation of CFC-11, CFC-12, and CT is favorable in reduced conditions. The transformation of CT has been explored more thoroughly than the transformation of CFC-11 or CFC-12, because CT causes more severe health hazards and is a regulated chemical in the subsurface under the clean water act.37,63,68,71 Reported reaction pathways for CT include hydrogenolysis, radical-radical coupling, free radical addition to alkenes, combination, carbene hydrolysis, and surface associated carbene reduction reactions

Figure 4.69,70,72–76 Figure 4 depicts transformation pathways for CT. Transformation products for CFC-11 and CFC-12 have been reported via hydrogenolysis, and carbene hydrolysis, since the formation of radicals from CFC-11 and CFC-12 is possible coupling and free radical addition reaction pathways are also included for CFC-11 and CFC-12 in Figure 1 - Figure 3.77,78

### Hydrogenolysis

Hydrogenolysis is a reductive chemical reaction where a single bond between a carbon and a halogen (ie., Cl or F) is cleaved (Equation 1).70,79 The halogen leaves, and is replaced by two electrons and a hydrogen. Hydrogenolysis can occur sequentially, in this process halogens are consecutively replaced by hydrogens and two electrons.70,80

﻿ ﻿RX + H+ + 2e– → RH + X–		(1)

### Radical-radical coupling

Radical-radical coupling reactions occure when two radicals form a covalent bond (Equation 2). For example, when two trichloromethyl radicals couple to form hexachloroethane Figure 1 and Figure 4.72 This can lead to the formation of tetrachloroethene when two of the chlorines leave the hexachloroethane compound.

RX• + RX•  → R2X2		(2)

### Free radical addition to alkenes

Free radical additions to alkenes occur when a radical attacks a carbon-carbon double bond (Equation 3). This forms a carbon radical that will subsequently attack other compounds, and perpetuate a chain of radical reactions (Equation 3). The hepatoxic nature of CT is related to the formation of the trichloromethyl radical and its subsequent tissue injury (i.e., lipid peroxidation).30,81

(3)

### Combination reactions

A combination reaction occurs when two compounds react to form one compound (Equation 4).82 An example of a combination reaction is when a trichloromethyl radical and oxygen gas combine to form a new compound Figure 4. This is the first step in the transformation path way that leads to the formation of phosgene. There is uncertainty regarding what the subsequent steps in the formation of phosgene are.83

A + B →  AB		(4)

### Carbene hydrolysis

Hydrolysis reactions occur when water breaks a chemical bond (Equation 5).70,76 A carbene hydrolysis reaction can occur between a ﻿dichlorocarbene and a water molecule react to form carbon monoxide Figure 4.

:CCl2 + H2O   →  CO + 2HCl	(5)

### Surface associated carbene reduction

Surface associated carbene reduction occurs when a carbene is associated with a reactive surface; it was proposed by McCormick and Adriens to explain the formation of methane without observing the transformation products associated with hydrogenolysis.76 During surface associated carbene reduction a reactive surface donates electrons and hydrogen  while stabilizing carbene and radical intermediates Figure 4.

### Transformation of CFC-11, CFC-12, and CT

Biologically mediated transformation of CFC-11, CFC-12, and CT occurs via co-metabolic processes that do not support organism growth. No evidence of an organism using CFC-11, CFC-12, or CT as terminal electron acceptors exists. Biotic transformation of CFC-11, CFC-12, and CT occur via redox-active compounds with low molecular weights.68,84 Transformation of CFC-11, CFC-12 and CT has been reported under various reducing conditions, including sulfate reducing, iron reducing, methanogenic, acetogenic, denitrifying, fermentative and halorespiring conditions.77,78,85–92 This review focuses on the redox-active compounds that have been shown to facilitate the transformation of CFC-11, CFC-12 or CT. Some of the redox active compounds are biologically produced (i.e., cobamides) while others are abiotic (i.e., zero valent iron), but the line between abiotic and biotic reactions is fuzzy, as many of the abiotic redox active compounds can be reduced by microbial processes, and the biologically produced redox active compounds rely on metal atoms. Many of the reactions that dehalogenate CFC-11, CFC-12, and CT fall into the category of biologically mediated abiotic reduction.

### Organometallic cofactors

Three organometallic cofactors that are able to dehalogenate CFC-11, CFC-12, or CT are cobamides, cytochromes (which contain heme groups), and coenzyme F430.93–98 All three of these cofactors contain highly conjugated porphyrin ring systems that stabilize a metal atom in the center of the ring.97,99,100 The metal components in these cofactors are cobalt in cobalamins, iron in cytochromes, and nickel in coenzyme F430. When the centrally coordinated metal atoms in these cofactors are reduced they can act as electron donors to halogenated compounds.94 Krone et al., reported the formation of carbon monoxide as the predominant transformation product of CFC-11 and CFC-12 dehalogenation by cobalamin with cobalt in the (+1) state (super-nucleophilic B12).93 Some carbon monoxide (~6% of observed transformation products) was formed when CT was dehalogenated by super-nucleophilic B12, but chloroform (CF) was the predominant transformation product (54% of observed transformation products). Krone and colleague suggest that the formation of carbon monoxide occurs via a carbene intermediate that undergoes hydrolysis (Figure 1-Figure 3), and that more carbon monoxide forms in reactions between super-nucleophilic B12 and CFC-11 or CFC-12 because trihaloradicals that contain fluorine are less stable than radicals that contain only chlorine (trichloromethyl radical), and that fluorine containing radicals are more likely to decompose into carbene intermediates (Figure 1-Figure 3). Cytochrome P450 reduced CFC-11 to carbon monoxide, and CT to CF.96,97 The formation of carbon monoxide likely occurs via a carbene intermediate. Unfortunately, carbon monoxide inhibits the dehalogenation activity of cytochrome P450.95–97 The carbon monoxide induced inhibition of cytochrome P450 makes the dehalogenation of CFC-11 by cytochrome P450 an inefficient degradation mechanism. Additional experiments have been shown that cytochrome-c and cytochrome-b produced by Shewanella putrefaciens 200 can transform CT to CF.101,102 Transformation of CT to CF is thought to occur via a trichloromethyl radical intermediate Figure 4. Experiments with showed that F-430 could dechlorinate CFC-11 and CT.98 Unfortunately, no defluorination activity was observed. An additional limitation of F-430 is that is it only produced my methanogenic bacteria, and thus is encountered less frequently in contaminated environments than cobamides and cytochromes which are produced by several microbial groups.87,96,97,101–104

### Extracellular electron shuttles

Extracellular electron shuttles are molecules that can be reversibly oxidised or reduced and commonly function outside of the cell. The regenerative nature of electron shuttles allows a single molecule to facilitate numerous redox reactions.105 Electron shuttles donate electrons to halogenated pollunents like CFC-11, CFC-12, and CT, and facilitate redox reactions by lowering the activation energy.106–110 The redox potencial of an electron shuttle is important, because the effectiveness of an electron shuttle is dependent on its ability to be reduced and its subsequent capacity to reduce the target contaminent.94,105 Some conditions that can effect the redox potencial of an electron shuttle are pH and whether the electron shuttle is carrying one or two electrons.105

Organic extracellular electron shuttles are mollecules that can be reversibly oxidised or reduced, and are primarily carbon based. Humic substances and quinones are organic extracellular electron shuttles, that have been studied in depth.94 Humic substances contain aromatic compounds that have redox activity, and quinones are a common constituent of humic substances.111 Quinones contain cyclic diketones that is fully conjugated. Humic substances and quinones can be reduced bioticly by iron reducing, sulfate reducing, halorespiring, fermentive, and methanogenic microbes, or abioticlly by reductants found in the subsurface (i.e., sulfide, zero valent iron (ZVI), Ti(III)citrate, and cysteine.).94,106,109,112–117 Standard redox potencial and chemical structure influence the effectiveness of humic substances and quinones as electron shuttles.105 In general, quinones and humic acids with lower standard redox potencials facilitate faster reduction of target contaminants.94,110,114,118 However, under some conditions chemical structure seems to have a greater effect on the rate of reduction than standard redox potencial.94,108 For example, the rate of reduction of CT by sulfide was faster in the presence of juglone, napthoquinone, and benzoquinone than in the presence of anthraquinone-2,6-disulfonate even though anthraquinone-2,6-disulfonate has a lower standard redox potencial.108 The authors atributed the increased reaction rates in the presence of juglone, napthoquinone, and benzoquinone to chemical structure.94,108 They observed the formation of mercaptoquinones when juglone, napthoquinone, and benzoquinone were reduced, while reduction of anthraquinone-2,6-disulfonate formed semiquinone radicals.

The most commonly encountered and best studied inorganic electron shuttles are reactive iron species.70 Reactive iron species contain solid ferrous iron, which can donate electrons to chlorinated solvents.70,94 He and collegues reviewed the literature on the degradation of chlorinated solvents by reactive iron minerals, and observed that the hierarchy of reactivity for reactive iron sprecies towards chlorinated solvents is: dissordered ferrous suifide > ordered ferrous sulfide > zero valent iron (ZVI) > sorbed ferrous iron > green rust = magnetite.70 Dehalogenation of CT by ﻿Mackinawite, FeS, FeS2, ﻿Pyrite, ﻿Pyrrhotite, Magnetite, ﻿Green rust, and surface associated ferrous iron has been reported Table 1. Additionally dehalogenation of CFC-11 has been reported by ZVI.119 Dehalogenation of CFC-12 has been reported by palladium catalystis, and by Molybdenum and Tungsten containing carbides, though these reactions are not environmentally relivant, as they occurred at temperatures greater than 140ºC.120,121 Information regaurding dehalogenation by inorganic electron shuttles, the transforamtions products formed, and the predicted dehalogenation pathways is available in Table 1. As mentioned above, the relationship beteween abiotic and biotic reaction is synergistic, examples of synergistic effects inculde ferrous sulfide production being supported by the byproducts of sulfate reducing bacteria, or the formation and regeneration of reactive iron minerals by dissimulatior iron reducing bacteria.70,74,94,122 Thus, understanding CFC-11, CFC-12, and CT degradation by reactive mineral phases requires the integrated study of biotic and abiotic processes.

Table 1. Transformation products and reaction pathways of dehalogenation of CFC-11, CFC-12, and CT by inorganic electron shuttles.

| Chemical | Inorganic electron shuttle | Transformation products | Transformation pathway | Reference |
| --- | --- | --- | --- | --- |
| CFC-11 | ZVI | not reported | Unknown | 119 |
| CFC-12 | Pd/AI2O3 at temperatures greater than 140ºC | CH2F2 | Hydrogenolysis | 120 |
| CFC-12 | W2C/y-Al2O3 and Mo2C/y-Al2O3 carbides | CH2F2 | Hydrogenolysis | 121 |
| CT | Mackinawite | CHCl3 | Hydrogenolysis | 123 |
| CT | FeS | CHCl3, CH4, C2H4, C2H6 | Hydrogenolysis, and radical-radical coupling | 124 |
| CT | FeS | not reported | Unknown | 125 |
| CT | FeS | CHCl3 | Hydrogenolysis | 73 |
| CT | FeS | CHCl3 | Hydrogenolysis | 126 |
| CT | FeS | CHCl3 | Hydrogenolysis | 74 |
| CT | Pyrite | CHCl3 | Hydrogenolysis | 127 |
| CT | Pyrite | CHCl3 | Hydrogenolysis | 128 |
| CT | Pyrrhotite | CHCl3 | Hydrogenolysis | 128 |
| CT | Magnetite | not reported | Unknown | 129 |
| CT | Magnetite | CHCl3, CH4, CO | Hydrogenolysis, carbene hydrolysis, surface associated carbene reduction | 122 |
| CT | Magnetite | CHCl3 and CO2 | Hydrogenolysis | 130 |
| CT | Magnetite | CHCl3 | Hydrogenolysis | 123 |
| CT | Mackinawite | CHCl3 | Hydrogenolysis | 123 |
| CT | Surface associated Fe2+ Goethite | CHCl3 | Hydrogenolysis | 123 |
| CT | lepidocrocite | CHCl3 | Hydrogenolysis | 123 |
| CT | Siderite | CHCl3 | Hydrogenolysis | 123 |
| CT | Hematite | CHCl3 | Hydrogenolysis | 123 |
| CT | Green rust | CHCl3 | Hydrogenolysis | 131 |
| CT | Green rust | CF, HCOOH, CO | Hydrogenolysis, carbene hydrolysis | 132 |
| CT | Surface associated Fe2+ Goethite | not reported | Unknown | 133 |
| CT | Surface associated Fe2+ Goethite | CHC3, CO, formate | Hydrogenolysis, carbene hydrolysis | 134 |
| CT | Surface associated Cu0 on green rust | CHCl3, methane | Hydrogenolysis, and possibly surface associated carbene reduction | 135 |

### Siderophores

Pyridine-2,6-bis(thiocarboxylate) (PDTC ) is a siderophore that dechlorinates CT.136,137

The primary transforamtion product formed when CT is dechlorinted by PDTC is carbon dioxide.138,139 Lewis and colligues suggest that carbon dioxide formation occurs via interactions with a copper:PDTC complex that forms a thioester intermediate.136 Unlike the extracellular electron shuttles previously discussed, the reaction between CT and PDTC is not catalytic.136,140 When PDCT reacts with CT dipicolinic acid is produced. PDCT is produced by Pseudomonas stutzeri strain KC.136,137 The dechlorination of CT occurs most rapidly when Pseudomonas stutzeri strain KC is grown under nitrate reducing conditions.141

### Conclusions and Future Work

Despite the numerous mechanisms of CFC degradation known, these contaminants still trouble remediation efforts, this is likely due to the inhibitory nature of these chemicals on bacterial growth.30,31,34,95 There is one report in the literature of an organisms regenerating a reactive iron barrier in the presence of CT.142  Future research should look to identify other organisms that can promote the formation of reactive iron minerals in the presence of halogenated methanes. Alternatively research efforts could go into developing a bioremediation tool that utilizes B12, as the literature suggests that the predominant transformation product formed via B12 reduction is CO. However, a deeper understanding of how environmental factors alter the composition of transformation product formation is needed. It is especially important to understand which transformation products are produced, as they can be more hazardous than the starting chemical (i.e., dichlorofluoromethane from CFC-11 or CFC-12).

## Appendix

Table 2. Physical and chemical properties of CFC-11, CFC-12, and CT.

| Chemical | Molecular Weight (g/mol) | Density (g/mL) | Aqueous Solubility (mg/L) at 25°C | Henry's law constant k°H (mol/kg*bar) | Octanol:water partitioning coefficient (log Kow) | Boiling point (°C) |
| --- | --- | --- | --- | --- | --- | --- |
| CFC-11 | 37.368 | 1.49 | 1,100 | 0.0012 | 2.44 | 23.7 |
| CFC-12 | 120.914 | 1.35 | 280 | 0.0025 | 2.05 | -29.8 |
| Carbon Tetrachloride | 153.823 | 1.59 | 793 | 0.033 | 2.83 | 76.7 |

Figure 1. Trichlorofluoromethane degradation pathways

Figure 2. Dichlorodifluoromethane transformation pathways part 1

Figure 3. Dichlorodifluoromethane transformation pathways part 2

Figure 4. Tetrachloromethane degradation pathways

# CHAPTER II

# High-resolution site charaterization and Laboratory-Scale Treatability Study enabling aquifer remediation at an Industrial Site.

## Abstract

This study characterizes the contamination of an aquifer at an industrial site in South America and evaluates the effectiveness of monitored natural attenuation, anaerobic-enhanced in situ bioremediation (EISB), and enhanced abiotic degradation for treating specific contaminant of concern (PCE, 1,1-DCE, CT, CF, CFC-11, and CFC-12). The techniques used to characterize the site included membrane interface probe-hydraulic profiling tool logging, measurements of contaminants in the groundwater and sediment, and qPCR of DNA from groundwater samples to determine the presence and abundance of biomarker genes for organohalide-respiring bacteria. Microcosms containing aquifer material were monitored to a) assess the potential of natural attenuation as a remediation strategy; b) determine whether native microorganisms or commercially available bioaugmentation consortia can degrade target contaminants when provided lactate as an electron donor; c) evaluate the potential for enhanced abiotic CT degradation; and d) examine the degradation of CT when microcosms receive reactive iron species (ZVI, FeS, Fe3O4), commercially available bioaugmentation consortia, and lactate. The microcosm data suggests that bioaugmentation could be an effective remediation strategy for PCE, 1,1-DCE, and CF. Unfortunately, CT, CFC-11, and CFC-12 were not susceptible to degradation by the bioaugmentation culture. Furthermore, CT was degraded by reactive iron species, but more work needs to be done to determine an effective remediation strategy for CFC-11 and CFC-12.

## Introduction

The industrial site was originally a Freon production plant, though Freon production has not occurred there for 26 years. Monitoring wells have identified various contaminants, including tetrachloroethene (PCE), trichloroethene (TCE), 1,1-dichloroethene (1,1-DCE), carbon tetrachloride (CT), chloroform (CF), trichlorofluoromethane (CFC-11), and dichlorodifluoromethane (CFC-12) in the groundwater at the site. An air stripper is currently being used to remove volatile organic compounds (VOCs) from the groundwater, but implementation of additional remediation strategies is desired to accelerate remediation efforts at the site. The area of interest for this study was a portion of the site referred to as Area A. Area A was chosen by the technical team as the area of interest. There is currently no industrial activity occurring at Area A.

Technologies were used to characterize Area A, include membrane interface probe-hydraulic profiling tool (MiHPT) pushes, measurements of volatile organic carbon in the groundwater and sediment, and measurements of dissolved iron. Four borehole locations were selected along a transect in Area A, and MiHPT pushes were performed at those locations. These pushes measure VOCs, soil electrical conductance, and soil permeability. Sampling intervals for sediment and groundwater were chosen based on the possible presence of contaminants and variation in soil permeability, as indicated by the MiHPT pushes. Portions of the collected aquifer material were sent off for measurements of contaminants and iron in the collected groundwater and sediment. The rest of the aquifer material was used to set up microcosms for the treatability study.

Laboratory-based bench-scale treatability studies are performed to assess the potential for various remediation strategies to meet site-specific cleanup goals, aid in selecting a remediation strategy, and inform pilot-scale testing or, in some cases, inform full-scale remediation design.143 Site-specific cleanup goals are selected to conform to regulatory standards. In the United States, the Environmental Protection Agency (EPA) sets maximum contaminant levels (MCLs) for contaminants, and site-specific cleanup goals are often set to meet these MCLs. Monitored natural attenuation (MNA), enhanced abiotic degradation, and enhanced in situ bioremediation (EISB) technologies are commonly tested during treatability studies for sites impacted by chlorinated, volatile organic solvents.

EISB technologies utilize organohalide-respiring bacteria (OHRB), which use contaminants as energy sources.144 In contaminated aquifers, OHRB are often limited by the availability of electron donors. The addition of fermentable substrate (i.e., biostimulation) increases the flux of hydrogen (H2), a key electron donor for OHRB. If OHRB are not present or active at a contaminated site, then contaminant-degrading microbial consortia containing OHRB are available, and their injection (i.e., bioaugmentation) into a contaminated aquifer can increase contaminant degradation rates and extents.145 Bioaugmentation is typically performed in combination with biostimulation (i.e., the addition of fermentable substrates) to increase H2 flux and to ensure efficient contaminant detoxification.

The presence and abundance of OHRB in an impacted aquifer can be determined by quantitative polymerase chain reaction (qPCR). qPCR assays targeting specific reductive dehalogenase (RDase) genes utilized by OHRB are available because the OHRB involved in the degradation of chlorinated solvents are reasonably well understood. For several contaminants, specific RDase enzyme systems have been identified.146 Moreover, qPCR-based monitoring regimes can provide quantitative data about OHRB and provide information about bioremediation progress.147

EISB technologies are not applicable to all contaminants. For example, OHRB capable of respiring CT, CFC-11, or CFC-12 have not been identified, though co-metabolic CT degradation has been documented.148,149 Microbial co-metabolism of CT is due to fortuitous reactions between CT and reduced transition metal co-factors, such as corrinoids (Krone et al., 1991). CT is also susceptible to degradation by magnetite (a reactive mixed ferrous/ferric iron mineral), and microbial processes generating magnetite can indirectly promote CT degradation.122,150 Furthermore, CT is susceptible to abiotic degradation by zero-valent iron (ZVI) and ferrous sulfide (FeS).151,152 No specific organisms have been identified that dehalogenate CFC-11 and CFC-12, but dechlorination has been observed under anoxic conditions.85,153

This study uses MiHPT data, and VOC measurements to characterize the contaminant profile of Area A at the industrial site. DNA is extracted from Sterivex filters collected at the site to determine the presence and abundance of biomarker genes for OHRB via qPCR. Microcosms are also set up to determine the potential of biostimulation, bioaugmentation, and amendments with reactive iron species as remediation strategies at an industrial site.

## Methods

### Membrane Interface Probe-Hydraulic Profiling Tool (MiHPT) Logging

MiHPT pushes were advanced with a Track Rig-TC10 drill rig along a transect in Area A. The MiHPT push locations were selected with an expectation of encountering zones of high and low contamination. The membrane interface probe (MIP) is 24 inches long and has a diameter of 1.5 inches. Components included on the MiHPT probe are a photoionization detector, a flame ionization detector, and a halogen-specific detector.154 The photoionization detector uses a 10.6 electron volt (eV) lamp and detects compounds with double or triple bonds, which have ionization energies below 10.6 eV. The flame ionization detector measures ions produced during combustion and detects any combustible compound. The halogen-specific detector detects halogens by measuring the current produced when halogens react with alkali atoms on the surface of electrically charged platinum beads. Checks of the MIP components were performed by testing known concentrations of TCE, CFC-11, CT, and 1,1-DCE on the MiHPT probe. The hydraulic profiling tool (HPT) portion of the MiHPT probe injects water at a rate of between 50 ml and 500 ml per minute and uses a downhole transducer to measure hydraulic pressure, and an electrical conductivity (EC) dipole measures electrical conductivity. The MiHPT probe was advanced at a rate of 12 inches per minute with a Track Rig-TC10 drill rig in boreholes formed by a cone penetration test (CPT) probe. The Track Rig-TC10 drill rig is a direct push machine that uses the vehicle’s static weight and percussive pushes to advance tools and probes into the ground.

Dissipation tests were performed by stopping the injection of water from the HPT and recording hydrostatic pressure once the pressure had stabilized. Measurements of atmospheric pressure and the hydrostatic pressure throughout the borehole were used to calculate the depth of the water table and the corrected HPT pressure. Then, the HPT flow rate and the calculated corrected pressure were fed into an empirical model to estimate hydraulic conductivity (K).155

### Aquifer Material Sampling

Sampling intervals were decided by the technical team, and the Löffler lab Table 12. Sampling intervals were selected to capture variation in lithological properties throughout the aquifer and variation in contaminant profile, as suggested by the MiHPT data. The depth and location of the four sampling intervals can be seen in Table 12. A Track Rig-TC10 drilling rig and a 30-cm Doris-tube piston samplers (Adara Systems, Richmond, BC) were used to collect aquifer material at the designated sampling intervals. Aquifer material was recovered in stainless steel sleeves, visually inspected for lithological properties, capped to limit atmospheric exposure, and labeled by members of the technical team. The aquifer material samples were sent to Pace Analytical laboratories for VOC analysis following the EPA method SW846-8260B, and to the Löffler laboratory for bench-scale treatability tests. The samples were shipped on wet ice to maintain a temperature of ~ 4 °C.

### Groundwater Sampling

Groundwater samples were taken at approximately the same sampling intervals as the aquifer material samples Table 12. Team members recommended the use of three groundwater sampling techniques due to the variable aquifer conditions. The Hydropunch technique was used in areas of the aquifer that had relatively high permeability, while temporary wells were set up in areas of the aquifer with relatively low permeability, and an existing monitoring well was used when the sampling interval was close to the aquitard. Estimates of aquifer permeability were based on hydraulic conductivity profiling provided by the CPT and MiHPT pushes.

Groundwater sampling at Borehole A1 (BHA1) used the hydropunch technique. The Hydropunch tool was advanced to the desired sampling depth by direct push using the Track Rig-TC10 drill rig. At the desired sampling depth, the Hydropunch’s cone-tipped, 30-cm slotted piston was unsheathed, exposing the screen to the aquifer and allowing water to enter the direct pushrods, and a stainless steel retention valve and disposable tubing were used to verify that the aquifer was responding consistently during manual development. Micro-purging and low-flow sampling were conducted using a Geotech 0.675-inch outer diameter bladder pump.

Temporary wells were set up for groundwater sampling at Boreholes A3 and A4. Temporary wells were installed by advancing direct push rods containing a 0.75-inch inner diameter polyvinyl chloride (PVC) well screen and riser pipes to the desired sampling depths. The well screen was exposed to the aquifer by raising the direct pushrods a minimum of 1.5 m once the desired sampling depth was reached. Well rods were secured at the surface with clamps. A stainless steel retention valve and disposable tubing were used to verify that the aquifer was responding consistently during manual development. Micro-purging and low-flow sampling were conducted using a Geotech 0.675-inch outer diameter bladder pump.

Groundwater sampling from an existing monitoring well occurred near Borehole A3 (BHA3) to access groundwater near the aquitard. A Geotech 1.66-inch outer diameter bladder pump was used to conduct micro-purging and low-flow sampling inside of the existing monitoring well (Geotech, Denver, CO).

A Horiba U-50 multiparameter meter was used to measure the temperature, pH, oxidation-reduction potential (ORP), specific conductance, turbidity, dissolved oxygen, water level, and purge rate of the groundwater. Team members from the technical team, and the Löffler lab took turns monitoring the Horiba U-50 multiparameter meter parameters during the micro-purging of wells under low flow conditions. Groundwater samples were not taken until the parameters measured by the Horiba U-50 device were stable. Four types of groundwater samples were taken: groundwater for the treatability study microcosms was collected in 1-l bottles; groundwater for the total metals analysis was filtered into 100-ml bottles containing 1% nitric acid; groundwater for VOC analysis was collected in 40-ml bottles; and groundwater was filtered through Sterivex filters for qPCR. These groundwater samples were stored and shipped on wet ice to maintain a temperature of ~4 °C.

### Borehole Abandonment

Team members used a Geoprobe grout pump to fill each borehole with cement-free bentonite slurry. Boreholes that were less than 20 m deep were grouted by connecting PVC pipes to the grout pump, and those greater than 20 m deep were grouted by connecting direct pushrods to the grout pump. Boreholes located in paved areas were cemented over at the surface once they had been grouted with bentonite slurry.

### Volatile Organic Compounds (VOC) analysis

Pace Analytical Laboratories performed VOC analysis of sediment and groundwater samples. The EPA method SW846-8260B was used to identify VOCs in the samples (Pace Analytical, Minneapolis, MN).

### Total Iron Measurements

Groundwater samples from each of the boreholes and monitoring well PZ2 were received for analysis of total iron by inductively coupled argon plasma optical emission spectrometry (ICP-OES). These samples were stored at 4 °C until they were given to the Water Quality Core Facility at the University of Tennessee, Knoxville, on January 7th. The ICP-OES method used by this facility is based on the EPA method 200.7.

### Quantitative Polymerase Chain Reaction (qPCR)

Team members from the technical team, and the Löffler lab collaborated in obtaining field samples for qPCR at the groundwater sampling locations. The Sterivex-GP cartridges (Millipore, Billerica, MA) were identified by borehole location and shipped on wet ice to the Löffler lab. Upon arrival, the cartridges were immediately stored at -80 ˚C to preserve the collected biomass. They were then processed by members of the Löffler lab for DNA analysis. Following the manufacturer’s recommendations, DNA was isolated from the Sterivex-GP cartridges using the MoBio PowerLyzer PowerSoil Kit (MoBio, Carlsbad, CA). Moreover, according to the manufacturer’s manual, DNA concentrations were quantified using the Qubit dsDNA BR assay (Life Technologies, Grand Island, NY). DNA solutions were stored at -80 °C until analysis. For qPCR analysis, each sample was diluted to three different dilutions using nuclease-free water (i.e., 1:10, 1:100, and 1:1000) to determine whether any inhibitors were present that would interfere with the qPCR analysis. The General Bacteria 16S rRNA gene-targeted qPCR assay was chosen to demonstrate any interference from unidentified compounds in each sample. Upon analysis of the qPCR results, if the results determined no interfering contaminants present, then the most diluted sample that exhibited the best fit within the template DNA standard curve was chosen for further qPCR analysis (i.e., 1:10 dilution). Standard curves were prepared using synthetic linear DNA fragments obtained from GeneArt (Life Technologies, Grand Island, NY). The synthetic linear DNA fragment standard curves spanned a concentration range of approximately 10E+1 to 10E+8 target gene copies/mL and were prepared using a 10-fold serial dilution series of template DNA. All standard curves had a total of eight calibration points. qPCR targets were the Dehalococcoides mccartyi 16S rRNA gene, the Dehalobacter restrictus 16S rRNA gene, and the Dehalogenimonas sp.16S rRNA gene to determine whether the chlorinated compound-degrading bacteria were present at levels significant enough to observe degradation activity.156–158

### Microcosm Setup and Monitoring

Sediment and groundwater samples were stored at 4 °C until microcosm setup which was performed by members of the Löffler lab and the technical team. All 60-mL glass serum bottles and black butyl rubber stoppers were cleaned and autoclaved before microcosm setup (Bellco Glass, Vineland, NJ, USA). Reduced anoxic mineral salts medium (RAMM) was prepared in 300-mL serum bottles with 200 mL of RAMM and 100 mL of headspace under anoxic conditions following established procedures.159 Groundwater (600 mL) was aliquoted into 1-L glass serum bottles closed with butyl rubber stoppers and crimp caps, and it was flushed with oxygen-free nitrogen (N2) for 30 min to remove any remaining VOCs associated with the groundwater. The compounds of concern, except CFC-11 and CFC-12, were added to individual 300-mL or 1-L bottles containing sterilized RAMM and groundwater, respectively, using dedicated Hamilton glass syringes Table 13. Additionally, the vessels containing RAMM received 5 mM lactate from a 1 M anoxic stock solution. The vessels were shaken at 60 rpm for 1–2 days at room temperature to achieve equilibration before the medium was dispensed inside the anoxic chamber into individual 60-mL serum bottles. This procedure ensured that all microcosms received the same amount of the respective contaminant of concern and an electron donor (RAMM microcosms only). Since CFC-11 and CFC-12 are highly volatile, these compounds were added directly into individual microcosms after setup Table 13. CFC-11 (liquid) or CFC-12 (gas) with a volume of 3 µL or 0.3 mL, respectively, was directly injected into microcosms to achieve aqueous phase concentrations of 0.25 mM. The physical properties used to calculate the aqueous phase concentrations of each contaminant are reported in Table 14.

A sediment extrusion apparatus was designed and built to remove the aquifer material from the stainless steel core casing Figure 42. Extruded aquifer material was collected in sterilized glass Mason jars under a constant flow of sterile N2 gas to avoid air exposure. The Mason jars containing the aquifer material, the 1-L vessels with groundwater, the 1-L vessels with RAMM, the sterilized 60-mL glass serum bottles, and sterilized metal spatulas were transferred into an anoxic chamber containing 2–3% H2 with a balance of N2 Figure 43.

Following the transfer of all materials into the anoxic chamber, the aquifer material, groundwater, and RAMM were used to construct 60-mL batch microcosms containing ~6 g (wet weight) of aquifer material and 40 mL of associated groundwater or RAMM. RAMM stocks containing 5 mM lactate were used for microcosms designated to receive 5 mM lactate to test the effects of biostimulation. Lactate was added by injecting 0.2 mL of a 1 M lactate stock solution with 1-mL plastic syringes for biostimulation microcosms prepared with groundwater.

Following microcosm setup, all glass serum bottles were immediately capped with butyl rubber stoppers, removed from the anoxic chamber, and sealed with aluminum crimp caps. All microcosms were equilibrated overnight, and negative control microcosms were autoclaved on two consecutive days.

The following day, each microcosm was measured for initial concentrations of chlorinated compounds in the headspace using a gas chromatograph equipped with a flame ionization detector (GC-FID). To quantify lower CF concentrations, microcosms containing CF were analyzed using a gas chromatograph equipped with a micro-electron capture detector (GC-µECD). The GC-FID (Agilent GC 7890A) was equipped with a DB-624 column (60-m length, 320-µm diameter, and 1.8-µm film thickness) and used the Agilent OpenLab (Rev. C.01.06[61]) software (Agilent, Santa Clara, CA). The GC-µECD (Agilent GC 7890A) was equipped with an Agilent HP-PLOT/Q column (30-m length, 320-µm diameter, 20-µm film thickness) and used Agilent ChemStation version G1701EA MSD. Headspace samples were collected with gastight Hamilton #1725 syringes fitted with a Luer Lock adapter to allow for the use of sterile needles (25G 7/8). Before sampling, the Hamilton syringes were flushed with sterile N2 gas. Once inserted into the sample bottle, the syringe was flushed three times with headspace gas before 100 µL of headspace sample was collected. The gas sample was manually injected into the GC. Once all samples had been run, the peak area  for each chlorinated compound present was recorded. Standard curves were applied to calculate each contaminant’s concentrations in a given sample based on the raw peak area data. Detailed instrument settings used for the analysis of headspace samples with gas chromatography are summarized in Table 15 and Table 16. To distinguish methane from ethene in the PCE or 1,1-DCE microcosms, 100-µL headspace measurements were made using a GC-FID (Agilent GC 7890B) equipped with a J&W HP-PlotQ column (30-m length, 535-µm diameter, and 40-µm film thickness) and used the Agilent OpenLab (Rev. C.01.06[61]) software (Agilent, Santa Clara, CA).

Additionally, samples were taken from each microcosm to measure the initial lactate concentrations using high-performance liquid chromatography (HPLC). A volume of 0.6 mL of liquid was removed from each microcosm with a 1-mL, N2-flushed, sterile plastic syringe and a 25G 7/8” needle. The samples were filtered through a 0.2-µm membrane into 1.5-mL plastic Eppendorf tubes and frozen at -20 ˚C until measurements could be taken. Preparation of samples for measurement on the HPLC required the samples to be thawed and centrifuged at 10,000 x g for 5 min. Then, 300 µL from each sample was transferred to an HPLC vial, acidified with 2 µL 1 M sulfuric acid, and placed in the HPLC autosampler rack where it was injected into the HPLC.

To ensure that reducing conditions had been established in the microcosms, an additional 0.2 mM Na2S was added to all microcosms from a 100 mM sterilized stock solution. Resazurin, a redox indicator, was added to the RAMM medium to monitor the redox condition in the microcosms visually. Resazurin is pink when the redox potential is above -110 mV, and it turns clear when the redox potential is below -110 mV. Once the medium had turned clear, the bioaugmentation consortia were added to the designated microcosms Table 18. Microcosms containing PCE or 1,1-DCE were inoculated with 1.2 mL (3% v/v) of the SDC-9 consortium (APTIM), and microcosms containing CT or CF were inoculated with 1.2 mL (3% v/v) of the KB-1 Plus consortium (SiREM). Dechlorination of CFC-11 and CFC-12 by sulfate-reducing bacteria was observed.160 To stimulate the growth of sulfate-reducing bacteria and possibly enhance the degradation of CFC-11 and CFC-12, sulfate was added to some CFC-11- and CFC-12-containing microcosms Table 16. Slurries (0.5 g/mL) of FeS, ZVI, and Fe3O4 were prepared inside the glove box by weighing out the minerals and transferring them to separate 60-mL glass serum bottles and adding anoxic water using a 25-mL plastic pipette. While dispensing the three aforementioned compounds to individual microcosms, the bottles with the stock slurries were gently mixed while the suspension was withdrawn with a 3-mL syringe with an 18-gauge needle. Then 2 mL of the slurry was immediately dispensed into the designated microcosm Table 18. Each microcosm received 1 g of the designated reactive iron mineral, and all microcosms were incubated upside down in the dark under static conditions at room temperature with periodic measurements, and the results were recorded in Excel spreadsheets.

A summary of the microcosms established as part of this treatability study is provided in Table 18. All microcosms were set up in duplicate, except for some heat-killed controls, which consisted of one heat-killed microcosm containing groundwater and a second heat-killed microcosm containing RAMM. Each microcosm received a sample ID. Each sample ID was assigned a letter a–e, with the letter representing the aquifer material used to set up the microcosms as follows: (a) microcosms containing aquifer material from BHA1-12 m, (b) microcosms containing aquifer material from BHA3-7 m, (c) microcosms containing aquifer material from BHA3-15 m, (d) microcosms containing aquifer material from BHA3-24 m, and (e) microcosms containing aquifer material from BHA4-10 m. The numerical portion of the sample ID is an identifier used to log information about that microcosm. Three microcosms were mislabeled during setup and are identified as b-15-2nd, b-27-2nd, and b80-2nd.

### Standard Curve Preparation of Gas Chromatograph Equipped with a Flame Ionization Detector (GC-FID) and Gas Chromatograph Equipped with a Micro-Electron Capture Detector (GC-µECD)

Standard curves for each chlorinated compound of interest and their predicted degradation products were established Table 17. Stock solutions of PCE, TCE, cDCE, 1,1-DCE, CT, and CF were prepared in ice-cold methanol and shaken vigorously. To prepare individual standards with the desired amounts of PCE, TCE, cDCE, 1,1-DCE, CT, and CF, designated volumes of the stock solutions were transferred to 60-mL glass serum bottles containing 40 mL of anoxic Milli-Q water. The bottles were shaken at room temperature for at least 24 h prior to sampling. Stocks for VC and ethene were prepared using 125-mL glass serum bottles filled with N2 at 1 atmosphere. VC and ethene were added with 3-mL plastic syringes, and serial dilutions of the VC and ethene stock bottles were performed in 60-mL bottles to achieve the desired amounts for the standards. All bottles containing gases without liquid received approximately 3–5 glass beads so that the gasses could be efficiently mixed by shaking.

CFC-11 has a boiling point of 23.82 °C at 1 atmosphere of pressure, and the stock solution of CFC-11 at a concentration of 10 mg/mL was prepared gravimetrically in ice-cold methanol. Standards of CFC-11 at different concentrations were prepared in 60-mL serum bottles containing 40 mL of MilliQ-H2O and 20 mL of headspace. Different volumes (1 µL to 1,000 µL) of the CFC-11 stock solution (10 mg/mL) were injected into the 60-mL serum bottles to achieve the desired amounts. Furthermore, CFC-12 has a boiling point of -29.79 °C and is a gas at room temperature. Standards of CFC-12 were also prepared in 60-mL serum bottles with 40 mL of MilliQ-H2O and 20 mL of headspace. Different volumes of CFC-12 gas, ranging from 10 µL to 2,000 µL, were injected into serum bottles to achieve the desired amounts for standard curve preparation.

Standards for PCE, TCE, cDCE, and 1,1-DCE, VC, ethene, CT, CF, DCM, methane, CFC-11, and CFC-12 were measured using the GC-FID, and CF, DCM, and CM standards were measured using the GC-µECD. Standard curves for the compounds of concern and possible degradation products are depicted in Figure 44 and Figure 45.

### Data Management

Data generated during the treatability studies (i.e., HPLC, qPCR and GC measurements) were manually entered into Excel spreadsheets. Calculations of contaminant, methane, ethene and organic acid concentrations were performed in Excel, and graphs were generated. A CSV spreadsheet containing data on the percentage of actively dehalogenating microcosms was uploaded into R and used to create the heat maps Figure 9 and Figure 10. The code used to generate these figures can be found in the appendix. The qPCR data was generated by the QuantStudion 12K Flex system (Applied Biosystems) in CSV format.  The quantity of each sample was imported in to Excel and then used to calculate the gene copies/mL.147 The qPCR data is show in Table 11.  The MiHPT push data were shared with the Löffler lab as PDF files. In addition, dissolved iron data were generated by the Water Quality Core Facility at the University of Tennessee, Knoxville, and shared with the Löffler lab in the form of an Excel spreadsheet. Finally, groundwater and sediment VOC data were generated by Pace Analytical and shared with the Löffler lab in the form of Excel spreadsheets.

## Results

### MiHPT Overview

Six separate graphs are presented in Figure 5 though Figure 8 for each MiHPT push location. The Y axis for all graphs is depth, which is reported in mbgs. From left to right, the graphs display EC dipole data, followed by the PID, FID, and XSD graphs. The HPT graph illustrates average pressure in black and HPT flow in red, and finally, estimated values for K are graphed in the last panel. EC is reported in (mS/m); coarse grained particles such as sand are indicated by low EC values, and fine grained particles such as silts and clays are indicted by high EC values. The PID, FID, and XSD data is all reported in (µV x 105), so that comparisons between boreholes can be made. Furthermore, HPT pressure is reported in (kPa), and HPT flow is reported in (ml/min). The rule of thumb for HPT data is that a lower pressure and a higher flow rate indicate greater permeability in the aquifer material, while a higher pressure and a lower flow rate indicate lower permeability in the aquifer material.

### Borehole A1 (BHA1)—MiHPT Data

The PID, FID, and XSD signals from the BHA1 MiHPT push suggest the presence of a combustible compound or compounds containing double or triple bonds, and a low degree of halogenation present between 12.50 mbgs and 14 mbgs Figure 5. The compound of interest that fits these data best is 1,1-DCE, or the degradation product VC. The PID, FID, and XSD peaks at 18.50 mbgs also suggest the presence of compounds containing double or triple bonds, and a low degree of halogenation, possibly 1,1-DCE or VC. Sediment and groundwater samples were taken between 12.25 mbgs and 13.15 mbgs for microcosm setup and VOC analysis. The HPT data revealed an average pressure of ~200 kPa between 8.5 mbgs and 18.5 mbgs. These pressure data indicate an interval of lower permeability materials and a potential storage zone for contaminants in the aquifer. The measurements from the EC dipole suggest the presence of silt in the interval spanning 0–12 mbgs and a transition to clay in the interval spanning 12–19 mbgs. Dissipation tests could not be performed in BHA1 due to failure of the HPT pump, so the depth of the water table and hydraulic conductivity were not estimated in BHA1.

Figure 5. Membrane interface probe-hydraulic profiling tool data from Borehole 1.

### Borehole A2 (BHA2)—MiHPT Data

The lack of any response from the PID, FID, and XSD detectors during the BHA2 push suggests that no contaminants of interest were present at BHA2 Figure 6. Therefore, no sediment or groundwater samples were taken from this location. The HPT data indicates that in BHA2, the average pressure from 8.25 to 11.25 mbgs was ~200 kPa, thereby implying a zone of lower permeability. In BHA2 from 11.50 mbgs to 20 mbgs, the average pressure was ~750 kPa; the high pressure in this interval highlights an area with low permeability and a potential storage zone for contaminants. Furthermore, the measurements from the EC dipole suggest the presence of silt in the interval from 8.25 mbgs to 11.25 mbgs and clay in the interval from 11.50 mbgs to 20 mbgs. The depth of the water table was estimated to be 8.5 mbgs at BHA2. Estimates for hydraulic conductivity report K being as low as 0 ml per day and as high as 9 mL per day in zones containing fine-grained, lower-permeability materials and courser-grained, higher-permeability materials, respectively.

Figure 6. Membrane interface probe-hydraulic profiling tool data from Borehole 2.

### Borehole A3 (BHA3)—MiHPT Data

The PID, FID, and XSD signals from the BHA3 MiHPT push suggest the presence of a noncombustible, highly halogenated compound between 7.00 mbgs and 8.00 mbgs Figure 7. The compounds of interest that these signals best represent are CT, CFC-11, and CFC-12. The PID, FID, and XSD peaks between 14.00 mbgs and 15.00 mbgs indicate the presence of a compound or compounds containing double or triple bonds, and a medium degree of halogenation, possibly TCE, 1,1-DCE, or the degradation product VC. The PID, FID, and XSD peaks between 24.25 mbgs and 26.00 mbgs point to the presence of a combustible compound or compounds containing double or triple bonds. The lack of halogenation suggests that the compound or compounds are not contaminants of interest. These signals demonstrate the presence of hydrocarbons or possibly the degradation product ethene. Moreover, the HPT data indicates that the average pressure in BHA3 from 8.00 mbgs to 27.00 mbgs was ~750 kPa. This interval of high pressure implies an area with low permeability, and a potential storage zone for contaminants. The EC measurements in BHA3 from 8.00 mbgs to 27.00 mbgs were consistently above 100 mS/m, thus suggesting the presence of clay materials. Additionally, the water table depth at BHA3 was estimated to be 6.5 mbgs. Finally, estimates for hydraulic conductivity report K being as low as 0 ml per day and as high as 45 mL per day in zones containing fine-grained. Lower-permeability materials and in pockets containing courser-grained, higher-permeability materials, respectively.

Figure 7. Membrane interface probe-hydraulic profiling tool data from Borehole 3.

### Borehole A4 (BHA4)—MiHPT Data

The PID, FID, and XSD signals from the BHA4 MiHPT push suggest the presence of double or triple bonds, and a medium degree of halogenation, possibly TCE, 1,1-DCE, or the degradation product VC Figure 8. The HPT data indicates that the average pressure in BHA4 from 4.5 mbgs to 9.00 mbgs was ~750 kPa. The high pressure in this interval demonstrates an area with lower permeability. In BHA4 from 9.00 mbgs to 11.50 mbgs, the average pressure was ~300 kPa; this pressure points to an area of higher permeability. Furthermore, the water table was estimated to begin at ~8.75 mbgs. EC measurements greater than 100 mS/m and high HPT pressure indicate the presence of a non-porous material such as clay in the area between the ground surface and the water table. The low HPT pressure and EC measurements around 60 mS/m between 9 mbgs and 12 mbgs suggest an area containing silt. Additionally, the K estimations suggest that there is some hydraulic conductivity throughout the interval from 9 mbgs and 12 mbgs. K values in the interval between 9 m and 12 m vary between 0 ml and 20 ml per day.

Figure 8. Membrane interface probe-hydraulic profiling tool data from Borehole 4.

### Sediment and Groundwater VOC Data

Table 3 shows that no contaminants were detected in the sediment samples from BHA1 (12.85–13.00 mbgs) or BHA3 (7.40–7.55 mbgs). CT and CF were detected in the sediment sample from BHA3 (14.55–14.75 mbgs), while CF was the only contaminant detected in BHA3 (24.50–24.75 mbgs). Moreover, PCE was detected in the sediment sample from BHA4 (10.55–10.75 mbgs). The VOC analysis of groundwater from BHA1 (12.25–12.50 mbgs) and BHA3 (12.80–14.50 mbgs) identified CT, CF, and PCE Table 4. The amounts of contaminants detected was higher in the samples from BHA3 (12.80–14.50 mbgs) than in samples from BHA1 (12.25–12.50 mbgs). The highest amounts of CT, CF, CFC-11, CFC-12, PCE, and 1,1-DCE were detected in BHA3 (19.70–24.70 mbgs). No contaminants were detected in the groundwater sample from BHA4 (9.95–11.30 mbgs).

Table 3. Amounts of chlorinated VOCs associated with aquifer materials collected in four different boreholes Area A.

| Sampling Location and Depth (mbgs) | BHA1 12.85–13.00 | BHA3 7.40–7.55 | BHA3 14.55–14.75 | BHA3 24.50–24.75 | BHA4 10.55–10.75 |
| --- | --- | --- | --- | --- | --- |
| CT (mg/kg) | ND | ND | 0.268 | ND | ND |
| CF (mg/kg) | ND | ND | 0.118 | 0.0179 | ND |
| CFC-11 (mg/kg) | ND | ND | ND | ND | ND |
| CFC-12 (mg/kg) | ND | ND | ND | ND | ND |
| PCE (mg/kg) | ND | ND | ND | ND | 0.00608 |
| 1,1-DCE (mg/kg) | ND | ND | ND | ND | ND |

Table 4. Groundwater Volatile Organic Compound Data.

| Sampling Location and Depth (mbgs) | BHA1 12.25–12.50 | BHA3 12.80–14.50 | BHA3 19.70–24.70 | BHA4 9.95–11.30 |
| --- | --- | --- | --- | --- |
| CT (µg/L) | 5.77 | 488 | 4570 | ND |
| CF (µg/L) | 11.5 | 530 | 1100 | ND |
| CFC-11 (µg/L) | ND | 19.1 | 97.4 | ND |
| CFC-12 (µg/L) | ND | ND | 5.47 | ND |
| PCE (µg/L) | 1.44 | 6.38 | 10.4 | ND |
| 1,1-DCE (µg/L) | ND | ND | 1.34 | ND |

### Dissolved Iron Data

Table 4 reports the values of dissolved iron in the groundwater samples identified by ICPOES. Notably, the groundwater sample from BHA4 has a higher concentration of iron than the other samples.

Table 5. Dissolved ferrous iron in groundwater samples.

| Sample Location | Fe (mg/L) |
| --- | --- |
| BHA1 | 2.2 |
| BHA3 | 1.3 |
| BHA4 | 33.3 |
| PZ2 | 0.1 |

### Overview of Treatability Study Microcosm Performance

To generate Figure 9, microcosm performance was deemed successful if contaminant degradation occurred, and unsuccessful if degradation of the target contaminant did not occur. Percentages were determined based on the number of successful microcosms from each borehole that received the same treatment. Microcosms tested for the same remediation strategy, but set up with different aqueous phases (i.e., groundwater or RAMM) were considered to be part of the same group when determining the percent of successful microcosms. VOC concentrations over time for each contaminant, borehole, and remediation strategy can be seen in Figure 11 through Figure 41.

Degradation of 1,1-DCE, CT, CF, CFC-11, or CFC-12 was not observed in any microcosms where MNA was tested as a remediation strategy Figure 9. In contrast, degradation of PCE and the formation of TCE, cDCE, VC, and ethene was observed under MNA conditions 1 year after initial set up in all microcosms set up with aquifer material from BHA3-7 m, BHA3-15 m, and BHA4-10 m. Furthermore, no activity was observed in any heat-killed microcosms. In addition, biostimulation was not a successful remediation strategy in microcosms that contained 1,1-DCE, CT, CF, CFC-11, or CFC-12. Some transformation of PCE was observed in biostimulation microcosms set up with aquifer material from BHA3-15 m, BHA3-7 m, and BHA4-10 m. Degradation was observed in all PCE and 1,1-DCE microcosms bioaugmented with SDC-9 and biostimulated with lactate. Degradation of CF was observed in microcosms set up with aquifer material from BHA1-12 m, BHA3-15 m, and BHA3-24 m that were bioaugmented with KB-1 Plus and biostimulated with lactate. CF degradation was observed in bioaugmented microcosms set up with sediment and RAMM from BHA3-7m and BHA4-10 m, but not in microcosms where groundwater was the liquid phase. It is possible that redox conditions were not favorable for reductive dechlorination in the microcosms containing groundwater from BHA3-7 m and BHA4-10 m, but this observation is ambiguous as a redox indicator was not added to the microcosm set up with groundwater.

Additional remediation strategies were tested for CT Figure 10. Transformation of CT to CF was observed in all microcosms that were amended with ZVI or FeS. Amending CT-containing microcosms with Fe3O4 was successful in microcosms set up with aquifer material from BHA4-10 m. Similarly, combining reactive iron species, biostimulation, and bioaugmentation followed the same pattern of activity as the reactive iron species alone. However, in microcosms where reactive iron species, biostimulation, and bioaugmentation were combined, CT was degraded to non-halogenated end products Figure 14, Figure 15, Figure 21, Figure 27, Figure 33, and Figure 39.

Figure 9. Performance of monitored natural attenuation, heat kill, biostimulation, and bioaugmentation microcosms.

Figure 10. Performance of microcosms amended with reactive iron species, and combined bioaugmentation with reactive iron species.

### Lactate Fermentation and Microcosm Performance: Borehole A1 at 12-m depth (BHA1-12 m)

Lactate fermentation was observed under biostimulation and bioaugmentation conditions in all microcosms that contained PCE, 1,1-DCE, or CF Table 6. In microcosms that contained CT, lactate fermentation occurred under bioaugmentation conditions, but not under biostimulation conditions. Table 6 lists the percentage of microcosms in which lactate fermentation occurred under those conditions for microcosms that contained PCE, 1,1-DCE, CT, and CF.

Table 6. Borehole A1-12 m lactate fermentation performance

### PCE (BHA1-12 m)

No PCE degradation products including TCE, DCEs, VC, and ethene were observed in killed control microcosms (a1, a2; Figure 11). PCE degradation products were also not observed in microcosms established with aquifer material from BHA1-12 m under natural attenuation (a3) and biostimulation (a4, a5 a9, a10) conditions, regardless of whether groundwater or RAMM was used for microcosm setup. The degradation of PCE and the formation of ethene were only observed in the microcosms that received lactate and were bioaugmented with the SDC-9 consortium (a6, a7 and a11, a12). The killed control vessels were heat inactivated, and some PCE loss occurred during the autoclave cycle, which explains the lower initial PCE amounts in these vessels (e.g., a2). Some loss of PCE occurred over time, as evidenced in vessels without transformation product formation (e.g., a3, a4, a9). Additional measurements were taken for these microcosms 1 year after the initial setup, and no additional PCE degradation was apparent.

Figure 11. Borehole A1-12 m microcosm performance (PCE).

### 1,1-DCE (BHA1-12 m)

No 1,1-DCE degradation products including VC and ethene were observed in killed control microcosms (a13, a14) established with BHA1-12 m aquifer material. 1,1-DCE degradation products were also not observed in microcosms mimicking natural attenuation (a15) or biostimulation (a16, a17 and a21, a22) conditions, regardless of whether groundwater or RAMM was used for microcosm setup Figure 12. The formation of VC and ethene was only observed in microcosms amended with 5 mM of lactate and the SDC-9 consortium (a18, a19 and a23, a24). Additional measurements were taken for these microcosms 1 year after the initial setup, and no additional 1,1-DCE degradation was seen.

Figure 12. Borehole A1-12 m microcosm performance (1,1-DCE).

### CF (BHA1-12 m)

The degradation of CF and the formation of DCM were observed in bioaugmentation microcosms established with aquifer material from BHA1-12 m for microcosms established with groundwater (a30, a31) or RAMM (a35, a36), as illustrated in Figure 13. No CF degradation was observed in microcosms without the addition of the bioaugmentation consortium KB-1 Plus. Active microcosms a30, a31, a35, and a36 were re-spiked with an additional 12 mol/bottle of CF on Day 112; this is indicated by a star in Figure 13. The killed control vessels were heat inactivated, and some CF loss occurred during the autoclave cycle (e.g., a25). Some loss of CF occurred over time, as evidenced in vessels without transformation product formation (e.g., a34). Additional measurements were taken for these microcosms 1 year after the initial setup, and no additional CF degradation was observed.

Figure 13. Borehole A1-12 m microcosm performance (CF).

### CT (BHA1-12 m)

Microcosms a55–a62 were set up without aquifer material but received the KB-1 Plus culture to test whether this culture alone could mediate CT transformation Figure 14. Moreover, microcosms a55 and a56 were set up without additional reactive iron minerals to determine whether the FeS present in the KB-1 Plus consortium was sufficient to degrade the amount of CT added to the microcosms. The potential degradation products of CT (i.e., CF, DCM, CM, and methane) were not observed in microcosms a55 and a56. Microcosms a57–a62 were then amended with KB-1 Plus and a reactive iron species. Microcosms a57 and a58 contained ZVI, while microcosms a59 and a60 contained additional FeS, and microcosms a61 and a62 contained Fe3O4 (magnetite). Degradation of CT was observed in microcosms a57, a58, a59, and a60, but not in microcosms a61 and a62. Unexpectedly, the magnetite used in these experiments (50–100-nm particles purchased from Sigma-Aldrich) was not active toward CT.

In microcosms established with aquifer material from BHA1-12m, CT degradation products including CF, DCM, CM, and methane were not observed in killed control microcosms (a37, a38) or natural attenuation microcosms (a63, a64), as illustrated in Figure 15. CT degradation products were also not observed in biostimulation (a39, a40, a41, a42) and bioaugmentation (a47, a48) microcosms, independent of whether groundwater or RAMM was used as the liquid phase. CT was transformed to CF in microcosms amended with ZVI (a43, a44) or FeS (a83, a84). However, degradation products were not observed in microcosms amended with Fe3O4 (a85, a86). In microcosms amended with KB-1 Plus and additional ZVI (a49, a50) or FeS (a51, a52), the degradation products CF and DCM were observed. Degradation products were also not observed in microcosms amended with KB-1 Plus and Fe3O4 (a53, a54). Furthermore, in heat-killed control vessels, some CT loss occurred during the autoclave cycle (e.g., a38). Some loss of CT occurred over time, as evidenced in vessels without transformation product formation (e.g., a46, a56, a64, a86). Active microcosms (a43, a44, a83, a84, a49, a50, a51, a52, a57, a58, a59, a60) were re-spiked with an additional 10 mol/bottle of CT on Day 118, as indicated by a star in Figure 15. Additional measurements were taken for these microcosms 1 year after the initial setup, and no additional CT degradation was observed.

Figure 14. Microcosm setup to test the activity of KB-1 Plus on CT in the absence of aquifer material.

Figure 15. Borehole A1-12 m microcosm performance (CT).

### CFC-11 (BHA1-12 m)

During a 7-month monitoring period, no degradation of CFC-11 was observed in any of the microcosms Figure 16. Slight decreases in CFC-11 concentrations were observed during the first 2 month of incubation.

Figure 16. Borehole A1-12 m microcosm performance (CFC-11).

### CFC-12 (BHA1-12 m)

CFC-12 was recalcitrant in all microcosms. During the 7-month monitoring period, no degradation of CFC-12 was observed in microcosms without additions (natural attenuation), biostimulation with lactate, biostimulation with sulfate, or biostimulation with lactate combined with bioaugmentation with the KB-1 Plus consortium Figure 17.

Figure 17. Borehole A1-12 m microcosm performance (CFC-12).

### Lactate Fermentation and Microcosm Performance: Borehole A3 at 7-m Depth (BHA3-7 m)

Lactate fermentation was observed under biostimulation conditions in all microcosms that contained PCE or CF Table 7. In microcosms that contained CT, lactate fermentation under biostimulation conditions was observed in microcosms that contained RAMM as the liquid phase (b43, b44), but not in microcosms that contained groundwater as the liquid phase (b41, b42). In addition, in microcosms containing 1,1-DCE lactate, fermentation occurred under biostimulation conditions in microcosms b16, b17, and b22, but not in b21, which contained RAMM as the liquid phase. Lactate fermentation was observed under bioaugmentation conditions in all microcosms that contained PCE, 1,1-DCE, CF, or CT Table 7. Table 7 lists the percentage of microcosms in which lactate fermentation occurred under biostimulation and bioaugmentation conditions for microcosms that contained PCE, 1,1-DCE, CT, and CF.

Table 7. Borehole A3-7 m lactate fermentation performance.

### PCE (BHA3-7 m)

No PCE degradation products including TCE, DCEs, VC, and ethene were observed in killed control microcosms (b1, b2). PCE degradation products were also not observed in microcosms established with aquifer material from BHA3-7 m under natural attenuation (b3, b3-2nd) and biostimulation (b4, b5 b9, b10) conditions, independent of whether groundwater or RAMM was used for microcosm setup Figure 18. Degradation of PCE and the formation of ethene were only observed in the microcosms that received lactate and were bioaugmented with the SDC-9 consortium (b6, b7 and b11, b12). The killed control vessels were heat inactivated, and some PCE loss occurred during the autoclave cycle (e.g., b2). Some loss of PCE occurred over time, as evidenced in vessels without transformation product formation (e.g., b3, b4, b9). Additional measurements were taken for these microcosms 1 year after the initial setup, and no additional PCE degradation was seen, with the exception of one microcosm (b10) initially established in RAMM and biostimulated with 5 mM lactate. Further investigation is required to confirm degradation.

Figure 18. Borehole A3-7 m microcosm performance (PCE).

### 1,1-DCE (BHA3-7 m)

No 1,1-DCE degradation products including VC and ethene were observed in killed control microcosms (b13, b14) established with BHA3-7 m aquifer material. 1,1-DCE degradation products were also not observed in microcosms mimicking natural attenuation (b15, b15-2nd) or biostimulation (b16, b17 and b21, b22) conditions, regardless of whether groundwater or RAMM was used for microcosm setup Figure 19. The formation of VC and ethene was only observed in microcosms amended with 5 mM lactate and the SDC-9 consortium (b18, b19 and b23, b24). Additional measurements were taken for these microcosms 1 year after the initial setup, and no additional 1,1-DCE degradation was seen.

Figure 19. Borehole A3-7 m microcosm performance (1,1-DCE).

### CF (BHA3-7 m)

The degradation of CF and the formation of DCM were observed in bioaugmentation microcosms established with aquifer material from BHA3-7 m for microcosms established with RAMM (b35, b36), as illustrated in Figure 20. No CF degradation was observed in groundwater microcosms with or without bioaugmentation with consortium KB-1 Plus. In contrast, CF degradation was observed in microcosms established with RAMM and bioaugmented with consortium KB-1 Plus (b35, b36). Active microcosms were re-spiked with an additional 12 mol/bottle of CF on Day 102, as indicated by a star in Figure 20. Some loss of CF occurred over time, as evidenced in vessels without transformation product formation (e.g., b29). Additional measurements were taken for these microcosms 1 year after the initial setup, and no additional CF degradation was observed.

Figure 20. Borehole A3-7 m microcosm performance (CF).

### CT (BHA3-7 m)

In microcosms established with aquifer material from BHA3-7 m, CT degradation products including CF, DCM, CM, and methane were not observed in killed control microcosms (b39, b40), as illustrated in Figure 21. CT degradation products were also not observed in biostimulation (b41, a42, b43, b44) and bioaugmentation (b51, b52, b53, b54) conditions, regardless of whether groundwater or RAMM was used as the liquid phase. However, CT was degraded to CF in near stoichiometric ratios when ZVI (b45, b46) or FeS (b47, b48) were added to the system. Furthermore, degradation products were not observed in microcosms amended with Fe3O4 (b49, b50), whereas in microcosms amended with KB-1 Plus and ZVI (b55, b56) or FeS (b57, b58), the degradation products CF and DCM were observed. Degradation products were not observed in microcosms amended with KB-1 Plus and Fe3O4 (b59, b60). Active microcosms (b45, b46, b47, b48, b55, b56, b57, b58) were re-spiked with an additional 10 mol/bottle of CT on Day 102, as indicated by a star in Figure 21. Some loss of CT occurred over time, as evidenced in vessels without transformation product formation (e.g., b51, b59). The obvious CT concentration decline in the heat-killed controls (b39, b40) was likely due to improper handling—these microcosms were accidently incubated in an upright position, causing increased sorption to the rubber stopper. Additional measurements were taken for these microcosms 1 year after the initial setup, and no additional CT degradation was observed.

Figure 21. Borehole A3-7 m microcosm performance (CT).

### CFC-11 (BHA3-7 m)

During the 7-month monitoring period, no degradation was observed in microcosms that tested natural attenuation capacity or that were biostimulated with either lactate, sulfate, or lactate combined with bioaugmentation with consortium KB-1 Plus Figure 22. Figure 22 illustrates the CFC-11 microcosm performance for Borehole A3-7 m.

Figure 22. Borehole A3-7 m microcosm performance (CFC-11).

### CFC-12 (BHA3-7 m)

During the 7-month monitoring period, no degradation of CFC-12 was observed in microcosms testing natural attenuation capacity. Degradation was also not observed in microcosms that were biostimulated with either lactate, sulfate, or lactate combined with bioaugmentation with consortium KB-1 Plus Figure 23. Figure 23 illustrates the CFC-11 microcosm performance for Borehole A3-7 m.

Figure 23. Borehole A3-7 m microcosm performance (CFC-12).

### Lactate Fermentation and Microcosm Performance: Borehole A3 at 15-m Depth (BHA3-15 m)

Lactate fermentation was observed under biostimulation and bioaugmentation conditions in all microcosms that contained PCE, 1,1-DCE, CF, or CT Table 8. Table 8 lists the percentage of microcosms in which lactate fermentation occurred under those two conditions.

Table 8. Borehole A3-15 m lactate fermentation performance.

### PCE (BHA3-15 m)

No PCE degradation products including TCE, DCEs, VC, and ethene were observed in killed control microcosms (c1, c2), nor were such products observed in microcosms established with aquifer material from BHA3-15 m under natural attenuation (c3) and biostimulation (c4, c5 c9, c10) conditions, independent of whether groundwater or RAMM was used for microcosm setup Figure 24. Degradation of PCE and the formation of ethene were only observed in the microcosms that received lactate and were bioaugmented with the SDC-9 consortium (c6, c7 and c11, c12). Furthermore, the killed control vessels were heat inactivated, and some PCE loss occurred during the autoclave cycle (e.g., c2). Some loss of PCE also occurred over time, as evidenced in vessels without transformation product formation (e.g., c3, c4, c9). As with all previously mentioned experiments, additional measurements were taken for these microcosms 1 year after the initial setup. Potential additional PCE degradation was seen under certain conditions, namely, natural attenuation (c3) and biostimulation with 5 mM lactate (c4, c5, c10). However, further investigation is required to confirm degradation.

Figure 24. Borehole A3-15 m microcosm performance (PCE).

### 1,1-DCE (BHA3-15 m)

No 1,1-DCE degradation products including VC and ethene were observed in killed control microcosms (c13, c14) established with BHA3-15 m aquifer material. 1,1-DCE degradation products were also not observed in microcosms mimicking natural attenuation (c15) or biostimulation (c16, c17 and c21, c22) conditions, regardless of whether groundwater or RAMM was used for microcosm setup Figure 25. The formation of VC and ethene was only observed in microcosms amended with 5 mM lactate and the SDC-9 consortium (c18, c19 and c23, c24). Additional measurements were taken for these microcosms 1 year after the initial setup, and no additional 1,1-DCE degradation was seen.

Figure 25. Borehole A3-15 m microcosm performance (1,1-DCE).

### CF (BHA3-15 m)

Degradation of CF and the formation of DCM were observed in bioaugmentation microcosms established with aquifer material from BHA3-15 m established with groundwater (c30, c31) or RAMM (c35, c36), as illustrated in Figure 26. In contrast, no CF degradation was observed in microcosms without the addition of the bioaugmentation consortium KB-1 Plus. Active microcosms c30, c31, c35, and c36 were re-spiked with an additional 12 mol/bottle of CF on Day 105, as indicated by a star in Figure 26. The killed control vessels were heat inactivated, and some CF loss occurred during the autoclave cycle, which explains the lower initial CF amounts in these vessels (e.g., c26). Some loss of CF also occurred over time, as evidenced in vessels without transformation product formation (e.g., c27, c28, c33). Additional measurements were taken for these microcosms 1 year after the initial setup, and no additional CF degradation was observed.

Figure 26. Borehole A3-15 m microcosm performance (CF).

### CT (BHA3-15 m)

Degradation of CT was not observed in heat-killed controls and in biostimulation or bioaugmentation microcosms. In addition, in microcosms established with aquifer material from BHA3-15 m, potential CT degradation products including CF, DCM, CM, and methane were not observed in killed control microcosms (c37, c38), as illustrated in Figure 27. CT degradation products were also not observed in biostimulation (c39, c40, c41, c42) and bioaugmentation (c49, c50, c51, c52) microcosms, independent of whether groundwater or RAMM was used as the liquid phase. However, CT was degraded to CF in near stoichiometric ratios when ZVI (c43, c44) or FeS (c45, c46) was added to the system. Moreover, degradation products were not observed in microcosms amended with Fe3O4 (c47, c48), but in microcosms amended with KB-1 Plus and additional ZVI (c53, c54) or FeS (c55, c56), the degradation products CF and DCM were observed. Degradation products were also not observed in microcosms amended with KB-1 Plus and Fe3O4 (c57, c58). Active microcosms (c43, c44, c45, c46, c53, c54, c55, c56) were re-spiked with an additional 10 mol/bottle of CT on Day 106, as indicated by a star in Figure 27. The killed control vessels were heat inactivated, and some CT loss occurred during the autoclave cycle (e.g., c38). Furthermore, some loss of CT occurred over time, as evidenced in vessels without transformation product formation (e.g., c36, c48, c49, c58). Additional measurements were taken for these microcosms 1 year after the initial setup, and no additional CT degradation was observed.

Figure 27. Borehole A3-15 m microcosm performance (CT).

### CFC-11 (BHA3-15 m)

During the 7-month monitoring period, no degradation of CFC-11 was observed in microcosms testing natural attenuation capacity, or microcosms that were biostimulated with either lactate, sulfate, or lactate combined with bioaugmentation with consortium KB-1 Plus Figure 28. Figure 28 illustrates the CFC-11 microcosm performance for Borehole A3-15 m.

Figure 28. Borehole A3-15 m microcosm performance (CFC-11).

### CFC-12 (BHA3-15 m)

During the 7-month monitoring period. No degradation of CFC-12 was observed in microcosms testing natural attenuation capacity or in microcosms that were biostimulated with either lactate, sulfate, lactate combined with bioaugmentation with consortium KB-1 Plus Figure 29. Figure 29 illustrates the CFC-12 microcosm performance for Borehole A3-15 m.

Figure 29. Borehole A3-15 m microcosm performance (CFC-12).

### Lactate Fermentation and Microcosm Performance: Borehole A3 at 24-m Depth (BHA3-24 m)

Lactate fermentation was observed under biostimulation conditions in all microcosms that contained PCE, CT, or CF Table 9. In microcosms that contained 1,1-DCE, lactate fermentation under biostimulation conditions was observed in microcosms that contained RAMM as the liquid phase (d21, d22), but not in microcosms that contained groundwater as the liquid phase (d15, d16). Table 9 lists the percentage of microcosms in which lactate fermentation occurred under those two conditions for microcosms that contained PCE, 1,1-DCE, CT, and CF.

Table 9. Borehole A3-24 m lactate fermentation performance

### PCE (BHA3-24 m)

No PCE degradation products including TCE, DCEs, VC, and ethene were observed in killed control microcosms (d1, d2). PCE degradation products were also not observed in microcosms established with aquifer material from BHA3-24 m under natural attenuation (d3) and biostimulation (d4, d5 d9, d10) conditions, regardless of whether groundwater or RAMM was used for microcosm setup Figure 30. Degradation of PCE and the formation of ethene were only observed in the microcosms that received lactate and were bioaugmented with the SDC-9 consortium (d6, d7 and d11, d12). The killed control vessels were heat inactivated, and some PCE loss occurred during the autoclave cycle (e.g., d2). In addition, some loss of PCE occurred over time, as evidenced in vessels without transformation product formation (e.g., d3, d4, d9). Additional measurements were taken for these microcosms 1 year after the initial setup, and no additional PCE degradation was seen.

Figure 30. Borehole A3-24 m microcosm performance (PCE).

### 1,1-DCE (BHA3-24 m)

No 1,1-DCE degradation products including VC and ethene were observed in killed control microcosms (d13, d14) established with BHA3-24 m aquifer material, nor were those degradation products observed in microcosms mimicking natural attenuation (d15) or biostimulation (d16, d17 and d21, d22) conditions, independent of whether groundwater or RAMM was used for microcosm setup Figure 31. The formation of VC and ethene was only observed in microcosms amended with 5 mM lactate and the SDC-9 consortium (d18, d19 and d23, d24). Additional measurements were taken for these microcosms 1 year after the initial setup, and no additional 1,1-DCE degradation was seen.

Figure 31. Borehole A3-24 m microcosm performance (1,1-DCE.)

### CF (BHA3-24 m)

Degradation of CF was observed in bioaugmentation microcosms established with aquifer material from BHA3-24 m for microcosms established with groundwater (d30, d31) or RAMM (d35, d36), as illustrated in Figure 32. Furthermore, the production of DCM was observed in microcosms d30 and d31, but not in microcosms d35 and d36. DCM was presumably further degraded in microcosms d35 and d36. Moreover, no CF degradation was observed in microcosms without the addition of the bioaugmentation consortium KB-1 Plus. Active microcosms d30, d31, d35, and d36 were re-spiked with an additional 12 mol/bottle of CF on Day 112, as indicated by a star in Figure 32. Some loss of CF occurred over time, as evidenced in vessels without transformation product formation (e.g., d29). Additional measurements were taken for these microcosms 1 year after the initial setup; degradation of CF in bioaugmented microcosms that contained groundwater (d30 and d31) has concluded, and no chlorinated methanes were identified in these microcosms.

Figure 32. Borehole A3-24 m microcosm performance (CF).

### CT (BHA3-24 m)

Degradation of CT was not observed in heat-killed controls, nor in biostimulation and bioaugmentation microcosms. Moreover, in microcosms established with aquifer material from BHA3-24 m, CT degradation products including CF, DCM, CM, and methane were not observed in killed control microcosms (d37, d38), as Figure 33 illustrates. CT degradation products were also not observed in biostimulation (d39, d40, d41, d42) and bioaugmentation (d49, d50, d51, d52) microcosms, regardless of whether groundwater or RAMM was used as the liquid phase. However, CT was degraded to CF in near stoichiometric ratios when ZVI (d43, d44) or FeS (d45, d46) was added to the system. Furthermore, degradation products were not observed in microcosms amended with Fe3O4 (d47, d48), whereas in microcosms amended with KB-1 Plus and ZVI (d53, d54) or FeS (d55, cd6), the degradation products CF and DCM were observed. Degradation products were also not observed in microcosms amended with KB-1 Plus and Fe3O4 (d57, d58). Active microcosms (d43, d44, d45, d46, d53, d54, d55, d56) were re-spiked with an additional 10 mol/bottle of CT on Day 113, as indicated by a star in Figure 33. Some loss of CT occurred over time, as evidenced in vessels without transformation product formation (e.g., d39, d41, d48, d50, d51, d60). Additional measurements were taken for these microcosms 1 year after the initial setup, and no additional CT degradation was observed.

Figure 33. Borehole A3-24 m microcosm performance (CT).

### CFC-11 (BHA3-24 m)

During the 7-month monitoring period, no degradation of CFC-11 was observed in microcosms testing natural attenuation capacity or in microcosms that were biostimulated with either lactate, sulfate, or lactate combined with bioaugmentation with consortium KB-1 Plus, as illustrated in Figure 34. Measurements after an extended incubation period of around 1 year revealed no significant degradation of CFC-11 in all microcosms, and CFC-11 remained persistent in the microcosms.

Figure 34. Borehole A3-24 m microcosm performance (CFC-11).

### CFC-12 (BHA3-24 m)

During the 7-month monitoring period at BHA3-24 m, no degradation of CFC-12 was observed in any of the microcosms (i.e., those that tested natural attenuation capacity or that were biostimulated with either lactate, sulfate, or lactate combined with bioaugmentation with consortium KB-1 Plus; Figure 35. Measurements after an extended incubation period of approximately 1 year showed no significant degradation of CFC-12 in all microcosms, and CFC-12 remained persistent in the microcosms.

Figure 35. Borehole A3-24 m microcosm performance (CFC-12).

### Lactate Fermentation and Microcosm Performance: Borehole A4 at 10-m Depth (BHA4-10 m)

Lactate fermentation was observed under biostimulation and bioaugmentation conditions in all microcosms that contained PCE, 1,1-DCE, CF, or CT Table 10. Table 10 presents the percentage of microcosms in which lactate fermentation occurred under those two conditions for the aforementioned microcosms.

Table 10. Borehole A4-10 m lactate fermentation performance.

### PCE (BHA4-10 m)

No PCE degradation products including TCE, DCEs, VC, and ethene were observed in killed control microcosms (e1, e2). PCE degradation products were also not observed in microcosms established with aquifer material from BHA4-10 m under natural attenuation (e3, e4) and biostimulation (e4, e5 e9, e10) conditions, independent of whether groundwater or RAMM was used for microcosm setup Figure 36 in the first 9 months of observation. Degradation of PCE and the formation of ethene were only observed in the microcosms that received lactate and were bioaugmented with the SDC-9 consortium (e6, e7 and e11, e12). The killed control vessels were heat inactivated, and some PCE loss occurred during the autoclave cycle (e.g., e2). In addition, some loss of PCE occurred over time, as evidenced in vessels without transformation product formation (e.g., e3, e4, e9). Additional measurements were taken for these microcosms 1 year after the initial setup. Potential additional PCE degradation was seen under biostimulated conditions in microcosms created with groundwater (e5, e6) as well as those created with RAMM (e9, e10). However, further investigation is required to confirm degradation.

Figure 36. Borehole A4-10 m microcosm performance (PCE).

### 1,1-DCE (BHA4-10 m)

No 1,1-DCE degradation products including VC and ethene were observed in killed control microcosms (e13, e14) established with BHA4-10 m aquifer material. Such products were also not observed in microcosms mimicking natural attenuation (e15, e16) or biostimulation (e17, e18 and e21, e22) conditions, regardless of whether groundwater or RAMM was used for microcosm setup Figure 37. The formation of VC and ethene was only observed in microcosms amended with 5 mM lactate and the SDC-9 consortium (e18, e19 and e23, e24). Additional measurements were taken for these microcosms 1 year after the initial setup. No additional 1,1-DCE degradation was seen.

Figure 37. Borehole A4-10 m microcosm performance (1,1-DCE).

### CF (BHA4-10 m)

Degradation of CF and the formation of DCM were observed in bioaugmentation microcosms established with aquifer material from BHA4-10 m for microcosms established with RAMM (e35, e36), depicted in Figure 38. However, no CF degradation was observed in microcosms established with KB-1 Plus and groundwater, or without the addition of the bioaugmentation consortium KB-1 Plus. Active microcosms e35 and e36 were re-spiked with an additional 12 mol/bottle of CF on Day 102, as indicated by a star in Figure 38. The killed control vessels were heat inactivated, and some CF loss occurred during the autoclave cycle (e.g., e25). Some loss of CF also occurred over time, as evidenced in vessels without transformation product formation (e.g., e27, e30). Additional measurements were taken for these microcosms 1 year after initial setup, and no additional CF degradation was observed.

Figure 38. Borehole A4-10 m microcosm performance (CF).

### CT (BHA4-10 m)

In microcosms established with aquifer material from BHA4-10 m, potential CT degradation products including CF, DCM, CM, and methane were not observed in killed control microcosms (e37, e38), as illustrated in Figure 39. CT degradation products were also not observed in biostimulation (e39, e40, e41, e42) and bioaugmentation (e49, e50, e51, e52) microcosms, independent of whether groundwater or RAMM was used as the liquid phase. Furthermore, CT was degraded to CF in near stoichiometric ratios when ZVI (e43, e44), FeS (e45, e46), or Fe3O4 (e47, e48) was added to the system. In microcosms amended with KB-1 Plus and additional ZVI (e53, e54), FeS (e55, ed6), or Fe3O4 (e57, e58), the degradation products CF and DCM were observed. Some loss of CT occurred over time, as evidenced in vessels without transformation product formation (e.g., e38, e41, e50, e52). By accident, all of the microcosms established with BHA4-10 m aquifer material were incubated with the stopper up and the headspace in contact with the stopper between Day 60 and Day 94, resulting in increased CT loss due to sorption. Active microcosms and microcosms that experienced CT loss (e37, e37, e41, e42, e43, e44, e45, e56, e47, e48, e49, e51, e52, e53, e54, e55, e56, e57, e58) were re-spiked with an additional 10 mol/bottle of CT on Day 104, as indicated by a star in Figure 39. Additional measurements were taken for these microcosms 1 year after the initial setup, and no additional CT degradation was observed.

Figure 39. Borehole A4-10 m microcosm performance (CT).

### CFC-11 (BHA4-10 m)

Degradation of CFC-11 was not observed in any microcosms during the 7-month monitoring period—neither in microcosms testing natural attenuation capacity nor in microcosms that were biostimulated with either lactate, sulfate, or lactate combined with bioaugmentation with consortium KB-1 Plus Figure 40. Figure 40 depicts the CFC-11 microcosm performance at BHA4-10 m.

Figure 40. Borehole A4-10 m microcosm performance (CFC-11).

### CFC-12 (BHA4-10 m)

During the 7-month monitoring period, no degradation of CFC-12 was observed in any microcosms (i.e., not in microcosms testing natural attenuation capacity, nor in microcosms that were biostimulated with one of the following: lactate, sulfate, or lactate combined with bioaugmentation with consortium KB-1 Plus) Figure 41. Figure 41 illustrates the CFC-12 microcosm performance at BHA4-10 m.

Figure 41. Borehole A4-10 m microcosm performance (CFC-12).

### qPCR Data from Sterivex Filters.

qPCR analysis was used to determine the abundance of OHRB (i.e., Dhc, Dhb, and Dhgm) in the groundwater from Sterivex cartridges collected at the site. The abundance of reductive dechlorination biomarkers (i.e., tceA, vcrA, bvcA, and cerA genes) is typically used to demonstrate reductive dechlorination activity in groundwater; however, to reliably determine the abundance of those biomarkers, sufficient quantities of OHRB must be present (Clark et al., 2018). A cell titer of 107 Dhc cell per liter is regarded as a threshold for sustaining generally useful rates of reductive dechlorination. Due to the low abundance of Dhc, Dhb, and Dhgm 16S rRNA genes in the groundwater, RDase genes were not quantified Table 11. The qPCR analysis demonstrated that the Dhc 16S rRNA gene was not detected in any of the borehole locations that were sampled in this study. The qPCR analysis also revealed that the Dhb 16S rRNA gene was present in boreholes BER-RT1-BHA4-SF1 (ST-5A), BER-RT1-BHA4-SF5 (ST-17A), and BER-RT1-BHA4-SF6 (ST-6B) at an abundance of approximately 10E+2 cells per liter. The Dhb 16S rRNA gene was not detected in any other borehole. Finally, based on the qPCR analysis, the Dhgm 16S rRNA gene was not detected in any of the boreholes.

Table 11. qPCR data from Sterivex filters collected at site A.

| Sample Name | Volume of Sample Filtered (mL) | DNA Conc. (ng/µl) | Volume of purified DNA (µl) | Dehalococcoides 16S rRNA |  | Dehalobacter 16S rRNA |  | Dehalogenimonas 16S rRNA |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Ave Gene copies/ml | STD Error | Ave Gene copies/ml | STD Error | Ave Gene copies/ml | STD Error |
| BER-RT1-BHA3-SF-1 (ST-2A) | 760 | 0.50 | 100 | ND | ND | DNQ | DNQ | ND | ND |
| BER-RT1-BHA3-SF-2 (ST-3A) | 760 | 0.60 | 100 | ND | ND | DNQ | DNQ | ND | ND |
| BER-RT1-BHA3-SF4 (ST-1A) | 760 | 0.36 | 100 | ND | ND | DNQ | DNQ | ND | ND |
| BER-RT1-BHA3-SF3 (ST-4B) | 790 | 0.37 | 100 | ND | ND | DNQ | DNQ | ND | ND |
| BER-RT1-BHA4-SF1 (ST-5A) | 790 | 0.38 | 100 | ND | ND | 170 | 6.10 | ND | ND |
| BER-RT1-BHA4-SF2 (ST-8A) | 310 | too low | 100 | ND | ND | DNQ | DNQ | ND | ND |
| BER-RT1-BHA4-SF3 (ST-7B) | 630 | 0.08 | 100 | ND | ND | DNQ | DNQ | ND | ND |
| BER-RT1-BHA4-SF4 (ST-9A) | 520 | 0.18 | 100 | ND | ND | DNQ | DNQ | ND | ND |
| BER-RT1-BHA4-SF5 (ST-17A) | 500 | 0.33 | 100 | ND | ND | 117 | 3.32 | ND | ND |
| BER-RT1-BHA4-SF6 (ST-6B) | 520 | 0.32 | 100 | ND | ND | 134 | 9.75 | ND | ND |

DNQ = Detected Not Quantifiable

ND = Not Detected

## Discussion

Data from the MiHPT pushes and VOC analysis of the sediment and groundwater provide information on contaminant distribution in Area A. Data from the MiHPT pushes and groundwater samples analyzed for VOCs identified a broader range of contaminants and higher contaminant concentrations at sampling locations BHA1 and BHA3 than in samples from BHA2 or BHA4. These data suggest that additional sampling near the diagonal southern boundary of Area A could be useful to further characterize the site.

The microcosm treatability study evaluated the potential for anaerobic microbial degradation of the target contaminants in microcosms established with site material collected from three borehole locations in Area A. The treatability study explored contaminant transformation and degradation under anoxic conditions because prior research has demonstrated that the target contaminants (e.g., PCE, 1,1-DCE, and CF) are susceptible to anaerobic degradation. The study tested the potential of native microbes (i.e., microorganisms associated with the aquifer materials and the groundwater collected from the site) to degrade the target contaminants (i.e., natural attenuation potential). First, microcosms amended with lactate tested whether the addition of an electron donor would have a positive effect on the degradation of the target contaminants by native microorganisms (i.e., biostimulation). Second, bioaugmentation combined with biostimulation tested the potential for initiating biodegradation of the target contaminants following the injection of biodegrading microbial consortia (i.e., consortium SDC-9 and consortium KB-1 Plus). Third, additional microcosms tested the potential for abiotic contaminant transformation (i.e., ZVI, FeS, and Fe3O4).

The microbial reductive dehalogenation process requires anoxia and low redox conditions. To facilitate the establishment of redox conditions conducive to microbial reductive dehalogenation, all microcosms received 0.2 mM of sodium sulfide as a reductant. The chemical resazurin changes color from pink to clear at a redox potential of -110 mV and serves as a visual indicator of whether reducing conditions have been established in the vessels. Resazurin was present in the microcosms with RAMM, and a single dose of 0.2 mM sulfide was observed to be insufficient to lower the redox potential in all of the microcosms, as was indicated by the pink sheen of the liquid phase. Additional sulfide was added to establish reducing conditions in the microcosms. The microcosm setup followed stringent procedures that avoided oxygen exposure and intrusion, and the observations indicate that the aquifer material was oxidizing with a positive redox potential. Groundwater samples should be analyzed for dissolved oxygen, methane, and ferrous iron to obtain additional information about the redox state of the aquifer. The keystone bacterial population involved in the degradation of the target compounds are strict anaerobes. The qPCR analysis of biomass collected from Area A groundwater samples provided little to no evidence of measurable biomarker genes of OHRB known to degrade chlorinated solvents, indicating that the site aquifer in Area A most likely does not harbor bacteria capable of degrading the target contaminants under anoxic conditions. The low abundance of known OHRB could be due to oxic aquifer conditions.

Following the addition of extra reductant (i.e., sulfide), reducing conditions were established in all microcosms included in the treatability study. The degradation of target contaminants was not observed under MNA or biostimulation conditions in microcosms contain any of the other target contaminants after a 7-month incubation period. Some degradation was observed in a subset of PCE-containing microcosms set up under MNA or biostimulation conditions from BHA3-7 m, BHA3-15 m, and BHA4-10 m after a 1-year incubation period. These microcosms have been transferred, and further monitoring is underway. These observations suggest that OHRB capable of degrading the target contaminants may be present in low abundances and are heterogeneously distributed throughout Area A aquifer.

A decline in the concentration of PCE, CT, CF, and CFC-11 without the formation of transformation products was observed in microcosms from all sampling locations and conditions. This decline in concentration is attributed to sorption onto the aquifer material or the stopper. It does not indicate degradation as it occurs in the heat-killed controls and transformation products were not observed.

The degradation of PCE and 1,1-DCE was observed in microcosms that received the bioaugmentation consortium SDC-9 and lactate as an electron donor. Ethene was observed as a dechlorination product, indicating that complete reductive dechlorination occurred and a non-toxic and environmentally benign product was formed. Furthermore, the degradation of CF was observed in microcosms amended with the bioaugmentation consortium KB-1 Plus. DCM was observed as a transformation product, and it was further degraded to innocuous products. These results indicate that the aquifer material does not contain inhibitory chemicals that would inhibit the growth of OHRB. Additionally, bioaugmentation combined with biostimulation can establish conditions conducive to reductive dechlorination and lead to the degradation and detoxification of chlorinated ethenes and CF.

CT is a strong inhibitor of bacteria involved in the degradation of the other target contaminants. Therefore, CT removal should be the first step in a remediation strategy and implemented prior to biological remedies for the other contaminants. CT is susceptible to abiotic degradation and readily reacts with ZVI and FeS. The microcosm study demonstrated rapid transformation of CT following the addition of ZVI and FeS, and CF was observed as the transformation product. Data from the bioaugmentation microcosms indicate that the amount of FeS present in KB-1 Plus is insufficient to transform the CT present in the microcosms vessels. Transformation of CT to nonhalogenated products was observed in microcosms where ZVI or FeS was added to combined bioaugmentation and biostimulation conditions. CT was also reported to be susceptible to degradation mediated by Fe3O4; however, the experiments performed in this treatability study failed to demonstrate CT degradation with Fe3O4 in microcosms set up with aquifer material from all sampling locations except for BHA4-10 m. The elevated dissolved iron concentration observed in samples from BHA4 could explain why CT was transformed to CF in microcosms amended with Fe3O4 and aquifer material from BHA4-10 m. Some literature suggests that surface-associated Fe(II) may be the principal reductant of CT in the presence of Fe3O4. Several reports in the literature suggest that the reactivity of Fe3O4 toward chlorinated solvents depends on the availability of ferrous iron in the system.161–164

In microcosms with CT that were bioaugmented with consortium KB-1 Plus and amended with ZVI or FeS, CF and DCM were observed as transformation products. Both CF and DCM were degraded, and complete removal of chlorinated methanes was achieved. This is a relevant finding and suggests that a combined abiotic/biotic treatment can remove CT and chlorinated transformation products (i.e., CF and DCM).

## Conclusions

This research aimed to characterize the contaminant profile in Area A, determine the presence and abundance of biomarker genes for OHRB at sampling locations, and determine the potential of biostimulation, bioaugmentation, and amendments with reactive iron species as remediation strategies. The MiHPT, and VOC data suggests that the region of highest contamination in Area A is along the diagonal southern boundary of Area A. Performance of the MNA and biostimulation microcosms along with the qPCR data suggest that known OHRBs are absent or present in low abundances in Area A. Bioaugmentation combined with biostimulation is a promising treatment approach to detoxify chlorinated ethenes as well as CF and DCM. However, CT removal should be the first step if the goal is the subsequent implementation of an in situ remedial strategy. The microcosm study suggested that in situ CT removal can occur via ZVI treatment. The treatability study provided no evidence of CFC-11 or CFC-12 degradation, and more research is needed to determine an effective remediation strategy for these contaminants.

## Appendix

Table 12. Sampling depth intervals for aquifer material collected in Area A.

| Borehole ID | Depth (mbgs) |
| --- | --- |
| BHA1 | 12.25 – 13.15 |
| BHA3 | 7.55 – 8.60 |
| BHA3 | 13.90 – 14.90 |
| BHA3 | 24.20 – 25.00 |
| BHA4 | 10.10 – 11.20 |

mbgs: meters below ground surface

Table 13. Chlorinated compounds added to microcosms.

| Chlorinated compound | Volume (µL) added | Target aqueous concentration in microcosms (mM) |
| --- | --- | --- |
| PCE | - 8 µL to 300-mL vessels containing 200 mL of RAMM - 20 µL to 1-L vessels containing 600 mL of groundwater | 0.2 |
| 1,1-DCE | - 8 µL to 300-mL vessels containing 200 mL RAMM - 20 µL to 1-L vessels containing 600 mL of groundwater | 0.2 |
| CFC-11 | - 3 µL of neat CFC-11 liquid was directly injected into individual microcosms | 0.25 |
| CFC-12 | - 0.3 mL of neat CFC-12 gas was directly injected into individual microcosms | 0.25 |
| CT | - 4 µL to 300-mL vessels containing 200 mL of RAMM - 12 µL to 1-L vessels containing 600 mL of groundwater | 0.1 |
| CF | - 2 µL to 300-mL vessels containing 200 mL RAMM - 6 µL to 1-L vessels containing 600 mL of groundwater | 0.1 |

Table 14. Physical properties of target contaminates and projected transformation products.

| Chemical | Molecular Weight (g/mol)165 | Density (g/mL)166 | Aqueous Solubility (mg/L)166 | K°H (mol/kg*bar)165 |
| --- | --- | --- | --- | --- |
| Tetrachloroethene | 165.833 | 1.62 | 206 | 0.057 |  |
| Trichloroethene | 131.388 | 1.4642 | 1,280 | 0.11 |  |
| cis-Dichloroethene | 96.943 | 1.2837 | 3500 | 0.27 |  |
| Vinyl chloride | 62.498 | 0.9106 | 2700 | 0.038 |  |
| Ethene | 28.0532 | – g | 131 | 0.0047 |  |
| Carbon Tetrachloride | 153.823 | 1.59 | 793 | 0.033 |  |
| Chloroform | 119.378 | 1.4788 | 7950 | 0.28 |  |
| Dichloromethane | 84.933 | 1.325 | 13,200 | 0.47 |  |
| Chloromethane | 50.488 | 0.911 | 5040 | 0.12 |  |
| Methane | 16.0425 | – g | 22 | 0.0013 |  |
| Trichlorofluoromethane | 37.368 | 1.49 | 1,100 | 0.0012 |  |
| Dichlorodifluoromethane | 120.914 | 1.35 | 280 | 0.0025 |  |

(–  g) these chemicals are gasses at environmentally relevant conditions. K°H = Henry's law constant for solubility in water at 298.15 K

Figure 42. Aquifer material extraction from stainless steel cores.

(A) The custom-build stainless steel core extruder apparatus.  (B) Aquifer material being extruded from the stainless steel core with constant flow of sterile N2 and placed inside a sterilized Mason jar for transfer into the anoxic chamber.

Figure 43. Microcosm setup inside an anoxic chamber.

(A) A Mason jar containing an extruded aquifer core. (B) Mason jars with aquifer material transferred inside the anoxic chamber. (C) Aquifer material (6 g) being distributed into 60-mL glass serum bottles using sterilized spatulas and a balance. (D) Labeled microcosms containing aquifer material ready for incubation and monitoring.

Table 15. Detailed instrument settings used for the analysis of headspace samples with gas chromatography - FID.

| Chlorinated compounds | Front Inlet | Inlet/Detector Gases | Detector (FID) or (µECD) | Program |
| --- | --- | --- | --- | --- |
| PCE, 1,1-DCE, CFC-11, CFC-12, CT | Heater = 200 ˚C Pressure = 23.193 psi Septum Purge flow = 1.5 ml/min Split = Split ratio of 50:1 w/Split flow of 150 ml/min | He = inlet N2 = detector | Heater = 200 C Air Flow = 400 ml/min H2 Fuel flow = 30 ml/min | Initial Rate ˚C/min = 0 Initial Value ˚C = 60 Initial Hold Time min = 2 Initial Run Time min = 2 Ramp 1 Rate ˚C/min = 25 Ramp 1 Value ˚C = 200 Ramp 1 Hold Time min = 1 Ramp 1 Run Time min = 8.6 |
| CF, DCM, CM | Heater = 200 ˚C Pressure = 32.45 psi Septum Purge flow = 3 ml/min Split = Split ratio of 100:1 w/Split flow of 400 ml/min | He = inlet N2 = detector | Heater = 250 C N2 Flow = 15 ml/min | Initial Rate ˚C/min = 0 Initial Value ˚C = 180 Initial Hold Time min = 6 Initial Run Time min = 6 |

Table 16. Detailed instrument settings used for the analysis of headspace samples with gas chromatography - µecd.

| Chlorinated compounds | Back Inlet | Inlet/Detector Gases | Detector (FID) or (µECD) | Program |
| --- | --- | --- | --- | --- |
| Methane, Ethene | Heater = 250 ˚C Pressure = 10 psi Septum Purge flow = 4 ml/min Split = Split ratio of 9:1 w/Split flow of 78.511 ml/min | He = inlet N2 = detector | Heater = 200 C Air Flow = 400 ml/min H2 Fuel flow = 30 ml/min | Initial Rate ˚C/min = 0 Initial Value ˚C = 60 Initial Hold Time min = 4 Initial Run Time min = 4 Ramp 1 Rate ˚C/min = 50 Ramp 1 Value ˚C = 240 Ramp 1 Hold Time min = 1 Ramp 1 Run Time min = 9.5 |

Table 17. Chlorinated compounds of concern and expected degradation products.

| Chlorinated Compound | Expected Degradation Products |
| --- | --- |
| PCE | TCE, cDCE, VC, ethene |
| 1,1-DCE | VC, ethene |
| CFC-11 | Dichlorofluoromethane, chlorofluoromethane, fluoromethane, CF, DCM, CM, methane |
| CFC-12 | Difluorochloromethane, difluoromethane, dichlorofluoromethane, chlorofluoromethane, DCM, CM, methane |
| CT | CF, DCM, CM, methane |
| CF | DCM, CM, methane |

Figure 44. Standard curves for target contaminants and degradation products (GC-FID).

Figure 45. Standard curves for target contaminants and degradation products (GC-ECD).

Table 18. Microcosms established with aquifer core materials to assess target contaminant degradation.

| Purpose | Sample ID | Aquifer Material | Liquid Phase | Compound of Concern | Methane Formation |
| --- | --- | --- | --- | --- | --- |
| heat kill control | a1 | BHA1-12 m | GW | PCE | No |
| heat kill control | a2 | BHA1-12 m | GW | PCE | No |
| natural attenuation | a3 | BHA1-12 m | GW | PCE | Yes |
| biostimulated | a4 | BHA1-12 m | GW | PCE | Yes |
| biostimulated | a5 | BHA1-12 m | GW | PCE | Yes |
| bioaugmented | a6 | BHA1-12 m | GW | PCE | Yes |
| bioaugmented | a7 | BHA1-12 m | GW | PCE | Yes |
| heat kill control | a8 | BHA1-12 m | RAMM | PCE | No |
| biostimulated | a9 | BHA1-12 m | RAMM | PCE | Yes |
| biostimulated | a10 | BHA1-12 m | RAMM | PCE | Yes |
| bioaugmented | a11 | BHA1-12 m | RAMM | PCE | Yes |
| bioaugmented | a12 | BHA1-12 m | RAMM | PCE | Yes |
| heat kill | a13 | BHA1-12 m | GW | 1,1-DCE | No |
| heat kill | a14 | BHA1-12 m | GW | 1,1-DCE | No |
| natural attenuation | a15 | BHA1-12 m | GW | 1,1-DCE | No |
| biostimulated | a16 | BHA1-12 m | GW | 1,1-DCE | No |
| biostimulated | a17 | BHA1-12 m | GW | 1,1-DCE | No |
| bioaugmented | a18 | BHA1-12 m | GW | 1,1-DCE | Yes |
| bioaugmented | a19 | BHA1-12 m | GW | 1,1-DCE | Yes |
| heat kill | a20 | BHA1-12 m | RAMM | 1,1-DCE | No |
| biostimulated | a21 | BHA1-12 m | RAMM | 1,1-DCE | No |
| biostimulated | a22 | BHA1-12 m | RAMM | 1,1-DCE | No |
| bioaugmented | a23 | BHA1-12 m | RAMM | 1,1-DCE | Yes |
| bioaugmented | a24 | BHA1-12 m | RAMM | 1,1-DCE | Yes |
| heat kill | a25 | BHA1-12 m | GW | CF | No |
| heat kill | a26 | BHA1-12 m | GW | CF | No |
| natural attenuation | a27 | BHA1-12 m | GW | CF | No |
| biostimulated | a28 | BHA1-12 m | GW | CF | No |
| biostimulated | a29 | BHA1-12 m | GW | CF | No |
| bioaugmented | a30 | BHA1-12 m | GW | CF | Yes |
| bioaugmented | a31 | BHA1-12 m | GW | CF | Yes |
| heat kill | a32 | BHA1-12 m | RAMM | CF | No |
| biostimulated | a33 | BHA1-12 m | RAMM | CF | No |
| biostimulated | a34 | BHA1-12 m | RAMM | CF | No |
| bioaugmented | a35 | BHA1-12 m | RAMM | CF | Yes |
| bioaugmented | a36 | BHA1-12 m | RAMM | CF | Yes |
| heat kill | a37 | BHA1-12 m | GW | CT | No |
| heat kill | a38 | BHA1-12 m | GW | CT | No |
| biostimulated | a39 | BHA1-12 m | GW | CT | No |
| biostimulated | a40 | BHA1-12 m | GW | CT | No |
| biostimulated | a41 | BHA1-12 m | RAMM | CT | No |
| biostimulated | a42 | BHA1-12 m | RAMM | CT | No |
| abiotic | a43 | BHA1-12 m | GW | CT | Yes |
| abiotic | a44 | BHA1-12 m | GW | CT | Yes |
| abiotic | a83 | BHA1-12 m | GW | CT | No |
| abiotic | a84 | BHA1-12 m | GW | CT | No |
| abiotic | a85 | BHA1-12 m | GW | CT | No |
| abiotic | a86 | BHA1-12 m | GW | CT | No |
| bioaugmented | a45 | BHA1-12 m | GW | CT | Yes |
| bioaugmented | a46 | BHA1-12 m | GW | CT | Yes |
| bioaugmented | a47 | BHA1-12 m | RAMM | CT | Yes |
| bioaugmented | a48 | BHA1-12 m | RAMM | CT | Yes |
| bioaugmented | a49 | BHA1-12 m | GW | CT | Yes |
| bioaugmented | a50 | BHA1-12 m | GW | CT | Yes |
| bioaugmented | a51 | BHA1-12 m | GW | CT | Yes |
| bioaugmented | a52 | BHA1-12 m | GW | CT | Yes |
| bioaugmented | a53 | BHA1-12 m | GW | CT | Yes |
| bioaugmented | a54 | BHA1-12 m | GW | CT | Yes |
| Positive control | a55 | none | RAMM | CT | Yes |
| Positive control | a56 | none | RAMM | CT | Yes |
| Positive control | a57 | none | RAMM | CT | Yes |
| Positive control | a58 | none | RAMM | CT | Yes |
| Positive control | a59 | none | RAMM | CT | Yes |
| Positive control | a60 | none | RAMM | CT | Yes |
| Positive control | a61 | none | RAMM | CT | No |
| Positive control | a62 | none | RAMM | CT | No |
| natural attenuation | a63 | BHA1-12 m | GW | CT | No |
| natural attenuation | a64 | BHA1-12 m | GW | CT | No |
| heat kill | a65 | BHA1-12 m | GW | CFC-11 | No |
| heat kill | a66 | BHA1-12 m | RAMM | CFC-11 | No |
| natural attenuation | a67 | BHA1-12 m | GW | CFC-11 | No |
| biostimulated | a68 | BHA1-12 m | GW | CFC-11 | No |
| biostimulated | a69 | BHA1-12 m | GW | CFC-11 | No |
| biostimulated | a70 | BHA1-12 m | RAMM | CFC-11 | No |
| biostimulated | a71 | BHA1-12 m | RAMM | CFC-11 | No |
| bioaugmented | a72 | BHA1-12 m | RAMM | CFC-11 | Yes |
| bioaugmented | a73 | BHA1-12 m | RAMM | CFC-11 | Yes |
| heat kill | a74 | BHA1-12 m | GW | CFC-12 | No |
| heat kill | a75 | BHA1-12 m | RAMM | CFC-12 | No |
| natural attenuation | a76 | BHA1-12 m | GW | CFC-12 | No |
| biostimulated | a77 | BHA1-12 m | GW | CFC-12 | No |
| biostimulated | a78 | BHA1-12 m | GW | CFC-12 | No |
| biostimulated | a79 | BHA1-12 m | RAMM | CFC-12 | No |
| biostimulated | a80 | BHA1-12 m | RAMM | CFC-12 | No |
| bioaugmented | a81 | BHA1-12 m | RAMM | CFC-12 | Yes |
| bioaugmented | a82 | BHA1-12 m | RAMM | CFC-12 | Yes |
| heat kill | b1 | BHA3-7 m | GW | PCE | No |
| heat kill | b2 | BHA3-7 m | GW | PCE | No |
| natural attenuation | b3 | BHA3-7 m | GW | PCE | No |
| natural attenuation | b3 | BHA3-7 m | GW | PCE | No |
| biostimulated | b4 | BHA3-7 m | GW | PCE | No |
| biostimulated | b5 | BHA3-7 m | GW | PCE | No |
| bioaugmented | b6 | BHA3-7 m | GW | PCE | Yes |
| bioaugmented | b7 | BHA3-7 m | GW | PCE | Yes |
| biostimulated | b9 | BHA3-7 m | RAMM | PCE | No |
| biostimulated | b10 | BHA3-7 m | RAMM | PCE | No |
| bioaugmented | b11 | BHA3-7 m | RAMM | PCE | Yes |
| bioaugmented | b12 | BHA3-7 m | RAMM | PCE | Yes |
| heat kill | b13 | BHA3-7 m | GW | 1,1-DCE |  |
| heat kill | b14 | BHA3-7 m | GW | 1,1-DCE | No |
| natural attenuation | b15 | BHA3-7 m | GW | 1,1-DCE | No |
| natural attenuation | b15-2nd | BHA3-7 m | GW | 1,1-DCE | No |
| biostimulated | b16 | BHA3-7 m | GW | 1,1-DCE | No |
| biostimulated | b17 | BHA3-7 m | GW | 1,1-DCE | No |
| bioaugmented | b18 | BHA3-7 m | GW | 1,1- DCE | Yes |
| bioaugmented | b19 | BHA3-7 m | GW | 1,1-DCE | Yes |
| biostimulated | b21 | BHA3-7 m | RAMM | 1,1-DCE | No |
| biostimulated | b22 | BHA3-7 m | RAMM | 1,1-DCE | No |
| bioaugmented | b23 | BHA3-7 m | RAMM | 1,1-DCE | Yes |
| bioaugmented | b24 | BHA3-7 m | RAMM | 1,1-DCE | Yes |
| heat kill | b25 | BHA3-7 m | GW | CF | No |
| heat kill | b26 | BHA3-7 m | GW | CF | No |
| natural attenuation | b27 | BHA3-7 m | GW | CF | No |
| natural attenuation | b27-2nd | BHA3-7 m | GW | CF | No |
| biostimulated | b28 | BHA3-7 m | GW | CF | No |
| biostimulated | b29 | BHA3-7 m | GW | CF | No |
| bioaugmented | b30 | BHA3-7 m | GW | CF | No |
| bioaugmented | b31 | BHA3-7 m | GW | CF | No |
| biostimulated | b33 | BHA3-7 m | RAMM | CF | No |
| biostimulated | b34 | BHA3-7 m | RAMM | CF | No |
| bioaugmented | b35 | BHA3-7 m | RAMM | CF | Yes |
| bioaugmented | b36 | BHA3-7 m | RAMM | CF | Yes |
| heat kill | b39 | BHA3-7 m | GW | CT | No |
| heat kill | b40 | BHA3-7 m | GW | CT | No |
| biostimulated | b41 | BHA3-7 m | GW | CT | No |
| biostimulated | b42 | BHA3-7 m | GW | CT | No |
| biostimulated | b43 | BHA3-7 m | RAMM | CT | No |
| biostimulated | b44 | BHA3-7 m | RAMM | CT | No |
| abiotic | b45 | BHA3-7 m | GW | CT | Yes |
| abiotic | b46 | BHA3-7 m | GW | CT | Yes |
| abiotic | b47 | BHA3-7 m | GW | CT | No |
| abiotic | b48 | BHA3-7 m | GW | CT | No |
| abiotic | b49 | BHA3-7 m | GW | CT | No |
| abiotic | b50 | BHA3-7 m | GW | CT | No |
| bioaugmented | b51 | BHA3-7 m | GW | CT | Yes |
| bioaugmented | b52 | BHA3-7 m | GW | CT | Yes |
| bioaugmented | b53 | BHA3-7 m | RAMM | CT | Yes |
| bioaugmented | b54 | BHA3-7 m | RAMM | CT | Yes |
| bioaugmented | b55 | BHA3-7 m | GW | CT | Yes |
| bioaugmented | b56 | BHA3-7 m | GW | CT | Yes |
| bioaugmented | b57 | BHA3-7 m | GW | CT | Yes |
| bioaugmented | b58 | BHA3-7 m | GW | CT | Yes |
| bioaugmented | b59 | BHA3-7 m | GW | CT | Yes |
| bioaugmented | b60 | BHA3-7 m | GW | CT | Yes |
| heat kill | b71 | BHA3-7 m | GW | CFC-11 | No |
| heat kill | b72 | BHA3-7 m | RAMM | CFC-11 | No |
| natural attenuation | b73 | BHA3-7 m | GW | CFC-11 | No |
| natural attenuation | b74 | BHA3-7 m | GW | CFC-11 | No |
| biostimulated | b75 | BHA3-7 m | GW | CFC-11 | No |
| biostimulated | b76 | BHA3-7 m | GW | CFC-11 | No |
| biostimulated | b77 | BHA3-7 m | RAMM | CFC-11 | No |
| biostimulated | b78 | BHA3-7 m | RAMM | CFC-11 | No |
| bioaugmented | b79 | BHA3-7 m | RAMM | CFC-11 | No |
| bioaugmented | b80 | BHA3-7 m | RAMM | CFC-11 | No |
| heat kill | b80-2nd | BHA3-7 m | GW | CFC-12 | No |
| heat kill | b81 | BHA3-7 m | RAMM | CFC-12 | No |
| natural attenuation | b82 | BHA3-7 m | GW | CFC-12 | No |
| natural attenuation | b83 | BHA3-7 m | GW | CFC-12 | No |
| biostimulated | b84 | BHA3-7 m | GW | CFC-12 | No |
| biostimulated | b85 | BHA3-7 m | GW | CFC-12 | No |
| biostimulated | b86 | BHA3-7 m | RAMM | CFC-12 | No |
| biostimulated | b87 | BHA3-7 m | RAMM | CFC-12 | No |
| bioaugmented | b88 | BHA3-7 m | RAMM | CFC-12 | No |
| bioaugmented | b89 | BHA3-7 m | RAMM | CFC-12 | No |
| heat kill | c1 | BHA3-15 m | GW | PCE | No |
| heat kill | c2 | BHA3-15 m | GW | PCE | No |
| natural attenuation | c3 | BHA3-15 m | GW | PCE | No |
| biostimulated | c4 | BHA3-15 m | GW | PCE | No |
| biostimulated | c5 | BHA3-15 m | GW | PCE | No |
| bioaugmented | c6 | BHA3-15 m | GW | PCE | Yes |
| bioaugmented | c7 | BHA3-15 m | GW | PCE | Yes |
| heat kill | c8 | BHA3-15 m | RAMM | PCE | No |
| biostimulated | c9 | BHA3-15 m | RAMM | PCE | No |
| biostimulated | c10 | BHA3-15 m | RAMM | PCE | No |
| bioaugmented | c11 | BHA3-15 m | RAMM | PCE | Yes |
| bioaugmented | c12 | BHA3-15 m | RAMM | PCE | Yes |
| heat kill | c13 | BHA3-15 m | GW | 1,1-DCE | No |
| heat kill | c14 | BHA3-15 m | GW | 1,1-DCE | No |
| natural attenuation | c15 | BHA3-15 m | GW | 1,1-DCE | No |
| biostimulated | c16 | BHA3-15 m | GW | 1,1-DCE | No |
| biostimulated | c17 | BHA3-15 m | GW | 1,1-DCE | No |
| bioaugmented | c18 | BHA3-15 m | GW | 1,1-DCE | Yes |
| bioaugmented | c19 | BHA3-15 m | GW | 1,1-DCE | Yes |
| heat kill | c20 | BHA3-15 m | RAMM | 1,1-DCE | No |
| biostimulated | c21 | BHA3-15 m | RAMM | 1,1-DCE | No |
| biostimulated | c22 | BHA3-15 m | RAMM | 1,1-DCE | No |
| bioaugmented | c23 | BHA3-15 m | RAMM | 1,1-DCE | Yes |
| bioaugmented | c24 | BHA3-15 m | RAMM | 1,1-DCE | Yes |
| heat kill | c25 | BHA3-15 m | GW | CF | No |
| heat kill | c26 | BHA3-15 m | GW | CF | No |
| natural attenuation | c27 | BHA3-15 m | GW | CF | No |
| biostimulated | c28 | BHA3-15 m | GW | CF | No |
| biostimulated | c29 | BHA3-15 m | GW | CF | No |
| bioaugmented | c30 | BHA3-15 m | GW | CF | Yes |
| bioaugmented | c31 | BHA3-15 m | GW | CF | Yes |
| heat kill | c32 | BHA3-15 m | RAMM | CF | No |
| biostimulated | c33 | BHA3-15 m | RAMM | CF | No |
| biostimulated | c34 | BHA3-15 m | RAMM | CF | No |
| bioaugmented | c35 | BHA3-15 m | RAMM | CF | Yes |
| bioaugmented | c36 | BHA3-15 m | RAMM | CF | Yes |
| heat kill | c37 | BHA3-15 m | GW | CT | No |
| heat kill | c38 | BHA3-15 m | GW | CT | No |
| biostimulated | c39 | BHA3-15 m | GW | CT | No |
| biostimulated | c40 | BHA3-15 m | GW | CT | No |
| biostimulated | c41 | BHA3-15 m | RAMM | CT | No |
| biostimulated | c42 | BHA3-15 m | RAMM | CT | No |
| abiotic | c43 | BHA3-15 m | GW | CT | Yes |
| abiotic | c44 | BHA3-15 m | GW | CT | Yes |
| abiotic | c45 | BHA3-15 m | GW | CT | Yes |
| abiotic | c46 | BHA3-15 m | GW | CT | Yes |
| abiotic | c47 | BHA3-15 m | GW | CT | No |
| abiotic | c48 | BHA3-15 m | GW | CT | No |
| bioaugmented | c49 | BHA3-15 m | GW | CT | Yes |
| bioaugmented | c50 | BHA3-15 m | GW | CT | Yes |
| bioaugmented | c51 | BHA3-15 m | RAMM | CT | Yes |
| bioaugmented | c52 | BHA3-15 m | RAMM | CT | Yes |
| bioaugmented | c53 | BHA3-15 m | GW | CT | Yes |
| bioaugmented | c54 | BHA3-15 m | GW | CT | Yes |
| bioaugmented | c55 | BHA3-15 m | GW | CT | Yes |
| bioaugmented | c56 | BHA3-15 m | GW | CT | Yes |
| bioaugmented | c57 | BHA3-15 m | GW | CT | Yes |
| bioaugmented | c58 | BHA3-15 m | GW | CT | Yes |
| natural attenuation | c59 | BHA3-15 m | GW | CT |  |
| natural attenuation | c60 | BHA3-15 m | GW | CT |  |
| heat kill | c61 | BHA3-15 m | GW | CFC-11 | No |
| heat kill | c62 | BHA3-15 m | RAMM | CFC-11 | No |
| natural attenuation | c63 | BHA3-15 m | GW | CFC-11 | No |
| biostimulated | c64 | BHA3-15 m | GW | CFC-11 | No |
| biostimulated | c65 | BHA3-15 m | GW | CFC-11 | No |
| biostimulated | c66 | BHA3-15 m | RAMM | CFC-11 | No |
| biostimulated | c67 | BHA3-15 m | RAMM | CFC-11 | No |
| bioaugmented | c68 | BHA3-15 m | RAMM | CFC-11 | No |
| bioaugmented | c69 | BHA3-15 m | RAMM | CFC-11 | No |
| heat kill | c70 | BHA3-15 m | GW | CFC-12 | No |
| heat kill | c71 | BHA3-15 m | RAMM | CFC-12 | No |
| natural attenuation | c72 | BHA3-15 m | GW | CFC-12 | No |
| biostimulated | c73 | BHA3-15 m | GW | CFC-12 | No |
| biostimulated | c74 | BHA3-15 m | GW | CFC-12 | No |
| biostimulated | c75 | BHA3-15 m | RAMM | CFC-12 | No |
| biostimulated | c76 | BHA3-15 m | RAMM | CFC-12 | No |
| bioaugmented | c77 | BHA3-15 m | RAMM | CFC-12 | No |
| bioaugmented | c78 | BHA3-15 m | RAMM | CFC-12 | No |
| heat kill | d1 | BHA3-24 m | GW | PCE | No |
| heat kill | d2 | BHA3-24 m | GW | PCE | No |
| natural attenuation | d3 | BHA3-24 m | GW | PCE | No |
| biostimulated | d4 | BHA3-24 m | GW | PCE | No |
| biostimulated | d5 | BHA3-24 m | GW | PCE | No |
| bioaugmented | d6 | BHA3-24 m | GW | PCE | Yes |
| bioaugmented | d7 | BHA3-24 m | GW | PCE | Yes |
| heat kill | d8 | BHA3-24 m | RAMM | PCE | No |
| biostimulated | d9 | BHA3-24 m | RAMM | PCE | No |
| biostimulated | d10 | BHA3-24 m | RAMM | PCE | No |
| bioaugmented | d11 | BHA3-24 m | RAMM | PCE | Yes |
| bioaugmented | d12 | BHA3-24 m | RAMM | PCE | Yes |
| heat kill | d13 | BHA3-24 m | GW | 1,1-DCE | No |
| heat kill | d14 | BHA3-24 m | GW | 1,1-DCE | No |
| natural attenuation | d15 | BHA3-24 m | GW | 1,1-DCE | No |
| biostimulated | d16 | BHA3-24 m | GW | 1,1-DCE | No |
| biostimulated | d17 | BHA3-24 m | GW | 1,1-DCE | No |
| bioaugmented | d18 | BHA3-24 m | GW | 1,1-DCE | Yes |
| bioaugmented | d19 | BHA3-24 m | GW | 1,1-DCE | Yes |
| heat kill | d20 | BHA3-24 m | RAMM | 1,1-DCE | No |
| biostimulated | d21 | BHA3-24 m | RAMM | 1,1-DCE | No |
| biostimulated | d22 | BHA3-24 m | RAMM | 1,1-DCE | No |
| bioaugmented | d23 | BHA3-24 m | RAMM | 1,1-DCE | Yes |
| bioaugmented | d24 | BHA3-24 m | RAMM | 1,1-DCE | Yes |
| heat kill | d25 | BHA3-24 m | GW | CF | No |
| heat kill | d26 | BHA3-24 m | GW | CF | No |
| natural attenuation | d27 | BHA3-24 m | GW | CF | No |
| biostimulated | d28 | BHA3-24 m | GW | CF | No |
| biostimulated | d29 | BHA3-24 m | GW | CF | No |
| bioaugmented | d30 | BHA3-24 m | GW | CF | Yes |
| bioaugmented | d31 | BHA3-24 m | GW | CF | Yes |
| heat kill | d32 | BHA3-24 m | RAMM | CF | No |
| biostimulated | d33 | BHA3-24 m | RAMM | CF | No |
| biostimulated | d34 | BHA3-24 m | RAMM | CF | No |
| bioaugmented | d35 | BHA3-24 m | RAMM | CF | Yes |
| bioaugmented | d36 | BHA3-24 m | RAMM | CF | Yes |
| heat kill | d37 | BHA3-24 m | GW | CT | No |
| heat kill | d38 | BHA3-24 m | GW | CT | No |
| biostimulated | d39 | BHA3-24 m | GW | CT | No |
| biostimulated | d40 | BHA3-24 m | GW | CT | No |
| biostimulated | d41 | BHA3-24 m | RAMM | CT | No |
| biostimulated | d42 | BHA3-24 m | RAMM | CT | No |
| abiotic | d43 | BHA3-24 m | GW | CT | Yes |
| abiotic | d44 | BHA3-24 m | GW | CT | Yes |
| abiotic | d45 | BHA3-24 m | GW | CT | Yes |
| abiotic | d46 | BHA3-24 m | GW | CT | Yes |
| abiotic | d47 | BHA3-24 m | GW | CT | No |
| abiotic | d48 | BHA3-24 m | GW | CT | Yes |
| bioaugmented | d49 | BHA3-24 m | GW | CT | Yes |
| bioaugmented | d50 | BHA3-24 m | GW | CT | Yes |
| bioaugmented | d51 | BHA3-24 m | RAMM | CT | Yes |
| bioaugmented | d52 | BHA3-24 m | RAMM | CT | Yes |
| bioaugmented | d53 | BHA3-24 m | GW | CT | Yes |
| bioaugmented | d54 | BHA3-24 m | GW | CT | Yes |
| bioaugmented | d55 | BHA3-24 m | GW | CT | Yes |
| bioaugmented | d56 | BHA3-24 m | GW | CT | Yes |
| bioaugmented | d57 | BHA3-24 m | GW | CT | Yes |
| bioaugmented | d58 | BHA3-24 m | GW | CT | Yes |
| natural attenuation | d59 | BHA3-24 m | GW | CT | No |
| natural attenuation | d60 | BHA3-24 m | GW | CT | No |
| heat kill | d61 | BHA3-24 m | GW | CFC-11 | No |
| heat kill | d62 | BHA3-24 m | RAMM | CFC-11 | No |
| natural attenuation | d63 | BHA3-24 m | GW | CFC-11 | No |
| biostimulated | d64 | BHA3-24 m | GW | CFC-11 | No |
| biostimulated | d65 | BHA3-24 m | GW | CFC-11 | No |
| biostimulated | d66 | BHA3-24 m | RAMM | CFC-11 | No |
| biostimulated | d67 | BHA3-24 m | RAMM | CFC-11 | No |
| bioaugmented | d68 | BHA3-24 m | RAMM | CFC-11 | No |
| bioaugmented | d69 | BHA3-24 m | RAMM | CFC-11 | No |
| heat kill | d70 | BHA3-24 m | GW | CFC-12 | No |
| heat kill | d71 | BHA3-24 m | RAMM | CFC-12 | No |
| natural attenuation | d72 | BHA3-24 m | GW | CFC-12 | No |
| biostimulated | d73 | BHA3-24 m | GW | CFC-12 | No |
| biostimulated | d74 | BHA3-24 m | GW | CFC-12 | No |
| biostimulated | d75 | BHA3-24 m | RAMM | CFC-12 | No |
| biostimulated | d76 | BHA3-24 m | RAMM | CFC-12 | No |
| bioaugmented | d77 | BHA3-24 m | RAMM | CFC-12 | No |
| bioaugmented | d78 | BHA3-24 m | RAMM | CFC-12 | No |
| heat kill | e1 | BHA4-10 m | GW | PCE | No |
| heat kill | e2 | BHA4-10 m | GW | PCE | No |
| natural attenuation | e3 | BHA4-10 m | GW | PCE | Yes |
| natural attenuation | e4 | BHA4-10 m | GW | PCE | Yes |
| biostimulated | e5 | BHA4-10 m | GW | PCE | Yes |
| biostimulated | e6 | BHA4-10 m | GW | PCE | Yes |
| bioaugmented | e7 | BHA4-10 m | GW | PCE | Yes |
| bioaugmented | e8 | BHA4-10 m | GW | PCE | Yes |
| biostimulated | e9 | BHA4-10 m | RAMM | PCE | Yes |
| biostimulated | e10 | BHA4-10 m | RAMM | PCE | Yes |
| bioaugmented | e11 | BHA4-10 m | RAMM | PCE | Yes |
| bioaugmented | e12 | BHA4-10 m | RAMM | PCE | Yes |
| heat kill | e13 | BHA4-10 m | GW | 1,1-DCE | No |
| heat kill | e14 | BHA4-10 m | GW | 1,1-DCE | No |
| natural attenuation | e15 | BHA4-10 m | GW | 1,1-DCE | No |
| natural attenuation | e16 | BHA4-10 m | GW | 1,1-DCE | No |
| biostimulated | e17 | BHA4-10 m | GW | 1,1-DCE | No |
| biostimulated | e18 | BHA4-10 m | GW | 1,1-DCE | No |
| bioaugmented | e19 | BHA4-10 m | GW | 1,1-DCE | Yes |
| bioaugmented | e20 | BHA4-10 m | GW | 1,1-DCE | Yes |
| biostimulated | e21 | BHA4-10 m | RAMM | 1,1-DCE | No |
| biostimulated | e22 | BHA4-10 m | RAMM | 1,1-DCE | No |
| bioaugmented | e23 | BHA4-10 m | RAMM | 1,1-DCE | Yes |
| bioaugmented | e24 | BHA4-10 m | RAMM | 1,1-DCE | Yes |
| heat kill | e25 | BHA4-10 m | GW | CF | No |
| heat kill | e26 | BHA4-10 m | GW | CF | No |
| natural attenuation | e27 | BHA4-10 m | GW | CF | No |
| natural attenuation | e28 | BHA4-10 m | GW | CF | No |
| biostimulated | e29 | BHA4-10 m | GW | CF | No |
| biostimulated | e30 | BHA4-10 m | GW | CF | No |
| bioaugmented | e31 | BHA4-10 m | GW | CF | No |
| bioaugmented | e32 | BHA4-10 m | GW | CF | No |
| biostimulated | e33 | BHA4-10 m | RAMM | CF | No |
| biostimulated | e34 | BHA4-10 m | RAMM | CF | No |
| bioaugmented | e35 | BHA4-10 m | RAMM | CF | Yes |
| bioaugmented | e36 | BHA4-10 m | RAMM | CF | Yes |
| heat kill | e37 | BHA4-10 m | GW | CT | No |
| heat kill | e38 | BHA4-10 m | GW | CT | No |
| biostimulated | e39 | BHA4-10 m | GW | CT | No |
| biostimulated | e40 | BHA4-10 m | GW | CT | No |
| biostimulated | e41 | BHA4-10 m | RAMM | CT | Yes |
| biostimulated | e42 | BHA4-10 m | RAMM | CT | Yes |
| abiotic | e43 | BHA4-10 m | GW | CT | Yes |
| abiotic | e44 | BHA4-10 m | GW | CT | Yes |
| abiotic | e45 | BHA4-10 m | GW | CT | Yes |
| abiotic | e46 | BHA4-10 m | GW | CT | Yes |
| abiotic | e47 | BHA4-10 m | GW | CT | Yes |
| abiotic | e48 | BHA4-10 m | GW | CT | Yes |
| bioaugmented | e49 | BHA4-10 m | GW | CT | Yes |
| bioaugmented | e50 | BHA4-10 m | GW | CT | Yes |
| bioaugmented | e51 | BHA4-10 m | RAMM | CT | Yes |
| bioaugmented | e52 | BHA4-10 m | RAMM | CT | Yes |
| bioaugmented | e53 | BHA4-10 m | GW | CT | Yes |
| bioaugmented | e54 | BHA4-10 m | GW | CT | Yes |
| bioaugmented | e55 | BHA4-10 m | GW | CT | Yes |
| bioaugmented | e56 | BHA4-10 m | GW | CT | Yes |
| bioaugmented | e57 | BHA4-10 m | GW | CT | Yes |
| bioaugmented | e58 | BHA4-10 m | GW | CT | Yes |
| natural attenuation | e59 | BHA4-10 m | GW | CT | No |
| natural attenuation | e60 | BHA4-10 m | GW | CT | No |
| heat kill | e61 | BHA4-10 m | GW | CFC-11 | No |
| heat kill | e62 | BHA4-10 m | RAMM | CFC-11 | No |
| natural attenuation | e63 | BHA4-10 m | GW | CFC-11 | No |
| natural attenuation | e64 | BHA4-10 m | GW | CFC-11 | No |
| biostimulated | e65 | BHA4-10 m | GW | CFC-11 | No |
| biostimulated | e66 | BHA4-10 m | GW | CFC-11 | No |
| biostimulated | e67 | BHA4-10 m | RAMM | CFC-11 | No |
| biostimulated | e68 | BHA4-10 m | RAMM | CFC-11 | No |
| bioaugmented | e69 | BHA4-10 m | RAMM | CFC-11 | No |
| bioaugmented | e70 | BHA4-10 m | RAMM | CFC-11 | No |
| heat kill | e71 | BHA4-10 m | GW | CFC-12 | No |
| heat kill | e72 | BHA4-10 m | RAMM | CFC-12 | No |
| natural attenuation | e73 | BHA4-10 m | GW | CFC-12 | No |
| natural attenuation | e74 | BHA4-10 m | GW | CFC-12 | No |
| biostimulated | e75 | BHA4-10 m | GW | CFC-12 | No |
| biostimulated | e76 | BHA4-10 m | GW | CFC-12 | No |
| biostimulated | e77 | BHA4-10 m | RAMM | CFC-12 | No |
| biostimulated | e78 | BHA4-10 m | RAMM | CFC-12 | No |
| bioaugmented | e79 | BHA4-10 m | RAMM | CFC-12 | No |
| bioaugmented | e80 | BHA4-10 m | RAMM | CFC-12 | No |

### Libraries and R code used to make Figure 7 and Figure 8

Libraries used: dplyr, tidyverse, ggplot2, gridExtra.

#read in file

CT <- read_csv("/Users/file-path")

CT_long<- gather(CT,contaminants,Percent,'CT', factor_key = TRUE)

#plot heatmaps

plot <- test_long %>% ggplot(., aes(x= Site, y= Treatment, fill = Percent)) + geom_tile(color='white', size=0.3) + facet_grid(~contaminants) + theme_classic() + theme(axis.text.x = element_text(angle = 90, vjust = .5)) + scale_fill_gradientn(colors = c("gray89","midnightblue"))

ggsave("/Users/ file-path.pdf",plot=last_plot(), width = 9.36, height = 5.79, units = "in", limitsize = FALSE)

grid.arrange(nrow=1, ncol=2, test_long %>% ggplot(., aes(x= Site, y= Treatment, fill = Percent)) + geom_tile(color='white', size=0.3) + facet_grid(~contaminants) + theme_classic() + theme(axis.text.x = element_text(angle = 90, vjust = .5)) + scale_fill_gradientn(colors = c("gray89","midnightblue")),

CT_long %>% ggplot(., aes(x= Site, y= Treatment, fill = Percent)) + geom_tile(color='white', size=0.35) + facet_grid(~contaminants) + theme_classic() + theme(axis.text.x = element_text(angle = 90, vjust = .5)) + scale_fill_gradientn(colors = c("gray89","midnightblue"))) ggsave("/file-path",plot=last_plot(), width = 4.68, height = 5.79, units = "in", limitsize = FALSE)

# CHAPTER III

# A Laboratory-Scale Treatability Study of Biostimulation, Bioaugmentation, and Amendments with Reactive Iron Species for tetrachloroethene, Tetrachloromethane, and trichloromethane

# Degradation at a contaminated site.

## Abstract

A bench-scale treatability study was performed with aquifer material from a contaminated site in the US, which is predominantly impacted by carbon tetrachloride (CT),  chloroform (CF), and tetrachloroethene (PCE). Microcosms were constructed in 160-mL glass vessels containing about 18 g (wet weight) of aquifer material and 80 mL of site groundwater or entirely synthetic, reduced anoxic mineral salt medium (RAMM). Chlorinated solvents were monitored by GC-FID, and the organic acid concentration was monitored by HPLC. Four remedial treatments were evaluated: monitored natural attenuation (MNA), biostimulation, biostimulation combined with bioaugmentation, and the amending of microcosms with zero-valent iron (ZVI). Groundwater was filtered through Sterivex filters to identify organohalide-respiring bacteria (OHRB) by qPCR. The lack of degradation of PCE, CT, or CF in microcosms set up to simulate MNA and biostimulation conditions, along with the limited detection of OHRB in qPCR assays, suggests that a community of OHRB are not active at this contaminated site. OHRB were detected at one of the four sampling locations in low abundance. In samples from location B-2-S, the qPCR assay for Dehalococcoides detected 5.5 gene copies per ml, and the Dehalogenimonas assay detected 6.5 gene copies per ml. Furthermore, bioaugmentation combined with biostimulation effectively degraded PCE and CF in microcosms that contained RAMM, but no activity was observed in microcosms that contained groundwater. This observation suggests that a constituent of the groundwater inhibits the growth of native and amended OHRB. Transformation of CT to CF was observed in microcosms amended with ZVI.

## Introduction

The samples used in this study were from a chlorofluorocarbon manufacturing facility. The aquifer at this contaminated site is impacted by various chlorinated volatile organic compounds (cVOCs), including PCE, CT, and CF Table 24. The presence of trichloroethene (TCE; 0.02 mg/L), cis-1,2-dichloroethene (cDCE; 0.02 mg/L), and vinyl chloride (VC; 0.01 mg/L), which are typical PCE transformation products, was observed in groundwater monitoring wells. In addition, dichloromethane (methylene chloride; DCM; 0.05 mg/L) and chloromethane (methyl chloride; CM; 0.01 mg/L), both potential transformation products of CT and CF, were detected in groundwater samples. These observations suggest that indigenous processes transform PCE, CT, and CF to products with a lower degree of chlorination; however, the extent of in situ dechlorination is limited, and the transformation products are also regulated compounds (i.e., detoxification is not achieved). In addition, fluorinated volatile organic compounds were observed in groundwater monitoring wells, 1,1,2-trichloro-1,2,2-trifluoroethane (CFC-113) was observed at a concentration of 1.10 mg/L, and dichlorodifluoromethane (CFC-12; 0.01 mg/L) and trichlorofluoromethane (CFC-11; 0.01 mg/L) were observed.

CT, CF, and PCE are recalcitrant cVOCs listed on the EPA’s priority pollutants list.167 These VOCs have densities of 1.49 g/cm³ (CF), 1.59 g/cm³ (CT), and 1.62 g/cm³ (PCE) and tend to form dense non-aqueous phase liquids in the subsurface upon release.168 Such dense non-aqueous phase liquid source zones can feed extensive groundwater contaminant plumes for many decades.145

A variety of processes, including biotic and abiotic degradation, convert cVOCs into non-hazardous products.68–70,169 Degradation of cVOCs by zero-valent iron (ZVI) or reactive mineral phases, such as iron sulfide (FeS) and magnetite (Fe3O4), has been documented in the literature, and installation of permeable reactive barriers can be a strategy to intercept plumes and degrade cVOCs.70 Reactions between mineral phases and cVOCs are influenced by various biogeochemical parameters, including pH, sulfate/sulfide concentrations, natural organic matter content, iron availability, and the resident microbial community (e.g., sulfate-reducing and ferric iron-reducing bacteria).

In addition to abiotic processes, microbial degradation is a significant attenuation process for cVOCs. A diversity of bacteria has been discovered that use cVOCs as respiratory electron acceptors in a process known as organohalide respiration.144,170 In the context of cVOCs, the focus has been on chlorinated ethenes such as PCE, TCE, cDCE, and VC. Microbes capable of reductively dechlorinating cVOCs have been reported in the literature.144,170 If these microbes are present and the environmental conditions support their activity, in situ contaminant degradation can lead to contaminant mass reduction and detoxification. If in such a scenario, the indigenous degradation rates meet remedial goals, the implementation of monitored natural attenuation (MNA) can be considered.

At many sites, bacteria capable of degrading cVOCs are present, but site-specific conditions are not favorable to support efficient contaminant degradation. In such cases, altering the site conditions can stimulate biodegradation activity. For instance, adding a fermentable substrate (i.e., biostimulation) to consume oxygen can establish favorable redox conditions and simultaneously increase the flux of hydrogen, which is the preferred electron donor for many organohalide-respiring bacteria. To further enhance contaminant degradation rates, or to initiate the microbial reductive dechlorination process at sites where the key dechlorinators are absent or present in low abundances, the injection of a microbial consortium (i.e., bioaugmentation) can be considered.171–174 Two commercial microbial consortia were tested in this study, including APTIM’s SDC-9 culture, which was used to stimulate the degradation of PCE in microcosms, and SiREM’s KB-1 Plus culture, which was used to stimulate the degradation of CT and CF in microcosms.

The overarching goal of this bench-scale treatability study was to assess the potential for in situ degradation of the target contaminants by MNA, enhanced in situ bioremediation (i.e., biostimulation alone and combined with bioaugmentation), and abiotic degradation of CT by ZVI in aquifer material from the site. Degradation activity was measured by following the concentrations of target contaminants (i.e., PCE, CT, and CF) and the formation of potential transformation products (i.e., cDCE, VC, DCM, CM, ethene, and methane). Additional measures included the consumption of the electron donor (i.e., lactate), the formation of lactate fermentation products (i.e., acetate and propionate), and the presence of known dechlorinating bacteria in biomass associated with the groundwater.

## Methodology

Sample Collection Aquifer material and groundwater samples were collected near a former miscellaneous landfill area at the contaminated site. Four temporary wells were established to a depth of 20 ft and placed 5–8 ft north, south, east, and west of the monitoring well Figure 52. Each aquifer core was 23 in long and encased in a cellulose acetate butyrate (CAB) liner. Groundwater was collected from each of the four temporary wells in 1-L plastic containers closed with gas-tight screw caps. Aquifer material and groundwater samples were classified with the label “B” (for borehole = temporary well) 1 through 4, with the numerals indicating the order in which the samples were obtained and with “N,” “S,” “E,” and “W” for the location of the boreholes relative to the monitoring well Figure 52. These identifiers (e.g., B-1-E, B-2-S, B-3-W, and B-4-N) were used to identify aquifer and groundwater origins.

The field crew measured basic groundwater parameters at the time of sampling, and the data are summarized in Table 25. In addition, groundwater from each of the four temporary wells was filtered on site through Sterivex cartridges to collect biomass associated with the groundwater. The volume of the groundwater that passed through each cartridge was recorded and noted on the respective cartridge housing Table 26. From each temporary well location, two Sterivex cartridges were obtained (eight cartridges total). Aquifer material cores, groundwater, and Sterivex cartridges were shipped in a cooler with blue ice packs via an overnight carrier to the University of Tennessee, and the shipment arrived on June 19, 2019.

Sample processing and microcosm setup Two liters of composite groundwater were prepared and used as the liquid phase for microcosm setup. To prepare composite groundwater, equal volumes (250 mL) of groundwater collected from temporary well locations B-1-E, B-2-S, B-3-W, and B-4-N were mixed in 2-L vessels under a constant flow of nitrogen (N2). To remove oxygen and VOCs, sterile, oxygen-free N2 gas was bubbled through the composite groundwater for 30 min. Prior to the vessels being caped, 20 mL was transferred from each vessel to a clean beaker, and pH was measured. The pH of the composite groundwater in both vessels was 7.7 ± 0.05 pH units. The 2-L vessels with N2 headspace were closed with butyl rubber stoppers secured with aluminum crimps and were stored at 4C until used for microcosm setup.

A total of four ~23-inch-long aquifer material cores were obtained with the designations B-1-E, B-2-S, B-3-W, and B-4-N. The aquifer cores were cut into two segments and immediately closed with a plastic cap, and the seal was secured with electrical tape. The bottom 33 cm portion of the core was considered the lower core, and the top ~10-inch portion of the core was considered the upper core Figure 53.

Following the separation of cores, the term “lower” refers to the bottom portions, and the term “upper” refers to the top portions. The cut cores were transferred inside an anoxic glove box filled with N2/H2 (97:3, vol/vol) to transfer aquifer material from the CAB core liners into sterile glass Mason jars. The aquifer material had a firm consistency that was very difficult to remove from the CAB liners. The upper portion of the core material contained more of a fine material, with some visible sand and white, visibly porous pebbles. The lower portion of the core contained a higher proportion of clay. Homogenization of the aquifer materials by physical mixing was impossible due to the material’s firm texture, and it was impossible to distribute the material into glass serum bottles for microcosm setup. Pipettable slurries were made in order to distribute the homogenized material into glass serum bottles for microcosm setup. To make the pipettable slurries groundwater from the corresponding sampling locations was added to the Mason jars containing the solids in an attempt to prepare pipettable slurries Table 19. Then, manual homogenization of the slurries was performed outside of the glove box under a constant stream of anoxic, sterile N2. This process was time-consuming due to the firm texture of the aquifer material, and it took several hours before this process yielded pipettable slurries. The Mason jars containing the slurries with N2 headspace were closed with gas-tight lids, and the contents were mixed with a magnetic stirrer at 130 rpm at room temperature overnight.

The following day, composite slurries were prepared to represent the upper and lower portions of the cores Table 19. The composite slurry representing the upper segments was prepared by mixing equal volumes (~200 mL) of the four slurries made from the top portions of the B-1-E, B-2-S, B-3-W, and B-4-N aquifer material cores Table 19. The composite slurry representing the lower portions was prepared by mixing ~200 mL of each of the slurries established with the lower portions of the cores. To prepare pipettable slurries, an additional volume of 100 mL of composite groundwater was added to the upper and lower composite slurries. The Mason jars containing the pipettable composite slurries were then capped and manually shaken for homogenization. Table 19 provides information about the amount of aquifer material and groundwater used to prepare the individual slurries. Table 19 summarizes the amounts of aquifer material and volumes of groundwater that were used to establish the different slurries. The individual upper and lower core slurries were then mixed to form the upper and lower composite slurries identified in the boxes on the right-hand side of Table 19.

Table 19. Preparation of pipettable composite slurries for microcosm setup.

Microcosms were prepared at the Hungate station under a constant flow of an anoxic, sterile mixture of N2 and carbon dioxide (CO2; 80/20, vol/vol; Figure 54 and Figure 55). All vessels were flushed with the 80/20 (vol/vol) N2/CO2 gas mixture to replace air. First, 18 mL of the designated composite slurry was transferred into sterile 160-mL glass serum bottles with a sterile 25-mL plastic pipette Figure 54. Then, 80 mL of either RAMM-containing resazurin as a redox indicator or anoxic composite groundwater was added Figure 56. The pH of the groundwater decreased from 7.7 to 7.2 ± 0.2 pH units. The microcosms were sealed with blue butyl rubber stoppers held in place with aluminum crimps. All microcosms received 0.2 mM sulfide from a 100-mM sterilized stock solution of sodium sulfide. Lactate (~5 mM) was added to the 2-L bottles of RAMM when the medium was prepared.175 For microcosms containing groundwater, lactate was added by injecting 1 mL from a 1-M stock solution with a 1-mL plastic syringe equipped with a sterile 0.2-m membrane filter Table 28. One replicate microcosm per treatment was autoclaved (40 min at 121ºC at 15 psi) immediately after microcosm setup, and a second time the following morning, to serve as heat-killed control. Heat-killed controls received PCE, CT, and CF after autoclaving. All cVOCs were added with designated Hamilton glass syringes equipped with reproducibility (Chaney) adaptors. The concentrations of PCE, CT, and CF added to the microcosms are shown in Table 27.

Figure 55 shows the system used to dispense groundwater or RAMM into 160-mL glass serum bottles containing the composite aquifer material slurries. The 2-L bottle containing anoxic groundwater or sterilized RAMM was inverted and placed on a three-legged stand (depicted on the left in Figure 55). The 2-L bottle was pressurized with anoxic, sterile N2 gas. A sterile PVC line with a needle penetrating the bottle’s rubber stopper delivered RAMM or groundwater to the serum bottles with the slurries. This RAMM transfer line was equipped with a valve to regulate flow. Each 160-mL serum bottle received 80 mL of RAMM or groundwater. A 160-mL serum bottle containing 100 mL of RAMM was bubbled continuously with the same N2/CO2 gas mixture to serve as a proxy for monitoring RAMM pH. A RAMM pH of 7.2 ± 0.2 pH units was maintained by adjusting the percentage of CO2 in the gas mixture. All serum bottles were flushed with this N2/CO2 (~80/20, vol/vol) gas mixture and capped with blue rubber butyl stoppers, which were secured with aluminum crimps Figure 56.

All microcosms were equilibrated overnight at room temperature with the stoppers down. The next morning, the redox indicator in microcosms established with RAMM was pink, indicating the redox potential was above -110 mV, potentially unfavorable for reductive dechlorination. To lower the redox potential in the microcosms, an additional 0.2 mM sodium sulfide was added to all microcosms, including those that had been established with groundwater (note that the groundwater microcosms did not contain the redox indicator resazurin and received the same treatment as did the microcosms established with RAMM.) After an additional day of incubation, the redox indicator remained pink. At this time, cVOCs were measured in headspace samples using a gas chromatograph (GC) equipped with a flame ionization detector (FID). Headspace samples were also injected into a GC equipped with a micro electron capture detector to detect low concentrations of oxygen. These measurements did not provide evidence for elevated concentrations of oxygen in any of the microcosms. Aliquots (1 mL) of the liquid phase were withdrawn from all microcosms to measure organic acids (i.e., lactate, propionate, and acetate) by using high-performance liquid chromatography (HPLC).

Bioaugmentation Microcosms containing PCE were amended with culture SDC-9, a commercial bioaugmentation consortium capable of degrading a variety of halogenated contaminants.176 The SDC-9 culture was received from APTIM on June 21, 2019 and was stored at 4°C until it was injected into the designated microcosms on July 1, 2019 Table 28.

Microcosms containing CT and CF were augmented with consortium KB-1 Plus provided by SiREM. The KB-1 Plus culture was received on June 20, 2019 and stored at 4°C until it was injected into the designated microcosms on July 1, 2019 Table 28. Microcosms received 3 mL of the respective bioaugmentation cultures using 3-mL, N2-flushed plastic syringes with 25G needles. Although the microcosms containing the redox indicator resazurin still had a pink sheen, the microcosms were incubated without further manipulations with the stoppers down in the dark and without agitation at room temperature (~21°C). For all treatments, negative control microcosms were established to document potential contaminant loss (e.g., sorption) and clearly demonstrate the benefits of the respective treatments.

Abiotic degradation Experiments were set up at the gassing (Hungate) station under a constant flow of N2 and CO2 (80/20, vol/vol). Composite aquifer material slurry (18 mL) was transferred to 160-mL glass serum bottles containing 80 mL of anoxic MilliQ water. The bottles were closed with butyl rubber stoppers and crimped prior to CT being added with a designated Hamilton syringe equipped with Chaney adaptors Table 27. The bottles were equilibrated overnight with the stoppers down at room temperature. The following morning, time zero measurements of contaminant concentrations in the headspace were acquired by manually injecting 100-µL headspace samples into a GC-FID. Immediately after sampling, bottles 37a, 38a, and 39a were amended with 3 mL of a ZVI slurry containing 500 mg/mL ZVI by using a 3-mL plastic syringe equipped with an 18G needle. The ZVI slurry was prepared inside the anoxic glove box using anoxic deionized water and -200 mesh iron powder from Alfa Aesar (99+% metals-basis grade). Contaminant concentrations were monitored over the next 48 hr via headspace sampling and GC-FID analysis.

Microcosm monitoring cVOCs and organic acids were measured every 14 days over a 98-day period. One additional cVOC measurement occurred after 226 days. For cVOC measurements, plastic 1-mL syringes were used to withdraw 100 µL of headspace sample, which was manually injected into a GC-FID. Organic acids were measured in the aqueous phase, and 1 mL of liquid was withdrawn with a 3-mL plastic syringe and transferred to a 1.5-mL plastic tube. Following centrifugation, 200 µL of the supernatant was transferred to a 200-µL plastic HPLC vial containing 2 µL of 1 M sulfuric acid. The HPLC vials were stored at 4°C overnight and analyzed the following day by HPLC. Concentrations of analytes were determined by comparing the peak areas to externally generated standard curves.

Data analysis Peak areas from the GC-FID chromatograms were determined by integration performed with the Agilent software package OpenLab ChemStation (Rev. C.01.10). Peak areas from the HPLC chromatograms were determined by integration performed with Agilent software ChemStation for LC 3D systems (Rev. B.04.02). The peak areas for GC-FID and HPLC were recorded in an Excel spreadsheet and plotted. If the GC-FID measurements indicated that replicate microcosms performed similarly, then the cVOC data were averaged and plotted as a single graph. Separate graphs were prepared if replicate microcosms showed distinct dechlorination performance. For GC-FID analysis, the limit of detection (LOD) and limit of quantification (LOQ) for each chemical were calculated using the standard deviation of the response of the slope method.177,178

Additional amendments On day 63 after microcosm setup, an additional dose of 0.5 mM sodium sulfide was added to all microcosms because the microcosms with RAMM, which contained the redox indicator resazurin, remained slightly pink. The bioaugmentation microcosms received an additional dose of 5 mM lactate and a second 3-mL inoculum of the respective bioaugmentation culture SDC-9 or KB-1 Plus on day 63 Table 28.

DNA isolation Biomass samples collected with Sterivex cartridges from temporary well locations B-1-E, B-2-S, B-3-W, and B-4-N were kept at -80°C upon delivery. To isolate DNA from Sterivex membrane filters, the filters were removed from the Sterivex cartridge housing, cut into ~1-cm-long pieces using a sterile scalpel, and placed into plastic tubes with beads provided with the Qiagen DNeasy PowerLyzer PowerSoil DNA Kit. DNA was isolated according to the manufacturer’s recommendations except for the application of a bead-beating step at 5 m/s for 3 min to improve cell lysis (OMNI Bead Rupter Homogenizer, OMNI International, GA, USA). DNA was eluted into nuclease-free water, and the DNA concentration and purity were determined with a NanoDrop spectrophotometer (NanoDrop Technologies, Wilmington, DE, USA). DNA was stored at -80°C until analysis.

Quantitative polymerase chain reaction (qPCR) qPCR assays using TaqMan detection chemistry were used to enumerate biomarker gene copy numbers associated with the biomass samples collected on the Sterivex membrane filters. The qPCR runs were performed with the specific assays targeting the 16S rRNA genes of Dehalococcoides mccartyi,179 Dehalobacter,180 and total bacteria.147 Prior to qPCR analysis, the DNA samples were diluted with nuclease-free water to achieve 1:10, 1:100, and 1:1,000 dilutions of the initial DNA concentrations. This routine procedure assists in the identification of potential PCR inhibitors in the samples.181 TaqMan qPCR assays (10 µL total assay volume) were prepared as follows: 5 µL of TaqMan Universal PCR Master Mix No AmpErase UNG (Applied Biosystems, Foster City, CA), 0.3 µM of each primer, 0.3 µM of the probe, and 2.0 µL of template DNA. The PCR cycle parameters were 10 min at 95°C, followed by 40 cycles of 15 s at 95°C and 1 min at 60°C. TaqMan qPCR assays were run using QuantStudio 12K Flex Real-Time PCR System (Life Technologies, Grand Island, NY) in 384-well plates. A synthetic 1,500 bp linear DNA fragment covering primer/probe binding sites of target genes was used as a DNA standard for the preparation of standard curves for the target assays. A range of 1.0E+01 to 10E+08 gene copies/µL was prepared for each target gene by a 10-fold serial dilution series of standard linear DNA fragments. Standards and samples were run in triplicate, and the results were analyzed with the QuantStudio 12K Flex System Software (Life Technologies).

## Results

### Detection and quantification of known dechlorinating bacteria with qPCR

Bacterial 16S rRNA genes were detected in groundwater collected from all four sampling locations. Total bacterial abundances ranged from 3.62E+02 gene copies per mL at sampling location B-1-E to 6.68E+04 gene copies per mL at sampling location B-2-S. Intermediate bacterial 16S rRNA gene copy numbers were observed in groundwater collected from sampling locations B-3-W and B-4-N Table 20. Bacterial 16S rRNA gene copy numbers varied significantly between the four groundwater sampling locations B-1-E, B-2-S, B-3-W, and B-4-N, suggesting that microorganisms in the aquifer surrounding the monitoring well are heterogeneously distributed. Dhc 16S rRNA genes were detected in a quantifiable amount in samples from B-2-S. Additionally, Dhc 16S rRNA genes were detected in samples from B-4-N, but were below the LOQ (10 gene copies/reaction) in groundwater sampling location B-4-N. At sampling locations B-1-E and B-3-W, Dhc 16S rRNA genes were below the LOD (5 gene copies/reaction; Table 20). Dehalobacter 16S rRNA genes were detected at all four groundwater sampling locations, but the abundances were generally too low to be quantified (i.e., DNQ; Table 20). The qPCR analysis indicated that organohalide-respiring bacteria belonging to the genera Dehalococcoides and Dehalobacter are present in groundwater near well OW-09DB but in very low abundances. Active reductive dechlorination has been observed at Dhc cell abundance exceeding 1.0E+04 per mL, and this cell titer is regarded as a threshold for sustaining generally useful rates of reductive dechlorination.182 Due to the low abundance of Dhc and Dhb 16S rRNA genes in the groundwater (<10 cells/mL), reductive dehalogenase genes were not quantified.

Table 20. Detection of total bacterial, Dhc, and Dhb 16S rRNA genes with specific qPCR assays in groundwater samples collected from the contaminated site.

| Sample location | Total bacterial 16S rRNA gene copies/mLa | Dhc 16S rRNA gene copies/mLb | Dhb 16S rRNA gene copies/mLb |
| --- | --- | --- | --- |
| B-1-E | 3.62E+02 ± 1.11E+02 | Not detected | DNQ c |
| B-2-S | 6.68E+04 ± 1.36E+04 | 5.52 ± .96 | 6.45 ± .54 |
| B-3-W | 1.23E+03 ± 3.02E+02 | Not detected | DNQ c |
| B-4-N | 1.04E+04 ± 2.79E+03 | DNQ b | DNQ c |

a Practical detection limit: 1 gene copy/mL, b Practical detection limit: 0.1 gene copies/mL, c detectable but not quantifiable

### Microcosm performance

No cVOC degradation was apparent in any of the microcosms after a 63-day incubation period. At that time, all of microcosms established with RAMM had a pink sheen, presumably due to the presence of some oxidized resazurin. The microcosms established with groundwater did not receive resazurin, and the pink sheen was not apparent, suggesting that the pink sheen was indeed caused by oxidized resazurin. Apparently, reducing conditions had not been established in the microcosms, and all microcosms received an additional dose of 0.5 mM sulfide. It is possible that oxygen was introduced during the extensive manipulation that was necessary to create a slurry of the aquifer material and set up microcosms, though anoxic techniques were used during the handling of aquifer material. Another explanation for reducing conditions having not been met is that the aquifer material may have been oxidizing. Data collected by Parsons on the day of sampling at the monituring well reported a redox value of 102.2 mV and the dissolved oxygen concentrated to be 0.53 mg/L. Furthermore, historical measurements which were taken biannually since 2005 show a mean redox value of 44 and a mean dissolved oxygen concentration of 1.12 mg/L.

Lactate consumption had only occurred in the bioaugmented microcosms by day 63, which received an additional dose of 5 mM lactate. In addition, the bioaugmented microcosm received a second 3-mL inoculum with the respective bioaugmentation culture SDC-9 or KB-1 Plus. Lactate was not consumed in any of the biostimulation microcosms that contained groundwater as the liquid phase, which was an unexpected observation. Lactate is used by aerobic microorganisms and is readily fermented by ubiquitous microorganisms under anoxic conditions. Numerous studies have used lactate as an electron donor to stimulate reductive dechlorination processes, and lactate fermentation is a robust process and generally not inhibited by the presence of cVOCs that support organohalide-respiring bacteria. Possible explanations for this unexpected observation include the presence of low amounts of CT and/or CFC-113 in the microcosms. The initial CT and CFC-113 concentrations in the groundwater were 2.1 mg/L and 1.1 mg/L, respectively, which were substantially reduced by flushing the groundwater with N2 gas for 30 min; however, it is possible that low levels of CT and CFC-113 remained in the groundwater or were introduced to the microcosms with the aquifer material. Neither compounds are utilized as electron acceptors by organohalide-respiring bacteria, and toxic effects have been documented.30,31,34,183,184 It is conceivable that the CT concentrations remained sufficiently high to prevent microbial lactate utilization in the microcosms. Little is known about the toxicity of CFC-113 for microorganism; however, a recent study demonstrated inhibitory effects on Dhc.185 Im et al. also showed that homoacetogen Sporomusa ovata is capable of reducing CFC-113 to cis-1,2-dichloroethene (cis-DCE). It is thought that the reduction of CFC-113 to cis-1,2-dichloroethene (cis-DCE) by Sporomusa ovata is due to its corrinoid-producing capabilities, as Im et al. also report the super nucleophilic corrinoids can reduce CFC-113 to chlorotrifluoroethene and trifluoroethene. Additionally, lactate consumption was observed in the bioaugmented microcosms, suggesting that increased biomass can circumvent potential toxicity caused by CT and/or CFC-113.

Microcosms were set up with i) upper aquifer material and groundwater, ii) upper aquifer material and RAMM, and iii) lower aquifer material and RAMM to explore the potential of MNA, biostimulation, and bioaugmentation as remediation strategies Table 28. Additionally, heat-killed control microcosms were set up and monitored. Microcosms were not set up with lower aquifer material and groundwater due to limitations in laboratory resources. Analysis of the GC-FID data for microcosms containing i) upper aquifer material and groundwater, ii) upper aquifer material and RAMM, and iii) lower aquifer material and RAMM revealed that the microcosms behaved consistently within MNA, biostimulation, and bioaugmentation treatment groups, with the exception of PCE and CF bioaugmentation microcosms that contained RAMM.

Data from all microcosms that underwent the same treatment (i.e., MNA, biostimulation, and bioaugmentation) and contained the same contaminant were averaged and plotted together due to the consistent behavior between microcosms that contained different combinations of aquifer material and groundwater or RAMM Figure 46, Figure 48, and Figure 49. Bioaugmentation microcosms that contained PCE and CF were plotted individually to highlight the differences between microcosms set up with i) upper aquifer material and groundwater, ii) upper aquifer material and RAMM, and iii) lower aquifer material and RAMM Figure 47 and Figure 50.

PCE transformation PCE can be reductively dechlorinated to TCE, cDCE, VC, and ethene, and these transformation products can be captured in headspace samples analyzed by GC-FID. PCE concentrations decreased in all microcosms; however, equivalent amounts of transformation products were not observed, suggesting that this decrease in PCE concentration is due to sorption to the stopper and/or aquifer material solids rather than reductive dechlorination Figure 46. The formation of some cDCE (1.0–4.5 µmol/bottle) was observed in all microcosms that had received PCE following the addition of 0.5 mM of sodium sulfide on day 63. A possible explanation for the formation of cDCE following the addition of 0.5 mM sodium sulfide on day 63 is that the added sulfide scavenged any ferrous iron (Fe[II]) in the system, resulting in the formation of ferrous sulfide (FeS).129 FeS has been demonstrated to dechlorinate chlorinated ethenes including PCE and TCE.152 Data for ferrous iron in the groundwater or associated with the aquifer material were not available, but ferrous iron (~0.1 g/bottle) was present in microcosms containing RAMM.

In PCE-containing heat-killed controls, an average decrease of 11.7 ± 0.5 mol of PCE per bottle was observed on day 226 Figure 46. Plausible explanations for the loss of PCE include sorption to the stopper and/or the aquifer material solids. Some cDCE formation (1.5 mol/bottle) was observed after the addition of sulfide. Prior to the addition of 0.5 mM sulfide on day 63, no cDCE was detected in heat-killed controls. In PCE-containing MNA microcosms, the measured starting amount of PCE in the MNA microcosms was 37.0 ± 0.5 mol per bottle, and decreases of 21.9 ± 0.5 mol of PCE per bottle were observed following a 226-day incubation period. No cDCE was formed prior to the addition of 0.5 mM sulfide on day 63. Following a 98-day incubation period, an average of 4.4 ± 0.5 mol of cDCE was detected in PCE-containing MNA microcosms.

Figure 46. Amounts of PCE and transformation products over time.

Panel (a) shows cVOC data from heat-killed controls (ID# 1a, 5a, 25a), panel (b) contains cVOC data from MNA microcosms (ID# 2a, 6a, 26b), panel (c) contains cVOC data from biostimulation microcosms (ID# 3a–b, 7a–b, 27a–b), and panel (d) contains cVOC data from bioaugmentation microcosms (ID# 4a–b, 8a–b, 28a–b). The star indicates the addition of a second dose of 0.5 mM of sodium sulfide on day 63.

Biostimulation microcosms that contained PCE had a measured starting amount of PCE of 38.7 mol per bottle. A decrease of 24.9 ± 0.5 mol of PCE per bottle was observed after a 226-day incubation period. Additionally, 2.5 ± 0.5 mol of cDCE was identified. Prior to the addition of 0.5 mM sulfide on day 63, no cDCE was detected in PCE-containing biostimulation microcosms. An average decrease of 28.8 ± 0.5 mol of PCE per bottle was observed in PCE-containing bioaugmentation microcosms. Variability in the performance of the bioaugmentation microcosms was observed; thus, they were plotted individually Figure 47. Microcosms #4a–b contained groundwater as the liquid phase, and an average decrease of 25.3 ± 0.5 mol of PCE per bottle was observed on day 226. Microcosms #4a–b produced 1.3 ± 0.5 mol of cDCE after the addition of 0.5 mM of sodium sulfide on day 63, similar to what was observed in the heat-killed control, MNA, and biostimulation microcosms. It is possible that reductive dechlorination was inhibited in the bioaugmentation microcosms established with groundwater due to the presence of CT or CFC-113.30,31,34,183–185 Evidence for PCE reductive dechlorination was observed in bioaugmentation microcosms containing RAMM as the liquid phase (#8a–b and 28a–b). In microcosms #8a–b, an average decline of 34.9 ± 0.5 mol of PCE per bottle and the formation of 3.4 ± 0.5 mol of cDCE and 12.4 ± 0.5 mol of ethene were observed on day 226 Figure 47. In microcosms #28a–b, an average decline of 29.9 ± 0.5 mol of PCE and the formation of 1.8 ± 0.5 mol of cDCE and 9.2 ± 0.5  mol of ethene were observed on day 226. Ethene was detected in all bioaugmentation microcosms (#4a–b, 8a–b, 28a–b), but was below the LOD in microcosms #4a–b, which had groundwater as the liquid phase. The LOD and LOQ for ethene were 0.3 ± 0.5 mol and 1.0 ± 0.5 mol, respectively. Quantifiable amounts of ethene were observed in bioaugmentation microcosms containing RAMM as the liquid phase (#8a–b and 28a–b) on day 226 Figure 47. VC was not detected in any of the microcosms. The LOD for VC is 1.0 ± 0.5 mol per bottle. Methane was detected in bioaugmentation microcosms on day 226. An average of 1.2 ± 0.5 mol of methane was detected in microcosms #4a–b, 29.7 ± 0.5 mol methane was observed in microcosms #8a–b, and 18.6 ± 0.5 mol methane was observed in microcosms #28a–b. The GC-FID is quite sensitive to methane. The LOD is 0.04 mol of methane per bottle, and the LOQ is 0.1 mol of methane per bottle. Methane formation was not observed in the heat-killed control, MNA, or biostimulation microcosms. The data generated from the bioaugmentation microcosms suggests that reductive dechlorination did not occur prior to the second addition of 0.5 mM sodium sulfide on day 63.

Figure 47. Changes in the amount of PCE, cDCE, and ethene in µmol per bottle over time.

Panel (a) shows cVOC data from microcosms #4a–b, which contain upper aquifer material and groundwater. Panel (b) shows cVOC data from microcosms #8a–b, which contain upper aquifer material and RAMM. Panel (c) shows cVOC data from microcosms #28a–b, which contain lower aquifer material and RAMM. All microcosms were bioaugmented with consortium SDC-9. The addition of a second dose of 0.5 mM sodium sulfide on day 63 is indicated by the star.

### Lactate fermentation in PCE-containing microcosms

Lactate is a fermentable substrate that generally leads to the formation of hydrogen, propionate, and acetate. Organic acid concentrations were monitored by HPLC every 14 days Table 21. The target concentration for microcosms containing RAMM as the aqueous phase was 5 mM, but the actual measured starting concentrations were 4.5 mM or 3.8 mM. Two 2-L bottles of RAMM were used to set up microcosms, and they had different concentrations of lactate. The variation in starting lactate concentrations between the two 2-L bottles of RAMM may have been due to the retention of some of the 60% (w/w) sodium DL-lactate syrup on the neck of the 2-L flask during medium preparation, causing incomplete dissolution of the lactate added to the medium. Microcosms (ID# 27, 28) were set up with RAMM that had 4.5 mM of lactate. Microcosms (ID# 7, 8) were set up with RAMM that had 3.8 mM of lactate. Microcosms (ID# 3, 4) contained groundwater as the aqueous phase, and lactate was added by injecting 0.5 mL from an 800-mM stock solution using a 3-mL syringe equipped with a 2-µm filter, resulting in a starting concentration of 4 mM for microcosms containing groundwater.

In biostimulated microcosms containing PCE, lactate fermentation occurred in microcosms that contained RAMM as the liquid phase (ID# 7, 27; Table 21). Lactate fermentation did not occur in biostimulated microcosms that contained groundwater as the liquid phase (ID# 3). These data suggest that the groundwater contains a chemical that is inhibitory to lactate-fermenting organisms. It is possible that residual amounts of CT or CFC-113 are responsible for the inhibition of lactate fermentation in the PCE-containing biostimulated microcosms.30,31,34,183–185 All microcosms set up to test the effectiveness of combined bioaugmentation and biostimulation as a remediation strategy fermented lactate regardless of the liquid phase. These data suggest that the bioaugmentation culture SDC-9 contains lactate fermenters that are resistant to the inhibitory effects of the compound in the groundwater.

Table 21. Consumption of lactate and formation of acetate and propionate in lactate-amended microcosms containing PCE.

The numbers at the top of each column refer to the sample ID#, as described in the text and Table 28 the “Day” column refers to the days after microcosm initiation. The reported value is the average of two replicates. A heat map function is used to indicate the consumption of lactate and the formation of propionate and acetate. All microcosms were amended with an additional 0.5 mM of sodium sulfide on day 63. Microcosms (ID# 8a–b, 28a–b) were also amended with an additional 5 mM of lactate on day 63.

CT transformation Some decline in the amounts of CT was observed in all microcosms, including the heat-killed controls, MNA, biostimulation, and bioaugmentation microcosms, but no transformation products were observed Figure 48. In heat-killed controls, CT decreased by 8.2 ± 0.5 mol per bottle following a 226-day incubation period. CT decreases of 6.2 ± 0.5 mol, 11.0 ± 0.5 mol, and 9.5 ± 0.5 mol were observed in MNA, biostimulation, and bioaugmentation microcosms, respectively, following a 226-day incubation period.

Bioaugmentation culture KB-1 Plus contains amorphous FeS, which has been shown to transform CT to CF.73,152 Based on information provided by SiREM, the amount of FeS present in the KB-1 Plus bioaugmentation culture is 0.02 g/L. Each bioaugmentation microcosm with CT was amended with 3 mL of the KB-1 Plus at the start, and an additional 3 mL of KB-1 Plus was added on day 63. Thus, the total amount of FeS added to each CT-containing bioaugmentation microcosm was 0.0012 g/L. This small amount of FeS added to the bioaugmentation microcosms did not result in observable increases in CT transformation compared to the other treatments.

Figure 48. Amounts of CT in the microcosms over time.

Panel (a) contains cVOC data from heat-killed control microcosms (ID# 9a, 13a, 29a). Panel (b) contains cVOC data from MNA microcosms (ID# 10a, 14a, 30a). Panel (c) contains cVOC data from biostimulation microcosms (ID# 11a–b,15a–b, 31a–b). Panel (d) contains cVOC data from bioaugmentation microcosms (ID# 12a–b, 16a–b, 32a–b). The star indicates the addition of a second dose of 0.5 mM of sodium sulfide on day 63.

### Lactate fermentation and formation in CT-containing microcosms

Organic acid concentrations were monitored by HPLC every 14 days Table 22. The target concentration for microcosms containing RAMM as the aqueous phase was 5 mM, but the actual measured starting concentrations were 4.5 mM or 3.8 mM. Two 2-L bottles of RAMM were used to set up microcosms, and they had different concentrations of lactate. The variation in starting lactate concentrations between the two 2-L bottles of RAMM may have been due to retention of some of the 60% (w/w) sodium DL-lactate syrup on the neck of the 2-L flask during medium preparation, causing incomplete dissolution of the lactate added to the medium. Microcosms (ID# 31, 32) were set up with RAMM that had 4.5 mM lactate. Microcosms (ID# 15, 16) were set up with RAMM that had 3.8 mM lactate. Microcosms (ID# 11, 12) contained groundwater as the aqueous phase, and lactate was added by injecting 0.5 mL from an 800-mM stock solution using a 3-mL syringe equipped with a 2-µm filter, resulting in a starting concentration of 4 mM for microcosms containing groundwater.

Lactate fermentation did not occur in any biostimulated microcosms that contained CT (ID# 11, 15, 31; Table 22). Lactate fermentation occurred in all CT-containing microcosms that were bioaugmented and biostimulated (ID# 12, 16, 32), though fermentation occurred more slowly here than it did in bioaugmented microcosms that contained PCE or CF. This suggests that the lactate-fermenting organisms present in KB1 Plus were able to overcome the inhibitory effects of the CT that these microcosms were treated with.

Table 22. Consumption of lactate and formation of acetate and propionate in lactate-amended microcosms containing CT.

The numbers at the top of each column refer to the sample ID#, as described in the text and Table 28. The “Day” column refers to the days after microcosm initiation. The reported value is the average of two replicates. A heat map function is used to indicate the consumption of lactate and the formation of propionate and acetate. All microcosms were amended with an additional 0.5 mM of sodium sulfide on day 63.

CF transformation CF concentrations decreased in all microcosms, including the heat-killed control microcosms. Transformation products were not observed, suggesting that the decrease in CF concentration was due to sorption to the stopper or aquifer material, not dechlorination Figure 49. On average, CF amounts declined by 4.1 ± 0.5 mol, 2.4 ± 0.5  mol, and 1.9 ± 0.5 mol per bottle in heat-killed control, MNA, and biostimulation microcosms, respectively. In bioaugmentation microcosms (ID# 20a–b, 24a–b, 36a–b), an average decline of 12.8 ± 0.5 mol per bottle was observed. There was variability in the performance of the bioaugmentation microcosms; thus, they were plotted individually Figure 50. Microcosms #20a–b were established with groundwater as the liquid phase, and an average decrease of 2.5 ± 0.5 mol of CF per bottle was observed on day 226. This decrease in CF is similar to the decrease observed in the heat-killed control, MNA, and biostimulation microcosms and is likely due to sorption to the stopper and/or the aquifer material. A possible explanation for the lack of dechlorination in the bioaugmentation microcosms containing groundwater is inhibition by residual CT and CFC-113.30,31,34,183,184 The bioaugmentation microcosms containing aquifer material and RAMM as the liquid phase (ID# 24a–b, 36a–b) provided evidence for CF dechlorination. In microcosms #24a–b, which contained upper aquifer material and RAMM, an average of 20.3 ± 0.5 mol of CF per bottle was consumed by day 86. These microcosms produced 7.4 ± 0.5 mol of DCM by day 86, which was completely consumed by day 98. Microcosms #36a–b, which were established with aquifer material from the lower section of the sediment cores and RAMM, consumed an average of 15.7 ± 0.5 mol of CF. In these microcosms, an average of 3.6 ± 0.5 mol of DCM per bottle had been produced by day 70 and consumed by day 86. Methane was detected in all bioaugmentation microcosms, though much less methane was formed in #20a–b (0.69 ± 0.5 mol, which contained groundwater as the liquid phase). In microcosms #24a–b, 634.71 ± 0.5 mol of methane were detected, and in microcosms #3a–b, 747.4 ± 0.5 mol of methane were detected. Methane formation was not observed in heat-killed control, MNA, or biostimulation microcosms containing CF. The LOD for methane is 0.04 mol, and the LOQ is 0.1 mol of methane per bottle. Bioaugmentation microcosms amended with CF were inoculated with the KB-1 Plus culture. Bioaugmentation culture KB-1 Plus contains a Dehalobacter population capable of reductively dechlorinating CF to DCM.186,187 DCM can then be degraded by organisms such as Dehalobacterium formicoaceticum or the ‘Candidatus Dichloromethanomonas elyunquensis’ strain RM.173,188 Dechlorination of CF was observed in microcosms containing aquifer material and RAMM as the liquid phase after the addition of a second dose of 0.5 mM of sodium sulfide on day 63, suggesting that non-reducing conditions had prevented dechlorination prior to day 63.

Figure 49. Amounts of CF and transformation products over time.

Panel (a) shows cVOC data from heat-killed control microcosms (ID# 17a, 21a, 33a). Panel (b) shows cVOC data from MNA microcosms (ID# 18a, 22a, 34a). Panel (c) depicts cVOC data from biostimulation microcosms (ID# 19a–b, 23a–b, 35a–b). Panel (d) shows cVOC data from bioaugmentation microcosms (ID# 20a–b, 24a–b, 36a–b). The star indicates the addition of a second dose of 0.5 mM of sodium sulfide on day 63.

Figure 50. Amounts of CF and transformation products measured in bioaugmentation microcosms #20a–b, 24 a–b, and 26 a–b over time.

Lactate fermentation in CF-containing microcosms

Organic acid concentrations were monitored by HPLC every 14 days Table 23. The target concentration for microcosms containing RAMM as the aqueous phase was 5 mM, but the actual measured starting concentrations were 4.5 mM or 3.8 mM. Two 2-L bottles of RAMM were used to set up microcosms, and they had different concentrations of lactate. The variation in starting lactate concentrations between the two 2-L bottles of RAMM may have been due to retention of some of the 60% (w/w) sodium DL-lactate syrup on the neck of the 2-L flask during medium preparation, causing incomplete dissolution of the lactate added to the medium. Microcosms (ID# 23, 24, 35, 36) were set up with RAMM that had 4.5 mM of lactate. Microcosms (ID# 19, 20) contained groundwater as the aqueous phase, and lactate was added by injecting 0.5 mL from an 800-mM stock solution using a 3-mL syringe equipped with a 2-µm filter, resulting in a starting concentration of 4 mM for microcosms containing groundwater.

In biostimulated microcosms containing CF, lactate fermentation occurred in microcosms that contained RAMM as the liquid phase (ID# 23, 35; Table 23). Lactate fermentation did not occur in biostimulated microcosms that contained groundwater as the liquid phase (ID# 19). These data suggest that the groundwater contains a chemical that is inhibitory to lactate-fermenting organisms. It is possible that residual amounts of CT or CFC-113 are responsible for the inhibition of lactate fermentation in the CF-containing biostimulated microcosms.30,31,34,183–185 All microcosms set up to test the effectiveness of combined bioaugmentation and biostimulation as a remediation strategy fermented lactate regardless of the liquid phase. These data suggest that the bioaugmentation culture KB1 Plus contains lactate fermenters that are resistant to the inhibitory effects of the compound in the groundwater.

Table 23. Consumption of lactate and formation of acetate and propionate in lactate-amended microcosms containing CF.

The numbers at the top of each column refer to the sample ID#, as described in the text and Table 28. The “Day” column refers to the days after microcosm initiation. The reported value is the average of two replicates. A heat map function is used to indicate the consumption of lactate and the formation of propionate and acetate. All microcosms were amended with an additional 0.5 mM of sodium sulfide on day 63. Microcosms (ID# 24a–b, 36a–b) were also amended with an additional 5 mM of lactate on day 63.

Microcosms testing abiotic degradation Microcosms were established to explore the capacity of ZVI to transform CT Figure 51. Contaminant concentrations were monitored for 96 hr using headspace samples. Complete degradation of CT and formation of CF were observed in all abiotic microcosms containing ZVI (ID# 37a, 38a, 39a) within 48 hr. The use of ZVI to remove CT may alleviate the inhibiting effects of CT and CFC-113 on microbial reduction of PCE and CF in the aquifer material.189,190

Figure 51. Abiotic transformation of CT by ZVI in microcosms established with upper aquifer material, lower aquifer material, and a control containing no aquifer material.

## Discussion

In this treatability study, four remediation strategies were tested for their effectiveness at degrading PCE, CT, and CF in the aquifer material from the site. Anoxic conditions were used because known OHRB required anaerobic conditions. Degradation was not observed under biostimulation or MNA conditions. Biostimulation and MNA as remediation strategies rely on the presence of native microorganisms (e.g., OHRB) in the groundwater aquifer capable of degrading the target contaminants. These observations combined with the low detection of known OHRB via qPCR assays suggest that there is not an active community of OHRB present at this site and that indigenous processes are unlikely to have a major impact on remediating the target contaminants.

Degradation of PCE and CF was observed in microcosms set up to test bioaugmentation combined with biostimulation that contained groundwater as the liquid phase, but degradation was unsuccessful in microcosms set up with groundwater as the liquid phase. These observations suggest that a compound that is inhibitory to OHRB is present in the groundwater. It is possible that CT or another contaminant is responsible for this inhibition. Degradation of CT was not observed in any microcosms set up to test bioaugmentation combined with biostimulation as a remediation strategy. This observation is not surprising, as CT is not respired by any known OHRB, and is known to be toxic to bacteria.30,31,34,183,184 If bioaugmentation combined with biostimulation was to be implemented as a remediation strategy, removal or transformation of the inhibitory compounds would be necessary at the site. Additionally, the aquifer would need to be reduced for bioaugmentation combined with biostimulation to be an effective remediation strategy.

Amendments of ZVI to microcosms containing CT facilitated the transformation of CT to CF. In addition to transforming CT to CF, ZVI is a reductant, and addition of ZVI to the aquifer may help to establish reducing conditions.189,190

## Appendix

Table 24. Contaminant concentrations measured in the groundwater at the contaminated site.

| Compound | Concentration (mg/L) |
| --- | --- |
| Carbon tetrachloride (CT) | 2.10 |
| Chloroform (CF) | 0.65 |
| Tetrachloroethene (PCE) | 1.10 |
| 1,1,2-Trichloro-1,2,2-trifluoroethane (CFC-113) | 1.10 |

Figure 52. Schematic showing the temporary well locations surrounding the monitoring well at the contaminated site.

Table 25. Groundwater parameters determined in the monitoring well located in the center of the four sampling locations (data provided by Parsons).

| Parameter | Value/Observation | Units |
| --- | --- | --- |
| Color | Clear | - |
| Water Table Depth a | 9.01 | ft |
| Dissolved Oxygen b | 0.53 | mg/L |
| Odor | None | - |
| pH | 7.28 | -log[H+] |
| Redox | 102.2 | mV |
| Specific Conductance b | 33,954 | µmho/cm |
| Temperature | 25.77 | C |
| Turbidity c | 3.9 | NTU |
| Total Depth | 19.15 | ft |

a The depth to water from the top of the well casing is reported in feet.

b The unit of specific conductance used is micromhos per centimeter (mho/cm).

c Turbidity is reported in nephelometric turbidity units (NTU).

Table 26. Volumes of groundwater filtered through each Sterivex cartridge.

|  | Volume of groundwater filtered (mL) |  |
| --- | --- | --- |
| Sampling location | Sterivex cartridge 1 | Sterivex cartridge 2 |
| B-1-E | 1,200 | 1,000 |
| B-2-S | 2,000 | 1,300 |
| B-3-W | 1,250 | 2,000 |
| B-4-N | 1,500 | 1,750 |

Figure 53. Aquifer material cores that were received prior to separating the cores into lower (bottom) and upper (top) cores.

Figure 54. Transfer of aquifer material slurry to sterile and N2/CO2-flushed 160-mL glass serum bottles.

Table 27. cVOCs added to microcosms.

| Contaminant | µL added | µmol per bottle | mM (aq) |
| --- | --- | --- | --- |
| PCE | 3 | 29 | 0.205 |
| CT | 2 | 20 | 0.118 |
| CF | 2 | 25 | 0.227 |

Figure 55. Dispensing of synthetic, defined reduced anoxic mineral salt medium (RAMM) to the glass serum bottles containing the composite slurry.

Figure 56. Microcosms established with aquifer slurries and composite groundwater or RAMM.

Table 28. Microcosms established to assess target contaminant degradation.

| Identifier (ID#) | Source Aquifer Material | Electron Donor a | Treatment | Liquid Phase b | cVOC (µmol/bottle) |
| --- | --- | --- | --- | --- | --- |
| 1a | Upper | - | Killed | GW | PCE (29) |
| 2a | Upper | - | Native | GW | PCE (29) |
| 3a | Upper | Yes | Stimulated | GW | PCE (29) |
| 3b | Upper Aquifer Material | Yes | Stimulated | GW | PCE (29) |
| 4a | Upper Aquifer Material | Yes | SDC-9 | GW | PCE (29) |
| 4b | Upper Aquifer Material | Yes | SDC-9 | GW | PCE (29) |
| 5a | Upper Aquifer Material | - | Killed | RAMM | PCE (29) |
| 6a | Upper Aquifer Material | - | Native | RAMM | PCE (29) |
| 7a | Upper Aquifer Material | Yes | Stimulated | RAMM | PCE (29) |
| 7b | Upper Aquifer Material | Yes | Stimulated | RAMM | PCE (29) |
| 8a | Upper Aquifer Material | Yes | SDC-9 | RAMM | PCE (29) |
| 8b | Upper Aquifer Material | Yes | SDC-9 | RAMM | PCE (29) |
| 9a | Upper Aquifer Material | - | Killed | GW | CT (20) |
| 10a | Upper Aquifer Material | - | Native | GW | CT (20) |
| 11a | Upper Aquifer Material | Yes | Stimulated | GW | CT (20) |
| 11b | Upper Aquifer Material | Yes | Stimulated | GW | CT (20) |
| 12a | Upper Aquifer Material | Yes | KB-1® Plus | GW | CT (20) |
| 12b | Upper Aquifer Material | Yes | KB-1® Plus | GW | CT (20) |
| 13a | Upper Aquifer Material | - | Killed | RAMM | CT (20) |
| 14a | Upper Aquifer Material | - | Native | RAMM | CT (20) |
| 15a | Upper Aquifer Material | Yes | Stimulated | RAMM | CT (20) |
| 15b | Upper Aquifer Material | Yes | Stimulated | RAMM | CT (20) |
| 16a | Upper Aquifer Material | Yes | KB-1® Plus | RAMM | CT (20) |
| 16b | Upper Aquifer Material | Yes | KB-1® Plus | RAMM | CT (20) |
| 17a | Upper Aquifer Material | - | Killed | GW | CF (25) |
| 18a | Upper Aquifer Material | - | Native | GW | CF (25) |
| 19a | Upper Aquifer Material | Yes | Stimulated | GW | CF (25) |
| 19b | Upper Aquifer Material | Yes | Stimulated | GW | CF (25) |
| 20a | Upper Aquifer Material | Yes | KB-1® Plus | GW | CF (25) |
| 20b | Upper Aquifer Material | Yes | KB-1® Plus | GW | CF (25) |
| 21a | Upper Aquifer Material | - | Killed | RAMM | CF (25) |
| 22a | Upper Aquifer Material | - | Native | RAMM | CF (25) |
| 23a | Upper Aquifer Material | Yes | Stimulated | RAMM | CF (25) |
| 23b | Upper Aquifer Material | Yes | Stimulated | RAMM | CF (25) |
| 24a | Upper Aquifer Material | Yes | KB-1® Plus | RAMM | CF (25) |
| 24b | Upper Aquifer Material | Yes | KB-1® Plus | RAMM | CF (25) |
| 25a | Lower Aquifer Material | - | Killed | RAMM | PCE (29) |
| 26b | Lower Aquifer Material | - | Native | RAMM | PCE (29) |
| 27a | Lower Aquifer Material | Yes | Stimulated | RAMM | PCE (29) |
| 27b | Lower Aquifer Material | Yes | Stimulated | RAMM | PCE (29) |
| 28a | Lower Aquifer Material | Yes | SDC-9 | RAMM | PCE (29) |
| 28b | Lower Aquifer Material | Yes | SDC-9 | RAMM | PCE (29) |
| 29a | Lower Aquifer Material | - | Killed | RAMM | CT (20) |
| 30a | Lower Aquifer Material | - | Native | RAMM | CT (20) |
| 31a | Lower Aquifer Material | Yes | Stimulated | RAMM | CT (20) |
| 31b | Lower Aquifer Material | Yes | Stimulated | RAMM | CT (20) |
| 32a | Lower Aquifer Material | Yes | KB-1® Plus | RAMM | CT (20) |
| 32b | Lower Aquifer Material | Yes | KB-1® Plus | RAMM | CT (20) |
| 33a | Lower Aquifer Material | - | Killed | RAMM | CF (25) |
| 34a | Lower Aquifer Material | - | Native | RAMM | CF (25) |
| 35a | Lower Aquifer Material | Yes | Stimulated | RAMM | CF (25) |
| 35b | Lower Aquifer Material | Yes | Stimulated | RAMM | CF (25) |
| 36a | Lower Aquifer Material | Yes | KB-1® Plus | RAMM | CF (25) |
| 36b | Lower Aquifer Material | Yes | KB-1® Plus | RAMM | CF (25) |
| 37a | Upper Aquifer Material | - | ZVI | Anoxic H2O | CT (10) |
| 38a | Lower Aquifer Material | - | ZVI | Anoxic H2O | CT (10) |
| 39a | - | - | ZVI | Anoxic H2O | CT (10) |

a Lactate (5 mM) was provided as an electron donor.

b RAMM: reduced anoxic mineral salt medium.

# CHAPTER IV

# Effect of the potential of hydrogen on the Biomimetic Dehalogenation of trichlorofluoromethane, dichlorodifluoromethane, and carbon tetrachloride by Super-Nucleophilic vitamin B12.

## Abstract

Chlorofluorocarbons are recalcitrant contaminants found in the atmosphere and subsurface. This study examined the dehalogenation of trichlorofluoromethane (CFC-11), dichlorodifluoromethane (CFC-12), and tetrachloromethane (CT) by super-nucleophilic vitamin B12 at pH 5, 7, and 9. Additionally, the dehalogenation by super-nucleophilic vitamin B12 of possible transformation products of CFC-11 and CFC-12 was studied at pH 7. The potential transformation products studied were chlorofluoromethane, chlorodifluoromethane, difluoromethane, and fluoromethane. All reactions in this study occurred in closed anaerobic batch reactors. Changes in contaminant concentrations were monitored by gas chromatography with a flame ionization detector. Super-nucleophilic vitamin B12 is capable of dehalogenating CFC-11, CFC-12, and carbon tetrachloride at pH 5, 7, and 9. Changes in pH affected which transformation products formed in reactions that contained CFC-11 and carbon tetrachloride. Chlorodifluoromethane was the only transformation product identified in the reactions with CFC-12 at all tested pHs. No dehalogenation was observed in the reactions with super-nucleophilic vitamin B12 and chlorofluoromethane, chlorodifluoromethane, difluoromethane, or fluoromethane.

## Introduction

Chlorofluorocarbons (CFCs) and tetrachloromethane (CT) are anthropogenic chemicals that were first synthesized as refrigerats.10,191 Their use was later expanded to many other industrial applications. Due to their extensive use and improper handling, CT and CFCs such as trichlorofluoromethane (CFC-11) and dichlorodifluoromethane (CFC-12) are common groundwater contaminants.37,84,191–193 Additionally, due to the high vapor pressure of CFC-11, CFC-12, and CT at ambient temperatures, these compounds collect in the atmosphere, where they facilitate ozone depletion.11,26,44,192,194,195 Production of CFC-11, CFC-12, and CT has been phased out under the 1987 Montreal Protocol, but they remain a challenge for environmental cleanup as they are recalcitrant.36

The environmental fate of CFC-11 and CFC-12 has been studied as they are used as groundwater tracers to calculate the age and recharge rate of young (~100 years old) groundwater.196 They are considered useful groundwater tracers as they are thought to react conservatively, were released globally, and their production and release rates are known and measured. Though the transformation of CFCs in groundwater is considered conservative, degradation of CFC-11 and CFC-12 has been observed in pyrite-oxidizing aquifers and sulfate-reducing aquifers.85,86,197 Laboratory studies have reported degradation of CFC-11, CFC-12, and CT under sulfate-reducing and methanogenic conditions.77,78,84,90,160,198,199 This suggests that methanogenic archaea and sulfate-reducing bacteria may contribute to the degradation of CFC-11, CFC-12, and CT, possibly through their production of cobamides that can react with CFCs.200–204

Cobamides are cofactors that contain a corrin ring with a cobalt center, an upper ligand coordinated with the cobalt center, and an exchangeable lower base which is connected to the corrin ring by a nucleotide loop.99 The oxidation state of cobalt can vary from -1 to +4, though the +2 and +3 oxidation states are most frequently observed.205 The oxidation state of a coordinated cobalt at the center of a corrin ring can be reduced from the +2 or +3 oxidation states to a +1 oxidation state in the presence of a strong reductant like Ti(III)citrate or ZVI.93,206 When the oxidation state of the centrally coordinated cobalt is +1, the cobamide is considered super-nucleophilic.185,206 Studies have shown that vitamin B12 and other cobamides can dehalogenate CFC-11, CFC-12, and CT.93,148,207 Dehalogenation by vitamin B12 occurs via electron donation from the cobalt center of the corrin ring.185,206,208,209 Vitamin B12 acts as a catalytic electron shuttle when it reduces CFC-11, CFC-12 or CT, as such a single vitamin B12 molecule can be regenerated and facilitate numerous reactions.94,209

Krone and collogues reported that carbon monoxide and a mixture of halogenated transformation products formed via the reduction of CFC-11, CFC-12, and CT by super-nucleophilic vitamin B12.93 They suggested that carbon monoxide was formed via a carbene intermediate that hydrolyzed to carbon monoxide. It is important to understand the factors that affect the distribution of transformation products, as some transformation products are less favorable than others. For example, the formation of dichlorofluoromethane or chlorofluoromethane from CFC-11 is not favorable, as dichlorofluoromethane is more toxic than CFC-11 and chlorofluoromethane is carcinogenic and flammable.58 It is conceivable that pH would affect the distribution of transformation products formed during a reaction between super-nucleophilic vitamin B12 and CFC-11, CFC-12, or CT, as the amount of protons available in a solution is likely to affect, the ionic interactions between a di-halogen carbene and the cobalt center of a super-nucleophilic vitamin B12 molecule.94,130 This work examined the effect of pH (5, 7, 9) on the distribution of transformation product formation from the dehalogenation of CFC-11, CFC-12, and CT by super-nucleophilic vitamin B12. Experiments were also performed that examined the ability of super-nucleophilic vitamin B12 to dehalogenate possible transformation products of CFC-11 and CFC-12; the transformation products tested were chlorofluoromethane, chlorodifluoromethane, difluoromethane, and fluoromethane. Hypothesis explored in this work are i) that changes in pH will effect which transformation products form during the dehalogenation of CFC-11, CFC-12, or CT by super-nucleophilic vitamin B12, ii) that super-nucleophilic vitamin B12 can dehalogenate chlorofluoromethane, chlorodifluoromethane, difluoromethane, and fluoromethane.

## Materials and Methods

### Chemicals

CFC-11 (99%), CFC-12 (99%), fluoromethane (99%), difluoromethane (99%), chlorofluoromethane (99%), and chlorodifluoromethane (99 %), vinyl chloride (99%) were purchased from SynQuest Laboratories. CT (99%) and Vitamin B12 were purchased from Fisher Scientific. Titanium(III)chloride (15%) in 10 % HCl, methane (99%), tetrachloroethene (99%), trichloroethene (99.5%), chloroform (99.5%), dichloromethane (99.8%), chloromethane (99.5%), cis-1,2-dichloroethene (97%), ethene (99.5%), dichlorofluoromethane (98%) were purchased from Millipore Sigma.

Cyanocobalamin-catalyzed dehalogenation

Dehalogenation of CFC-11, CFC-12, and CT by super-nucleophilic vitamin B12 was assessed in 30 mL anoxic water at pH 5, 7, or 9 and 30 mL of N2 headspace in 60 mL glass serum bottles. CFC-11 (108 µmol per bottle) and CT (103 µmol per bottle) were added by designated Hamilton syringes, and CFC-12 (122 µmol per bottle) was added with 3 mL plastic syringes fitted with 25 G needles. Reaction vessels containing CFC-11, CFC-12, or CT were stored stopper down for 24 hours to allow the chemicals to equilibrate. Once equilibrated, headspace samples were taken to determine the starting concentration of the chemical in each bottle. Stock solutions of vitamin B12 (1 mM) and Titanium(III)-citrate (200 mM) were prepared fresh for each experiment. Titanium(III)-citrate was added to a final concentration of 5.3 mM in each reaction vessel. The concentration of Titanium(III)-citrate was approximately 100 times that of the vitamin B12 added to each reaction; this was done to ensure that the vitamin B12 was reduced to the super-nucleophilic Co(I) state and that the super-nucleophilic vitamin B12 could be regenerated. Solutions of NaOH (10 M and 1 M) and HCl (6 M and 1 M) prepared in anoxic MilliQ water were used to adjust the solution's pH before the addition of vitamin B12. Samples were taken from each reaction vessel, and the pH was measured. Finally, vitamin B12 (49.5 µM) was added to the reaction vessels, and the reactions were monitored by gas chromatography with a flame ionization detector. All stock solutions were made with deionized, degassed Milli-Q water (Milli-Q Corporation, Bedford, MA), and the Hungate technique was used to remove oxygen. Control bottles that lacked either vitamin B12 or titanium(III)-citrate accompanied each experiment.

Degradation of chlorofluoromethane, chlorodifluoromethane, difluoromethane, and fluoromethane by super-nucleophilic vitamin B12 was explored in 60 mL glass serum bottles containing 30 mL anoxic water pH 7. Each reaction vessel received 122 µmol of chlorodifluoromethane, difluoromethane, fluoromethane, or chlorofluoromethane. 3 mL plastic syringes fitted with 25 G needles were used to inject the halogenated gasses. Titanium(III)-citrate (5.3 mM) and vitamin B12 (49.5 µM) were added to the reaction vessels, and the reactions were monitored by gas chromatography with a flame ionization detector for 24 hours.

### Analytical methods.

Experiments in which CFC-11, CFC-12, chlorodifluoromethane, difluoromethane, fluoromethane, chlorofluoromethane were the starting chemical, were monitored by injecting headspace samples (100 μL) into an Agilent 7890 gas chromatograph equipped with a PLOTQ column (30 m × 535 µm × 40 µm), and a flame ionization detector. Experiments with CT as the starting chemical were monitored by injecting headspace samples (100 μL) into an Agilent 7890 gas chromatograph equipped with a DB-624 capillary column (60 m × 0.32 mm × 1.8 μm), and a flame ionization detector. Quantification was attained by applying external standard curves. Standards were established by adding known amounts of each compound to 60 mL bottles with the same liquid-to-headspace ratio. Liquid compounds were added neat using dedicated microliter Hamilton syringes, and gaseous compounds were added using 3 mL plastic syringes.

Standard curves for CFC- 11, CFC-12, chlorofluoromethane, chlorodifluoromethane, dichlorofluoromethane, fluoromethane, difluoromethane, CT, CF, DCM, CM, CH4, PCE, TCE, cis-DCE, VC, and ethene were analyzed using the Agilent 7890 gas chromatograph equipped a PLOTQ column (30 m × 535 µm × 40 µm), and a flame ionization detector Figure 58. Additionally, standard curves for CT, CF, DCM, CM, CH4, PCE, TCE, cis-DCE, VC, and ethene were also analyzed on an Agilent 7890 gas chromatograph equipped with a DB-624 capillary column (60 m × 0.32 mm × 1.8 μm) Figure 59. Transformation products were determined by aligning the retention times on the chromatograms of peaks generated during an experiment with the known retention times of chemicals for which standard curves were established. Analytical conditions of the gas chromatography methods and the established standard curves are available in the appendix Table 33 and Table 34. To calculate the total amounts of the halogenated compound in the incubation vessels, dimension-less Henry's constants were used Table 35.

## Results

Experiments to determine if super-nucleophilic vitamin B12 could dehalogenate CFC-11, CFC-12, or CT were performed at pH 5, 7, and 9. Complete transformation of CFC-11, CFC-12, and CT was observed at pH 5, 7, and 9 in the presence of super-nucleophilic vitamin B12 Figure 57. Declines in the CFC-11, CFC-12, and CT concentrations were observed in control vessels that contained vitamin B12 without a reductant and in control vessels that had only Ti(III)citrate. However, the declines in CFC-11, CFC-12, and CT concentration were less than those observed in the vessels containing both vitamin B12 and Ti(III)citrate. Transformation products were not observed in control vessels that contained CFC-11 or CFC-12 Table 29 and Table 30. CF and PCE were observed in control vessels that contained CT and Ti(III)citrate at pH 7.

The percentages of carbon accounted for as identified transformation products were less than 25% for CFC-12 and CT and greater than 50% for CFC-11 Table 29 and Table 30. A greater percentage of carbon was accounted for as transformation products as the pH increased for vessels that contained CFC-11 and CFC-12. In contrast, the percentage of carbon accounted for as transformation products did not vary with pH for CT. Four transformation products were identified in vessels in which CFC-11 was degraded: chlorofluoromethane, fluoromethane, methane, and ethene Table 29. Chlorodifluoromethane was the only transformation product identified in vessels where CFC-12 was degraded Table 30. Chloroform, dichloromethane, chloromethane, methane, tetrachloroethene, and ethene were identified as transformation products in vessels where CT was degraded Table 31.

### Transformation product experiment.

The ability of super-nucleophilic vitamin B12 to dehalogenate possible transformation products of CFC-11 and CFC-12 was explored by monitoring vessels that contained vitamin B12, Ti(III)citrate, and either chlorodifluoromethane, difluoromethane, fluoromethane, or chlorofluoromethane. Dehalogenation or a decline in the starting concentration of the target chemical was not observed in any vessels Table 32.

Figure 57. Reactions with super-nucleophilic vitamin B12 and Ti(III)citrate.

Table 29. CFC-11 pH variation experiment performance at 24 hours.

| Chemical | Experimental conditions | Decline in CFC-11 concentration (%) | Carbon identified as transformation products (%) | Methane (µmol) | Ethene (µmol) | Fluoromethane (µmol) | Chlorofluoromethane (µmol) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CFC-11 | Vitamin B12, Ti(III)citrate at pH 5 | 100 | 43 | 34 | 5 | 0 | 5 |
| CFC-11 | Vitamin B12, Ti(III)citrate at pH 7 | 100 | 39 | 7 | 2 | 11 | 11 |
| CFC-11 | Vitamin B12, Ti(III)citrate at pH 9 | 100 | 102 | 7 | 2 | 0 | 106 |
| CFC-11 | Vitamin B12 pH 5 | 34 | 0 | 0 | 0 | 0 | 0 |
| CFC-11 | Vitamin B12 pH 7 | 3 | 0 | 0 | 0 | 0 | 0 |
| CFC-11 | Vitamin B12 pH 9 | 0 | 0 | 0 | 0 | 0 | 0 |
| CFC-11 | Ti(III)citrate pH 5 | 8 | 0 | 0 | 0 | 0 | 0 |
| CFC-11 | Ti(III)citrate pH 7 | 11 | 0 | 0 | 0 | 0 | 0 |
| CFC-11 | Ti(III)citrate pH 9 | 13 | 0 | 0 | 0 | 0 | 0 |

Table 30. CFC-12 pH variation experiment performance at 24 hours.

| Chemical | Experimental conditions | Decline in CFC-12 concentration (%) | Carbon identified as transformation products (%) | Chlorodifluoromethane (µmol) |
| --- | --- | --- | --- | --- |
| CFC-12 | Vitamin B12 and Ti(III)citrate at pH 5 | 100 | 8 | 59 |
| CFC-12 | Vitamin B12 and Ti(III)citrate at pH 7 | 100 | 6 | 47 |
| CFC-12 | Vitamin B12 and Ti(III)citrate at pH 9 | 100 | 14 | 131 |
| CFC-12 | Vitamin B12 pH 5 | 23 | 0 | 0 |
| CFC-12 | Vitamin B12 pH 7 | 0 | 0 | 0 |
| CFC-12 | Vitamin B12 pH 9 | 0 | 0 | 0 |
| CFC-12 | Ti(III)citrate pH 5 | 46 | 0 | 0 |
| CFC-12 | Ti(III)citrate pH 7 | 0 | 0 | 0 |
| CFC-12 | Ti(III)citrate pH 9 | 0 | 0 | 0 |

Table 31. CT pH variation experiment performance at 24 hours.

| Chemical | Experimental conditions | Decline in CT concentration (%) | Carbon identified as transformation products (%) | Methane (µmol) | CF (µmol) | DCM (µmol) | CM (µmol) | PCE (µmol) | Ethene (µmol) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CT | Vitamin B12 and Ti(III)citrate at pH 5 | 100 | 22 | 4 | 0 | 3 | 6 | 11 | 0 |
| CT | Vitamin B12 and Ti(III)citrate at pH 7 | 100 | 19 | 12 | 0 | 0 | 25 | 0 | 0 |
| CT | Vitamin B12 and Ti(III)citrate at pH 9 | 100 | 25 | 13 | 0 | 0 | 0 | 0 | 10 |
| CT | Vitamin B12 pH 5 | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| CT | Vitamin B12 pH 7 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| CT | Vitamin B12 pH 9 | 4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| CT | Ti(III)citrate pH 5 | 13 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| CT | Ti(III)citrate pH 7 | 10 | 2 | 0 | 4 | 0 | 0 | 1 | 0 |
| CT | Ti(III)citrate pH 9 | 6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Table 32. Performance of reactions with super-nucleophilic B12 and transformation products at 24 hours.

| Chemical | Experimental conditions | Percent of starting chemical degraded |
| --- | --- | --- |
| Chlorodifluoromethane | Vitamin B12 and Ti(III)citrate at pH 7 | 0 |
| Chlorodifluoromethane | Vitamin B12 pH 7 | 0 |
| Chlorodifluoromethane | Ti(III)citrate pH 7 | 0 |
| Difluoromethane | Vitamin B12 and Ti(III)citrate at pH 7 | 0 |
| Difluoromethane | Vitamin B12 pH 7 | 0 |
| Difluoromethane | Ti(III)citrate pH 7 | 0 |
| Fluoromethane | Vitamin B12 and Ti(III)citrate at pH 7 | 0 |
| Fluoromethane | Vitamin B12 pH 7 | 0 |
| Fluoromethane | Ti(III)citrate pH 7 | 0 |
| Chlorofluoromethane | Vitamin B12 and Ti(III)citrate at pH 7 | 0 |
| Chlorofluoromethane | Vitamin B12 pH 7 | 0 |
| Chlorofluoromethane | Ti(III)citrate pH 7 | 0 |

## Discussion

Strategies to degrade CFCs are needed in order to remove them from the groundwater and atmosphere. This study explored the dehalogenation capacity of biomimetic degradation by super-nucleophilic vitamin B12. Degradation of CFC-11, CFC-12, and CT was observed in the presence of super-nucleophilic vitamin B12. Unfortunately, no dehalogenation was observed in reactions with super-nucleophilic vitamin B12 and chlorodifluoromethane, difluoromethane, fluoromethane, chlorofluoromethane. The lower degree of halogenation of these chemicals may make them less likely to be reduced, as their reduction potential has decreased.69

In vessels that contained CFC-11 and super-nucleophilic vitamin B12, shifts in the formation of transformation products occurred as pH increased. The dominant transformation product at pH 5 was methane, with small amounts of ethene and chlorofluoromethane identified. At pH 9, chlorofluoromethane was the predominant transformation product. The formation of chlorofluoromethane is an unfavorable outcome, as it is carcinogenic and flammable.58 Additionally, in reactions that occurred at pH 9 all of the carbon measured as CFC-11 at time zero was accounted for as transformation products at the 24 hour time point. In reactions that occurred at pH 5 and 7 roughly 40% or the carbon from the time zero measurement of CFC-11 was accounted for as transformation products at the 24 hour time point, this suggests that transformation products formed which were not identifiable by the method used in this study. It is possible that carbon monoxide formed via a carbene intermediate at pH 5 and 7, as reported by Krone and collogues,93 and that reaction pathways that form radical intermediates and halogenated transformation products are more favorable at pH 9, than reaction pathways that favor carbene intermediates and non-halogenated end products (Figure 1, Chapter 1). The ionic interaction between a carbene intermediate and the cobalt center of vitamin B12 may be destabilized in the less protonated environment of a pH 9 solution.

The influence of pH on reactions between super-nucleophilic vitamin B12 and CFC-12 was observable in the amounts of transformation products identified, but not in the composition of the transformation product, as chlorodifluoromethane was the sole transformation product observed. However, more chlorodifluoromethane was observed at the 24 hour time point in reactions that occurred at pH 9 than in reactions that occurred at pH 5 or 7. The formation of chlorodifluoromethane as a transformation product is favorable as it is not toxic, carcinogenic, or flammable.58 It is unclear if defluorination occurred in reactions with super-nucleophilic vitamin B12 and CFC-12 as less than 15% of the carbon measured as CFC-12 at time zero was accounted for as transformation products at the 24 hour time point. It appears that transformation products formed that were not identifiable with the given methodology.

In reactions between CT and super-nucleophilic vitamin B12 at pH 9 only non-halogenated transformation products were observed. While in reactions between CT and super-nucleophilic vitamin B12 at pH 5 and 7 a mixture of halogenated end products and methane formed. Twenty-five percent or less of the carbon measured as CT at time zero was accounted for as transformation products at the 24 hour time point, suggesting that transformation products formed that were not identifiable with the given methodology.

There are significant limitations to the methodologies used in this study. For example, the flame ionization detector is not the ideal detector for these compounds, as some halogenated methanes are considered non-combustible. High starting concentrations were used in this study to overcome the non-combustible nature CFC-11, CFC-12, and of possible transformation products. Additionally, though a PLOTQ column is useful in separating the volatile methanes likely to form via dehalogenation of CFC-11, CFC-12, or CT, a PLOTQ column is not ideal if halogenated ethanes or ethenes form via radical-radical coupling reactions, as they are retained on the column. For example, the method used in this study is 17.5 minutes long and reaches a temperature of 240ºC, but a sample of tetrachloroethene when injected will not elute from the column during the runtime. Given the limitations of the methodologies used in this study, more research should be done to confirm the identity of the transformation products produced. Gas chromatography-mass spectrometry would be a useful method to verify the identity of the transformation products. This methodology was unavailable for this study due to limitations in laboratory resources.

## Conclusions

The hypothesis that super-nucleophilic vitamin B12 can dehalogenate chlorofluoromethane, chlorodifluoromethane, difluoromethane, and fluoromethane was not supported. No chlorofluoromethane, chlorodifluoromethane, difluoromethane, and fluoromethane was dehalogenated in reactions containing super-nucleophilic vitamin B12. The hypothesis that changes in pH will effect which transformation products form during the dehalogenation of CFC-11, CFC-12, or CT by super-nucleophilic vitamin B12 was supported. Changes in the formation of transformation products were observed in reactions between CFC-11, CFC-12, or CT and super-nucleophilic vitamin B12 at pH 5, 7, 9. Unfortunately, the majority of the carbon measured as starting chemicals was not accounted for as transformation products at the 24 hour time point. This suggests that transformation products formed that were not detectable with this methodology. The development of tools that utilize super-nucleophilic vitamin B12 could be useful in the degradation of these chemicals. Though, more research should confirm the identity of the transformation products formed, as it would be unwise to develop a tool for bioremediation that forms transformation products that are more toxic, carcinogenic, flammable, or recalcitrant.

## Appendix

Table 33. Detailed instrument settings used for the analysis of headspace samples with GC – FID set up with the DB-624 column.

| Chlorinated compounds | Front Inlet | Inlet/Detector Gases | Detector (FID) or (µECD) | Program |
| --- | --- | --- | --- | --- |
| CT, CF, DCM, CM, CH4, PCE, TCE, cis-DCE, VC, methane, and ethene | Heater = 200 ˚C Pressure = 23.193 psi Septum Purge flow = 1.5 ml/min Split = Split ratio of 50:1 w/Split flow of 150 ml/min | He = inlet N2 = detector | Heater = 200 C Air Flow = 400 ml/min H2 Fuel flow = 30 ml/min | Initial Rate ˚C/min = 0 Initial Value ˚C = 60 Initial Hold Time min = 2 Initial Run Time min = 2 Ramp 1 Rate ˚C/min = 25 Ramp 1 Value ˚C = 200 Ramp 1 Hold Time min = 1 Ramp 1 Run Time min = 8.6 |

Table 34. Detailed instrument settings used for the analysis of headspace samples with GC – FID PLOTQ column.

| Chlorinated compounds | Back Inlet | Inlet/Detector Gases | Detector (FID) | Program |
| --- | --- | --- | --- | --- |
| CFC- 11, CFC-12, chlorofluoromethane, chlorodifluoromethane, dichlorofluoromethane, fluoromethane, difluoromethane, CT, CF, DCM, CM, CH4, PCE, TCE, cis-DCE, VC, methane, and ethene | Heater = 250 ˚C Pressure = 4.0198 psi Septum Purge flow = 4 ml/min Split = Split ratio of 10:1 w/Split flow of 30 ml/min | He = inlet N2 = detector | Heater = 300 C Air Flow = 400 ml/min H2 Fuel flow = 30 ml/min | Initial Rate ˚C/min = 0 Initial Value ˚C = 40 Initial Hold Time min = 5 Initial Run Time min = 5 Ramp 1 Rate ˚C/min = 80 Ramp 1 Value ˚C = 80 Ramp 1 Hold Time min = 2 Ramp 1 Run Time min = 7.5 Ramp 2 Rate ˚C/min = 80 Ramp 2 Value ˚C = 120 Ramp 2 Hold Time min = 2 Ramp 2 Run Time min = 10 Ramp 3 Rate ˚C/min = 80 Ramp 3 Value ˚C = 160 Ramp 3 Hold Time min = 2 Ramp 3 Run Time min = 12.5 Ramp 4 Rate ˚C/min = 80 Ramp 4 Value ˚C = 200 Ramp 4 Hold Time min = 2 Ramp 4 Run Time min = 15 Ramp 5 Rate ˚C/min = 80 Ramp 5 Value ˚C = 240 Ramp 5 Hold Time min = 2 Ramp 5 Run Time min = 17.5 |

Figure 58. Standards run on GC-FID with a PLOTQ column.

Figure 59. Standards run on GC-FID with a DB-624 column.

Table 35. Physical and Chemical Properties

| Compound | Molecular Weight (g/mol) | Density (g/mL) | Aqueous Solubility (mg/L) | KºH |
| --- | --- | --- | --- | --- |
| Tetrachloroethene | 165.833 | 1.62 | 206 | 0.057 |  |
| Trichloroethene | 131.388 | 1.4642 | 1,280 | 0.11 |  |
| cis-Dichloroethene | 96.943 | 1.2837 | 3500 | 0.27 |  |
| Vinyl Chloride | 62.498 | 0.9106 | 2700 | 0.038 |  |
| Ethene | 28.0532 | – g | 131 | 0.0047 |  |
| Carbon Tetrachloride | 153.823 | 1.59 | 793 | 0.033 |  |
| Chloroform | 119.378 | 1.4788 | 7950 | 0.28 |  |
| Dichloromethane | 84.933 | 1.325 | 13,200 | 0.47 |  |
| Chloromethane | 50.488 | 0.911 | 5040 | 0.12 |  |
| Methane | 16.0425 | – g | 22 | 0.0013 |  |
| CFC-11 | 37.368 | 1.49 | 1,100 | 0.0012 |  |
| CFC-12 | 120.914 | 1.35 | 280 | 0.0025 |  |
| Chlorodifluoromethane | 86.468 | 1.194 | 2770 | 0.034 |  |
| Chlorofluoromethane | 68.478 | – g | 1190 | 0.15 |  |
| Dichlorofluoromethane | 102.923 | 1.48 | 18800 | 0.19 |  |
| Difluoromethane | 52.0234 | – g | 12850 | 0.086 |  |
| Fluoromethane | 34.0329 | – g | 1787 | 0.059 |  |

NIST – for Molecular weight and KºH

PubChem – for Density and aqueous solubility

# CHAPTER V

# Characterization of Sulfidogenic Enrichment Culture Grown in the Presence of Carbon Tetrachloride.

## Abstract

Carbon tetrachloride (CT) is a recalcitrant groundwater contaminant that is susceptible to degradation by reactive iron species (i.e., magnetite and ferrous sulfide). There is interest in the potential of biologically mediated abiotic degradation to tackle the CT problem. The basis for BMAD is the observation that some microorganisms form extracellular reactive iron species, which can then degrade CT. Unfortunately, the trichloromethyl radical that forms upon a 1-electron transfer to CT is inhibitory to many microbes, including dissimilatory iron-reducing bacteria, which are a group of organisms known for their ability to reduce ferric iron to ferrous iron. This study explored the ability of a precipitate-forming sulfidogenic enrichment culture to degrade CT, examined the enrichment cultures’ microbial community composition with 16S rRNA gene amplicon sequencing, and characterized the precipitates formed in the presence of FeCl2, NaSO4, and Na2S, FeCl2 and Na2S, FeCl2, and NaSO4, and FeCl2 with X-ray powder diffraction. A decline in the amount of CT per bottle was greater in vessels with active enrichment culture than in corresponding heat-killed control incubations. An analysis of the 16S rRNA gene amplicon the sulfidogenic enrichment culture identified 11 bacterial classes, with the majority of  sequences representing Campylobacteria, Clostridia, Spirochaetia, and Bacteroidia. Vivianite was the only mineral phase identified in the X-ray diffraction patterns of precipitates from inoculated enrichments and abiotic controls containing FeCl2, FeCl2, and Na2S, FeCl2, or NaSO4, and the abiotic control containing FeCl2, Na2S, and NaSO4. In the enrichments amended with FeCl2, Na2S, NaSO4 and inoculum vivianite and mackinawite were observed in the X-ray diffraction pattern.

## Introduction

Carbon tetrachloride (CT) is a toxic chlorinated solvent, a predicted carcinogen, and an ozone-depleting agent.37 It was widely used during the 20th century as a dry cleaning agent, industrial degreaser, grain fumigant, fire extinguisher, and in the synthesis of chlorofluorocarbons. Research into the health and environmental impacts of CT and other chlorinated solvents began to raise concerns in the 1960s.10 Awareness of the health and environmental impacts, combined with increased publicization of the prevalence of industrial dumpsites, led to public outcry for governmental regulatory action, which facilitated the creation of the Environmental Protection Agency in the United States. In the latter half of the 20th century, production and use of CT and other hazardous chemicals were phased out globally under the Montreal protocal.210 Currently, minimal amounts of CT are produced for use as an extraction solvent and as an intermediate for the production of other chemicals.11 Unfortunately, CT is still found at many hazardous waste sites.12

Abiotic reductive dechlorination of CT by magnetite has been reported in the literature.76,130,150 The transformation of CT to nontoxic substances by magnetite may be of environmental significance. McCormick and Adriaens (2004) identified carbon monoxide, formate, and methane as transformation products of CT via the carbine-reduction pathway.161 Magnetite’s chemical structure is Fe3O4. It is a mixed-valence iron oxide that has a cubic closed-packed structure.211 The production of magnetite by dissimilatory iron-reducing bacteria (DIRB) is well established.212–215 However, DIRB cannot produce Fe3O4 in the presence of CT, due to the inhibitory effects of the trichloromethyl radical that forms from the reduction of CT by one electron.76 Currently, only one study in the literature describes biogenic magnetite formation through an oxidative pathway.216 The work by Chaunduri et al, describes magnetite formation by an organism that couples the oxidation of ferrous iron (Fe(II)) to the reduction of nitrate in an anoxic system. The biogenic production of magnetite or another reactive iron species could facilitate biologically mediated abiotic degradation of CT. This work characterized a sulfidogenic enrichment culture that forms iron containing precipitates in the presence of CT and Fe(II). This work explores the hypothesis that magnetite formation in this system occurs via a biogenic oxidative pathway in the presence of CT.

The enrichment cultures used in this work were developed by transferring microcosms established with sediment from a CT-contaminated railyard. Interest in the sediment from the CT-contaminated railyard developed because chloroform (CF) formation was observed after a wetland was installed. The aims of this study were i) to track chlorinated volatile organic compound (VOC) concentrations over time in the enrichment culture, ii) identify the phylogenetic diversity of the microbial consortium present in this culture, and iii) identify the precipitate formed by the anaerobic enrichment culture.

## Methods

Chemicals CT (99%) and sodium sulfide nonahydrate were purchased from ACROS organics. Chloroform (CF) (99%), dichloromethane (DCM) (99.8%), chloromethane (CM) (99.5%), methane (99%), and sodium DL-lactate 60% w/w syrup were purchased from Sigma-Aldrich. Ferrous chloride (FeCl2) (99.5%) and sodium sulfate anhydrous (NaSO4) (99.0%) were purchased from Fisher Scientific. MilliQ water (18.2 MΩ.cm) was used to prepare all solutions and medium. All other chemicals used were reagent grade or better. The Hungate technique was used to make anoxic solutions.217

Transformation of carbon tetrachloride in enrichment cultures Enrichment cultures and microcosms were established in an anaerobic glove box (Coy, Ann Arbor, MI ) containing N2/H2 (97/3 vol/vol). The original microcosms contained ~10 g sediment (wet weight) from a CT-contaminated rail yard, 90 mL anoxic bicarbonate-buffered mineral salts medium, and 60 mL of headspace.218 CT (10 µmol/bottle) was added to each microcosm using a Hamilton syringe equipped with a Chaney adapter. The microcosms were maintained over six transfers in anoxic bicarbonate-buffered mineral salts medium pH (7.2) amended with lactate (5 mM), FeCl2 (5 mM), and NaSO4 (5 mM), and CT (10 µmol/bottle).

This work reports observations of an enrichment culture which was developed by transferring  microcosms established with sediment from a CT-contaminated railyard. The microcosms were established in 160 mL bottles containing inoculum (3%), 100 mL of anoxic bicarbonate-buffered mineral salts medium, 60 mL of headspace containing a mixture of N2/CO2 (80/20, vol/ vol), lactate (5 mM), and CT (10 µmol/bottle). Details describing additional amendments of FeCl2 (5 mM), NaSO4 (5 mM), and NaS2 (0.2 mM) are available in Table 37. The Hungate method was used to make anoxic stock solutions of FeCl2 (1 M), NaSO4 (1 M), and NaS2 (100 mM). All amendments and the inoculum were transferred into the 160 mL bottles with 3 mL N2 flushed syringes.

Analytical techniques CT, CF, DCM, chloromethane (CM), and methane concentrations were monitored by headspace injection (100 µl) into an Agilent 7890 gas chromatograph (GC) with a DB-624 capillary column (60 m × 0.32 mm × 1.8 μm) and a flame ionization detector (FID). Further details of the GC-FID method are found in Table 38. Quantification of CT, CF, DCM, CM, and methane was obtained by measuring standards with known amounts of each compound. Standards were prepared in 160 mL bottles with 100 mL of liquid and 60 mL of headspace. Dimensionless Henry’s constants used to calculate the concentration of each chemical were as follows: CT (1.244), CF (0.150), DCM (0.0895), and CM (0.361).219

### Statistical analysis of chlorinated VOC data.

The effect of treatment on CT was examined using a one-way analysis of variance (ANOVA). Diagnostic analysis of the data was conducted to verify the model assumptions for normality and equal variance. Post hoc multiple comparisons were performed with Tukey’s adjustment. Statistical significance was identified at the level of 0.05. Analyses were conducted in SAS 9.4 TS1M7 for Windows 64x (SAS Institute Inc., Cary, NC).

Microbial community analysis via 16S rRNA gene amplicon sequence analysis. A cell pellet was collected by vacuum filtration of 20 mL of liquid from a microcosm which contained FeCl2, NaSO4, and Na2S. The cell pellet was collected on a sterile 0.22 μm membrane filter. DNA was extracted from the membrane filter using a PowerSoil kit (Qiagen, Hilden Germany). Extracted DNA was purified using the Genomic DNA Clean and Concentrator Kit (Zymo Research, Irvine CA). The V4 region of the 16S rRNA gene was amplified via polymerase chain reaction (PCR) using barcoded-primers F515/R806220 Target amplicon length was 291 base pairs. The total volume of the PCR assays was 50 μL and contained 10 μL of sample DNA, 1 μL of barcoded primer (10 μM), and 39 μL of a master mix containing 22 μL de-ionized water (5 PRIME, Gaithersburg, MD), 1 μL CAP 515F primer (10 μM), 5 μL Invitrogen Pfx50™ buffer (Invitrogen, Carlsbad CA), 1 μL Invitrogen Pfx50™ Polymerase, 5 μL dNTP, and 5 μL of MgCl2 (25 mM). The thermocycling program consisted of an initial denaturing at 94°C for 3 min, followed by 35 cycles of 94°C for 45 seconds, 55°C for 60 seconds, 72°C for 90 seconds, and a final extension of 10 minutes at 72°C. After PCR, the DNA concentration was determined on a NanoDrop 2000 and then pooled with DNA of similar concentration. Further purification of the pooled samples was performed with Solid Phase Reversible Immobilization (SPRI) magnetic beads (Beckman Coulter, Inc., Indianapolis IN). Purified pooled DNA was quantified and checked for quality and presence of primer dimers on a Bioanalyzer 2100 (Agilent, Santa Clara CA) with a high-sensitivity DNA kit. The final amplicon library was diluted to 10 nM and prepared for paired-end sequencing and loaded onto the Illumina MiSeq (Illumina, Inc., San Diego CA) following the manufacturer’s protocol. The default parameters of the MiSeq reporter were used to base call, demultiplex, and trim adopters.

Analysis of the raw 16S rRNA gene amplicon sequences was performed using the CLC Genomics Workbench (v11.0) microbial genomic module (v3.6.1) from Qiagen. Overlapping pairs were merged if they had a minimum alignment score of 8, and a gap penalty of 3. A mismatch penalty of 2 was applied. Merged reads were trimmed using a quality limit of 0.001 and an ambiguous nucleotide limit of 2. Adapter sequences were removed. Sequences shorter than 240 base pairs in length were discarded. Resulting sequences were clustered using a reference-based method, and taxonomic assignments were determined by comparison to the SILVA 16S v132 database. Sequences that did not have 97% or greater similarity to a sequence in the SILVA 16S v132 database were clustered based on the similarity of sequences within a given cluster to other sequences within the cluster at 97% similarity threshold. Clustering sequences based on the similarity of sequences within a sample is referred to as De Novo Clustering. 221 To generate a phylogenetic tree the sequences were then aligned using ClustalOmega,222 and a maximum likelihood tree was created using Phyml223 with a bootstrap value of 100. Then, iTOL224 was used to visualize and annotate the phylogenetic tree.

Precipitate characterization Microcosms for precipitate characterization were established in 2 L glass bottles with 1 L of anoxic bicarbonate-buffered mineral salts medium supplemented with vitamins, lactate (5 mM), and CT (103 µmol/bottle). The amendment scheme is shown in Table 39. Hamilton syringes equipped with reproducibility (Chaney) adapters were used to add CT to microcosms (Hamilton, Reno, NV). Microcosms were incubated for 120 days in the dark. After incubation, the precipitate phase was collected via gravity filtration using a 1 L Stericup-GP sterile vacuum filtration system (Millipore Sigma) in an anaerobic chamber (Coy, Ann Arbor, MI) containing N2/H2 (97/3 vol/vol). After 24 hours any liquid that had not passed through the filter was poured out of the filtration system as the precipitate phase had settled to the bottom. The filters were then transferred to Mason jars. Each mason jar was wrapped in parafilm prior to being transferred out of the anaerobic glove box and moved to the XRD facility at the Joint Institute for Advanced Materials (JIAM).

At JIAM, each precipitate was transferred to a PANalytical sample holder and scanned in a PANalytical Empyrean powder XRD instrument with a co-tube and a PIXcel3D-Medipix3 1 × 1 detector. Scanning 1-D line mode was used. The XRD scan was continuous, with a starting position of 10.0129 Θ, an ending position of 109.9869 Θ, and a step size of 0.0263. Further details of the XRD collection scheme are found in Table 40. Highscore PDF4+ was used for qualitative phase identification, as displayed in Figure 63–Figure 66. The background signal of diffraction patterns was calculated and then subtracted from diffraction patterns using a bending factor of 50 and a granularity of 9 in Highscore PDF4+. Reference files for vivianite (04-015-9091) and mackinawite (04-007-8222) were obtained from the International Center for Diffraction Data (ICDD). Prior to plotting samples were normalized by dividing all samples by the highest peak and multiplying by 100.

## Results

### Chlorinated VOC

To determine if the presence of the inoculum led to a greater decline in CT concentration under four treatments Table 37, microcosms and heat killed controls were grown in the presence of CT. Head space measurements of cVOCs were taken to track the concentrations of CT and the predicted transformation products CF, DCM, CM, and methane. The results showed that the presence of the inoculum facilitated a greater decline in the concentration of CT under all four treatments, as seen in Figure 60 and Figure 61, which depict chlorinated VOC concentrations over time for microcosms containing CT. The graphs show averaged data of the three replicates, the error bars show the standard error of the three replicates at each time point. Standard error was calculated by calculating the standard deviation of the three replicates and dividing by the square route of the sample size, which was 3 for all conditions. Figure 61 shows the average decline in µmol of CT per bottle for each treatment. The p-values incidate that there are statistically significant differences between the inoculated treatment and the corresponding heat-killed control for all treatments.

Figure 60. Chlorinated volatile organic compound concentrations over time in carbon-tetrachloride-containing microcosms.

Figure 61. One-way analysis of variance of the effect of treatment on carbon tetrachloride. Each purple bar indicates the average decline in µmol of CT per treatment. The error bars show the standard error of each treatment. The p-value for each comparison is shown above the brackets which indicate the two treatments being compared.

Microbial community analysis. In order to identify members of the Elkhart enrichment culture

16S rRNA gene amplicons were sequenced. Processing the 16S rRNA gene amplicons from the FeCl2, Na2SO4, and Na2S microcosms resulted in 41,704 sequence reads after processing, which were grouped into 64 predicted operational taxonomic units (OTUs). Of these OTUs, 47 were based on the SILVA 16S v132 database, and 17 were De Novo OTUs. The average read length was 253 base pairs. Figure 62 is a phylogenetic tree built using maximum likelihood method with a bootstrap value of 100. All of the 64 predicted OTUs are included on the phylogenetic tree. The GenBank accession numbers are included from OTUs based on the SILVA 16S v132 database.

This analysis identified 11 bacterial classes, as shown in Figure 62. In Figure 62, each representative of the 11 bacterial classes is highlighted with a different color: the corresponding colors and classes are identified in the legend on the upper-right-hand corner. In general, OTUs from the bacterial classes grouped together, with the exception of the Clostridia, which was intersected by the Bacilli and Alphaproteobacteria classes. The largest class of organisms identified in this analysis was the Clostridia, which has 29 representatives from 15 genera. The next largest class was the Campylobacteria, which has 12 representatives, all of which are from the Sulfurospirillum genus. The most identified organism was a Sulfurospirillum, which was identified 22,720 times. There were seven representatives from the Bacteroidia class, with representatives from three genera. There were five OTUs identified as Spirochaetia, all of which were unidentified at the genus level. Three Gammaproteobacteria representatives were identified, all from different genera. Two Alphaproteobacteria were identified, one of which was from the Pleomorphomonas genera. A single OTU was identified from each of the Actinobacteria, Dehalococcoidia, Anaerolineae, and Oxyphotobacteria classes.

Figure 62. Maximum likelihood tree built from predicted Elkhart operational taxonomic unit.

In Figure 62, each class is highlighted by a different color, as shown in the legend. The lowercase letter before each the operational taxonomic unit name identified the level of phylogenetic identification that was determined for each operational taxonomic unit. Additionally, the red bars on the right-hand side of the tree report how many times each operational taxonomic unit was identified.

Precipitate characterization In order to determine the identities of the precipitates formed under four treatments Table 39, X-ray diffraction patterns were collected and analyzed. The results showed that vivianite (Fe32+(PO4)2 · 8H2O) was the only mineral phase identified in the diffraction patterns of the precipitates from vessels that contained FeCl2, FeCl2, and Na2S, and FeCl2 and NaSO4 Figure 63–Figure 66, and Table 36. The presence or absence of CT had no effect on the formation of the precipitate phase in any of the experimental vessels.

Additionally, vivianite and mackinawite (FeS) were identified in the diffraction pattern of the precipitate from the vessels that contained FeCl2, Na2S, NaSO4, and cells Figure 63. Only vivianite was identified in the diffraction pattern of the precipitate from the vessels that contained FeCl2, Na2S, NaSO4 without cells. A reference pattern (04-007-8222) from the ICDD was used to identify FeS. Key peak alignments that suggest the presence of FeS in the diffraction pattern from the precipitate formed in the vessel that contained FeCl2, Na2S, NaSO4, and cells are the forked peak at 20 of 2-theta, the peak at 40 of 2-theta, and the forked peaks at 58 2-theta.

Table 36. Description of precipitate formation in microcosms.

| Biotic or Abiotic | FeCl2 | NaSO4 | Na2S | CT | Mineral Phase |
| --- | --- | --- | --- | --- | --- |
| Biotic | 5 mM | 5 mM | 0.2 mM | 10 µmol/bottle | Vivianite, Mackinawite |
| Biotic | 5 mM | 5 mM | 0.2 mM | 0 µmol/bottle | Vivianite, Mackinawite |
| Abiotic | 5 mM | 5 mM | 0.2 mM | 10 µmol/bottle | Vivianite |
| Biotic | 5 mM |  | 0.2 mM | 10 µmol/bottle | Vivianite |
| Biotic | 5 mM |  | 0.2 mM | 0 µmol/bottle | Vivianite |
| Abiotic | 5 mM |  | 0.2 mM | 10 µmol/bottle | Vivianite |
| Biotic | 5 mM | 5 mM |  | 10 µmol/bottle | Vivianite |
| Biotic | 5 mM | 5 mM |  | 0 µmol/bottle | Vivianite |
| Abiotic | 5 mM | 5 mM |  | 10 µmol/bottle | Vivianite |
| Biotic | 5 mM |  |  | 10 µmol/bottle | Vivianite |
| Biotic | 5 mM |  |  | 0 µmol/bottle | Vivianite |
| Abiotic | 5 mM |  |  | 10 µmol/bottle | Vivianite |

Figure 63. Diffraction patterns of the FeCl2, Na2S, and NaSO4 amended microcosms.

Figure 64. Diffraction patterns of the FeCl2 and Na2S amended microcosms.

Figure 65. Diffraction patterns of the FeCl2 and Na2SO4 amended microcosms.

Figure 66. Diffraction patterns of the FeCl2 amended microcosms.

## Discussion

Biologically mediated abiotic degradation of CT via the formation or regeneration of reactive iron species in contaminated aquifers has the potential to facilitate degradation of this recalcitrant contaminant. A deeper understanding of the microbial processes that can generate reactive iron species in the presence of CT is needed. This study tracked the concentration of CT and its predicted transformation products in microcosms, used 16S rRNA amplicon sequencing to identify members of the microbial consortium, and identified the precipitate phases formed under different conditions. This study found that there was greater decline in CT concentration in microcosms that contained inoculum than in heat killed controls, identified FeS in microcosms that contained FeCl2, NaSO4, Na2S and cells, and observed 16S rRNA gene sequences of sulfate reducing bacteria, which suggest that the formation of FeS could be a biproduct of sulfate reduction.

There were greater declines in the amount of CT per bottle in bottles that were inoculated with the enrichment culture than in the bottles that were inoculated with the enrichment culture and then heat-killed for all treatments. Despite the statistically significant difference between inoculated and heat-killed control vessels, stoichiometric amounts of transformation products were not observed. It is possible that transformation products that were not detectable by the GC-FID method were formed. For example, McCormick and Adriaens observed the formation of carbon monoxide and formate in their 2004 study, and some studies report the formation of carbon dioxide and phosgene.68,76 The analytical methods used in this study were not established to capture the formation of phosgene or carbon dioxide or carbon monoxide.

The identification of vivianite as the only mineral phase is robust in vessels that contained FeCl2, FeCl2, and Na2S, and FeCl2 and NaSO4 Figure 63–Figure 66, as all peaks in these samples match the peaks in the vivianite diffraction pattern obtained through the ICDD (01-080-9696), and there are no unaccounted for peaks in these diffraction patterns. It seems that the formation of vivianite is an abiotic process, as vivianite was identified in samples both with and without cells. The ratio of phosphate to ferrous iron in vivianite is 2:3, as seen in vivianite’s chemical formula Fe3(PO4)2(H2O). Further, the medium used in these experiments contained 1.47 mM of monobasic potassium phosphate (KH2PO4). Therefore, if all the KH2PO4 precipitated with ferrous iron, 2.2 mM of ferrous iron could be consumed. As 5.0 mM FeCl2 was used in each experiment, this reaction could precipitate out half of the iron in each experimental vessel.

Identification of FeS in the vessels that contained FeCl2, Na2S, NaSO4, and cells is not as robust as the identification of vivianite, as the other peaks associated with FeS are small and obscured by the vivianite diffraction pattern. Precipitate formation was the same in the inoculated vessels that contained FeCl2, Na2S, and NaSO4 whether or not CT was present. A possible explanation for the formation of FeS in vessels that contained FeCl2, Na2S, NaSO4, and cells is that the activity of a sulfate-reducing organism (i.e., Desulfosporosinus meridiei DSM 13257, which was identified in the 16S rRNA gene amplicon analysis) increased the amount of sulfide present in the system. An increase in sulfide concentration could lead to the formation of an identifiable amount of FeS, as sulfide precipitation with ferrous iron could form FeS.

Analysis of microbial community structure by 16S rRNA gene amplicon sequencing has limitations that are inherent in the methodology. Examples of these limitations are primer bias, sequencing error, and variation in starting DNA concentration.225 Primer bias can cause some sequences to be amplified more than others, leading to an overestimate of their prevalence in the population. Furthermore, all general primers will entirely miss some members of diverse microbial populations. During PCR amplification cycles, incorrect base pairing occurs, which creates OTUs that do not represent real organisms. This leads to an overestimation of OTUs in the sample. Finally, variation in starting concentration of DNA can select for increased amplification of more abundant DNA. Knowledge of these errors can be addressed to some extent in experimental design and data processing. For example, errors can be minimized by using high-quality primer sets, being precise with starting DNA concentration when preparing the amplicon library, and removing singly represented OTUs during data processing. Despite these limitations, benchmark studies like the one performed by D’Amore and collogues (2016) suggest 16S rRNA gene amplicon sequencing still has some quantitative value.225 This report mentions the relative abundance of community members at the phylum level identified during 16S rRNA gene amplicon sequencing, with the understanding that there is error in the reported values.

The most frequently detected OTU represents an uncultured Sulfurspirillum sp. identified by a sequencing effort to characterize the microbial community of the Danshui river estuary of Northern Taiwan. The metabolic strategy of this uncultured Sulfurspirillum is unknown.226 Members of the Sulfurspirillum genus are known to use toxic compounds such as arsenate, selenite, and organohalides as electron acceptors coupled to the oxidation of pyruvate, hydrogen, and formate as electron donors.144 Members of this genus can also use nitrate and oxidized sulfur compounds as electron donors and perform pyruvate fermentation, which produces H2.227 Sulfurspirillum is commonly found in contaminated sites, and research suggests that menaquinones and cytochromes are involved in their anaerobic respiratory chains.144 Menaquinones and cytochromes have been shown to facilitate CT transformation when produced by other organisms.101,109,137,228 It is possible that the menaquinones or corrinoid produced by Sulfurspirillum spp. could facilitate co-metabolic CT degradation. All of the representatives from the Campylobacteria class are Sulfurospirillum Figure 62.

The second most frequently identified OTU was a member Spirochaetaceae family with the GenBank accession number JN685463. The uncultured Spirochaetaceae was identified by a sequencing effort to characterize the microbial community in a water-flooded oil reservoir.229 Members of the Spirochaetia class can be free-living or host-associated, and they are generally motile, spiral-shaped organisms that can be obligate or facultative anaerobes.144 A saccharolytic metabolism is often employed by members of the Spirochaetia class. The representative identified in the enrichment culture is of the Spirochaetaceae family, which contains the genera Borrelia, Brevinema, Cristispira, Spirochaeta, Spironema, and Treponema.

The third, fourth, and fifth most frequently identified OTUs were members of the Clostridia class. All members of the clostridia phylum are obligate anaerobes. The third most frequently identified OTU was Desulfosporosinus meridiei DSM 13257, which can be identified by the GenBank accession number CP003629. This bacterium was isolated from groundwater contaminated with motor fuel. Desulfosporosinus meridiei DSM 13257 is a grampositive organism that utilizes sulfate reduction as a metabolic strategy.230,231 Other members of the Desulfosporosinus genus have been isolated from acid mine drainage and are slightly acidophilic.230 The fourth most frequently identified OTU is an ambiguous member of the Caproiciproducens genus, identified by the GenBank accession number CCFG01000002. Not much information is associated with this GenBank accession number. A cultured member of the Caproiciproducens genus, Caproiciproducens galactitolivorans, was isolated from a wastewater treatment plant and is produces caproic acid from galactitol.232 The fifth most frequently detected OTU was an ambiguous member of the Ruminococcaceae family, which was identified in samples from Chinese mines.233 Members of the Ruminococcaceae family are obligate anaerobes, and the type strain of the Ruminiclostridium genera is Ruminiclostridium cellobioparum, which was isolated from bovine rumen and employs a fermentative metabolism.217

Other less frequently detected members of the Clostridia class belonged to the genera: Desulfitobacterium, Hydrogenoanaerobacterium, Intestimona, Oscillibacter, Family XIII, and

Lutispora. Many organisms from these genera are known to utilize fermentative metabolisms.234–240 Of note are the members of the Desulfitobacterium genera, which are known to be metabolically diverse. Representatives of the Desulfitobacterium genera are known to utilize nitrate, sulfite, metals, humic acids, and man-made or naturally occurring halogenated organic compounds as electron acceptors.241

Fermentative metabolisms are also commonly observed in members of less frequently detected classes (i.e., Bacilli, Actinobacteria, Anaerolineae). There are two representatives from the Bacilli, both are from the Bacillus genus. Members of the Bacillus genus are known to be rod-shaped, gram-positive, spore-forming, aerobic, or facultatively anaerobic bacteria that use fermentative metabolic strategies.242 The representative of the Actinobacteria is unidentified at the species level. The type strain of the genus is Propionicicella superfundia, which was isolated from groundwater contaminated with chlorinated ethanes and vinyl chloride. Propionicicella superfundia is a gram-positive, rod-shaped, non-motile, non-spore-forming bacterium that is a facultative anaerobe and utilized a fermentative metabolic strategy.243 The Anaerolineae representative sequence was originally identified as part of a methanogenic community developed using subseafloor sediments and cultivated in a bioreactor prior to sequencing efforts.244 The type strain of the Pelolinea genera is Pelolinea submarina, which is described as a filamentous anaerobic gram-negative bacterium that is non-motile and non-spore former that can grow on several carbohydrates in the presence of yeast extract.245

Two OTUs identified at low abundances that likely utilize distinct metabolic strategies were the uncultured Dehalogenimonas, first discovered [this is not true. check Jun Yang’s first paper in EMI in 2009]in a microcosm developed from grape pomace compost, which is capable of dechlorinated trichloroethene and vinyl chloride to the non-chlorinated end product ethene,246 and a member of the Oxyphotobacteria class identified as a Cyanobium gracile PCC 6307, which is described by Uniprot247 and Jgi248 as a non-motile unicellular gram-negative bacteria isolated from a freshwater system. Cyanobacteria are typically oxygenic phototrophic organisms.

## Conclusions

This study sought to gain a deeper understanding of the processes that contribute to biologically mediated abiotic degradation of CT. The hypothesis that magnetite formation in this system occurs via a biogenic oxidative pathway in the presence of CT was not supported. Magnetite was not identified in these experiments, and the precipitate that did form did not contain oxidized (ferric) iron. FeS formation was observed in the vessels that contained FeCl2, Na2S, NaSO4 and were inoculated with the enrichment culture. FeS was not identified under any other treatment condition. The formation of FeS was likely facilitated by a sulfate-reducing microbe. Additionally, the decline in CT concentration was greater in the inoculated microcosms than in their corresponding heat-killed controls. The greater decline in CT concentration in inoculated bottles could be due to the activity of Desulfosporosinus and Sulfurspirillum, as their sulfate reducing metabolisms are likely responsible for the formation of FeS.249,250 Furthermore, the production of menaquinones and cytochromes by Sulfurspirillum may facilitate co-metabolic CT transformation.101,109,137,228

## Appendix

Table 37. Amendment schemes for Elkhart microcosms.

| ID | Inoculum | FeCl2 | NaSO4 | Na2S | Lactate | CT |
| --- | --- | --- | --- | --- | --- | --- |
| 1 a, b, and c | 3% | 5 mM | 5 mM | 0.2 mM | 5 mM | 10 µmol/bottle |
| 2 a, b, and c | 3% HK | 5 mM | 5 mM | 0.2 mM | 5 mM | 10 µmol/bottle |
| 3 a, b, and c | 3% |  | 5 mM |  | 5 mM | 10 µmol/bottle |
| 4 a, b, and c | 3% HK |  | 5 mM |  | 5 mM | 10 µmol/bottle |
| 5 a, b, and c | 3% | 5 mM |  | 0.2 mM | 5 mM | 10 µmol/bottle |
| 6 a, b, and c | 3% HK | 5 mM |  | 0.2 mM | 5 mM | 10 µmol/bottle |
| 7 a, b, and c | 3% | 5 mM |  |  | 5 mM | 10 µmol/bottle |
| 8 a, b, and c | 3% HK | 5 mM |  |  | 5 mM | 10 µmol/bottle |

HK = heat kill

Table 38. Detailed instrument settings used for the analysis of headspace samples with gas chromatography.

| Chlorinated compounds | Front Inlet | Inlet/Detector Gases | Detector (FID) | Program |
| --- | --- | --- | --- | --- |
| PCE, 1,1-DCE, CT, CF, DCM, CM, methane | Heater = 200 ˚C Pressure = 23.193 psi Septum purge flow = 1.5 mL /min Split = Split ratio of 50:1 w/Split flow of 150 mL /min | He = inlet N2 = detector | Heater = 200 C Air Flow = 400 mL /min H2 Fuel flow = 30 mL /min | Initial rate ˚C/min = 0 Initial value ˚C = 60 Initial hold time min = 2 Initial run time min = 2 Ramp 1 rate ˚C/min = 25 Ramp 1 value ˚C = 200 Ramp 1 hold time min = 1 Ramp 1 run time min = 8.6 |

Table 39. Amendment Schemes for X-ray diffraction microcosms.

| Biotic or Abiotic | FeCl2 | NaSO4 | Na2S | CT |
| --- | --- | --- | --- | --- |
| Biotic | 5 mM | 5 mM | 0.2 mM | 10 µmol/bottle |
| Biotic | 5 mM | 5 mM | 0.2 mM | 10 µmol/bottle |
| Abiotic | 5 mM | 5 mM | 0.2 mM | 10 µmol/bottle |
| Biotic | 5 mM |  | 0.2 mM | 10 µmol/bottle |
| Biotic | 5 mM |  | 0.2 mM | 10 µmol/bottle |
| Abiotic | 5 mM |  | 0.2 mM | 10 µmol/bottle |
| Biotic | 5 mM | 5 mM |  | 10 µmol/bottle |
| Biotic | 5 mM | 5 mM |  | 10 µmol/bottle |
| Abiotic | 5 mM | 5 mM |  | 10 µmol/bottle |
| Biotic | 5 mM |  |  | 10 µmol/bottle |
| Biotic | 5 mM |  |  | 10 µmol/bottle |
| Abiotic | 5 mM |  |  | 10 µmol/bottle |

Table 40. X-ray diffraction collection parameters

| Radiation |  |  |  |
| --- | --- | --- | --- |
| Co Kα1 [Å] |  | 1.789 |  |
| Co kα2 [Å] |  | 1.793 |  |
| Tube voltage [kV] |  | 45 |  |
| Tube current [mA] |  | 40 |  |
| Collimation |  |  |  |
| Divergence slit [°] | 1/4 |  | 1/8 |
| Anti-scatter slit [°] | 1/2 |  | 1/4 |
| Soller slit [Rad] | 0.04 |  | 0.04 |
| Environment |  |  |  |
| Atmosphere |  | Inert |  |
| Temperature |  | Ambient |  |
| Pressure |  | Ambient |  |
| Collection |  |  |  |
| Scan range [2Θ] | 10.0 ≤ 2Θ ≤ 90.0 |  | 10.0 ≤ 2Θ ≤ 90.0 |
| Step size [2Θ] | 0.02 |  | 0.013 |
| Counting time [s] | 39.00 |  | 159.00 |
| Spin time [s/rotation] | 4 |  | 4 |

Table 41. Pairwise comparisons for the effect of treatment on carbon tetrachloride.

| treatment | treatment | Estimate | Standard Error | DF | t Value | Pr > ¦t¦ | Adj P |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FeCl2 | FeCl2, Na2S | 0.1127 | 0.7026 | 16 | 0.16 | 0.8746 | 1.0000 |
| FeCl2 | FeCl2, NaSO4, Na2S | -2.2252 | 0.7026 | 16 | -3.17 | 0.0060 | 0.0860 |
| FeCl2 | Heat-Killed FeCl2 | 3.5208 | 0.7026 | 16 | 5.01 | 0.0001 | 0.0025 |
| FeCl2 | Heat-Killed FeCl2, Na2S | 4.3940 | 0.7026 | 16 | 6.25 | <.0001 | 0.0002 |
| FeCl2 | Heat-Killed FeCl2, NaSO4, Na2S | 3.3800 | 0.7026 | 16 | 4.81 | 0.0002 | 0.0037 |
| FeCl2 | Heat-Killed NaSO4 | 5.6333 | 0.7026 | 16 | 8.02 | <.0001 | <.0001 |
| FeCl2 | NaSO4 | 2.0843 | 0.7026 | 16 | 2.97 | 0.0091 | 0.1226 |
| FeCl2, Na2S | FeCl2, NaSO4, Na2S | -2.3378 | 0.7026 | 16 | -3.33 | 0.0043 | 0.0642 |
| FeCl2, Na2S | Heat-Killed FeCl2 | 3.4082 | 0.7026 | 16 | 4.85 | 0.0002 | 0.0034 |
| FeCl2, Na2S | Heat-Killed FeCl2, Na2S | 4.2813 | 0.7026 | 16 | 6.09 | <.0001 | 0.0003 |
| FeCl2, Na2S | Heat-Killed FeCl2, NaSO4, Na2S | 3.2673 | 0.7026 | 16 | 4.65 | 0.0003 | 0.0050 |
| FeCl2, Na2S | Heat-Killed NaSO4 | 5.5207 | 0.7026 | 16 | 7.86 | <.0001 | <.0001 |
| FeCl2, Na2S | NaSO4 | 1.9717 | 0.7026 | 16 | 2.81 | 0.0127 | 0.1611 |
| FeCl2, NaSO4, Na2S | Heat-Killed FeCl2 | 5.7460 | 0.7026 | 16 | 8.18 | <.0001 | <.0001 |
| FeCl2, NaSO4, Na2S | Heat-Killed FeCl2, Na2S | 6.6192 | 0.7026 | 16 | 9.42 | <.0001 | <.0001 |
| FeCl2, NaSO4, Na2S | Heat-Killed FeCl2, NaSO4, Na2S S | 5.6052 | 0.7026 | 16 | 7.98 | <.0001 | <.0001 |
| FeCl2, NaSO4, Na2S | Heat-Killed NaSO4 | 7.8585 | 0.7026 | 16 | 11.18 | <.0001 | <.0001 |
| FeCl2, NaSO4, Na2S | NaSO4 | 4.3095 | 0.7026 | 16 | 6.13 | <.0001 | 0.0003 |
| Heat-Killed FeCl2 | Heat-Killed FeCl2, Na2S | 0.8732 | 0.7026 | 16 | 1.24 | 0.2319 | 0.9067 |
| Heat-Killed FeCl2 | Heat-Killed FeCl2, NaSO4, Na2S | -0.1408 | 0.7026 | 16 | -0.20 | 0.8437 | 1.0000 |
| Heat-Killed FeCl2 | Heat-Killed NaSO4 | 2.1125 | 0.7026 | 16 | 3.01 | 0.0084 | 0.1144 |
| Heat-Killed FeCl2 | NaSO4 | -1.4365 | 0.7026 | 16 | -2.04 | 0.0577 | 0.4852 |
| Heat-Killed FeCl2, Na2S | Heat-Killed FeCl2, NaSO4, Na2S | -1.0140 | 0.7026 | 16 | -1.44 | 0.1683 | 0.8248 |
| Heat-Killed FeCl2, Na2S | Heat-Killed NaSO4 | 1.2393 | 0.7026 | 16 | 1.76 | 0.0968 | 0.6501 |
| Heat-Killed FeCl2, Na2S | NaSO4 | -2.3097 | 0.7026 | 16 | -3.29 | 0.0046 | 0.0691 |
| Heat-Killed FeCl2, NaSO4, Na2S | Heat-Killed NaSO4 | 2.2533 | 0.7026 | 16 | 3.21 | 0.0055 | 0.0800 |
| Heat-Killed FeCl2, NaSO4, Na2S | NaSO4 | -1.2957 | 0.7026 | 16 | -1.84 | 0.0838 | 0.6026 |
| Heat-Killed NaSO4 | NaSO4 | -3.5490 | 0.7026 | 16 | -5.05 | 0.0001 | 0.0023 |

## Conclusions

CFC-11, CFC-12, and CT are greenhouse gasses, ozone depleting substances and groundwater pollutants found globally.4,6,251 Though CFC-11 and CFC-12 are not very toxic they do inhibit the biodegradation of other contaminants, and their transformation products can be more toxic, carcinogenic, and flammeble then they are, for these reasons it is important to understand the how these chemicals can be degraded in the subsurface.58 It is also important to understand the mechanisms of degradation for CT, as it is a toxic substance, suspected carcinogen, and inhibitory to dehalogenating bacteria.8,20,21,26

Data from two treatability studies contaminated with CFC-11, CFC-12, and CT and other chemicals was presented in this dissertation. At the first contaminated site a soil gas survey (SGS) and membrane interface hydrolic profiling tool (MiHPT) were used to characterize contaminante distribution at the site prior to taking sediment and groundwater samples (Chapter 2). Data from the SGS survey and MiHPT transect revealed that contaminant concentration was highest along the dagonal southern bourndry of the contaminated site. This finding was consistent with historical information regaurding land use at the site, and was used to inform sediment and groundwater sampling locations. Analysis of the volatile organic cabons in the groundwater indicate that the borehole A3 from 19.70–24.70 mbgs containe the most contaminants, as PCE, 1,1-DCE, CFC-11, CFC-12, and CT were present in this sample. A sucessful remediation strategy for CFC-11 or CFC-12 was not identified by the treatability study. However, the data from the treatability study did suggest that bioaugmentation combined with biostimulation and the addition of zero valent iron or ferrous sulfide would transform CT into non-halogenated transforamtion products. The treatability study data also suggested that bioaugmentaion would be an effective remediation strategy for PCE and 1,1-DCE. However, degradation of CFC-11, CFC-12, and CT is recommended prior to implememnting bioaugmentation as a remediation strategy, as these chemicals are likely to inhibit the activity of the bioaugmentation culture.

It appeared that the aquifer material from the second contaminated site was oxic, because repeated additions of sulfide were needed to reduce the microcosms set up with aquifer material from this site. The microcosm data from the second contaminated site suggest that a component in the ground water is inhibitory to the bioaugmentation cultures tested; degradation of PCE and CF was achieved in bioaugmenation microcosms that contained reduced anoxic mineral salts medium but not in microcosms that contained groundwater. It is possible that the inhibitory component in the groundwater is CFC-11, CFC-12 or CT, as these chemicals are known to be inhibitory to bacteria and were present at the site. Similarly to the finding at the first contaminated site, it is recomened that degradation of CFC-11, CFC-12, and CT occur prior to an effort to degrade PCE or CF via bioaugmentation. CT was transformed to CF in microcosms set up with aquifer material and additional zero valent iron.

In chapter four the effect of pH on the dehalogenation of CFC-11, CFC-12 and CT by super-nucleophilic B12 was explored. Experiments in chapter four indicate that pH does have an effect on the dehalogenation of CFC-11, CFC-12 and CT by super-nucleophilic B12. The formation and distribution of transportation products changes as pH increased. Additionally, it was observed that super-nucleophilic vitamin B12 is unable to dehalogenate chlorodifluoromethane, difluoromethane, fluoromethane, chlorofluoromethane.

In chapter five a precipitate forming sulfidogenic cenrichment culture was characterized. The data from this chapter indicate that the presence of active enrichment culture facilitated a greater decline in CT concentration than was observed in heat killed controls. Additionally, the two precipitate phases were identified in microcosms that contained FeCl2, NaSO4, Na2S and enrichment culture. The two mineral phases were, mackinawite (FeS) and vivianite (Fe32+(PO4)2 · 8H2O). Vivianite formation occurred via precipitation of added ferrous iron and the phosphate present in the medium, as this mineral phase was observed in all treatments including heat killed controls. Mackinawite formation was only observed in the traetments that contained FeCl2, NaSO4, Na2S and enrichment culture, and likey occurred via precipitation of ferrous iron and hydrogen sulfide formed as a byproduct of sulfate reduction. Sulfate reducing bacteria were identified in 16S rRNA gene sequences from the enrichment culture.

This work indicates that there is a need to implement effective remedition strategys for the CFC-11, CFC-12, and CT, as they inhibit the degradation of other contaminantes (i.e., chloronated ethenes). However, an understanding of the transforamtion products that will be produced is importat, as some transformation products are more hazourdous then the starting chemicals.58 Stimulation of microbial comunities like the sulfate reducing community characterized in chapter five may help facilitate transformation of CFC-11, CFC-12, and CT, but the literature indicates that more hazourdous transformation products are likely to form.73,74,126 Development of a remediation strategy that utilizes B12 may be effective, as carbon monoxide is reportedly the predominant transforamtion product. Some barriors that would have to be overcome in order to develop a remediation strategy that utilizes B12 would be preparing an environment that has the capacity to reduce B12 to the super-nucleophilic state, and the necessity of immobilizing B12 in the subsurface, so that it would not difuse, get taken up by microbes, or travel away from the prepared reducing environment.

# References

1. 	Plummer N, Busenberg E. Environmental Tracers in Subsurface Hydrology. 1st ed. (Cook P, Herczeg A, eds.). Ney York: Springer Science+Business Media; 2000. doi:10.1007/978-1-4615-4557-6

2. 	Siegemund G, Schwertfeger W, Feiring A, et al. Organic Fluorine Compounds. In: Ullmann’s Encyclopedia of Industrial Chemistry. Wiley‐VCH Verlag GmbH & Co; 2016:349-392. https://onlinelibrary.wiley.com/doi/abs/10.1002/14356007.a11_349.pub2.

3. 	AFEAS. Alternative Fluorocarbons Environmental Acceptability Study. www.afeas.org.

4. 	Watson R, Rodhe H, Oeschger H, Siegenthaler U. Greenhouse Gases and Aerosols, in Climate Change. Cambridge: Cambridge University Press; 1990.

5. 	Molina MJ, Rowland FS. Stratospheric sink of chlorofluoromrthanes: Chlorine atom-catalyzed destruction of ozone. Nature. 1974;249(6):810. https://www.nature.com/articles/249810a0.pdf.

6. 	Molina LT, Molina JM. Production of Cl202 from the Self-Reaction of the CIO Radical. J Phys Chem. 1987;(2):433-436. https://pubs-acs-org.proxy.lib.utk.edu/doi/abs/10.1021/j100286a035.

7. 	Smart BE, Fernandez RE. Fluorinated Aliphatic Compounds. Kirk‐Othmer Encycl Chem Technol. 2000. doi:10.1002/0471238961.0612211519130118.a01

8. 	WHO. Carbon Tetrachloride in Drinking-water. 2004.

9. 	Faith L, Lowenheim A, Marguerite M. Industrial Chemicals. 4th ed. New York: Wiley; 1975.

10. 	Doherty RE. A History of the Production and Use of Carbon Tetrachloride, Tetrachloroethylene, Trichloroethylene and 1,1,1-Trichloroethane in the United States: Part 1--Historical Background; Carbon Tetrachloride and Tetrachloroethylene. Environ Forensics. 2000;1(2):69-81. doi:10.1006/enfo.2000.0010

11. 	Sherry D, Mcculloch A, Liang Q, Reimann S, Newman PA. Environmental Research Letters Current sources of carbon tetrachloride (CCl 4 ) in our atmosphere Current sources of carbon tetrachloride (CCl 4 ) in our atmosphere. Environ Res Lett. 2018;13:24004. doi:10.1088/1748-9326/aa9c87

12. 	EPA. Superfund Site Information. Superfund Site Search Results.

13. 	National Toxicology Program. Bioassay of 1,1,2,2-tetrachloroethane for possible carcinogenicity. Natl Cancer Inst Carcinog Tech Rep Ser. 1978;27(106):1-86. http://www.ncbi.nlm.nih.gov/pubmed/12844181.

14. 	Epstein SS, Joshi S, Andrea J, Clapp P, Falk H, Mantel N. Synergistic toxicity and carcinogenicity of “freons” and piperonyl butoxide. Nature. 1967;214(5087):526-528. doi:10.1038/214526a0

15. 	Aviado D. Toxicity of Aerosol Propellants in the Respiratory and Circulatory Systems. IX. Summary of the Most Toxic: Trichlorofluoromethane (FC 11). Toxicology. 1975;3(3):311-219.

16. 	Himmel HM. Mechanisms involved in cardiac sensitization by volatile anesthetics: General applicability to halogenated hydrocarbons? Crit Rev Toxicol. 2008;38(9):773-803. doi:10.1080/10408440802237664

17. 	Garcia H. Spacecraft Maximum Allowable Concentratioins for Selected Airborne Concnetrations: Volumn 4. Houston, Texas: Johnson Space Center Toxicology Group, Medical Operations Branch; 2000. https://www.nap.edu/read/9786/chapter/18.

18. 	Maltoni C, Lefemine G, Tovoli D, Perino G. Long-Term Carcinogenicity Bioassays on Three Chlorofluorocarbons (Trichlorofluoromethane, FC11; Dichlorodifluoromethane, FCl2; Chlorodifluorome thane, FC22) Administered by Inhalation to Sprague-Dawley Rats and Swiss Mice. New York Acad Sci. (534):261-282.

19. 	Nuckolls H. Underwriter’s Laboratories’ Report on the Comparative Life, Fire, and Explosion Hazards of Common Refrigerants. Miscellaneous Hazard No. 237. Chicago, Il; 1933.

20. 	Zimmerman H. Syndromes of Environmental Hepatotoxicity. In: Hepatotoxicity — the Adverse Effect of Drugs and Other Chemicals on the Liver. New York: Appleton-Century-Crofts; 1978.

21. 	Berman E, House D, Allis J, Simmons J. Hepatotoxic interactions of ethanol with allyl alcohol or carbon tetrachloride in rats. J Toxicol Env Heal. 1992;37(1):161-176. doi:10.1080/15287399209531663

22. 	Gehring P. Hepatotoxic potency of various chlorinated hydrocarbon vapours relative to their narcotic and lethal potencies in mice. Toxicol Appl Pharmacol. 1968;13(3):287-298. doi:10.1016/0041-008x(68)90102-6

23. 	Klaassen C, Plaa G. Susceptibility of male and female mice to the nephrotoxic and hepatotoxic properties of chlorinated hydrocarbons. Proc Soc Exp Biol Med. 1967;124(4):1163-1166.

24. 	Prendergast J, Jones R, Jenkins L, Siegel J. Effects on experimental animals of long-term inhalation of trichloroethylene, carbon tetrachloride, 1,1,1-trichloroethane, dichlorodifluoromethane, and 1,1-dichloroethylene. Toxicol Appl Pharmacol. 1967;10(2):270-289.

25. 	Towner R, Reinke L, Janzen E, Yamashiro S. In vivo magnetic resonance imaging study of Kupffer cell involvement in CCl4-induced hepatotoxicity in rats. J physio Pharmacol. 1994;72(5):441-446. doi:10.1139/y94-064

26. 	IPCS. Environmental health criteria for carbon tetrachloride. Environ Heal Criteria. 1999;(208). https://www.who.int/ipcs/publications/ehc/who_ehc_208.pdf?ua=1.

27. 	Pääkkö P, Anttila S, Sormunen R, et al. Biochemical and morphological characterization of carbon tetrachloride-induced lung fibrosis in rats. Arch Toxicol. 1996;70(9):540-552. doi:10.1007/s002040050311

28. 	McDERMOTT W, Hardy H. Cirrhosis of the Liver Following Chronic Exposure to Carbon Tetrachloride. J Occup Med. 1963;5(5):249-251.

29. 	Weisburg E. Carcinogenicity studies on halogenated hydrocarbons. Env Heal Perspect. 1977:7-16. doi:10.1289/ehp.77217

30. 	Forni LG, Packer JE, Slater TF, Willson RL. Reaction of the trichloromethyl and halothane-derived peroxy radicals with unsaturated fatty acids: A pulse radiolysis study. Chem Biol Interact. 1983;45(2):171. doi:0.1016/0009-2797(83)90066-2

31. 	Slater TF. Free-radical mechanisms in tissue injury. Biochem J. 1984;222(1):1-15. doi:10.1042/bj2220001

32. 	Slater TF, McBrien D. Activation of Carbon Tetrachloride: Chemical Principles and Biological Significance. In: Free Radicals, Lipid Peroxidation and Cancer. New York, London, San Francisco: Academic Press; 1982.

33. 	Dianzani MU. Lipid peroxidation and haloalkylation: two distinct mechanisms for carbon tetrachloride-induced liver damage. Int Congr Ser Exerpta Med. 1984;632:39-50.

34. 	Tomasi A, Albano E, Banni S, et al. Free-radical metabolism of carbon tetrachloride in rat liver mitochondria. A study of the mechanism of activation. Biochem J. 1987;246(2):313-317. doi:10.1042/bj2460313

35. 	Recknagel R, Glende E. Carbon tetrachloride hepatotoxicity: An example of lethal cleavage. Crit Rev Toxicol. 1973;2:263-297.

36. 	Bredsdorff L, Nielsen E. Chlorofluorocarbons: CFC-11, CFC-12, CFC-113, HCFC-21, HCFC-31, HCFC-133a. Copenhagen: The Danish Environmental Protection Agency; 2014. https://www2.mst.dk/Udgiv/publications/2014/04/978-87-93178-40-3.pdf.

37. 	United States Envrionmental Protection Agency. Risk Evaluation for Carbon Tetrachloride. United States Envrionmental Prot Agency. 2020;(EPA-740-R1-8014). https://www.epa.gov/assessing-and-managing-chemicals-under-tsca/risk-evaluation-carbon-tetrachloride.

38. 	Cunnold D, Fraser PJ, Weiss RF, et al. Global trends and annual releases of CCl3F and CCl2F2 estimated from ALE/GAGE and other measurements from July 1978 to June 1991. J Geophys. 1994;99:1107–1126.

39. 	Oster H, Sonntag C, Munnich K. Groundwater age dating with chlorofluorocarbons. Water Resour Res. 1996;32:2989–3001.

40. 	Wang JL, Chew C, Chen S, Kuo S. Concentration variability of anthropogenic halocarbons and applications as internal reference in volatile organic compound measure- ments. Env Sci Technol. 2000;34:2243–2248.

41. 	Chang C, Lo G, Tsai C, Wand J. Concentration variability of halocarbons over an electronics industrial park and its implication in compliance with the Montreal protocol. Enivironmental Sci Technol. 2001;35:3273–3279.

42. 	Ho D, Schlosser P, Smethie W, Simpson H. Variability in atmospheric chlorofluo- rocarbons (CCl3F and CCl2F2) near a large urban area: Implications for groundwater dating. Environ Sci Technol. 1998;32:2377–2382.

43. 	Earth System Research Laboratories: Global Monitoring Laboratory. National Oceanic and Atmospheric Administration.

44. 	Newman PA, Oman LD, Douglass AR, et al. What would have happened to the ozone layer if chlorofluorocarbons (CFCs) had not been regulated? Atmos Chem Phys. 2009;9(6):2113-2128. doi:10.5194/acp-9-2113-2009

45. 	Massmann J, Farrier D. Effects of atmospheric pressures on gas transport in the vadose zone. Water Resour Res. 1992;28(3):777-791.

46. 	Severinghaus J, Keeling R, Miller B, Weiss R, Deck B, Broecker W. Feasibility of using sand dunes as archives of old air. J Geophys Res - Atmos. 1997;102(14):16783-16792.

47. 	Nilson R, Peterson E, Lie K, Burkhard N, Hearst J. Atmospheric pumping: A mechanism causing vertical transport of contaminated gases through fractured permeable media. J Geophys Res - Solid Earth. 1991;96(13):21933-21948.

48. 	Thorstenson D, Weeks E, Haas H, Busenberg E, Plummer L, Peters C. Chemistry of unsaturated zone gases sampled in open boreholes at the crest of Yucca Mountain, Nevada: Data and basic concepts of chemical and physical processes in the mountain. Water Resour Res. 1998;34(6):1507-1529.

49. 	Weeks E, Earp D, Thompson G. Use of atmospheric fluorocarbons F-11 and F-12 to determine the diffusion parameters of the unsaturated zone in the southern high plains of Texas. Water Resour Res. 1982;15:1365-1378.

50. 	Busenberg E, Weeks E, Plummer L, Bartholemay R. Age dating ground water by use of chlorofluorocarbons (CCl3F and CCl2F2), and distribution of chlorofluorocarbons in the unsaturated zone, Snake River Plain aquifer, Idaho National Engineering Laboratory, Idaho. US Geol Surv Water-Resources Investig. 1993;93(47).

51. 	Cook P, Solomon D. The transport of atmospheric trace gases to the water table: Implications for groundwater dating with chlorofluorocarbons and Krypton-85. Water Resour Res. 1995;31(2):263-270.

52. 	Ciccioli P, Cooper W, Hammer P, Hayes J. Organic solute-mineral surface interactions: A new method for the determination of groundwater velocities. Water Resour Res. 1980;16(1):217-223.

53. 	Brown J. M.S. Thesis - Evolution of Fluorocarbon Compounds as Ground Water Tracers-Soil Column Studies. 1980.

54. 	Russell A, Thompson G. Mechanism leading to enrichment of the atmospheric fluorocarbons CCl3F and CCl2F2 in groundwater. Water Resour Res. 1983;19(1):57-60.

55. 	Duffy C, McCallister S, Renken R. Carbon Tetrachloride Retention by Modern and Buried Soil A Horizons. J Environ Qual. 1997;26(4):1123-1127.

56. 	Dural NH, Chen CH, Puri RK. Adsorption equilibrium of carbon tetrachloride on dry soils. Chem Eng Commun. 1997;162(May):75-92. doi:10.1080/00986449708936633

57. 	Deitsch J, Smith J, Arnold M, Bolus J. Sorption and Desorption Rates of Carbon Tetrachloride and 1,2-Dichlorobenzene to Three Organobentonites and a Natural Peat Soil. Enivironmental Sci Technol. 1998;32(20):3169–3177.

58. 	Höhener P, Werner D, Balsiger C, Pasteris G. Worldwide occurrence and fate of chlorofluorocarbons in groundwater. Crit Rev Environ Sci Technol. 2003;33(1):1-29. doi:10.1080/10643380390814433

59. 	Laugwitz R, Poller T, Deipser A. The contribution of domestic waste landfills to the chlorofluorocarbon cycle, in German. Muell Abfall. 1990;(22):316–318.

60. 	Deipser A, Stegmann R. The origin and fate of volatile trace components in municipal solid waste landfills. Waste Manag Res. 1994;(12):129–139.

61. 	Thompson G, Hayes J. Trichlorofluoromethane in groundwater — a possible tracer and indicator of groundwater age. Water Resour Res. 1979;(15):547–554.

62. 	Ekwurzel B, Schlosser P, Smethie W, et al. Dating of shallow groundwater: Comparison of the transient tracers 3H/4H, chlorofluorocarbons, and 85Kr. Water Resour Res. 1994;(30):1693–170.

63. 	Atsdr. Toxicological Profile of Carbon Tetrachloride.; 2005.

64. 	Bu X, Warner M. Solubility of chlorofluorocarbon 113 in water and seawater. Deep Sea Res. 1995;(42):1151–1161.

65. 	Warner J, Weiss R. Solubilities of chlorofluorocarbons 11 and 12 in water and seawater. Deep Res. 1985;(32):1485–1497.

66. 	Vollrath A, Hohl C, Seiler H. Trace analysis of chlorofluorocarbons/partially halogenated chlorofluorohydrocarbons (CFC/HCFC) in polymeric foams by headspace capillary gas chromatography with electron-capture detection or ion trap detection combined with preconcentration on cold trap. Fresenius J Anal Chem. 1995;351:251–259.

67. 	Ward R, Williams G, Hills C. Changes in major and trace components of landfill gas during subsurface migration. Waste Manag Res. 1996;(14):243–261.

68. 	Penny C, Vuilleumier S, Bringel F. Microbial degradation of tetrachloromethane: mechanisms and perspectives for bioremediation. FEMS Microbiol Ecol. 2010;74(2):257-275. doi:10.1111/j.1574-6941.2010.00935.x

69. 	Vogel TM, McCarty PL. Transformations of halogenated aliphatic compounds. Environ Sci Technol. 1987;21(8):722-736. doi:10.1021/es00162a001‚

70. 	He YT, Wilson JT, Su C, Wilkin RT. Review of Abiotic Degradation of Chlorinated Solvents by Reactive Iron Minerals in Aquifers. Groundw Monit Remediat. 2015;35(3):57-75. doi:10.1111/gwmr.12111

71. 	He YT, Wilson JT, Wilkin RT. Transformation of Reactive Iron Minerals in a Permeable Reactive Barrier (Biowall) Used to Treat TCE in Groundwater. Environ Sci Technol. 2008;42:6690–6696.

72. 	Erbs M, Hansen HCB, Olsen CE. Reductive dechlorination of carbon tetrachloride using iron(II) iron(III) hydroxide sulfate (green rust). Environ Sci Technol. 1999;33(2):307-311. doi:10.1021/es980221t

73. 	Assaf-Anid N, Lin K-Y. Carbon Tetrachloride Reduction by Fe2+, S2- and FeS with vitamin B12 as an organic Amendment. 2002:94-99.

74. 	Kenneke JF, Weber EJ. Reductive Dehalogenation of Halomethanes in Iron- and Sulfate-Reducing Sediments. Enivironmental Sci Technol. 2003;37:713-720.

75. 	Woojin Lee, Batchelor B. Abiotic Reductive Dechlorination of Chlorinated Ethylenes by Iron-Bearing Soil Minerals. 1. Pyrite and Magnetite. Environ Sci Technol. 2002;36:5147-5154.

76. 	McCormick ML, Adriaen P. Carbon Tetrachloride Transformation on the Surface of Nanoscale Biogenic Magnetite Particles, Environmental Science and Technology. Environ Sci Technol. 2004;38:1045-1053.

77. 	Balsiger C, Holliger C, Höhener P. Reductive dechlorination of chlorofluorocarbons and hydrochlorofluorocarbons in sewage sludge and aquifer sediment microcosms. Chemosphere. 2005;61(3):361-373. doi:10.1016/j.chemosphere.2005.02.087

78. 	Deipser A, Stegmann R. Biological degradation of VCCs and CFCs under simulated anaerobic landfill conditions in laboratory test digesters. Environ Sci Pollut Res. 1997;4(4):209-216. doi:10.1007/BF02986348

79. 	Tobiszewski M, Namiesnik J. Abiotic degradation of chlorinated ethanes and ethenes in water. Environ Sci Pollut Res. 2012;19:1994–2006.

80. 	Holliger C, Regeard C, Diekert G. Dehalogenation by Anaerobic Bacteria. In Dehalogenation: Microbial Processes and Environmental Applications. (Häggblom M, Bossert I, eds.). Boston: Kluwer Academic Publishers; 2003.

81. 	Slater T, Cheeseman K, Ingold K. Carbon tetrachloride toxicity as a model for studying free-radical mediated liver injury. Philos Trans R Soc L B Biol Sci. 1985;311:633-645. doi:10.1098/rstb.1985.0169

82. 	Tro N. Chemistry: A Molecular Approach. 5th ed. Pearson; 2020.

83. 	Pohl L, Schulick R, Highet R. Reductive-oxygenation mechanism of metabolism of carbon tetrachloride to phosgene by cytochrome P-450. Mol Pharmacol. 1985;(25):318-321.

84. 	Lovley DR, Woodward JC. Consumption of Freons CFC-11 and CFC-12 by Anaerobic Sediments and Soils. Environ Sci Technol. 1992;26(5):925-929. doi:10.1021/es00029a009

85. 	Horneman A, Stute M, Schlosser P, et al. Degradation rates of CFC-11, CFC-12 and CFC-113 in anoxic shallow aquifers of Araihazar, Bangladesh. J Contam Hydrol. 2008;97(1-2):27-41. doi:10.1016/j.jconhyd.2007.12.001

86. 	Sebol LA, Robertson WD, Busenberg E, Plummer LN, Ryan MC, Schiff SL. Evidence of CFC degradation in groundwater under pyrite-oxidizing conditions. J Hydrol. 2007;347(1-2):1-12. doi:10.1016/j.jhydrol.2007.08.009

87. 	Egli C, Tschan T, Scholtz R, Cook A, Leisinger T. Transformation of Tetrachloromethane to Dichloromethane and Carbon Dioxide by Acetobacterium woodii. Appl Env Microbiol. 1988;54.

88. 	Novak PJ, Daniels L, Parkin GF. Enhanced dechlorination of carbon tetrachloride and chloroform in the presence of elemental iron and Methanosarcina barkeri, Methanosarcina thermophila, or Methanosaeta concillii. Environ Sci Technol. 1998;32(10):1438-1443. doi:10.1021/es970785h

89. 	Novak PJ, Daniels L, Parkin GF. Rapid dechlorination of carbon tetrachloride and chloroform by extracellular agents in cultures of Methanosarcina thermophila. Environ Sci Technol. 1998;32(20):3132-3136. doi:10.1021/es971069i

90. 	Best JH de, Salminen E, Doddema HJ, Janssen DB, Harder W. Transformation of carbon tetrachloride under sulfate reducing conditions. Biodegradation. 1998;8:429-436.

91. 	Sherwood J, Petersen J, Skeen R. Effects of nitrate and acetate availability on chloroform production during carbon tetrachloride destruction. Biotechnol Bioeng. 1999;(64):342–348.

92. 	Gälli R, McCarty PL. Biotransformation of 1,1,1- trichloroethane, trichloromethane, and tetrachloromethane by a Clostridium sp. Appl Env Microbiol. 1989;(5):837–844.

93. 	Krone UE, Thauer RK, Hogenkamp HPC, Steinbach K. Reductive Formation of Carbon Monoxide from CC14 and FREONs 11, 12, and 13 Catalyzed by Corrinoids. Biochemistry. 1991;30(10):2713-2719. doi:10.1021/bi00224a020

94. 	Van der Zee FP, Cervantes FJ. Impact and application of electron shuttles on the redox (bio)transformation of contaminants: A review. Biotechnol Adv. 2009;27(3):256-277. doi:10.1016/j.biotechadv.2009.01.004

95. 	Sylvestre M, Bertrand JL, Viel G. Feasibility study for the potential use of biocatalytic systems to destroy chlorofluorocarbons (CFCs). Crit Rev Environ Sci Technol. 1997;27(2):87-111. doi:10.1080/10643389.1997.10737058

96. 	Li S, Wackett LP. Reductive Dehalogenation by Cytochrome P450CAM: Substrate Binding and Catalysis. Biochemistry. 1993;32(36):9355-9361. doi:10.1021/bi00087a014

97. 	Castro CE, Wade RS, Belser NO. Biodehalogenation : Reactions of Cytochrome P-450 with Polyhalomethanes1 ". 1985:204-210.

98. 	Castro CE, Helvenston M, Belser N. Biodehalogenation, reductive dehalogenation by Methanobacterium thermoautotrophicum, comparison with nickel(l)octaethylisobacteriochlorin anion and F-430 model. Environ Toxicol Chern. 1994;13(429-433).

99. 	Nomenclature of Corrinoids. Pure Appl Chem. 1975;49:495-502.

100. 	Friedmann H, Klein A, Thauer R. Structure and function of the nickel porphinoid, coenzyme F430, and of its enzyme, methyl coenzyme M reductase. Microbiol Rev. 1990;87:339-348.

101. 	Picardal FW, Arnold RG, Couch H, Little AM, Smith ME. Involvement of cytochromes in the anaerobic biotransformation of tetrachloromethane by Shewanella putrefaciens 200. Appl Environ Microbiol. 1993;59(11):3763-3770.

102. 	Picardal F, Arnold RG, Huey BB. Effects of electron donor and acceptor conditions on reductive dehalogenation of tetrachloromethane by Shewanella putrefaciens 200. Appl Environ Microbiol. 1995;61(1):8-12. doi:10.1016/j.visres.2004.12.003

103. 	Gantzer CJ, Wackett LP. Reductive Dechlorination Catalyzed by Bacterial Transition-Metal Coenzymes. Environ Sci Technol. 1991;25(4):715-722. doi:10.1021/es00016a017

104. 	Chiu P-C, Reinhard M.  Transformation of Carbon Tetrachloride by Reduced Vitamin B 12 in Aqueous Cysteine Solution . Environ Sci Technol. 2002;30(6):1882-1889. doi:10.1021/es950477o

105. 	Glasser NR, Saunders SH, Newman DK. The Colorful World of Extracellular Electron Shuttles. Annu Rev Microbiol. 2017;71:731-751. doi:10.1146/annurev-micro-090816-093913

106. 	Curtis G, Reinhard M. Reductive dehalogenation of hexachloroethane, carbon tetrachloride, and bromoform by anthraquinone disulfonate and humic acid. Env Sci Technol. 1994;(28):2393–2401.

107. 	Tratnyek PG, Macalady D. Abiotic reduction of nitro aromatic pesticides in anaerobic laboratory systems. J Agric Food Chem. 1989;(37):248–254.

108. 	Doong R-A, Chiang HC. Transformation of carbon tetrachloride by thiol reductants in the presence of quinone compounds. Environ Sci Technol. 2005;39(19):7460-7468. doi:10.1021/es047956k

109. 	Cervantes FJ, Vu-Thi-Thu L, Lettinga G, Field JA. Quinone-respiration improves dechlorination of carbon tetrachloride by anaerobic sludge. Appl Microbiol Biotechnol. 2004;64(5):702-711. doi:10.1007/s00253-004-1564-z

110. 	Guerrero-Barajas C, Field JA. Riboflavin- and cobalamin-mediated biodegradation of chloroform in a methanogenic consortium. Biotechnol Bioeng. 2005;89:539–550.

111. 	Wolf M, Kappler A, Jiang J, Meckenstock RU. Effects of humic substances and quinones at low concentrations on ferrihydrite reduction by Geobacter metallireducens. Environ Sci Technol. 2009;43(15):5679-5685. doi:10.1021/es803647r

112. 	Field J, Cervantes FJ. Microbial Redox Reactions Mediated by Humus and Structurally Related Quinones. In: Use of Humic Substances to Remediate Polluted Environments: From Theory to Practice, Vol 52. (Perminova I, Hatfield K, Hertkorn N, eds.). Dordrecht, The Netherlands: Springer; 2005.

113. 	Schwarzenbach RP, Stierli R, Lanz K, Zeyer J. Quinone and iron porphyrin mediated reduction of nitroaromatic compounds in homogeneous aqeous solution. Env Sci Technol. 1990;24:1566–1574.

114. 	Tratnyek PG, Scherer MM, Deng B, Hu S. Effects of natural organic matter, anthropogenic surfactants, and model quinones on the reduction of contaminants by zero-valent iron. Water Resour. 2001;35:4435–4443.

115. 	Dror I, Schlautman M. Cosolvent effect on the catalytic reductive dechlorination of PCE. Chemosphere. 2004;57:1505–1514.

116. 	Hashsham S, Freedman DL. Enhanced biotransformation of carbon tetrachloride by Acetobacterium woodii upon addition of hydroxocobalamin and fructose. Appl Env Microbiol Env Microbiol. 1999;65:4537–4542.

117. 	Collins R, Picardal F. Enhanced anaerobic transformations of carbon tetrachloride by soil organic matter. Env Toxicol Chem. 1999;18:2703–2710.

118. 	Doong R, Lai Y. Dechlorination of tetrachloroethylene by palladized iron in the presence of humic acid. Water Resour. 2005;39:2309–2318.

119. 	Scheutz C, Winther K, Kjeldsen P. Removal of halogenated organic compounds in landfill gas by top covers containing zero-valent iron. Environ Sci Technol. 2000;34(12):2557-2563. doi:10.1021/es991301f

120. 	Juszczyk W, Malinowski A, Karpiński Z. Hydrodechlorination of CCl2F2 (CFC-12) over γ-alumina supported palladium catalysts. Appl Catal A Gen. 1998;166(2):311-319. doi:10.1016/S0926-860X(97)00265-2

121. 	Dhandapani B, Oyama ST. Novel catalysts for selective dehalogenation of CCl2F2(CFC 12). Catal Letters. 1995;35(3-4):353-360. doi:10.1007/BF00807192

122. 	McCormick, Michael L.; Adriaens P. Carbon Tetrachloride Transformation on the Surface of Nanoscale. Environ Sci Technol. 2004;38:1045-1053.

123. 	Zwank L, Elsner M, Aeberhard A, Schwarzenbach RP, Haderlein SB. Carbon isotope fractionation in the reductive dehalogenation of carbon tetrachloride at iron (hydr)oxide and iron sulfide minerals. Environ Sci Technol. 2005;39(15):5634-5641. doi:10.1021/es0487776

124. 	Choi J, Choi K, Lee W. Effects of transition metal and sulfide on the reductive dechlorination of carbon tetrachloride and 1,1,1-trichloroethane by FeS. J Hazard Mater. 2009;162(2-3):1151-1158. doi:10.1016/j.jhazmat.2008.06.007

125. 	Lipczynska-Kochany E, Harms S, Milburn R, Sprah G, Nadarajah N. Degradation of carbon tetrachloride in the presence of iron and sulphur containing compounds. 1994;29(7):1477-1489.

126. 	Butler E, Hayes K. Kinetics of the transformation of halogentated compounds by iron sulfide. Enivironmental Sci Technol. 2000;34:422-429.

127. 	Kriegman-King MR, Reinhard M. Transformation of Carbon Tetrachloride by Pyrite in Aqueous Solution. Environ Sci Technol. 1994;28(4):692-700. doi:10.1021/es00053a025

128. 	Devlin JF, Muller D. Field and Laboratory Studies of Carbon Tetrachloride Transformation in a Sandy Aquifer under Sulfate Reducing Conditions. Enivironmental Sci Technol. 1999;33:1021-1027.

129. 	McCormick ML, Jung PT, Groos V, et al. Assessing biotic and abiotic contributions to chlorinated solvent transformation in iron-reducing and sulphidogenic environments. IAHS-AISH Publ. 2002;(275):119-126.

130. 	Danielsen KM, Hayes KF. pH dependence of carbon tetrachloride reductive dechlorination by magnetite. Environ Sci Technol. 2004;38(18):4745-4752. doi:10.1021/es0496874

131. 	Liang X, Butler EC. Effects of natural organic matter model compounds on the transformation of carbon tetrachloride by chloride green rust. Water Res. 2010;44(7):2125-2132. doi:10.1016/j.watres.2009.12.026

132. 	Ayala-Luis KB, Cooper NGA, Bender Koch C, Hansen HCB. Erratum: Efficient dechlorination of carbon tetrachloride by hydrophobic green rust intercalated with dodecanoate anions (Environmental Science and Technology (2012) 46:6 (3390-3397) DOI: 10.1021/es204368u). Environ Sci Technol. 2012;46(8):4683. doi:10.1021/es301121a

133. 	Shao H, Butler EC. The influence of iron and sulfur mineral fractions on carbon tetrachloride transformation in model anaerobic soils and sediments. Chemosphere. 2007;68(10):1807-1813. doi:10.1016/j.chemosphere.2007.04.048

134. 	Elsner M, Haderlein SB, Kellerhals T, et al. Mechanisms and Products of Surface-Mediated Reductive Dehalogenation of Carbon Tetrachloride by Fe(II) on Goethite. Environ Sci Technol. 2004;38(7):2058-2066. doi:10.1021/es034741m

135. 	O’Loughlin EJ, Kemner KM, Burris DR.  Effects of Ag I , Au III , and Cu II on the Reductive Dechlorination of Carbon Tetrachloride by Green Rust . Environ Sci Technol. 2007;37(13):2905-2912. doi:10.1021/es030304w

136. 	Lewis TA, Paszczynski A, Gordon-Wylie SW, Jeedigunta S, Lee CH, Crawford RL. Carbon tetrachloride dechlorination by the bacterial transition metal chelator pyridine-2,6-bis(thiocarboxylic acid). Environ Sci Technol. 2001;35(3):552-559. doi:10.1021/es001419s

137. 	Lee C, Lewis TA, Paszczynski A, Crawford RL. Identication of an Extracellular Catalyst of Carbon Tetrachloride Dehalogenation from. Biochem Biophys Res Commun. 1999;566:562-566.

138. 	Dybas MJ, Tatara GM, Criddle CS. Localization and characterization of the carbon tetrachloride transformation activity of Pseudomonas sp. strain KC. Appl Environ Microbiol. 1995;61(2):758-762.

139. 	Criddle CS, DeWitt JT, Grbic-Galic D, McCarty PL. Transformation of Carbon Tetrachloride by Pseudomonas sp. Strain KC under Denitrification Conditions. Appl Environ Microbiol. 1990;56(11):3240-3246. doi:10.1128/aem.56.11.3240-3246.1990

140. 	Penny C, Vuilleumier S, Bringel F. Microbial degradation of tetrachloromethane: Mechanisms and perspectives for bioremediation. FEMS Microbiol Ecol. 2010;74(2):257-275. doi:10.1111/j.1574-6941.2010.00935.x

141. 	Lewis TA, Crawford RL. Physiological factors affecting carbon tetrachloride dehalogenation by the denitrifying bacterium Pseudomonas sp. strain KC. Appl Environ Microbiol. 1993;59(5):1635-1641.

142. 	Gerlach R, Cunningham AB, Caccavo F. Dissimilatory iron-reducing bacteria can influence the reduction of carbon tetrachloride by iron metal. Environ Sci Technol. 2000;34(12):2461-2464. doi:10.1021/es991200h

143. 	Rawe J, Krietemeyer S, Meagher-Hartzell E. Guide for Conducting Treatability Studies Under CERCLA: Biodegradation Remedy Selection. US Environ Prot Agency, Washington, DC. 1993.

144. 	Lorenz A, Löffler F, eds. Organohalide-Respiring Bacteria. 1st ed. Berlin: Springer-Verlag; 2016. doi:10.1007/978-3-662-49875-0_9

145. 	Stroo HF, Leeson A, Marqusee JA, et al. Chlorinated ethene source remediation: Lessons learned. Environ Sci Technol. 2012;46(12):6438-6447. doi:10.1021/es204714w

146. 	Fincker M, Spormann A. Biochemistry of catabolic reductive dehalogenation. Annu Rev Biochem. 2017;86:357-386.

147. 	Ritalahti KM, Amos BK, Sung Y, Wu Q, Koenigsberg SS, Löffler FE. Quantitative PCR targeting 16S rRNA and reductive dehalogenase genes simultaneously monitors multiple Dehalococcoides strains. Appl Environ Microbiol. 2006;72(4):2765-2774. doi:10.1128/AEM.72.4.2765-2774.2006

148. 	Syed A Hashsham, Richard Scholze DLF. Cobalamin - Enhanced Anaerobic Biotransformation of Carbon Tetrachloride. Enivironmental Sci Technol. 1995;29(11):2856-2863.

149. 	Shan H, Wang H, Yu R, Jacob P, Freedman DL. Biodegradation of high concentrations of halomethanes by a fermentative enrichment culture. AMB Express. 2014;4(1):1-10. doi:10.1186/s13568-014-0048-5

150. 	Vikesland PJ, Heathercock AM, Rebodos RL, Makus EE. Particle Size and Aggregation Effects on Magnetite Reactivity toward Carbon Tetrachloride. Enivironmental Sci Technol. 2007;41:5277-5283.

151. 	Helland B, Alvarez P, Schnoor J. Reductive dechlorination of carbon tetrachloride with elemental iron. J Hazard Mater. 1995;41(2-3):205-216.

152. 	Buttler E, Hayes K. Kinetics of the Transformation of Halogenated Aliphatic Compounds by Iron Sulfide. Environ Sci Technol. 2000;(34):422-429.

153. 	Krone U, Thauer R, Hogenkamp H, Steinbach K. Reductive formation of carbon monoxide from carbon tetrachloride and FREONS 11, 12, and 13 catalyzed by corrinoids. Biochemistry. 1991;30(10):2713-2719.

154. 	Mccall W, Christy TM, Pipp D, et al. Field Application of the Combined Membrane-Interface Probe and Hydraulic Profiling Tool (MiHpt). Groundw Monit Remediat. 2014;34(2):85-95. doi:10.1111/gwmr.12051

155. 	Mccall W. Tech Guide for Calculation of Estimated Hydraulic Conductivity ( Est . K ) Log from HPT Data. 2010:1-20. https://geoprobe.com/sites/default/files/storage/pdfs/tech_guide_estk_v5_0_0.pdf.

156. 	He J, Ritalahti KM, Aiello MR, Löffler FE. Complete detoxification of vinyl chloride by an anaerobic enrichment culture and identification of the reductively dechlorinating population as a Dehalococcoides species. Appl Environ Microbiol. 2003;69(2):996-1003. doi:10.1128/AEM.69.2.996-1003.2003

157. 	Yan J, Rash BA, Rainey FA, Moe WM. Detection and quantification of Dehalogenimonas and “ Dehalococcoides” populations via PCR-based protocols targeting 16S rRNA genes. Appl Environ Microbiol. 2009;75(23):7560-7564. doi:10.1128/AEM.01938-09

158. 	Murdoch FK, Murdoch R, Swift C, et al. Nanofluidic high-throughput qPCR to assess, monitor and predict microbial activities at chlorinated solvent sites.

159. 	Löffler FE, Sanford RA, Ritalahti KM. Chapter 5: Enrichment, Cultivation, and Detection of Reductivly Dechlorinating Bacteria. In: Leadbetter J, ed. Methods in Enzymology: Environmental Microbiology, Volume 397. 397th ed. Elsevier Inc.; 2005:77-111.

160. 	Sonier DN, Duran NL, Smith GB. Dechlorination of trichlorofluoromethane (CFC-11) by sulfate-reducing bacteria from an aquifer contaminated with halogenated aliphatic compounds. Appl Environ Microbiol. 1994;60(12):4567-4572. doi:10.1128/aem.60.12.4567-4572.1994

161. 	McCormick,Michael, Bouwer E, Adrieans P. Carbon Tetrachloride Transformation in a Model Iron-Reducing Culture: Relative Kinetics of Biotic and Abiotic Reactions. Environ Sci Technol. 2002;36:403-410.

162. 	Maithreepala RA, Doong R a. Effect of biogenic iron species and copper ions on the reduction of carbon tetrachloride under iron-reducing conditions. Chemosphere. 2008;70(8):1405-1413. doi:10.1016/j.chemosphere.2007.09.021

163. 	Culpepper JD, Scherer MM, Robinson TC, Neumann A, Cwiertny D, Latta DE. Reduction of PCE and TCE by magnetite revisited. Environ Sci Process Impacts. 2018;20(10):1340-1349. doi:10.1039/c8em00286j

164. 	Liu C, Zachara JM, Gorby YA, Szecsody JE, Brown CF. Microbial reduction of Fe(III) and sorption/precipitation of Fe(II) on Shewanella putrefaciens strain CN32. Environ Sci Technol. 2001;35(7):1385-1393. doi:10.1021/es0015139

165. 	NIST Chemistry WebBook, SRD 69. Natl Inst Stand Technol.

166. 	PubChem Compound Summary. Natl Cent Biotechnol Inf. 2021. https://pubchem.ncbi.nlm.nih.gov/compound/297.

167. 	Priority Pollutant List. United States Environ Prot Agency. 2014;(40 CFR Part 423, Appendix A.):1-2.

168. 	Huling SG, Weaver JW. Ground water issue: Dense nonaqueous phase liquids. United States Environ Prot Agency. 1992:1-25. doi:10.1056/NEJMoa030660

169. 	Loffler FE, Edwards EA. Harnessing microbial activities for environmental cleanup. Curr Opin Biotechnol. 2006;17(3):274-284. doi:10.1016/j.copbio.2006.05.001

170. 	Löffler FE, Edwards EA. Harnessing microbial activities for environmental cleanup. Curr Opin Biotechnol. 2006;17(3):274-284. doi:10.1016/j.copbio.2006.05.001

171. 	Morgan P, Major DW, McMaster ML, et al. Successful field demonstration of bioaugmentation to degrade PCE and TCE to ethene. IAHS-AISH Publ. 2002;(275):369-376.

172. 	Lendvay JM, Barcelona MJ, Daniels G, et al. Plume control using bioaugmentation with halorespiring microorganisms. IAHS-AISH Publ. 2002;(275):325-332.

173. 	Justicia-Leon SD, Higgins S, Mack EE, et al. Bioaugmentation with distinct Dehalobacter strains achieves chloroform detoxification in microcosms. Environ Sci Technol. 2014;48(3):1851-1858. doi:10.1021/es403582f

174. 	Lendvay JM, Löffler FE, Dollhopf M, et al. Bioreactive barriers: A comparison of bioaugmentation and biostimulation for chlorinated solvent remediation. Environ Sci Technol. 2003;37(7):1422-1431. doi:10.1021/es025985u

175. 	Löffler FE, Sanford RA, Tiedje JM. Initial characterization of a reductive dehalogenase from desulfitobacterium chlororespirans Co23. Appl Environ Microbiol. 1996;62(10):3809-3813. doi:10.1128/aem.62.10.3809-3813.1996

176. 	Vainberg S, Condee CW, Steffan RJ. Large-scale production of bacterial consortia for remediation of chlorinated solvent-contaminated groundwater. J Ind Microbiol Biotechnol Vol. 2009;36:1189–1197.

177. 	Dettmer-Wilde K, Engewald W. Practical Gas Chromatography: A Comprehensive Reference. In: Dettmer-Wilde K, Engewald W, eds. Heidelberg, New York, Dordrecht, London: Springer; 2014:271-302. doi:0.1007/978-3-642-54640-2

178. 	International Conference on Harmonization of Technical Requirments for Registration of Pharmacuticals for Human Use. In: Validation of Analytical Procedures: Text and Methodology Q2(R1). The International Conference on Harmonisation of Technical Requirements for Registration of Pharmaceuticals for Human Use; 2005:541-545. doi:10.1007/978-3-642-35099-3_86

179. 	Hatt JK, Ritalahti KM, Ogles DM, Lebrón CA., Löffler FE. Design and Application of an Internal Amplification Control to Improve Dehalococcoides mccartyi 16S rRNA Gene Enumeration by qPCR. Env Sci Technol. 2013;47(19):11131-11138.

180. 	Simsir B, Yan J, Im J, Graves D, Löffler FE. Natural Attenuation in Streambed Sediment Receiving Chlorinated Solvents from Underlying Fracture Networks. Env Sci Technol. 2017;51(9):4821-4830. doi:10.1021/acs.est.6b05554.Natural

181. 	Ritalahti KM, Hatt JK, Lugmayr V, et al. Comparing On-site to off-site biomass collection for dehalococcoides biomarker gene quantification to predict in situ chlorinated ethene detoxification potential. Environ Sci Technol. 2010;44(13):5127-5133. doi:10.1021/es100408r

182. 	Lu X, Wilson JT, Kampbell DH. Relationship between Dehalococcoides DNA in ground water and rates of reductive dechlorination at field scale. Water Res. 2006;40(16):3131-3140. doi:10.1016/j.watres.2006.05.030

183. 	Packer JE, Mahood JS, Willson RL, Wolfenden BS. Reactions of the Trichloromethylperoxy free radical (Cl3COO.) With Tryptophan Tryptophanyl-Tyrosine and Lysozyme. Int J Radiat Biol Relat Stud Phys Chem Med. 1981;39(2):135-141. doi:10.1080/09553008114550151

184. 	Connor HD, Thurman RG, Galizi MD, Mason RP. The formation of a novel free radical metabolite from CCl4 in the perfused rat liver and in vivo. J Biol Chem. 1986;261(10):4542-4548.

185. 	Im J, Mack EE, Seger ES, Löffler FE. Biotic and Abiotic Dehalogenation of 1,1,2-Trichloro-1,2,2-trifluoroethane (CFC-113): Implications for Bacterial Detoxification of Chlorinated Ethenes. Environ Sci Technol. 2019;53(20):11941-11948. doi:10.1021/acs.est.9b04399

186. 	Duhamel M, Grostern A, Dworatzek S, Edwards EA. Chloroform respiration to dichloromethane by a Dehalobacter population. Environ Microbiol. 2010;12(4):1053-1060. doi:10.1111/j.1462-2920.2009.02150.x

187. 	Lee M, Low A, Zemb O, Koenig J, Michaelsen A, Manefield M. Complete chloroform dechlorination by organochlorine respiration and fermentation. Environ Microbiol. 2012;14(4):883-894. doi:10.1111/j.1462-2920.2011.02656.x

188. 	Kleindienst S, Chourey K, Chen G, et al. Proteogenomics reveals novel reductive dehalogenases and methyltransferases expressed during anaerobic dichloromethane metabolism. Appl Environ Microbiol. 2019;85(6):1-16. doi:10.1128/AEM.02768-18

189. 	Archbold ME, Elliot T, Kalin RM. Carbon isotopic fractionation of CFCs during abiotic and biotic degradation. Environ Sci Technol. 2012;46(3):1764-1773. doi:10.1021/es203386a

190. 	Alvarado JS, Rose C, Lafreniere L. Degradation of carbon tetrachloride in the presence of zero-valent iron. J Env Monit. 2010;12(8):1524-1530. doi:10.1039/c0em00039f

191. 	Midgley T, Henne AL. Organic Fluorides as Refrigerants. Ind Eng Chem. 1930;22(5):542-545. doi:10.1021/ie50245a031

192. 	Rigby M, Park S, Saito T, et al. Increase in CFC-11 emissions from eastern China based on atmospheric observations. Nature. 2019;569(7757):546-550. doi:10.1038/s41586-019-1193-4

193. 	M.R. Bauer JBY. Processes and Mechanisms Controlling Consumption of Cfc-I1 and Cfc-12 By Peat From a Conifer-Swamp and Black Spruce-Tamarack Bog in New York State M.R. 1996;32(4):159-168.

194. 	Cicerone RJ. Halogens in the atmosphere. Rev Geophys. 1981;19(1):123-139. doi:10.1029/RG019i001p00123

195. 	Anderson JG, Toohey DW, Brune WH. Free radicals within the Antarctic vortex: The role of CFCs in Antarctic ozone loss. Science (80- ). 1990;251(4989):39-46. doi:10.1126/science.251.4989.39

196. 	Chambers LA, Gooddy DC, Binley AM. Use and application of CFC-11, CFC-12, CFC-113 and SF6 as environmental tracers of groundwater residence time: A review. Geosci Front. 2019;10(5):1643-1652. doi:10.1016/j.gsf.2018.02.017

197. 	Hinsby K, Højberg AL, Engesgaard P, et al. Transport and degradation of chlorofluorocarbons (CFCs) in the pyritic Rabis Creek aquifer, Denmark. Water Resour Res. 2007;43(10):1-15. doi:10.1029/2006WR005854

198. 	Koenig JC, Lee MJ, Manefield M. Successful microcosm demonstration of a strategy for biodegradation of a mixture of carbon tetrachloride and perchloroethene harnessing sulfate reducing and dehalorespiring bacteria. Hazard Mater. 2012;219-220:169-175. doi:10.1016/j.jhazmat.2012.03.076

199. 	Denovan B, Strand S. Biological Degradation of Chlorofluorocarbons in Anerobic Environments. 1992;24(7):935-940.

200. 	Yang Y, Zhang Z, Lu J, Maekawa T. Continuous methane fermentation and the production of vitamin B 12 in a fixed-bed reactor packed with loofah. Bioresour Technol. 2004;92(3):285-290. doi:10.1016/j.biortech.2003.09.011

201. 	Mazumder TK, Nishio N, Fukuzaki S, Nagai S. Production of extracellular vitamin B-12 compounds from methanol by Methanosarcina barkeri. Appl Microbiol Biotechnol. 1987;26(6):511-516. doi:10.1007/BF00253023

202. 	Kräutler B, Kohler HE, Stupperich E. 5′‐Methylbenzimidazolyl‐cobamides are the corrinoids from some sulfate‐reducing and sulfur‐metabolizing bacteria. Eur J Biochem. 1988;176(2):461-469. doi:10.1111/j.1432-1033.1988.tb14303.x

203. 	Guimarães DH, Weber A, Klaiber I, Vogler B, Renz P. Guanylcobamide and hypoxanthylcobamide-Corrinoids formed by Desulfovibrio vulgaris. Arch Microbiol. 1994;162(4):272-276. doi:10.1007/BF00301850

204. 	Men Y, Erica S, Ti S, et al. Identification of specific corrinoids reveals corrinoid modification in dechlorinating microbial communities. Env Microbiol. 2015;17(12):4873–4884. doi:10.1111/1462-2920.12500

205. 	Cobalt. In: Britannica. Web. The Editors of Encyclopaedia; 2020:https://www.britannica.com/science/cobalt-chemical.

206. 	Park S, De Perre C, Lee LS. Alternate Reductants with VB12 to Transform C8 and C6 Perfluoroalkyl Sulfonates: Limitations and Insights into Isomer-Specific Transformation Rates, Products and Pathways. Environ Sci Technol. 2017;51(23):13869-13877. doi:10.1021/acs.est.7b03744

207. 	Assaf-Anld N, Hayes KF, Vogel TM. Reductive Dechlorination of Carbon Tetrachloride by Cobalamin(II) in the Presence of Dithiothreitoi: Mechanistic Study, Effect of Redox Potential and pH. Environ Sci Technol. 1994;28(2):246-252. doi:10.1021/es00051a010

208. 	Liu J, Van Hoomissen DJ, Liu T, et al. Reductive Defluorination of Branched Per- and Polyfluoroalkyl Substances with Cobalt Complex Catalysts. Environ Sci Technol Lett. 2018;5(5):289-294. doi:10.1021/acs.estlett.8b00122

209. 	Banerjee R, Ragsdale SW. The many faces of vitamin B12: Catalysis by cobalamin-dependent enzymes. Annu Rev Biochem. 2003;72:209-247. doi:10.1146/annurev.biochem.72.121801.161828

210. 	Amendment To the Montreal Protocol on Substances That Deplete the Ozone Layer: Report (to Accompany Treaty Doc. 103-9). Washington, D.C.: United States. Congress.; 1993.

211. 	Gorski CA, Scherer MM. Influence of Magnetite Stoichiometry on FeII Uptake and Nitrobenzene Reduction. 2009.

212. 	Weber KA, Achenbach LA, Coates JD. Microorganisms pumping iron: anaerobic microbial iron oxidation and reduction. Nat Rev Microbiol. 2006;4(10):752-764. doi:10.1038/nrmicro1490

213. 	Swades K. Chaudhuri, Joseph G. Lack, John D. Coates. Biogenic Magnetite Formation through Anaerobic Biooxidation of Fe(II). Appl Environ Microbiol. 2001;67(6):2844-2848. doi:10.1128/AEM.67.6.2844

214. 	Dennis A. Bazylinski, Richard B. Frankel, Konhauser KO. Modes of Biomineralization of Magnetite by Microbes. Geomicrobiol . 2007;24(665-475).

215. 	Roh Y, Zhang CL, Vali H, Lauf RJ, Zhou J, Phelps TJ. Biogeochemical and environmental factors in Fe biomineralization: Magnetite and siderite formation. Clays Clay Miner. 2003;51(1):83-95. doi:10.1346/CCMN.2003.510110

216. 	Chaudhuri SK, Lack JG, Coates JD. Biogenic Magnetite Formation through Anaerobic Biooxidation of Fe(II). Appl Environ Microbiol. 2001;67(6):2844-2848. doi:10.1128/AEM.67.6.2844-2848.2001

217. 	Hungate RE. Studies on Cellulose Fermentation: I. The Culture and Physiology of an Anaerobic Cellulose-digesting Bacterium. J Bacteriol. 1944;48(5):499-513. doi:10.1128/JB.48.5.499-513.1944

218. 	Onley JR, Ahsan S, Sanford RA. Denitrification by Anaeromyxobacter dehalogenans, a Common Soil Bacterium Lacking the Nitrite Reductase Genes nirS and nirK. 2018;84(4):1-14.

219. 	Gossett JM. Measurement of Henry ’ s Law Constants for Hydrocarbons. Environ Sci Technol. 1987;21(2):202-208.

220. 	Caporaso JG, Lauber CL, Walters WA, et al. Ultra-high-throughput microbial community analysis on the Illumina HiSeq and MiSeq platforms. ISME J. 2012;6(8):1621-1624. doi:10.1038/ismej.2012.8

221. 	Westcott SL, Schloss PD. De novo clustering methods outperform reference-based methods for assigning 16S rRNA gene sequences to operational taxonomic units. PeerJ. 2015;2015(12). doi:10.7717/peerj.1487

222. 	Goujon M, McWilliam H, Li W, et al. A new bioinformatics analysis tools framework at EMBL-EBI. Nucleic Acids Res. 2010;38(SUPPL. 2):695-699. doi:10.1093/nar/gkq313

223. 	Guindon S, Gascuel O. A Simple, Fast, and Accurate Algorithm to Estimate Large Phylogenies by Maximum Likelihood. Syst Biol. 2003;52(5):696-704. doi:10.1080/10635150390235520

224. 	Letunic I, Bork P. Materials for photoelectrocatalytic hydrogen production. PCT Int Appl. 2011;(WO2011085731A1):37pp.

225. 	D’Amore R, Ijaz UZ, Schirmer M, et al. A comprehensive benchmarking study of protocols and sequencing platforms for 16S rRNA community profiling. BMC Genomics. 2016;17(1). doi:10.1186/s12864-015-2194-9

226. 	Liao PC, Huang BH, Huang S. Microbial community composition of the Danshui River estuary of Northern Taiwan and the practicality of the phylogenetic method in microbial barcoding. Microb Ecol. 2007;54(3):497-507. doi:10.1007/s00248-007-9217-2

227. 	Kruse S, Goris T, Westermann M, Adrian L, Diekert G. Hydrogen production by Sulfurospirillum species enables syntrophic interactions of Epsilonproteobacteria. Nat Commun. 2018;9(1). doi:10.1038/s41467-018-07342-3

228. 	Criddle CS, Tang W, Fu QS, Boonchayaanant B, Trost BM. Simple menaquinones reduce carbon tetrachloride and iron (III). Biodegradation. 2008;20(1):109-116. doi:10.1007/s10532-008-9204-4

229. 	Zhao L, Ma T, Gao M, et al. Characterization of microbial diversity and community in water flooding oil reservoirs in China. World J Microbiol Biotechnol. 2012;28(10):3039-3052. doi:10.1007/s11274-012-1114-2

230. 	Pester M, Brambilla E, Alazard D, et al. Complete genome sequences of Desulfosporosinus orientis DSM765T, Desulfosporosinus youngiae DSM17734T, Desulfosporosinus meridiei DSM13257T, and Desulfosporosinus acidiphilus DSM22704T. J Bacteriol. 2012;194(22):6300-6301. doi:10.1128/JB.01392-12

231. 	Robertson WJ, Bowman JP, Franzmann PD, Mee BJ. Desulfosporosinus meridiei sp. nov., a spore-forming sulfate-reducing bacterium isolated from gasolene-contaminated groundwater. Int J Syst Evol Microbiol. 2001;51(1):133-140. doi:10.1099/00207713-51-1-133

232. 	Kim BC, Jeon BS, Kim S, Kim H, Um Y, Sang BI. Caproiciproducens galactitolivorans gen. Nov., sp. nov., a bacterium capable of producing caproic acid from galactitol, isolated from a wastewater treatment plant. Int J Syst Evol Microbiol. 2015;65(12):4902-4908. doi:10.1099/ijsem.0.000665

233. 	Quast C, Pruesse E, Yilmaz P, et al. The SILVA ribosomal RNA gene database project: Improved data processing and web-based tools. Nucleic Acids Res. 2013;41(D1):590-596. doi:10.1093/nar/gks1219

234. 	Lee GH, Rhee MS, Chang DH, et al. Oscillibacter ruminantium sp. nov., isolated from the rumen of Korean native cattle. Int J Syst Evol Microbiol. 2013;63(PART6):1942-1946. doi:10.1099/ijs.0.041749-0

235. 	Shiratori H, Ohiwa H, Ikeno H, et al. Lutispora thermophila gen. nov., sp. nov., a thermophilic, spore-forming bacterium isolated from a thermophilic methanogenic bioreactor digesting municipal solid wastes. Int J Syst Evol Microbiol. 2008;58(4):964-969. doi:10.1099/ijs.0.65490-0

236. 	Song L, Dong X. Hydrogenoanaerobacterium saccharovorans gen. nov., sp. nov., isolated from H2-producing UASB granules. Int J Syst Evol Microbiol. 2009;59(2):295-299. doi:10.1099/ijs.0.000349-0

237. 	Kläring K, Hanske L, Bui N, et al. Intestinimonas butyriciproducens gen. nov., sp. nov., a butyrate-producing bacterium from the mouse intestine. Int J Syst Evol Microbiol. 2013;63(PART 12):4606-4612. doi:10.1099/ijs.0.051441-0

238. 	Iino T, Mori K, Tanaka K, Suzuki KI, Harayama S. Oscillibacter valericigenes gen. nov., sp. nov., a valerate-producing anaerobic bacterium isolated from the alimentary canal of a Japanese corbicula clam. Int J Syst Evol Microbiol. 2007;57(8):1840-1845. doi:10.1099/ijs.0.64717-0

239. 	Grabowski A, Tindall BJ, Bardin V, Blanchet D, Jeanthon C. Petrimonas sulfuriphila gen. nov., sp. nov., a mesophilic fermentative bacterium isolated from a biodegraded oil reservoir. Int J Syst Evol Microbiol. 2005;55(3):1113-1121. doi:10.1099/ijs.0.63426-0

240. 	Hahnke S, Langer T, Koeck DE, Klocke M. Description of Proteiniphilum saccharofermentans sp. nov., Petrimonas mucosa sp. nov. and Fermentimonas caenicola gen. nov., sp. nov., isolated from mesophilic laboratory-scale biogas reactors, and emended description of the genus Proteiniphilum. Int J Syst Evol Microbiol. 2016;66(3):1466-1475. doi:10.1099/ijsem.0.000902

241. 	Villemur R, Lanthier M, Beaudet R, Lépine F. The Desulfitobacterium genus. FEMS Microbiol Rev. 2006;30(5):706-733. doi:10.1111/j.1574-6976.2006.00029.x

242. 	Baron S, ed. Medical Microbiology, 4th Edition. University of Texas Medical Branch at Galveston; 1996.

243. 	Bae HS, Moe WM, Yan J, Tiago I, da Costa MS, Rainey FA. Propionicicella superfundia gen. nov., sp. nov., a chlorosolvent-tolerant propionate-forming, facultative anaerobic bacterium isolated from contaminated groundwater. Syst Appl Microbiol. 2006;29(5):404-413. doi:10.1016/j.syapm.2005.11.004

244. 	Imachi H, Aoi K, Tasumi E, et al. Cultivation of methanogenic community from subseafloor sediments using a continuous-flow bioreactor. ISME J. 2011;5(12):1913-1925. doi:10.1038/ismej.2011.64

245. 	Imachi H, Sakai S, Lipp JS, et al. Pelolinea submarina gen. nov., sp. nov., an anaerobic, filamentous bacterium of the phylum Chloroflexi isolated from subseafloor sediment. Int J Syst Evol Microbiol. 2014;64(PART 3):812-818. doi:10.1099/ijs.0.057547-0

246. 	Yang Y, Higgins SA, Yan J, et al. Grape pomace compost harbors organohalide-respiring Dehalogenimonas species with novel reductive dehalogenase genes. ISME J. 2017;11(12):2767-2780. doi:10.1038/ismej.2017.127

247. 	Consortium U. UniProt: A worldwide hub of protein knowledge. Nucleic Acids Res. 2019;47(D1):D506-D515. doi:10.1093/nar/gky1049

248. 	Nordberg H, Cantor M, Dusheyko S, et al. The genome portal of the Department of Energy Joint Genome Institute: 2014 updates. Nucleic Acids Res. 2014;42(D1):26-31. doi:10.1093/nar/gkt1069

249. 	Berg JS, Jézéquel D, Duverger A, Lamy D, Laberty-Robert C, Miot J. Microbial diversity involved in iron and cryptic sulfur cycling in the ferruginous, low-sulfate waters of Lake Pavin. PLoS One. 2019;14(2):1-21. doi:10.1371/journal.pone.0212787

250. 	Jørgensen BB, Findlay AJ, Pellerin A. The biogeochemical sulfur cycle of marine sediments. Front Microbiol. 2019;10(APR):1-27. doi:10.3389/fmicb.2019.00849

251. 	Molina M, Rowland F. Stratospheric sink for chlorofluoromethanes: Chlorine atom catalyzed destruction of ozone. Nature. 1974;249:810-812.

# VITA

Briana M. McDowell was born at Beale Air Force Base in Yuba County, California, to Lisa R. Bryan and Glenn A. McDowell. Growing up, she was passionate about musical theater and regularly performed in theatrical productions and choral ensembles. During her high school years, she became beguiled by the wildness and beauty of the Yuba and American rivers. She frequented these rivers as often as possible, spending entire days climbing up their granite pool drop rapids, exploring their nooks and crannies, and learning about the relentless power of these and all rivers. She became a raft guide on the American River after high school and spent two summers rafting and exploring as many rivers as possible. Feeling the need to explore more of the world, she moved to Boston in 2011, where she attended Suffolk University.

At Suffolk University, Briana acquired a B.S. in Biochemistry. During her undergraduate career, she was selected to participate in the McNair Scholars Program, where she was encouraged to find undergraduate research opportunities and attend graduate school. With the McNair Scholars advisors' encouragement, she gained a position as a Visiting Scientist at MIT Sea Grant. She worked with Dr. Julia Simpson on carbon sequestration in coastal eelgrass meadows. After graduating from Suffolk University in 2015, Briana entered graduate school in the Department of Microbiology at the University of Tennessee in Knoxville, where she joined the lab of Dr. Frank Löffler and began studying the degradation of halogenated methanes.