Transport of Fecal Pollution Indicators: Impacts from the Land Spreading of Liquid Manure on Water Quality

A Thesis Presented for the

Master of Science

Degree

The University of Tennessee, Knoxville

Derek Lee Street

December 2014

Copyright © 2014 by Derek Lee Street

All rights reserved.

# Dedication

For my wife Nicole, who has helped and inspired me more than she will ever know. Without her love and support I never would have dreamed of pursuing my degree.

# Acknowledgements

I would like to acknowledge my committee members Dr. Larry McKay, Dr. Alice Layton, and Dr. Andrea Ludwig for all their assistance along the way in making this thesis possible. I would also like to thank Dr. Ed Perfect for the wisdom he has imparted on hydrogeology and statistics, and Dan Williams for helping me improve my laboratory skills.

I would also like to acknowledge Cody Yankey and Casey Martin. Cody provided numerous hours of assistance in the field and lab, and helped trouble shoot several unexpected situations. Casey selflessly gave his time to assist me in improving me skills and processing samples when my own time was short.

# Abstract

This thesis is a case study to determine if groundwater and/or drainage tiles are important pathways for fecal migration to streams resulting from the application of liquid manure to cropland at a small dairy farm, the Little River Animal and Environmental Unit, near Townsend, TN. Traditional biological indicators, coliforms and E.coli, were used in conjunction with a bovine specific Bacteroides assay to measure fecal microbes. Total nitrates, turbidity, and other chemical parameters for water quality also were used to identify related fecal contamination. This thesis covers three separate manure applications. The first manure application occurred in May 2013, the second in November 2013, and the third in May 2014. Water samples were collected from shallow groundwater wells, a drainage ditch, drainage tiles, and the river adjacent to the dairy

The first liquid manure application resulted in significant changes in E.coli, AllBac, and nitrates. During the first manure application E.coli concentrations in the drainage tiles, drainage ditch, and Little River had statistically significant increases compared to samples collected before the application. AllBac only significantly increased in the Little River, and cannot be definitely ascribed to the liquid manure application because of possible upstream inputs. However, the sudden presence of detectable levels of BoBac in the surface drainage ditch, wells, and Little River after the application provides a strong line of evidence that fecal pollution was transported around the study site.

In contrast to the first manure application, no significant changes were observed for any of the fecal markers in subsequent applications. During the second application, November 4th through 7th of 2013, and third manure application, May 21st through 24th of 2014, there was no precipitation in the weeks before and after the applications, and the depth to the water table was greater than the first application.

These results suggest it is possible to apply large volumes of liquid manure and have minimal/non-detectable impacts on surface and ground water quality. However, this is only true when the liquid manure occurs during optimum hydrologic conditions and best management practices are in place to prevent direct application of liquid manure to surface waters.

Table of Contents

1. Introduction	1

2. Study Site and Previous Investigations	9

2.1 Study Site	9

2.2 LRAEU Regional and Local Geology and Hydrogeology	11

2.3 Previous Water Quality Monitoring	19

3. Methods	22

3.1 Sampling Plan	22

3.2 Water Quality Testing	35

3.2.1 Field Parameter Measurements	35

3.2.2 Nitrates	36

3.2.3 Coliforms and E.coli	36

3.2.4 DNA Extraction and Bacteroides Analysis	37

3.2.5 Analysis of Microbial Community Structure	42

3.4 Statistical Analysis	44

4. Results	45

4.1 Hydrologic Monitoring	45

4.1.1 Precipitation	45

4.1.2 Ground Water Levels	51

4.1.3 Gaining and Losing Status of the Drainage Ditch	55

4.2 Liquid Manure Chemical and Taxonomic Structure	57

4.3 Data and Simple Statistics	61

4.4 Changes in Fecal Indicators Before and After Liquid Manure Applications	71

4.5 Correlations and Significance	80

5. Discussion and Conclusions	83

5.1 Overview of this Study	83

References	88

Appendices	94

Appendix A: Bore Logs for Wells	95

Appendix B: Statistical Errata	99

Appendix C: Water Quality Data-Supplemental Data	123

VITA	142

# List of Tables

Table 1: Sampling and manure application dates at the LRAEU.	26

Table 2: Chemical and physical properties measured for manure and water samples.	27

Table 3: Descriptive details on wells sampled.	28

Table 4: Target organisms and primer/probe gene sequences.	40

Table 6: Depth to water and elevations in wells adjacent to the drainage ditch along with the elevation of the water in the drainage ditch at surveyed markers.	56

Table 7: Mean chemical analysis data and Bacteroides concentrations in liquid manure from each application	58

Table 8: Microbial community relative abundance and the ten most abundant genera, plus Escherichia.	59

Table 9: Untransformed E.coli concentrations in MPN/100mls for each sampling location by date. Values below the detection limit of 1 MPN/100mls were given a value of 0.5 MPN/100mls.	62

Table 10: Untransformed coliform  concentrations in MPN/100mls for each sampling location by date.  Values below the detection limit of 1 MPN/100mls were given a value of 0.5 MPN/100mls.	66

Table 11: Untransformed nitrate  concentrations in mg/L for each sampling location by date.  Values below the detection limit of 0.3 mg/L were given a value of 0.15 mg/L	67

Table 12: Untransformed AllBac concentrations in copies/100mls for each sampling location by date.  Dark grey cells were either not sampled or sampled and the filters not analyzed.	68

Table 13: Untransformed BoBac concentrations in copies/100mls for each sampling location by date.  Dark grey cells were either not sampled or sampled and the filters not analyzed.	69

Table 14: Simple statistics for chemical and biological data for each sample location site type.	70

Table 15: Significance of change in fecal indicators by manure application analyzing the time frame of 28 days before and after the start of each manure application. Data was fit to a normal distribution and considered significant at p< 0.05. BoBac is the exception to this statistical analysis and is either below detection limit or above the detection limit.	79

Table 16: Pearson correlation coefficients for the biological indicators used in this study with their probability of significance below.	81

Table 17: Pearson correlation coefficients for the chemical and biological indicators used in this study with their probability of significance below.	82

# List of Figures

Figure 1: Pathways for fecal transport at a typical dairy farm.	8

Figure 2: Site map of the LRAEU bonded in red.	10

Figure 3: Projected flow direction and surface runoff divide.  Elevation contours are 0.1524m increments (0.5 ft).	15

Figure 4: Average elevation of water table in selected wells at the LRAEU and average depth to water table (Hunter, 2013).	16

Figure 5: A cross sectional diagram of the study site at the LRAEU. Reprinted with permission from Robert Hunter.	17

Figure 6: Generalized cross section of a gaining and losing streams.	18

Figure 7: Sampling locations at the LRAEU.	25

Figure 8: Images of wells sampled at the LRAEU.	29

Figure 9: Images of drainage tiles sampled at the LRAEU.	30

Figure 10: Images of surface drainage ditch sampled at LRAEU. Site S3 was photographed during dry conditions.	31

Figure 11: Images of Little River sampling locations at the LRAEU.	32

Figure 12: Cumulative precipitation by month over the course of the study along with the cumulative historical averages by month from the weather station at Tyson McGee airport.	47

Figure 13: Precipitation during the 28 day period preceding and following the start of the first liquid manure application.	48

Figure 14: Precipitation during the 28 day period preceding and following the start of the second liquid manure application.	49

Figure 15: Precipitation during the 28 day period preceding and following the start of the third liquid manure application.	50

Figure 16: Depth to groundwater manually measured on each day of sampling at the four monitored wells. Daily precipitation totals are shown with blue bars.	53

Figure 17: Depth to groundwater measured by the pressure transducers placed into monitoring wells with the liquid manure applications shown in the grey columns.	54

Figure 18: Taxonomic composition of bacteria sequences identified in the liquid manure used in the first application at the LRAEU. Figure created using data uploaded into MG-RAST and graphic created by Krona.	60

Figure 19: Graphs of log E.coli mean values during the 28 days before and after the start of all three liquid manure applications. Confidence intervals are one standard deviation.	74

Figure 20:  Graphs of log coliform mean values during the 28 days before and after the start of all three liquid manure applications. Confidence intervals are one standard deviation.	75

Figure 21: Graphs nitrate mean values during the 28 days before and after the start of all three liquid manure applications. Confidence intervals are one standard deviation.	76

Figure 22: Graphs of log AllBac mean values during the 28 days before and after the start of all three liquid manure applications. Confidence intervals are one standard deviation.	77

Figure 23: Graphs of log BoBac mean values during the 28 days before and after the start of all three liquid manure applications. Confidence intervals are one standard deviation.	78

# 1. Introduction

Successful livestock management requires development and implementation of practices that safely utilize or dispose of manure without negatively impacting water quality.  Poor manure management can lead to fecal material entering waterways, with the resultant spread of pathogenic bacteria, viruses and excess nutrients. This not only threatens livestock, but can impact aquatic ecosystems and human health (Baxter-Potter, 1988; Jamieson, 2002; EPA, 2005). Offsite migration of fecal pollution can be exacerbated by liquid manure being applied beyond the nutrient needs of crops, or by applying it during periods of high precipitation, both of which are common occurrences (Harter, 2002).  One of the most notable examples of off-farm transport of fecal pollution is the outbreak of diarrheal disease in Walkerton, Ontario in May 2000, where several people died and many others became ill from drinking well water contaminated with E.coli O157:H7 from a cattle farm (Hudrey, 2003). Bacteria of concern in liquid manure may include Escherichia coli O157:H7 (Hancock, 1997), Campylobacter spp., Salmonella spp., and Mycobacterium spp, (McGarvey, 1997), along with viruses such as bovine enterovirus (BEV) (Ley, 2002; Jiménez-Clavero, 2005).

The management of fecal waste must comply with federal and state regulations, which are typically based on monitoring of the fecal indicator E.coli, in surface water at or near livestock facilities. The Federal Water Pollution Control Act, referred to as the Clean Water Act (CWA), sets out many of the requirements that protect water quality. The act is administered by the Environmental Protection Agency (EPA) but, in most instances, enforcement is delegated to the states, as is the case in Tennessee. Despite several decades of implementation of the CWA, many waterways across the United States fail to achieve federal water quality standards.  As recently as 2012, 25% of monitored stream miles in Tennessee failed because of pathogen impairment or elevated E.coli levels, and the primary suspected cause was agriculture (TDEC, 2012). Acceptable E.coli levels for surface water for recreational use are 126 CFU/100mls, based on the geometric mean of 5 samples over a maximum of 30 days. No single sample can be greater than 941 CFU/100mls or 487 CFU/100mls for a water body designated as an Outstanding National Resource Water (TDEC, 2013). For groundwater in any setting, agricultural or otherwise, Tennessee regulations are vague, stating that pollutant levels must not be detrimental to public health or impair usefulness of the groundwater. If groundwater discharges to a surface water body, it is then subject to surface water regulations for each pollutant. The most stringent regulations apply to drinking water standards, which are <1 CFU/100mls for E.coli in any sample (TDEC, 2013).

There is evidence that relying solely on E.coli may not be the optimum method for detecting livestock-related fecal pollution (Cook, 2011).  One concern is there are typically many sources of fecal contamination in a watershed (livestock, wildlife, humans, etc.), and E.coli generated from these sources is difficult to distinguish (Savichtcheva, 2007). If a fecal indicator has the ability to reproduce outside of the host, as E.coli does under some conditions, it may cause an over estimation of fecal pollution (Brennan, 2010). This has been observed in temperate soils where resident E.coli populations fluctuated throughout the year from <1 CFU/g of soil to over 3x10^3 CFU/g of soil (Ishii, 2006). E.coli concentrations in surface water are often influenced by seasonal and short-term variations in precipitation (Van Donsel, 1967). As a result of these factors it can be difficult to assess the impact of a specific livestock operation or manure management practice on water quality in a stream or other surface water body.

Using E.coli in conjunction with host-specific indicators may provide evidence of the impact of specific fecal contamination sources. In the case of dairy farms, bovine-specific fecal indicators may help determine which management practices on farms are having a significant impact on water quality (Jaffrezic, 2011; Scott, 2002). Fecal related Bacteroides are a well-studied alternative indicators to E.coli (Layton, 2006; Savichtcheva, 2007).  They are present in the intestines of mammals, constitute a large percentage of total fecal bacteria, have host specific strains (Bernhard, 2000), and have limited potential to reproduce outside the host (Kreader, 1998). Real-time polymerase chain reaction (PCR) assays targeting Bacteroides make it possible to quantify the amount of total Bacteroides (AllBac), and bovine specific Bacteroides (BoBac) in both surface and groundwater samples (Layton, 2006).

Chemical constituents of manure, such as nitrates, ammonia, and phosphorus, are also commonly used as indicators of fecal contamination at or near dairy farms (Birchall, 2008). Nitrates are common in surface and groundwater around agricultural activities and have an allowable maximum contaminant level of 10.0 mg/L (as total nitrates) for drinking water (EPA, 2008). Excess nitrates not sorbed by soil or taken up by plants in agricultural fields are easily transported in water, often leading to serious water quality issues (Ayebo, 2006).  In some dairy operations, leaching of nitrate, not E.coli, is the primary concern for water contamination, and may result from applications of liquid manure at rates that exceed the natural denitrification capability of soil and groundwater (Hubbard, 1998; Spalding, 1993). A statistically significant increase in nitrates in water around livestock facilities or manure applications, can often be used as an indicator of fecal pollution (Ward, 2005).

A herd of 200 dairy cows, typical of a small dairy in Tennessee, can produce as much as 10,900 kg per day of feces and urine (EPA, 2012). One approach to managing this large waste stream is to flush the dairy barn floor with water to remove the urine, feces, and other detritus, which is then collected in a pit or storage pond. The solids and liquid are then separated and the solid material is sold as a composting fertilizer. The liquids, or a combination of liquids and solids, are sprayed on cropland or pasture land as a fertilizer. The liquid portion is frequently less than 1% solids and is stored in holding tanks or in-ground ponds for 6-12 months before application to cropland. As liquid manure ages, the fecal coliforms and E.coli tend to die off, especially during the summer (Lorimor, 2004). It also undergoes a decrease in nitrogen, chemical oxygen demand, and total phosphorus (Kowalski, 2013; Martin, 2004), which reduces the environmental hazards related to manure application to cropland.

At a typical dairy, there are multiple pathways along which liquid manure can enter the aquatic environment and impair water quality. Storage tanks or in-ground ponds pose one of the greatest threats to nearby water quality due to their large volume and potential for leakage. There have been many instances where above ground tanks or ponds have failed, causing large spills, or long-term leakage (Flemming, 1999). For example, in February 2014, a spill at a 2200 cow dairy farm in Michigan caused by a malfunctioning valve on a multi-million gallon liquid manure storage tank fouled several miles of local streams (Dowling, 2014).  Application of liquid manure to fields can also result in contamination of streams. This can occur through surface runoff, during periods of very high precipitation. It can also occur during less extreme hydrologic conditions as a result of transport through drainage tiles or through discharge from shallow unconfined aquifers (Figure 1). Additionally, fecal pollution can be transported during spray applications as a bioaerosol, especially if applied during windy conditions. Management practices intended to reduce the likelihood of liquid manure transport through surface runoff and bioaerosols are commonly applied. These include restrictions on manure application during periods of high precipitation or winds, both of which are highly visible and predictable, at least over short time frames.

This thesis focuses on transport through drainage tiles and groundwater (figure 1), both of which are common in many row crop fields, and are a complicating factor in predicting transport of land-applied liquid manure. These tiles are installed because of inadequate existing soil drainage and are intended to make land more accessible for planting and harvesting. The tiles lower the water table and increase the pore space available for water to infiltrate instead of becoming surface runoff. However, the tiles create the potential for large amounts of nitrates, salts, pesticides, bacteria, and fertilizer to be discharged very rapidly (Wesstöm, 2002). In some cases, E.coli has been detected in drainage tiles only a few hours after an application of liquid manure. Others have documented increases in less than twenty-four hours (Cook, 2001; Dean, 1992). However, still other experiments have shown it is possible to safely apply manure to tiled fields without impacts on surrounding water (Randal, 2000; McGechan, 2000).

Many streams and ditches receive groundwater discharge (baseflow), which can contain manure constituents. In areas of rapid recharge and high hydraulic conductivity, fecal pollution may contaminate shallow aquifers and subsequently surface waters in gaining streams or ditches.  Transport along any these pathways is very complex and is controlled by a variety of environmental and anthropogenic factors, including soil type, depth to water table, topography, precipitation, vegetation, and seasonality (Dungan, 2010; Geohring, 1998). The relative importance of each transport pathways vary greatly between dairy farms and are poorly understood. Thus multiple different settings are needed to understand the role each plays so that generalized management practices can be developed and tested.

The main objective of this thesis is to determine whether groundwater discharge and/or drainage tile discharge are important pathways for transport of fecal contamination from land applications of liquid manure to surface water drainages at a typical dairy farm. Secondary objectives are to compare transport of E.coli and chemical indicators (nitrates, turbidity, etc.) with alternative fecal indicators, especially Bacteroides. A relatively simple hydrologic setting was chosen for the study, so that transport pathways could be readily identified and quantified. The site consists of low relief agricultural fields and shallow subsurface drains, leading to a central ditch which discharges into a nearby river. The fields are underlain by 2-6m of silt and sand, with a few gravel layers, above relatively low hydraulic conductivity bedrock. At least part of each year, groundwater from the shallow sediments discharges into the central ditch. Fecal transport by surface runoff is expected to be small, because of the low relief and because liquid manure is applied during seasonally dry periods. There is also a vegetated buffer (minimum 15 m wide) between the land application area and the central drainage ditch, which is expected to reduce fecal inputs from surface runoff. The main hypothesis for the study is that the drainage tiles will be the primary pathway, with groundwater secondary. Airborne transport is assumed to be negligible as a pathway to surface water at this site. Surface runoff is also expected to be minimal because of management controls against application of liquid manure applications during precipitation events and because of the buffer zone.

Figure 1: Pathways for fecal transport at a typical dairy farm.

# 2. Study Site and Previous Investigations

## 2.1 Study Site

This investigation was carried out at the Little River Animal and Environmental Unit (LRAEU) in Walland, TN, which is a 214 hectare dairy operated by the University of Tennessee. The dairy officially began operations on November 2nd 2011, with a primary focus on Holstein cow milk production. The dairy is located at the foot of the Smoky Mountains and is bounded on two sides by the Little River and on a third side by Ellejoy Creek (Figure 2). The proximity to the streams makes the dairy an area of interest for studying transport of fecal contaminants, especially since the upper portion of the Little River is listed as an Outstanding National Resource Waters (TDEC, 2012). The Little River is a source of recreation and drinking water for the City of Maryville and other communities in Blount County. The Little River upstream of the LRAEU is predominantly mountainous and rural. The Little River originates in the Great Smoky Mountains National Park (GSMNP), with its headwaters near Clingmans Dome. Along its course the Little River passes through the towns of Gatlinburg, Townsend, and Walland, Tennessee before reaching the LRAEU. The other creek bordering the LRAEU, Ellejoy Creek, runs through farmland and rural residential areas before joining the Little River. Ellejoy Creek is listed on Tennessee’s 303(d) list for impaired streams due to sediment and high levels of E.coli.

Figure 2: Site map of the LRAEU bounded in red.

## 2.2 LRAEU Regional and Local Geology and Hydrogeology

The LRAEU is located in the USGS Wildwood quadrangle, in an area classified as part of the Valley and Ridge physiographic province. It is situated at a point on the Little River that contains a catchment area of approximately 250 square miles. All outcropping rocks within the quadrangle are sedimentary in origin, and the depth to undeformed basement rock is approximately 4300 m. Major outcrops are composed of quartzite, sandstone, and shale, while most of the carbonates tend are covered with residuum or floodplain sediment. Several notable faults cut across the quadrangle, including the Dumplin Valley, the Wildwood Fault, Guess Creek, and the Great Smoky Fault (Neuman, 1960).

Approximately half of the 214 hectares LRAEU, including all of the cropland, sits on a low-relief flood plain adjacent to the Little River and Ellejoy Creek. The flood plain is made up of Pleistocene-aged alluvium. To the southeast of the flood plain are hills underlain by middle Ordovician sedimentary rocks of the Tellico Formation, which are made up of sandstone and shale. These materials tend to be highly weathered to depths of several meters or more (Neuman, 1960). The flood plain is mostly fine grained sand, with some silt and clay. There are also intermittent gravel layers. The alluvial deposits range from 3 to 9 m in thickness and are underlain by bedrock. About two-thirds of the floodplain portion of the LRAEU is underlain by the Blockhouse and Tellico Formation shales. Near the Little River, on the northwest edge of the property, the floodplain is underlain by the Lenoir Formation and lower Ordovician Knox group carbonates. At least three sinkholes have been documented in this area. One large sinkhole that opened near Ellejoy Creek Road, on the northwest corner of the property, was excavated, a geosynthetic liner installed, and backfilled with crushed rock in the fall of 2012 (Hunter, 2013).

When overland flow occurs, which is usually a rare event, a large portion of the cropland drains into the excavated central ditch.  The runoff catchment area is approximately 55 hectares at the LRAEU. The field to the east of the LRAEU is privately owned, and not studied, but based on the natural drainage that connects it to the LRAEU, it contributes approximately 11 hectares to the catchment area for the excavated drainage ditch. Although some of the predicted overland flow paths for the fields to the north of the central drainage are away from it, two of six drainage tiles (D1 & D2) run under those fields and channel water back towards the central drainage (Figure 3). The LRAEU contains two sets of artificial wetlands, one adjacent to the main road entering the property, and another on the backside of the property to the northeast of the liquid manure storage pit. Both sets of wetlands discharge to Ellejoy Creek and are not connected to the central drainage ditch.

The LRAEU has been the site of several investigations, including by two classes at the University of Tennessee and in a thesis by Robert Hunter. The class, Geology 586: Field Methods in Hydrogeology, instructed by Dr. Larry McKay, first worked on the site in 2007. This initial project was a characterization of the geology at the LRAEU and an identification of groundwater flow patterns. The final recommendation was the installation of ground wells on the site. In 2009 the Geology 586 class returned and installed, developed, and tested 15 groundwater wells, which was the first groundwater monitoring at the site. The 2009 class report recommended installing more monitoring wells. The most comprehensive analysis of the hydrogeology of the LREAU was carried out by Robert Hunter for his thesis, “Hydrogeology of the Little River Animal Agriculture Environmental Research Unit and Impacts of Dairy Operations on Groundwater” in 2013.  Mr. Hunter determined that the alluvial sediments of the floodplain, upon which the LRAEU is built, are poorly sorted. Based on his results, the alluvium is mostly silt and sand, with a small amount of clay. An extensive gravel layer was also observed along the central drainage, approximately 0.5 to 2 m thick.

Mr. Hunter determined that infiltration from the ground surface to the wells is rapid and occurs within a few hours to days after a precipitation event. Hydraulic conductivity values measured in wells located near the central drainage ditch were as follows: well 6 1.2E-6 m/s, well 5A and 5B having the same hydraulic conductivity of 1.75E-5 m/s, and well 10B having a value of 1.0E-5 m/s (Hunter, 2013). See appendix A for bore logs of the wells used. Flow of groundwater is expected to be predominately horizontal in the alluvium because of the much lower hydraulic conductivity expected in the shale bedrock compared to the alluvium. Also of relevance to this study, Hunter found changes in specific conductance in the Little River along Old Walland Highway was likely from a spring connected to a sinkhole located near western edge of the property near the intersection of Ellejoy Creek Road and Old Walland Highway. The sinkhole was filled with crushed rock in 012, but still is expected to act as a pathway for groundwater discharge to the river.

The depth to groundwater in the floodplain varies depending on the season and amount of precipitation. The mean depth to water table tends to be the shallowest in the interior of the property, and greatest near the property boundaries, where there are steep slopes down to the Little River and Ellejoy Creek (Figure 4) (Hunter, 2013). The hydraulic gradient running from W6 to W10b, is roughly in line with the central drainage and runs the entire length of the study area (Figure 5). The mean depth to water at W6 is 0.34 m and 3.31 m at W10b. Depending on the season and precipitation, the central drainage is expected to act as either a gaining stream, receiving recharge directly from groundwater on either side, or a losing stream (Figure 6). During low precipitation periods the water level in the central drainage ditch can be higher than the water table and lose water to the water table. The shale bedrock is expected to have much lower hydraulic conductivity than the alluvium, so most of the recharge and groundwater flow in areas underlain by shale is expected to occur in the alluvium. However, in the carbonate bedrock there is likely substantial groundwater flow, especially near the sinkhole.

Figure 3: Projected flow direction and surface runoff divide.  Elevation contours are 0.1524m increments (0.5 ft).

Figure 4: Average elevation of water table in selected wells at the LRAEU and, in brackets, average depth to water table (Hunter, 2013).

Figure 5: A cross sectional diagram of the study site at the LRAEU. Reprinted with permission from Robert Hunter.

Figure 6: Generalized cross section of a gaining and losing streams.

## 2.3 Previous Water Quality Monitoring

Monitoring of surface water quality at the LRAEU began in May 2005, when the University of Tennessee AgResearch staff members set up stations at nine locations, including sites in the central drainage ditch, the crop fields, the Little River, and Ellejoy Creek. Each point is/was (monitoring of some locations has changed over time) sampled with a Teledyne ISCO Portable Sampler. The sampler either collects water samples at set time intervals or collects samples based on water flow conditions. The individual water samples are aggregated in a 2.5 gallon Nalgene bottle, which is periodically retrieved by a research associate. All samples collected by UT AgResearch are submitted to the BESS Water Quality Laboratory for analysis, which includes E.coli, total solids (ppm), biochemical oxygen demand, total nitrogen (ppm), chlorine (ppm), nitrate (ppm), and nitrite (ppm). In addition to surface water monitoring, air sampling at the LRAEU began in September 2008 for ammonia, nitrous oxide, carbon dioxide, methane, and water vapor. Since 2006  a weather station was added by the UT AgResearch that collects meteorological data on wind speed, wind direction, rainfall, soil temperature ( at a depth of 15.2 cm ), humidity, incident sunlight, and air temperature.

Prior to the LREAEU being established on its current site, the flood plain portion of the property was managed by a tenant farmer, who applied commercial fertilizer to the cropland. No data exists on application rates or type of fertilizer. Thus, the site is not “pristine”, but there were no cows on site or liquid animal waste being applied as a fertilizer, which marks a major shift in site management.

The first time liquid manure was applied was in late spring 2012 at a rate of 150,000 liters per hectare. The second manure application occurred in the fall of 2012, at a similar application rate. Neither of these applications is included in this thesis, but preliminary data on the first application was studied by Mr. Hunter in his 2013 thesis.

The liquid manure applications analyzed in this thesis occurred  May 29th to 31st of 2013, November 4th to 8th of 2013, and May 21st to 24th of 2014. Both applications in 2013 involved applying an average of 140,000 liters per hectare across approximately 81 hectares, while the application in the spring of 2014 was 178,000 liters per hectare across approximately 81 hectares. The liquid manure was generated on site by the Holstein cows, and stored in an above ground holding tank, after having some solids separated from the liquids. In these applications, liquid manure was pumped directly from a large above ground tank, with a volume maximum capacity of seventy-five million liters, via a hose to a spreader pulled by a tractor.

In groundwater sampling by Mr. Hunter, positive results for E.coli  were found in every well sampled at the LRAEU at least once, with results ranging from 1 to 23 MPN/100mls. The wells that were the most impacted by fertilizer and manure applications during Mr. Hunter’s study were the wells closest to the central drainage (W10B, W5A, W5B), and those along the edge of the property due to the increase in hydraulic gradient to the Little River and Ellejoy Creek. Since there was an observed impact in the wells, it appeared there was a need to better understand the transport pathways (drainage tiles, groundwater, surface runoff, etc.). Mr. Hunter’s analysis also concluded that a small incremental increase in nutrients in the groundwater occurred after cows arrived and after crop cultivation began. This conclusion was based on the fact background water samples at the LRAEU were similar to other groundwater parameters in the southeast of the United States, but after operations began at the dairy small increases in nutrients occurred.

# 3. Methods

## 3.1 Sampling Plan

Water samples for this thesis were collected from the central drainage ditch, wells near the ditch, drainage tiles, and two sites along the Little River (Figure 7). Samples were collected over a period of 14 months, from April 2013 through June 2014 (Table 1). This time period included three separate manure application events. During each application event, the manure was sampled 3-5 times. Between manure applications, water samples were typically collected every 2-4 weeks. During and immediately after manure applications sampling was increased to 2-3 samples per week to capture possible impacts on water quality. Within a few weeks after the manure application was completed, frequency of sampling returned to the regular schedule. During the time frame of this thesis, water samples were collected on 36 days during the 424 days covered. Parameters measured during sample collection or in the laboratory are listed in Table 2.

A total of four wells were sampled in this thesis. These are abbreviated W5A, W5B, W6, and W10B. Locations are shown on Figure 7 and photographs of the wells are shown on Figure 8. W6 is the furthest from the excavated portion of the drainage ditch at 522 m, but is 163 m from a seasonally wet area and the natural drainage that occasionally flows into the excavated drainage ditch during high precipitation events. The well was drilled to bedrock, which was encountered at 2.6 m and has a screened interval and sand pack of 1.8 m to 2.6 m. W5A is 32 m from the excavated drainage ditch and is located in a small (≈5 m x 8 m) non-cultivated area within the crop fields. The well is 2.6 m deep, and is screened in alluvial deposits from a depth of 1.8 m to 2.6 m. Sampling site W5B is adjacent to W5A in the non-cultivated area in the crop field. It is 33 m away from the excavated drainage ditch and is screened from a depth of 3.2 m to 4.0 m. During drilling, bedrock was encountered at 4.7 m below ground surface.  Sampling site W10B is 7.6 m from the excavated drainage ditch and is located in the non-cultivated zone (buffer) around the drainage ditch. W10B is the deepest well at 5.2 m and is screened from a depth from 4.4 m to the bottom, which is at the top of bedrock (Table 3).

The first two drainage tile sampling points (D1 & D2) are located at the upstream end of the excavated portion of the central drainage. D1 has a diameter of 15 cm, and D2 has a diameter of 30 cm. The next sampling point, D3, is 49 m downstream. The D4 sampling point is 60 m downstream, followed by D5 at 217 m, and D6 at 657 m. Locations of the drainage tile sampling points are shown in Figure 7 and photographs are shown in Figure 9.

Three sampling points were selected in the excavated portion of the central drainage ditch. Locations are shown on Figure 7 and photographs are shown on Figure 10. The first sampling point is 19 m from the beginning of the excavated portion of the drainage ditch, which was measured from sampling site D2. The second sampling point, S2, is 409 m from the beginning of the ditch and is located just before a sharp bend in the drainage ditch.  Drainage ditch site S3 is 954 m from the beginning of the drainage. S3 is 54 m from the discharge of the ditch to the Little River. The total length of the excavated portion of the surface drainage ditch is approximately 1008 m.

Two sampling locations are located in the Little River on the opposite side of Ellejoy Creek Road from the LRAEU. The first sampling location, LR1, is 12 m upstream of the discharge of the drainage ditch. The second sampling site, LR2, is 191 m downstream of the discharge point. Locations are shown on Figure 7 and photographs are shown on Figure 11. Between the discharge of the drainage ditch and LR2 there are no discharge tiles, culverts, and/or streams entering the Little River.

LR1

LR2

S3

W10B

S2

S1

D6

D5

D4

D3

D1 & D2

W5A & W5B

W6

Figure 7: Sampling locations at the LRAEU.

Table 1: Sampling and manure application dates at the LRAEU.

| Date | Manure Application | Water Sampling |  | Date | Manure Application | Water Sampling |
| --- | --- | --- | --- | --- | --- | --- |
| 4/13/2013 |  | X |  | 12/11/2013 |  | X |
| 5/9/2013 |  | X |  | 1/12/2014 |  | X |
| 5/14/2013 |  | X |  | 2/8/2014 |  | X |
| 5/29/2013 | X | X |  | 2/23/2014 |  | X |
| 5/30/2013 | X | X |  | 3/1/2014 |  | X |
| 5/31/2013 | X |  |  | 3/11/2014 |  | X |
| 6/8/2013 |  | X |  | 4/6/2014 |  | X |
| 6/16/2013 |  | X |  | 4/21/2014 |  | X |
| 6/21/2013 |  | X |  | 5/7/2014 |  | X |
| 6/30/2013 |  | X |  | 5/14/2014 |  | X |
| 7/13/2013 |  | X |  | 5/21/2014 | X | X |
| 7/20/2013 |  | X |  | 5/22/2014 | X |  |
| 8/4/2013 |  | X |  | 5/23/2014 | X | X |
| 8/15/2013 |  | X |  | 5/24/2014 | X |  |
| 8/20/2013 |  | X |  | 5/26/2014 |  | X |
| 9/14/2013 |  | X |  | 5/29/2014 |  | X |
| 10/17/2013 |  | X |  | 6/4/2014 |  | X |
| 11/2/2013 |  | X |  | 6/11/2014 |  | X |
| 11/4/2013 | X |  |  |  |  |  |
| 11/5/2013 | X | X |  |  |  |  |
| 11/6/2013 | X |  |  |  |  |  |
| 11/7/2013 | X |  |  |  |  |  |
| 11/8/2013 |  | X |  |  |  |  |
| 11/13/2013 |  | X |  |  |  |  |
| 11/17/2013 |  | X |  |  |  |  |
| 11/22/2013 |  | X |  |  |  |  |

Table 2: Chemical and physical properties measured for manure and water samples.

| Parameter | Units | Manure Samples | Water Samples |
| --- | --- | --- | --- |
| Date | D/M/YR |  | X |
| Total Coliforms | MPN/100ml |  | X |
| E.coli | MPN/100ml |  | X |
| Bacteroides (AllBac) | copies/100ml |  | X |
| Bacteroides(BoBac) | copies/100ml |  | X |
| Bovine Enterovirus PS 87 | copies/100ml |  | X |
| Conductivity | µs |  | X |
| Resistivity | KΩ |  | X |
| Total Dissolved Solids | Ppm |  | X |
| Oxidation Reduction Potential | Mv |  | X |
| pH |  |  | X |
| Turbidity | Ntu |  | X |
| Temperature (Ground Water) | Co |  | X |
| Temperature (Atmospheric) | Co |  | X |
| Depth to Water Table | M |  | X |
| Nitrates | mg/L |  | X |
| Precipitation | Mm |  | X |
| Discharge | L/min |  | X |
| Electrical Conductivity | umhos/cm | X |  |
| Total Nitrogen | mg/L | X |  |
| Total Phosphorous | mg/L | X |  |
| Total Potassium | mg/L | X |  |
| Total Calcium | mg/L | X |  |
| Ammonia-N | mg/L | X |  |
| Nitrate-N | mg/L | X |  |
| Total Magnesium | mg/L | X |  |
| Total Sulfur | mg/L | X |  |
| Total Iron | mg/L | X |  |
| Total Manganese | mg/L | X |  |
| Total Zinc | mg/L | X |  |
| Total Copper | mg/L | X |  |
| Water Extractable Phosphorus | mg/L | X |  |

Table 3: Descriptive details on wells sampled.

| Location Abbreviation | Location Type | Elevation (m) | Screen Depth Bottom (m) | Screen Depth Top (m) | Geologic Material at Screen Interval | Distance from Natural Surface Drainage (m) |
| --- | --- | --- | --- | --- | --- | --- |
| W5A | Well | 271.261 | 2.6 | 1.8 | silt with gravel | 31.9 |
| W5B | Well | 271.335 | 4.0 | 3.2 | fine sand, silt | 33.3 |
| W6 | Well | 271.826 | 2.6 | 1.8 | silty/fine sand | 163* |
| W10B | Well | 269.472 | 5.2 | 4.4 | silty/fine sand | 7.6 |

*W6 is located near a seasonally wet area that drains towards the excavated central drainage ditch, located approximately 500m away.

Figure 8: Images of wells sampled at the LRAEU.

Figure 9: Images of drainage tiles sampled at the LRAEU.

Figure 10: Images of surface drainage ditch sampled at LRAEU. Site S3 was photographed during dry conditions.

Figure 11: Images of Little River sampling locations at the LRAEU.

Water samples were collected in autoclaved Nalgene bottles and pre-sterilized Corning 50 ml centrifuge tubes. Prior to being autoclaved, Nalgene bottles were washed, scrubbed, and then rinsed with deionized water and allowed to dry. For groundwater samples, 1000 mls of water were collected for Bacteroides (occasionally, high turbidity made filtering this volume impossible), 100 mls of water in a separate Nalgene bottle for coliforms and E.coli, and 50 mls of water in a Corning centrifuge tube for total nitrates. The Little River and surface drainage had 500mls of water collected for Bacteroides in a Nalgene bottle, with a goal of filtering 300 mls of water. A separate 100 mls of water was collected for coliforms and E.coli, and 50 mls of water collected in a Corning centrifuge tube for total nitrates. Drainage tiles had 100 mls of water collected for coliforms and E.coli, and 50 mls of water in a Corning centrifuge tube for total nitrates. Drainage tile samples were not analyzed for Bacteroides.

Surface water samples were collected following the USGS protocol for fecal indicator sampling. An opened autoclaved bottle was plunged mouth-down into the water. The bottle was then slowly rotated so that it filled. The bottle was then removed with the mouth pointing upward. Once removed the bottles were capped and packed in ice (Myers, 2010).

Prior to sampling the wells, the water-tight well caps were removed and water levels were measured after 5-10 minutes. This allowed the water level in the well to equilibrate with the atmosphere. Starting in May of 2014, water level elevations were also measured in the nearby ditch relative to steel stakes that had been driven into the sediment at two sites, one near wells W5A and W5B, and the other near well W5B. The top of the stakes were surveyed, so water level elevations in the ditch could be compared to water level elevations in the nearby wells to determine whether the ditch was gaining or losing water to the aquifer at that location.

A Proactive 12V Typhoon submersible pump was used to purge and sample the wells. Water was pumped until the geochemical parameters stabilized, with special attention paid to the stabilization of pH and turbidity. Stabilization was defined as a change of +/- 0.1 standard units and a change of turbidity +/- 10.0 NTU over multiple readings. This act of purging was performed to remove stagnant water in the well that would not be representative of the surrounding aquifer (Nielsen, 1991). Between sampling events a 10% bleach solution was circulated through the pumps for 30 minutes to sterilize the tubing and pump, and then deionized water was circulated for 15 minutes to remove the bleach. The cord and tubing was wiped with a 10% bleach solution and then rinsed, before sampling the next well.

Purging time, flow rates, and volume purged varied widely depending on hydraulic conductivity of each well and the water level. When the depth to the water table was shallow, wells could be pumped at higher rates without drawing air into the pump intake, which was typically set at 1-2 m above the bottom of the well. When depth to the water level was relatively large for the site, pumping rates had to be diminished so wells would not be completely emptied. Well W6 was pumped at a rate ranging from 0.8L/min to 0.05 L/min. Well W5A was pumped at rates ranging from 3.3 L/min to 0.2 L/min. Well W5B ranged from 3.5 L/min to 0.3 L/min. Well W10B pumping rates ranged from 1.0 L/min to 0.1 L/min. During times of extremely low pumping rates, the tubing connected to the pump was covered with a white painter’s tarp to reduce direct sun exposure and decrease solar heating of the water to be sampled. Volume pumped to stabilize parameters, especially pH, was as high as 40 L when sampling occurred shortly after rain events, but more often was approximately 20 L for the deepest wells, and 10 L for the shallowest.

Liquid manure samples were collected on each day of land spreading by the staff of the LRAEU and shipped to the University of Arkansas Agricultural Diagnostic Laboratory in Fayetteville, Arkansas for chemical analysis. The chemical properties tested were pH, electric conductivity, percent solids, total nitrogen, total phosphorus, total potassium, total calcium, total ammonium, total nitrate, total iron, total manganese, total zinc, total copper, and water extractable phosphorus. The samples were typically collected in the morning and afternoon of each day of land spreading to understand the heterogeneity of the manure application. Additional 50ml samples of liquid manure were collected in sterile Corning (Corning, NY) centrifuge tubes and stored at -20o C. These samples were analyzed at UTK to quantify Bacteroides concentrations and determine the taxonomic structure of the microbial community.

## 3.2 Water Quality Testing

### 3.2.1 Field Parameter Measurements

All water samples were tested in the field for conductivity, resistivity, total dissolved solids, oxidation reduction potential, pH, and temperature using a Hach (Loveland, Colorado) MP6 portable meter. Turbidity was measured with a Hach portable turbidimeter. All of these parameters were measured and recorded at the time of sampling.

### 3.2.2 Nitrates

Total nitrates (NO3-) were measured in all water samples in the laboratory using a cadmium reduction powder pillow method with a range of 0.3 to 30.0 mg/L as quantified on a Hach DR 3900 spectrophotometer. The method has a precision of +/- 0.3 mg/L. Liquid waste and other materials that came in contact with the contents of the powder pillow were properly disposed of as hazardous waste.

### 3.2.3 Coliforms and E.coli

Total coliforms and E.coli concentrations were measured using the EPA approved IDEXX Quanti-Tray/2000 method (IDEXX Laboratories, Westbrook, ME) (EPA, 2007). Water samples collected for coliforms and E.coli analyzes were carefully packed in ice and processed in the lab in less than 6 hours from the time of collection. Use of the Quanti-Tray system followed the manufacture’s protocol.

For quantification of coliforms, the number of yellow cells (large and small) was recorded. An ultraviolet light was then used to determine the ad number of cells that were both yellow and fluoresced, which represented E.coli positive cells. The most probable number (MPN)/100mls were calculated using the IDEXX laboratories MPN Generator Version 3.2 software. The MPN is generated from a statistical test, also known as the Poisson zeros. The IDEXX method has a range of <1.0 MPN/100mls to > 2419.6 MPN/100mls. Once results were recorded the trays were autoclaved to kill all cultured bacteria.

### 3.2.4 DNA Extraction and Bacteroides Analysis

For molecular analysis of Bacteroides, 1000 mls of water from wells, and 300 mls from all other sites, were filtered through a sterile 0.22 μm nitrocellulose filters using a 47 mm Millipore glass vacuum system. Occasionally high turbidity values made it impossible to filter the desired volume, in which case the number of milliliters filtered was recorded. The filters were put in petri dishes, wrapped in parafilm, and stored at -20 oC until DNA was extracted.

DNA extractions were performed using a MP Biomedical Fast DNA Spin Kit for Soil (MP Biomedical, Santa Anna, CA). To extract DNA a quarter of a filter was cut into several small pieces using a sterile razor blade and placed into a Lysing Matrix E tube containing ceramic and silica particles with 978 µl of sodium phosphate buffer and 122 μL of MT soil lysis buffer. A MP Biomedical FastPrep Instrument set for 40 seconds on a setting of 6.0 was used to break down the bacterial cells and filter pieces. The tubes were centrifuged at 14,000 x g for 10 minutes and the supernatant was transferred to a sterile 2.0 ml microcentrifuge tube with 250 μL of protein precipitation solution (PPS). The microcentrifuge tube was then inverted by hand several times. Each tube was then placed in the centrifuge for 14,000 x g for 5 minutes and the supernatant transferred to a 15 ml tube with 1.0 ml of binding matrix. The 15 ml tubes were inverted by hand for 2 minutes to allow binding of DNA, and placed in a rack for 3 minutes to allow for the settling of silica. A total of 650μL of supernatant was removed and discarded. Next, the remaining mixture was shaken and 650μL transferred to a SPIN ™ Filter (MP Biomedical, Santa Anna, CA) and centrifuged at 14,000 x g for 1 minute. The catch tube was then emptied and the remaining supernatant added and centrifuged again at 14,000 x g for 1 minute. Then 500 μL of SEWS-M was added to resuspend the pellet in the SPIN Filter. The mixture was centrifuged at 14,000 x g for 1 minute and the catch tube emptied and replaced, and centrifuged at 14,000 x g for 2 minutes. The catch tube was discarded and replaced with a clean catch tube. The SPIN ™ Filter was then air dried for 7 minutes. Finally 50 μL of DNase/Pyrogen-Free Water (DES) was added and centrifuged at 14,000 x g for 1 minute. The final product was then either immediately cleaned and concentrated, or stored at -80o C and cleaned later.

Because of impurities in the DNA samples it was necessary to clean and concentrate the DNA to limit PCR inhibition. This was done using a Zymo Research Genomic DNA Clean & Concentrator kit (Zymo Research, Irvine, CA). ChIP DNA Binding Buffer was added to the sample in a microcentrifuge tube at a rate of 2:1 and mixed. The sample was then added to a Zymo-Spin IC-XL Column in a collection tube and centrifuged for 30 seconds and the flow through discarded. Next 200 μL of DNA Wash Buffer was added to the column and centrifuged for 1 minute. The flow through was discarded and the step repeated with 200 μL of DNA Wash Buffer. Last, the column matrix had 50μL of HPLC water added and centrifuged for 30 seconds. The final product was quantified using a NanoDrop Spectrophotmeter (Thermo Scientific, Waltham, MA) to obtain 260/280, 260/230, and ng/μL readings. The samples were then stored at -80oC.

Two previously developed quantitative PCR assays were used to target specific Bacteroides gene sequences; all Bacteroides species, AllBac, and the second was bovine-associated Bacteroides, BoBac (Table 4) (Layton, 2006). For each real time-PCR run each well of the plate contained 12.5 μL of Absolute Blue Master Mix, 0.75 μL of the forward primer, 0.75 μL of the reverse primer, 0.50 μL of the fluorescent probe, 10.0 μL of HPLC water, and 2.5 μL of each sample to be analyzed. Starting in January 2014 the use of Absolute Blue was discontinued and 5 PRIME HotMasterMix was used as a replacement. Samples were placed in triplicate in a 96 well plate, along with standards ranging from 107 to 101 copies plasmid standard and a blank. For each sample a fourth well was spiked with a 106 copies standard to determine PCR inhibition. The Bacteroides assay involved annealing  at 50 Co for 2 minutes, 95 Co for 10 minutes (5 minutes for BoBac), up to 50 repetitions at 95 Co for 30 seconds and 60 Co for 45 seconds, with a plate reading before each repetition. Results from the manure were obtained as copies per well on the plate. Results were then standardized across multiple runs using combined standard curves generated from the C(T) values for both AllBac and BoBac assays. Adjusted concentrations were converted to copies/100mls to be more comparable with E.coli data. The equations to adjust the AllBac and BoBac values were obtained by averaging the slope and x-intercept for the standard curves generated in each run, respectively, that were used (Table 5). Standard curves from PCR runs were considered acceptable if their C(T) values for the 10^7 JC-5 standard were between 14 and 18.

Table 4: Target organisms and primer/probe gene sequences.

| Target Organism | Primer/Probe Sequence |  | Reference |
| --- | --- | --- | --- |
| AllBac | Forward Primer | 5'-GAGAGGAAGGTCCCCCAC-3' | Layton, 2006 |
|  | Reverse Primer | 5'-CGCTACTTGGCTGGTTCAG-3' |  |
|  | Probe | 5'-(FAM)CCATTGACCAATATTCCTCACTGCTGCCT(BHQ-1)-3' |  |
| BoBac | Forward Primer | 5'-GAAG(G/A)CTGAACCAGCCAAGTA-3' |  |
|  | Reverse Primer | 5'-GCTTATTCATACGGTACATACAAG-3' |  |
|  | Probe | 5'-(FAM)TGAAGGATGAAGGTTCTATGGATTGTAAACTT(BHQ-1)-3' |  |

AllBac Copies= 10(-0.275*Ct+11.26)

BoBac Copies= 10(-0.25*Ct+11.40)

Equation 1: Equations to standardize AllBac and BoBac copies based on average slope and x-intercept from all used runs.

### 3.2.5 Analysis of Microbial Community Structure

DNA extractions from field samples of liquid manure was performed using 250 µls in place of the filters outlined in section 3.2.4. After adding the 250 µls, the extraction protocol and clean and concentrate protocol does not differ from section 3.2.4. To amplify the V4 region of the 16S rRNA a barcode primer, positive control, and a negative control were added to the sample. The master mix was created with 33µl HPLC water, 5µl pfx50 buffer (Life Technologies, Carlsbad, CA), 1µl Pfx Taq polymerase, 1µl 515f, 1µl dntp 10mM mix, and 0.5µl DM50. After creating the master mix, 40µl was placed into each tube, with 10µl of DNA extracted from liquid manure samples, and 1µl of reverse 806r barcode primer. The thermal cycler program ran for 3 minutes at 94Co once, followed by 35 cycles of 94 Co for 45 seconds, 55C for 1minute, and 72 Co for 1.5 minutes. After completing the 35 cycles, the program ran at 72 Co for 10 minutes and held at 4 Co indefinitely.

The samples were analyzed using a DNA 7500 kit gene chip (Agilent Technologies, Santa Clara, CA) to verify correct product size ~350bp. Samples with correct product size were purified using solid phase reversible immobilization (SPRI) cleanup to remove residual primers. PCR amplicoms were pooled with approximately 40 other samples in equal concentration. Kapa PCR was used to quantify pooled library concentration. Pooled amplicoms were sequenced on a MiSeq Sequencer (Illumina Inc., San Diego, CA) using 2x150bp cartridge following the protocol described by Caporoso et al (Caporoso, 2012). Sequences were demultiplexed on the MiSeq instrument to generate Read 1 and Read 2 fast q files. Read 1 and Read 2 files were uploaded, joined, and annotated through the MG-RAST pipeline to provide taxonomic information (Meyer, 2008).

3.4 Statistical Analysis

All data collected was organized in Microsoft Excel 2013 and imported into SAS 9.4 for statistical analysis. Statistical tests were conducted with an α value of 0.05 to be considered significant. Biological data and other water quality parameters were analyzed for a Pearson product-moment correlation coefficient in SAS 9.4 both on a global scale and on a site specific level.  Analysis of variance testing was performed using the PROC glm procedure in SAS. If significant relationships were found, Tukey’s studentized range test was used to test differences in mean values. When appropriate a Shapiro-Wilk test for normality and Durbin-Watson statistic for autocorrelation of the residuals were also run and the results compiled in appendix B.

# 4. Results

## 4.1 Hydrologic Monitoring

### 4.1.1 Precipitation

The first manure application (May 29th to 31st) occurred during a relatively wet spring season. Monthly precipitation in April and June of 2013 were approximately 80 mm higher than the historic average, although the value for May was just below the average (Figure 12). In the 28 days before the start of the liquid manure application there were 11 measureable rain events with a range of 0.76 mm to 25.4 mm. In the 28 days following the start of the liquid manure application there were 10 measureable precipitation events with a range of 0.5 mm to 44.7 mm. The closest precipitation events to the manure application occurred two days prior and two days after completion (Figure 13).

The second manure application occurred in early November 2011 (Nov. 4th to 7th) and was preceded by an abnormally dry October, which was approximately 54mm below the historic average. November precipitation was only slightly greater than the average, but the overall period around the manure application was dry (Figure 12). There were 2 measurable events in the 28 days leading up to the second liquid manure application with a range of 9.4 mm to 15 mm. Following the liquid manure application there were 8 precipitation events with a range of 2 mm to 56.6 mm. During the manure application there was 1 precipitation event of 10.2 mm, but application was postponed during the event. A precipitation event occurred 3 days before the application and another occurred 7 days after the completion of the application (Figure 14).

The third manure application occurred in late May 2014  (May 21st to 24th) , which was preceded by a wetter than average April by approximately 20 mm. May was drier than average by almost 79 mm. June began as a dry month, but data collection was only for a portion of the month (Figure 12). There were 10 precipitation events ranging 0.3 mm to 16 mm during the 28 day window leading up to the liquid manure application. One small precipitation event occurred during the liquid manure application of 2.8 mm. There were 8 precipitation events following the completion of the manure application and ranged from 2 mm to 10.8 mm (Figure 15). A precipitation even occurred 5 days before the manure application, and another occurred 1 day after the completion of the event.

Figure 12: Cumulative precipitation by month over the course of the study along with the cumulative historical averages by month from the weather station at Tyson McGee airport. Manure Applications occurred in May of 2013, November of 2013, and May of 2014.

Figure 13: Precipitation during the 28 day period preceding and following the start of the first liquid manure application.

Figure 14: Precipitation during the 28 day period preceding and following the start of the second liquid manure application.

Figure 15: Precipitation during the 28 day period preceding and following the start of the third liquid manure application.

### 4.1.2 Ground Water Levels

During the first liquid manure application the depth to the water table, measured with an electric water level tape, was shallower across all the sites than during the subsequent manure applications. This reflects the greater than average precipitation observed in spring of 2013. Notably, water levels in W6 were close to the surface and actually rose to the top of the well casing for part of spring 2013. The water level in W6 declined over 1 m during the summer and fall before the second liquid manure application occurred. During the winter the water level in W6 rose again to at or near the top of the casing, and declined again throughout the spring of 2014 (Figure 16). Similar observations were observed in measurements of depth to water table with the pressure transducers in W6 (Figure 17).

Depth to water level in W5A and W5B varied from 0.7 to 1.7 m and followed the same trend throughout the study as shown in figure 16. The values measured with a water level tape consistently showed W5A to be slightly shallower than W5B, but the inverse was shown with the pressure transducers.  As with W6, W5A and W5B both had the shallowest depth to the water table during the first manure application, and were deeper in the subsequent two applications. The water table declined in both wells during the summer and fall of 2013, and then rose in the winter 2013/2014, and declined again during the spring of 2014.

Well W10B had the deepest level to the water table throughout the entire study, ranging from 2.3 to 3.5 m, but exhibited the same seasonal trends as the other wells. The shallowest depth to groundwater in W10B was observed during the first manure application, while the second and third were approximately half a meter deeper.

Figure 16: Depth to groundwater manually measured on each day of sampling at the four monitored wells. Daily precipitation totals are shown with blue bars.

Figure 17: Depth to groundwater measured by the pressure transducers placed into monitoring wells with the liquid manure applications shown in the grey columns.

### 4.1.3 Gaining and Losing Status of the Drainage Ditch

Starting on 5/14/2014, the elevation of the surface water in the ditch at locations near well pair W5A/W5B and W10B was surveyed. At the same time, the elevation of the water level in the nearby wells was measured. The top of the well casings have a known elevation. On 5/14/2014, 5/21/2014, 5/23/2014, and 5/26/2014 the water level elevations in the drainage ditch, approximately 150 m from the start, was lower than the water level elevations in W5A and W5B. This indicates that the ditch near W5A and W5B (and likely the entire upstream portion of the ditch) was acting as a “gaining stream” (Table 6).

Well W10B on 5/14/2014, 5/21/2014, 5/23/2014, and 5/26/2014 had a water elevation that was lower than the adjacent drainage ditch, which was approximately 50 m from the discharge to the Little River. This indicates that the ditch near W10B was acting as a “losing stream”. Measurement in the ditch adjacent to W10B were not possible after 5/26/14 because there was no water in the drainage ditch from the discharge to the Little River and going upstream approximately 125 m.

Table 6: Depth to water and elevations in wells adjacent to the drainage ditch along with the elevation of the water in the drainage ditch at surveyed markers.

## 4.2 Liquid Manure Chemical and Taxonomic Structure

The first two liquid manure application had samples sent for analysis by the staff of the LRAEU. Liquid manure composition did not change dramatically in most respects, other than electrical conductivity, which increased 796 umhos/cm from the first to second application. From the first liquid manure application to the second, there was an increase in electrical conductivity, ammonia-nitrogen, total magnesium, and AllBac. All other chemical and biological factors decreased. Over the course of the liquid manure applications, the percentage of AllBac attributable to BoBac decreased from 18% in the first application, 12% in the second application, and 8% in the third application (Table 7).

A sample of liquid manure from the first application was further analyzed for its microbial community.  Out of a total sequence count of 203,447,16% were Eubacterium. The target of the AllBac assay covers approximately 14% of the total sequence count, which includes Bacteroides (7.95%), and Prevotella (6.52%). In addition to the ten most abundant genera present in the liquid manure Eschirichia, the genus to which E.coli belongs, was 0.02% of the total sequence counts (Table 8). Figure 18 has percentages rounded to the nearest whole number, but shows the phylum, class, order family, and genus percentages present for all bacteria in the liquid manure sample.

Table 7: Mean chemical analysis data and Bacteroides concentrations in liquid manure from each application

\

Table 8: Microbial community relative abundance and the ten most abundant genera, plus Escherichia.

| Manure Application 1 |  |
| --- | --- |
| bp Count | Sequence Count |
| 51436155 | 203447 |
| Bacteria (Genus) | Relative Abundance (%) |
| Eubacterium | 15.88 |
| Bacteroides | 7.95 |
| Prevotella | 6.52 |
| unclassified (derived from Bacteria) | 6.45 |
| Clostridium | 5.24 |
| Coptotermes | 3.54 |
| Blattabacterium | 2.99 |
| Treponema | 2.83 |
| Barnesiella | 2.64 |
| Porphyromonas | 2.50 |
| Escherichia | 0.02 |

Figure 18: Taxonomic composition of bacteria sequences identified in the liquid manure used in the first application at the LRAEU. Figure created using data uploaded into MG-RAST and graphic created by Krona.

## 4.3 Data and Simple Statistics

Five samples exceeded the maximum detection limit of 2419. 6 CFU/100 mls for E.coli during the study, and all instances were in surface water or drainage tiles. The first was on 5/30/2013 and 3 more on 6/3/2013. Both dates fall during or immediately after the first liquid manure application. Additionally, 56% of sites sampled on 5/30/2013 exceeded 126 MPN/100mls, and 27% of samples exceeded the regulatory single sample maximum for recreational waters of 941 MPN/100mls. On 6/3/2013, 91% of sites exceeded 126 MPN/100mls and 36% exceeded the maximum regulatory limit for recreational waters. The fifth instance on 10/17/2013 was not during or immediately after any manure application. After this date only 3 samples exceeded the 941 MPN/100mls maximum single sample regulatory limit (Table 9).

Wells exceeded the drinking water standard for E.coli in approximately 41% of samples collected over the course of the study. Wells exceeded the surface water quality standard of 126 MPN/100 mls in four samples. Two of these samples followed the first liquid manure application. The third sample was two days after the second liquid manure application was completed, and the final instance was 108 days after the second manure application.

Coliform concentrations were frequently over the maximum detection limit of 2419.6 MPN/100mls. Samples from the surface drainage ditch were most often above the maximum detection limit, followed by the Little River, drainage tiles, and, lastly, the wells. Only 3 samples were ever below the detection limit, all of which came from wells

Table 9: Untransformed E.coli concentrations in MPN/100mls for each sampling location by date. Values below the detection limit of 1 MPN/100mls were given a value of 0.5 MPN/100mls.

(Table 10). Wells were also the only sampling locations that had a median value that was below the maximum detection limit, which was 791 MPN/100mls (Table 14).

Nitrate values were typically below the drinking water standard of 10.0 mg/L, but did exceed it in 10 samples, with 6 of those samples being on 1/12/2014, which was not close in time to a liquid manure application. The Little River and wells were the only sampling locations that contained samples below the detection limit of 0.3 mg/L (Table 11). The surface drainage ditch samples had the highest median and mean value, followed by the drainage tiles (Table 14).

Generally, surface water standards for streams and rivers are not codified, but Florida has standards more stringent for waterways, based on classes, that are below the 10.0 mg/L limit for drinking water. These range from 0.67mg/L to 1.84 mg/L (EPA, 2010). Based on the higher standard of 1.84 mg/L, the drainage tiles and surface drainage consistently had concentrations that exceeded the 1.84mg/L. Wells frequently exceeded the standard, but were more often below the standard than the previously two mentioned sited.  The Little River typically had concentrations below the Florida surface water standard, with a few exceptions.

AllBac was above the minimum detection limit in every sample until 6/21/13, when all sampling locations were below the detection limit of 10.7 copies/100 mls. After this date samples were generally above the detection limit, but on several occasions S1 was below the minimum detection limit in the summer and fall of 2013, which is located at the beginning of the excavated portion of the drainage ditch. The Little River did have some samples below the minimum detection limit, but never had an instance in which both sampling locations were simultaneously below it (Table 11). The highest mean values for the study were from the surface drainage (9.9E3 copies/100mls). The second highest mean value was the Little River, at 3.54E3 copies/100mls, and lastly the wells, with a mean of 2.9E3 copies/100mls (Table 12).

BoBac concentrations were frequently below the detection limit, especially for the wells and the drainage ditch. After the first manure application, and in one sample in late June 2013 in the drainage ditch, all samples were below the detection limit. Samples from the Little River were above the detection limit during the first manure application on 5/30/2013, and again on 6/3/2013. BoBac concentrations in samples continued to be below the detection limit until the samples collected prior to the start of the second liquid manure application. However, BoBac was not detectable during or after the second application. Samples before the third manure application had detectable levels of BoBac, and concentrations continued to be detectable during and after the application until 5/26/2014 (Table 13). The Little River, surface drainage and wells all had a median concentrations below the detection limit, but the surface drainage had the highest mean concentration over the course of the study, followed by the Little River, and, lastly, the wells (Table 14).

Water quality parameters by site and date can be found in Appendix C. Mean turbidity generally was low ranging from 13.4 NTU for the drainage tiles up to 24.1 NTU for the surface drainage. The median value was below 10.0 NTU in all sites, except the surface drainage, which was 16.9 NTU. Conductivity was lower in the Little River than any of the other water types. The surface drainage ditch, drainage tiles, and wells all had similar conductivity median values (225 μS to 276 μS), and similar mean values (217.1 μS to 283.9 μS). The highest mean and median values were in the wells. The only location with resistivity values above the detection limit was the Little River. Total dissolved solids had the highest median and mean value in the wells, and the lowest in the Little River. ORP mean and median values were relatively similar amongst the water types, but one well and one drainage tile did have negative values, which indicate a reducing environment. The highest mean pH values were in the Little River (7.6), followed by the surface drainage (7.0), while the drainage tiles and wells both had a mean value of 7.0. Temperature values varied greatly over the course of the study depending on season, but the drainage tiles were consistently the lowest. The range of temperature values went from a low of 4.9oC in the Little River to a maximum temperature of 31.2 oC in a well sample (Table 14).

Table 10: Untransformed coliform  concentrations in MPN/100mls for each sampling location by date.  Values below the detection limit of 1 MPN/100mls were given a value of 0.5 MPN/100mls.

Table 11: Untransformed nitrate  concentrations in mg/L for each sampling location by date.  Values below the detection limit of 0.3 mg/L were given a value of 0.15 mg/L

Table 12: Untransformed AllBac concentrations in copies/100mls for each sampling location by date.  Dark grey cells were either not sampled or sampled and the filters not analyzed.

Table 13: Untransformed BoBac concentrations in copies/100mls for each sampling location by date.  Dark grey cells were either not sampled or sampled and the filters not analyzed.

Table 14: Simple statistics for chemical and biological data for each sample location site type.

## 4.4 Changes in Fecal Indicators Before and After Liquid Manure Applications

The mean log transformed E.coli values for the first manure application showed an increase in mean values after the start of the liquid manure application in the drainage tiles, surface drainage, and Little River. The confidence interval also shows that these values are higher than the values measured before the application (Figure 19). The mean values for the well data did not show a clear pattern of change based E.coli concentrations prior to the application (Table 15).

The mean log transformed E.coli values in the second and third manure applications did not exhibit a clear increase in value in any water sample type based on either the confidence intervals in Figure 19, or with the statistical analysis shown in Table 15. The one exception is the Little River did have a log transformed mean value that was significantly higher than other mean values during the second application, but it was before the liquid manure application began (Figure 19) and thus is not attributable to the dairy farm.

Coliform values were frequently close to or above the maximum detection limit. During the first liquid manure application only the wells had enough values below the maximum detection limit to warrant a statistical test. Based on the SAS results there was no significant change in log transformed mean concentrations in the wells. During the second liquid manure application statistical tests were performed on the drainage tiles and the wells. There was a significant change in the coliform values in the drainage tiles, but there was a continued decrease in mean values after the application. Wells had no values that were significantly different over the data set analyzed. The third manure application had values below the maximum detection limit in the drainage tiles and wells. Based on the statistical results in SAS, there was no significant change in the drainage tiles or wells (Figure 20).

Arithmetic mean values for nitrates only had one clear significantly different concentration during any of the manure applications in any water sample type. After the first liquid manure application mean nitrate values were significantly higher in the surface drainage ditch based on the confidence intervals and the statistical analysis in SAS (Figure 21).

During the first manure application only the log transformed mean for the Little River showed a significant increase in AllBac concentrations after the liquid manure application. The surface drainage showed a decrease after the application, and the wells had no clear pattern of change before or after the application. The second application had no significant change in any of the water quality types. The surface drainage did show an increase in the mean values after the liquid manure application, but the change is not statistically significant. The third application only had a significant change in the wells, but the statistically different value is from a decrease, not an increase (Figure 22, Table 15).

Unlike the other data, BoBac values are considered significant on a detection/non-detection basis. The Little River had detectable BoBac values in samples after the first application and detectable values in samples before and after the third application. The surface drainage had two samples with detectable concentrations during and after the first liquid manure application, and no detectable levels in the subsequent applications. Likewise, the wells had detectable concentrations in two samples after the first application, but no detectable concentration in samples in the subsequent two applications (Figure 23, Table 15).

Drainage Tiles

Little River

Surface Drainage Ditch

Wells

Figure 19: Graphs of log E.coli mean values during the 28 days before and after the start of all three liquid manure applications. Confidence intervals are one standard deviation.

Drainage Tiles

Surface Drainage Ditch

Wells

Little River

Figure 20:  Graphs of log coliform mean values during the 28 days before and after the start of all three liquid manure applications. Confidence intervals are one standard deviation.

Figure 21: Graphs nitrate mean values during the 28 days before and after the start of all three liquid manure applications. Confidence intervals are one standard deviation.

Drainage Tiles

Little River

Surface Drainage Ditch

Wells

Little River

Drainage Tiles

Little River

Surface Drainage Ditch

Wells

Figure 22: Graphs of log AllBac mean values during the 28 days before and after the start of all three liquid manure applications. Confidence intervals are one standard deviation.

Wells

Little River

Surface Drainage Ditch

Figure 23: Graphs of log BoBac mean values during the 28 days before and after the start of all three liquid manure applications. Confidence intervals are one standard deviation.

Table 15: Significance of change in fecal indicators by manure application analyzing the time frame of 28 days before and after the start of each manure application. Data was fit to a normal distribution and considered significant at p< 0.05. BoBac is the exception to this statistical analysis and is either below detection limit or above the detection limit.

| Liquid Manure Application | Site | Dependent Variables |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | AllBac | BoBac | Coliforms | E.coli | Nitrates |
| 1 | Drainage Tiles |  |  |  |  |  |
|  | Wells |  |  |  |  |  |
|  | Surface Drainage Ditch |  |  |  |  |  |
|  | Little River |  |  |  |  |  |
| 2 | Drainage Tiles |  |  | * |  |  |
|  | Wells |  |  |  |  |  |
|  | Surface Drainage Ditch |  |  |  |  |  |
|  | Little River |  |  |  | * |  |
| 3 | Drainage Tiles |  |  |  |  |  |
|  | Wells | * |  |  |  |  |
|  | Surface Drainage Ditch |  |  |  |  |  |
|  | Little River |  |  |  |  |  |
|  |  | p>0.05 or below detection (BoBac Only) |  |  |  |  |
|  |  | p<0.05 or detectable (BoBac Only) |  |  |  |  |
|  |  | Data above detection limit |  |  |  |  |
|  |  | NA |  |  |  |  |
| * |  | Significance not attributable to application |  |  |  |  |

## 4.5 Correlations and Significance

All biological data was analyzed with the SAS PROC Corr procedure to determine the Pearson correlation coefficient and determine if the correlations between biological factors separate from the other water quality parameters were significant. Even if the correlation is significant, only correlations equal to or greater than 0.1 are considered noteworthy. The strongest correlation of 0.61 was between the log AllBac and log BoBac data sets (Table 16). A moderate correlation also exists between the log E.coli and log coliform data set, while a weak correlation exists between log E.coli and log AllBac, log E.coli and log BoBac, and log coliforms and log AllBac. No significant correlation exists between log coliforms and log BoBac.

The data set of water quality and biological parameters was also analyzed with the same tests for correlations and significance of correlations. The strongest significant correlation (0.93) exits between TDS and conductivity. Other significant correlations exist between TDS and resistivity, resistivity and turbidity, and BoBac and water table level, which was negatively correlated (Table 17). The correlation between BoBac and the water table indicates that lower water table elevations were correlated with higher BoBac concentrations.

Table 16: Pearson correlation coefficients for the biological indicators used in this study with their probability of significance below.

Table 17: Pearson correlation coefficients for the chemical and biological indicators used in this study with their probability of significance below.

# 5. Discussion and Conclusions

## 5.1 Overview of this Study

Transport of fecal pollution from agriculture is an issue in many areas, so there is a greater need to understand the transport pathways it takes for both surface and groundwater. It was possible to look for significant changes in the indicators at the sampled sites through culturing and quantitative PCR assays, E.coli, coliforms, AllBac, and BoBac, along with measuring nitrates. This study looked at chemical and microbial data covering 3 liquid manure applications, along with the chemical and biological makeup of the liquid manure used in the applications. At least one line of evidence indicated contamination during and after the first liquid manure application in all the sampled water types. No clear evidence was found that the subsequent two applications had detectable impacts on any of the sampled locations. It is likely that precipitation, soil moisture, and a shallow depth to groundwater played an important part in the transport of the fecal indicators in the first application. In the second and third applications there was less precipitation and a much lower water table level, thus making the fecal indicators less mobile along the potential transport pathways.

5.2 Transport and impact of fecal indicators in surface and subsurface water

Based on the compilation of the statistical analysis, there is evidence that only the first liquid manure application in the spring of 2013 had an impact on the water at the site. The drainage tiles, surface drainage ditch, and Little River all had a statistically significant increase in E.coli. There was also a significant change in AllBac concentrations in the Little River, concentrations of BoBac present in samples from the Little River, surface drainage ditch, and wells 3 days and 6 days from the start of the liquid manure application. With the exception of the Little River, it is unlikely that the bacteria targeted by the BoBac assay could have entered these sampling locations from another source. At 19 days there was a small detectable level of BoBac in the drainage ditch, but it was no longer present in the Little River or wells. In the second and third manure applications BoBac was not detectable in the surface drainage or the wells.

During the first manure application the Little River had detectable levels of BoBac in both sampling locations, which either indicates contamination coming from agricultural activity further upstream, or that fecal pollution was being transported offsite from the LRAEU. Since BoBac was present in the surface drainage and the wells, it is possible that the LRAEU is the source of the contamination and it was transported via one, or both, of these pathways. It could also be that it was transported through unmonitored pathways, such as the sinkholes in the carbonate area by the Little River.

BoBac concentrations were detectable in the Little River prior to the third liquid manure application, and continued to be detectable during and after the application. The highest concentration was in the samples immediately before the start of the third application, and cannot be from the LRAEU. Even during the application it is unlikely the LRAEU is the source of any fecal contamination because the highest concentration was before the application, and BoBac was below the detection limit in the surface drainage and wells which might have been possible transport pathways.

5.3 Impacts from seasonal hydrology on detection of fecal indicators

The hydrology and precipitation patterns at the site, in conjunction with best management practices, likely prevented liquid manure from being transported in surface and/or subsurface water. The rainfall prior to and after the first manure application is likely responsible for the observed increases in fecal indicators in groundwater and surface water. Rainfall can cause transport in the drainage tiles, even when they are located above the seasonal water table. Rainfall can also reduce the depth to the water table in the aquifer resulting in less filtration in the vadose zone and in the aquifer. The fecal pollutants that were detectable in the surface drainage ditch could have been transported there via the drainage tiles and the movement of water from the aquifer to the drainage ditch along the hydraulic gradient. With the more saturated soil it is also possible that with the high application rate, not all the liquid manure could infiltrate fast enough and some portion was transported as surface runoff into the drainage ditch. Although, this was not observed during the sampling events, which occurred after manure applications were complete

5.4 Suggestions for Future Study

One of the greatest limitations with Bacteroides is that it cannot definitely prove that contamination in the Little River is from the dairy farm. Even when BoBac is detectable only during a liquid manure application at the LRAEU, it is still possible what is being detected came from an unknown upstream source. One option is to identify a more appropriate source tracker based on the microbial community structure of the liquid manure being used, if one exists. Other bacteria in high concentrations include Eubacterium, Clostridium, and Coptotermes, BoBac is still a useful tool for the wells and drainage ditch, but in the Little River the assay cannot differentiate from the upstream sources and those at the dairy farm.

Some of the biological indicators used had moderate or weak correlations that may allow them to be used in additional work to estimate fecal pollution. This would be particularly useful in cases where it was not possible to perform lab work to determine coliforms/E.coli or Bacteroides.  One unexpected result from the Pearson correlation coefficients was the negative relationship between BoBac and the water table level. This result is likely because the Little River was included in the analysis, and can contain detectable levels of BoBac from upstream inputs, which are not controlled by conditions at the LRAEU. When the Little River is removed from the analysis, the relationship is a positive relationship of higher BoBac concentrations with elevated water table levels.

Liquid manure contains multiple potential bacteria that can be used as fecal indicators. The presence of these fecal indicators in both surface and groundwater indicates the application of liquid manure does pose a real threat to water quality, but negative water quality impacts is not a universal outcome. Fecal indicators were found in all the potential pathways at the LRAEU showing that drainage tiles, groundwater transport, and surface runoff can play a role in pollutant transport.  Continued monitoring of the sampling locations will provide more data on the roll precipitation, depth to groundwater, and seasonality play in the transport at this site. Additional analysis of liquid manure samples to look for less common fecal indicator in the surrounding watershed will provide more information about the LRAEU impacts on the Little River.

# List of References

Ayebo, A., Plowman, D., States, S., 2006, Nitrate, coliforms, and cryptosporidium spp. As indicators of stream water quality in western Pennsylvania: Journal of Environmental Health, v. 69(3), p. 16-21.

Baxter-Potter, W., Gilliland, M., 1988, Bacterial Pollution in Runoff from Agricultural Lands: Journal of Environmental Quality, v. 17(1), p. 27-34.

Bernhard, A.E., Field, K.G., 2000, PCR assay to discriminate human and ruminant feces on the basis of host differences in Bacteroides-Prevotella: Applied and Environmental Microbiology, v. 66(10), p. 4571-4574.

Birchall, S., Dilon, C., Wrigley, R., 2008, Effluent and manure management database for the Australian dairy industry: Dairy Australia.

Brennan, F.P., O’Flaherty, V., Kramers, G., Grant, J., Richards, K.G., 2010, Long-term persistence of leaching of Escherichia coli in temperate maritime soils, Applied Environmental Microbiology, v. 76(5), p. 1449-1455.

Caporaso, J.G., Christian, L.L., Walters, W.A., Berg-Lyons, D., Huntley, J., Fierer, N., Owens, S.M., Betley, J., Fraser, L., Bauer, M., Gormley, N., Gilbert, J.A., Smith, G., Knight, R., 2012, Ultra-high-throughput microbial community analysis on the Illumina HiSeq and MiSeq platforms: International Society for Microbial Ecology, v.6, p. 1621-1624.

Cook, M.J., Baker, J.L., 2001, Bacteria and nutrient transport to tile lines shortly after application of large volumes of liquid swine manure: Transactions of the American Society of Agricultural Engineers, v. 44(3), p. 495-503.

Dean, D.M., Foran, M.E., 1992, The effect of farm liquid waste application on tile drainage: Journal of Soil and Water Conservation, v. 47(5), p. 368-369.

Dowling, J., 2014, Fox 17 exposes massive manure spill in Allegan County: Fox 17 West Michigan, accessed 4/26/2014 from http://fox17online.com/2014/02/15/fox-17-exposes-massive-manure-spill-in-allegan-county-waters/#axzz30EhYFWCm

Dungan, R.S., 2010, Fate and transport of bioaerosols associated with livestock operations and manures: Journal of Animal Science, v. 88(11), p. 3693-3706.

Environmental Protection Agency, 2007, Guidelines Establishing Test Procedures for the Analysis of Pollutants; Analytical Methods for the Biological Pollutants in Wastewater and Sewage Sludge; Final Rule: United States Federal Register, v. 72(57), p. 14221.

Environmental Protection Agency, 2010, Water Quality Standards for the State of Florida’s Lakes and Flowing Water. [Updated2010; cited 2014]; Available from http://water.epa.gov/lawsregs/rulesregs/florida_factsheet.cfm

Environmental Protection Agency, 2008, National Primary Drinking Water Standards.

Environmental Protection Agency. 2005, Protecting Water Quality from Agricultural Runoff. [Updated 2005; cited 2014]; Available from http://water.epa.gov/polwaste/nps/outreach/facts_index.cfm

Flemming, R., Johnston, J., Fraser, H., 1999, Leaking of liquid manure storages-literature review: University of Guelph.

Geohring, L.D., Wright, P.E., Steenhuis, T.S., 1998, Preferential flow of liquid manure to subsurface drains: Drainage in the 21st Century: Food production and the environment, p. 8-10.

Harter, T., Davis. H, Mathews, M.C., Meyer, R.D., Shallow groundwater quality on dairy farms with irrigated forage crops: Journal of Contaminant Hydrology, v. 55(3-4), p. 287-315.

Haack, S., 2008, E. coli and Alternatives as Indicators of Fecal Pollution. United States Geological Survey. Michigan Water Science Center, [Updated 2005; cited 2013] Available from http://www.glc.org/stclair/documents/Haack_PP.pdf

Hancock, D.D., Rice, D.H.., Herriott, D.E., Besser, T.E., Ebel, E.D., Carpenters, L.V., 1997, Effects of Farm Manure-Handling Practices on Escherichia coli O157 Prevalence in Cattle: Journal of Food Protection, no. 4, p. 363-366.

Harter, T., Davis. H, Mathews, M.C., Meyer, R.D., Shallow groundwater quality on dairy farms with irrigated forage crops: Journal of Contaminant Hydrology, v. 55(3-4), p. 287-315.

Hrudey, S.E., Payment, P., Huck, P.M., Gillham, R.W., Hrudey E.J., 2003, A fatal waterborne disease epidemic in Walkerton, Ontario: comparison with other waterborne outbreaks in the developed world: Water Science and Technology, vol. 47(3), pp 7-14.

Hubbard, R.K., Lowrance, R.R., 1998, Management of dairy cattle manure: Agricultural Uses of Municipal Animal, and Industrial Byproducts. US Department of Agriculture, Agricultural Research Service, p. 91-102.

Hunter, Robert Wesley, Hydrogeology of the Little River Animal Agriculture Environmental Research Unit and Impacts of Dairy Operations on Groundwater.  Master's Thesis, University of Tennessee, 2013.

Ishii, S., Ksoll, W.B., Hicks, R.E., Sadowsky, M.J., 2006, Presence and Growth of Naturalized Escherichia coli in Temperate Soils from Lake Superior Watersheds: Applied and Environmental Microbiology, v. 72(1), p. 612-621.

Jafferzic, A. E., Jarde, A., Pourcher, M., Gourmelon, M., Caprais, D., Heddadj, P.C., Bilal, M., 2011, Microbial and Chemical Markers: Runoff Transfer in Animal Manure-Amended Soils: Journal of Environmental Quality, v. 40(3), p. 959-968.

Jamieson, R., Gordon, R., Sharples, K., Stratton, G., Madani, A., 2002, Movement and Persistence of Fecal Bacteria in Agricultural Soils and Subsurface Drainage Water: A Review: Canadian Biosystems Engineering. V. 44(1), p. 1-9.

Jimenez-Clavero, M.A., Escribano-Romero, E., Mansilla, C., Gomez, N., Roblas, N., Ponz, F., Ley, V., Saiz, J.C., 2005, Survey of bovine enterovirus in biological and environmental samples by a highly sensitive real-time reverse transcription-PCR: Applied Environmental Microbiology, v. 71(7), p. 3536-3543.

Kowalski, Z., Makara, A., Fijorek, K., 2013, Changes in the properties of manure slurry: Acta Biochmica Polonica, v 60 (4), p. 845-850.

Kreader, C.A., 1998, Persistence of PCR-detectable Bacteroides distasonis from human feces in river water, Applied and Environmental Microbiology, v. 64(10), p. 4102-4105.

Lambertini, E., Spencer, S.K., Bertz, P.D., Loge, F.J., Kieke, Burney, A.K., Borchardt, M.A., 2008, Concentration of enteroviruses, adenoviruses, noroviruses from drinking water by glass wool: Applied Environmental and Microbiology, v. 74(10), p. 2990-2996.

Layton, A., McKay, L., Williams, D., Garrett, V., Gentry, R., Sayler, G., 2006, Development of Bacteroides 16s Rrna Gene Taqman-Based Real-Time Pcr Assays for Estimation of Total, Human, and Bovine Fecal Pollution in Water: Applied and Environmental Microbiology, v. 72(6), p. 4214–4224.

Ley, V., Higgins, J., Fayer, R., 2002, Bovine enterovirus as indicators of fecal contamination: Applied Environmental Microbiology, v. 68(7), p. 3455-3461.

Lorior, J., Powers, W., Sutton, A., 2004, Manure characteristics: manure management systems series: MidWest Plan Service Iowa State University.

Martin, J, 2004, A comparison of dairy cattle manure management with and without anaerobic digestion and biogas utilization: AgSTAR Program United States Environmental Protection Agency.

Mathews, M.C., Frate, C., Harter, T., Sather, S., 2001, Lagoon water composition, sampling and field analysis: Proceedings 2001 California Plant and Soil Conference, p. 43-39.

McGarvey, J.A., Miller, W.G., Sanchez, S., Stanker, L., 2004, Identification of Bacterial Populations in Dairy Wastewater by use of 16s rRNA Gene Sequences and Other genetic Markers: Applied Environmental Microbiology, v. 70(7), p. 4267-4275.

McGechan, M.B., Lewis, D.R., 2000, Watercourse pollution due to surface runoff following slurry spreading part 2: decision support to minimize pollution: Journal of Agricultural Engineering Research, v. 75(4), p. 429-447.

Meyer, F., Paarmann, D., D’Souza M., Olson, R., Glass, E.M., Kubal, M., Paczian, T., Rodriguez, A., Stevens, R., Wilke, A., Wilkening, J., Edwards, R.A., 2008, The metagenomics RAST server- a public resource for the automatic phylogentic and functional analysis of metagenomes; BMC Bioinformatics, v. 9, p. 386.

Millen, H.T., Gonnering, J.C., Berg, R.K., Spencer, S.K., Jokela, W.E., Pearce, J.M. Borchardt, J.N., Borchardt, M.A., 2012, Glass wool filters for concentrating waterborne viruses and agricultural zoonotic pathogens: Journal of Visualized Experiments, v. 61.

Myers, D.N., Stoeckel, D.M., Bushon, R.N., Francy, D.S., and Brady, A.M.G., 2007, Fecal indicator bacteria (ver. 2.0): U.S. Geological Survey Techniques of Water-Resources Investigations, book 9, chap. A7, section 7.1, February, accessed 1/05/2014 from http://pubs.water.usgs.gov/twri9A/.

Neuman, R.B., 1960, Geology of the Wildwood quadrangle, Tennessee: U.S. Geological Survey, Geologic Quadrangle Map, GQ-130, scale 1:24000.

Nielsen, D.M., 1991, Practical Handbook of Ground-Water Monitoring: New York, Lewis Publishers, Inc., p. 466.

Randall, G.W., Iragavarapu, T.K., Schmitt, M.A., 2000, Nutrient losses in subsurface drainage water from manure and urea applied for corn: Journal of Environmental Quality, v. 29(4), p. 1244-1252.

Savichtcheva, A., Okayama, N., Okabe, S., 2007, Relationships between Bacteroides 16S rRNA genetic markers and presence of bacterial enteric pathogens and conventional fecal indicators: Water Research, v. 41(16), p. 3615-3628.

Scott, T.M., Rose, J.B., Jenkins, T.M., Farrah, S.R., Lukasik, J., 2002, Microbial source tracking: current methodology and future directions: Applied Environmental Microbiology, v. 68(12), p. 5796-5803.

Spalding, R.F., Exner, M.E., 1993, Occurrence of nitrate in groundwater-a review: Journal of Environmental Quality, v. 22, p. 392-402.

Tennessee Department of Environment and Conservation, 2012, The Status of Water Quality in Tennessee, p. 9-67.

Tennessee Department of Environment and Conservation, 2013, General Water Quality Criteria, §0400-40-03.

Thornley, S., Bos, A.W., 1985, Effects of livestock wastes and agricultural drainage on water quality: An Ontario case study: Journal of Soil and Water Conservation, v. 40 (1), p. 173-175.

Van Donsel, D.J., Geldreich, E.E., Clarke, N.A., 1967, Seasonal variations in survival of indicator bacteria in soil and their contribution to storm-water pollution: Applied Environmental Microbiology, v. 15(6), p. 1362-1370.

Ward, M.H., deKok, T.M., Levallois, P., Brender, J., Gulis, G., Nolan, B.T., VanDerslice, J., 2005, Drinking-water and health: Environmental Health Perspective, v. 113(11), p. 1607-1614.

Wesström, I., Ekbohm, G., Linnér, H., Messing, I., 2003, The effect of controlled drainage on subsurface outflow from level agricultural fields: Hydrological Processes, v. 17(8), p. 1525-1538.

# Appendices

## Appendix A: Bore Logs for Wells

;

## Appendix B: Statistical Errata

| Drainage Tiles: Application 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Dependent Variable: LogEcoli |  | Model: LogEcoli = Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.94 | Pr < W | 0.04 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 5 | 19.83 | 3.97 | 7.4 | 0.0002 |
| Error | 27 | 14.48 | 0.54 |  |  |
| Corrected Total | 32 | 34.31 |  |  |  |
| R-Square | Coeff Var | Root MSE | LogEcoli Mean |  |  |
| 0.578002 | 44.7444 | 0.73 | 1.64 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 5 | 19.83 | 3.97 | 7.4 | 0.0002 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 5 | 19.83 | 3.97 | 7.4 | 0.0002 |
| Tukey Grouping |  | Mean | N | Date |  |
|  | A | 2.9158 | 5 | 6/3/2013 |  |
|  | A | 2.5521 | 5 | 5/30/2013 |  |
| B | A | 1.7351 | 5 | 6/8/2013 |  |
| B |  | 1.0803 | 6 | 6/21/2013 |  |
| B |  | 1.0259 | 6 | 5/9/2013 |  |
| B |  | 0.893 | 6 | 6/16/2013 |  |
| Drainage Tiles: Application 1 |  |  |  |  |  |
| Dependent Variable: LogColiforms |  | Model: LogColiforms = Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.37 | Pr < W | <0.01 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 5 | 0.18846705 | 0.03769341 | 1.4 | 0.2554 |
| Error | 27 | 0.72631032 | 0.02690038 |  |  |
| Corrected Total | 32 | 0.91477737 |  |  |  |
| R-Square | Coeff Var | Root MSE | LoColiform Mean |  |  |
| 0.206025 | 4.926322 | 0.164013 | 3.329327 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 5 | 0.18846705 | 0.03769341 | 1.4 | 0.2554 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 5 | 0.18846705 | 0.03769341 | 1.4 | 0.2554 |
| Tukey Grouping |  | Mean | N | Date |  |
| Test Not Applicable |  |  |  |  |  |
| Drainage Tiles: Application 1 |  |  |  |  |  |
| Dependent Variable: Nitrates |  | Model: Nitrates = Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.752506 | Pr < W | <0.0001 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 5 | 26.5520885 | 5.3104177 | 1.22 | 0.3268 |
| Error | 26 | 112.9478333 | 4.3441474 |  |  |
| Corrected Total | 31 | 139.4999219 |  |  |  |
| R-Square | Coeff Var | Root MSE | Nitrate Mean |  |  |
| 0.190338 | 55.2351 | 2.084262 | 3.773438 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 5 | 26.55208854 | 5.31041771 | 1.22 | 0.3268 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 5 | 26.55208854 | 5.31041771 | 1.22 | 0.3268 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Drainage Tiles: Application 2 |  |  |  |  |  |
| Dependent Variable: LogE.coli |  | Model: LogE.coli = Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.691616 | Pr < W | <0.0001 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 7 | 0.8425735 | 0.12036764 | 1.6 | 0.2199 |
| Error | 13 | 0.97722717 | 0.07517132 |  |  |
| Corrected Total | 20 | 1.81980067 |  |  |  |
| R-Square | Coeff Var | Root MSE | LogEcoli Mean |  |  |
| 0.463003 | 138.0989 | 0.274174 | 0.198534 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 7 | 0.8425735 | 0.12036764 | 1.6 | 0.2199 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 7 | 0.8425735 | 0.12036764 | 1.6 | 0.2199 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Drainage Tiles: Application 2 |  |  |  |  |  |
| Dependent Variable: Log Coliforms |  | Model: Log Coliforms = Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.824923 | Pr < W | 0.0021 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 7 | 1.75657875 | 0.25093982 | 3.29 | 0.0342 |
| Error | 12 | 0.91639877 | 0.07636656 |  |  |
| Corrected Total | 19 | 2.67297752 |  |  |  |
| R-Square | Coeff Var | Root MSE | LogColiforms Mean |  |  |
| 0.657162 | 9.137744 | 0.276345 | 3.024215 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 7 | 1.75657875 | 0.25093982 | 3.29 | 0.0342 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 7 | 1.75657875 | 0.25093982 | 3.29 | 0.0342 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Drainage Tiles: Application 2 |  |  |  |  |  |
| Dependent Variable: Nitrates |  | Model: Nitrates = Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.884363 | Pr < W | 0.0309 |  |
| The GLM Procedure |  |  |  |  |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 6 | 2.81277778 | 0.4687963 | 1.49 | 0.2693 |
| Error | 11 | 3.47166667 | 0.31560606 |  |  |
| Corrected Total | 17 | 6.28444444 |  |  |  |
| R-Square | Coeff Var | Root MSE | Nitrates Mean |  |  |
| 0.447578 | 20.47002 | 0.561788 | 2.744444 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 6 | 2.81277778 | 0.4687963 | 1.49 | 0.2693 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 6 | 2.81277778 | 0.4687963 | 1.49 | 0.2693 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Drainage Tiles: Application 3 |  |  |  |  |  |
| Dependent Variable: Log E.coli |  | Model: Log E.coli = Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.574204 | Pr < W | <0.0001 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 7 | 0.34671433 | 0.04953062 | 0.87 | 0.547 |
| Error | 17 | 0.96474167 | 0.05674951 |  |  |
| Corrected Total | 24 | 1.311456 |  |  |  |
| R-Square | Coeff Var | Root MSE | LogEcoli Mean |  |  |
| 0.264374 | 202.5694 | 0.238222 | 0.1176 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 7 | 0.34671433 | 0.04953062 | 0.87 | 0.547 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 7 | 0.34671433 | 0.04953062 | 0.87 | 0.547 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Drainage Tiles: Application 3 |  |  |  |  |  |
| Dependent Variable: Log Coliforms |  | Model: Log Coliforms = Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.916942 | Pr < W | 0.0437 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 7 | 2.53191433 | 0.36170205 | 2.11 | 0.0993 |
| Error | 17 | 2.91817927 | 0.1716576 |  |  |
| Corrected Total | 24 | 5.4500936 |  |  |  |
| R-Square | Coeff Var | Root MSE | Coliforms Mean |  |  |
| 0.464563 | 14.97677 | 0.414316 | 2.766389 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 7 | 2.53191433 | 0.36170205 | 2.11 | 0.0993 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 7 | 2.53191433 | 0.36170205 | 2.11 | 0.0993 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Drainage Tiles: Application 3 |  |  |  |  |  |
| Dependent Variable: Nitrates |  | Model: Nitrates = Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.449603 | Pr < W | <0.0001 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 7 | 4886070872 | 698010125 | 0.95 | 0.4976 |
| Error | 17 | 12527809840 | 736929991 |  |  |
| Corrected Total | 24 | 17413880712 |  |  |  |
| R-Square | Coeff Var | Root MSE | Nitrate Mean |  |  |
| 0.280585 | 229.4094 | 27146.45 | 11833.19 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 7 | 4886070872 | 698010125 | 0.95 | 0.4976 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 7 | 4886070872 | 698010125 | 0.95 | 0.4976 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Surface Drainage: Application 1 |  |  |  |  |  |
| Dependent Variable: Log E.coli |  | Model: Log E.coli = Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.927352 | Pr < W | 0.1744 |  |
| Surface Drainage: Application 1 |  |  |  |  |  |
| Dependent Variable: sqrt E.coli |  | Model: sqrt E.coli = Date |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.742626 | Pr < W | 0.0003 |  |
| The GLM Procedure |  |  |  |  |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 5 | 2917.269778 | 583.453956 | 13.53 | 0.0001 |
| Error | 12 | 517.483774 | 43.123648 |  |  |
| Corrected Total | 17 | 3434.753552 |  |  |  |
| R-Square | Coeff Var | Root MSE | LogEcoli Mean |  |  |
| 0.849339 | 43.03208 | 6.56686 | 15.26038 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 5 | 2917.269778 | 583.453956 | 13.53 | 0.0001 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 5 | 2917.269778 | 583.453956 | 13.53 | 0.0001 |
| Tukey Grouping | Mean | N | Date |  |  |
| A | 42.838 | 3 | 6/3/2013 |  |  |
| B | 16.437 | 3 | 5/30/2013 |  |  |
| B | 9.724 | 3 | 6/16/2013 |  |  |
| B | 7.787 | 3 | 6/21/2013 |  |  |
| B | 7.558 | 3 | 6/8/2013 |  |  |
| B | 7.218 | 3 | 5/9/2013 |  |  |
| Surface Drainage: Application 1 |  |  |  |  |  |
| Dependent Variable: Log AllBac |  | Model: Log AllBac= Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.97022 | Pr < W | 0.8018 |  |
| Surface Drainage: Application 1 |  |  |  |  |  |
| Dependent Variable: Sqrt AllBac |  | Model: Sqrt AllBac= Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.340389 | Pr < W | <0.0001 |  |
| The GLM Procedure |  |  |  |  |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 5 | 5137802.51 | 1027560.5 | 0.98 | 0.4676 |
| Error | 12 | 12558861.5 | 1046571.79 |  |  |
| Corrected Total | 17 | 17696664.01 |  |  |  |
| R-Square | Coeff Var | Root MSE | Sqrt AllBac Mean |  |  |
| 0.290326 | 301.8387 | 1023.021 | 338.9297 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 5 | 5137802.511 | 1027560.502 | 0.98 | 0.4676 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 5 | 5137802.511 | 1027560.502 | 0.98 | 0.4676 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Surface Drainage: Application 1 |  |  |  |  |  |
| Dependent Variable: Nitrates |  | Model: Nitrates= Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.748312 | Pr < W | 0.0003 |  |
| The GLM Procedure |  |  |  |  |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 5 | 48.85684028 | 9.77136806 | 22.23 | <.0001 |
| Error | 12 | 5.275 | 0.43958333 |  |  |
| Corrected Total | 17 | 54.13184028 |  |  |  |
| R-Square | Coeff Var | Root MSE | Nitrate Mean |  |  |
| 0.902553 | 13.6041 | 0.663011 | 4.873611 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 5 | 48.85684028 | 9.77136806 | 22.23 | <.0001 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 5 | 48.85684028 | 9.77136806 | 22.23 | <.0001 |
| Tukey Grouping | Mean | N | Date |  |  |
| A | 8.4583 | 3 | 6/3/2013 |  |  |
| B | 4.7667 | 3 | 5/9/2013 |  |  |
| B | 4.4833 | 3 | 6/8/2013 |  |  |
| B | 4.0667 | 3 | 6/21/2013 |  |  |
| B | 3.8333 | 3 | 6/16/2013 |  |  |
| B | 3.6333 | 3 | 5/30/2013 |  |  |
| Surface Drainage: Application 2 |  |  |  |  |  |
| Dependent Variable: Log E.coli |  | Model: Log E.coli =Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.691616 | Pr < W | <0.0001 |  |
| The GLM Procedure |  |  |  |  |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 7 | 0.8425735 | 0.12036764 | 1.6 | 0.2199 |
| Error | 13 | 0.97722717 | 0.07517132 |  |  |
| Corrected Total | 20 | 1.81980067 |  |  |  |
| R-Square | Coeff Var | Root MSE | LogEcoli Mean |  |  |
| 0.463003 | 138.0989 | 0.274174 | 0.198534 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 7 | 0.8425735 | 0.12036764 | 1.6 | 0.2199 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 7 | 0.8425735 | 0.12036764 | 1.6 | 0.2199 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Surface Drainage: Application 2 |  |  |  |  |  |
| Dependent Variable:10^nitrate |  | Model: 10^nitrate =Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.467854 | Pr < W | <0.0001 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 7 | 1821265507 | 260180787 | 1.07 | 0.4523 |
| Error | 9 | 2192432293 | 243603588 |  |  |
| Corrected Total | 16 | 4013697800 |  |  |  |
| R-Square | Coeff Var | Root MSE | 10^nitrate Mean |  |  |
| 0.453762 | 239.281 | 15607.81 | 6522.792 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 7 | 1821265507 | 260180787 | 1.07 | 0.4523 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 7 | 1821265507 | 260180787 | 1.07 | 0.4523 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Surface Drainage: Application 2 |  |  |  |  |  |
| Dependent Variable: sqrt AllBac |  | Model: sqrt AllBac =Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.609326 | Pr < W | <0.0001 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 7 | 43691.9044 | 6241.7006 | 0.5 | 0.816 |
| Error | 9 | 113174.4297 | 12574.9366 |  |  |
| Corrected Total | 16 | 156866.3341 |  |  |  |
| R-Square | Coeff Var | Root MSE | sqrt AllBac Mean |  |  |
| 0.27853 | 196.6633 | 112.138 | 57.02032 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 7 | 43691.90438 | 6241.70063 | 0.5 | 0.816 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 7 | 43691.90438 | 6241.70063 | 0.5 | 0.816 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Surface Drainage: Application 3 |  |  |  |  |  |
| Dependent Variable: Log E.coli |  | Model: Log E.coli =Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.898413 | Pr < W | <0.0001 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 7 | 0.34671433 | 0.04953062 | 0.87 | 0.547 |
| Error | 17 | 0.96474167 | 0.05674951 |  |  |
| Corrected Total | 24 | 1.311456 |  |  |  |
| R-Square | Coeff Var | Root MSE | LogEcoli Mean |  |  |
| 0.264374 | 202.5694 | 0.238222 | 0.1176 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 7 | 0.34671433 | 0.04953062 | 0.87 | 0.547 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 7 | 0.34671433 | 0.04953062 | 0.87 | 0.547 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Surface Drainage: Application 3 |  |  |  |  |  |
| Dependent Variable: Nitrates |  | Model: Nitrates =Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.449603 | Pr < W | <0.0001 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 7 | 4886070872 | 698010125 | 0.95 | 0.4976 |
| Error | 17 | 12527809840 | 736929991 |  |  |
| Corrected Total | 24 | 17413880712 |  |  |  |
| R-Square | Coeff Var | Root MSE | Nitrate Mean |  |  |
| 0.280585 | 229.4094 | 27146.45 | 11833.19 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 7 | 4886070872 | 698010125 | 0.95 | 0.4976 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 7 | 4886070872 | 698010125 | 0.95 | 0.4976 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Wells: Application 1 |  |  |  |  |  |
| Dependent Variable: LogE.coli |  | Model: LogE.coli =Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.859705 | Pr < W | 0.0033 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 5 | 3.18560096 | 0.63712019 | 1.03 | 0.4305 |
| Error | 18 | 11.14931559 | 0.61940642 |  |  |
| Corrected Total | 23 | 14.33491655 |  |  |  |
| R-Square | Coeff Var | Root MSE | LogEcoli Mean |  |  |
| 0.222227 | 101.9703 | 0.787024 | 0.771817 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 5 | 3.18560096 | 0.63712019 | 1.03 | 0.4305 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 5 | 3.18560096 | 0.63712019 | 1.03 | 0.4305 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Wells: Application 1 |  |  |  |  |  |
| Dependent Variable: Log Coliforms |  | Model: Log Coliforms =Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.796711 | Pr < W | 0.0003 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 5 | 5.3255041 | 1.06510082 | 1.34 | 0.2943 |
| Error | 18 | 14.36009802 | 0.79778322 |  |  |
| Corrected Total | 23 | 19.68560213 |  |  |  |
| R-Square | Coeff Var | Root MSE | Log Coliforms Mean |  |  |
| 0.270528 | 33.53496 | 0.893187 | 2.66345 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 5 | 5.3255041 | 1.06510082 | 1.34 | 0.2943 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 5 | 5.3255041 | 1.06510082 | 1.34 | 0.2943 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Wells: Application 1 |  |  |  |  |  |
| Dependent Variable: Nitrates |  | Model: Nitrates =Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.856544 | Pr < W | 0.0029 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 5 | 35.2605208 | 7.0521042 | 1.05 | 0.4175 |
| Error | 18 | 120.454375 | 6.6919097 |  |  |
| Corrected Total | 23 | 155.7148958 |  |  |  |
| R-Square | Coeff Var | Root MSE | Nitrates Mean |  |  |
| 0.226443 | 94.42577 | 2.586873 | 2.739583 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 5 | 35.26052083 | 7.05210417 | 1.05 | 0.4175 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 5 | 35.26052083 | 7.05210417 | 1.05 | 0.4175 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Wells: Application 1 |  |  |  |  |  |
| Dependent Variable: Log AllBac |  | Model: Log AllBac =Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.970113 | Pr < W | 0.7998 |  |
| Wells: Application 1 |  |  |  |  |  |
| Dependent Variable: Sqrt AllBac |  | Model: Sqrt AllBac =Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.566571 | Pr < W | <0.0001 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 5 | 106300.0362 | 21260.0072 | 0.69 | 0.6434 |
| Error | 12 | 372084.7252 | 31007.0604 |  |  |
| Corrected Total | 17 | 478384.7615 |  |  |  |
| R-Square | Coeff Var | Root MSE | Sqrt AllBac Mean |  |  |
| 0.222206 | 194.0331 | 176.0882 | 90.75166 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 5 | 106300.0362 | 21260.0072 | 0.69 | 0.6434 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 5 | 106300.0362 | 21260.0072 | 0.69 | 0.6434 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Wells: Application 2 |  |  |  |  |  |
| Dependent Variable: Log E.coli |  | Model: Log E.coli =Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.755739 | Pr < W | <0.0001 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 7 | 1.09305972 | 0.15615139 | 0.23 | 0.9741 |
| Error | 24 | 16.33471545 | 0.68061314 |  |  |
| Corrected Total | 31 | 17.42777516 |  |  |  |
| R-Square | Coeff Var | Root MSE | LogEcoli Mean |  |  |
| 0.062719 | 142.692 | 0.824993 | 0.578163 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 7 | 1.09305972 | 0.15615139 | 0.23 | 0.9741 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 7 | 1.09305972 | 0.15615139 | 0.23 | 0.9741 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Wells: Application 2 |  |  |  |  |  |
| Dependent Variable: Log Coliforms |  | Model: Log Coliforms =Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.818909 | Pr < W | <0.0001 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 7 | 1.20954636 | 0.17279234 | 0.91 | 0.5173 |
| Error | 24 | 4.57027172 | 0.19042799 |  |  |
| Corrected Total | 31 | 5.77981808 |  |  |  |
| R-Square | Coeff Var | Root MSE | Log Coliforms Mean |  |  |
| 0.209271 | 14.4272 | 0.436381 | 3.024708 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 7 | 1.20954636 | 0.17279234 | 0.91 | 0.5173 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 7 | 1.20954636 | 0.17279234 | 0.91 | 0.5173 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Wells: Application 2 |  |  |  |  |  |
| Dependent Variable: Nitrates |  | Model: Nitrates =Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.836073 | Pr < W | 0.0002 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 7 | 3.7896875 | 0.54138393 | 0.24 | 0.972 |
| Error | 24 | 55.0725 | 2.2946875 |  |  |
| Corrected Total | 31 | 58.8621875 |  |  |  |
| R-Square | Coeff Var | Root MSE | Nitrates Mean |  |  |
| 0.064382 | 79.0772 | 1.514823 | 1.915625 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 7 | 3.7896875 | 0.54138393 | 0.24 | 0.972 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 7 | 3.7896875 | 0.54138393 | 0.24 | 0.972 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Wells: Application 2 |  |  |  |  |  |
| Dependent Variable: Sqrt AllBac |  | Model: Sqrt AllBac =Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.699342 | Pr < W | 0.0004 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 6 | 6363.46913 | 1060.57819 | 0.49 | 0.7993 |
| Error | 7 | 15193.44342 | 2170.49192 |  |  |
| Corrected Total | 13 | 21556.91255 |  |  |  |
| R-Square | Coeff Var | Root MSE | AllBac Mean |  |  |
| 0.295194 | 161.4386 | 46.58854 | 28.85837 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 6 | 6363.46913 | 1060.578188 | 0.49 | 0.7993 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 6 | 6363.46913 | 1060.578188 | 0.49 | 0.7993 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Wells: Application 3 |  |  |  |  |  |
| Dependent Variable: Log E.coli |  | Model: Log E.coli =Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.443129 | Pr < W | <0.0001 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 7 | 0.55945483 | 0.07992212 | 1.72 | 0.151 |
| Error | 24 | 1.11329557 | 0.04638732 |  |  |
| Corrected Total | 31 | 1.6727504 |  |  |  |
| R-Square | Coeff Var | Root MSE | Ecoli Mean |  |  |
| 0.334452 | 246.0881 | 0.215377 | 0.08752 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 7 | 0.55945483 | 0.07992212 | 1.72 | 0.151 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 7 | 0.55945483 | 0.07992212 | 1.72 | 0.151 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Wells: Application 3 |  |  |  |  |  |
| Dependent Variable: Nitrate |  | Model: Nitrate =Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.836073 | Pr < W | 0.0002 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 7 | 68.425 | 9.775 | 1.43 | 0.2412 |
| Error | 24 | 164.59 | 6.8579167 |  |  |
| Corrected Total | 31 | 233.015 |  |  |  |
| R-Square | Coeff Var | Root MSE | Nitrates Mean |  |  |
| 0.293651 | 89.14936 | 2.618762 | 2.9375 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 7 | 68.425 | 9.775 | 1.43 | 0.2412 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 7 | 68.425 | 9.775 | 1.43 | 0.2412 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Wells: Application 3 |  |  |  |  |  |
| Dependent Variable: Log AllBac |  | Model: Log AllBac = Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.566571 | Pr < W | <0.0001 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 7 | 23.1564877 | 3.30806967 | 5.4 | 0.015 |
| Error | 8 | 4.89857516 | 0.6123219 |  |  |
| Corrected Total | 15 | 28.05506286 |  |  |  |
| R-Square | Coeff Var | Root MSE | AllBac Mean |  |  |
| 0.825394 | 26.03195 | 0.78251 | 3.00596 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 7 | 23.1564877 | 3.30806967 | 5.4 | 0.015 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 7 | 23.1564877 | 3.30806967 | 5.4 | 0.015 |
| Tukey Grouping |  | Mean | N | Date |  |
|  | A | 4.0065 | 2 | 5/7/2014 |  |
|  | A | 3.8721 | 2 | 5/26/2014 |  |
|  | A | 3.8444 | 2 | 5/21/2014 |  |
|  | A | 3.562 | 2 | 5/14/2014 |  |
|  | A | 3.3582 | 2 | 5/23/2014 |  |
| B | A | 2.8072 | 2 | 6/4/2014 |  |
| B | A | 2.4863 | 2 | 5/29/2014 |  |
| B |  | 0.1109 | 2 | 6/10/2014 |  |
| Little River Application 1 |  |  |  |  |  |
| Dependent Variable: Log E.coli |  | Model: Log E.coli = Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.78153 | Pr < W | 0.0058 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 5 | 1.30367147 | 0.26073429 | 14.07 | 0.0029 |
| Error | 6 | 0.111151 | 0.01852517 |  |  |
| Corrected Total | 11 | 1.41482248 |  |  |  |
| R-Square | Coeff Var | Root MSE | LogEcoli Mean |  |  |
| 0.921438 | 6.383982 | 0.136107 | 2.132011 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 5 | 1.30367147 | 0.26073429 | 14.07 | 0.0029 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 5 | 1.30367147 | 0.26073429 | 14.07 | 0.0029 |
| Tukey Grouping | Mean | N | Date |  |  |
| A | 2.8424 | 2 | 6/3/2013 |  |  |
| B | 2.1095 | 2 | 6/8/2013 |  |  |
| B |  |  |  |  |  |
| B | 2.0661 | 2 | 5/30/2013 |  |  |
| B |  |  |  |  |  |
| B | 1.9759 | 2 | 6/21/2013 |  |  |
| B |  |  |  |  |  |
| B | 1.968 | 2 | 6/16/2013 |  |  |
| B |  |  |  |  |  |
| B | 1.8302 | 2 | 5/9/2013 |  |  |
| Little River: Application 1 |  |  |  |  |  |
| Dependent Variable: Nitrates |  | Model: Nitrates = Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.779456 | Pr < W | 0.0051 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 5 | 4.02352273 | 0.80470455 | 3.65 | 0.091 |
| Error | 5 | 1.10375 | 0.22075 |  |  |
| Corrected Total | 10 | 5.12727273 |  |  |  |
| R-Square | Coeff Var | Root MSE | Nitrates Mean |  |  |
| 0.78473 | 59.06565 | 0.46984 | 0.795455 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 5 | 4.02352273 | 0.80470455 | 3.65 | 0.091 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 5 | 4.02352273 | 0.80470455 | 3.65 | 0.091 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Wells: Application 2 |  |  |  |  |  |
| Dependent Variable: Log E.coli |  | Model: Log E.coli = Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.838299 | Pr < W | 0.0092 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 7 | 1.33398635 | 0.19056948 | 10.66 | 0.0017 |
| Error | 8 | 0.14301932 | 0.01787741 |  |  |
| Corrected Total | 15 | 1.47700566 |  |  |  |
| R-Square | Coeff Var | Root MSE | LogEcoli Mean |  |  |
| 0.903169 | 6.772193 | 0.133706 | 1.974345 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 7 | 1.33398635 | 0.19056948 | 10.66 | 0.0017 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 7 | 1.33398635 | 0.19056948 | 10.66 | 0.0017 |
| Tukey Grouping | Mean | N | Date |  |  |
| A | 2.6552 | 2 | 11/2/2013 |  |  |
| B | 2.0783 | 2 | 10/17/2013 |  |  |
| B | 2.0293 | 2 | 11/8/2013 |  |  |
| B | 1.98 | 2 | 11/4/2013 |  |  |
| B | 1.8069 | 2 | 11/13/2013 |  |  |
| B | 1.7887 | 2 | 11/6/2013 |  |  |
| B | 1.7826 | 2 | 11/22/2013 |  |  |
| B | 1.6739 | 2 | 11/17/2013 |  |  |
| Wells: Application 2 |  |  |  |  |  |
| Dependent Variable: Nitrates |  | Model: Nitrate = Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.673313 | Pr < W | <0.0001 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 7 | 1.544375 | 0.220625 | 2.17 | 0.1505 |
| Error | 8 | 0.815 | 0.101875 |  |  |
| Corrected Total | 15 | 2.359375 |  |  |  |
| R-Square | Coeff Var | Root MSE | Nitrate Mean |  |  |
| 0.65457 | 42.20544 | 0.319179 | 0.75625 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 7 | 1.544375 | 0.220625 | 2.17 | 0.1505 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 7 | 1.544375 | 0.220625 | 2.17 | 0.1505 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Wells: Application 2 |  |  |  |  |  |
| Dependent Variable: Sqrt AllBac |  | Model: Sqrt AllBac = Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.839 | Pr < W | 0.0158 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 6 | 952.410657 | 158.735109 | 1.02 | 0.4799 |
| Error | 7 | 1084.356291 | 154.908042 |  |  |
| Corrected Total | 13 | 2036.766947 |  |  |  |
| R-Square | Coeff Var | Root MSE | AllBac Mean |  |  |
| 0.467609 | 91.50128 | 12.44621 | 13.60222 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 6 | 952.4106566 | 158.7351094 | 1.02 | 0.4799 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 6 | 952.4106566 | 158.7351094 | 1.02 | 0.4799 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Wells: Application 3 |  |  |  |  |  |
| Dependent Variable: Log E.coli |  | Model: SqrtLog E.coli = Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.833623 | Pr < W | 0.0079 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 7 | 4.81080603 | 0.687258 | 0.61 | 0.7348 |
| Error | 10 | 11.21110694 | 1.12111069 |  |  |
| Corrected Total | 17 | 16.02191297 |  |  |  |
| R-Square | Coeff Var | Root MSE | Ecoli Mean |  |  |
| 0.300264 | 66.34269 | 1.058825 | 1.595994 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 7 | 4.81080603 | 0.687258 | 0.61 | 0.7348 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 7 | 4.81080603 | 0.687258 | 0.61 | 0.7348 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Wells: Application 3 |  |  |  |  |  |
| Dependent Variable: Nitrates |  | Model: Nitrates = Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.759138 | Pr < W | 0.0008 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 7 | 26.66833333 | 3.8097619 | 1.56 | 0.2531 |
| Error | 10 | 24.45166667 | 2.44516667 |  |  |
| Corrected Total | 17 | 51.12 |  |  |  |
| R-Square | Coeff Var | Root MSE | Nitrate Mean |  |  |
| 0.521681 | 49.90541 | 1.563703 | 3.133333 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 7 | 26.66833333 | 3.8097619 | 1.56 | 0.2531 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 7 | 26.66833333 | 3.8097619 | 1.56 | 0.2531 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |
| Wells: Application 3 |  |  |  |  |  |
| Dependent Variable: LogAllBac |  | Model: LogAllBac = Date |  |  |  |
| Tests for Normality |  |  |  |  |  |
| Test | Statistic |  | p Value |  |  |
| Shapiro-Wilk | W | 0.566571 | Pr < W | <0.0001 |  |
| Source | DF | Sum of Squares | Mean Square | F Value | Pr > F |
| Model | 6 | 1685099328 | 280849888 | 0.77 | 0.6152 |
| Error | 9 | 3302818953 | 366979884 |  |  |
| Corrected Total | 15 | 4987918281 |  |  |  |
| R-Square | Coeff Var | Root MSE | AllBac Mean |  |  |
| 0.337836 | 138.3812 | 19156.72 | 13843.44 |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| Date | 6 | 1685099328 | 280849888 | 0.77 | 0.6152 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| Date | 6 | 1685099328 | 280849888 | 0.77 | 0.6152 |
| Tukey Grouping | Mean | N | Date |  |  |
| Test Not Applicable |  |  |  |  |  |

## Appendix C: Water Quality Data-Supplemental Data

| Site | Date | Cond (µs) | Res (KΩ) | TDS (ppm) | Orp (Mv) | pH | Temp (Co) | WT (ft) | Turbidity (ntu) | Nirtate (mg/L) | Coliforms (CFU/100mls) | E.coli (CFU/100mls) | AllBac (Copies/100mls) | BoBac (Copies/100mls) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D1 | 4/13/2013 | 190.3 |  | 124.2 | 136 | 7.32 | 13.7 |  |  | 9.8 | 2419.6 | 18.9 |  |  |
| D2 | 4/13/2013 | 198 |  | 130 | 146 | 7.36 | 13.7 |  |  | 8.7 | 2419.6 | 24.3 |  |  |
| D3 | 4/13/2013 | 210.1 |  | 140 | 120 | 7.15 | 13.48 |  |  | 5.6 | 1413.6 | 2 |  |  |
| D4 | 4/13/2013 | 213.5 |  | 140 | 164 | 7.09 | 13.5 |  |  | 5.8 | 2419.6 | 24.6 |  |  |
| D5 | 4/13/2013 | 326.6 |  | 216.7 | -15 | 7.05 | 13.7 |  |  | 4.9 | 2419.6 | 34.1 |  |  |
| D6 | 4/13/2013 | 370 |  | 251 | 226 | 7.14 | 13.4 |  |  | 3.3 | 2419.6 | 8.6 |  |  |
| LR1 | 4/13/2013 | 30.95 | 20.2 | 30.95 | 192 | 7.67 | 12.4 |  |  | 1.3 | 1011.2 | 39.3 |  |  |
| LR2 | 4/14/2013 | 34.21 | 20.9 | 27.39 | 202 | 7.43 | 12.5 |  |  |  | 1413.6 | 41.8 |  |  |
| S1 | 4/13/2013 | 191.4 |  | 124 | 147 | 7.47 | 14 |  |  | 4.2 | 2419.6 | 21.2 | 1030.19 | 16.70 |
| S2 | 4/13/2013 | 185.5 |  | 117.7 | 214 | 7.35 | 21.9 |  |  | 7.3 | 2419.6 | 106.3 | 416.49 | 16.70 |
| S3 | 4/13/2013 | 198.6 |  | 129.3 | 164 | 7.25 | 12.9 |  |  | 8.45 | 2419.6 | 393.3 | 353.45 |  |
| W10B | 4/13/2013 | 222 |  | 146 | 211 | 7.1 | 15.5 | 7.5 | 28 | 2.25 | 80.2 | 2 | 237.81 | 5.00 |
| W5A | 4/13/2013 | 118 |  | 75.4 | 765 | 5.95 | 15.6 | 2.46 | 25 | 4.85 | 1 | 1 | 5.00 | 5.00 |
| W5B | 4/13/2013 | 351 |  | 233.2 | -60 | 7.3 | 14.6 | 2.5 | 30 | 1.2 | 2 | 1 | 10.78 | 5.00 |
| W6 | 4/13/2013 | 406 |  | 273 | 43 | 7.07 | 25 | 0 |  | 1.55 | 2419.6 | 5.75 | 23506.52 | 5.00 |
| D1 | 5/9/2013 | 202.4 |  | 132.4 | 165 | 6.88 | 15.6 |  |  | 4.7 | 2419.6 | 24.1 |  |  |
| D2 | 5/9/2013 | 208.5 |  | 137.8 | 161 | 6.95 | 15.4 |  |  | 8.4 | 396.8 | 26.2 |  |  |
| D3 | 5/9/2013 | 207 |  | 135 | 129 | 7.5 | 14.7 |  |  | 2.15 | 2419.6 | 1.0 |  |  |
| D4 | 5/9/2013 | 214 |  | 139 | 125 | 7.5 | 14.1 |  |  | 2.7 | 2419.6 | 4.1 |  |  |
| D5 | 5/9/2013 | 306.8 |  | 200.8 | -82 | 7.24 | 16.5 |  |  | 1.4 | 2419.6 | 178.2 |  |  |
| D6 | 5/9/2013 | 379.8 |  | 258.3 | 98 | 7.22 | 14.7 |  |  | 5.7 | 866.4 | 3.1 |  |  |
| LR1 | 5/9/2013 | 53.08 | 18.65 | 35.13 | 99 | 7.93 | 15 |  |  |  | 1186.7 | 78.2 |  |  |
| LR2 | 5/9/2013 | 50.3 | 19.48 | 32.07 | 126 | 7.88 | 14.8 |  |  | 0.6 | 1916.6 | 58.6 |  |  |
| S1 | 5/9/2013 | 202.1 |  | 132.1 | 212 | 6.65 | 16.1 |  |  | 4.4 | 2419.6 | 19.4 |  |  |
| S2 | 5/9/2013 | 193.2 |  | 124.4 | 46 | 7.42 | 21.5 |  |  | 3.75 | 2419.6 | 36.1 |  |  |
| S3 | 5/9/2013 |  |  |  |  |  |  |  |  | 6.15 | 2419.6 | 126.5 |  |  |
| W10B | 5/9/2013 | 236.2 |  | 155.7 | 147 | 7.1 | 15.8 | 8.9 |  | 4 | 488.4 | 4.7 |  |  |
| W5A | 5/9/2013 | 120 |  | 76.13 | 192 | 6.2 | 17.4 | 2.45 |  | 6.8 | 579.4 | 1.0 |  |  |
| W5B | 5/9/2013 | 348 |  | 234.4 | -55 | 7.1 | 13.3 | 2.5 |  | 0.45 | 9.8 | 1.0 |  |  |
| W6 | 5/9/2013 | 440 |  | 307 | 57 | 7.3 | 17.1 | 0 |  | 9.3 | 37.4 | 7.0 |  |  |
| D1 | 5/13/2013 | 216 |  | 142.3 | 230 | 6.58 | 15.8 |  |  | 4.95 | 59.2 | 6.3 |  |  |
| D2 | 5/13/2013 | 214.1 |  | 140.6 | 211 | 6.85 | 15.2 |  |  | 4.5 | 86.1 | 7.5 |  |  |
| D3 | 5/13/2013 | 212.5 |  | 139.7 | 214 | 6.84 | 15.2 |  |  | 3.45 | 58.8 | 1 |  |  |
| D4 | 5/13/2013 | 201.6 |  | 131.8 | 186 | 6.89 | 14.5 |  |  | 3.45 | 2419.6 | 3 |  |  |
| D5 | 5/13/2013 |  |  |  |  |  |  |  |  |  |  | 1 |  |  |
| D6 | 5/13/2013 | 427.1 |  | 287.7 | 183 | 7.04 | 14.8 |  |  | 2.4 | 230 | 12.1 |  |  |
| S1 | 5/13/2013 | 214.9 |  | 142.9 | 204 | 6.95 | 15.6 |  |  | 4.05 | 2419.6 | 10.25 |  |  |
| S2 | 5/13/2013 | 205.6 |  | 131.7 | 185 | 7.22 | 25.2 |  |  | 3.55 | 2419.6 | 5.8 |  |  |
| S3 | 5/13/2013 | 211.8 |  | 134.6 | 164 | 7.57 | 29 |  |  | 3.3 | 2419.6 | 4.75 |  |  |
| D1 | 5/30/2013 | 232.5 |  | 150.2 | 226 | 6.84 | 18.1 |  | 19.1 | 3.8 | 2419.6 | 31.05 |  |  |
| D2 | 5/30/2013 | 229.7 |  | 151.8 | 214 | 7 | 18.1 |  | 16.4 | 3.55 | 2419.6 | 28.5 |  |  |
| D3 | 5/30/2013 | 243.1 |  | 156.2 | 165 | 6.8 | 18.1 |  | 7.26 | 3.05 | 2419.6 | 1732.9 |  |  |
| D4 | 5/30/2013 | 517 |  | 408.5 | 186 | 7.1 | 21.4 |  | 134 | 35 | 2419.6 | 2419.6 |  |  |
| D5 | 5/30/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 5/30/2013 | 489.7 |  | 334.5 | 179 | 6.88 | 17.5 |  | 28.8 | 2 | 2419.6 | 1553.1 |  |  |
| LR1 | 5/30/2013 | 81.7 | 12.19 | 51.83 | 155 | 7.66 | 19.5 |  | 4.68 | 1.05 | 2419.6 | 104.6 | 7964.23 | 270.82 |
| LR2 | 5/30/2013 | 85.14 | 11.9 | 52.46 | 104 | 7.76 | 20.1 |  | 5.1 | 0.75 | 2419.6 | 129.6 | 730.76 | 142.30 |
| S1 | 5/30/2013 | 198 |  | 149.6 | 195 | 6.92 | 18.1 |  | 21.5 | 4.05 | 2419.6 | 42.45 | 7779.86 | 428.83 |
| S2 | 5/30/2013 | 217.3 |  | 141.8 | 150 | 7.31 | 18 |  | 18.5 | 3.55 | 2419.6 | 359.1 | 214043.38 | 7242.70 |
| S3 | 5/30/2013 | 173.2 |  | 255.5 | 123 | 7.34 | 18.7 |  | 32.9 | 3.3 | 2419.6 | 568.6 | 198313.79 | 12013.23 |
| W10B | 5/30/2013 | 226.9 |  | 149.9 | 169 | 6.89 | 15.5 | 10.22 | 75.9 | 4.7 | 2419.6 | 14.65 | 362.00 | 5.00 |
| W5A | 5/30/2013 | 116.5 |  | 74.24 | 231 | 5.64 | 18.2 | 3.3 | 2.34 | 5.95 | 184.2 | 1 | 65.70 | 55.56 |
| W5B | 5/30/2013 | 355.2 |  | 239.2 | 60 | 7.18 | 16.2 | 3.5 | 2.11 | 0.65 | 1 | 1 | 11.75 | 24.65 |
| W6 | 5/30/2013 | 448.5 |  | 302.3 | 99 | 7.15 | 20.6 | 1.52 | 8.76 | 1.05 | 235.9 | 1 | 258.75 | 187.75 |
| D1 | 6/3/2013 | 270.1 |  | 175.6 | 184 | 6.96 | 19.4 |  | 51.9 | 8.95 | 2419.6 | 727 |  |  |
| D2 | 6/3/2013 | 268.4 |  | 174.9 | 174 | 7.08 | 19.5 |  | 70.3 | 11.1 | 2419.6 | 1732.9 |  |  |
| D3 | 6/3/2013 | 243.1 |  | 158.1 | 94 | 6.93 | 18.7 |  | 9.72 | 2.6 | 2419.6 | 360.9 |  |  |
| D4 | 6/3/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D5 | 6/3/2013 | 310.1 |  | 204.3 | 65 | 6.6 | 20.4 |  | 10.8 | 2.65 | 2419.6 | 2419.6 |  |  |
| D6 | 6/3/2013 | 398.8 |  | 268.4 | 251 | 6.98 | 18.8 |  | 7.33 | 3 | 2419.6 | 344.8 |  |  |
| LR1 | 6/3/2013 | 67.95 | 16.14 | 39.38 | 211 | 7.23 | 17 |  | 29.1 | 2.7 | 2419.6 | 766.95 | 83718.83 | 1235.60 |
| LR2 | 6/3/2013 | 62.39 | 16.03 | 39.25 | 91 | 8 | 18 |  | 24.2 | 1.25 | 2419.6 | 630.95 | 22408.87 | 399.24 |
| S1 | 6/3/2013 | 266.6 |  | 173.8 | 198 | 6.87 | 19.6 |  | 63.2 | 9.025 | 2419.6 | 908.15 | 39540.95 | 316.05 |
| S2 | 6/3/2013 | 219.7 |  | 142.7 | 217 | 7.22 | 20.3 |  | 57.3 | 7.775 | 2419.6 | 2419.6 | 37431.34 | 340.46 |
| S3 | 6/3/2013 | 235.4 |  | 153.2 | 245 | 7.07 | 19.9 |  | 61.4 | 8.575 | 2419.6 | 2419.6 | 4884.67 | 16.70 |
| W10B | 6/3/2013 | 225.8 |  | 148.2 | 268 | 6.95 | 16 | 9.81 | 8.38 | 0.9 | 2419.6 | 104.9 | 2512.40 | 5.00 |
| W5A | 6/3/2013 | 123.9 |  | 78.65 | 245 | 5.69 | 19.8 | 2.29 | 2.69 | 4.7 | 2419.6 | 6.9 | 226.16 | 5.00 |
| W5B | 6/3/2013 | 354.3 |  | 236.9 | -67 | 7.23 | 16.1 | 2.3 | 7.9 | 0.55 | 142.1 | 1 | 96.81 | 169.14 |
| W6 | 6/3/2013 | 420.2 |  | 284.9 | 24 | 7.2 | 22.4 | 0 | 12.8 | 1.9 | 2419.6 | 50.8 | 4643.87 | 91.33 |
| D1 | 6/8/2013 | 238 |  | 155.2 | 199 | 6.95 | 19.3 |  | 16.1 | 4 | 2419.6 | 3.1 |  |  |
| D2 | 6/8/2013 | 240.6 |  | 156.8 | 205 | 7.05 | 19.3 |  | 13.2 | 4.45 | 2419.6 | 325.5 |  |  |
| D3 | 6/8/2013 | 232.3 |  | 151.6 | 153 | 6.79 | 18.4 |  | 5.02 | 2.55 | 2419.6 | 13 |  |  |
| D4 | 6/8/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D5 | 6/8/2013 | 399.2 |  | 266.6 | -83 | 6.8 | 21.5 |  | 12.9 | 3.1 | 2419.6 | 172.2 |  |  |
| D6 | 6/8/2013 | 414.8 |  | 280.3 | 63 | 6.94 | 18.7 |  | 2.95 | 3.15 | 1732.9 | 209.8 |  |  |
| LR1 | 6/8/2013 | 361.1 | 27.19 | 23.23 | 70 | 7.83 | 18 |  | 16.4 | 0.55 | 2419.6 | 207.65 | 742.35 | 16.70 |
| LR2 | 6/8/2013 | 370.7 | 26.25 | 24.04 | 108 | 7.62 | 17.3 |  | 15.4 | 0.65 | 2419.6 | 79.75 | 1153.24 | 16.70 |
| S1 | 6/8/2013 | 236.5 |  | 154.2 | 219 | 6.75 | 19.6 |  | 20.8 | 4.1 | 2419.6 | 190.05 | 2517.30 | 16.70 |
| S2 | 6/8/2013 | 222.7 |  | 143.7 | 27 | 7.26 | 23.1 |  | 24.1 | 4.35 | 2419.6 | 23.9 | 243.59 | 16.70 |
| S3 | 6/8/2013 | 243.8 |  | 157.8 | 93 | 7.26 | 23 |  | 20.5 | 5 | 2419.6 | 16 | 417.37 | 16.70 |
| W10B | 6/8/2013 | 243.1 |  | 158.5 | 125 | 6.96 | 17.9 | 9.96 | 6.48 | 0.75 | 2419.6 | 2 | 42.38 | 5.00 |
| W5A | 6/8/2013 | 123.7 |  | 78.9 | 226 | 5.69 | 18 | 2.49 | 2.39 | 4.65 | 2419.6 | 1 | 27.99 | 5.00 |
| W5B | 6/8/2013 | 352 |  | 237.3 | -62 | 7.16 | 15.6 | 2.58 | 1.51 | 0.2 | 146.7 | 1 | 5.00 | 5.00 |
| W6 | 6/8/2013 | 417.7 |  | 278.3 | 18 | 7.16 | 23.4 | 0 | 24.3 | 1.35 | 2419.6 | 113.2 | 8911.25 | 5.00 |
| D1 | 6/16/2013 | 241.8 |  | 157.6 | 107 | 7.07 | 19.9 |  | 15 | 5.15 | 2419.6 | 24.3 |  |  |
| D2 | 6/16/2013 | 245.8 |  | 160.1 | 108 | 7.04 | 19.8 |  | 12.1 | 4.05 | 2419.6 | 13.4 |  |  |
| D3 | 6/16/2013 | 222.7 |  | 145.1 | 84 | 6.85 | 19.1 |  | 2.98 | 2.35 | 2419.6 | 1 |  |  |
| D4 | 6/16/2013 | 221.1 |  | 144.3 | 87 | 6.81 | 18 |  | 5.62 | 2.55 | 2419.6 | 6.3 |  |  |
| D5 | 6/16/2013 | 428.6 |  | 286.8 | -95 | 6.83 | 22.1 |  | 8.25 | 2.45 | 2419.6 | 55.6 |  |  |
| D6 | 6/16/2013 | 420.1 |  | 283.6 | 82 | 6.94 | 19.2 |  | 1.93 | 3.8 | 920.8 | 2 |  |  |
| LR1 | 6/16/2013 | 47.83 | 20.9 | 30.08 | 78 | 7.98 | 19.4 |  | 4.13 | 0.1 | 2419.6 | 108.35 | 400.97 |  |
| LR2 | 6/16/2013 | 47.13 | 20.79 | 29.91 | 125 | 7.8 | 17.8 |  | 2.7 | 0.15 | 2419.6 | 79.65 | 328.37 |  |
| S1 | 6/16/2013 | 240.8 |  | 156.8 | 112 | 7.09 | 19.8 |  | 23 | 4.4 | 2419.6 | 50.45 | 920.46 | 16.70 |
| S2 | 6/16/2013 | 226.5 |  | 146 | 15 | 7.28 | 24.4 |  | 20.2 | 3.6 | 2419.6 | 169.25 | 4504.83 | 16.70 |
| S3 | 6/16/2013 | 243.8 |  | 157.5 | 69 | 7.39 | 23.4 |  | 16.9 | 3.5 | 2419.6 | 82.1 | 42340.96 | 40.14 |
| W10B | 6/16/2013 | 248.8 |  | 162.6 | 128 | 6.89 | 17.6 | 9.77 | 7.21 | 0.25 | 1553.1 | 10.35 | 51.29 | 5.00 |
| W5A | 6/16/2013 | 128.3 |  | 81.74 | 246 | 5.53 | 19.1 | 2.58 | 1.77 | 4.95 | 1986.3 | 8 |  |  |
| W5B | 6/16/2013 | 352.7 |  | 236.2 | -58 | 7.01 | 15.9 | 3.73 | 1.97 | 0.1 | 68.3 | 1 |  |  |
| W6 | 6/16/2013 | 438.8 |  | 287.6 | 54 | 7.2 | 28.8 | 0.8 | 4.64 | 0.95 | 2419.6 | 223.1 | 15707.36 | 5.00 |
| D1 | 6/21/2013 | 243.7 |  | 158.2 | 115 | 6.94 | 20.7 |  | 12.7 | 3.5 | 2419.6 | 57.3 |  |  |
| D2 | 6/21/2013 | 246.8 |  | 160.2 | 112 | 7.08 | 20.9 |  | 10.2 | 3.6 | 2419.6 | 24.6 |  |  |
| D3 | 6/21/2013 | 217.9 |  | 140.7 | 136 | 6.79 | 19.4 |  | 2.67 | 2.4 | 2419.6 | 6.3 |  |  |
| D4 | 6/21/2013 | 227.3 |  | 147.1 | 141 | 6.59 | 19.3 |  | 3.91 | 2.4 | 2419.6 | 4.1 |  |  |
| D5 | 6/21/2013 | 392.6 |  | 261.5 | -113 | 6.77 | 22.2 |  | 16.9 | 2.2 | 2419.6 | 28.8 |  |  |
| D6 | 6/21/2013 | 428.8 |  | 289.3 | 181 | 6.99 | 19.5 |  | 3.73 | 3.3 | 2419.6 | 2.8924895 |  |  |
| LR1 | 6/21/2013 | 73 | 13.61 | 46.01 | 205 | 6.73 | 17.7 |  | 5.06 | 0.5 | 2419.6 | 95.9 | 16.70 | 16.70 |
| LR2 | 6/21/2013 | 56.92 | 17.5 | 36.09 | 105 | 8.13 | 18 |  |  | 0.45 | 2419.6 | 93.3 | 16.70 | 16.70 |
| S1 | 6/21/2013 | 241.6 |  | 157.3 | 105 | 6.96 | 20.7 |  | 13.1 | 4.35 | 2419.6 | 24.63054 | 16.70 | 16.70 |
| S2 | 6/21/2013 | 230.9 |  | 149.9 | 141 | 7.29 | 21.2 |  | 20.6 | 3.75 | 2419.6 | 101.7 | 16.70 | 16.70 |
| S3 | 6/21/2013 | 255.3 |  | 165.8 | 196 | 6.93 | 20.6 |  | 26 | 4.1 | 2419.6 | 69.1 | 16.70 | 16.70 |
| W10B | 6/21/2013 | 259.9 |  | 170.3 | 173 | 6.88 | 17.1 | 9.72 | 26 | 3.05 | 1413.6 | 9.8 | 5.00 | 5.00 |
| W5A | 6/21/2013 | 138.2 |  | 87.86 | 213 | 5.71 | 20.6 | 2.44 | 2.01 | 6.3 | 2419.6 | 40.4 |  |  |
| W5B | 6/21/2013 | 350.5 |  | 234.3 | -71 | 7.24 | 18 | 2.49 | 4.19 | 1 | 43.5 | 2 |  |  |
| W6 | 6/21/2013 | 417.3 |  | 278.6 | -11 | 7.21 | 22.3 | 0 | 10.2 | 1.25 | 2419.6 | 57.3 | 5.00 | 5.00 |
| D1 | 6/30/2013 | 241.2 |  | 156.9 | 52 | 7.06 | 20.06 |  | 19.3 | 5.1 | 2419.6 | 1 |  |  |
| D2 | 6/30/2013 | 245.8 |  | 159.8 | 61 | 7.1 | 20.5 |  | 18.4 | 4.8 | 2419.6 | 67.7 |  |  |
| D3 | 6/30/2013 | 220.8 |  | 143.5 | 60 | 6.33 | 19.9 |  | 3.39 | 3 | 2419.6 | 6.3900562 |  |  |
| D4 | 6/30/2013 | 235.1 |  | 153.1 | 71 | 6.94 | 19.4 |  | 4.12 | 2.8 | 2419.6 | 8.4 |  |  |
| D5 | 6/30/2013 | 393.6 |  | 261.6 | -115 | 6.81 | 22.9 |  | 8.12 | 3.5 | 2419.6 | 1203.3 |  |  |
| D6 | 6/30/2013 | 422.7 |  | 284.8 | 159 | 7.02 | 20.2 |  | 1.49 | 3.3 | 1553.1 | 1 |  |  |
| LR1 | 6/30/2013 | 65.18 | 15.16 | 41.71 | 128 | 8.08 | 17.5 |  | 8.44 | 0.9 | 2419.6 | 178.5 | 48.12 | 16.70 |
| LR2 | 6/30/2013 | 65.5 | 15.06 | 46.02 | 32 | 7.94 | 18.1 |  | 4.62 | 1.4 | 2419.6 | 129.6 | 16.70 | 16.70 |
| S1 | 6/30/2013 | 240.1 |  | 156 | 48 | 7.1 | 21.6 |  | 20.5 | 5.2 | 2419.6 | 32.3 | 36.47 |  |
| S2 | 6/30/2013 | 229.5 |  | 148 | 129 | 7.32 | 25 |  | 21 | 4.9 | 2419.6 | 206.4 | 1382.95 |  |
| S3 | 6/30/2013 | 251.9 |  | 163.8 | 149 | 7.61 | 20.4 |  | 20.3 | 4.2 | 2419.6 | 517.2 | 1717.64 |  |
| W10B | 6/30/2013 | 266.6 |  | 174.1 | 168 | 7.08 | 18.7 | 9.2 | 2.94 | 0.9 | 770.1 | 2.498666 | 5.00 |  |
| W5A | 6/30/2013 | 141 |  | 89.1 | 162 | 5.74 | 21.2 | 2.45 | 2 | 0.8 | 13.2 | 1 |  |  |
| W5B | 6/30/2013 | 351.2 |  | 234.4 | -91 | 7.28 | 19.3 |  | 1.4 | 6.7 |  | 1 |  |  |
| W6 | 6/30/2013 | 421.5 |  | 277.8 | -11 | 7.09 | 26.8 | 0 | 3.89 | 0.9 | 2419.6 | 160.7 | 553.22 |  |
| D1 | 7/20/2013 | 246.8 |  | 160.7 | 151 | 7.03 | 21.7 |  | 6.24 | 3.5 | 2419.6 | 16 |  |  |
| D2 | 7/20/2013 | 251.8 |  | 163.5 | 146 | 7.12 | 21.2 |  | 6.18 | 3.3 | 1986.3 | 12.1 |  |  |
| D3 | 7/20/2013 | 222.1 |  | 144.6 | 148 | 7 | 21.1 |  | 2.27 | 3.1 | 613.1 | 1 |  |  |
| D4 | 7/20/2013 | 206.1 |  | 133.7 | 141 | 7.01 | 19.1 |  | 2.26 | 3.1 | 1553.1 | 1 |  |  |
| D5 | 7/20/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 7/20/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LR1 | 7/20/2013 | 68.6 | 14.52 | 43.56 | 160 | 6.88 | 21 |  | 2.79 | 0.8 | 2419.6 | 122.3 | 420.87 | 16.70 |
| LR2 | 7/20/2013 | 69 | 14.42 | 42.11 | 87 | 7.82 | 21.2 |  | 2.15 | 1 | 2419.6 | 90.8 | 35.21 | 16.70 |
| S1 | 7/20/2013 |  |  |  |  |  |  |  |  |  | 2419.6 | 23.2 | 36.45 | 16.70 |
| S2 | 7/20/2013 | 225.3 |  | 145.6 | 153 | 7.4 | 23.3 |  |  | 3.1 | 2419.6 | 118.7 | 16.70 | 16.70 |
| S3 | 7/20/2013 | 238.1 |  | 153.6 | 167 | 6.93 | 24.6 | 8.73 | 7.81 | 3 | 2419.6 | 272.83565 |  |  |
| W10B | 7/20/2013 | 276.3 |  | 179.8 | 163 | 7.07 | 20.2 | 9.87 | 1.59 | 0.8 | 109.5 | 1 | 5.00 | 5.00 |
| W5A | 7/20/2013 | 135.7 |  | 85.9 | 223 | 5.53 | 22.9 | 3.29 | 1.73 | 6.2 | 2419.6 | 2 | 5.00 | 5.00 |
| W5B | 7/20/2013 | 362.6 |  | 242.1 | -80 | 7.24 | 19.9 | 3.51 | 1.14 | 1.7 | 1986.3 | 1 | 5.00 | 5.00 |
| W6 | 7/20/2013 | 437.7 |  | 285.6 | 112 | 7.17 | 31.7 | 1.82 | 2.35 | 0.8 | 235.9 | 6.3 | 41.42 | 5.00 |
| D1 | 8/4/2013 | 254.9 |  | 165.5 | 166 | 6.84 | 20.8 |  | 8.76 | 4.1 | 2419.6 | 22.8 |  |  |
| D2 | 8/4/2013 | 259.2 |  | 168.6 | 174 | 7.17 | 21.2 |  | 5.06 | 3.6 | 2419.6 | 17.1 |  |  |
| D3 | 8/4/2013 | 231.5 |  | 148 | 179 | 6.7 | 21.3 |  | 2.41 | 3.8 | 1046.2 | 4.1 |  |  |
| D4 | 8/4/2013 | 214.1 |  | 139.6 | 197 | 6.62 | 20 |  | 2.41 | 3.3 | 1046.2 | 3.1 |  |  |
| D5 | 8/4/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 8/4/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LR1 | 8/4/2013 | 72.08 | 11.82 | 45.77 | 186 | 7.15 | 21.2 |  | 2.43 | 1 | 2419.6 | 96 | 213.10 | 16.70 |
| LR2 | 8/4/2013 | 72.7 | 13.69 | 46.13 | 103 | 7.85 | 21.7 |  | 2.31 | 1.1 | 2419.6 | 105 | 52.37 | 16.70 |
| S1 | 8/4/2013 | 250.6 |  | 162.3 | 181 | 6.99 | 22.4 |  | 3.5 | 3.6 | 2419.6 | 18.9 | 16.70 |  |
| S2 | 8/4/2013 | 224.4 |  | 144.7 | 161 | 7.49 | 25.2 |  | 15.2 | 3.7 | 2419.6 | 80.9 | 884.54 |  |
| S3 | 8/4/2013 | 245.5 |  | 157 | 205 | 7 | 24.1 |  | 9.23 | 3.9 | 2419.6 | 28.2 |  |  |
| W10B | 8/4/2013 | 247.1 |  | 160.2 | 162 | 6.96 | 20.8 | 10.37 | 1.43 | 3.8 | 191.8 | 2 | 5.00 |  |
| W5A | 8/4/2013 | 129.1 |  | 81.79 | 238 | 5.44 | 23 | 3.61 | 1.66 | 4.5 | 224.7 | 1 |  |  |
| W5B | 8/4/2013 | 359.6 |  | 239.5 | -64 | 7.16 | 19.5 | 3.8 | 1.57 | 1.5 | 2419.6 | 1 |  |  |
| W6 | 8/4/2013 | 453.8 |  | 298.2 | 113 | 7.1 | 20.1 | 1.72 | 5.8 | 2.3 |  | 9.8 | 433.23 |  |
| D1 | 8/20/2013 | 253.1 |  | 164.4 | 195 | 6.69 | 22 |  | 18.1 | 5.5 | 2420 | 40.3 |  |  |
| D2 | 8/20/2013 | 259.7 |  | 168.4 | 185 | 6.96 | 21.8 |  | 1.45 | 4.9 | 2420 | 43 |  |  |
| D3 | 8/20/2013 | 256.2 |  | 166.1 | 156 | 6.76 | 21.6 |  | 2.48 | 3.3 | 2420 | 24 |  |  |
| D4 | 8/20/2013 | 235.3 |  | 152.9 | 140 | 6.79 | 21.6 |  | 5.16 | 3.5 | 2420 | 48 |  |  |
| D5 | 8/20/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 8/20/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LR1 | 8/20/2013 | 96.4 | 13.45 | 46.73 | 236 | 7.37 | 20 |  | 8.43 | 1.5 | 2420 | 49 | 676.33 | 16.70 |
| LR2 | 8/20/2013 | 71.96 | 13.7 | 46.02 | 109 | 7.87 | 20.5 |  | 7.3 | 1.4 | 2420 | 48 | 259.33 | 16.70 |
| S1 | 8/20/2013 |  |  |  |  |  |  |  |  | 5 | 2420 | 39 | 16.70 | 16.70 |
| S2 | 8/20/2013 |  |  |  |  |  |  |  |  | 4.3 | 2420 | 44 | 67.23 | 16.70 |
| S3 | 8/20/2013 |  |  |  |  |  |  |  |  | 11.1 | 2420 | 49 | 189.72 |  |
| W10B | 8/20/2013 | 242.6 |  | 157.7 | 181 | 7.06 | 20.9 | 10.56 | 7.42 | 1.1 | 850 | 10 | 965.77 | 5.00 |
| W5A | 8/20/2013 | 116.9 |  | 74.09 | 183 | 5.67 | 22.7 | 2.72 | 1.45 | 2.4 | 232 | 21 | 5388.87 | 5.00 |
| W5B | 8/20/2013 | 361.6 |  | 241.4 | -68 | 7.18 | 18.7 | 2.96 | 7.3 | 1.7 | 2420 | 2 | 5.00 | 5.00 |
| W6 | 8/20/2013 | 455.5 |  | 299.5 | 84 | 7.21 | 29.8 | 1.6 | 3.91 | 3 | 650 | 1 | 556.15 | 5.00 |
| D1 | 9/14/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D2 | 9/14/2013 | 280 |  | 182.8 | 154 | 7.28 | 22.8 |  | 4.55 | 2.2 | 2419.6 | 45.7 |  |  |
| D3 | 9/14/2013 | 253.5 |  | 164.2 | 118 | 6.88 | 22.4 |  | 2.2 | 1.5 | 2419.6 | 4.1 |  |  |
| D4 | 9/14/2013 | 214.5 |  | 138.9 | 110 | 7.04 | 21 |  | 1.74 | 1.8 | 2419.6 | 15.8 |  |  |
| D5 | 9/14/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 9/14/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LR1 | 9/14/2013 | 69.31 | 14.32 | 44.12 | 177 | 7.03 | 18.6 |  | 6.88 | 0.3 | 2419.6 | 307.03333 | 1701.51 | 16.70 |
| LR2 | 9/14/2013 | 68.01 | 14.63 | 42.82 | 91 | 7.93 | 20.1 |  | 5.56 | 0.8 | 2419.6 | 185 | 1422.89 | 214.00 |
| S1 | 9/14/2013 | 274.1 |  | 178.2 | 165 | 7.01 | 22.1 |  | 35.6 | 4.3 | 2419.6 | 56.5 | 16.70 |  |
| S2 | 9/14/2013 | 227.6 |  | 146.9 | 92 | 8.43 | 24.3 |  | 15.9 | 1.6 | 2419.6 | 11.9 | 69.73 |  |
| S3 | 9/14/2013 | 216.9 |  | 141.4 | 181 | 7.39 | 18.3 |  | 5.33 | 1.5 | 2419.6 | 139.1 | 4160.73 |  |
| W10B | 9/14/2013 | 262.8 |  | 169.9 | 42 | 6.88 | 25.5 | 10.73 | 7.13 | 0.6 | 2419.6 | 9.7 | 103.26 |  |
| W5A | 9/14/2013 | 123.3 |  | 78.14 | 162 | 5.76 | 23.1 | 4.83 | 5 | 3.2 | 980.4 | 16.1 |  |  |
| W5B | 9/14/2013 | 360.5 |  | 239.4 | -79 | 7.2 | 21.4 | 4.98 | 6.57 | 0.1 | 435.2 | 1 |  |  |
| W6 | 9/14/2013 | 469.1 |  | 311.6 | 61 | 7.19 | 25.6 | 2.83 | 13.1 | 0.2 | 2419.6 | 3.1 | 11.55 |  |
| D1 | 10/17/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D2 | 10/17/2013 | 281.4 |  | 184.2 | 134 | 7.36 | 19.6 |  | 3.31 | 1.9 | 1046.2 | 6.3 |  |  |
| D3 | 10/17/2013 | 261.7 |  | 170.2 | 134 | 6.94 | 20.1 |  | 2.88 | 2 | 1413.6 | 1 |  |  |
| D4 | 10/17/2013 | 224.1 |  | 146 | 133 | 7.05 | 19.3 |  | 3.3 | 2.2 | 2419.6 | 2 |  |  |
| D5 | 10/17/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 10/17/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LR1 | 10/17/2013 | 98.9 | 10.57 | 59.7 | 227 | 7.35 | 17.85 |  | 2.59 | 0.4 | 2419.6 | 186 | 103.94 |  |
| LR2 | 10/17/2013 | 89.46 | 11.09 | 56.95 | 72 | 7.65 | 18.7 |  | 5.5 | 0.6 | 2419.6 | 77.1 | 1161.81 |  |
| S1 | 10/17/2013 | 27.3 |  | 179.2 | 132 | 7.17 | 19.6 |  | 15.1 | 1.8 | 2419.6 | 26.2 | 16.70 |  |
| S2 | 10/17/2013 | 264.8 |  | 172.6 | 108 | 7.45 | 19 |  | 12.8 | 3.1 | 2419.6 | 686.7 | 16.70 |  |
| S3 | 10/17/2013 | 231.1 |  | 160 | 262 | 7.07 | 19.1 |  | 12.2 | 0.8 | 2419.6 | 2419.6 | 103635.13 |  |
| W10B | 10/17/2013 | 239.7 |  | 155.4 | 40 | 7.16 | 20.1 | 10.72 | 9.4 | 0.8 | 2419.6 | 14.8 | 12.35 |  |
| W5A | 10/17/2013 | 132.7 |  | 84.57 | 121 | 5.99 | 19.6 | 4.82 | 17.4 | 4.1 | 2419.6 | 5.2 |  |  |
| W5B | 10/17/2013 | 370.5 |  | 247.6 | -56 | 7.17 | 19 | 4.9 | 4.49 | 1.1 | 1553.1 | 3 |  |  |
| W6 | 10/17/2013 | 454.1 |  | 306.8 | 95 | 7.21 | 20.4 | 3 | 30.5 | 1.2 | 2419.6 | 9.8 | 383.11 |  |
| D1 | 11/2/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D2 | 11/2/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D3 | 11/2/2013 | 238.3 |  | 155.8 | 131 | 7.24 | 17.7 |  | 2.25 | 3.1 | 1553.1 | 1 |  |  |
| D4 | 11/2/2013 | 209.7 |  | 136.6 | 12.5 | 7.46 | 17.5 |  | 1.3 | 3.3 | 2419.6 | 3.1 |  |  |
| D5 | 11/2/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 11/2/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LR1 | 11/2/2013 | 94.9 | 10.41 | 61.07 | 201 | 7.47 | 14.8 |  | 2.65 | 0.8 | 2419.6 | 613.1 | 540.54 |  |
| LR2 | 11/2/2013 | 91.85 | 10.5 | 59.1 | 137 | 7.67 | 15.8 |  | 3.08 | 0.8 | 2419.6 | 333.33333 | 334.62 |  |
| S1 | 11/2/2013 |  |  |  |  |  |  |  |  |  |  |  | 16.70 |  |
| S2 | 11/2/2013 | 235.1 |  | 154.7 | 90 | 8.41 | 14.6 |  | 8.48 | 3.4 | 2419.6 | 75.2 | 195.35 |  |
| S3 | 11/2/2013 | 254 |  | 166.2 | 120 | 7.45 | 17.3 |  | 17.2 | 4.8 | 2419.6 | 33.1 |  |  |
| W10B | 11/2/2013 | 240.5 |  | 157 | 66 | 7.36 | 19 | 10.94 | 3.91 | 1.2 | 1553.1 | 58.6 | 11.50 |  |
| W5A | 11/2/2013 | 124.9 |  | 79.71 | 140 | 6.37 | 19.2 | 5.37 | 2.23 | 3.9 | 360.9 | 1 |  |  |
| W5B | 11/2/2013 | 366.5 |  | 244.4 | -65 | 7.17 | 20.3 | 5.58 | 1.47 | 1.6 | 980.4 | 1 |  |  |
| W6 | 11/2/2013 | 418.4 |  | 282.6 | 102 | 6.75 | 17.2 | 3.48 | 1.56 | 1 | 2419.6 | 29.5 | 3172.06 |  |
| D1 | 11/4/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D2 | 11/4/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D3 | 11/4/2013 | 240.9 |  | 157 | 152 | 7.06 | 17.9 |  | 3.6 | 1.7 | 1413.6 | 1 |  |  |
| D4 | 11/4/2013 | 210.4 |  | 136.5 | 159 | 7 | 17.9 |  | 1.59 | 2.9 | 1413.6 | 1 |  |  |
| D5 | 11/4/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 11/4/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LR1 | 11/4/2013 | 85.84 | 11.46 | 55.91 | 147 | 7.05 | 11.2 |  | 1.22 | 0.8 | 499.6 | 111.2 | 33.47 |  |
| LR2 | 11/4/2013 | 84.65 | 11.66 | 54.54 | 118 | 8.15 | 13.3 |  | 4.33 | 0.9 | 2419.6 | 82 | 16.70 |  |
| S1 | 11/4/2013 |  |  |  |  |  |  |  |  |  |  |  | 16.70 |  |
| S2 | 11/4/2013 | 224.7 |  | 148 | -106 | 8.44 | 11.2 |  | 8.15 | 2.4 | 2419.6 | 25.8 | 142.78 |  |
| S3 | 11/4/2013 | 250.1 |  | 162.7 | 150 | 7.23 | 18 |  | 11.6 | 3.5 | 2419.6 | 1732.9 |  |  |
| W10B | 11/4/2013 | 247.3 |  | 162 | 63 | 7.46 | 18 | 11.2 | 6.17 | 3.9 | 1299.7 | 35.5 | 5975.08 |  |
| W5A | 11/4/2013 | 118.8 |  | 76.11 | 148 | 6.28 | 18.3 |  | 2.11 | 1.7 | 2419.6 | 1 |  |  |
| W5B | 11/4/2013 | 367.7 |  | 245.7 | -43 | 7.19 | 19.8 | 5.78 | 7.15 | 1.3 | 1553.1 | 1 |  |  |
| W6 | 11/4/2013 | 435.8 |  | 294.6 | 48 | 7.2 | 18 | 3.64 | 5.18 | 0.5 | 2419.6 | 13.4 | 15.47 |  |
| D1 | 11/6/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D2 | 11/6/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D3 | 11/6/2013 | 242.1 |  | 158.6 | 133 | 6.84 | 16.8 |  | 8.51 | 2.7 | 1732.9 | 2 |  |  |
| D4 | 11/6/2013 | 213 |  | 139.1 | 133 | 6.9 | 16.7 |  | 3.31 | 3 | 1732.9 | 2 |  |  |
| D5 | 11/6/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 11/6/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LR1 | 11/6/2013 | 90.48 | 10.88 | 58.87 | 162 | 6.94 | 11.5 |  | 4.8 | 0.8 | 2419.6 | 73.8 | 359.94 | 16.70 |
| LR2 | 11/6/2013 | 91.9 | 11.1 | 55.35 | 150 | 7 | 11.4 |  | 2.71 | 0.7 | 2419.6 | 51.2 | 469.93 | 16.70 |
| S1 | 11/6/2013 |  |  |  |  |  |  |  |  |  |  |  | 16.70 | 16.70 |
| S2 | 11/6/2013 | 259.3 |  | 171.6 | 167 | 7.45 | 10.8 |  | 11.9 | 2.1 | 2419.6 | 727 | 1013.05 | 16.70 |
| S3 | 11/6/2013 | 253.2 |  | 161 | 132 | 7.04 | 11 |  | 12.7 | 3.3 | 2419.6 | 30.9 |  |  |
| W10B | 11/6/2013 | 372.8 |  | 254.4 | 132 | 7.29 | 11.4 | 11.32 | 31.1 | 1.3 | 416 | 18.9 | 5.00 | 5.00 |
| W5A | 11/6/2013 | 121.7 |  | 77.58 | 121 | 6.56 | 19 | 5.5 | 3.08 | 3.4 | 1119.9 | 1 |  |  |
| W5B | 11/6/2013 | 366.8 |  | 245.5 | -17 | 7.11 | 19.1 | 5.7 | 9.78 | 1.1 | 920.8 | 1 |  |  |
| W6 | 11/6/2013 | 449.6 |  | 304.7 | 44 | 7.3 | 19.2 | 3.71 | 21.1 | 1 | 2419.6 | 1 | 5.00 | 5.00 |
| D1 | 11/8/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D2 | 11/8/2013 | 257.4 |  | 169.3 | 163 | 7.51 | 15.3 |  | 4.61 | 3.5 |  | 1 |  |  |
| D3 | 11/8/2013 | 241.1 |  | 157.6 | 196 | 6.85 | 17.5 |  | 13.7 | 3.3 | 1733 | 1 |  |  |
| D4 | 11/8/2013 | 211.3 |  | 138.3 | 148 | 6.99 | 16.4 |  | 4.63 | 2.4 | 579 | 2 |  |  |
| D5 | 11/8/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 11/8/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LR1 | 11/8/2013 | 88.62 | 11.11 | 57.31 | 99 | 8.04 | 12.7 |  | 12.8 | 0.9 | 2420 | 125.9 | 927.94 | 16.70 |
| LR2 | 11/8/2013 | 94.35 | 10.41 | 61.68 | 160 | 6.47 | 10.3 |  | 3.24 | 2.1 | 2420 | 90.9 | 16.70 | 16.70 |
| S1 | 11/8/2013 |  |  |  |  |  |  |  |  |  |  |  | 38.19 | 16.70 |
| S2 | 11/8/2013 | 232.9 |  | 154 | 124 | 8.11 | 12.1 |  | 15.2 | 2.7 | 2420 | 172.3 | 214.72 | 16.70 |
| S3 | 11/8/2013 | 247.1 |  | 161.2 | 171 | 7.27 | 17 |  | 21.1 | 4.4 | 2420 | 47.1 |  |  |
| W10B | 11/8/2013 | 249.1 |  | 162.9 | 95 | 7.19 | 18.1 | 10.79 | 23.1 | 2.6 | 512 | 193.5 | 14.78 | 5.00 |
| W5A | 11/8/2013 | 116.61 |  | 75.35 | 141 | 6.44 | 13.2 | 5.5 | 3.86 | 3.7 | 1542 | 1 |  |  |
| W5B | 11/8/2013 | 375.9 |  | 254.3 | -42 | 7.35 | 18.5 | 5.76 | 2.2 | 1.1 | 876 | 1 |  |  |
| W6 | 11/8/2013 | 428.7 |  | 291.6 | 56 | 7.42 | 16.6 | 3.58 | 22.1 | 0.9 | 2420 | 3 | 10299.36 | 5.00 |
| D1 | 11/13/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D2 | 11/13/2013 | 260 |  | 165.7 | 171 | 7.39 | 14.2 |  | 5 | 3.3 | 178.2 | 1 |  |  |
| D3 | 11/13/2013 | 239 |  | 149.9 | 176 | 7 | 14.1 |  | 4.62 | 3.3 | 1732.9 | 1 |  |  |
| D4 | 11/13/2013 | 214.2 |  | 143.4 | 170 | 7.34 | 13.8 |  | 5.05 | 2 | 293.3 | 1 |  |  |
| D5 | 11/13/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 11/13/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LR1 | 11/13/2013 | 87.64 | 11.92 | 57.55 | 170 | 7.03 | 9.8 |  | 2.76 | 0.6 | 2419.6 | 64.4 | 75.86 | 16.70 |
| LR2 | 11/13/2013 | 80.88 | 11.34 | 53 | 84 | 7.83 | 11 |  | 2.34 | 0.4 | 2419.6 | 63.8 | 16.70 | 16.70 |
| S1 | 11/13/2013 |  |  |  |  |  |  |  |  |  |  |  | 35.60 | 16.70 |
| S2 | 11/13/2013 | 229.1 |  | 153 | 160 | 7.55 | 13.5 |  | 10.1 | 3 | 2419.6 | 260.3 | 5048.68 | 16.70 |
| S3 | 11/13/2013 | 233 |  | 160 | 189 | 7.27 | 15.5 |  | 12.2 | 1.6 | 2419.6 | 100.8 |  |  |
| W10B | 11/13/2013 | 243.3 |  | 145.8 | 140 | 7.17 | 18.9 | 10.87 | 4.85 | 1.7 | 1413.6 | 60.2 | 760.04 | 5.00 |
| W5A | 11/13/2013 | 133 |  | 84.5 | 118 | 6.36 | 20.6 | 5.5 | 11 | 1.9 | 648.8 | 1 |  |  |
| W5B | 11/13/2013 | 369.7 |  | 245.8 | -38 | 7.2 | 22.5 | 5.68 | 8.66 | 1 | 106.7 | 1 |  |  |
| W6 | 11/13/2013 | 442.5 |  | 296.9 | 140 | 7.07 | 22 |  | 7.85 | 0.4 | 2419.6 | 1 | 66.03 | 5.00 |
| D1 | 11/17/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D2 | 11/17/2013 | 262.2 |  | 172.7 | 172 | 7.45 | 14.5 |  | 4.7 | 3.4 | 162.4 | 1 |  |  |
| D3 | 11/17/2013 | 233.5 |  | 153.9 | 175 | 7.1 | 13.9 |  | 4.77 | 3.2 | 461.1 | 1 |  |  |
| D4 | 11/17/2013 | 217.1 |  | 143.4 | 170 | 7.34 | 13.8 |  | 5 | 2.2 | 547.5 | 1 |  |  |
| D5 | 11/17/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 11/17/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LR1 | 11/17/2013 | 87.64 | 11.22 | 57.45 | 168 | 7 | 9.2 |  | 2.55 | 0.8 | 131.4 | 51.2 |  |  |
| LR2 | 11/17/2013 | 80.88 | 12.14 | 53 | 84 | 7.95 | 10.5 |  | 2.34 | 0.5 | 509.9 | 43.5 | 34.44 |  |
| S1 | 11/17/2013 |  |  |  |  |  |  |  |  |  |  |  | 16.70 |  |
| S2 | 11/17/2013 | 234.2 |  | 154.1 | 164 | 7.72 | 13.9 |  | 9.33 | 3.1 | 2419.6 | 49.5 | 3086.10 |  |
| S3 | 11/17/2013 | 246.5 |  | 162 | 173 | 7.27 | 15.5 |  | 12.3 | 2 | 2419.6 | 160.7 |  |  |
| W10B | 11/17/2013 | 245.7 |  | 160.1 | 140 | 7.17 | 18.9 | 10.87 | 4.85 | 2.1 | 2419.6 | 50.4 | 153.28 |  |
| W5A | 11/17/2013 | 133 |  | 84.5 | 118 | 6.36 | 20.6 | 5.5 | 11 | 4.9 | 436.6 | 1 |  |  |
| W5B | 11/17/2013 | 369.7 |  | 245.8 | -50 | 7.24 | 21.5 | 5.68 | 8.66 | 1.1 | 2419.6 | 1 |  |  |
| W6 | 11/17/2013 | 442.5 |  | 296.9 | 142 | 7.07 | 22.2 | 4 | 24.4 | 1 | 281.2 | 1 | 381.28 |  |
| D1 | 11/22/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D2 | 11/22/2013 | 260.5 |  | 171.5 | 200 | 7.56 | 14.2 |  | 6.11 | 3.6 | 2419.6 | 7.5 |  |  |
| D3 | 11/22/2013 | 241.8 |  | 159.4 | 209 | 7.34 | 14 |  | 2.69 | 2.3 | 1732.9 | 1 |  |  |
| D4 | 11/22/2013 | 234.8 |  | 154.7 | 195 | 7.6 | 14 |  | 2.21 | 1.8 | 2419.6 | 6.3 |  |  |
| D5 | 11/22/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 11/22/2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LR1 | 11/22/2013 | 67.45 | 15.14 | 42.47 | 163 | 7.32 | 9.5 |  | 3.2 | 0.5 | 2419.6 | 63.8 | 16.70 |  |
| LR2 | 11/22/2013 | 65 | 15.26 | 42 | 166 | 7.3 | 10 |  | 1.62 | 0.5 | 2419.6 | 57.6 | 39.73 |  |
| S1 | 11/22/2013 |  |  |  |  |  |  |  |  |  |  |  | 16.70 |  |
| S2 | 11/22/2013 | 231.3 |  | 152.6 | 187 | 7.87 | 12.3 |  | 15.5 | 2.6 | 2419.6 | 146.7 | 3657.12 |  |
| S3 | 11/22/2013 | 246.8 |  | 162.5 | 211 | 7.38 | 14.7 |  | 12.5 | 4 | 2419.6 | 31.3 |  |  |
| W10B | 11/22/2013 | 234.8 |  | 154.3 | 115 | 7.35 | 15.4 | 10.77 | 26.4 | 2.6 | 192.7 | 51.2 | 6897.10 |  |
| W5A | 11/22/2013 | 266.6 |  | 81.35 | 128 | 6.66 | 15.7 | 5.38 | 75.9 | 5.6 | 2419.6 | 1 |  |  |
| W5B | 11/22/2013 | 373.1 |  | 250.6 | -65 | 7.19 | 19.1 |  | 6.31 | 1.2 | 65.2 | 1 |  |  |
| W6 | 11/22/2013 | 437.5 |  | 299 | 123 | 7.19 | 15 | 2 | 3.71 | 0.4 | 1986.3 | 1 | 5.00 |  |
| D1 | 12/11/2013 | 185.11 |  | 122.2 | 159 | 7.01 | 8.7 |  | 4.6 | 5.4 | 2419.6 | 20.9 |  |  |
| D2 | 12/11/2013 | 190.3 |  | 125.8 | 159 | 7 | 8.8 |  | 3.9 | 4.4 | 2419.6 | 12.2 |  |  |
| D3 | 12/11/2013 | 210.9 |  | 139 | 164 | 6.84 | 13.2 |  | 5.3 | 3.5 | 2419.6 | 1 |  |  |
| D4 | 12/11/2013 | 208.5 |  | 137.3 | 161 | 6.75 | 12.3 |  | 3.1 | 3.8 | 2419.6 | 191.8 |  |  |
| D5 | 12/11/2013 | 324.4 |  | 218.5 | 74 | 7.15 | 11.7 |  | 5.3 | 3.1 | 2419.6 | 18.7 |  |  |
| D6 | 12/11/2013 | 129 |  | 251.6 | 129 | 7.13 | 12.2 |  | 6.4 | 5.1 | 1986.3 | 1 |  |  |
| LR1 | 12/11/2013 | 41.77 | 23.22 | 27.5 | 91 | 7.75 | 6.6 |  | 10.8 | 0.6 | 1413.6 | 31.3 | 309.99 | 16.70 |
| LR2 | 12/11/2013 | 46.56 | 22.24 | 28.36 | 124 | 7.76 | 6.6 |  | 8.9 | 0.9 | 1732.9 | 27.5 | 779.64 | 16.70 |
| S1 | 12/11/2013 | 133.5 |  | 120.8 | 159 | 6.95 | 8.8 |  | 32.9 | 6.2 | 2419.6 | 17.3 | 71.59 |  |
| S2 | 12/11/2013 | 183.9 |  | 121.2 | 130 | 7.32 | 9.8 |  | 43.9 | 6.6 | 2419.6 | 90.6 | 75.95 |  |
| S3 | 12/11/2013 | 205.1 |  | 136.5 | 123 | 7.43 | 7.4 |  | 25.8 | 4.3 | 2419.6 | 135.4 | 346.75 |  |
| W10B | 12/11/2013 | 225.3 |  | 148.5 | 132 | 7.14 | 13.3 | 10.1 | 15.1 | 2.2 | 98 | 10.9 | 12.35 |  |
| W5A | 12/11/2013 | 132.7 |  | 86.16 | 172 | 6.22 | 11.5 | 2.2 | 3.14 | 4 | 1986.3 | 6.3 |  |  |
| W5B | 12/11/2013 | 351.9 |  | 237.4 | -14 | 6.99 | 16 | 2.28 | 6.09 | 1.2 | 5.1 | 1 |  |  |
| W6 | 12/11/2013 | 357.4 |  | 243 | 129 | 7.19 | 10.8 | 0 | 12.5 | 1.1 | 161.5 | 1 | 874.59 |  |
| D1 | 1/12/2014 | 149 |  | 95.86 | 165 | 7.23 | 9.9 |  | 95 | 19 | 2419.6 | 365.4 |  |  |
| D2 | 1/12/2014 | 159.5 |  | 104.2 | 164 | 7.25 | 9.5 |  | 88 | 15.3 | 2419.6 | 387.3 |  |  |
| D3 | 1/12/2014 | 202 |  | 132.3 | 174 | 7.03 | 10.5 |  | 24.5 | 2.7 | 1986.3 | 2 |  |  |
| D4 | 1/12/2014 | 223.4 |  | 147 | 189 | 6.86 | 10.6 |  | 44.4 | 6 | 2419.6 | 22.8 |  |  |
| D5 | 1/12/2014 | 278.6 |  | 186.1 | 189 | 7.29 | 14.1 |  | 21.1 | 0.9 | 2419.6 | 2 |  |  |
| D6 | 1/12/2014 | 315.4 |  | 211.6 | 206 | 7.03 | 10.5 |  | 58.3 | 10.5 | 2419.6 | 5.2 |  |  |
| LR1 | 1/12/2014 | 38.5 | 25.41 | 25.02 | 206 | 7.44 | 10.6 |  | 18.7 | 1.4 | 1986.3 | 25.9 | 452.39 | 16.70 |
| LR2 | 1/12/2014 | 20.2 | 22.31 | 27.31 | 198 | 7.4 | 10.6 |  | 19.5 | 1.6 | 87.1 | 41.7 | 233.71 | 16.70 |
| S1 | 1/12/2014 | 151.7 |  | 98.55 | 173 | 7.15 | 11.9 |  | 98.8 | 22.7 | 2419.6 | 325.5 | 238.75 | 16.70 |
| S2 | 1/12/2014 | 158.7 |  | 103.4 | 197 | 7.28 | 11.2 |  | 99.1 | 19.9 | 2419.6 | 146.7 | 513.07 | 16.70 |
| S3 | 1/12/2014 | 266.6 |  | 170 | 215 | 6.8 | 8.5 |  | 93.2 | 16.3 | 2419.6 | 184.2 | 1152.19 | 16.70 |
| W10B | 1/12/2014 | 235.9 |  | 154.4 | 202 | 7.12 | 17.3 | 8.68 | 14.2 | 2 | 816.4 | 1 | 5.00 | 5.00 |
| W5A | 1/12/2014 | 133.5 |  | 86.22 | 183 | 7.25 | 13.9 | 2.16 | 22.5 | 2 | 2419.6 | 1 |  |  |
| W5B | 1/12/2014 | 354.9 |  | 239.3 | 8 | 7.07 | 15.1 | 2.25 | 5.21 | 5.7 | 218.7 | 1 |  |  |
| W6 | 1/12/2014 | 355.4 |  | 239.7 | 46 | 7.27 | 15 | 0 | 7.46 | 1.3 | 2419.6 | 25.9 | 5819.91 | 5.00 |
| D1 | 2/8/2014 | 170.5 |  | 112.5 | 195 | 7.14 | 7.1 |  | 32.1 | 5.8 | 56.4 | 2 |  |  |
| D2 | 2/8/2014 | 174.9 |  | 115.6 | 195 | 7.12 | 7 |  | 28.7 | 6.1 | 104.5 | 5.2 |  |  |
| D3 | 2/8/2014 | 194.3 |  | 128.1 | 203 | 6.94 | 9.6 |  | 2.8 | 2.7 | 52.6 | 1 |  |  |
| D4 | 2/8/2014 | 200.1 |  | 132.6 | 207 | 6.8 | 9.5 |  | 7.54 | 2.4 | 2419.6 | 9.8 |  |  |
| D5 | 2/8/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 2/8/2014 | 358.3 |  | 244 | 209 | 6.78 | 8.5 |  | 3.21 | 3.2 | 225.8 | 1 |  |  |
| LR1 | 2/8/2014 | 76.2 | 13.6 | 49.5 | 198 | 7.11 | 4.9 |  | 3.27 | 1.1 | 332.5 | 24.1 | 53.65 | 16.70 |
| LR2 | 2/8/2014 | 69.2 | 14.5 | 43.4 | 82 | 7.8 | 5.4 |  | 3.32 | 0 | 394.5 | 18.7 | 16.70 | 16.70 |
| S1 | 2/8/2014 | 199.1 |  | 132.3 | 210 | 6.84 | 5.8 |  | 24.4 | 5.4 | 2419.6 | 2 | 38.19 |  |
| S2 | 2/8/2014 | 180.8 |  | 119 | 205 | 7.12 | 7.7 |  | 26.5 | 5.7 | 352.4 | 5.2 | 93.96 |  |
| S3 | 2/8/2014 | 171 |  | 113 | 198 | 7.07 | 7.4 |  | 29.2 | 4.7 | 1011.2 | 2 |  |  |
| W10B | 2/8/2014 | 241.1 |  | 149.3 | 199 | 7.14 | 8.3 | 9.55 | 6.84 | 2 | 20.9 | 2 | 2027.29 |  |
| W5A | 2/8/2014 | 122 |  | 79.78 | 207 | 6.91 | 8.5 | 2.44 | 2.73 | 3.4 | 2419.6 | 1 |  |  |
| W5B | 2/8/2014 | 346.1 |  | 223.8 | -7 | 7.14 | 13 | 2.48 | 3.39 | 0.8 | 10.9 | 1 |  |  |
| W6 | 2/8/2014 | 322.7 |  | 118.1 | 184 | 7.14 | 8.6 | 0 | 12 | 1 | 665.3 | 1 | 16.87 |  |
| D1 | 2/23/2014 | 163.5 |  | 106.4 | 173 | 7.15 | 10.4 |  | 49.8 | 7.5 | 241.1 | 4.1 |  |  |
| D2 | 2/23/2014 | 169.7 |  | 109.7 | 151 | 7.24 | 15.6 |  | 43.7 | 7.1 | 533.5 | 8.4 |  |  |
| D3 | 2/23/2014 | 195.5 |  | 128.1 | 181 | 6.93 | 11.1 |  | 3.68 | 5.8 | 76.6 | 1 |  |  |
| D4 | 2/23/2014 | 202.9 |  | 132 | 183 | 6.81 | 10.9 |  | 4.2 | 3.4 | 2419.6 | 3.1 |  |  |
| D5 | 2/23/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 2/23/2014 | 355 |  | 239.6 | 194 | 6.72 | 11.1 |  | 15 | 6.21 | 343 | 1 |  |  |
| LR1 | 2/23/2014 | 80.4 | 12.69 | 50.47 | 184 | 7.48 | 8.7 |  | 6.73 | 2.2 | 461.1 | 36.4 | 474.44 | 16.70 |
| LR2 | 2/23/2014 | 52.34 | 18.7 | 34.26 | 120 | 7.79 | 9.6 |  | 1.1 | 1.1 | 273.3 | 27.9 | 137.90 | 16.70 |
| S1 | 2/23/2014 | 164.3 |  | 11.06 | 171 | 7.18 | 12.2 |  | 51.8 | 2.2 | 1011.2 | 7.5 | 272.26 |  |
| S2 | 2/23/2014 | 181 |  | 116 | 183 | 7.2 | 19.6 |  | 39.3 | 6.6 | 2419.6 | 10.8 | 36.48 |  |
| S3 | 2/23/2014 | 198.5 |  | 130 | 201 | 6.78 | 9.4 |  | 33.5 | 2.5 | 2419.6 | 12.2 | 290.84 |  |
| W10B | 2/23/2014 | 215.8 |  | 140.2 | 80 | 7.12 | 20.4 | 9.15 | 22.1 | 0.9 | 55.6 | 5.2 | 56.93 |  |
| W5A | 2/23/2014 | 133 |  | 85.9 | 178 | 6.48 | 11 | 2.35 | 4.67 | 4.3 | 257.2 | 1 |  |  |
| W5B | 2/23/2014 | 353.5 |  | 237.4 | -40 | 7.25 | 14.3 | 2.43 | 1.52 | 1.2 | 2 | 1 |  |  |
| W6 | 2/23/2014 | 334.3 |  | 223.4 | 174 | 7.15 | 17 | 0 | 23.7 | 2.7 | 2419.6 | 365.4 | 5991.75 |  |
| D1 | 3/1/2014 | 178 |  | 116.8 | 173 | 6.74 | 9.7 |  | 18.2 | 4.2 | 301.5 |  |  |  |
| D2 | 3/1/2014 | 180.4 |  | 118.5 | 175 | 7.23 | 10 |  | 15 | 4 | 499.6 |  |  |  |
| D3 | 3/1/2014 | 194.1 |  | 127.4 | 175 | 7 | 11.5 |  | 5.04 | 1.9 | 1203.3 |  |  |  |
| D4 | 3/1/2014 | 196 |  | 128.9 | 123 | 6.97 | 10.9 |  | 3.18 | 1.8 | 34.5 |  |  |  |
| D5 | 3/1/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 3/1/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LR1 | 3/1/2014 | 71.35 | 13.71 | 46.95 | 173 | 7.46 | 7.8 |  | 2.23 | 0.6 | 230 | 34.535458 |  | 16.70 |
| LR2 | 3/1/2014 | 68.56 | 14.24 | 45.22 | 116 | 8.15 | 7.5 |  | 2.1 | 0.8 | 501.2 | 1.8371518 |  | 16.70 |
| S1 | 3/1/2014 | 179.5 |  | 117.1 | 180 | 7.07 | 10.9 |  | 19.4 | 5 | 2419.6 | 213.1328 |  |  |
| S2 | 3/1/2014 | 181.6 |  | 119 | 156 | 7.31 | 11.8 |  | 15 | 2.9 | 2419.6 | 371.7902 |  |  |
| S3 | 3/1/2014 | 196.5 |  | 130.4 | 133 | 6.88 | 7.6 |  | 13.8 | 3.1 | 2419.6 | 708.54141 |  |  |
| W10B | 3/1/2014 | 219.8 |  | 144.1 | 80 | 7.5 | 15.6 | 9.65 | 24.2 | 3.9 | 157.6 | 209.2772 |  |  |
| W5A | 3/1/2014 | 131.3 |  | 85.28 | 157 | 6.95 | 11.7 | 3.19 | 3.22 | 3.8 | 866.4 |  |  |  |
| W5B | 3/1/2014 | 331.9 |  | 222.2 | 17 | 7.46 | 15.4 | 3.5 | 3.54 | 1.1 | 4.1 |  |  |  |
| W6 | 3/1/2014 | 400.9 |  | 271.5 | 168 | 7.07 | 17.3 | 0.91 | 8.44 | 1 | 816.4 | 1787.0983 |  |  |
| D1 | 3/11/2014 | 186.7 |  | 122.5 | 176 | 7 | 11.1 |  | 16.7 | 16.7 | 2419.6 |  |  |  |
| D2 | 3/11/2014 | 190.5 |  | 124.8 | 179 | 7.01 | 11.7 |  | 3.96 | 15.4 | 2419.6 |  |  |  |
| D3 | 3/11/2014 | 189.4 |  | 124.1 | 178 | 6.75 | 12.1 |  | 0.7 | 2.1 | 50.4 |  |  |  |
| D4 | 3/11/2014 | 197 |  | 128.5 | 201 | 6.49 | 12 |  | 5.57 | 2.9 | 1299.7 |  |  |  |
| D5 | 3/11/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 3/11/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LR1 | 3/11/2014 | 79.14 | 11.43 | 51.32 | 185 | 7.3 | 10.5 |  | 3 | 0.8 | 150.3 | 1733.7843 |  |  |
| LR2 | 3/11/2014 | 81 | 12.46 | 52.3 | 188 | 7.15 | 10.9 |  | 3.75 | 0.6 | 324.1 | 401.36076 |  |  |
| S1 | 3/11/2014 | 203.4 |  | 134.6 | 181 | 6.73 | 7.8 |  | 12.1 | 3.9 | 2419.6 | 1404.2765 |  |  |
| S2 | 3/11/2014 | 185.7 |  | 121.5 | 187 | 7.03 | 11.9 |  | 18.1 | 3.9 | 2419.6 | 1269.9421 |  |  |
| S3 | 3/11/2014 | 187 |  | 121 | 179 | 6.9 | 12.2 |  | 22.2 | 4 | 2419.6 |  |  |  |
| W10B | 3/11/2014 | 220 |  | 147.2 | 76 | 7.7 | 14.9 | 9.99 | 15.8 | 2.1 | 95.7 | 814.46133 |  |  |
| W5A | 3/11/2014 | 123.4 |  | 80.13 | 208 | 6.92 | 11.5 | 3.13 | 2.16 | 2.8 | 79.9 |  |  |  |
| W5B | 3/11/2014 | 338.5 |  | 226.4 | 17 | 7.03 | 15.1 | 3.28 | 4.04 | 0.7 | 261.3 |  |  |  |
| W6 | 3/11/2014 | 398.2 |  | 265.2 | 170 | 7.05 | 17 | 0.7 | 14 | 1.1 | 344.1 | 125.19918 |  |  |
| D1 | 4/6/2014 | 188.8 |  | 123.7 | 125 | 7.4 | 12.6 |  |  | 5.9 | 816.4 |  |  |  |
| D2 | 4/6/2014 | 192.4 |  | 126.9 | 127 | 7.48 | 12.5 |  |  | 4.5 | 2419.6 |  |  |  |
| D3 | 4/6/2014 | 194.1 |  | 127.3 | 122 | 7.29 | 12.8 |  |  | 2.7 | 1986.3 |  |  |  |
| D4 | 4/6/2014 | 213.2 |  | 139.9 | 121 | 7.22 | 12.8 |  |  | 2.4 | 2419.6 |  |  |  |
| D5 | 4/6/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 4/6/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LR1 | 4/6/2014 | 61.8 | 15.8 | 40.23 | 58 | 8.93 | 11.9 |  |  | 1.1 | 173.1 | 2734.1819 |  |  |
| LR2 | 4/6/2014 | 55.4 | 17.7 | 36.12 | 92 | 7.98 | 13.3 |  |  | 1 | 257.7 | 2473.4572 |  |  |
| S1 | 4/6/2014 | 189.4 |  | 123.8 | 126 | 7.49 | 14 |  |  | 5.4 | 138.2 | 4316.7491 |  |  |
| S2 | 4/6/2014 | 185.2 |  | 121 | 10 | 7.95 | 14.1 |  |  | 3.5 | 665.3 | 1643.7709 |  |  |
| S3 | 4/6/2014 | 197.5 |  | 130.1 | 95 | 8.18 | 11 |  |  | 3.4 | 2419.6 | 4574.7354 |  |  |
| W10B | 4/6/2014 | 206.5 |  | 135.2 | 78 | 7.39 | 16.8 | 10.4 |  | 3.3 | 30.7 | 37.968627 |  |  |
| W5A | 4/6/2014 | 123 |  | 79.82 | 104 | 7.52 | 12.2 | 3.52 |  | 3.4 | 102.9 |  |  |  |
| W5B | 4/6/2014 | 349.9 |  | 235.9 | -27 | 7.23 | 13.6 | 3.71 |  | 0.6 | 1 |  |  |  |
| W6 | 4/6/2014 | 410.4 |  | 276.1 | 68 | 7.22 | 19.2 | 0.6 |  | 1.7 | 2419.6 | 2090.3448 |  |  |
| D1 | 4/24/2014 | 206.5 |  | 132.2 | 189 | 7.15 | 14.5 |  | 11 | 3.8 | 1203.3 |  |  |  |
| D2 | 4/24/2014 | 205.1 |  | 130.9 | 186 | 7.2 | 14.4 |  | 9.81 | 4.1 | 1119.9 |  |  |  |
| D3 | 4/24/2014 | 192.3 |  | 125.9 | 200 | 7.05 | 14.9 |  | 3.1 | 6.7 | 40.1 |  |  |  |
| D4 | 4/24/2014 | 204 |  | 132 | 211 | 7 | 15 |  | 3.17 | 3.3 | 1413.6 |  |  |  |
| D5 | 4/24/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 4/24/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LR1 | 4/24/2014 | 64.14 | 15.38 | 41.29 | 209 | 7.26 | 14.1 |  | 2.43 | 2.4 | 2419.6 | 3602.0284 |  | 16.70 |
| LR2 | 4/24/2014 |  |  |  |  |  |  |  | 2.05 | 0.9 | 209.1 | 1440.5583 |  | 16.70 |
| S1 | 4/24/2014 | 149.4 |  | 129.9 | 191 | 7.13 | 15.3 |  | 11.1 | 4.1 | 1203.3 | 973.25939 |  |  |
| S2 | 4/24/2014 | 191.2 |  | 124.5 | 204 | 7.14 | 16 |  | 10.3 | 3.4 | 2419.6 | 5802.5616 |  |  |
| S3 | 4/24/2014 | 249.5 |  | 166.2 | 158 | 7.58 | 13.8 |  | 4.85 | 4.2 | 1986.3 | 6608.4633 |  |  |
| W10B | 4/24/2014 | 199.69 |  | 130.74 | 75.426 | 7.1461 | 16.246 | 10.37 | 43.6 | 3.4 | 1732.9 | 236.28999 |  |  |
| W5A | 4/24/2014 | 118.94 |  | 77.186 | 100.57 | 7.2718 | 11.797 | 4.1 | 2.04 | 1.3 | 143 |  |  |  |
| W5B | 4/24/2014 | 338.35 |  | 228.12 | -26.109 | 6.9914 | 13.151 | 4.29 | 3.67 | 1.2 | 8.4 |  |  |  |
| W6 | 4/24/2014 | 396.86 |  | 266.99 | 65.756 | 6.9817 | 18.566 | 1.55 | 1.38 | 5.4 | 1732.9 | 2698.7259 |  |  |
| D1 | 5/7/2014 |  |  |  |  |  |  |  | 10.6 | 3 | 162 |  |  |  |
| D2 | 5/7/2014 |  |  |  |  |  |  |  | 10.4 | 3.4 | 73 |  |  |  |
| D3 | 5/7/2014 | 189.4 |  | 124.1 | 178 | 6.75 | 14 |  | 2.77 | 3.2 | 288 |  |  |  |
| D4 | 5/7/2014 | 197 |  | 128.5 | 201 | 6.49 | 14.1 |  | 4.21 | 3.1 | 1203 |  |  |  |
| D5 | 5/7/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 5/7/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LR1 | 5/7/2014 | 70.1 | 12.33 | 44.58 | 101 | 7.99 | 22 |  | 1.99 | 1.1 | 2420 | 59065.231 |  | 1262.11 |
| LR2 | 5/7/2014 | 75.1 | 14.66 | 43.58 | 95 | 7.81 | 19.8 |  | 2.02 | 0.8 | 2420 | 7413.5048 |  | 2690.77 |
| S1 | 5/7/2014 | 210.4 |  | 136 | 94 | 7.82 | 22 |  | 9.86 | 1.3 | 2420 | 37187.085 |  | 16.70 |
| S2 | 5/7/2014 | 212.7 |  | 136.8 | 73 | 7.87 | 24.3 |  | 17.2 | 3.1 | 2420 | 22043.926 |  | 16.70 |
| S3 | 5/7/2014 | 214.3 |  | 137.4 | 84 | 7.83 | 21.8 |  | 15.6 | 3.8 | 1986 | 2431.4944 |  | 16.70 |
| W10B | 5/7/2014 |  |  |  |  |  |  | 10.91 | 149 | 6.7 | 11 | 1537.7064 |  | 5.00 |
| W5A | 5/7/2014 | 145.7 |  | 92.57 | 70 | 7.9 | 18.4 | 4.81 | 3.82 | 6 | 2420 |  |  |  |
| W5B | 5/7/2014 | 324.82 |  | 218.99 | -25.065 | 6.7118 | 12.625 | 4.98 | 12.3 | 1.1 | 29 |  |  |  |
| W6 | 5/7/2014 | 40.14 |  | 264.3 | 114 | 7.52 | 25.4 | 1.94 | 9.3 | 0.9 | 2420 | 66999.708 |  | 5.00 |
| D1 | 5/14/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D2 | 5/14/2014 | 147 |  | 162.3 | 153 | 7.37 | 16.2 |  | 6.68 | 3.5 | 980 |  |  |  |
| D3 | 5/14/2014 | 203.4 |  | 132.8 | 151 | 7.29 | 16.2 |  | 3.78 | 2.8 | 2420 |  |  |  |
| D4 | 5/14/2014 | 213.2 |  | 139.3 | 147 | 7.31 | 16.7 |  | 4.38 | 3.8 | 980 |  |  |  |
| D5 | 5/14/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 5/14/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LR1 | 5/14/2014 | 67.7 | 14.67 | 43.01 | 138 | 6.86 | 20.5 |  | 5.03 | 1.4 | 2420 | 1519.6023 |  |  |
| LR2 | 5/14/2014 | 69.78 | 14.33 | 44.2 | 85 | 6.5 | 21.9 |  | 4.49 | 2 | 2420 | 8432.6443 |  |  |
| S1 | 5/14/2014 | 221.5 |  | 143 | 152 | 6.9 | 23.6 |  | 11.5 | 3 | 2420 | 70748.282 |  |  |
| S2 | 5/14/2014 | 212.8 |  | 136.4 | 154 | 7.3 | 27 |  | 19 | 2.4 | 2420 | 16316.593 |  |  |
| S3 | 5/14/2014 | 240.9 |  | 157.3 | 149 | 7.28 | 19.1 |  | 8.51 | 3.9 | 2420 | 2052.3253 |  |  |
| W10B | 5/14/2014 | 221.4 |  | 142.7 | 140 | 7.13 | 27.1 | 10.45 | 348 | 5 | 921 | 4159.9912 |  |  |
| W5A | 5/14/2014 | 128.1 |  | 81.05 | 127 | 7.07 | 24.2 | 5.1 | 42.8 | 4.3 | 2420 |  |  |  |
| W5B | 5/14/2014 | 255.3 |  | 235.8 | -28 | 7.24 | 16.7 | 5.2 | 4.25 | 4.1 | 293 |  |  |  |
| W6 | 5/14/2014 | 434.4 |  | 281.1 | 58 | 7.25 | 33 | 2.21 | 45 | 5.1 | 59 | 3198.6999 |  |  |
| D1 | 5/21/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D2 | 5/21/2014 | 245.7 |  | 161.3 | 147 | 7.36 | 16.3 |  | 7.65 | 4.2 | 129.6 |  |  |  |
| D3 | 5/21/2014 | 205.5 |  | 134.5 | 153 | 7.01 | 16.5 |  | 8.37 | 3.8 | 167 |  |  |  |
| D4 | 5/21/2014 | 214.4 |  | 139.2 | 163 | 7.16 | 16.3 |  | 9.95 | 3.9 | 394.5 |  |  |  |
| D5 | 5/21/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 5/21/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LR1 | 5/21/2014 | 60.55 | 16.45 | 38.38 | 130 | 7.81 | 20.3 |  | 7.3 | 1 | 2420 | 4361.8588 |  | 232.62 |
| LR2 | 5/21/2014 | 61.22 | 15.98 | 27.32 | 130 | 7.79 | 20.4 |  | 8.1 | 1.1 | 2420 | 12807.231 |  | 231.86 |
| S1 | 5/21/2014 | 240.4 |  | 155 | 152 | 7.22 | 20.8 |  | 28.5 | 4 | 2420 | 21510.129 |  | 16.70 |
| S2 | 5/21/2014 | 207.8 |  | 132.5 | 144 | 7.5 | 27.5 |  | 34.5 | 1.8 | 2420 | 13868.234 |  | 16.70 |
| S3 | 5/21/2014 |  |  |  |  |  |  |  |  |  |  |  |  | 16.70 |
| W10B | 5/21/2014 | 221.8 |  | 141.3 | 106 | 7.3 | 31.2 | 10.55 | 8.67 | 1 | 214.2 | 58305.366 |  | 5.00 |
| W5A | 5/21/2014 | 132.9 |  | 83.1 | 111 | 6.85 | 26.3 | 5.42 | 5.52 | 4.9 | 547.5 |  |  |  |
| W5B | 5/21/2014 | 343.8 |  | 223.2 | -28 | 7.28 | 28.6 | 5.54 | 5.51 | 1.2 | 189.2 |  |  |  |
| W6 | 5/21/2014 | 433 |  | 282.2 | 63 | 7.28 | 32.9 | 2.48 |  | 1.9 | 18.7 | 837.82048 |  | 5.00 |
| D1 | 5/23/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D2 | 5/23/2014 | 246.9 |  | 161.5 | 161 | 7.23 | 16.6 |  | 5.16 | 2.8 | 280.9 |  |  |  |
| D3 | 5/23/2014 | 211.2 |  | 137.5 | 172 | 6.82 | 16.5 |  | 3.07 | 2.5 | 2420 |  |  |  |
| D4 | 5/23/2014 | 213 |  | 138.4 | 176 | 6.72 | 16.9 |  | 3.4 | 2.6 | 387.3 |  |  |  |
| D5 | 5/23/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 5/23/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LR1 | 5/23/2014 | 203.9 | 11.76 | 53.84 | 104 | 6.97 | 20 |  | 0.9 | 0.8 | 2420 | 3027.049 |  | 495.55 |
| LR2 | 5/23/2014 | 66.3 | 15.04 | 41.8 | 146 | 7.56 | 20.8 |  | 0.8 | 0.5 | 1011.2 | 2583.0486 |  | 364.70 |
| S1 | 5/23/2014 | 238.5 |  | 154.8 | 164 | 7.02 | 19.6 |  | 11.7 | 3.1 | 2420 | 1895.202 |  | 16.70 |
| S2 | 5/23/2014 | 220.3 |  | 141.4 | 134 | 7.73 | 28.2 |  | 9.4 | 1.9 | 2420 | 5003.7792 |  | 16.70 |
| S3 | 5/23/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| W10B | 5/23/2014 | 218.1 |  | 140.2 | 157 | 7.04 | 24 | 10.59 | 10.7 | 0.9 | 509.9 | 5856.834 |  | 5.00 |
| W5A | 5/23/2014 | 137.6 |  | 86.49 | 109 | 6.63 | 29.2 | 5.47 | 5.6 | 2.9 | 2420 |  |  |  |
| W5B | 5/23/2014 | 346.9 |  | 229.7 | -54 | 7.34 | 20.7 | 5.61 | 17.2 | 1.2 | 57.8 |  |  |  |
| W6 | 5/23/2014 | 440.5 |  | 286.7 | 141 | 7.28 | 31 | 2.58 | 18.4 | 1.6 | 275.5 | 888.63689 |  | 5.00 |
| D1 | 5/26/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D2 | 5/26/2014 | 248.7 |  | 162.4 | 93 | 7.82 | 17 |  | 5 | 3.5 | 3.43 |  |  |  |
| D3 | 5/26/2014 | 214.2 |  | 139.4 | 89 | 7.42 | 17.2 |  | 2.48 | 2.7 | 2.646 |  |  |  |
| D4 | 5/26/2014 | 208.5 |  | 135.7 | 98 | 7.4 | 17.6 |  | 8.4 | 5.1 | 4.998 |  |  |  |
| D5 | 5/26/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 5/26/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LR1 | 5/26/2014 | 71.5 | 13.88 | 45.37 | 118 | 7.94 | 19.7 |  | 7.21 | 0.8 | 2420 | 5468.4018 |  | 106.10 |
| LR2 | 5/26/2014 | 71.5 | 13.94 | 45.5 | 134 | 7.69 | 19.2 |  | 6.4 | 1.1 | 2420 | 2599.4058 |  | 128.06 |
| S1 | 5/26/2014 | 240.5 |  | 156.8 | 81 | 7.64 | 19.1 |  | 14.3 | 4.9 | 2420 | 281.05234 |  | 16.70 |
| S2 | 5/26/2014 | 225.7 |  | 145 | 132 | 7.77 | 26.5 |  | 9.57 | 2.2 | 1011.2 | 5609.4768 |  | 16.70 |
| S3 | 5/26/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| W10B | 5/26/2014 | 221.3 |  | 143.7 | 82 | 7.27 | 21.2 | 10.66 | 19.2 | 1.5 | 509.9 | 38836.265 |  | 5.00 |
| W5A | 5/26/2014 | 143.3 |  | 90.03 | 152 | 6.51 | 28.2 | 5.63 | 9.7 | 5.7 | 2420 |  |  |  |
| W5B | 5/26/2014 | 340.2 |  | 224.9 | -27 | 7.3 | 21 | 5.73 | 15.2 | 13.9 | 57.8 |  |  |  |
| W6 | 5/26/2014 | 432.7 |  | 285,7 | 147 | 7.16 | 26 | 2.76 | 18.9 | 1.8 | 275.5 | 1428.5758 |  | 5.00 |
| D1 | 5/29/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D2 | 5/29/2014 | 249.2 |  | 161.6 | 115 | 7.43 | 19.6 |  | 66.7 | 4.8 | 328.2 |  |  |  |
| D3 | 5/29/2014 | 225 |  | 146.2 | 129 | 7.28 | 17.6 |  | 4.71 | 3.2 | 866.4 |  |  |  |
| D4 | 5/29/2014 | 209.8 |  | 134 | 109 | 7.31 | 18.5 |  | 2.64 | 4.2 | 1299.7 |  |  |  |
| D5 | 5/29/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 5/29/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LR1 | 5/29/2014 | 71.25 | 13.97 | 45.31 | 75 | 7.95 | 21.2 |  | 3.08 | 0.9 | 2420 | 10669.667 |  | 344.37 |
| LR2 | 5/29/2014 | 71.88 | 13.9 | 45.11 | 40 | 7.97 | 22.2 |  | 5.74 | 1.2 | 2420 | 2903.7011 |  | 16.70 |
| S1 | 5/29/2014 |  |  |  |  |  |  |  |  |  | 2420 | 11006.874 |  | 16.70 |
| S2 | 5/29/2014 | 230 |  | 147.1 | 81 | 7.83 | 30.8 |  | 7.26 | 1.3 | 2420 | 6792.2207 |  | 16.70 |
| S3 | 5/29/2014 | 243.1 |  | 157.7 | 116 | 7.29 | 22.8 |  | 157 | 4.5 |  |  |  |  |
| W10B | 5/29/2014 | 227 |  | 148.8 | 118 | 7.1 | 18 | 10.98 | 8.4 | 1.2 | 549.3 | 667.89643 |  | 5.00 |
| W5A | 5/29/2014 | 144.8 |  | 91.78 | 86 | 6.64 | 25.7 | 5.64 | 6.38 | 5.3 | 2420 |  |  |  |
| W5B | 5/29/2014 | 351.3 |  | 235.7 | -36 | 7.29 | 16.5 | 5.75 | 21.6 | 1.9 | 2420 |  |  |  |
| W6 | 5/29/2014 | 415.6 |  | 174.5 | 92 | 7.09 | 15.2 | 2.9 | 9.56 | 0.6 | 435.2 | 140.57849 |  | 5.00 |
| D1 | 6/4/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D2 | 6/4/2014 | 256 |  | 166.1 | 89 | 7.38 | 18.5 |  | 9.1 | 4.2 | 172.8 |  |  |  |
| D3 | 6/4/2014 | 249.3 |  | 162.7 | 88 | 7.01 | 18.7 |  | 3.75 | 3.1 | 235.9 |  |  |  |
| D4 | 6/4/2014 | 220.5 |  | 143.8 | 87 | 7.1 | 20 |  | 1.4 | 4 | 1986.3 |  |  |  |
| D5 | 6/4/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 6/4/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LR1 | 6/4/2014 | 80.5 | 12.41 | 50.63 | 114 | 8.21 | 21 |  | 4.07 | 3.4 | 2420 | 582.68638 |  | 16.70 |
| LR2 | 6/4/2014 | 78 | 12.81 | 49.17 | 33 | 8.08 | 22 |  | 2.69 | 0.8 | 1011.2 | 822.41276 |  | 16.70 |
| S1 | 6/4/2014 | 256.4 |  | 159.6 | 86 | 7.26 | 22.5 |  | 16.9 | 4.2 | 1011.2 | 145.39495 |  | 16.70 |
| S2 | 6/4/2014 | 242.2 |  | 155.4 | 82 | 7.78 | 22.3 |  | 8.22 | 8.2 | 2420 | 4602.9495 |  | 16.70 |
| S3 | 6/4/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| W10B | 6/4/2014 | 227.1 |  | 148.9 | 101 | 7.18 | 18.8 | 11.23 | 10.4 | 1.4 | 2420 | 1408.9296 |  | 5.00 |
| W5A | 6/4/2014 | 179.4 |  | 113 | 81 | 6.99 | 26 | 5.7 | 36 | 4.1 | 2420 |  |  |  |
| W5B | 6/4/2014 | 350.1 |  | 235.3 | -35 | 7.34 | 15.8 | 5.78 | 6.31 | 0.7 | 791.5 |  |  |  |
| W6 | 6/4/2014 | 406.5 |  | 273.5 | 79 | 7.09 | 21.5 | 3.15 | 3.28 | 1.3 | 2420 | 292.13972 |  | 5.00 |
| D1 | 6/10/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D2 | 6/10/2014 | 251.3 |  | 163.9 | 134 | 7.57 | 19 |  | 6.53 | 3.8 | 1553.1 |  |  |  |
| D3 | 6/10/2014 | 235.3 |  | 153.5 | 110 | 7.32 | 19.4 |  | 28 | 3.5 | 2420 |  |  |  |
| D4 | 6/10/2014 | 228.6 |  | 148.6 | 108 | 7.54 | 20.8 |  | 3.66 | 3.1 | 2420 |  |  |  |
| D5 | 6/10/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D6 | 6/10/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LR1 | 6/10/2014 | 82.79 | 12.01 | 152.56 | 92 | 8.5 | 22 |  | 2.43 | 3.5 | 2420 | 3.5714286 |  | 16.70 |
| LR2 | 6/10/2014 | 79.3 | 12.58 | 50.12 | 29 | 7.89 | 22.8 |  | 2.46 | 2 | 691 | 3.3333333 |  | 16.70 |
| S1 | 6/10/2014 | 241.8 |  | 156.3 | 100 | 7.45 | 22.6 |  | 17.9 | 1.2 | 2420 | 3.3333333 |  | 16.70 |
| S2 | 6/10/2014 | 236.2 |  | 151.7 | 103 | 7.84 | 26.8 |  | 2.62 | 1.6 | 2420 | 3.3333333 |  | 16.70 |
| S3 | 6/10/2014 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| W10B | 6/10/2014 | 238.8 |  | 155.4 | 94 | 7.45 | 19.3 |  | 43.6 | 2.7 | 601.5 | 1.6666667 |  | 5.00 |
| W5A | 6/10/2014 | 203.1 |  | 129.1 | 48 | 6.9 | 28 |  | 361 | 1.5 | 2420 |  |  |  |
| W5B | 6/10/2014 | 345.5 |  | 230.4 | -35 | 6.86 | 17.2 |  | 91.6 | 1.1 | 2420 |  |  |  |
| W6 | 6/10/2014 | 416.5 |  | 273.1 | 81 | 7.71 | 26.2 |  | 10.4 | 0.5 | 2420 | 1 |  | 5.00 |

# VITA

Derek Street was born on September 17th, 1985 in Winchester, Tennessee to Daryl and Patricia Street. He attended high school at St. Andrews Sewanee School in Sewanee, Tennessee. He went on to earn a Bachelor’s degree in Natural Resources with a minor in History from the University of the South in 2008. He then earned a Master of Science in Urban Planning with a concentration in Environmental Planning form the University of Tennessee in 2010. He then spent a year as a volunteer in AmeriCorps working on the Cumberland Habitat Conservation Plan, and the following year working for a Nashville, Tennessee based non-profit, the Tennessee Environmental Council. In 2012 he began his Master of Science in Geology and worked as a teaching assistant. He is expected to graduate in the fall of 2014 from the Department of Earth and Planetary Sciences with a Master of Science degree in Geology. Currently he is employed as a staff geologist with Hart & Hickman, PC in Charlotte, North Carolina.