A Cost Benefit Model for Traffic Incident Management

A Thesis Presented for the

Master of Science

Degree

The University of Tennessee, Knoxville

Sam Dallas Moss

December 2011

ACKNOWLEDGEMENTS

Funding for this research was provided by the Tennessee Department of Transportation (TDOT).  I would like to thank Mr. Frank Horne of TDOT who provided the direction and initiative for this project.  I would also like to thank Mr. Mark Dykes, also of TDOT, who provided insight to the traffic management center and its operations in Knoxville, Tennessee.  Mr. Mark Best provided me with information regarding the HELP trucks and their responsibilities and duties.  Dr. Lee D. Han provided me with the guidance and direction needed to complete this project.  Dr. Demin Xiong provided the calculations that were essential to completing the project.  Dr. Moraes Oliviero also helped in providing some of the code for the equations to calculate delay.  Ms. Stephanie Hargrove provided me with all the information that I needed to understand and begin working on the project and helped introduce me to the project.  Mr. Clement Oigbokie helped to organize the data that were collected.

ABSTRACT

Every day, non-recurring incidents cause delay on major roadways and cost the public valuable time and, hence, money.  Some public agencies have systems that help them to identify and mitigate the problems associated with incidents such as delay.  These systems are referred to as traffic incident management systems.  Many different agencies have searched for a way to quantify the benefits that these programs provide.  This thesis details how a model was developed to help these agencies easily quantify the benefits that can be derived from saving delay through efficient incident management.  This model uses real data to find the actual delay experienced by roadway users.  This dataset was collected for a case study that took place in Knoxville, Tennessee to help quantify the delay associated with incidents on the interstate.  Part of this dataset was an incident log that recorded every incident that was reported or identified in Knoxville for the year 2009.  Another part of the dataset was 30 second aggregate traffic data that measured the speed, occupancy, and flow at over 200 stations along the interstate system in Knoxville.  Once the delays were quantified for each of the stations for the whole year, the benefits of a traffic incident management system could be calculated.  The benefits were calculated by finding the delay savings due to the incident management program.  Knowing the delay savings makes it possible to quantify how much money is saved every year for each incident by the incident management system.  The benefit in delay saved for the city of Knoxville was found to be $12.1 million dollars.  The costs for the Knoxville system were $1.43 million dollars, thus giving a benefit cost ratio of approximately 8.5:1.  The model developed to determine this benefit cost ratio can be applied to other places and reused with only minor adjustments that do not require extensive data collection.  This is advantageous because it requires less time and effort to calculate the benefits of any particular incident management system.

TABLE OF CONTENTS

CHAPTER I  Introduction	1

CHAPTER II  Literature Review	3

CHAPTER III  Analysis methods	9

Incident Log	9

Data	10

Delay Calculation	10

Incident Comparison	12

Delay Savings	16

Value of Delay	20

Value of Time	20

Truck Percentages	21

Matrix Model	26

Input Layer	26

Calculations Layer	26

Savings Layer	27

Model Calibration	27

Capacity and Demand	27

CHAPTER IV  Results and discussion	29

LIST OF REFERENCES	31

APPENDIX	35

VITA	68

LIST OF TABLES

Table 1:  Incident reduction times used by previous studies.	4

Table 2:  Average incident reduction times for the eight types of incidents as identified by TDOT.	6

Table 3:  Benefit cost ratios of previous reviews.	8

Table 4:  The number of incidents for each type of cause and location occurring during the “Morning Peak” time period and lasting for less than one hour.	10

Table 5.  Variables used in the previous equations to obtain delay.	12

Table 6:  Initial values for a random screening of incidents with greater than 90% of the data available.	13

Table 7:  Values used in the filter to find the delay for each incident type at the appropriate stations.	16

Table 8:  Variables for the equations derived from the queuing diagram.	19

Table 9.  Value of time calculations and values for parameters.	26

Table 10:  Capacities used in the Knoxville, Tennessee case study for each location type	28

LIST OF FIGURES

Figure 1:  A map of the RTS stations for downtown Knoxville, Tennessee courtesy of TDOT.	14

Figure 2:  A map of the RTS stations to the west of downtown Knoxville, Tennessee courtesy of TDOT.	15

Figure 3:  The standard queuing diagram.	17

Figure 4:  Queuing diagram showing the effect of extended duration on total delay.	18

Figure 5:  30 second aggregate counts of traffic volumes on eastbound I-40 west of the I-640 interchange	22

Figure 6:  30 second aggregate traffic volume count for southbound I-275	23

Figure 7.  The location of the traffic counts and the locations where truck percentages were assumed to be 50% of the total number of trucks.	25

CHAPTER I 
Introduction

The purpose of traffic incident management (TIM) is to quickly and efficiently detect, respond to, and clear traffic incidents so that traffic flow may be restored as safely and efficiently as possible (FHWA 2011).  In order to evaluate the effectiveness of a TIM program, one must quantify the benefits in a way that objectively and accurately reflect the program’s goals.  There are several ways to quantify the benefits including improvements in safety, mobility, capacity, customer satisfaction, and environmental impacts.  One of the safety benefits derived from a TIM program is a reduction in the number of secondary crashes.  In terms of mobility, a TIM program helps in reducing the travel time delay of individual vehicles.  Capacity can be helped by a TIM program through optimizing the use of existing right of way facilities.  Customer satisfaction is a measure of the effectiveness of the program from the user’s perspective.  The environmental benefits can be measured in terms of reduction in emissions and fuel consumption (Han 2010).

Tennessee Department of Transportation’s (TDOT) “SmartWay” was implemented in 1999 and consisted of freeway service patrols in Knoxville and Nashville, Tennessee which patrol the freeways and provide assistance to motorists and aid in accident clearance.  Since 1999 the system has been expanded and now includes 700 Roadway Traffic Sensors (RTS), 250 traffic cameras, 78 Dynamic message signs, three transportation management centers, and freeway service patrols for Knoxville, Nashville, Memphis, and Chattanooga, Tennessee (TDOT 2011).

The SmartWay system is currently active in Knoxville, Nashville, Chattanooga, and Memphis (TDOT 2011).  Customer satisfaction has been reviewed previously by TDOT but the benefits of delay savings were not quantified.  A study of previous benefit costs analyses for other TIM programs revealed that there was a need for an easy-to-use model that could be applied to all locations in the country and could be used on a macroscopic or a microscopic scale.

The purpose of this thesis is to develop a model that can derive the benefits of a TIM program and be adjusted to any region in the country.  This model is capable of calculating only the delay savings that come from an effective TIM program.  This model will also be adjustable for use in planning purposes and for review of current systems and in either microscopic or macroscopic applications.  The model can be used in conjunction with an incident log if the demand, or average annual daily traffic (AADT), and network constraints, such as capacity, are known.

This model was developed using an incident log provided by TDOT, the capacity of the interstate network in Knoxville, Tennessee, and 30 second aggregate data that included volume, occupancy, and speed data for over 200 locations in Knoxville.  These data were chosen because of their high accuracy and reliability.  The incident log was provided by the Knoxville transportation management center and was used to identify when an incident occurred.

This model will only discuss the productivity improvements in terms of money saved due to delay reduction.  This method of evaluation was chosen because it is the easiest method to directly quantify and can be measured with the existing infrastructure in the program studied.  The program that was studied was the Tennessee Department of Transportation’s (TDOT’s) SmartWay Intelligent Transportation System (ITS) program.  The data used in this model comes from the Knoxville, Tennessee roadway traffic sensors and consists of speed, traffic flow, and occupancy data for each lane and an average of all lanes on the freeway.  The incident log was only available for Knoxville at the time when the study began which is why the study encompasses only Knoxville.

# CHAPTER II 
Literature Review

Many of the public agencies that have TIM programs have conducted reviews of them.  Some of these reviews relied on modeling software to assess the benefits of the programs.  Other reviews used analytical approaches such as a queuing model or kinematic wave theory to analyze delay caused by incidents.  These reviews typically focused on incident delay savings but could also include reduction in fuel consumption, lower emissions, avoided secondary crashes, and other safety benefits.

According to Hendrickson, the biggest assumption that most reviews make is the effect that a TIM program will have on the duration of the incident.  Most reviews assumed that the TIM program reduced the duration of a type of incident by a certain time while others studied the before and after effects of a TIM program and calculated the actual time saved.  The average of the stated incident reduction times either assumed or calculated is 17.8 minutes.  A list of all the times used by other reviews can be found in Table 1.  A summary of the average incident reduction times used by other reviews for each of the incident types that the Tennessee Department of Transportation (TDOT) uses can be found in Table 2.  TDOT states that the law enforcement officers in the state have reported a reduction in accident investigation time of 30% (Baird 2003) while Georgia Department of Transportation (GDOT) cited a reduction in incident response and identification time (Guin 2007).

Previous reviews can be grouped into four categories:  ones that used real data to calculate delay, reviews that used real data to calculate delay and compared them to simulation models that also calculated delay, reviews that only used a simulation model to calculate delay, and reviews that used an existing evaluation model to calculate a benefit cost ratio.

Some of the program reviews that used data collected from roadway sensors are the Georgia Navigator review (Guin 2007) and the St. Louis, Missouri review (Sun 2010). One of the benefits of a review that used data to calculate delay is that there was no estimation of the delay; instead, it was calculated directly.  A drawback to having used only data to calculate delay for a review is that there was nothing to compare the delay values to.  This could be misleading if an error occurred in the analysis which might have caused a misleading estimation of delay or benefits.

The “FIRST” program review used real data to calculate delay savings due to the program and compared the results to data generated by a model (Hendrickson 2004).  Washington State’s TIM program review also used real data to calculate delay savings and compared the results to model data (Wang 2008).  Because these reviews used data collected from traffic and compared the

Table 1:  Incident reduction times used by previous studies.

| Region | Incident Type | Incident Duration Reduction |
| --- | --- | --- |
| Boston | minor incident. | 15 |
|  | disabled | 25 |
|  | moved to shoulder | 25 |
|  | debris | 30 |
|  | accident in lane | 20 |
| Chicago | accidents on the shoulder | 20 |
|  | accidents in 1 lane | 35 |
|  | accidents in 2 or more lanes | 40 |
| Denver | lane blockers | 10.5 |
|  | non-blockers | 8.6 |
| Gary, IN | crash/ in lane assist | 10 |
|  | others | 15 |
| Houston | minor incident average | 16.6 |
| San Francisco | breakdowns | 16.5 |
|  | crashes | 12.6 |
| Minnesota | stall less than 30 | 8 |
|  | stall thirty to an hour | 5 |
|  | stall over hour | 0 |
| Virginia | accident | 43.4575 |
|  | breakdown | 24.97875 |
|  | debris | 5 |

Table 1 (continued)

| Region | Incident Type | Incident Duration Reduction |
| --- | --- | --- |
| Washington | disabled | 4.275 |
| Missouri | all | 15 |
| Georgia Navigator | incident | 23 |
| Florida | all | 20 |
| New Jersey | all | 7-20 |

Table 2:  Average incident reduction times for the eight types of incidents as identified by TDOT.

|  | Location |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Incident Type | shoulder | 1 lane | 2 lanes | 3 lanes | 4+ lanes | median |
| accident | 20.3 | 20.5 | 21.0 | 21.0 | 21.0 | 20.3 |
| fire | 17.6 | 17.6 | 17.6 | 17.6 | 17.6 | 17.6 |
| disabled | 15.6 | 15.6 | 15.6 | 15.6 | 15.6 | 15.6 |
| abandoned | 17.6 | 17.6 | 17.6 | 17.6 | 17.6 | 17.6 |
| debris | 17.6 | 17.6 | 17.6 | 17.6 | 17.6 | 17.6 |
| pedestrian | 17.6 | 17.6 | 17.6 | 17.6 | 17.6 | 17.6 |
| other | 17.1 | 17.1 | 17.1 | 17.1 | 17.1 | 17.1 |

delay savings calculations made on this data to a computer model there is good redundancy in the review.  One of the drawbacks of these reviews is that they could contain over estimates of the delay because of the limited number of actual cases used in the delay calculations.

One of the reviews that used only simulation data is the evaluation of the “CHART” program in Maryland (Chang 2002).  This evaluation used CORSIM to model the benefits from incident delay.  This evaluation also compared two different years of data to analyze any changes in the benefits of the “CHART” program.  Another review that used only model simulation data was the one conducted on Arizona’s “REACT” program (Battelle 2002).  “REACT’s” objective is to provide management for emergencies on arterial roadways (Battelle 2002).  This review used a simulation to assess the benefits of the program but due to the nature of the program, only two simulations were used to estimate the benefits.  One of the benefits to using only simulation data is that there is no need to collect large amounts of data.  One of the drawbacks to this is that there is no data to compare the simulation results to.  This results in conclusions that cannot be verified.

The Florida Road Ranger program used a model know as Freeway Service Patrol Beat Evaluation (FSPE) in its review (Hagen 2005).  This model was developed to analyze the benefits of a freeway service patrol program.  This is an effective model; however, there is no adaptability to any specific area.  Because the review was located throughout the entire state of Florida, the same model was used for parts that can be very different in accessibility, geometry, or other characteristics.  In addition, there are a limited number of types of incidents that can be distinguished using FSPE.  The review in Hampton Roads, Virginia used FSPE to evaluate their freeway service patrol program (Dougald 2007).  The main distinctions of this program were its limited area in addition to a before and after study.  The before and after study provided a detailed assessment of the actual clearance time improvements.  One of the drawbacks to these reviews is that there was no data for overall delay to use in comparison with the FSPE output.

The average benefit cost ratio for previous reviews was about 13.1:1 ranging from a low of 2.3:1 to a high of 41.5:1.  A list of some of the reviews analyzed along with their respective benefit cost ratios can be found in Table 3.

This research differs from the previous reviews in that its purpose is to develop a model that estimates delay saved by a TIM program using actual data and has the ability to be adapted to other locations.  This model is also different because the data that were used to calculate delay was a sample of many days taken from delay data that were calculated for an entire year.  The samples of data were later compared to the actual data from the entire year to see if the estimates for total delay values were similar to the measured total delay values.  Another benefit of this model is that it will reduce the amount of data needed to perform a benefit cost analysis on an existing TIM program by allowing minor adjustments for changes in capacity and demand in different locations.  Finally, this model can be easily adjusted to reflect different types of incidents beyond breakdowns, accidents, and debris.  It can calculate delay savings for the eight different categories that TDOT uses to identify different incidents.

Table 3:  Benefit cost ratios of previous reviews.

| Region | C/B Ratio |
| --- | --- |
| Boston | 19:1 |
| Chicago | 17:1 |
| Denver | 10-16:1 |
| Gary, IN | 13:1 |
| Houston | 7-36:1 |
| Houston Special Program | 19.4:1 |
| Los Angeles | 3.8-5.5:1 |
| San Francisco | 3.4:1 |
| Minnesota | 4.4:1 |
| Virginia | 4.6:1 |
| Missouri | 38:1 |
| Georgia | 4.4:1 |
| Florida Overall | 25:1 |
| Florida | 2.3-41.5:1 |
| North Carolina | 3.5-4.3:1 |
| South Carolina | 8:1 |
| Arizona | 6.5-8.5:1 |

# CHAPTER III 
Analysis methods

## Incident Log

An incident is defined as any non-recurring event that causes a reduction in the capacity of a roadway or an abnormal increase in demand (Owens 2010).  Each incident that is reported by the Traffic Management Center (TMC), roadway service patrols (HELP trucks), the local authorities, or by any citizen is recorded in an incident log that is kept for records with TDOT.  The incident log contains information about the time the incident was identified, the type of incident that occurred, when a HELP truck was dispatched, the location of the incident, what service was provided to aid motorists, the date it occurred, and when the incident was resolved.  For the year 2009, 14,996 incidents were recorded in the incident log for the interstate within the boundaries of the Knoxville SmartWay system alone.

Each incident was classified into four different groups:  incident cause, the time of day that the incident occurred, the duration of the incident, and the location on the roadway that the incident occurred. The incident causes as identified by TDOT are:  Accidents, Vehicle Fires, Disabled Vehicles, Abandoned Vehicles, Road Way Debris, Pedestrians on the Freeway, Other Incidents, and No Note Incidents.  The time of day group was meant to be representative of the demands experienced at different times of the day.  The times were:  a Morning Peak category that was from 6 AM until 10 AM, a Mid-Day category that was from 10AM until 3 PM, an Afternoon Peak category that was from 3 PM until 7 PM, and an Off Peak category that was from 7 PM until 6 AM.  The duration group was divided into four lengths:  less than one hour, one to three hours, three to five hours, and greater than 5 hours.  The location group took into account the different effects an incident’s location would have on the delay.  The locations were:  Shoulder, One Lane, Two Lanes, Three Lanes, Four or more lanes, and Median.  These groupings allow delays to be estimated based on common criteria.

Once the incidents were classified they were organized into different matrices that represent groups of incidents with similar delays.  Each matrix consisted of a count of all the incidents for each cause and each location.   There is one matrix for each combination of time of day and duration categories.  There are 768 individual cells, each representing a different classification of incident, comprising sixteen similar matrices corresponding to each time and duration category.  A sample of one of the organizational matrices is presented in Table 4.  All of the matrices can be found in Appendix 1.

Table 4:  The number of incidents for each type of cause and location occurring during the “Morning Peak” time period and lasting for less than one hour.

| CAUSE | LOCATION |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | TOTAL |
| Accident | 193 | 17 | 7 |  |  | 17 | 234 |
| Vehicle Fire | 4 | 1 |  |  |  | 0 | 5 |
| Disabled | 1486 | 181 | 51 | 7 |  | 97 | 1822 |
| Abandoned | 511 | 172 | 29 | 9 |  | 58 | 779 |
| Debris | 266 | 27 | 6 | 1 |  | 14 | 314 |
| Pedestrian | 2 |  |  |  |  | 1 | 3 |
| Other | 170 | 13 | 6 | 2 |  | 19 | 210 |
| No Note | 11 | 2 |  |  |  | 0 | 13 |
| TOTAL | 2643 | 413 | 99 | 19 | 0 | 206 | 3380 |

## Data

Speed, volume, and occupancy data collected by 213 radar and video roadway traffic sensors were gathered from TDOT.  The data were for the year 2009 and covered most of the freeways through and around the city of Knoxville.  The roadway traffic sensors were powered by solar panels; therefore, some of the data were incomplete.  On days when there was not much sunlight there could be missing data.

There were also missing values occurring infrequently throughout some days due to random errors in the data collection devices.  The data were imputed to correct the missing data points where possible.  The imputations followed an algorithm that derives the likely values of speed, volume, and occupancy separately during a given time window.  This time window was defined by the number of data points before and after the missing data and with a maximum of eight minutes.  The algorithm also weighted the data points more if they were closer in time to the missing values.

## Delay Calculation

The speed, volume, and occupancy data were used to calculate the delay for every thirty seconds of every day.  This was done using the fundamental traffic stream relationship where flow is equal to density multiplied by velocity.  A total travel time for each segment can be calculated using equation 1:

(3.1)

Over a duration of m for n segments, we have the total travel time, T, as:

(3.2)

Le is dependent on the mix of traffic and can change over i and j.  But if the mix of traffic were known and relatively stable, a fixed value, e.g. 26.4 feet, may be used.  This transforms equation 3.2 into:

(3.3)

During normal, non-congested conditions, either side of the equation should work.  If vij were somehow unavailable, one can assume an average operational speed of Vb for all i and j:

(3.4)

If there are periods of congestion where traffic count Cij = 0, an alternative way of calculating the total travel time over a duration of m for n segments is:

(3.5)

Using these equations, the total delay over the duration of m for n segments can be calculated as:

(3.6)

These equations illustrate that the delay is equal to the travel time under congested conditions minus the travel time under normal conditions where the variables are given in Table 5.

A program was written to apply these equations to the average speed, flow, and occupancy data.  The program consisted of three conditions.  The first condition was that if the traffic speed was greater than or equal to the speed limit, then the delay would be equal to zero.  The second condition stated if the traffic speed was less than the speed limit and the traffic speed was greater than zero, then the delay was equal to the actual travel time for the segment minus the travel time at the speed limit as per equation 3.6.  The third condition was that if the traffic speed was equal to zero the traffic count and the occupancy must also be checked to see if they were equal to zero.  If so, then the delay was zero.  However, if this was not true when the speed was equal to zero then traffic is present but not moving and the delay can be calculated as the lane count multiplied by the segment length multiplied by 200 multiplied by thirty multiplied by occupancy divided by 100 as per equation 3.5.

## Incident Comparison

After the delay is calculated for every thirty second interval and every station for every day, the delay can be compared to the incident log to find the total delay for each type of incident.  In order to assure the accuracy of the start time in the incident log, a preliminary check of each type of incident was done to find the

Table 5.  Variables used in the previous equations to obtain delay.

| Variable | Meaning |
| --- | --- |
| Ci | Traffic count at station i for dt, in number of vehicles |
| dt | Size of time slice, e.g. 30 seconds |
| i | A roadway segment centered around sensor station i |
| j | A time slice of the size of dt, e.g. 30 seconds |
| ki | Density in segment i, in vehicles/mile/lane |
| Le | Length of an average vehicle, in feet, about 26.4 feet |
| Li | Length of segment i, in miles |
| m | Number of (30-second) time slices in the analysis |
| n | Number of roadway segments in the analysis |
| Ni | Number of lanes in roadway segment i |
| qi | Flow rate at station i, in vehicles/hour/lane |
| vi | Average speed at sensor i |
| Vb | Average normal speed for the duration of dt at sensor i, in miles/hour |
| Vi | Average speed for the duration of dt at sensor i, in miles/hour |
| Φ[phi]i | Occupancy at station i, in percent |

average duration and length in stations for each type of incident.  Only the days where 90% of the data were available were used in order to accurately describe each incident type.  Once these higher reliability days were found, a random sample was drawn from the incident log and checked using the speed, flow, and occupancy data to find the actual duration and number of stations that were affected by each incident.  Each incident was compared to the station maps, shown in figures 1 and 2, to match the location of the incidents to the correct stations for analysis.  For each incident type the average values calculated are shown in Table 6.

There were no incidents in the log that fell under the category of “No Note” and were therefore excluded from the initial analysis.  These incidents were assumed to have been added to the “Other” category and the categories were combined.

After the initial average values of the incident types were found, another random sample was taken.  This random sample was taken from the days that contained 80% of the data available and consisted of fifty days for each type of incident where possible.  In the cases where it was not possible to have a sample of fifty incidents due to a small number of incidents of that cause, all of the days available were sampled.  The sample of incidents was then compared to the incident log to find out when and where the incidents occurred.  Once the location of the incidents was known, a map of the stations was used to determine and record the closest downstream stations that would be affected by the incident.  After this was determined for each incident the delay was calculated using a filter that found the delay for the average number of stations for each category and for a prescribed period of time before and after the incident discovery time was recorded.  A list of the filter parameters for each incident type is in table 7.

Once an initial recording of the delays were found, a second check of the data was performed.  The second check entailed filtering out the incidents in the sample that had over 50,000 seconds of delay.  This value was chosen because it was found to be roughly equal to a thirty minute incident blocking one lane.  Once these days were identified, a thorough investigation of these incidents was done by hand to determine the exact extent of the incident in time and number of stations.  This investigation included checking all of the surrounding stations to

Table 6:  Initial values for a random screening of incidents with greater than 90% of the data available.

|  | Accident | Fire | Disabled | Abandoned | Debris | Pedestrian | Other |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Duration | 31.875 | 78.33 | 10 | 50 | 35 | 30 | 13.33 |
| Stations | 2 | 5.75 | 1 | 1 | 1 | 1 | 1 |

Figure 1:  A map of the RTS stations for downtown Knoxville, Tennessee courtesy of TDOT.

Figure 2:  A map of the RTS stations to the west of downtown Knoxville, Tennessee courtesy of TDOT.

Table 7:  Values used in the filter to find the delay for each incident type at the appropriate stations.

| Filter Parameter | Accident | Fire | Disabled, Abandoned, Pedestrian, and Other | Debris |
| --- | --- | --- | --- | --- |
| Stations Used | 3 | 5 | 1 | 1 |
| Time Before (min.) | 30 | 90 | 30 | 45 |
| Time After (min.) | 30 | 60 | 30 | 30 |

determine the geographic extent of an incident to ensure that all of the stations that could have recorded delay caused by the incident were used.  The investigation also included checking the beginning and ending time of the incident to ensure that the entire incident was counted by the delay comparison.

Once the delay values were found for each of the incidents, the recurring delay was subtracted out.  This was accomplished by recording the delay on another day in the same place and time using the same filter as the incident day.  Once the delay was known for both days the delay from the non-incident day was subtracted from the delay measured on the day of the incident.

## Delay Savings

The total delay caused by an incident can be directly measured using the fundamental traffic stream relationship.  This will give the delay that was caused by the incident but will not tell how much an incident management system benefits a given city.  In order to find the benefits of an incident management system, the delay caused by an incident without the incident management system must be known.  The queuing diagram was used to estimate what the delay would have been in the absence of the incident management program.  The basic queuing diagram is shown in Figure 3.  The queuing diagram was modified to represent the total delay if an incident’s duration were extended.  Figure 4 illustrates a comparison between the actual delay and the delay that would occur without an incident management system.  The delay for the extended duration can be calculated by using equations derived from the queuing diagram.  These equations whose variables are defined in Table 8 are:

λ[lambda]TH+Δ[delta]t1=SI*TH+S*Δ[delta]t1  (3.7)

AH=12x(TH+Δ[delta]t1)  (3.8)

x=(λ[lambda]-SI)TH  (3.9)

Figure 3:  The standard queuing diagram.

Incident cleared without TIM

Incident cleared with TIM

Incident Begins

Time

Delay with TIM

Demand

Cumulative Vehicles

Time

Additional delay without TIM

Figure 4:  Queuing diagram showing the effect of extended duration on total delay.

λ[lambda]TH+Δ[delta]t2+Δ[delta]T=SI(TH+Δ[delta]T)+S*Δ[delta]t2  (3.10)

y=(λ[lambda]-SI)TH+Δ[delta]T  (3.11)

A=12yTH+Δ[delta]t2+Δ[delta]T  (3.12)

A-AHAH  (3.13)

Equation 3.13 represents the delay savings ratio.  The delay savings ratio is the delay saved divided by the delay calculated.  This delay savings ratio can be multiplied by the delay calculated to get the theoretical delay savings.  In order to find the delay savings for each cell in the matrices, the actual delay was calculated and multiplied by the delay savings ratio.

For cells that had no samples available to obtain the delay needed, the queuing diagram was used to help determine the actual value of delay that would occur.  In some cases, the demand was less than the reduced capacity caused by an incident.  For these cases, the analytical queuing model does not accurately reflect delay data found.  In these cases, a linear relationship with respect to time was found to be a more accurate representation of the delay calculated for the incident type.  The analytical queuing model shows that if the reduced capacity is greater than the demand, there should be no queuing delay.  However, because some delay can be calculated at the location of the incidents, the most accurate estimate for calculating the delay was found to be a linear relationship between the duration of the incident and the amount of delay.  A comparison of the

Table 8:  Variables for the equations derived from the queuing diagram.

| Variable | Meaning |
| --- | --- |
| λ[lambda] | Demand |
| TH | Incident Duration |
| Δ[delta]t1 | Time until normal conditions are restored after incident with incident management |
| SI | New capacity caused by the incident |
| AH | Delay caused by the incident with incident management |
| x | Maximum number of vehicles delayed with incident management |
| Δ[delta]T | Time savings from incident management |
| Δ[delta]t2 | Time until normal conditions restored after incident without incident management |
| y | Maximum number of vehicles delayed without incident management |
| A | Delay of the incident without incident management |

reduced capacity to demand can be found in Appendix 2.

In order to linearly interpolate the delay, the delay measured was multiplied by the ratio of the average duration of the incident without delay data over the average duration of the incident with delay data.  The savings ratios for these data were different from the savings ratio for the data that used queuing estimates.  The savings ratios for the linearly interpolated data were calculated by dividing the time saved by the average duration of the incident.  All of the final values for delay and delay savings that were used in the model can be found in Appendix 3 and Appendix 4 respectively.

Some of the incidents are not represented well in this analysis.  For instance, a car can be abandoned on the interstate and the job of the HELP trucks is to assist the police in tagging these vehicles with the date discovered.  Once the vehicle is tagged it is allowed to sit on the side of the road for forty-eight hours.  An extended comparison was not made for abandoned cars for forty-eight hours.  Most of the abandoned cars were cleared before this time and if an extended analysis was made, the error would likely be increased.  Many of the days analyzed initially showed low amounts of delay with the exception being accidents and vehicle fires.  When the days were compared with an alternate day, they sometimes showed a decrease in delay on the days when the incident occurred.  This can be explained by daily variances in traffic patterns and by the lack of knowledge of what is actually occurring on each particular day.  For instance, an accident could occur at the same spot there was an abandoned vehicle the previous day.  If the abandoned vehicle was compared to the data with the accident, there is a chance that the delay would be much less than the day with the accident.  In cases where the delay for an incident was less than a corresponding similar day, additional days were analyzed in order to understand what the true baseline was for that area.  In instances where the corresponding similar days had a majority of similar days being greater than the day with the incident being studied, the delay was recorded as zero.

### Value of Delay

The value of delay is quantified by the product of the delay and the value of time.  The value of delay saved is the product of the value of the delay and the delay savings ratio.  The value of delay can be quantified by the following equation:

Value of Delay=Amount of Time Saved ×Value of Time (3.14)

### Value of Time

The value of time can be found using the following equation:

Value of Time=(Percent cars×Average Occupancy of Vehicles×Value of Time for Passenger Cars)+(Pecent Trucks×Value of Time for Trucks)  (3.15)

The value of time is estimated for different regions of the United States by the Texas Transportation Institute (TTI).  The value of time for Knoxville for 2009 was estimated as $16.01 per hour of person travel and $105.67 per hour of truck time (TTI 2009).

### Truck Percentages

The percentage of passenger cars on the interstate is roughly equal to 100 minus the percentage of trucks on the interstate.  If one of these is known, the other can be found easily.  The number of through trucks on the interstate in Knoxville is assumed to be relatively constant throughout the day and the year and is roughly twelve million trucks per year.  This volume is for the I-40/ I-75 weigh station to the west of Knoxville.  This number is equal to about 33,000 trucks per day or roughly 700 trucks per hour.  The total number of trucks is assumed to be relatively constant throughout the day with slight peaks in local truck traffic during the day.

The number of vehicles on the interstate at one time varies for different locations in Knoxville.  Figures 5 and 6 show the vehicle counts for two locations in Knoxville on the same day.  Graphs of the number of vehicles in various locations around Knoxville can be found in Appendix 5.

Knoxville is at the intersection of two major interstates, one a major north-south corridor and the other a major east-west corridor. Therefore, the truck volume estimates had to be calculated in a way that reflected the difference in the combined traffic, where the interstates are combined and the truck volumes are measured, and the split traffic, where the interstates are separate.  To assume that the same number of trucks travelled on combined I-75 and I-40 corridor as the individual roadways would result in double counting trucks along those separate roadways.  Since the amount of trucks is known along the combined corridors, a proportion of the total would be the appropriate number to assume for each individual interstate after they split at the I-640 and I-75 interchange.  The total amount of vehicles was counted on each roadway but the proportion of trucks that follow each route was unknown and assumed to be 50% for each direction.  One roadway may have a higher proportion of trucks than the other but the total of the proportions is still 100%.  This allows the number of trucks to remain constant but the number of cars can be combined for an overall

Figure 5:  30 second aggregate counts of traffic volumes on eastbound I-40 west of the I-640 interchange

Figure 6:  30 second aggregate traffic volume count for southbound I-275

percentage of trucks.  This means that the number of trucks at the last place where they shared the roadway is the sum of the two after they have split.  Therefore, the number of trucks on I-640 and the combined I-75 and I-40 corridor was the same as the combined total of trucks on I-75, I-40, and I-640 when the roadways were not combined.

The total truck percentage was calculated as:

Truck Percentage=Number of 100% Truck Count Locations+12×Number of 50% Truck Count Locations×Number of Trucks÷[Number of 100% Truck Count Locations+Number of 50% Truck Count Locations×Total Vehicles]  (3.16)

The number of total vehicles was calculated as the sum of the number of vehicles on I-40 just west of I-640, the number of vehicles just to the west of I-140, the total number of vehicles on I-640 to the west of the I-640/ I-75 interchange, the number of vehicles on I-75 north of I-640, and the number of vehicles on I-40 east of I-640.  The number of trucks had to be adjusted on I-75 and I-40 east of I-640 to reflect the unknown proportional split.  The number of total vehicles was counted but the number of trucks was assumed to be the sum of both.  Figure 7 shows the locations used in the counting of the volumes of vehicles.  The diagram also shows where the truck counts were assumed to be half of the number of the total.

The number of vehicles and percentage of trucks were taken for two different days of the year and were found to be similar.  One of these days was before the completion of “Smartfix 40” and the second was after.  The value of time that was used in the calculation and the respective components can be found in Table 10.

Location of data where 100% truck volume was assumed

Location of data where 50% truck volume was assumed

Figure 7.  The location of the traffic counts and the locations where truck percentages were assumed to be 50% of the total number of trucks.

| Time | Truck Value | Percent Trucks | Passenger Value | Occupancy | Percent Cars | Value of Time |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | $100.00 | 20% | $16.00 | 1.25 | 80% | $36.00 |
| 2 | $100.00 | 20% | $16.00 | 1.25 | 80% | $36.00 |
| 3 | $100.00 | 15% | $16.00 | 1.25 | 85% | $32.00 |
| 4 | $100.00 | 50% | $16.00 | 1.25 | 50% | $60.00 |

Table 9.  Value of time calculations and values for parameters.

## Matrix Model

Once the delay savings and the value of those savings are known, the benefit cost ratio can be calculated.  The purpose of the model is to provide the user with an interface that can be manipulated and adjusted depending on local conditions to give a benefit cost ratio that accurately reflects each situation.  The matrix model consists of three layers with a specific role and function for each layer.  Each layer is a separate Microsoft Excel worksheet within the same workbook.  The first layer is the input layer, the second is the calculations layer and the third is the savings or output layer.  Each layer has sixteen separate matrices that represent the different time and duration categories for the incidents.  Within these sixteen matrices are the categories for incident type and location.

### Input Layer

The input layer contains the number of each type of incident.  The matrices for this layer are the same as the incident log categories.  The inputs used in the analysis of Knoxville for 2009 can be found in Appendix 1.  This layer can be updated to reflect the number of incidents for any year and any location.

### Calculations Layer

The calculations layer contains the delay savings calculated for each type of incident.  It also has the value of time for each different time period.  This layer is adjustable via a ratio factor that changes all the values in a given matrix or the values themselves can be adjusted accordingly.  The numbers found for delay savings are very sensitive to changes in capacity and demand and, therefore, delay savings should be recalculated for different years and different regions.

### Savings Layer

This layer acts as the output because it multiplies the number of incidents of each type by the delay values and the value of time for each type of incident.  All 768 cells are filled with a value that represents the total amount of money saved in delay for each type of incident for the whole year.  The sum of all the cells is also on this page along with the total costs for the incident management system for that year.  The total benefit cost ratio has been calculated and is also on this page.

## Model Calibration

The model has been calibrated to give the appropriate delays for the Knoxville area for the year 2009.  Because the delay savings are highly sensitive to capacity and demand, these numbers should be adjusted for different regions or when there is a change in infrastructure or demand.

### Capacity and Demand

The standard capacity for the freeway in Knoxville is estimated to be around 1500 vehicles per hour per lane including trucks in rolling terrain.  In the first half of the year 2009 Knoxville’s interstate highway system was under construction from mile marker 388 to mile marker 389.  The interstate was completely shut down for through traffic and an alternate route had to be used for most traffic.  This caused a need to add an additional lane to I-640 around Knoxville in order to handle the increased flow on this roadway.  Because of this expansion a majority of the city of Knoxville had four or more lanes on the interstate in each direction.  The exceptions were I-275 and I-75.  During the portion of the year after Smart Fix 40 was completed, most of the interstate in Knoxville remained 4 or more lanes.  The Knoxville analysis assumes that most of the incidents occurred on a section of the interstate that contained four or more lanes in each direction.  This leads to the determination that a majority of the interstate roadway in Knoxville in 2009 had a capacity of around 6000 vehicles per hour per direction.

Capacity is constant for most of the roadway in Knoxville except in certain areas where weaving sections reduce the effective capacity and other natural bottlenecks affect traffic flow.  When an incident occurs it causes a drop in the capacity of the roadway.  Other studies have shown that the location of the incident along with the type of incident and how many lanes are affected all have an effect on the actual capacity of the roadway (Chou 2009).  The actual capacity after an incident occurs is hard to quantify but a range of values were used in the calibration of the model in order to fit the measured delays to the calculated delays.  The ranges are found in Table 10.

Demand is the actual number of users who use the roadway.  Knoxville has a large variance in demand from one roadway to another.  For this reason it was necessary to use average values for the region when calculating the delay savings.  It was also important to know the correct demand amounts for interpolating the amount of delay for incidents that were not included in the sample.  The demands used were:  4500-5000 vehicles per hour for the AM and PM peaks, 4000-4500 vehicles per hour for the midday period, and 2500-3000 vehicles per hour for the off peak or night period.

Because a summary of delay values was made, the amount of delay for each day was known for the whole year.  Extensive calibration was done to ensure the values estimated for total delay was approximately equal to the amount of delay calculated.  The first step was to find the values of delay for the whole year.  This was done by using the days in which 70% of the data were collected for each day.  This data encompassed 238 days for the year 2009.  The total delay for these 238 days was about 733,000 hours of delay.  This delay includes recurring delay which was subtracted out to find the delay caused by incidents.  To find the recurring delay a thirty day moving average was calculated for 208 of the 238 days.  Once the total delay for each of the 208 days was subtracted by the thirty day moving average, the total incident delay was found to be approximately 325,000 hours of delay.  Any negative values were taken to be equal to zero.  This gives an estimate of about 570,000 hours of delay for 365 days.

Once the total adjusted delay was known for the year a check of the calculated delay values was done to make sure the values were similar.  The total delay calculated by the model after calibration was about 447,000 hours.  Because the recurring delay was only an estimate the model was calibrated to reflect a conservative estimate of the actual delay for the year.

Table 10:  Capacities used in the Knoxville, Tennessee case study for each location type

| Location | Proportion of Capacity |
| --- | --- |
| Shoulder | .85-.99 |
| One Lane | .58-.66 |
| Two Lanes | .25-.28 |
| Three Lanes | .13-.15 |
| Four or More Lanes | 0 |
| Median | .74-.80 |

# CHAPTER IV 
Results and discussion

The third layer of the matrix model is an output page that shows the actual amount of money saved for the year by implementing the traffic incident management program.  The analysis for Knoxville, Tennessee shows that the total money in delay savings was $12.1 million in 2009.  The total costs for the year 2009 were $1.43 million.  This gives a benefit to cost ratio of 8.5:1.  This falls in the lower middle of the range of values calculated by other program reviews.

This estimation is probably lower because it only accounts for delay savings and does not account for safety benefits, environmental impacts, or goodwill.  Another reason that it is lower is due to the low amount of demand in Knoxville.  The amount of delay for an incident is related to the demand at the time of the incident.  Because it is an exponential relationship, even small increases in demand result in very large increases in delay.  This is beneficial for cities that wish to install a system when demand is low in anticipation of increases in demand over time.  The larger expenses can be made upfront when they are cheaper and the benefits can be derived from those investments over a long period of time.

For a city like Nashville, Tennessee the demand is higher in more places.  There is also higher capacity infrastructure.  Because the capacity is higher, there is less relative reduction in capacity due to an incident.  However, because the demand is greater, and the queuing equations are more sensitive to changes in demand than capacity, there would still be a gain in the delay savings.

A simple tool to adjust this model for use in other places has been included in the model itself.  A ratio multiplier is included on the calculations page.  This can be used to adjust for the amount of delay difference from one place to the next.  A simple ratio of the difference in delay savings for each of the sixteen category groupings can be input into the calculations page.  This ratio would represent the difference between delay savings due to capacity and demand changes in different areas.  A calculation could be made by using the queuing diagram without using actual data for this purpose; however, the appropriate values should be checked for all of the cells in each category.  This can be done by comparing the reduced capacity to the demand in each area and adjusting the values in the model accordingly.

This model can also be used for reviews of current systems and for planning purposes.  If a certain type of incident was producing only a small benefit to the system, this type of incident could be ignored by the incident management program in order to increase the efficiency of the program during times of high activity such as peak hours.  If an agency wanted to implement a new TIM program, this model would give them a way to help make decisions about the extent of the program they wish to implement and to help in determining which services they feel would best benefit their area.  This model can also be used either macroscopically as in the Knoxville, Tennessee case study or microscopically for greater precision.

This analysis is a useful way to quantify the benefits of an incident management program.  The model has been calibrated to reflect the conditions for the year 2009 in Knoxville, Tennessee.  The model can be adjusted and used in other places without the need of extensive data collection.  This model is different from previous models because it relies on real data collected by roadway traffic sensors.  The model can estimate a benefit cost ratio and can present the savings in a way that shows program managers where and how much delay savings occur and can help in policy decision making.

The data were collected at a time when Smart Fix 40 was ongoing.  The delay data were not affected by this project because the interstates in Knoxville had more lane consistency with the project in place and most of the data used were from the time that Smart Fix 40 occurred.  Another way the data were preserved was by excluding the stations between the I-640 and I-40 interchanges.  This ignored the unusually low volumes of traffic along this stretch of I-40 during Smart Fix 40.  The volume of traffic and the percentage of trucks were found for days during and after the Smart Fix 40 project and were found to be similar.  The benefit cost ratio found for SmartWay in Knoxville is only reflective of the conditions experienced in 2009 but can be adjusted for any other year.  The model can be adjusted to reflect different capacity and demand values and should be done so for future analyses in Knoxville.

Future research could build on this model by calculating other types of benefits such as secondary crashes avoided and other safety benefits.  Public perception could also be included in future models.  Further research in demand and capacity analysis could change the way this model functions but it can be adjusted to reflect these changes.

# LIST OF REFERENCES

"Traffic Incident Management - Emergency Transportation Operations." FHWA Operations. Web. 15 Nov. 2011. <http://ops.fhwa.dot.gov/eto_tim_pse/about/tim.htm>.

Han, Lee D., Demin Xiong. “A Preliminary Review on The State-of-the-Art of Assessment of Incident Response Programs”. UT Center for Transportation Research. April 2010.

"TDOT SmartWay -." Tennessee Department of Transportation. Web. 15 Nov. 2011. <http://www.tdot.state.tn.us/tdotsmartway/schedule.htm>.

Short Elliott Hendrickson Inc. “FIRST Program Evaluation”. Prepared for Minnesota Department of Transportation. November 2004.

Baird, Malcolm, Lori Cove, Frank Horne, and Bill Jacobs. “Development of Tennessee’s Freeway Service Patrol (HELP) Program”. TRB 2003 Annual Meeting.

Guin, Angshuman, Christopher Porter, Bayne Smith, and Carla Holmes. “Benefits Analysis for Incident Management Program Integrated with Intelligent Transportation Systems Operations - Case Study”. Transportation Research Record: Journal of the Transportation Research Board, No. 2000. Transportation Research Board of the National Academies, Washington, D.C., 2007. pp. 78–87.

Sun, Carlos, Venkat Chilukuri, Tom Ryan, Michael Trueblood. “Evaluation of Freeway Motorist Assist Program”. Prepared for Missouri Department of Transportation and Federal Highway Administration. University of Missouri, E2509 Lafferre Hall, Columbia, MO 65211. February 2010.

Wang, Yinhai, Patikhom Cheevarunothai, and Mark Hallenbeck. “Quantifying Incident-Induced Travel Delays On Freeways Using Traffic Sensor Data”. Prepared for Washington State Dept of Transportation and U.S. Department of Transportation. Department of Civil and Environmental Engineering, University of Washington, Seattle, Washington. May 2008.

Stamatiadis, C., Gartner, N. and J. Winn. “Evaluation of the Massachusetts Motorist Assistance Program: An Assessment of the Impacts of this Project on Congestion and Air Quality”. Prepared for Massachusetts Highway department. University of Massachusetts Transportation Center. Report No. 1997

Fenno, D. “Technical Support for Traffic Management, Traveler Information, and ITS Initiatives; Incident Management Support Materials; Freeway Service Patrols: A Nationwide Assessment”. Prepared for Federal Highway Administration. PB Farradyne Inc. in association with Texas Transportation Institute. 1997.

Cuciti, P., and B. Janson. “Incident Management Via Courtesy Patrol: Evaluation of a Pilot Program in Colorado”. Presented at 74th Annual Meeting of the Transportation Research Board. 1995.

Latoski, S., Pal, R., and K. Sinha. “Cost-Effectiveness Evaluation of Hoosier Helper Freeway Service Patrol”. Journal of Transportation Engineering-ASCE 125 (5), pp. 429-438. 1999.

Hawkins, P. “Evaluation of the Southwest Freeway Motorist Assistance Program in Houston”. Prepared for Texas Department of Transportation. Texas Transportation Institute. 1993.

Skabardonis, A., Noeimi, H., Petty, K., Rydzewski, D., Varaiya, P., and H. Al-Deek “Freeway Service Patrol Evaluation”. Institute of Transportation Studies, University of California Berkeley. 1995.

Minnesota Department of Transportation. “Highway Helper Summary Report”. Freeway Operations Section, Office of Traffic and Maintenance Operations. 2000.

Dougald, Lance E. “A Return on Investment Study of the Hampton Roads Safety Service Patrol Program”. Prepared for Virginia Department of Transportation. Virginia Transportation Research Council, 530 Edgemont Road, Charlottesville, VA 22903. June 2007.

Nee, Jennifer, Mark E. Hallenbeck. “Evaluation of the Service Patrol Program in the Puget Sound Region”. Prepared for the Research Office of the Washington State Department of Transportation. Washington State Transportation Center, University of Washington, Box 354802, University District Building; 1107 NE 45th Street, Suite 535 Seattle, Washington 98105-4631. September 2001.

Hagen, Larry, Huaguo Zhou, Harkanwal Singh. “Raod Ranger Benefit Cost Analysis”. Prepared for Florida Department of Transportation, Office of Project Management, Research & Development. Center for Urban Transportation Research and Development, Universtiy of South Florida, 4202 E. Fowler Avenue, CUT 100, Tampa, FL 33260-5375. November 2005.

Ozbay, Kaan, Weihua Xiao, Gaurav Jaiswal, Bekir Bartin. “Evaluation of Incident Management Strategies”. Prepared for Federal Highway Administration and New Jersey Department of Transportation. The State University of New Jersey, Civil and Environmental Engineering Department, 623 Bowser Road, Piscataway, NJ 08854. November 2005.

Chang, G. L., Ying Liu, Pei-Wei Lin, Nan Zou, and J. Y. “Point-Du-Jour, Performance Evaluation of CHART- Coordinated Highways Action Response Team”’. Maryland State Highway Administration. University of Maryland, College Park. November 2003.

Battelle Memorial Institute. “Regional Emergency Action Coordination Team (REACT) Evaluation”. Prepared for Maricopa County Department of Transportation, Phoenix, Arizona. July 19, 2002.

Siegfried, R., and W. McCasland. “Houston Motorist Assistance Program Annual Report: August 1990-July 1991”. Prepared for Texas Department of Transportation and Metropolitan Transit Authority of Harris County. Texas Transportation Institute. July 1991.

Skabardonis, A., Petty, K., Varaiya, P., and R. Bertini. “Evaluation of the Freeway Service Patrol (FSP) in Los Angeles”. Institute of Transportation Studies, University of California Berkeley. 1998.

Khattak, Asad J. and Nagui Rouphail. “Incident Management Assistance Patrols: Assessment of Investment Benefits and Costs”. Prepared for North Carolina Department of Transportation. Institute for Transportation Research and Education, NC State University, Centennial Campus, Box 8601, Raleigh NC 27695-8601. January 2005.

Chowdhury, Mashrur, Ryan Fries, Yongchang Ma, Ann Dunning, Carol Hamlin, Kaan Ozbay, Jennifer Ogle, and Wayne Sarasua. “Benefit Cost Analysis of Accelerated Incident Clearance”. Prepared for South Carolina Department of Transportation. Clemson University, Department of Civil Engineering, Lowry Hall, Box 340911, Clemson, South Carolina 29634-0911. April 2007.

Owens, Nicholas, April Armstrong, Paul Sullivan, Carol Mitchell, Diane Newton, Rebecca Brewster, Todd Trego. “Traffic Incident Management Handbook”. Prepared for Federal Highway Administration. Science Applications International Corporation, 8301 Greensboro Drive, McLean, VA 22102. January 2010.

Texas Transportation Institute. “Urban Mobility Report: Performance Measure Summary – Knoxville, TN”. Texas Transportation Institute, Texas A&M University System 3135 TAMU, College Station, Texas 77843-3135. 2009.

APPENDIX

Appendix 1

Numbers used for the input page of the model.

| TIME 1, DURATION 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median |
| Accident | 193 | 17 | 7 |  |  | 17 |
| Vehicle Fire | 4 | 1 |  |  |  | 0 |
| Disabled | 1486 | 181 | 51 | 7 |  | 97 |
| Abandoned | 511 | 172 | 29 | 9 |  | 58 |
| Debris | 266 | 27 | 6 | 1 |  | 14 |
| Pedestrian | 2 |  |  |  |  | 1 |
| Other | 170 | 13 | 6 | 2 |  | 19 |
| No Note | 11 | 2 |  |  |  | 0 |

| TIME 1, DURATION 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median |
| Accident | 13 |  |  |  |  | 1 |
| Vehicle Fire | 1 |  |  |  |  |  |
| Disabled | 7 | 4 |  |  |  |  |
| Abandoned | 1 |  |  | 1 |  |  |
| Debris |  |  |  |  |  |  |
| Pedestrian |  |  |  |  |  |  |
| Other | 1 |  |  |  |  |  |
| No Note |  |  |  |  |  |  |

| TIME 1, DURATION 3 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median |
| Accident | 2 |  |  |  |  |  |
| Vehicle Fire |  |  |  |  |  |  |
| Disabled |  | 1 |  |  |  |  |
| Abandoned |  |  |  |  |  |  |
| Debris |  |  |  |  |  |  |
| Pedestrian |  |  |  |  |  |  |
| Other |  |  |  |  |  |  |
| No Note |  |  |  |  |  |  |

| TIME 1, DURATION 4 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median |
| Accident |  |  |  |  |  |  |
| Vehicle Fire |  |  |  |  |  |  |
| Disabled | 1 |  |  |  |  |  |
| Abandoned |  |  |  |  |  |  |
| Debris |  |  |  |  |  |  |
| Pedestrian |  |  |  |  |  |  |
| Other |  |  |  |  |  |  |
| No Note |  |  |  |  |  |  |

| TIME 2, DURATION 1 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |  |  |  |  |
|  | Shoulder | One Lane |  | Two Lanes |  | Three Lanes |  | 4 or More |  | Median |  |
| Accident | 70 | 114 |  | 54 |  | 6 |  | 1 |  | 44 |  |
| Vehicle Fire | 7 | 7 |  | 2 |  | 1 |  |  |  |  |  |
| Disabled | 2724 | 97 |  | 15 |  | 1 |  |  |  | 223 |  |
| Abandoned | 773 | 9 |  |  |  |  |  |  |  | 23 |  |
| Debris | 25 | 483 |  | 108 |  | 21 |  | 3 |  | 27 |  |
| Pedestrian | 2 |  |  |  |  |  |  |  |  |  |  |
| Other | 412 | 68 |  | 8 |  | 15 |  | 8 |  | 20 |  |
| No Note | 2 | 1 |  | 1 |  | 1 |  |  |  | 1 |  |
| TIME 2, DURATION 2 |  |  |  |  |  |  |  |  |  |  |
| CAUSE | LOCATION |  |  |  |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes |  | Three Lanes |  | All Lanes |  | Median |  |
| Accident | 3 | 10 | 7 |  | 3 |  | 3 |  |  |  |
| Vehicle Fire |  |  |  |  | 1 |  |  |  |  |  |
| Disabled | 11 | 3 |  |  | 1 |  |  |  |  |  |
| Abandoned | 3 |  |  |  |  |  |  |  |  |  |
| Debris |  |  | 2 |  |  |  | 1 |  | 2 |  |
| Pedestrian |  |  |  |  |  |  |  |  |  |  |
| Other |  |  | 1 |  | 1 |  |  |  |  |  |
| No Note | 1 | 2 |  |  |  |  |  |  | 1 |  |

| TIME 2, DURATION 3 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median |
| Accident | 1 |  |  |  |  |  |
| Vehicle Fire |  |  |  |  |  |  |
| Disabled | 2 | 1 |  |  |  |  |
| Abandoned |  |  |  |  |  |  |
| Debris |  |  | 1 |  |  |  |
| Pedestrian |  |  |  |  |  |  |
| Other |  |  |  |  |  |  |
| No Note |  |  |  |  |  |  |

| TIME 2, DURATION 4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes |  | All Lanes |  | Median |  |
| Accident |  | 1 |  |  |  |  |  |  |  |
| Vehicle Fire |  |  |  |  |  |  |  |  |  |
| Disabled | 2 |  |  |  |  |  |  |  |  |
| Abandoned |  |  |  |  |  |  |  |  |  |
| Debris |  |  |  |  |  |  |  |  |  |
| Pedestrian |  |  |  |  |  |  |  |  |  |
| Other |  |  |  |  |  |  |  |  |  |
| No Note |  |  |  |  |  |  |  |  |  |
| TIME 3, DURATION 1 |  |  |  |  |  |  |  |  |
| CAUSE | LOCATION |  |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | 4 or More |  | Median |  |
| Accident | 102 | 133 | 69 | 15 |  |  | 47 |  |
| Vehicle Fire | 2 | 5 | 1 | 1 |  |  | 1 |  |
| Disabled | 2565 | 105 | 5 |  |  |  | 236 |  |
| Abandoned | 463 | 8 |  |  |  |  | 13 |  |
| Debris | 28 | 282 | 67 | 11 | 1 |  | 12 |  |
| Pedestrian | 1 |  |  |  |  |  |  |  |
| Other | 290 | 9 | 4 | 1 |  |  | 20 |  |
| No Note |  |  |  |  |  |  |  |  |

| TIME 3, DURATION 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median |
| Accident | 2 | 6 | 4 | 1 | 1 | 2 |
| Vehicle Fire |  |  |  |  |  |  |
| Disabled | 14 |  |  |  |  | 2 |
| Abandoned |  |  |  |  |  |  |
| Debris |  |  | 1 |  |  |  |
| Pedestrian |  |  |  |  |  |  |
| Other |  | 1 | 1 |  |  |  |
| No Note |  |  |  |  |  |  |

| TIME 3, DURATION 3 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median |
| Accident |  |  |  |  |  |  |
| Vehicle Fire |  |  |  |  |  |  |
| Disabled | 1 |  | 1 |  |  |  |
| Abandoned |  |  |  |  |  |  |
| Debris |  |  |  |  |  |  |
| Pedestrian |  |  |  |  |  |  |
| Other |  | 3 |  |  |  | 1 |
| No Note |  |  |  |  |  |  |
| TIME 3, DURATION 4 |  |  |  |  |  |  |
| CAUSE | LOCATION |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median |
| Accident |  |  |  |  |  |  |
| Vehicle Fire |  |  |  |  |  |  |
| Disabled |  |  |  |  |  |  |
| Abandoned |  |  |  |  |  |  |
| Debris |  |  |  |  |  |  |
| Pedestrian |  |  |  |  |  |  |
| Other |  | 1 |  |  |  |  |
| No Note |  |  |  |  |  |  |

| TIME 4, DURATION 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | 4 or More | Median |
| Accident | 32 | 35 | 24 | 6 | 1 | 20 |
| Vehicle Fire | 3 | 1 | 3 |  |  |  |
| Disabled | 816 | 47 | 3 |  |  | 59 |
| Abandoned | 260 | 2 |  |  |  | 5 |
| Debris | 11 | 87 | 21 | 2 |  | 6 |
| Pedestrian | 1 |  |  |  |  |  |
| Other | 69 | 9 | 2 | 2 | 1 | 7 |
| No Note |  | 1 | 1 |  |  |  |

| TIME 4, DURATION 2 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes |  | Three Lanes |  | All Lanes |  | Median |  |
| Accident | 2 | 4 | 5 |  | 2 |  | 1 |  | 1 |  |
| Vehicle Fire |  |  |  |  |  |  |  |  |  |  |
| Disabled | 2 | 3 |  |  |  |  |  |  | 1 |  |
| Abandoned |  | 1 |  |  |  |  |  |  |  |  |
| Debris |  |  |  |  |  |  |  |  |  |  |
| Pedestrian |  |  |  |  |  |  |  |  |  |  |
| Other |  | 3 | 1 |  |  |  |  |  |  |  |
| No Note |  |  |  |  |  |  |  |  |  |  |
| TIME 4, DURATION 3 |  |  |  |  |  |  |  |  |  |
| CAUSE | LOCATION |  |  |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes |  | All Lanes |  | Median |  |
| Accident |  |  | 1 |  |  |  |  |  |  |
| Vehicle Fire |  |  |  |  |  |  |  |  |  |
| Disabled |  | 1 |  |  |  |  |  |  |  |
| Abandoned |  |  |  |  |  |  |  |  |  |
| Debris |  |  |  |  |  |  |  |  |  |
| Pedestrian |  |  |  |  |  |  |  |  |  |
| Other |  |  |  |  |  |  |  |  |  |
| No Note |  |  |  |  |  |  |  |  |  |

| TIME 4, DURATION 4 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median |
| Accident |  |  |  | 1 |  |  |
| Vehicle Fire |  |  |  |  |  |  |
| Disabled | 2 | 1 |  |  |  |  |
| Abandoned |  |  |  |  |  |  |
| Debris |  |  |  |  |  |  |
| Pedestrian |  |  |  |  |  |  |
| Other | 2 |  | 2 |  | 1 |  |
| No Note |  |  |  |  |  |  |

Appendix 2

Test results showing when to use linear interpolation versus the queuing diagram.

For time periods 1 and 3 where capacity is 6000 vehicles per hour and demand is 5000 vehicles per hour.

| Cause | Location |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median |  |
| Blocking | Linear | Queuing | Queuing | Queuing | Queuing | Queuing |  |
| Non-Blocking | Queuing | Queuing | Queuing | Queuing | Queuing | Queuing |  |

Reduced capacity estimates for time periods 1 and 3.

| Cause | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median |
| --- | --- | --- | --- | --- | --- | --- |
| Blocking | 0.85 | 0.6 | 0.3 | 0.15 | 0 | 0.8 |
| Non-Blocking | 0.8325 | 0.8317 | 0.8 | 0.6667 | 0.5 | 0.8317 |

For time period 2 where capacity is 6000 vehicles per hour and demand is 4500 vehicles per hour.

| Cause | Location |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median |  |
| Blocking | Linear | Queuing | Queuing | Queuing | Queuing | Linear |  |
| Non-Blocking | Linear | Linear | Linear | Queuing | Queuing | Linear |  |

Reduced capacity estimates for time period 2

| Cause | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median |
| --- | --- | --- | --- | --- | --- | --- |
| Blocking | 0.85 | 0.6 | 0.3 | 0.15 | 0 | 0.8 |
| Non-Blocking | 0.8325 | 0.8317 | 0.8 | 0.6667 | 0.5 | 0.8317 |

For time period 4 where capacity is 6000 vehicles per hour and demand is 3000 vehicles per hour.

| Cause | Location |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median |  |
| Blocking | Linear | Linear | Queuing | Queuing | Queuing | Linear |  |
| Non-Blocking | Linear | Linear | Linear | Linear | Linear | Linear |  |

Reduced capacity estimates for time period 4

| Cause | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median |
| --- | --- | --- | --- | --- | --- | --- |
| Blocking | 0.85 | 0.6 | 0.25 | 0.15 | 0 | 0.8 |
| Non-Blocking | 0.8325 | 0.8317 | 0.8 | 0.6667 | 0.5 | 0.8317 |

Appendix 3

Values of delay used in the model.

| TIME 1, DURATION 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | Value of Time |
| Accident | 5.4 | 58.05 | 1289.3 | 2343.6 | 3690.9 | 10.8 | $36.00 |
| Vehicle Fire | 13.5 | 526.5 | 3164.4 | 5273.1 | 7909.7 | 27 |  |
| Disabled | 1.35 | 2.7 | 63.45 | 527.85 | 1582.2 | 2.7 |  |
| Abandoned | 1.35 | 2.7 | 63.45 | 527.85 | 1582.2 | 2.7 |  |
| Debris | 1.35 | 2.7 | 63.45 | 527.85 | 1582.2 | 2.7 |  |
| Pedestrian | 1.35 | 2.7 | 63.45 | 527.85 | 1582.2 | 2.7 |  |
| Other | 1.35 | 2.7 | 63.45 | 527.85 | 1582.2 | 2.7 |  |
| No Note | 1.35 | 2.7 | 63.45 | 527.85 | 1582.2 | 2.7 |  |
| Ratio | 1 |  |  |  |  |  |  |

| TIME 1, DURATION 2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | Value of Time |
| Accident | 14.4 | 159.84 | 3519.7 | 6399.7 | 10080 | 29.88 | $36.00 |
| Vehicle Fire | 37.8 | 1440 | 8640 | 14400 | 21600 | 79.2 |  |
| Disabled | 3.6 | 7.2 | 172.8 | 1440 | 4320 | 7.2 |  |
| Abandoned | 3.6 | 7.2 | 172.8 | 1440 | 4320 | 7.2 |  |
| Debris | 3.6 | 7.2 | 172.8 | 1440 | 4320 | 7.2 |  |
| Pedestrian | 3.6 | 7.2 | 172.8 | 1440 | 4320 | 7.2 |  |
| Other | 3.6 | 7.2 | 172.8 | 1440 | 4320 | 7.2 |  |
| No Note | 3.6 | 7.2 | 172.8 | 1440 | 4320 | 7.2 |  |
| Ratio | 1 |  |  |  |  |  |  |

| TIME 1, DURATION 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | Value of Time |
| Accident | 28.525 | 310.98 | 6844.4 | 12444 | 19600 | 58.275 | $36.00 |
| Vehicle Fire | 73.5 | 2800 | 16800 | 28000 | 42000 | 154 |  |
| Disabled | 7 | 14 | 336 | 2800 | 8400 | 14 |  |
| Abandoned | 7 | 14 | 336 | 2800 | 8400 | 14 |  |
| Debris | 7 | 14 | 336 | 2800 | 8400 | 14 |  |
| Pedestrian | 7 | 14 | 336 | 2800 | 8400 | 14 |  |
| Other | 7 | 14 | 336 | 2800 | 8400 | 14 |  |
| No Note | 7 | 14 | 336 | 2800 | 8400 | 14 |  |
| Ratio | 1 |  |  |  |  |  |  |

| TIME 1, DURATION 4 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | Value of Time |
| Accident | 41.838 | 456 | 10032 | 18240 | 28728 | 85.272 | $36.00 |
| Vehicle Fire | 107.73 | 4104 | 24624 | 41040 | 61560 | 225.72 |  |
| Disabled | 10.26 | 20.52 | 492.48 | 4104 | 12312 | 20.52 |  |
| Abandoned | 10.26 | 20.52 | 492.48 | 4104 | 12312 | 20.52 |  |
| Debris | 10.26 | 20.52 | 492.48 | 4104 | 12312 | 20.52 |  |
| Pedestrian | 10.26 | 20.52 | 492.48 | 4104 | 12312 | 20.52 |  |
| Other | 10.26 | 20.52 | 492.48 | 4104 | 12312 | 20.52 |  |
| No Note | 10.26 | 20.52 | 492.48 | 4104 | 12312 | 20.52 |  |
| Ratio | 1 |  |  |  |  |  |  |

| TIME 2, DURATION 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | 4 or More | Median | Value of Time |
| Accident | 2.1333 | 27 | 351.56 | 878.91 | 1582 | 4.2666 | $36.00 |
| Vehicle Fire | 4.2666 | 175.5 | 1757.7 | 3075.3 | 4746.6 | 10.667 |  |
| Disabled | 0.5333 | 1.0667 | 1.6667 | 175.5 | 791.1 | 1.0667 |  |
| Abandoned | 0.5333 | 1.0667 | 1.6667 | 175.5 | 791.1 | 1.0667 |  |
| Debris | 0.5333 | 1.0667 | 1.6667 | 175.5 | 791.1 | 1.0667 |  |
| Pedestrian | 0.5333 | 1.0667 | 1.6667 | 175.5 | 791.1 | 1.0667 |  |
| Other | 0.5333 | 1.0667 | 1.6667 | 175.5 | 791.1 | 1.0667 |  |
| No Note | 0.5333 | 1.0667 | 1.6667 | 175.5 | 791.1 | 1.0667 |  |
| Ratio | 1 |  |  |  |  |  |  |

| TIME 2, DURATION 2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | Value of Time |
| Accident | 2.1333 | 74.88 | 960 | 2400 | 4320 | 4.2666 | $36.00 |
| Vehicle Fire | 4.2666 | 74.88 | 960 | 2400 | 4320 | 10.667 |  |
| Disabled | 0.5333 | 1.0667 | 5.3333 | 479.88 | 2160 | 1.0667 |  |
| Abandoned | 0.5333 | 1.0667 | 5.3333 | 479.88 | 2160 | 1.0667 |  |
| Debris | 0.5333 | 1.0667 | 5.3333 | 479.88 | 2160 | 1.0667 |  |
| Pedestrian | 0.5333 | 1.0667 | 5.3333 | 479.88 | 2160 | 1.0667 |  |
| Other | 0.5333 | 1.0667 | 5.3333 | 479.88 | 2160 | 1.0667 |  |
| No Note | 0.5333 | 1.0667 | 5.3333 | 479.88 | 2160 | 1.0667 |  |
| Ratio | 1 |  |  |  |  |  |  |

| TIME 2, DURATION 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | Value of Time |
| Accident | 2.1333 | 145.78 | 1866.7 | 4666.7 | 8400 | 4.2666 | $36.00 |
| Vehicle Fire | 4.2666 | 145.78 | 1866.7 | 4666.7 | 8400 | 10.667 |  |
| Disabled | 0.5333 | 1.0667 | 5.3333 | 933.28 | 4200 | 1.0667 |  |
| Abandoned | 0.5333 | 1.0667 | 5.3333 | 933.28 | 4200 | 1.0667 |  |
| Debris | 0.5333 | 1.0667 | 5.3333 | 933.28 | 4200 | 1.0667 |  |
| Pedestrian | 0.5333 | 1.0667 | 5.3333 | 933.28 | 4200 | 1.0667 |  |
| Other | 0.5333 | 1.0667 | 5.3333 | 933.28 | 4200 | 1.0667 |  |
| No Note | 0.5333 | 1.0667 | 5.3333 | 933.28 | 4200 | 1.0667 |  |
| Ratio | 1 |  |  |  |  |  |  |

| TIME 2, DURATION 4 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | Value of Time |
| Accident | 2.1333 | 213.75 | 2736 | 6840 | 12312 | 4.2666 | $36.00 |
| Vehicle Fire | 4.2666 | 213.75 | 2736 | 6840 | 12312 | 10.667 |  |
| Disabled | 0.5333 | 1.0667 | 5.3333 | 1368 | 6156 | 1.0667 |  |
| Abandoned | 0.5333 | 1.0667 | 5.3333 | 1368 | 6156 | 1.0667 |  |
| Debris | 0.5333 | 1.0667 | 5.3333 | 1368 | 6156 | 1.0667 |  |
| Pedestrian | 0.5333 | 1.0667 | 5.3333 | 1368 | 6156 | 1.0667 |  |
| Other | 0.5333 | 1.0667 | 5.3333 | 1368 | 6156 | 1.0667 |  |
| No Note | 0.5333 | 1.0667 | 5.3333 | 1368 | 6156 | 1.0667 |  |
| Ratio | 1 |  |  |  |  |  |  |

| TIME 3, DURATION 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | 4 or More | Median | Value of Time |
| Accident | 5.4 | 58.05 | 1289.3 | 2343.6 | 3690.9 | 10.8 | $32.00 |
| Vehicle Fire | 13.5 | 526.5 | 3164.4 | 5273.1 | 7909.7 | 27 |  |
| Disabled | 1.35 | 2.7 | 63.45 | 527.85 | 1582.2 | 2.7 |  |
| Abandoned | 1.35 | 2.7 | 63.45 | 527.85 | 1582.2 | 2.7 |  |
| Debris | 1.35 | 2.7 | 63.45 | 527.85 | 1582.2 | 2.7 |  |
| Pedestrian | 1.35 | 2.7 | 63.45 | 527.85 | 1582.2 | 2.7 |  |
| Other | 1.35 | 2.7 | 63.45 | 527.85 | 1582.2 | 2.7 |  |
| No Note | 1.35 | 2.7 | 63.45 | 527.85 | 1582.2 | 2.7 |  |
| Ratio | 1 |  |  |  |  |  |  |

| TIME 3, DURATION 2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | Value of Time |
| Accident | 14.4 | 159.84 | 3519.7 | 6399.7 | 10080 | 29.88 | $32.00 |
| Vehicle Fire | 37.8 | 1440 | 8640 | 14400 | 21600 | 79.2 |  |
| Disabled | 1.152 | 2.304 | 172.8 | 1440 | 4320 | 2.304 |  |
| Abandoned | 1.152 | 2.304 | 172.8 | 1440 | 4320 | 2.304 |  |
| Debris | 1.152 | 2.304 | 172.8 | 1440 | 4320 | 2.304 |  |
| Pedestrian | 1.152 | 2.304 | 172.8 | 1440 | 4320 | 2.304 |  |
| Other | 1.152 | 2.304 | 172.8 | 1440 | 4320 | 2.304 |  |
| No Note | 1.152 | 2.304 | 172.8 | 1440 | 4320 | 2.304 |  |
| Ratio | 1 |  |  |  |  |  |  |

| TIME 3, DURATION 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | Value of Time |
| Accident | 28.525 | 310.98 | 6844.4 | 12444 | 19600 | 58.275 | $32.00 |
| Vehicle Fire | 73.5 | 2800 | 16800 | 28000 | 42000 | 154 |  |
| Disabled | 1.12 | 2.24 | 336 | 2800 | 8400 | 2.24 |  |
| Abandoned | 1.12 | 2.24 | 336 | 2800 | 8400 | 2.24 |  |
| Debris | 1.12 | 2.24 | 336 | 2800 | 8400 | 2.24 |  |
| Pedestrian | 1.12 | 2.24 | 336 | 2800 | 8400 | 2.24 |  |
| Other | 1.12 | 2.24 | 336 | 2800 | 8400 | 2.24 |  |
| No Note | 1.12 | 2.24 | 336 | 2800 | 8400 | 2.24 |  |
| Ratio | 1 |  |  |  |  |  |  |

| TIME 3, DURATION 4 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | Value of Time |
| Accident | 41.838 | 456 | 10032 | 18240 | 28728 | 85.272 | $32.00 |
| Vehicle Fire | 107.73 | 4104 | 24624 | 41040 | 61560 | 225.72 |  |
| Disabled | 1.0944 | 2.1888 | 492.48 | 4104 | 12312 | 2.1888 |  |
| Abandoned | 1.0944 | 2.1888 | 492.48 | 4104 | 12312 | 2.1888 |  |
| Debris | 1.0944 | 2.1888 | 492.48 | 4104 | 12312 | 2.1888 |  |
| Pedestrian | 1.0944 | 2.1888 | 492.48 | 4104 | 12312 | 2.1888 |  |
| Other | 1.0944 | 2.1888 | 492.48 | 4104 | 12312 | 2.1888 |  |
| No Note | 1.0944 | 2.1888 | 492.48 | 4104 | 12312 | 2.1888 |  |
| Ratio | 1 |  |  |  |  |  |  |

| TIME 4, DURATION 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | 4 or More | Median | Value of Time |
| Accident | 2.1333 | 8.1 | 151.2 | 565.65 | 1130 | 4.2666 | $60.00 |
| Vehicle Fire | 4.2666 | 10.8 | 151.2 | 565.65 | 1130 | 8.5333 |  |
| Disabled | 0.5333 | 1.0667 | 1.0667 | 5.3333 | 10.667 | 1.0667 |  |
| Abandoned | 0.5333 | 1.0667 | 1.0667 | 5.3333 | 10.667 | 1.0667 |  |
| Debris | 0.5333 | 1.0667 | 1.0667 | 5.3333 | 10.667 | 1.0667 |  |
| Pedestrian | 0.5333 | 1.0667 | 1.0667 | 5.3333 | 10.667 | 1.0667 |  |
| Other | 0.5333 | 1.0667 | 1.0667 | 5.3333 | 10.667 | 1.0667 |  |
| No Note | 0.5333 | 1.0667 | 1.0667 | 5.3333 | 10.667 | 1.0667 |  |
| Ratio | 1 |  |  |  |  |  |  |

| TIME 4, DURATION 2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | Value of Time |
| Accident | 2.1333 | 6.912 | 411.48 | 1543 | 3085.6 | 4.2666 | $60.00 |
| Vehicle Fire | 4.2666 | 9.216 | 411.48 | 1543 | 3085.6 | 8.5333 |  |
| Disabled | 0.5333 | 1.0667 | 1.0667 | 5.3333 | 10.667 | 1.0667 |  |
| Abandoned | 0.5333 | 1.0667 | 1.0667 | 5.3333 | 10.667 | 1.0667 |  |
| Debris | 0.5333 | 1.0667 | 1.0667 | 5.3333 | 10.667 | 1.0667 |  |
| Pedestrian | 0.5333 | 1.0667 | 1.0667 | 5.3333 | 10.667 | 1.0667 |  |
| Other | 0.5333 | 1.0667 | 1.0667 | 5.3333 | 10.667 | 1.0667 |  |
| No Note | 0.5333 | 1.0667 | 1.0667 | 5.3333 | 10.667 | 1.0667 |  |
| Ratio | 1 |  |  |  |  |  |  |

| TIME 4, DURATION 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | Value of Time |
| Accident | 2.1333 | 6.72 | 799.93 | 3000 | 6000.1 | 4.2666 | $60.00 |
| Vehicle Fire | 4.2666 | 8.96 | 799.93 | 3000 | 6000.1 | 8.5333 |  |
| Disabled | 0.5333 | 1.0667 | 1.0667 | 5.3333 | 10.667 | 1.0667 |  |
| Abandoned | 0.5333 | 1.0667 | 1.0667 | 5.3333 | 10.667 | 1.0667 |  |
| Debris | 0.5333 | 1.0667 | 1.0667 | 5.3333 | 10.667 | 1.0667 |  |
| Pedestrian | 0.5333 | 1.0667 | 1.0667 | 5.3333 | 10.667 | 1.0667 |  |
| Other | 0.5333 | 1.0667 | 1.0667 | 5.3333 | 10.667 | 1.0667 |  |
| No Note | 0.5333 | 1.0667 | 1.0667 | 5.3333 | 10.667 | 1.0667 |  |
| Ratio | 1 |  |  |  |  |  |  |

| TIME 4, DURATION 4 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | Value of Time |
| Accident | 2.1333 | 6.5664 | 1172.6 | 4397.1 | 8794.3 | 4.2666 | $60.00 |
| Vehicle Fire | 4.2666 | 8.7552 | 1172.6 | 4397.1 | 8794.3 | 8.5332 |  |
| Disabled | 0.5333 | 1.0667 | 1.0667 | 5.3333 | 10.667 | 1.0667 |  |
| Abandoned | 0.5333 | 1.0667 | 1.0667 | 5.3333 | 10.667 | 1.0667 |  |
| Debris | 0.5333 | 1.0667 | 1.0667 | 5.3333 | 10.667 | 1.0667 |  |
| Pedestrian | 0.5333 | 1.0667 | 1.0667 | 5.3333 | 10.667 | 1.0667 |  |
| Other | 0.5333 | 1.0667 | 1.0667 | 5.3333 | 10.667 | 1.0667 |  |
| No Note | 0.5333 | 1.0667 | 1.0667 | 5.3333 | 10.667 | 1.0667 |  |
| Ratio | 1 |  |  |  |  |  |  |

Appendix 4

Delay savings calculated by the model.

| TIME 1, DURATION 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | TOTAL |
| Accident | $37,519.20 | $35,526.60 | $324,891.00 | $0.00 | $0.00 | $6,609.60 | $404,546.40 |
| Vehicle Fire | $1,944.00 | $18,954.00 | $0.00 | $0.00 | $0.00 | $0.00 | $20,898.00 |
| Disabled | $72,219.60 | $17,593.20 | $116,494.20 | $133,018.20 | $0.00 | $9,428.40 | $348,753.60 |
| Abandoned | $24,834.60 | $16,718.40 | $66,241.80 | $171,023.40 | $0.00 | $5,637.60 | $284,455.80 |
| Debris | $12,927.60 | $2,624.40 | $13,705.20 | $19,002.60 | $0.00 | $1,360.80 | $49,620.60 |
| Pedestrian | $97.20 | $0.00 | $0.00 | $0.00 | $0.00 | $97.20 | $194.40 |
| Other | $8,262.00 | $1,263.60 | $13,705.20 | $38,005.20 | $0.00 | $1,846.80 | $63,082.80 |
| No Note | $534.60 | $194.40 | $0.00 | $0.00 | $0.00 | $0.00 | $729.00 |
| TOTAL | $158,338.80 | $92,874.60 | $535,037.40 | $361,049.40 | $0.00 | $24,980.40 | $1,172,280.60 |

| TIME 1, DURATION 2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | TOTAL |
| Accident | $6,739.20 | $0.00 | $0.00 | $0.00 | $0.00 | $1,075.68 | $7,814.88 |
| Vehicle Fire | $1,360.80 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $1,360.80 |
| Disabled | $907.20 | $1,036.80 | $0.00 | $0.00 | $0.00 | $0.00 | $1,944.00 |
| Abandoned | $129.60 | $0.00 | $0.00 | $51,840.00 | $0.00 | $0.00 | $51,969.60 |
| Debris | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Pedestrian | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Other | $129.60 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $129.60 |
| No Note | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| TOTAL | $9,266.40 | $1,036.80 | $0.00 | $51,840.00 | $0.00 | $1,075.68 | $63,218.88 |

| TIME 1, DURATION 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | TOTAL |
| Accident | $2,053.80 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $2,053.80 |
| Vehicle Fire | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Disabled | $0.00 | $504.00 | $0.00 | $0.00 | $0.00 | $0.00 | $504.00 |
| Abandoned | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Debris | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Pedestrian | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Other | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| No Note | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| TOTAL | $2,053.80 | $504.00 | $0.00 | $0.00 | $0.00 | $0.00 | $2,557.80 |

| TIME 1, DURATION 4 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | TOTAL |
| Accident | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Vehicle Fire | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Disabled | $369.36 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $369.36 |
| Abandoned | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Debris | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Pedestrian | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Other | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| No Note | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| TOTAL | $369.36 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $369.36 |

| TIME 2, DURATION 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | 4 or More | Median | TOTAL |
| Accident | $5,375.97 | $110,808.00 | $683,437.50 | $189,843.75 | $56,953.13 | $6,758.36 | $1,053,176.70 |
| Vehicle Fire | $1,075.19 | $44,226.00 | $126,554.40 | $110,710.80 | $0.00 | $0.00 | $282,566.39 |
| Disabled | $52,300.47 | $3,724.78 | $900.00 | $6,318.00 | $0.00 | $8,563.15 | $71,806.39 |
| Abandoned | $14,841.51 | $345.60 | $0.00 | $0.00 | $0.00 | $883.19 | $16,070.30 |
| Debris | $480.00 | $18,547.08 | $6,479.97 | $132,678.00 | $85,438.80 | $1,036.79 | $244,660.65 |
| Pedestrian | $38.40 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $38.40 |
| Other | $7,910.35 | $2,611.18 | $480.00 | $94,770.00 | $227,836.80 | $768.00 | $334,376.33 |
| No Note | $38.40 | $38.40 | $60.00 | $6,318.00 | $0.00 | $38.40 | $6,493.20 |
| TOTAL | $82,060.29 | $180,301.04 | $817,911.87 | $540,638.55 | $370,228.73 | $18,047.89 | $2,009,188.36 |

| TIME 2, DURATION 2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | TOTAL |
| Accident | $230.40 | $26,956.80 | $241,920.00 | $259,200.00 | $466,560.00 | $0.00 | $994,867.20 |
| Vehicle Fire | $0.00 | $0.00 | $0.00 | $86,400.00 | $0.00 | $0.00 | $86,400.00 |
| Disabled | $211.20 | $115.20 | $0.00 | $17,275.68 | $0.00 | $0.00 | $17,602.08 |
| Abandoned | $57.60 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $57.60 |
| Debris | $0.00 | $0.00 | $384.00 | $0.00 | $77,760.00 | $76.80 | $78,220.80 |
| Pedestrian | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Other | $0.00 | $0.00 | $192.00 | $17,275.68 | $0.00 | $0.00 | $17,467.68 |
| No Note | $19.20 | $76.80 | $0.00 | $0.00 | $0.00 | $38.40 | $134.40 |
| TOTAL | $518.40 | $27,148.80 | $242,496.00 | $380,151.36 | $544,320.00 | $115.20 | $1,194,749.75 |

| TIME 2, DURATION 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | TOTAL |
| Accident | $76.80 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $76.80 |
| Vehicle Fire | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Disabled | $38.40 | $38.40 | $0.00 | $0.00 | $0.00 | $0.00 | $76.80 |
| Abandoned | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Debris | $0.00 | $0.00 | $192.00 | $0.00 | $0.00 | $0.00 | $192.00 |
| Pedestrian | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Other | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| No Note | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| TOTAL | $115.20 | $38.40 | $192.00 | $0.00 | $0.00 | $0.00 | $345.60 |

| TIME 2, DURATION 4 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | TOTAL |
| Accident | $0.00 | $7,695.00 | $0.00 | $0.00 | $0.00 | $0.00 | $7,695.00 |
| Vehicle Fire | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Disabled | $38.40 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $38.40 |
| Abandoned | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Debris | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Pedestrian | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Other | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| No Note | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| TOTAL | $38.40 | $7,695.00 | $0.00 | $0.00 | $0.00 | $0.00 | $7,733.40 |

| TIME 3, DURATION 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | 4 or More | Median | TOTAL |
| Accident | $17,625.60 | $247,060.80 | $2,846,664.00 | $1,124,928.00 | $0.00 | $16,243.20 | $4,252,521.60 |
| Vehicle Fire | $864.00 | $84,240.00 | $101,260.80 | $168,739.20 | $0.00 | $864.00 | $355,968.00 |
| Disabled | $110,808.00 | $9,072.00 | $10,152.00 | $0.00 | $0.00 | $20,390.40 | $150,422.40 |
| Abandoned | $20,001.60 | $691.20 | $0.00 | $0.00 | $0.00 | $1,123.20 | $21,816.00 |
| Debris | $1,209.60 | $24,364.80 | $136,036.80 | $185,803.20 | $50,630.40 | $1,036.80 | $399,081.60 |
| Pedestrian | $43.20 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $43.20 |
| Other | $12,528.00 | $777.60 | $8,121.60 | $16,891.20 | $0.00 | $1,728.00 | $40,046.40 |
| No Note | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| TOTAL | $163,080.00 | $366,206.40 | $3,102,235.20 | $1,496,361.60 | $50,630.40 | $41,385.60 | $5,219,899.20 |

| TIME 3, DURATION 2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | TOTAL |
| Accident | $921.60 | $30,689.28 | $450,524.16 | $204,791.04 | $322,560.00 | $1,912.32 | $1,011,398.40 |
| Vehicle Fire | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Disabled | $516.10 | $0.00 | $0.00 | $0.00 | $0.00 | $147.46 | $663.55 |
| Abandoned | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Debris | $0.00 | $0.00 | $5,529.60 | $0.00 | $0.00 | $0.00 | $5,529.60 |
| Pedestrian | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Other | $0.00 | $73.73 | $5,529.60 | $0.00 | $0.00 | $0.00 | $5,603.33 |
| No Note | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| TOTAL | $1,437.70 | $30,763.01 | $461,583.36 | $204,791.04 | $322,560.00 | $2,059.78 | $1,023,194.88 |

| TIME 3, DURATION 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | TOTAL |
| Accident | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Vehicle Fire | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Disabled | $35.84 | $0.00 | $10,752.00 | $0.00 | $0.00 | $0.00 | $10,787.84 |
| Abandoned | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Debris | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Pedestrian | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Other | $0.00 | $215.04 | $0.00 | $0.00 | $0.00 | $71.68 | $286.72 |
| No Note | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| TOTAL | $35.84 | $215.04 | $10,752.00 | $0.00 | $0.00 | $71.68 | $11,074.56 |

| TIME 3, DURATION 4 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | TOTAL |
| Accident | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Vehicle Fire | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Disabled | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Abandoned | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Debris | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Pedestrian | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Other | $0.00 | $70.04 | $0.00 | $0.00 | $0.00 | $0.00 | $70.04 |
| No Note | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| TOTAL | $0.00 | $70.04 | $0.00 | $0.00 | $0.00 | $0.00 | $70.04 |

| TIME 4, DURATION 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | 4 or More | Median | TOTAL |
| Accident | $4,095.97 | $17,010.00 | $217,728.00 | $203,634.00 | $67,797.00 | $5,119.97 | $515,384.94 |
| Vehicle Fire | $768.00 | $648.00 | $27,216.00 | $0.00 | $0.00 | $0.00 | $28,632.00 |
| Disabled | $26,111.84 | $3,007.98 | $192.00 | $0.00 | $0.00 | $3,775.98 | $33,087.79 |
| Abandoned | $8,319.95 | $128.00 | $0.00 | $0.00 | $0.00 | $320.00 | $8,767.95 |
| Debris | $352.00 | $5,567.97 | $1,343.99 | $640.00 | $0.00 | $384.00 | $8,287.95 |
| Pedestrian | $32.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $32.00 |
| Other | $2,207.99 | $576.00 | $128.00 | $640.00 | $640.00 | $448.00 | $4,639.97 |
| No Note | $0.00 | $64.00 | $64.00 | $0.00 | $0.00 | $0.00 | $128.00 |
| TOTAL | $41,887.74 | $27,001.94 | $246,671.99 | $204,913.99 | $68,437.00 | $10,047.94 | $598,960.59 |

| TIME 4, DURATION 2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | TOTAL |
| Accident | $256.00 | $1,658.88 | $123,444.00 | $185,155.20 | $185,133.60 | $256.00 | $495,903.68 |
| Vehicle Fire | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Disabled | $64.00 | $192.00 | $0.00 | $0.00 | $0.00 | $64.00 | $320.00 |
| Abandoned | $0.00 | $64.00 | $0.00 | $0.00 | $0.00 | $0.00 | $64.00 |
| Debris | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Pedestrian | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Other | $0.00 | $192.00 | $64.00 | $0.00 | $0.00 | $0.00 | $256.00 |
| No Note | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| TOTAL | $320.00 | $2,106.88 | $123,508.00 | $185,155.20 | $185,133.60 | $320.00 | $496,543.68 |

| TIME 4, DURATION 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | TOTAL |
| Accident | $0.00 | $0.00 | $47,995.50 | $0.00 | $0.00 | $0.00 | $47,995.50 |
| Vehicle Fire | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Disabled | $0.00 | $64.00 | $0.00 | $0.00 | $0.00 | $0.00 | $64.00 |
| Abandoned | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Debris | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Pedestrian | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Other | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| No Note | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| TOTAL | $0.00 | $64.00 | $47,995.50 | $0.00 | $0.00 | $0.00 | $48,059.50 |

| TIME 4, DURATION 4 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUSE | LOCATION |  |  |  |  |  |  |
|  | Shoulder | One Lane | Two Lanes | Three Lanes | All Lanes | Median | TOTAL |
| Accident | $0.00 | $0.00 | $0.00 | $263,825.64 | $0.00 | $0.00 | $263,825.64 |
| Vehicle Fire | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Disabled | $64.00 | $64.00 | $0.00 | $0.00 | $0.00 | $0.00 | $128.00 |
| Abandoned | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Debris | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Pedestrian | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| Other | $64.00 | $0.00 | $128.00 | $0.00 | $639.99 | $0.00 | $831.99 |
| No Note | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 | $0.00 |
| TOTAL | $128.00 | $64.00 | $128.00 | $263,825.64 | $639.99 | $0.00 | $264,785.63 |

Appendix 5

Graphs showing number of vehicles at selected locations for thirty second aggregate data.

# VITA

Sam Moss was born in Knoxville, Tennessee and lived there for most of his life.  He attended elementary through high school in Knoxville and upon graduation from high school attended the University of Tennessee.  He studied Civil Engineering as an undergraduate student and completed a co-op in Atlanta, Georgia.  Upon graduation he received his Engineer Intern license in the state of Tennessee and accepted a graduate research position at the University of Tennessee.  Currently he is a Master’s degree candidate in Civil Engineering with a concentration in transportation engineering at the University of Tennessee.

Stamatiadis 1997

Fenno 1997

Cuciti 1995

Latoski 1999

Hawkins 1993

Skabardonis 1995

Minnesota DOT 2000

Dougald 2007

Nee 2001

Sun 2010

Guin 2007

Hagen 2005

Ozbay 2005

Stamatiadis 1997

Fenno 1997

Cuciti 1995

Latoski 1999

Seigfried 1991

Hawkins 1993

Skabardonis 1998

Skabardonis 1995

Minnesota DOT 2000

Dougald 2007

Sun 2010

Guin 2007

Clark 2005

Clark 2005

Khattak 2005

Chowdhury 2007

Battelle 2002