To the Graduate Council:

I am submitting herewith a thesis written by James Tate Geren entitled “Streamflow Reconstructions of Southern Appalachian (North Carolina) Headwater Gages Using Tree Rings.”  I have examined the final electronic copy of this thesis for form and content and recommend that it be accepted in partial fulfillment of the requirements for the degree of Master of Science, with a major in Environmental Engineering.

Glenn Tootle, Major Professor

We have read this thesis

and recommend its acceptance:

John Schwartz

Henri Grissino-Mayer

Accepted for the Council:

Carolyn R. Hodges

Vice Provost and Dean of the Graduate School

(Original signatures are on file with official student records.)
Streamflow Reconstructions of Southern Appalachian (North Carolina) Headwater Gages Using Tree Rings

A Thesis Presented for the

Master of Science

Degree

The University of Tennessee, Knoxville

James Tate Geren

December 2010

ACKNOWLEDGEMENTS

This research is supported by the University of Tennessee.  It is funded by the United States Geological Survey 104B program.  A special thanks to all those that reviewed this thesis:  Glenn Tootle, Henri Grissino-Mayer, John Schwartz, Cody Moser, SallyRose Anderson, Cole Geren, and Ross Ogle.

ABSTRACT

Tree rings have been used as a proxy in reconstructing streamflow in the western U.S. for many years, but few reconstructions have been attempted in the eastern United States.  Clear limitations exist for streamflow reconstructions in the eastern U.S. compared to the western U.S., but value can be established as demonstrated in this research.  The primary goal of this research was to reconstruct streamflow using data from five headwater gages in the Appalachian Mountains of North Carolina.  These gages are located on the Valley River, the Oconaluftee River, the Nantahala River, the Little Tennessee River, and the Watauga River.  Tree-ring chronologies were used to reconstruct streamflow.  Tree-ring chronology predictors were selected using a seasonal correlation analysis.  Seasonal correlation analysis revealed May-June-July (MJJ) streamflow variability being highly correlated with tree-ring chronologies in the study region and vicinity.  Stepwise linear regression methods were used to reconstruct MJJ streamflow.  The reconstructions for the Valley, Oconaluftee, and Nantahala Rivers were considered acceptable reconstructions because the models explained approximately 50% of the total variance in historic period MJJ streamflow records.  These three streamflow reconstruction models have predictive skill indicated by a positive reduction of error (RE) values.  The root mean square error (RMSE) statistic was 11.5 million cubic meters (MCM) for the Valley River (26% of the mean reconstructed MJJ flow), 15.9 MCM for the Oconaluftee River (16% of the mean reconstructed MJJ flow), and 8.2 MCM for the Nantahala River (20% of the mean reconstructed MJJ flow).  Analysis of the reconstructed streamflow data for these three rivers revealed low flow periods from 1710 to 1712 at all three sites.  The research presented here shows the potential benefit of using tree-ring chronologies to reconstruct streamflow in the Tennessee Valley region by demonstrating the ability of proxy-based reconstructions to provide useful data beyond the instrumental record.  These useful data include identification of extreme wet or dry periods and oscillations in the historical reconstructions that are not visible in the instrumental data.

Table of Contents

Introduction	1

Research Background	1

Site Description	3

Data	4

Streamflow Data	4

Tree-Ring Chronologies	5

Methods	6

Pre-Screening Methods	6

Reconstruction Methods	7

Streamflow Reconstruction Analysis Methods	9

Results	10

Seasonal Correlations	10

Calibration and Verification of Reconstructions	11

Discussion	15

Importance of Climate Reconstructions Eastern U.S.	15

Limitations of Eastern U.S. Reconstructions	17

Utilizing Larger Tree-Ring Chronology Search Radius	18

Streamflow Reconstruction Analysis	19

Conclusions	21

Literature Cited	22

Appendices	26

Appendix 1: Tables and Figures	27

Tables	27

Figures	35

Appendix 2:  Reconstruction Data	51

Tree Based	51

Kelsey Tract II Residuals	52

Pearl River Residuals	55

Piney Creek Pocket Wilderness Residuals	60

Ramsey’s Draft Recollection Residuals	64

Black River Residuals	68

Grandfather Mountain Residuals	84

Scotts Gap Residuals	88

Instrumental Based	91

Valley River	91

Oconaluftee River	94

Nantahala River	96

Little Tennessee River	98

Watauga River	100

Appendix 3:  Minitab Output	102

Valley River	102

Oconaluftee River	104

Nantahala River	106

Little Tennessee River	108

Watauga River	110

Vita	112

LIST OF TABLES

Table 1:  USGS streamflow gage information.	27

Table 2:  Tree-ring chronologies used in the research along with latitude, longitude, period of record, and species.	28

Table 3:  Tree-ring chronologies that are retained in the regression model and the state, period, species, and model(s) the chronology entered.	30

Table 4:  Valley River basic statistics:  mean, median, standard deviation, minimum MJJ streamflow, and maximum MJJ streamflow.	31

Table 5:  Reconstruction statistics for each river including the R-squared, R-squared adjusted, R-squared predicted, Durbin-Watson, VIF,  RE, and RMSE.	32

Table 6:  Oconaluftee River basic statistics:  mean, median, standard deviation, minimum MJJ streamflow, and maximum MJJ streamflow.	33

Table 7:  Nantahala River basic statistics:  mean, median, standard deviation, minimum MJJ streamflow, and maximum MJJ streamflow.	33

Table 8:  Little Tennessee River basic statistics:  mean, median, standard deviation, minimum MJJ streamflow, and maximum MJJ streamflow.	33

Table 9:  Watauga River basic statistics:  mean, median, standard deviation, minimum MJJ streamflow, and maximum MJJ streamflow.	33

Table 10:  Valley River 5, 10, and 25-year extreme wet and dry events for both the instrumental and reconstructed period.  The values in the table are the percent departures from the mean.	34

Table 11:  Oconaluftee River 5, 10, and 25-year extreme wet and dry events for both the instrumental and reconstructed period.  The values in the table are the percent departures from the mean.	34

Table 12:  Nantahala River 5, 10, and 25-year extreme wet and dry events for both the instrumental and reconstructed period.  The values in the table are the percent departures from the mean.	34

LIST OF FIGURES

Figure 1:  Map of the USGS streamflow gages and tree-ring chronologies used.	35

Figure 2:  Seasonal correlation plot.	36

Figure 3:  Three month seasonal correlation plot.	37

Figure 4:  Four month seasonal correlation plot.	37

Figure 5:  Six month and annual seasonal correlation plot.	38

Figure 6:  Climate/tree growth relationship for the Valley River MJJ streamflow/Piney Creek Tree-Ring Chronology using a 10-year filter and the 20-year filter over the periods (1928–1980) and (1938–1980) respectively.	39

Figure 7:  Map of the Valley River streamflow gage and retained tree-ring chronologies.	40

Figure 8:  Valley River calibration plot of MJJ average streamflow from 1919 to 1980.  The black line represents the instrumental streamflow, and the gray line represents the reconstructed streamflow.	41

Figure 9:  Valley River plot of combined reconstructed (1661–1918) and instrumental (1919–2008) MJJ streamflow using a 10-year end-year filter.	41

Figure 10:  Valley River plot of combined reconstructed (1671–1918) and instrumental (1919–2008) MJJ streamflow using a 20-year end-year filter.	41

Figure 11:  Map of Oconaluftee River streamflow gage and retained tree-ring chronologies.	42

Figure 12:  Oconaluftee River calibration plot of MJJ average streamflow from 1949 to 1980.  The black line represents the instrumental streamflow, and the gray line represents the reconstructed streamflow.	43

Figure 13:  Oconaluftee River plot of combined reconstructed (1688–1948) and instrumental (1949–2008) MJJ streamflow using a 10-year end-year filter.	43

Figure 14:  Oconaluftee River plot of combined reconstructed (1698–1948) and instrumental (1949–2008) MJJ streamflow using a 20-year end-year filter.	43

Figure 15:  Map of Nantahala River streamflow gage and retained tree-ring chronologies.	44

Figure 16:  Nantahala River calibration plot of MJJ average streamflow from 1941 to 1980.  The black line represents the instrumental streamflow, and the gray line represents the reconstructed streamflow.	45

Figure 17:  Nantahala River plot of combined reconstructed (1688–1940) and instrumental (1941–2008) MJJ streamflow using a 10-year end-year filter.	45

Figure 18:  Nantahala River plot of combined reconstructed (1698–1940) and instrumental (1941–2008) MJJ streamflow using a 20-year end-year filter.	45

Figure 19:  Map of Little Tennessee River streamflow gage and retained tree-ring chronologies.	46

Figure 20:  Map of Watauga River streamflow gage and retained tree-ring chronologies.	46

Figure 21:  Little Tennessee River calibration plot of MJJ average streamflow from 1945 to 1980.  The black line represents the instrumental streamflow, and the gray line represents the reconstructed streamflow.	47

Figure 22:  Little Tennessee River plot of combined reconstructed (1688–1944) and instrumental (1945–2008) MJJ streamflow using a 10-year end-year filter.	47

Figure 23:  Little Tennessee River plot of combined reconstructed (1698–1944) and instrumental (1945–2008) MJJ streamflow using a 20-year end-year filter.	47

Figure 24:  Watauga River calibration plot of MJJ average streamflow from 1941 to 1980.  The black line represents the instrumental streamflow, and the gray line represents the reconstructed streamflow.	48

Figure 25:  Watauga River plot of combined reconstructed (1695–1940) and instrumental (1941–2008) MJJ streamflow using a 10-year end-year filter.	48

Figure 26:  Watauga River plot of combined reconstructed (1705–1940) and instrumental (1941–2008) MJJ streamflow using a 20-year end-year filter.	48

Figure 27:  Valley River 5-year averages distribution of reconstructed values within the lowest 10th percentile.  The y-axis represents the rank for the period of low flow, and the x-axis represents the end year for the 5-year average period of the reconstructed (1656–1918) and instrumental (1919–2008) record.	49

Figure 28:  Oconaluftee River 5-year averages distribution of reconstructed values within the lowest 10th percentile.  The y-axis represents the rank for the period of low flow, and the x-axis represents the end year for the 5-year average period of the reconstructed (1683–1948) and instrumental (1949–2008) record.	49

Figure 29:  Nantahala River 5-year averages distribution of reconstructed values within the lowest 10th percentile.  The y-axis represents the rank for the period of low flow, and the x-axis represents the end year for the 5-year average period of the reconstructed (1683–1940) and instrumental (1941–2008) record.	49

Figure 30:  Valley River percent normal plot of combined reconstructed (1671–1918) and instrumental (1919–2008) MJJ streamflow using a 20-year end-year filter.  Gray areas represent wet periods and black areas represent dry periods.	50

Figure 31:  Oconaluftee River percent normal plot of combined reconstructed (1698–1948) and instrumental (1949–2008) MJJ streamflow using a 20-year end-year filter.  Gray areas represent wet periods and black areas represent dry periods.	50

Figure 32:  Nantahala River percent normal plot of combined reconstructed (1698–1940) and instrumental (1941–2008) MJJ streamflow using a 20-year end-year filter. Gray areas represent wet periods and black areas represent dry periods.	50

# Introduction

## Research Background

Instrumental data and information about the nation’s water resources became available to water managers and planners across the country with the creation of the United States Geological Survey’s (USGS) streamflow-gaging program in 1887 (Mason Jr., R.R. and T.H. Yorke, 1997).  Proxy indicators such as tree rings are the only reliable source of information on climate occurring before the late 19th century (Meko et al., 1995).  Throughout the western U. S., proxies to the instrumental data were constructed using tree-ring chronologies because they were strong annual indicators of climate. Precipitation and evapotranspiration control the climate of the western U.S., and also control the growth of moisture sensitive trees used in tree-ring reconstructions (Meko et al., 1995).  The landscapes of the western U.S. are also an ideal for sampling moisture sensitive tree species because the preferred sites are typically those that are dry, high in elevation, and extreme in slope (Fritts, 1976).

In the western U.S., tree rings were the most reliable and preferred method of obtaining multi-century reconstructions of streamflow variability (Meko et al., 2007; Watson et al., 2009; Woodhouse et al., 2006).   Tree-ring reconstructions have been used to reconstruct drought (Meko et al., 1995; Cook et al., 2007; Girardin et al., 2006), precipitation (Dettinger et al., 1998; Blasing et al., 1981; Stahle and Cleaveland, 1994; Stahle and Cleaveland, 1992; Grissino-Mayer, 1995), El Niño-Southern Oscillation climate index (Braganza et al., 2009), Atlantic Multidecadal Oscillation climate index (Gray et al., 2004), North Atlantic Oscillation climate index (Cook et al., 2002), and temperature (Wiles et al., 1998).

While the practice of reconstructing streamflow is prevalent in the western U.S. and abroad, few tree-ring reconstruction studies have been performed in the southeastern United States.  Many misconceptions linger among scientists that tree-ring research simply is not possible in the southeastern U.S. because of high decomposition and decay rates and a lack of trees that are long lived or have sensitive patterns of tree rings to facilitate crossdating (Grissino-Mayer, 2009).  These misconceptions are likely the reason that more streamflow reconstruction studies have not been performed in the southeastern United States.

One study demonstrated the possibility of creating successful streamflow reconstructions in the south central U.S. involved the White River in Arkansas (Cleaveland, 2000).  He reconstructed the June-July-August (JJA) average streamflow using tree-ring chronologies as a proxy to the instrumental record.  Baldcypress (Taxodium distichum), a lowland forest species, was used in the reconstruction.  The reconstruction of April through August monthly streamflow for the Occoquan River basin in Virginia was a study performed in the eastern U.S. (Phipps, 1983).  Tree species used in the reconstructions of the Occoquan River basin were hemlock (Tsuga canadensis), post oak (Quercus stellata), hickory (Carya glabra), beech (Fagus gradnifolia), chestnut oak (Quercus montana), and white oak (Quercus alba).  Many of the tree species used by Phipps (1983) and Cleaveland (2000) are also used in this research.

The reconstruction of the White River in Arkansas was able to obtain results with the use of the three-month summer seasonal average June-July-August (JJA) (Cleaveland, 2000).  Precipitation reconstructions have been performed for the southeastern U.S. in which the season to reconstruct precipitation was different for each statewide precipitation reconstruction (Stahle and Cleaveland, 1994; Stahle and Cleaveland, 1992).  Statewide spring rainfall reconstructions were performed for North Carolina, South Carolina, and Georgia.  The reconstructed precipitation seasons were:  North Carolina (April-June), South Carolina (March-June), and Georgia (March-June) (Stahle and Cleaveland, 1994; Stahle and Cleaveland, 1992).

The primary goal of this research was to reconstruct streamflow records from five gages in the Appalachian Mountains of North Carolina using tree-ring chronologies.  Many of the methods used in previous western U.S. streamflow reconstructions were applied in this research to demonstrate the value and need for additional climate reconstructions in the southeastern United States.  Another goal of this research was to examine the reconstructions that were able to explain approximately 50% of the variance to determine if historical drought periods exist for these reconstructions.  Wet periods were examined to determine if these periods were more extreme or more prolonged than the instrumental streamflow records indicate.

## Site Description

The headwaters of many rivers within the Appalachian Mountains of North Carolina are a vital water downstream supply and are used to generate power by the Tennessee Valley Authority (TVA), one of largest utility corporations in the United States.  Although the reconstruction gages are located in mountainous regions, the topography of the region contains many large valleys.  The western border of North Carolina has a unique geology in that this mountainous region contains metamorphic, igneous, and sedimentary rocks that have little or no primary porosity due to intense heat and pressure related to mountain building (Bales et al., 2009).  The Soils of this region are shallow due to the underlying geology (Bales et al., 2009).  The climate in the Appalachian Mountains is very diverse as pockets of this region are temperate rain forests due to the large amount of rainfall.  Parts of the region mirror the climate of the northeastern U.S. with very cold winters and mild summers while others mirror the climate in the southeastern U.S. with mild winters and hot, humid summers.  The effects of these climate variations are evident in the tree-ring widths.  Western U.S. tree-ring widths do not experience the same extreme variations in climate that the eastern U.S. tree rings do, and the effect of these variations in climate in the eastern U.S. on streamflow reconstructions is unknown.  Land cover change in the Blue Ridge Mountains of the southern Appalachian Mountains is unlike much of the eastern United States.  The percentage of land area where land cover changed at least once between 1973 and 2000 was the lowest with only a 2.0% change (Taylor and Kurtz, 2010).  This small change in land cover is important in this study because it can be hypothesized that the less land cover change in the study region, the more likely the reconstructed data will represent consistently accurate historical streamflow over the entire period.

Tree species used in this study are located in the Tennessee Valley, the Appalachian Mountains, and within a 640 kilometer radius of each of the gages studied (Figure 1 in Appendix 1).  Tree-ring chronologies used in this research included, but were not limited to, the following species:  tulip-poplar (Liriodendron tulipifera), red spruce (Picea rubens), white oak (Quercus alba), baldcypress (Taxodium distichum), eastern hemlock (Tsuga canadensis), and Carolina hemlock (Tsuga caroliniana).

# Data

## Streamflow Data

One of the most important components in a streamflow reconstruction is the accuracy and length of existing streamflow gage records.  Although the USGS streamflow-gaging program began collecting streamflow data as early as 1887, not all of the USGS gage stations have the same period of record.  Many USGS gage stations have gaps in the records due to a number of different technical, mechanical, or otherwise unknown reasons.  The USGS gage stations that were selected for reconstruction in the current research contain no gaps in the data and have an acceptable overlapping period to calibrate with regional tree-ring chronologies (http://waterdata.usgs.gov/nwis/sw, 2009).  The five USGS streamflow gages (Figure 1 and Table 1 in Appendix 1) selected for reconstruction were:  Watauga River near Sugar Grove, North Carolina; Little Tennessee River near Prentiss, North Carolina; Nantahala River near Rainbow Springs, North Carolina; Oconaluftee River at Birdtown, North Carolina; and Valley River at Tomotla, North Carolina.  The elevations of these gages vary from 474 m to 937 m.  The drainage areas of these rivers vary from 134 km2 to 477 km2.  Although these rivers are located in close proximity, each are unique in location and elevation.

## Tree-Ring Chronologies

The tree-ring chronology data used in this study came from the International Tree-Ring Data Bank (http://www.ncdc.noaa.gov/paleo/treering.html, 2009) which is maintained by the National Oceanic and Atmospheric Administration (NOAA) Paleoclimatology Program.  Available data include raw ring widths, wood types, site chronologies, and wood density measurements.  Each of the tree-ring chronologies used were uniformly processed using the ARSTAN program (Cook, 1985).  Methods for detrending the multiple series into a single chronology were either the negative exponential/straight line fit or a cubic spline two-thirds the length of each individual series (Cook et al., 1990).  Residual chronologies were used because the low-order autocorrelation was removed from these chronologies.  Low-order autocorrelations are often due to a number of different changes in growth factors including age, alteration in forest community, geological changes, or variations in climate (Fritts, 1976).

The 64 tree-ring chronologies selected were from ten states:  Tennessee, Alabama, Georgia, Mississippi, Florida, Virginia, North Carolina, Kentucky, Ohio, and West Virginia (Table 2).  To ensure a reconstruction of significant length, only residual chronologies extending beyond 1790 were investigated in the current research.

# Methods

## Pre-Screening Methods

Correlation is a process by which the degree of association between data sets of two variables is defined (Riggs, 1977).  The two samples for which a degree of association was required were the tree-ring chronologies and averaged monthly values of streamflow.  The degree of association between the two samples can be described by the correlation coefficient (r-value), and the significance (p-value) depends on the sample size (number of years the samples collectively span).  An increase in sample size decreases the r-value required for significance.  The opposite was true for a decrease in sample size.

Correlation analysis can yield positive or negative significance depending on the sign of the r-value.  In this study, positive r-values of 95% significance or greater are used. A positive r-value indicates an increase in both, the seasonal streamflow and the annual tree-ring width.  A negative r-value indicates an increase (decrease) in streamflow and a decrease (increase) in tree-ring width, and was not desired because a decrease (increase) in tree-ring width should not indicate an increase (decrease) in streamflow for the MJJ season.

The correlation analysis performed in this research helped determine which trees to use as predictors during the reconstruction process and helped determine the streamflow season to use as the most significant predictand.   A similar correlation process was used in a study by Blasing et al. (1981) in the Knoxville, Tennessee area of the Tennessee Valley.   This study used a correlation analysis to determine which seasonal precipitation period to reconstruct and which tree-ring chronologies to use as reconstruction predictors.  Blasing et al. (1981) determined that that May-June precipitation was most highly correlated to the tree-ring growth in the region.  The absence of streamflow reconstructions in the Tennessee Valley and the use of different tree-ring chronologies may cause the season for which streamflow and tree-ring widths were most highly correlated in this research to differ from the work done by Blasing et al. (1981).  This study investigated many different seasonally averaged streamflow periods including: three-month, four-month, six-month, and an annual average, in order to be exhaustive.  For example, the three-month periods began at January-February-March (JFM) and shift by one month for each analysis i.e. February-March-April (FMA).  A similar approach was done for four-month periods and six-month periods beginning at January-February-March-April (JFMA) and January-February-March-April-May-June (JFMAMJ), respectively.  This analysis assured that the most highly correlated season was evaluated.

Temporal stability between the tree-ring chronologies and streamflow was analyzed using DendroClim2002 from Biondi and Waikul (2004).  This analysis resulted in significant correlation of May, June, July streamflow with May and June showing the strongest moisture signal with the tree-ring chronologies.  DendroClim2002 was used to identify climate signals and confirm that no significant changes over time were imbedded in the tree-ring records.  These changes could be attributed to a number of different things including:  wildfires, insects, and lightning.

## Reconstruction Methods

After the prescreening process, positively significant tree-ring chronologies and seasonal streamflow data sets were used to reconstruct each of the five USGS streamflow gages.  Stepwise linear regression is a proven method of statistical streamflow reconstruction in the western U.S. (Woodhouse et al., 2006; Woodhouse, 2001) and was used for this research.  In the simplest case, a linear regression equation is used to reconstruct past values of a single climatic variable from ring-width indices of a single tree-ring chronology, or from a mean of two or more chronologies which have been merged to form a single chronology (Blasing et al., 1981).

Forward and backward stepwise regression was used to determine which tree-ring chronologies entered into the regression model.  The alpha level for the entry of a predictor was set to allow a maximum p-value of 0.05 and 0.10 for removal in the regression model.

The five reconstructed streamflow stations were validated over the gage-specific calibration periods.  For each gage, the strength of the regression model was measured by the adjusted R2 value, which quantifies the explanatory power of the regression and accounts for lost degrees of freedom (Axelson et al., 2009).  Along with the adjusted R2 value, the squared correlation coefficient (R2) and the reduction of error (RE) statistic evaluated the strength of the reconstruction models.  The overall variance explained within the calibration period was given by the R2 value.  The RE statistic tests the skill of the model over the verification period, compared to estimates based on the mean of the observed calibration data (Woodhouse, 2001).  The RE statistic ranges from minus infinity to +1.  A positive value for the RE demonstrates predictive skill in the model (Fritts, 1976).  The root mean square error (RMSE) further validated model strength.  A single validation series was produced using a leave-one-out cross validation scheme (Woodhouse et al., 2006).  The fit of the regression models was evaluated by the predicted R2 value, and the predicted R2 value was calculated from the prediction sum of squares test (PRESS) statistic.  The Durbin-Watson statistic was used to confirm that no autocorrelation existed within the residuals (Watson et al., 2009; Axelson et al., 2009).  The Durbin-Watson statistic ranges from 0 to 4.  Generally, Durbin-Watson values near 2 indicate no autocorrelation between predictor variables.  A value near 4 indicates negative autocorrelation, and a value near 0 indicates positive autocorrelation which was of the most concern in this research because the residual tree-ring widths were used in which autocorrelation was thought to be removed.  The variance inflation factor (VIF) examines multicollinearity of the predictors (Watson et al., 2009; Axelson et al., 2009).   A VIF near 1 was desired because, as the VIF increases towards infinity, multicollinearity arises between predictors.  This is a problem because it indicates that one or more of the tree-ring chronologies may be highly correlated with one or more of the other tree-ring chronologies, causing an exaggerated R2 value.

## Streamflow Reconstruction Analysis Methods

Due to the proximity of the streamflow gages, the extreme periods of wet or dry years were hypothesized to be consistent for all the reconstructions.  Water managers and planners are concerned with the severity and longevity of these periods.  In a study done by Woodhouse (2001), the distribution of extreme low flows over the streamflow reconstruction were ranked for periods of lowest flow.  The single year, three-year, and five-year average values that fell within the lowest 10th percentile were then plotted by rank over time (Woodhouse, 2001).  The five-year seasonal streamflow was ranked for all of the gages reconstructed according to the lowest 10th percentile flows.

The instrumental and reconstructed extreme wet and dry periods were examined for each of the gages using a 5-year, 10-year, and 25-year moving-average approach.  Streamflow percent departure from the mean of the instrumental and reconstruction periods was used to examine 5-year, 10-year, and 25-year extreme wet and dry periods.  These moving averages allow for comparison of the reconstruction streamflow and the instrumental record.  These examinations helped determine if the majority of prolonged wet or dry periods occurred in the reconstruction period or the instrumental period.  The percent departures were compared to determine the severity of the wet and dry periods.  Similarities in the percent departures for the reconstructed and instrumental records may indicate that the streamflow variance is likely comparable for the reconstructed and instrumental periods.

# Results

## Seasonal Correlations

One of the most important aspects of the seasonal correlation analysis was to determine a consistent streamflow season to reconstruct for all five of the USGS streamflow gages.  A seasonal correlation analysis was performed to examine the number of significant tree-ring chronologies over three-month, four-month, and six-month/annual periods for each season and river to determine the most consistent season (Figures 2–5).  The three-month period May-June-July (MJJ) was the season containing the largest number of significant tree-ring chronologies for the majority of the five gages subsequent to the seasonal correlation analysis (Figure 2).  In a few cases, the number of significant tree-ring chronologies increased for the three-month period (JJA) for the Watauga River and the four-month period (May-June-July-August) for the Valley River.  The number of significant trees was higher for the three-month period May-June-July (MJJ) than the three-month period April-May-June (AMJ).  At no time during the seasonal correlation analysis did the winter months yield a large number of highly correlated tree-ring chronologies.  Therefore, MJJ was used in this research because this seasonal period was the most consistent for all the gages.  This seasonal consistency was critical for comparison of the individual reconstructions.  A seasonal correlation plot showed the number of significant tree-ring chronologies retained against seasonal streamflow for each river (Figure 2).  Regional tree-growth contained the highest signal with May-June-July streamflow in the Valley River (29 significant tree-ring chronologies).

The stability of the climate/tree-growth relationship was analyzed over time for each of the tree-ring chronologies retained for each river.  This relationship is represented using correlation between tree-ring chronologies and MJJ streamflow over the calibration period (Figure 6).  The retained tree-ring chronologies demonstrate a positive relationship with MJJ streamflow over the entire calibration period for all of the stations indicating stability.  This relationship was also found to be significant for the majority of the calibration periods for all of the streamflow gage stations.

## Calibration and Verification of Reconstructions

Three of the five MJJ streamflow reconstructions were determined to be acceptable based on an R2 value greater than or equal to 0.48.  The longest reconstruction model (Valley River at Tomotla, North Carolina) dates back to 1652, and retained four tree-ring chronologies: Piney Creek Pocket Wilderness (white oak), Ramsey’s Draft Recollection (eastern hemlock), Black River, and Grandfather Mountain (red spruce).  The Piney Creek Pocket Wilderness chronology was also retained in the reconstruction models of the Oconaluftee River at Birdtown, North Carolina and the Nantahala River near Rainbow Springs, North Carolina.

The Oconaluftee River calibration model retained two tree-ring chronologies: Kelsey Tract II and Piney Creek Pocket Wilderness.  Along with the Piney Creek Pocket Wilderness chronology, the Pearl River (baldcypress) and the Kelsey Tract II (Carolina hemlock) chronologies were also retained in the Nantahala calibration model.  The retained tree-ring chronologies are located in four different states: North Carolina, Mississippi, Tennessee, and Virginia (Table 3).

The Valley River was the easternmost reconstructed streamflow stations (Figure 7).  The Valley River calibration period was from 1919–1980, and the model explains 51% of the total variance (Figure 8).  The Valley River reconstruction dates back to 1652.  The resulting reconstruction for the Valley River was smoothed using a 10-year end-year filter (Figure 9) in which the reconstructed data (1661–1918) were combined with the instrumental record (1919–2008).  A 20-year filter for the streamflow reconstruction was also performed for the Valley River (Figure 10) where the reconstructed data (1671–1918) were combined with the instrumental record (1919–2008).  The average seasonal streamflow volume for the entire period of reconstructed and instrumental streamflow for the Valley River was also plotted on both reconstructions.  Statistical parameters for the instrumental and reconstructed streamflow data including the mean, median, standard deviation, minimum streamflow, and maximum streamflow are provided (Table 4).  These statistics were calculated to compare the reconstructed and instrumental streamflow and determine if any major variations existed between the two datasets.  The mean for the reconstructed period and the instrumental period was 44.8 MCM and 45.6 MCM respectively.  One major difference in the reconstructed and instrumental statistics was the minimum MJJ flow.  The minimum MJJ streamflow for the reconstructed period and the instrumental period was 1.3 MCM and is 13.5 MCM, respectively.  The reconstruction of the Valley River was verified by the R2 statistic, RE statistic, and the RMSE statistic (Table 5).  The RE statistic for the Valley River was +0.51, indicating the model has predictive skill.  The RMSE statistic for the Valley River was 11.5 MCM, which was approximately 26% of the average seasonal flow for the reconstructed period.  The Durbin-Watson statistic for the Valley River was 1.7, indicating autocorrelation in the residuals was not a concern.  The VIF was 1.1, which suggests that the predictors contain minimal multicollinearity (Table 5).

Located northeast of the Valley River, the Oconaluftee River reconstruction explains 48% of the total variance in the streamflow.  The reconstruction model retained two tree-ring chronologies (Figure 11).  The calibration period for the Oconaluftee River covers 1949 to 1980 (Figure 12), while the reconstruction spans 1679 to 1980.  Reconstructed streamflow was smoothed using a 10-year end-year filter (Figure 13) in which the combined reconstruction (1688–1948) and instrumental period (1949–2008) was shorter than the previous Valley Reconstruction.  A 20-year end-year filter average streamflow reconstruction was performed for the Oconaluftee River (Figure 14) where the reconstructed data (1698–1948) was combined with the instrumental record (1948–2008).  The mean, median, standard deviation, minimum streamflow, and maximum streamflow were calculated for the reconstructed and instrumental period for the Oconaluftee River (Table 6).  The Oconaluftee River had a mean streamflow of 97.7 MCM and 99.9 MCM for the reconstructed and the instrumental period, respectively.  The minimum flow was lower for the reconstructed period, and the maximum flow was higher for the instrumental period.  The value of the Oconaluftee River reconstruction was demonstrated by the RE statisticand the RMSE which were respectively 0.48 and 15.9 MCM (Table 5).  A RMSE of 15.9 MCM was approximately 16% of the mean flow of the reconstructed period.  For the Oconaluftee River reconstruction, the Durbin-Watson statistic was 2.1, suggesting that autocorrelation between the model residuals was minimal, and insignificant multicollinearity exists between predictors (Table 5).

The final streamflow reconstruction was the Nantahala River, located east of the Valley River and south of the Oconaluftee River (Figure 15).  The Nantahala River streamflow reconstruction retained three tree-ring chronologies.  The variance explained by the Nantahala calibration model was 48%.  The period from 1941 to 1980 was used for the model calibration (Figure 16).  The Nantahala streamflow was reconstructed from 1679 to 1980.  A 10-year end-year filter (Figure 17) was used to smooth the Nantahala average streamflow reconstruction with a combined reconstruction period (1688–1940) and instrumental period (1941–2008).  Further smoothing was performed using a 20-year end-year filter (Figure 18) for the Nantahala average streamflow with a combined reconstruction period (1698–1940) and instrumental period (1941–2008).  The mean, median, and standard deviation statistics were calculated for the reconstructed period and the instrumental period for the Nantahala River along with the minimum and maximum streamflow values (Table 7).  The mean for the reconstructed and the instrumental periods were 40.0 MCM and 39.5 MCM, respectively.  The differences in the maximum and minimum flow values were not as extreme as those for the Valley and Oconaluftee River.  The maximum flow was greater for the instrumental period (83.5 MCM) than the reconstructed period (63.1 MCM), and the minimum flow was lower for the reconstructed period (12.2 MCM) than for the instrumental period (16.7 MCM).  Reconstruction statistics R2, RE, and RMSE were 0.48, 0.48, and 8.2 MCM respectively for the Nantahala River (Table 5).  A RMSE of 8.2 MCM was approximately 20% of the mean for the reconstructed flow.  The Durbin-Watson and VIF statistic were also calculated for the Nantahala and were 2.3 and 1.1 respectively (Table 5).

The Little Tennessee River (Figure 19) and the Watauga River (Figure 20) were also reconstructed.  The calibration period for the Little Tennessee River was from 1945 to 1980 (Figure 21).  A 10-year filter was used to smooth the streamflow reconstruction of the Little Tennessee River for the combined reconstructed period (1688–1944) and the instrumental period (1945–2008) (Figure 22).  Further smoothing was applied using a 20-year filter of the streamflow reconstruction of the Little Tennessee River for the combined reconstructed period (1698–1944) and the instrumental period (1945–2008) (Figure 23).  The calibration period for the Watauga River was from 1941 to 2008 (Figure 24).  A 10-year filter was used to smooth the streamflow reconstruction of the Watauga River for the combined reconstructed period (1695–1940) and the instrumental period (1941–2008) (Figure 25).  A 20-year filter was performed for the Watauga River reconstructed period (1705–1940) and instrumental period (1941–2008) (Figure 26).  However, these reconstructions did not perform as well as the previously discussed reconstructions as demonstrated by the R2 value (Table 5).  The Little Tennessee River was able to explain 42% of the total variance in streamflow, and the Watauga River explained 27% of the total variance in streamflow.  For this reason, these rivers were considered not reliable even though some of the statistics appear consistent.  For example, the Little Tennessee River had a positive RE of +0.42 indicating some predictive skill, but the RMSE was 19.1 MCM, the largest of all the reconstructed gages, which was approximately 24% of the reconstructed mean (Table 5).  No large differences in the mean or median exist between the reconstructed or instrumental periods for the Little Tennessee River and the Watauga River (Tables 8 and 9).  Because the streamflow reconstructions for the Little Tennessee River and the Watauga River were considered not accurate, a drought analysis was not performed for these rivers.

Leave-one-out cross validation resulted in predicted R2 values ranging from 21% to 41% (Table 5).  These predicted R2 values suggested that minimal overfitting existed in these models.  The most extreme case was the Watauga River that had a predicted R2 value of 21%.  The predicted R2 value differed by 12% or less than the R2 value for the Valley River, Oconaluftee River, and the Nantahala River.

# Discussion

## Importance of Climate Reconstructions Eastern U.S.

This research demonstrated that eastern U.S. streamflow reconstructions using tree-ring chronologies as proxies can be performed using similar techniques to those in the western United States.  Three of the five streamflow reconstructions in the Appalachian Mountains were comparable in the amount of variance explained to previous studies in the south central and the eastern United States.  Although the study performed by Cleaveland (2000) on the White River in Arkansas was able to explain 68% of the variance of the reconstruction model, the presented research was able to explain approximately 50% of the variance for three of the five rivers.  A possible explanation for this difference is likely due to the differences in location.  The geology of the White River watershed is very different from the southern Appalachian Mountains.  The White River watershed demonstrates consistent areas of karst topography in which the fractured limestone bedrock allows surface water to easily escape to the groundwater (http://extra.mdc.mo.gov/fish/watershed/whriver/geology/, 2010).  The White River also contains sedimentary rocks other than limestone including sandstone and dolomite which differ greatly from the metamorphic and igneous bedrock of the Appalachian Mountains (http://extra.mdc.mo.gov/fish/watershed/whriver/geology/, 2010 and Bales et al., 2009).  The metamorphic and igneous rock have little to no porosity and allow very little surface water to enter the groundwater (Bales et al., 2009).  This drastic contrast in geology between the White River watershed and the Appalachian Mountains was likely mirrored by drastic differences in streamflow reactions after precipitation and drought events.  Another difference between the present research and the work done by Cleaveland (2000) was the season used to create the streamflow reconstructions.  Because a different season was chosen over which to reconstruct streamflow, it is likely that different tree-ring chronologies correlate with different seasons due to variations in environmental and climatic factors occurring in the southeastern and south central United States.  A number of different moisture sensitive tree species were used in this research. It is possible that factors other than precipitation and evapotranspiration which control the majority of growth in trees in the western U.S. (Meko et al., 1995) were recorded by tree-ring chronologies in the eastern U.S because of location and climate.

The results of the present research yielded comparable R2 values to that of the research done by Phipps (1983).  The reconstructions performed by Phipps (1983) were able to explain 39% to 53% of the variability of monthly streamflow for rivers in the Occoquan River basin.  These results were thought to be similar to the present research due to the similarities in location and tree species used to perform the reconstructions.  The Occoquan River basin is located in northern Virginia.  The tree species utilized in the Phipps (1983) study were hemlock, post oak, hickory, beech, chestnut oak, and white oak.  Many of the tree species used by Phipps (1983) were also retained in the streamflow reconstruction models.

In a study conducted by Blasing et al. (1981), tree-ring chronologies were used as a proxy to reconstruct precipitation in Knoxville, Tennessee using similar regression techniques for reconstructing May-June precipitation.  Another precipitation reconstruction was performed for the southeastern region of the U.S. in which March through June average statewide precipitation was reconstructed for North Carolina, South Carolina, and Georgia (Stahle and Cleaveland, 1994).  The success of the present research in conjunction with Cleaveland (2000) and Phipps (1983) demonstrated the possibility of many more successful reconstruction studies in the eastern U.S. These studies should serve as a base for reconstructing a number of different climate variables in the eastern U.S., including streamflow, precipitation, drought, and temperature.

## Limitations of Eastern U.S. Reconstructions

One of the main restrictions of this study and future climate reconstruction work in this region was the limited calibration period.  When attempting to calibrate two data sets, a maximum number of years for the calibration period is optimal to capture the long-term variability in regional climate records.  The calibration period can be limited by two factors.  The first factor is the availability of the instrumental gage data.  Streamflow gage data are a limiting factor because they begin in the early to mid-1900s, and the data were collected through 2008.  Only streamflow gages that have complete data sets can be calibrated with tree-ring chronologies.  The second factor limiting the calibration period was the ending year of usable tree-ring chronologies.  For example, the majority of the tree-ring chronologies in this research end in 1980.  Both factors affected the length of the calibration periods in the present research.  The majority of the tree-ring chronologies extend only to 1980, and the majority of streamflow gage data begins in early to mid 1900’s.  The only solution to eliminate this issue for future research in this region is to extend the tree-ring chronologies because most streamflow gages have data available through the present year.  Unfortunately, updating the tree-ring chronology datasets has not been performed in this region on a large number of tree-ring chronology datasets.

## Utilizing Larger Tree-Ring Chronology Search Radius

Initially, tree-ring chronologies within a 240 kilometer radius of each USGS streamflow gages were collected for testing.  No baldcypress tree species were collected for use in the model with the 240 kilometer radius.  Next, the radius was expanded to 640 kilometers primarily to include baldcypress species.  This process was used in order to use the baldcypress species found to contain a high moisture signal in the Cleaveland (2000) study.  After expanding the radius, three out of the five reconstructions retained a baldcypress chronology.  The reconstructions that retained the baldcypress were able to explain more of the total variance in the models than the reconstructions that were performed without a baldcypress chronology.  An expanded search radius should be used in any study in which tree-ring chronologies are being used as a proxy because certain moisture sensitive tree species are often only found in very specific regions that may not be within the original scope of study.

## Streamflow Reconstruction Analysis

The reconstruction plots demonstrated several oscillating periods of wet and dry streamflow.  Common dry periods of streamflow were 1708 to 1734, 1795 to 1803, and 1843 to 1851 for the reconstructions of the Valley River, the Oconaluftee River, and the Nantahala River.  Consistent wet periods shown in the three reconstructions were 1738 to 1759, 1809 to 1833, and 1869 to 1873.  Water managers and planners can use streamflow reconstructions in a number of different ways.  One way is by performing drought analysis on the reconstruction.  The rank of the 5-year average streamflow values that fall within the lowest 10th percentile were plotted for each of the three reconstructed rivers (Figures 27–29).  The three streamflow reconstructions the Valley River, Nantahala River, and Oconaluftee River were analyzed to determine low flow periods over the reconstructed and instrumental periods.   A period of 5-year average low flow events for the three reconstructed rivers was the period from 1710 to 1712.  For two of the reconstructions (Valley River and Oconaluftee River) the 5-year average low flow events were extended out to 1718.  This period was the most prolonged and extreme drought period over the reconstructed and instrumental record for the Valley River and Oconaluftee River.  Another low flow period consistent for all three rivers occurred from 1954 to 1955.  The Valley River and the Nantahala River consistently demonstrated low flow years during the early 1700s as discussed, and the late 19th (1895, 1897, and 1898) into the 20th century (1914, 1915) also have short periods of extreme dry.  The Valley River and the Nantahala River demonstrated multiple periods of low flows around the middle of the 1900s, but the results were spaced over a period of time instead of a consistent period of prolonged low flow.  The Oconaluftee River was unique over one prolonged period from 1788 to 1792 during which the 5-year low flow analysis revealed an extended drought period for this river only.

The 5-year, 10-year, and 25-year extreme dry and wet events for each of the three rivers were determined according to the percent departure from the mean of the instrumental and reconstructed periods separately (Tables 10–12).  For each of the three rivers, the wettest 5-year average period was at least 25% wetter than the average for the reconstruction periods compared to what was shown in instrumental records.  The Valley River was the only river in which the extreme dry periods for the 5-year, 10-year, and 25-year percent departure from the mean was greater than the wet percent departure over the reconstructed period only.  This demonstrated the fact that the driest periods were more extreme than the wettest periods for the Valley River over the reconstructed period.  Without the reconstruction of the Valley River streamflow, this distinction between historical wet and dry periods would not be known.  With the exception of the 5-year average for the Nantahala River, the low percent departure from the mean over the reconstructed period was higher than that of the instrumental period for the each of the three rivers for the 5-year, 10-year, and 25-year averages.  This conclusion was mirrored by the fact that the minimum flow value was lower for the reconstruction period than the instrumental period for all three rivers (Tables 4, 6, and 7).

Many other techniques have been published for use in drought analysis, and one such technique was used by Zhang et al. (2004) in which tree-ring data were calibrated against the instrumental record of summer drought (June-August Palmer Drought Severity Index) based on application of the ‘Regularized Expectation Maximization’ algorithm to relate large-scale patterns of variation in proxy and instrumental data over a common (20th century) interval.  This type of regionalized study of drought in the Tennessee Valley would help confirm the results of the present research.  A study of “worst case” drought scenario for the Upper Colorado River Basin has also been performed using tree rings (Meko et al., 1995).  Another way to examine the reconstructions is to graph the percent normal flow over the reconstructed period and the calibration period.  The percent normal reconstruction graphs with 20-year end-year filters were created for each of the three rivers (Figures 30–32).  These figures demonstrated the percent departure of the streamflow value from the average streamflow over the reconstructed and instrumental periods. Many of the previously mentioned periods of extreme wet and dry were visible graphically in these reconstruction plots.

# Conclusions

Although few studies exist that provide a solid method of reconstructing streamflow in the eastern U.S., this research reconstructed streamflow gages for three rivers in this region with predictive skill using techniques similar to western U.S. studies.  The results of this work provided a valuable group of tree-ring based streamflow estimates that can be used along with the instrumental records by water managers and planners in the Tennessee Valley region.

Another important conclusion from this study was that the last century has not experienced a prolonged period of drought like that of the early 18th century for the reconstructions of the Valley River, the Oconaluftee River, and the Nantahala River.  This conclusion presents a serious concern for the Tennessee Valley and especially the Appalachian Mountains of North Carolina in that the recurrence of more extreme low flow periods than that seen over the last century may be a real possibility in the future given past occurrences.  The current research also established a method of climate reconstruction for use in other studies in this region.  Valuable information about the variability of streamflow in the Appalachian Mountains was uncovered in this study, and future studies in the eastern U.S. may help further explain the natural variations in streamflow and the climate of this region.

# Literature Cited

Axelson, J. N., D. J. Sauchyn, and J. Barichivich 2009, New reconstructions of streamflow variability in the South Saskatchewan River Basin from a network of tree ring chronologies, Alberta, Canada, Water Resources Research, 45, W09422, DOI:10.1029/2008WR007639.

Bales, J.D., M.J. Chapman, C.J. Oblinger, and J.C. Robbins. 2009.  North Carolina District Science Plan.  U.S. Geological Survey, U.S. Department of the Interior. <http://nc.water.usgs.gov/reports/ofr041025/report.html>. (Oct. 2010).

Blasing, T.J., D.N. Duvick, and D.C. West. 1981. Dendroclimatic calibration and verification using regionally averaged and single station precipitation data. Tree-Ring Bulletin 41: 37–43.

Braganza, K., J.L. Gergis, S.B. Power, J.S. Risbey, and A.M. Fowler. 2009. A multiproxy index of the El Niño-Southern Oscillation, AD 1525–1982. Journal of Geophysical Research-Atmospheres 114: DO5106.

Cleaveland, M.K. 2000. A 963-year reconstruction of summer (JJA) streamflow in the White River, Arkansas, USA, from tree-rings. Holocene 10(1): 33–41.

Cook, E.R. 1985. A time series approach to tree-ring standardization, Ph.D. dissertation, University of Arizona, Tucson.

Cook, E.R., K. Briffa, S. Shiyatov, and V. Mazepa. 1990. Tree-ring standardization and growth-trend estimation, in Methods of Dendrochronology: Applications in the Environmental Sciences, edited by E. R. Cook and L. A. Kairiukstis. Springer, New York 104–123 pp.

Cook, E.R., R.D. D’Arrigo, and M.E. Mann. 2002. A well-verified, multiproxy reconstruction of the winter North Atlantic Oscillation index since AD 1400. Journal of Climate 15(13): 1754–1764.

Cook, E.R., R. Seager, M.A. Cane, and D.W. Stahle. 2007. North American drought: reconstructions, causes, and consequences. Earth-Sciences Reviews 81(1–2): 93–134.

Dettinger, M.D., D.R. Cayan, H.F. Diaz, and D.M. Meko. 1998. North-south precipitation patterns in western North America on interannual-to-decadal timescales. Journal of Climate 11(12): 3095–3111.

Fritts, H.C. 1976. Tree Rings and Climate. Elsevier, New York.

Girardin, M.P., J.C. Tardiff, M.D. Flannigan, and Y. Bergeron. 2006. Synoptic-scale atmospheric circulation and Boreal Canada summer drought variability of the past three centuries. Journal of Climate 19(10): 1922–1947.

Gray, S.T., L.J. Graumlich, J.L. Betancourt, and G.T. Pederson. 2004. A tree-ring based reconstruction of the Atlantic Multidecadal Oscillation since 1567 AD. Geophysical Research Letters 31(12): L12205.

Grissino-Mayer, H.D. 2009. An introduction to dendroarchaeology in the southeastern United States. Tree-Ring Research 65(1): 5–10.

Grissino-Mayer, H.D. 1995. Tree-ring reconstructions of climate and fire history at El Malpais National Monument, New Mexico. Ph.D. dissertation, University of Arizona, Tucson, AZ. 407 pp.

International Tree-Ring Data Bank (ITRDB). Tree-ring data search. Available online at

http://www.ncdc.noaa.gov/paleo/treering.html. (Nov. 2009)

Mason Jr., R.R. and T.H. Yorke. 1997.  Streamflow Information for the Nation.  U.S. Geological Survey, U.S. Department of the Interior.

Meko, D.M., C.A. Woodhouse, C.A. Baisan, T. Knight, J.J. Lukas, M.K. Hughes, and M.W. Salzer. 2007. Medieval drought in the Upper Colorado River Basin, Geophysical Research Letters 34(10): L10705.

Meko, D.M., C.W. Stockton, W.R. Boggess. 1995.  The tree-ring record of severe sustained drought.  Water Resources Bulletin 31: 789–801.

National Weather Information System (NWIS). (2009).  USGS surface-water data for the nation, <http://waterdata.usgs.gov/nwis/sw> (Nov. 2009).

Phipps, R.L. 1983. Streamflow of the Occoquan River in Virginia as reconstructed from tree-ring series. Water Resources Bulletin 19: 735–743.

Riggs, H.C. 1977. U.S. Geological Survey of Water-Resources Investigations Book 4, Chapter A3. Some Statistical Tools in Hydrology. United States Government Printing Office, Washington. 1–39 pp.

Stahle, D.W. and M.K. Cleaveland. 1994. Tree-ring reconstructed rainfall over the southeastern USA during the medieval warm period and little ice-age. Climatic Change 26(2–3): 199–212.

Stahle, D.W. and M.K. Cleaveland. 1992. Reconstruction and analysis of spring rainfall over the Southeastern U.S. for the past 1000 Years. Bulletin American Meteorological Society. 73(12):  1947–1961.

Taylor, J.L. and R. Kurtz. 2010. Blue Ridge Mountains. U.S. Geological Survey, U.S. Department of the Interior. <http://landcovertrends.usgs.gov/east/eco66Report.html#_ftn1>. (Oct. 2010).

Watson, T.A., F.A. Barnett, S.T. Gray, and G.A. Tootle, 2009. Reconstructed streamflows for the headwaters of the Wind River, Wyoming, United States. Journal of the American Water Resources Association (JAWRA) 45(1):224–236. DOI: 10.1111/j.1752-1688.2008.00274.x

White River Watershed. 2010. Conservation Commission of Missouri. <http://extra.mdc.mo.gov/fish/watershed/whriver/geology/>. (Oct. 2010).

Wiles, G.C., R.D. D’Arrigo, and G.C. Jacoby. 1998. Gulf of Alaska atmosphere-ocean variability over recent centuries inferred from coastal tree-ring records. Climatic Change 38(3): 289–306.

Woodhouse, C.A. 2001. A tree-ring reconstruction of streamflow for the Colorado front range. Journal of the American Water Resources Association 37(3): 561–569.

Woodhouse, C.A., S.T. Gray, and D.M. Meko. 2006. Updated streamflow reconstructions for the Upper Colorado River Basin. Water Resources Research 42(5): WO5415.

Zhang, Z., M.E. Mann, and E.R. Cook. 2004. Alternative methods of proxy-based climate field reconstruction:  application to summer drought over the conterminous United States back to AD 1700 from tree-ring data. Holocene 14(4): 502–516.

# Appendices

# Appendix 1: Tables and Figures

## Tables

Table 1:  USGS streamflow gage information.

| Station # | Description | Drainage Area (km2) | Elevation (m) | Start | End |
| --- | --- | --- | --- | --- | --- |
| 03479000 | Watauga River | 239 | 795 | 1941 | 2008 |
| 03500000 | Little Tennessee River | 363 | 612 | 1945 | 2008 |
| 03504000 | Nantahala River | 134 | 937 | 1941 | 2008 |
| 03512000 | Oconaluftee River | 477 | 562 | 1949 | 2008 |
| 03550000 | Valley River | 269 | 474 | 1919 | 2008 |

Table 2:  Tree-ring chronologies used in the research along with latitude, longitude, period of record, and species.

| Station | Lat. | Lon. | Period | Species |
| --- | --- | --- | --- | --- |
| Altamaha River | 31.6 | -81.8 | 933–1985 | TADI BALDCYRPESS |
| Andrew Johnson Woods | 40.9 | -81.8 | 1626–1985 | QUAL WHITE OAK |
| Balsam Gap Mount Mitchell | 35.7 | -82.3 | 1610–1983 | PCRU RED SPRUCE |
| Black River | 34.3 | -78.2 | 385–1985 | TADI BALDCYPRESS |
| Blackwater River | 36.8 | -76.9 | 937–1985 | TADI BALDCYPRESS |
| Blue Ridge Parkway | 37.6 | -79.5 | 1594–1982 | QUPR CHESTNUT OAK |
| Bluff Mountain, Nature Conservancy | 36.4 | -81.6 | 1713–1993 | TSCR HEMLOCK |
| Boogerman Trail Tennessee | 35.6 | -83.1 | 1738–1995 | LITU TULIPTREE |
| Buffalo Beats Prairie | 39.5 | -82.2 | 1681–1995 | QUAL WHITE OAK |
| Chickahominy River | 37.4 | -76.9 | 1665–1984 | TADI BALDCYPRESS |
| Clingman's Dome | 35.6 | -83.4 | 1560–1983 | PCRU RED SPRUCE |
| Davis Purdue | 39.9 | -84.4 | 1662–1985 | QUAL WHITE OAK |
| Dragon Run | 37.6 | -76.7 | 1377–1984 | TADI BALDCYPRESS |
| Dysart Woods | 40.0 | -81.0 | 1625–1998 | QUAL WHITE OAK |
| Ebenezer Creek | 32.4 | -81.2 | 995–1985 | TADI BALDCYPRESS |
| Grandfather Mountain | 36.1 | -82.8 | 1561–1983 | PCRU RED SPRUCE |
| Greenbrier II | 35.7 | -83.4 | 1737–1995 | QUPR CHESTNUT OAK |
| Hampton Hills | 35.8 | -78.7 | 1770–1992 | QUAL WHITE OAK |
| Hemlock Cove - Sunset Field | 37.5 | -79.5 | 1533–1982 | TSCA EASTERN HEMLOCK |
| Hen Wallow Falls | 35.8 | -83.2 | 1768–1995 | TSCA EASTERN HEMLOCK |
| Joyce Kilmer Memorial Forest II | 35.4 | -83.9 | 1672–1997 | LITU TULIPTREE |
| Joyce Kilmer Wilderness | 35.2 | -84.0 | 1643–1983 | QUAL WHITE OAK |
| Kelsey Tract I | 35.1 | -83.2 | 1561–1983 | TSCA EASTERN HEMLOCK |
| Kelsey Tract II | 35.1 | -83.2 | 1679–1983 | TSCR CAROLINA HEMLOCK |
| Kit Springs Ranch, Nantahala | 35.3 | -83.9 | 1660–1993 | TSCA HEMLOCK |
| Knob Job | 38.9 | -79.4 | 1476–1981 | JUVI EASTERN RED CEDAR |
| Land Between The Lakes | 36.8 | -88.1 | 1689–2005 | QUST POST OAK |
| Lassiter Swamp (Merchants Millpond State Park) | 36.5 | -76.6 | 1528–1984 | TADI BALDCYPRESS |
| Lilley Cornett Tract | 37.1 | -83.0 | 1665–1982 | QUAL WHITE OAK |
| Linville Gorge | 35.9 | -81.9 | 1620–1977 | QUAL WHITE OAK |
| Lower Withlacoochee River | 29.0 | -82.7 | 1618–2005 | TADI BALDCYPRESS |
| Lynn Hollow and Look Rock Trail | 35.6 | -85.9 | 1685–1996 | PIEC SHORTLEAF PINE |
| Lynn Hollow I | 35.6 | -85.4 | 1755–1997 | QUPR CHESTNUT OAK |
| Lynn Hollow II | 35.6 | -85.4 | 1745–1997 | QUVE BLACK OAK |
| Mammoth Cave (Recollect) | 37.2 | -86.1 | 1651–1985 | QUAL WHITE OAK |
| Middle Withlacoochee River | 28.9 | -82.3 | 1516–2003 | TADI BALDCYPRESS |
| Montpelier National Landmark Forest | 38.2 | -78.2 | 1713–2001 | QUAL WHITE OAK |
| Mount Rogers | 36.7 | -81.7 | 1647–1982 | TSCA EASTERN HEMLOCK |
| Mountain Lake Virginia I | 37.4 | -80.5 | 1695–1982 | QUAL WHITE OAK |
| Mountain Lake Virginia II | 37.4 | -80.5 | 1554–1980 | QUAL WHITE OAK |
| Mt. Collins | 35.5 | -83.5 | 1654–1986 | PCRU RED SPRUCE |
| Norris Dam State Park | 36.2 | -84.1 | 1633–1981 | QUAL WHITE OAK |
| Nottoway River | 36.8 | -77.1 | 1175–1984 | TADI BALDCYPRESS |
| Ocmulgee River | 32.1 | -83.3 | 1206–1984 | TADI BALDCYPRESS |
| Pascagoula River | 30.6 | -88.6 | 1470–1992 | TADI BALDCYPRESS |
| Patty's Oaks, Blue Ridge Pkwy | 37.9 | -79.8 | 1572–1982 | QUAL WHITE OAK |
| Pearl River | 32.9 | -89.1 | 1550–1983 | TADI BALDCYPRESS |
| Piney Creek Pocket Wilderness | 35.7 | -84.9 | 1651–1982 | QUAL WHITE OAK |
| Pinnacle Point/Hawksbill Gap | 38.5 | -78.4 | 1615–1981 | QUAL WHITE OAK |
| Porter's Creek II | 35.7 | -82.4 | 1700–1997 | LITU TULIPTREE |
| Pulaski Woods | 41.1 | -86.7 | 1692–1985 | QUAL WHITE OAK |
| Rainbow Falls Trail II | 35.7 | -83.5 | 1689–1995 | TSCA EASTERN HEMLOCK |
| Ramseys Draft Recollection | 38.3 | -79.3 | 1600–1981 | TSCA EASTERN HEMLOCK |
| Reelfoot Lake (Quake Effects After 1811) | 36.4 | -89.4 | 1677–1990 | TADI BALDCYPRESS |
| Savage Gulf | 35.5 | -85.6 | 1616–1985 | TSCA EASTERN HEMLOCK |
| Scotts Gap | 35.6 | -83.9 | 1684–1981 | LITU TULIP POPLAR |
| Shenandoah National Park | 38.5 | -78.5 | 1757–1997 | TSCA EASTERN HEMLOCK |
| Shot Beech Ridge | 35.6 | -83.4 | 1772–1997 | QURU RED OAK |
| Sipsey Wilderness | 34.3 | -87.5 | 1682–1985 | QUAL WHITE OAK |
| Stebbin's Gulch | 41.6 | -81.3 | 1612–1983 | QUPR CHESTNUT OAK |
| Suwannee River | 29.7 | -83.0 | 1725–1993 | QULY OVERCUP OAK |
| Sweet Briar College Old-Growth White Oak | 37.6 | -79.1 | 1754–1998 | QUAL WHITE OAK |
| Upper Withlacoochee River | 28.3 | -82.1 | 1622–2005 | TADI BALDCYPRESS |
| Watch Dog, Massenhutten Mtn | 38.5 | -78.4 | 1645–1981 | QUPR CHESTNUT OAK |

Table 3:  Tree-ring chronologies that are retained in the regression model and the state, period, species, and model(s) the chronology entered.

| Station | State | Period | Species | Model |
| --- | --- | --- | --- | --- |
| Kelsey Tract II | NC | 1679–1983 | CAROLINA HEMLOCK | Little TN, Nantahala, Oconaluftee |
| Pearl River | MS | 1550–1983 | BALDCYPRESS | Little TN, Nantahala |
| Piney Creek Pocket Wilderness | TN | 1651–1982 | WHITE OAK | Nantahala, Oconaluftee, Valley |
| Ramseys Draft Recollection | VA | 1600–1981 | EASTERN HEMLOCK | Valley |
| Black River | NC | 385–1985 | BALDCYPRESS | Valley |
| Grandfather Mountain | NC | 1561–1983 | RED SPRUCE | Valley |
| Scotts Gap | TN | 1684–1981 | TULIP POPLAR | Watauga |

Table 4:  Valley River basic statistics:  mean, median, standard deviation, minimum MJJ streamflow, and maximum MJJ streamflow.

| Units = MCM | Reconstructed | Instrumental |
| --- | --- | --- |
| Period | 1652-1918 | 1919-2008 |
| Mean | 44.8 | 45.6 |
| Median | 45.1 | 40.9 |
| Standard Deviation | 12.9 | 19.2 |
| Min MJJ (Year) | 1.3 (1911) | 13.5 (1988) |
| Max MJJ (Year) | 82.5 (1680) | 104.1 (2003) |

Table 5:  Reconstruction statistics for each river including the R-squared, R-squared adjusted, R-squared predicted, Durbin-Watson, VIF,  RE, and RMSE.

| River | R2 | R2 (adj) | R2 (pred) | Durbin-Watson | VIF | RE | RMSE (MCM) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Little TN | 42.0 | 38.4 | 31.3 | 2.1 | 1.0 | 0.42 | 19.1 |
| Nantahala | 48.2 | 43.9 | 35.7 | 2.3 | 1.1 | 0.48 | 8.2 |
| Oconaluftee | 48.4 | 44.8 | 38.8 | 2.1 | 1.0 | 0.48 | 15.9 |
| Valley | 51.1 | 47.7 | 41.4 | 1.7 | 1.1 | 0.51 | 11.5 |
| Watauga | 27.7 | 25.8 | 21.3 | 1.4 | 1.0 | 0.28 | 11.6 |

Table 6:  Oconaluftee River basic statistics:  mean, median, standard deviation, minimum MJJ streamflow, and maximum MJJ streamflow.

| Units = MCM | Reconstructed | Instrumental |
| --- | --- | --- |
| Period | 1679-1948 | 1949-2008 |
| Mean | 97.7 | 99.9 |
| Median | 97.8 | 92.5 |
| Standard Deviation | 22.0 | 33.4 |
| Min MJJ (Year) | 29.9 (1708) | 46.2 (2007) |
| Max MJJ (Year) | 170.8 (1739) | 227.4 (1989) |

Table 7:  Nantahala River basic statistics:  mean, median, standard deviation, minimum MJJ streamflow, and maximum MJJ streamflow.

| Units = MCM | Reconstructed | Instrumental |
| --- | --- | --- |
| Period | 1679-1940 | 1941-2008 |
| Mean | 40.0 | 39.5 |
| Median | 40.3 | 38.2 |
| Standard Deviation | 9.4 | 13.8 |
| Min MJJ (Year) | 12.2 (1936) | 16.7 (1986) |
| Max MJJ (Year) | 63.1 (1679) | 83.5 (1989) |

Table 8:  Little Tennessee River basic statistics:  mean, median, standard deviation, minimum MJJ streamflow, and maximum MJJ streamflow.

| Units = MCM | Reconstructed | Instrumental |
| --- | --- | --- |
| Period | 1679-1944 | 1945-2008 |
| Mean | 78.7 | 76.1 |
| Median | 78.1 | 70.5 |
| Standard Deviation | 17.9 | 29.2 |
| Min MJJ (Year) | 30.5 (1926) | 29.2 (1986) |
| Max MJJ (Year) | 131.0 (1679) | 152.3 (1976) |

Table 9:  Watauga River basic statistics:  mean, median, standard deviation, minimum MJJ streamflow, and maximum MJJ streamflow.

| Units = MCM | Reconstructed | Instrumental |
| --- | --- | --- |
| Period | 1686-1940 | 1941-2008 |
| Mean | 32.3 | 32.3 |
| Median | 32.0 | 29.8 |
| Standard Deviation | 10.6 | 14.1 |
| Min MJJ (Year) | .5 (1854) | 11.9 (1988) |
| Max MJJ (Year) | 61.4 (1916) | 74.2 (1976) |

Table 10:  Valley River 5, 10, and 25-year extreme wet and dry events for both the instrumental and reconstructed period.  The values in the table are the percent departures from the mean.

|  | Instrumental (1919-2008) |  | Reconstruction (1652-1918) |  |
| --- | --- | --- | --- | --- |
| # of Years | Dry | Wet | Dry | Wet |
| 5 | -32.4 | 35.2 | -37.5 | 29.6 |
| 10 | -18.7 | 24.2 | -30.1 | 19.1 |
| 25 | -11.9 | 8.4 | -13.6 | 7.5 |

Table 11:  Oconaluftee River 5, 10, and 25-year extreme wet and dry events for both the instrumental and reconstructed period.  The values in the table are the percent departures from the mean.

|  | Instrumental (1949-2008) |  | Reconstruction (1679-1948) |  |
| --- | --- | --- | --- | --- |
| # of Years | Dry | Wet | Dry | Wet |
| 5 | -19.0 | 19.5 | -26.3 | 28.5 |
| 10 | -13.3 | 18.4 | -23.9 | 18.4 |
| 25 | -6.0 | 8.0 | -16.6 | 6.1 |

Table 12:  Nantahala River 5, 10, and 25-year extreme wet and dry events for both the instrumental and reconstructed period.  The values in the table are the percent departures from the mean.

|  | Instrumental (1941-2008) |  | Reconstruction (1679-1940) |  |
| --- | --- | --- | --- | --- |
| # of Years | Dry | Wet | Dry | Wet |
| 5 | -27.5 | 31.5 | -21.6 | 26.4 |
| 10 | -11.7 | 21.5 | -17.8 | 19.8 |
| 25 | -5.8 | 6.6 | -9.3 | 8.6 |

## Figures

Figure 1:  Map of the USGS streamflow gages and tree-ring chronologies used.

Figure 2:  Seasonal correlation plot.

Figure 3:  Three month seasonal correlation plot.

Figure 4:  Four month seasonal correlation plot.

Figure 5:  Six month and annual seasonal correlation plot.

Figure 6:  Climate/tree growth relationship for the Valley River MJJ streamflow/Piney Creek Tree-Ring Chronology using a 10-year filter and the 20-year filter over the periods (1928–1980) and (1938–1980) respectively.

Figure 7:  Map of the Valley River streamflow gage and retained tree-ring chronologies.

Figure 8:  Valley River calibration plot of MJJ average streamflow from 1919 to 1980.  The black line represents the instrumental streamflow, and the gray line represents the reconstructed streamflow.

Figure 9:  Valley River plot of combined reconstructed (1661–1918) and instrumental (1919–2008) MJJ streamflow using a 10-year end-year filter.

Figure 10:  Valley River plot of combined reconstructed (1671–1918) and instrumental (1919–2008) MJJ streamflow using a 20-year end-year filter.

Figure 11:  Map of Oconaluftee River streamflow gage and retained tree-ring chronologies.

Figure 12:  Oconaluftee River calibration plot of MJJ average streamflow from 1949 to 1980.  The black line represents the instrumental streamflow, and the gray line represents the reconstructed streamflow.

Figure 13:  Oconaluftee River plot of combined reconstructed (1688–1948) and instrumental (1949–2008) MJJ streamflow using a 10-year end-year filter.

Figure 14:  Oconaluftee River plot of combined reconstructed (1698–1948) and instrumental (1949–2008) MJJ streamflow using a 20-year end-year filter.

Figure 15:  Map of Nantahala River streamflow gage and retained tree-ring chronologies.

Figure 16:  Nantahala River calibration plot of MJJ average streamflow from 1941 to 1980.  The black line represents the instrumental streamflow, and the gray line represents the reconstructed streamflow.

Figure 17:  Nantahala River plot of combined reconstructed (1688–1940) and instrumental (1941–2008) MJJ streamflow using a 10-year end-year filter.

Figure 18:  Nantahala River plot of combined reconstructed (1698–1940) and instrumental (1941–2008) MJJ streamflow using a 20-year end-year filter.

Figure 19:  Map of Little Tennessee River streamflow gage and retained tree-ring chronologies.

Figure 20:  Map of Watauga River streamflow gage and retained tree-ring chronologies.

Figure 21:  Little Tennessee River calibration plot of MJJ average streamflow from 1945 to 1980.  The black line represents the instrumental streamflow, and the gray line represents the reconstructed streamflow.

Figure 22:  Little Tennessee River plot of combined reconstructed (1688–1944) and instrumental (1945–2008) MJJ streamflow using a 10-year end-year filter.

Figure 23:  Little Tennessee River plot of combined reconstructed (1698–1944) and instrumental (1945–2008) MJJ streamflow using a 20-year end-year filter.

Figure 24:  Watauga River calibration plot of MJJ average streamflow from 1941 to 1980.  The black line represents the instrumental streamflow, and the gray line represents the reconstructed streamflow.

Figure 25:  Watauga River plot of combined reconstructed (1695–1940) and instrumental (1941–2008) MJJ streamflow using a 10-year end-year filter.

Figure 26:  Watauga River plot of combined reconstructed (1705–1940) and instrumental (1941–2008) MJJ streamflow using a 20-year end-year filter.

Figure 27:  Valley River 5-year averages distribution of reconstructed values within the lowest 10th percentile.  The y-axis represents the rank for the period of low flow, and the x-axis represents the end year for the 5-year average period of the reconstructed (1656–1918) and instrumental (1919–2008) record.

Figure 28:  Oconaluftee River 5-year averages distribution of reconstructed values within the lowest 10th percentile.  The y-axis represents the rank for the period of low flow, and the x-axis represents the end year for the 5-year average period of the reconstructed (1683–1948) and instrumental (1949–2008) record.

Figure 29:  Nantahala River 5-year averages distribution of reconstructed values within the lowest 10th percentile.  The y-axis represents the rank for the period of low flow, and the x-axis represents the end year for the 5-year average period of the reconstructed (1683–1940) and instrumental (1941–2008) record.

Figure 30:  Valley River percent normal plot of combined reconstructed (1671–1918) and instrumental (1919–2008) MJJ streamflow using a 20-year end-year filter.  Gray areas represent wet periods and black areas represent dry periods.

Figure 31:  Oconaluftee River percent normal plot of combined reconstructed (1698–1948) and instrumental (1949–2008) MJJ streamflow using a 20-year end-year filter.  Gray areas represent wet periods and black areas represent dry periods.

Figure 32:  Nantahala River percent normal plot of combined reconstructed (1698–1940) and instrumental (1941–2008) MJJ streamflow using a 20-year end-year filter. Gray areas represent wet periods and black areas represent dry periods.

# Appendix 2:  Reconstruction Data

## Tree Based

Note:  Residual tree-ring chronology index values are unitless.  A value of 1000 represents the long term mean.  Higher or lower values for a given year represent proportionally higher or lower tree growth for that year

### Kelsey Tract II Residuals

| Year | Kelsey Tract II Residual |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1679 | 1376 | 1714 | 885 | 1749 | 1148 |
| 1680 | 1050 | 1715 | 669 | 1750 | 972 |
| 1681 | 763 | 1716 | 1277 | 1751 | 970 |
| 1682 | 1122 | 1717 | 1174 | 1752 | 1031 |
| 1683 | 1126 | 1718 | 866 | 1753 | 1024 |
| 1684 | 1043 | 1719 | 1267 | 1754 | 786 |
| 1685 | 874 | 1720 | 983 | 1755 | 964 |
| 1686 | 1097 | 1721 | 1063 | 1756 | 983 |
| 1687 | 895 | 1722 | 914 | 1757 | 875 |
| 1688 | 733 | 1723 | 1067 | 1758 | 991 |
| 1689 | 904 | 1724 | 976 | 1759 | 1156 |
| 1690 | 828 | 1725 | 771 | 1760 | 635 |
| 1691 | 1172 | 1726 | 1072 | 1761 | 1360 |
| 1692 | 915 | 1727 | 945 | 1762 | 842 |
| 1693 | 806 | 1728 | 1083 | 1763 | 1124 |
| 1694 | 990 | 1729 | 1197 | 1764 | 845 |
| 1695 | 1052 | 1730 | 945 | 1765 | 1085 |
| 1696 | 932 | 1731 | 992 | 1766 | 1273 |
| 1697 | 742 | 1732 | 704 | 1767 | 1058 |
| 1698 | 980 | 1733 | 1228 | 1768 | 1211 |
| 1699 | 1249 | 1734 | 1271 | 1769 | 1078 |
| 1700 | 761 | 1735 | 902 | 1770 | 895 |
| 1701 | 901 | 1736 | 1158 | 1771 | 940 |
| 1702 | 1305 | 1737 | 939 | 1772 | 756 |
| 1703 | 980 | 1738 | 940 | 1773 | 1214 |
| 1704 | 999 | 1739 | 1014 | 1774 | 919 |
| 1705 | 890 | 1740 | 902 | 1775 | 1016 |
| 1706 | 792 | 1741 | 1070 | 1776 | 1009 |
| 1707 | 949 | 1742 | 1082 | 1777 | 1073 |
| 1708 | 719 | 1743 | 881 | 1778 | 1103 |
| 1709 | 975 | 1744 | 850 | 1779 | 852 |
| 1710 | 1205 | 1745 | 952 | 1780 | 966 |
| 1711 | 1177 | 1746 | 899 | 1781 | 1065 |
| 1712 | 1219 | 1747 | 992 | 1782 | 681 |
| 1713 | 824 | 1748 | 886 | 1783 | 1206 |
| 1784 | 959 | 1819 | 719 | 1854 | 1406 |
| 1785 | 961 | 1820 | 841 | 1855 | 911 |
| 1786 | 1099 | 1821 | 753 | 1856 | 869 |
| 1787 | 824 | 1822 | 1068 | 1857 | 1237 |
| 1788 | 1064 | 1823 | 1220 | 1858 | 977 |
| 1789 | 796 | 1824 | 949 | 1859 | 963 |
| 1790 | 914 | 1825 | 1061 | 1860 | 972 |
| 1791 | 1028 | 1826 | 852 | 1861 | 1204 |
| 1792 | 1190 | 1827 | 995 | 1862 | 982 |
| 1793 | 1038 | 1828 | 957 | 1863 | 1166 |
| 1794 | 1095 | 1829 | 1083 | 1864 | 1011 |
| 1795 | 948 | 1830 | 830 | 1865 | 1039 |
| 1796 | 941 | 1831 | 1073 | 1866 | 996 |
| 1797 | 801 | 1832 | 853 | 1867 | 987 |
| 1798 | 1154 | 1833 | 1171 | 1868 | 805 |
| 1799 | 856 | 1834 | 1054 | 1869 | 1016 |
| 1800 | 1047 | 1835 | 1018 | 1870 | 1239 |
| 1801 | 807 | 1836 | 936 | 1871 | 728 |
| 1802 | 883 | 1837 | 831 | 1872 | 1311 |
| 1803 | 1023 | 1838 | 1064 | 1873 | 932 |
| 1804 | 1340 | 1839 | 891 | 1874 | 886 |
| 1805 | 1045 | 1840 | 913 | 1875 | 886 |
| 1806 | 990 | 1841 | 1031 | 1876 | 716 |
| 1807 | 1252 | 1842 | 1157 | 1877 | 918 |
| 1808 | 908 | 1843 | 908 | 1878 | 1055 |
| 1809 | 949 | 1844 | 1115 | 1879 | 990 |
| 1810 | 856 | 1845 | 882 | 1880 | 949 |
| 1811 | 989 | 1846 | 1164 | 1881 | 961 |
| 1812 | 999 | 1847 | 989 | 1882 | 1146 |
| 1813 | 997 | 1848 | 994 | 1883 | 685 |
| 1814 | 1285 | 1849 | 1125 | 1884 | 1144 |
| 1815 | 1095 | 1850 | 1113 | 1885 | 987 |
| 1816 | 1153 | 1851 | 1021 | 1886 | 865 |
| 1817 | 1259 | 1852 | 1094 | 1887 | 903 |
| 1818 | 623 | 1853 | 1046 | 1888 | 1100 |
| 1889 | 964 | 1924 | 803 | 1959 | 1179 |
| 1890 | 888 | 1925 | 962 | 1960 | 780 |
| 1891 | 825 | 1926 | 600 | 1961 | 1042 |
| 1892 | 872 | 1927 | 1085 | 1962 | 1087 |
| 1893 | 963 | 1928 | 989 | 1963 | 1075 |
| 1894 | 1127 | 1929 | 1148 | 1964 | 616 |
| 1895 | 656 | 1930 | 988 | 1965 | 795 |
| 1896 | 877 | 1931 | 1102 | 1966 | 924 |
| 1897 | 1128 | 1932 | 766 | 1967 | 1051 |
| 1898 | 861 | 1933 | 733 | 1968 | 1078 |
| 1899 | 981 | 1934 | 1007 | 1969 | 911 |
| 1900 | 1091 | 1935 | 1129 | 1970 | 954 |
| 1901 | 1208 | 1936 | 707 | 1971 | 1166 |
| 1902 | 991 | 1937 | 937 | 1972 | 1099 |
| 1903 | 1263 | 1938 | 840 | 1973 | 1148 |
| 1904 | 1115 | 1939 | 942 | 1974 | 1079 |
| 1905 | 1157 | 1940 | 1131 | 1975 | 967 |
| 1906 | 1072 | 1941 | 853 | 1976 | 1217 |
| 1907 | 966 | 1942 | 1014 | 1977 | 922 |
| 1908 | 1369 | 1943 | 924 | 1978 | 784 |
| 1909 | 948 | 1944 | 949 | 1979 | 997 |
| 1910 | 947 | 1945 | 1048 | 1980 | 1020 |
| 1911 | 932 | 1946 | 1118 |  |  |
| 1912 | 795 | 1947 | 1004 |  |  |
| 1913 | 919 | 1948 | 1248 |  |  |
| 1914 | 790 | 1949 | 1246 |  |  |
| 1915 | 1126 | 1950 | 998 |  |  |
| 1916 | 881 | 1951 | 893 |  |  |
| 1917 | 867 | 1952 | 1067 |  |  |
| 1918 | 1023 | 1953 | 869 |  |  |
| 1919 | 1233 | 1954 | 768 |  |  |
| 1920 | 1267 | 1955 | 1160 |  |  |
| 1921 | 1028 | 1956 | 954 |  |  |
| 1922 | 928 | 1957 | 1188 |  |  |
| 1923 | 872 | 1958 | 1114 |  |  |

### Pearl River Residuals

| Year | Pearl River Residual |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1549 | 639 | 1584 | 1344 | 1619 | 857 |
| 1550 | 942 | 1585 | 1271 | 1620 | 1541 |
| 1551 | 1096 | 1586 | 2072 | 1621 | 668 |
| 1552 | 1021 | 1587 | 556 | 1622 | 943 |
| 1553 | 898 | 1588 | 882 | 1623 | 729 |
| 1554 | 1014 | 1589 | 1906 | 1624 | 653 |
| 1555 | 1125 | 1590 | 1176 | 1625 | 746 |
| 1556 | 938 | 1591 | 1180 | 1626 | 1867 |
| 1557 | 1250 | 1592 | 1375 | 1627 | 842 |
| 1558 | 898 | 1593 | 927 | 1628 | 679 |
| 1559 | 1519 | 1594 | 601 | 1629 | 1231 |
| 1560 | 1368 | 1595 | 1003 | 1630 | 1215 |
| 1561 | 1173 | 1596 | 884 | 1631 | 1095 |
| 1562 | 863 | 1597 | 804 | 1632 | 872 |
| 1563 | 1519 | 1598 | 639 | 1633 | 805 |
| 1564 | 1840 | 1599 | 554 | 1634 | 752 |
| 1565 | 267 | 1600 | 1273 | 1635 | 882 |
| 1566 | 493 | 1601 | 1067 | 1636 | 1067 |
| 1567 | 823 | 1602 | 903 | 1637 | 569 |
| 1568 | 1343 | 1603 | 1179 | 1638 | 879 |
| 1569 | 438 | 1604 | 1009 | 1639 | 1287 |
| 1570 | 635 | 1605 | 1237 | 1640 | 1187 |
| 1571 | 700 | 1606 | 1063 | 1641 | 906 |
| 1572 | 885 | 1607 | 1158 | 1642 | 790 |
| 1573 | 1027 | 1608 | 550 | 1643 | 766 |
| 1574 | 1284 | 1609 | 944 | 1644 | 797 |
| 1575 | 1227 | 1610 | 759 | 1645 | 1356 |
| 1576 | 606 | 1611 | 773 | 1646 | 718 |
| 1577 | 1061 | 1612 | 719 | 1647 | 1111 |
| 1578 | 1239 | 1613 | 981 | 1648 | 1266 |
| 1579 | 829 | 1614 | 780 | 1649 | 1109 |
| 1580 | 985 | 1615 | 849 | 1650 | 1026 |
| 1581 | 1079 | 1616 | 1003 | 1651 | 661 |
| 1582 | 817 | 1617 | 982 | 1652 | 1067 |
| 1583 | 989 | 1618 | 714 | 1653 | 968 |
| 1654 | 775 | 1689 | 1871 | 1724 | 1347 |
| 1655 | 806 | 1690 | 1110 | 1725 | 391 |
| 1656 | 1193 | 1691 | 840 | 1726 | 829 |
| 1657 | 878 | 1692 | 1108 | 1727 | 854 |
| 1658 | 1535 | 1693 | 1017 | 1728 | 1216 |
| 1659 | 912 | 1694 | 645 | 1729 | 1138 |
| 1660 | 771 | 1695 | 1150 | 1730 | 727 |
| 1661 | 1153 | 1696 | 1119 | 1731 | 855 |
| 1662 | 786 | 1697 | 568 | 1732 | 937 |
| 1663 | 804 | 1698 | 1197 | 1733 | 1010 |
| 1664 | 889 | 1699 | 902 | 1734 | 1398 |
| 1665 | 1452 | 1700 | 1210 | 1735 | 730 |
| 1666 | 838 | 1701 | 1528 | 1736 | 1168 |
| 1667 | 1086 | 1702 | 981 | 1737 | 864 |
| 1668 | 1028 | 1703 | 740 | 1738 | 755 |
| 1669 | 734 | 1704 | 659 | 1739 | 771 |
| 1670 | 844 | 1705 | 768 | 1740 | 1207 |
| 1671 | 614 | 1706 | 1221 | 1741 | 876 |
| 1672 | 1087 | 1707 | 900 | 1742 | 1380 |
| 1673 | 1200 | 1708 | 802 | 1743 | 969 |
| 1674 | 1316 | 1709 | 476 | 1744 | 782 |
| 1675 | 758 | 1710 | 873 | 1745 | 694 |
| 1676 | 632 | 1711 | 1263 | 1746 | 1715 |
| 1677 | 777 | 1712 | 1097 | 1747 | 862 |
| 1678 | 780 | 1713 | 1333 | 1748 | 1217 |
| 1679 | 1537 | 1714 | 725 | 1749 | 1009 |
| 1680 | 980 | 1715 | 623 | 1750 | 1371 |
| 1681 | 1057 | 1716 | 1240 | 1751 | 830 |
| 1682 | 943 | 1717 | 1053 | 1752 | 785 |
| 1683 | 1000 | 1718 | 773 | 1753 | 927 |
| 1684 | 932 | 1719 | 1165 | 1754 | 987 |
| 1685 | 956 | 1720 | 787 | 1755 | 971 |
| 1686 | 874 | 1721 | 1494 | 1756 | 609 |
| 1687 | 1263 | 1722 | 541 | 1757 | 884 |
| 1688 | 1095 | 1723 | 1038 | 1758 | 1153 |
| 1759 | 773 | 1794 | 471 | 1829 | 1435 |
| 1760 | 577 | 1795 | 1103 | 1830 | 1055 |
| 1761 | 1125 | 1796 | 1227 | 1831 | 689 |
| 1762 | 998 | 1797 | 781 | 1832 | 922 |
| 1763 | 1449 | 1798 | 830 | 1833 | 858 |
| 1764 | 964 | 1799 | 944 | 1834 | 596 |
| 1765 | 869 | 1800 | 781 | 1835 | 1326 |
| 1766 | 806 | 1801 | 703 | 1836 | 1625 |
| 1767 | 553 | 1802 | 866 | 1837 | 926 |
| 1768 | 896 | 1803 | 1009 | 1838 | 1268 |
| 1769 | 1390 | 1804 | 1154 | 1839 | 937 |
| 1770 | 801 | 1805 | 2233 | 1840 | 756 |
| 1771 | 691 | 1806 | 1314 | 1841 | 1025 |
| 1772 | 739 | 1807 | 230 | 1842 | 1432 |
| 1773 | 967 | 1808 | 615 | 1843 | 1162 |
| 1774 | 1401 | 1809 | 792 | 1844 | 610 |
| 1775 | 1024 | 1810 | 994 | 1845 | 986 |
| 1776 | 1148 | 1811 | 1035 | 1846 | 539 |
| 1777 | 1183 | 1812 | 658 | 1847 | 881 |
| 1778 | 998 | 1813 | 1187 | 1848 | 949 |
| 1779 | 1006 | 1814 | 706 | 1849 | 964 |
| 1780 | 507 | 1815 | 1215 | 1850 | 1088 |
| 1781 | 899 | 1816 | 709 | 1851 | 1157 |
| 1782 | 1685 | 1817 | 938 | 1852 | 942 |
| 1783 | 558 | 1818 | 801 | 1853 | 967 |
| 1784 | 693 | 1819 | 640 | 1854 | 1157 |
| 1785 | 1137 | 1820 | 926 | 1855 | 806 |
| 1786 | 1033 | 1821 | 1098 | 1856 | 598 |
| 1787 | 1148 | 1822 | 733 | 1857 | 1193 |
| 1788 | 1289 | 1823 | 1382 | 1858 | 1382 |
| 1789 | 1263 | 1824 | 720 | 1859 | 632 |
| 1790 | 972 | 1825 | 917 | 1860 | 875 |
| 1791 | 730 | 1826 | 1046 | 1861 | 757 |
| 1792 | 1244 | 1827 | 759 | 1862 | 1025 |
| 1793 | 1609 | 1828 | 648 | 1863 | 668 |
| 1864 | 1422 | 1899 | 984 | 1934 | 1005 |
| 1865 | 784 | 1900 | 1667 | 1935 | 1275 |
| 1866 | 1341 | 1901 | 797 | 1936 | 494 |
| 1867 | 1469 | 1902 | 563 | 1937 | 793 |
| 1868 | 562 | 1903 | 1013 | 1938 | 845 |
| 1869 | 1256 | 1904 | 407 | 1939 | 1319 |
| 1870 | 987 | 1905 | 777 | 1940 | 810 |
| 1871 | 653 | 1906 | 1287 | 1941 | 517 |
| 1872 | 1011 | 1907 | 1478 | 1942 | 907 |
| 1873 | 1115 | 1908 | 1504 | 1943 | 658 |
| 1874 | 814 | 1909 | 1958 | 1944 | 884 |
| 1875 | 897 | 1910 | 1175 | 1945 | 708 |
| 1876 | 1029 | 1911 | 318 | 1946 | 1258 |
| 1877 | 728 | 1912 | 1221 | 1947 | 629 |
| 1878 | 1427 | 1913 | 920 | 1948 | 748 |
| 1879 | 1145 | 1914 | 640 | 1949 | 1435 |
| 1880 | 1119 | 1915 | 741 | 1950 | 599 |
| 1881 | 933 | 1916 | 1325 | 1951 | 1190 |
| 1882 | 1122 | 1917 | 729 | 1952 | 489 |
| 1883 | 1006 | 1918 | 891 | 1953 | 844 |
| 1884 | 1387 | 1919 | 1584 | 1954 | 985 |
| 1885 | 993 | 1920 | 1167 | 1955 | 845 |
| 1886 | 1173 | 1921 | 832 | 1956 | 450 |
| 1887 | 666 | 1922 | 1184 | 1957 | 951 |
| 1888 | 1014 | 1923 | 1394 | 1958 | 924 |
| 1889 | 1331 | 1924 | 398 | 1959 | 1421 |
| 1890 | 946 | 1925 | 738 | 1960 | 982 |
| 1891 | 907 | 1926 | 573 | 1961 | 1605 |
| 1892 | 1436 | 1927 | 960 | 1962 | 637 |
| 1893 | 1040 | 1928 | 849 | 1963 | 983 |
| 1894 | 581 | 1929 | 1153 | 1964 | 1576 |
| 1895 | 742 | 1930 | 854 | 1965 | 1122 |
| 1896 | 869 | 1931 | 802 | 1966 | 1448 |
| 1897 | 709 | 1932 | 834 | 1967 | 1294 |
| 1898 | 1010 | 1933 | 1113 | 1968 | 1185 |
| 1969 | 675 |  |  |  |  |
| 1970 | 1238 |  |  |  |  |
| 1971 | 889 |  |  |  |  |
| 1972 | 884 |  |  |  |  |
| 1973 | 1058 |  |  |  |  |
| 1974 | 1535 |  |  |  |  |
| 1975 | 912 |  |  |  |  |
| 1976 | 851 |  |  |  |  |
| 1977 | 598 |  |  |  |  |
| 1978 | 687 |  |  |  |  |
| 1979 | 1185 |  |  |  |  |
| 1980 | 794 |  |  |  |  |

### Piney Creek Pocket Wilderness Residuals

| Year | Piney Creek Residual |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1652 | 687 | 1687 | 1097 | 1722 | 712 |
| 1653 | 738 | 1688 | 1151 | 1723 | 809 |
| 1654 | 936 | 1689 | 1097 | 1724 | 826 |
| 1655 | 960 | 1690 | 959 | 1725 | 1094 |
| 1656 | 1020 | 1691 | 1249 | 1726 | 1096 |
| 1657 | 979 | 1692 | 871 | 1727 | 971 |
| 1658 | 882 | 1693 | 777 | 1728 | 703 |
| 1659 | 908 | 1694 | 1066 | 1729 | 940 |
| 1660 | 1013 | 1695 | 1211 | 1730 | 775 |
| 1661 | 879 | 1696 | 861 | 1731 | 1170 |
| 1662 | 913 | 1697 | 860 | 1732 | 782 |
| 1663 | 1001 | 1698 | 950 | 1733 | 1322 |
| 1664 | 1224 | 1699 | 916 | 1734 | 1189 |
| 1665 | 1087 | 1700 | 963 | 1735 | 835 |
| 1666 | 803 | 1701 | 1412 | 1736 | 877 |
| 1667 | 919 | 1702 | 902 | 1737 | 1397 |
| 1668 | 981 | 1703 | 815 | 1738 | 1227 |
| 1669 | 1177 | 1704 | 1171 | 1739 | 1623 |
| 1670 | 649 | 1705 | 1336 | 1740 | 1182 |
| 1671 | 946 | 1706 | 939 | 1741 | 908 |
| 1672 | 692 | 1707 | 1127 | 1742 | 837 |
| 1673 | 1006 | 1708 | 540 | 1743 | 791 |
| 1674 | 1114 | 1709 | 782 | 1744 | 948 |
| 1675 | 1113 | 1710 | 721 | 1745 | 1200 |
| 1676 | 660 | 1711 | 1004 | 1746 | 988 |
| 1677 | 1100 | 1712 | 657 | 1747 | 1316 |
| 1678 | 1262 | 1713 | 838 | 1748 | 925 |
| 1679 | 1066 | 1714 | 784 | 1749 | 975 |
| 1680 | 1552 | 1715 | 869 | 1750 | 1185 |
| 1681 | 536 | 1716 | 829 | 1751 | 1057 |
| 1682 | 1274 | 1717 | 994 | 1752 | 934 |
| 1683 | 933 | 1718 | 745 | 1753 | 992 |
| 1684 | 1361 | 1719 | 974 | 1754 | 1115 |
| 1685 | 772 | 1720 | 828 | 1755 | 812 |
| 1686 | 953 | 1721 | 1041 | 1756 | 867 |
| 1757 | 907 | 1792 | 886 | 1827 | 1024 |
| 1758 | 1160 | 1793 | 1017 | 1828 | 1023 |
| 1759 | 1043 | 1794 | 999 | 1829 | 854 |
| 1760 | 902 | 1795 | 888 | 1830 | 1064 |
| 1761 | 1155 | 1796 | 1051 | 1831 | 955 |
| 1762 | 910 | 1797 | 1067 | 1832 | 1089 |
| 1763 | 1096 | 1798 | 978 | 1833 | 838 |
| 1764 | 960 | 1799 | 932 | 1834 | 929 |
| 1765 | 1010 | 1800 | 1069 | 1835 | 961 |
| 1766 | 831 | 1801 | 927 | 1836 | 937 |
| 1767 | 851 | 1802 | 1321 | 1837 | 915 |
| 1768 | 1153 | 1803 | 1045 | 1838 | 958 |
| 1769 | 959 | 1804 | 961 | 1839 | 730 |
| 1770 | 1026 | 1805 | 1203 | 1840 | 1192 |
| 1771 | 1006 | 1806 | 855 | 1841 | 897 |
| 1772 | 750 | 1807 | 902 | 1842 | 1056 |
| 1773 | 799 | 1808 | 1130 | 1843 | 983 |
| 1774 | 821 | 1809 | 1122 | 1844 | 823 |
| 1775 | 1151 | 1810 | 1102 | 1845 | 1049 |
| 1776 | 1076 | 1811 | 929 | 1846 | 1029 |
| 1777 | 929 | 1812 | 1124 | 1847 | 1019 |
| 1778 | 1272 | 1813 | 938 | 1848 | 897 |
| 1779 | 696 | 1814 | 1334 | 1849 | 1092 |
| 1780 | 1131 | 1815 | 1226 | 1850 | 784 |
| 1781 | 1141 | 1816 | 882 | 1851 | 854 |
| 1782 | 1464 | 1817 | 1082 | 1852 | 983 |
| 1783 | 941 | 1818 | 1063 | 1853 | 936 |
| 1784 | 798 | 1819 | 903 | 1854 | 1043 |
| 1785 | 926 | 1820 | 1217 | 1855 | 1040 |
| 1786 | 1050 | 1821 | 814 | 1856 | 881 |
| 1787 | 632 | 1822 | 951 | 1857 | 1074 |
| 1788 | 905 | 1823 | 1197 | 1858 | 912 |
| 1789 | 805 | 1824 | 1092 | 1859 | 889 |
| 1790 | 838 | 1825 | 817 | 1860 | 1078 |
| 1791 | 927 | 1826 | 850 | 1861 | 915 |
| 1862 | 1132 | 1897 | 908 | 1932 | 908 |
| 1863 | 948 | 1898 | 815 | 1933 | 1038 |
| 1864 | 1055 | 1899 | 1020 | 1934 | 1108 |
| 1865 | 1101 | 1900 | 901 | 1935 | 1197 |
| 1866 | 993 | 1901 | 967 | 1936 | 616 |
| 1867 | 1033 | 1902 | 916 | 1937 | 1186 |
| 1868 | 900 | 1903 | 1051 | 1938 | 1327 |
| 1869 | 1208 | 1904 | 1008 | 1939 | 873 |
| 1870 | 834 | 1905 | 1063 | 1940 | 1085 |
| 1871 | 948 | 1906 | 1066 | 1941 | 954 |
| 1872 | 837 | 1907 | 1132 | 1942 | 1041 |
| 1873 | 1029 | 1908 | 974 | 1943 | 919 |
| 1874 | 755 | 1909 | 884 | 1944 | 956 |
| 1875 | 1027 | 1910 | 1079 | 1945 | 1260 |
| 1876 | 1111 | 1911 | 599 | 1946 | 1159 |
| 1877 | 970 | 1912 | 1053 | 1947 | 949 |
| 1878 | 930 | 1913 | 864 | 1948 | 698 |
| 1879 | 798 | 1914 | 823 | 1949 | 1106 |
| 1880 | 914 | 1915 | 1146 | 1950 | 1115 |
| 1881 | 1057 | 1916 | 1074 | 1951 | 1133 |
| 1882 | 1058 | 1917 | 1082 | 1952 | 847 |
| 1883 | 1011 | 1918 | 896 | 1953 | 821 |
| 1884 | 932 | 1919 | 981 | 1954 | 990 |
| 1885 | 923 | 1920 | 942 | 1955 | 1035 |
| 1886 | 1090 | 1921 | 936 | 1956 | 958 |
| 1887 | 932 | 1922 | 1251 | 1957 | 825 |
| 1888 | 1075 | 1923 | 1223 | 1958 | 1050 |
| 1889 | 1316 | 1924 | 1142 | 1959 | 896 |
| 1890 | 832 | 1925 | 649 | 1960 | 952 |
| 1891 | 959 | 1926 | 997 | 1961 | 1009 |
| 1892 | 1135 | 1927 | 1178 | 1962 | 797 |
| 1893 | 848 | 1928 | 1207 | 1963 | 1023 |
| 1894 | 809 | 1929 | 1102 | 1964 | 1008 |
| 1895 | 900 | 1930 | 895 | 1965 | 1059 |
| 1896 | 1044 | 1931 | 877 | 1966 | 799 |
| 1967 | 1029 |  |  |  |  |
| 1968 | 942 |  |  |  |  |
| 1969 | 877 |  |  |  |  |
| 1970 | 1046 |  |  |  |  |
| 1971 | 1060 |  |  |  |  |
| 1972 | 1058 |  |  |  |  |
| 1973 | 1090 |  |  |  |  |
| 1974 | 1063 |  |  |  |  |
| 1975 | 967 |  |  |  |  |
| 1976 | 1102 |  |  |  |  |
| 1977 | 863 |  |  |  |  |
| 1978 | 1016 |  |  |  |  |
| 1979 | 882 |  |  |  |  |
| 1980 | 835 |  |  |  |  |

### Ramsey’s Draft Recollection Residuals

| Year | Ramseys Draft Recollection Residual |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1598 | 661 | 1633 | 994 | 1668 | 1208 |
| 1599 | 731 | 1634 | 930 | 1669 | 670 |
| 1600 | 988 | 1635 | 1182 | 1670 | 1102 |
| 1601 | 913 | 1636 | 718 | 1671 | 1002 |
| 1602 | 888 | 1637 | 937 | 1672 | 1241 |
| 1603 | 1415 | 1638 | 1206 | 1673 | 1100 |
| 1604 | 1287 | 1639 | 1109 | 1674 | 1129 |
| 1605 | 1498 | 1640 | 1048 | 1675 | 1059 |
| 1606 | 1081 | 1641 | 1030 | 1676 | 737 |
| 1607 | 768 | 1642 | 1098 | 1677 | 1241 |
| 1608 | 786 | 1643 | 588 | 1678 | 1139 |
| 1609 | 828 | 1644 | 945 | 1679 | 1037 |
| 1610 | 1207 | 1645 | 1011 | 1680 | 896 |
| 1611 | 1536 | 1646 | 783 | 1681 | 908 |
| 1612 | 1256 | 1647 | 796 | 1682 | 823 |
| 1613 | 1177 | 1648 | 917 | 1683 | 974 |
| 1614 | 1338 | 1649 | 846 | 1684 | 1138 |
| 1615 | 503 | 1650 | 937 | 1685 | 1049 |
| 1616 | 916 | 1651 | 783 | 1686 | 1165 |
| 1617 | 974 | 1652 | 888 | 1687 | 619 |
| 1618 | 1152 | 1653 | 1014 | 1688 | 1180 |
| 1619 | 1178 | 1654 | 1224 | 1689 | 1025 |
| 1620 | 1143 | 1655 | 1077 | 1690 | 1105 |
| 1621 | 567 | 1656 | 1028 | 1691 | 897 |
| 1622 | 1271 | 1657 | 1121 | 1692 | 730 |
| 1623 | 1281 | 1658 | 1097 | 1693 | 1039 |
| 1624 | 844 | 1659 | 1147 | 1694 | 890 |
| 1625 | 884 | 1660 | 1139 | 1695 | 982 |
| 1626 | 1077 | 1661 | 891 | 1696 | 839 |
| 1627 | 1165 | 1662 | 1159 | 1697 | 877 |
| 1628 | 1071 | 1663 | 1119 | 1698 | 773 |
| 1629 | 933 | 1664 | 914 | 1699 | 885 |
| 1630 | 881 | 1665 | 1096 | 1700 | 892 |
| 1631 | 1132 | 1666 | 925 | 1701 | 1024 |
| 1632 | 892 | 1667 | 1097 | 1702 | 1082 |
| 1703 | 924 | 1738 | 968 | 1773 | 1108 |
| 1704 | 988 | 1739 | 975 | 1774 | 1048 |
| 1705 | 1004 | 1740 | 957 | 1775 | 941 |
| 1706 | 1138 | 1741 | 825 | 1776 | 1331 |
| 1707 | 1037 | 1742 | 1034 | 1777 | 1091 |
| 1708 | 757 | 1743 | 1014 | 1778 | 953 |
| 1709 | 868 | 1744 | 973 | 1779 | 923 |
| 1710 | 1184 | 1745 | 1148 | 1780 | 914 |
| 1711 | 979 | 1746 | 798 | 1781 | 1013 |
| 1712 | 903 | 1747 | 1077 | 1782 | 817 |
| 1713 | 921 | 1748 | 789 | 1783 | 914 |
| 1714 | 1015 | 1749 | 1182 | 1784 | 1028 |
| 1715 | 783 | 1750 | 1059 | 1785 | 997 |
| 1716 | 944 | 1751 | 1015 | 1786 | 1219 |
| 1717 | 969 | 1752 | 1031 | 1787 | 785 |
| 1718 | 918 | 1753 | 889 | 1788 | 1042 |
| 1719 | 851 | 1754 | 976 | 1789 | 1124 |
| 1720 | 946 | 1755 | 869 | 1790 | 895 |
| 1721 | 939 | 1756 | 1169 | 1791 | 912 |
| 1722 | 1201 | 1757 | 1089 | 1792 | 737 |
| 1723 | 1052 | 1758 | 975 | 1793 | 1073 |
| 1724 | 924 | 1759 | 1021 | 1794 | 811 |
| 1725 | 1388 | 1760 | 1028 | 1795 | 988 |
| 1726 | 1237 | 1761 | 1209 | 1796 | 1054 |
| 1727 | 1029 | 1762 | 887 | 1797 | 872 |
| 1728 | 888 | 1763 | 989 | 1798 | 928 |
| 1729 | 1088 | 1764 | 926 | 1799 | 961 |
| 1730 | 1050 | 1765 | 897 | 1800 | 1088 |
| 1731 | 1093 | 1766 | 1069 | 1801 | 1144 |
| 1732 | 1021 | 1767 | 1078 | 1802 | 859 |
| 1733 | 1068 | 1768 | 938 | 1803 | 909 |
| 1734 | 890 | 1769 | 1156 | 1804 | 905 |
| 1735 | 894 | 1770 | 884 | 1805 | 938 |
| 1736 | 1117 | 1771 | 1071 | 1806 | 1043 |
| 1737 | 913 | 1772 | 959 | 1807 | 1110 |
| 1808 | 1237 | 1843 | 944 | 1878 | 1125 |
| 1809 | 920 | 1844 | 756 | 1879 | 857 |
| 1810 | 996 | 1845 | 869 | 1880 | 770 |
| 1811 | 917 | 1846 | 993 | 1881 | 1019 |
| 1812 | 935 | 1847 | 1025 | 1882 | 964 |
| 1813 | 1059 | 1848 | 1058 | 1883 | 1195 |
| 1814 | 975 | 1849 | 924 | 1884 | 1051 |
| 1815 | 1068 | 1850 | 1002 | 1885 | 867 |
| 1816 | 1052 | 1851 | 854 | 1886 | 998 |
| 1817 | 1072 | 1852 | 1001 | 1887 | 1212 |
| 1818 | 922 | 1853 | 974 | 1888 | 862 |
| 1819 | 760 | 1854 | 952 | 1889 | 1175 |
| 1820 | 819 | 1855 | 872 | 1890 | 1091 |
| 1821 | 991 | 1856 | 928 | 1891 | 805 |
| 1822 | 1066 | 1857 | 1093 | 1892 | 1032 |
| 1823 | 1125 | 1858 | 1174 | 1893 | 1060 |
| 1824 | 1116 | 1859 | 1017 | 1894 | 824 |
| 1825 | 1030 | 1860 | 1029 | 1895 | 764 |
| 1826 | 814 | 1861 | 1039 | 1896 | 962 |
| 1827 | 1272 | 1862 | 1017 | 1897 | 1218 |
| 1828 | 1067 | 1863 | 939 | 1898 | 843 |
| 1829 | 953 | 1864 | 1078 | 1899 | 971 |
| 1830 | 989 | 1865 | 767 | 1900 | 940 |
| 1831 | 1004 | 1866 | 957 | 1901 | 951 |
| 1832 | 1270 | 1867 | 1074 | 1902 | 1070 |
| 1833 | 1122 | 1868 | 985 | 1903 | 1145 |
| 1834 | 1255 | 1869 | 853 | 1904 | 1255 |
| 1835 | 884 | 1870 | 879 | 1905 | 1010 |
| 1836 | 1105 | 1871 | 1048 | 1906 | 941 |
| 1837 | 952 | 1872 | 902 | 1907 | 745 |
| 1838 | 1056 | 1873 | 927 | 1908 | 1031 |
| 1839 | 663 | 1874 | 1044 | 1909 | 982 |
| 1840 | 1116 | 1875 | 872 | 1910 | 944 |
| 1841 | 1232 | 1876 | 1134 | 1911 | 742 |
| 1842 | 880 | 1877 | 986 | 1912 | 898 |
| 1913 | 1234 | 1948 | 1073 |  |  |
| 1914 | 737 | 1949 | 1132 |  |  |
| 1915 | 841 | 1950 | 1100 |  |  |
| 1916 | 1228 | 1951 | 1253 |  |  |
| 1917 | 1177 | 1952 | 720 |  |  |
| 1918 | 946 | 1953 | 918 |  |  |
| 1919 | 1002 | 1954 | 1079 |  |  |
| 1920 | 918 | 1955 | 1072 |  |  |
| 1921 | 884 | 1956 | 1145 |  |  |
| 1922 | 921 | 1957 | 983 |  |  |
| 1923 | 1022 | 1958 | 1055 |  |  |
| 1924 | 933 | 1959 | 1125 |  |  |
| 1925 | 1031 | 1960 | 754 |  |  |
| 1926 | 756 | 1961 | 1219 |  |  |
| 1927 | 800 | 1962 | 937 |  |  |
| 1928 | 1211 | 1963 | 1017 |  |  |
| 1929 | 1060 | 1964 | 754 |  |  |
| 1930 | 819 | 1965 | 883 |  |  |
| 1931 | 683 | 1966 | 971 |  |  |
| 1932 | 1089 | 1967 | 969 |  |  |
| 1933 | 1054 | 1968 | 1072 |  |  |
| 1934 | 1107 | 1969 | 769 |  |  |
| 1935 | 1092 | 1970 | 861 |  |  |
| 1936 | 1080 | 1971 | 1036 |  |  |
| 1937 | 795 | 1972 | 941 |  |  |
| 1938 | 1096 | 1973 | 1185 |  |  |
| 1939 | 1394 | 1974 | 913 |  |  |
| 1940 | 928 | 1975 | 1016 |  |  |
| 1941 | 928 | 1976 | 1127 |  |  |
| 1942 | 952 | 1977 | 935 |  |  |
| 1943 | 1207 | 1978 | 745 |  |  |
| 1944 | 970 | 1979 | 986 |  |  |
| 1945 | 946 | 1980 | 1073 |  |  |
| 1946 | 980 |  |  |  |  |
| 1947 | 1010 |  |  |  |  |

### Black River Residuals

| Year | Black River Residual |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 367 | 1235 | 402 | 565 | 437 | 355 |
| 368 | 832 | 403 | 1363 | 438 | 775 |
| 369 | 929 | 404 | 989 | 439 | 852 |
| 370 | 717 | 405 | 581 | 440 | 665 |
| 371 | 752 | 406 | 641 | 441 | 1770 |
| 372 | 1427 | 407 | 509 | 442 | 803 |
| 373 | 1600 | 408 | 685 | 443 | 1177 |
| 374 | 2462 | 409 | 1113 | 444 | 582 |
| 375 | 2000 | 410 | 1372 | 445 | 527 |
| 376 | 840 | 411 | 893 | 446 | 1120 |
| 377 | 535 | 412 | 1236 | 447 | 573 |
| 378 | 710 | 413 | 603 | 448 | 1015 |
| 379 | 625 | 414 | 725 | 449 | 903 |
| 380 | 899 | 415 | 1095 | 450 | 1307 |
| 381 | 1486 | 416 | 426 | 451 | 835 |
| 382 | 1748 | 417 | 1129 | 452 | 2396 |
| 383 | 1393 | 418 | 631 | 453 | 1016 |
| 384 | 354 | 419 | 1212 | 454 | 1542 |
| 385 | 746 | 420 | 278 | 455 | 770 |
| 386 | 606 | 421 | 234 | 456 | 786 |
| 387 | 805 | 422 | 957 | 457 | 1023 |
| 388 | 850 | 423 | 1273 | 458 | 503 |
| 389 | 735 | 424 | 1852 | 459 | 480 |
| 390 | 728 | 425 | 976 | 460 | 1150 |
| 391 | 267 | 426 | 637 | 461 | 781 |
| 392 | 637 | 427 | 1056 | 462 | 857 |
| 393 | 1323 | 428 | 1356 | 463 | 1233 |
| 394 | 1874 | 429 | 1000 | 464 | 1440 |
| 395 | 1872 | 430 | 1000 | 465 | 1042 |
| 396 | 1459 | 431 | 379 | 466 | 1756 |
| 397 | 802 | 432 | 564 | 467 | 52 |
| 398 | 441 | 433 | 1138 | 468 | 966 |
| 399 | 339 | 434 | 1632 | 469 | 2976 |
| 400 | 1157 | 435 | 1186 | 470 | 725 |
| 401 | 1207 | 436 | 352 | 471 | 913 |
| 472 | 1852 | 507 | 1006 | 542 | 241 |
| 473 | 922 | 508 | 395 | 543 | 398 |
| 474 | 347 | 509 | 720 | 544 | 941 |
| 475 | 1070 | 510 | 433 | 545 | 1912 |
| 476 | 717 | 511 | 623 | 546 | 1280 |
| 477 | 946 | 512 | 1099 | 547 | 1357 |
| 478 | 826 | 513 | 1322 | 548 | 520 |
| 479 | 423 | 514 | 1572 | 549 | 1023 |
| 480 | 2262 | 515 | 910 | 550 | 421 |
| 481 | 1387 | 516 | 981 | 551 | 1738 |
| 482 | 1661 | 517 | 1593 | 552 | 347 |
| 483 | 755 | 518 | 1259 | 553 | 966 |
| 484 | 1043 | 519 | 48 | 554 | 1309 |
| 485 | 670 | 520 | 197 | 555 | 1414 |
| 486 | 795 | 521 | 780 | 556 | 71 |
| 487 | 401 | 522 | 956 | 557 | 451 |
| 488 | 531 | 523 | 967 | 558 | 1474 |
| 489 | 670 | 524 | 1879 | 559 | 394 |
| 490 | 1601 | 525 | 2091 | 560 | 668 |
| 491 | 341 | 526 | 732 | 561 | 796 |
| 492 | 610 | 527 | 310 | 562 | 599 |
| 493 | 592 | 528 | 438 | 563 | 475 |
| 494 | 1584 | 529 | 1525 | 564 | 722 |
| 495 | 1125 | 530 | 1144 | 565 | 2719 |
| 496 | 1825 | 531 | 1441 | 566 | 396 |
| 497 | 463 | 532 | 888 | 567 | 1705 |
| 498 | 1850 | 533 | 1184 | 568 | 820 |
| 499 | 1940 | 534 | 412 | 569 | 68 |
| 500 | 570 | 535 | 1803 | 570 | 1114 |
| 501 | 448 | 536 | 2641 | 571 | 620 |
| 502 | 1844 | 537 | 1197 | 572 | 1559 |
| 503 | 639 | 538 | 1237 | 573 | 847 |
| 504 | 85 | 539 | 215 | 574 | 183 |
| 505 | 662 | 540 | 877 | 575 | 942 |
| 506 | 492 | 541 | 818 | 576 | 1303 |
| 577 | 1060 | 612 | 829 | 647 | 1023 |
| 578 | 1364 | 613 | 574 | 648 | 601 |
| 579 | 1006 | 614 | 655 | 649 | 574 |
| 580 | 1786 | 615 | 1691 | 650 | 1016 |
| 581 | 1024 | 616 | 1495 | 651 | 397 |
| 582 | 366 | 617 | 1077 | 652 | 1232 |
| 583 | 1387 | 618 | 1128 | 653 | 928 |
| 584 | 1037 | 619 | 552 | 654 | 2196 |
| 585 | 286 | 620 | 796 | 655 | 448 |
| 586 | 1601 | 621 | 1794 | 656 | 1802 |
| 587 | 534 | 622 | 1043 | 657 | 378 |
| 588 | 329 | 623 | 874 | 658 | 618 |
| 589 | 624 | 624 | 1599 | 659 | 746 |
| 590 | 523 | 625 | 1070 | 660 | 1991 |
| 591 | 1148 | 626 | 1206 | 661 | 2372 |
| 592 | 1389 | 627 | 1610 | 662 | 675 |
| 593 | 1700 | 628 | 161 | 663 | 2101 |
| 594 | 913 | 629 | 390 | 664 | 540 |
| 595 | 467 | 630 | 664 | 665 | 196 |
| 596 | 2315 | 631 | 2165 | 666 | 526 |
| 597 | 1419 | 632 | 672 | 667 | 354 |
| 598 | 1713 | 633 | 137 | 668 | 2628 |
| 599 | 957 | 634 | 607 | 669 | 1499 |
| 600 | 1172 | 635 | 631 | 670 | 977 |
| 601 | 900 | 636 | 1234 | 671 | 621 |
| 602 | 496 | 637 | 832 | 672 | 548 |
| 603 | 1254 | 638 | 1515 | 673 | 1083 |
| 604 | 409 | 639 | 1338 | 674 | 1191 |
| 605 | 398 | 640 | 1931 | 675 | 1134 |
| 606 | 853 | 641 | 866 | 676 | 1544 |
| 607 | 853 | 642 | 700 | 677 | 818 |
| 608 | 737 | 643 | 1230 | 678 | 472 |
| 609 | 1049 | 644 | 482 | 679 | 760 |
| 610 | 818 | 645 | 1098 | 680 | 881 |
| 611 | 1057 | 646 | 470 | 681 | 584 |
| 682 | 558 | 717 | 299 | 752 | 1146 |
| 683 | 972 | 718 | 683 | 753 | 1862 |
| 684 | 594 | 719 | 642 | 754 | 893 |
| 685 | 968 | 720 | 723 | 755 | -101 |
| 686 | 548 | 721 | 960 | 756 | 426 |
| 687 | 577 | 722 | 1023 | 757 | 598 |
| 688 | 436 | 723 | 1059 | 758 | 946 |
| 689 | 564 | 724 | 1386 | 759 | 1244 |
| 690 | 728 | 725 | 3031 | 760 | 605 |
| 691 | 1721 | 726 | 1259 | 761 | 1058 |
| 692 | 1835 | 727 | 2346 | 762 | 892 |
| 693 | 723 | 728 | 1573 | 763 | 1085 |
| 694 | 728 | 729 | 442 | 764 | 1378 |
| 695 | 2046 | 730 | 1356 | 765 | 534 |
| 696 | 621 | 731 | 1043 | 766 | 1530 |
| 697 | 287 | 732 | 1148 | 767 | 1599 |
| 698 | 292 | 733 | 788 | 768 | 848 |
| 699 | 930 | 734 | 916 | 769 | 1644 |
| 700 | 1778 | 735 | 395 | 770 | 966 |
| 701 | 490 | 736 | 1767 | 771 | 938 |
| 702 | 911 | 737 | 660 | 772 | 1002 |
| 703 | 416 | 738 | 1164 | 773 | 891 |
| 704 | 459 | 739 | 1797 | 774 | 466 |
| 705 | 683 | 740 | 1440 | 775 | 883 |
| 706 | 711 | 741 | 1132 | 776 | 1070 |
| 707 | 1100 | 742 | 844 | 777 | 1030 |
| 708 | 1423 | 743 | 1172 | 778 | 950 |
| 709 | 871 | 744 | 900 | 779 | 232 |
| 710 | 697 | 745 | 1178 | 780 | 802 |
| 711 | 481 | 746 | 966 | 781 | 1109 |
| 712 | 872 | 747 | 474 | 782 | 787 |
| 713 | 469 | 748 | 597 | 783 | 1806 |
| 714 | 665 | 749 | 585 | 784 | 361 |
| 715 | 1032 | 750 | 1110 | 785 | 1951 |
| 716 | 444 | 751 | 1023 | 786 | 1722 |
| 787 | 459 | 822 | 1724 | 857 | 442 |
| 788 | 2739 | 823 | 474 | 858 | 326 |
| 789 | 398 | 824 | 253 | 859 | 521 |
| 790 | 1256 | 825 | 476 | 860 | 1582 |
| 791 | 865 | 826 | 976 | 861 | 697 |
| 792 | 270 | 827 | 754 | 862 | 691 |
| 793 | 926 | 828 | 617 | 863 | 2734 |
| 794 | 1104 | 829 | 1002 | 864 | 1495 |
| 795 | 645 | 830 | 538 | 865 | 398 |
| 796 | 1269 | 831 | 1041 | 866 | 1605 |
| 797 | 716 | 832 | 1156 | 867 | 659 |
| 798 | 1087 | 833 | 805 | 868 | 1483 |
| 799 | 1165 | 834 | 1176 | 869 | 818 |
| 800 | 638 | 835 | 2077 | 870 | 458 |
| 801 | 700 | 836 | 1026 | 871 | 864 |
| 802 | 1378 | 837 | 1759 | 872 | 428 |
| 803 | 772 | 838 | 945 | 873 | 534 |
| 804 | 1992 | 839 | 637 | 874 | 1771 |
| 805 | 396 | 840 | 1399 | 875 | 584 |
| 806 | 352 | 841 | 1180 | 876 | 813 |
| 807 | 873 | 842 | 1291 | 877 | 419 |
| 808 | 1156 | 843 | 588 | 878 | 808 |
| 809 | 224 | 844 | 2560 | 879 | 824 |
| 810 | 442 | 845 | 1220 | 880 | 1174 |
| 811 | 645 | 846 | 1327 | 881 | 453 |
| 812 | 916 | 847 | 413 | 882 | 594 |
| 813 | 721 | 848 | 1318 | 883 | 1339 |
| 814 | 503 | 849 | 240 | 884 | 816 |
| 815 | 763 | 850 | 834 | 885 | 1021 |
| 816 | 769 | 851 | 915 | 886 | 2155 |
| 817 | 2191 | 852 | 768 | 887 | 1292 |
| 818 | 1234 | 853 | 799 | 888 | 103 |
| 819 | 1612 | 854 | 1061 | 889 | 1730 |
| 820 | 111 | 855 | 669 | 890 | 1394 |
| 821 | 991 | 856 | 940 | 891 | 1740 |
| 892 | 437 | 927 | 1577 | 962 | 1075 |
| 893 | 1109 | 928 | 328 | 963 | 1151 |
| 894 | 727 | 929 | 459 | 964 | 1379 |
| 895 | 768 | 930 | 584 | 965 | 1206 |
| 896 | 1134 | 931 | 1091 | 966 | 1769 |
| 897 | 422 | 932 | 1188 | 967 | 1502 |
| 898 | 756 | 933 | 1297 | 968 | 1689 |
| 899 | 750 | 934 | 290 | 969 | 656 |
| 900 | 548 | 935 | 2149 | 970 | 801 |
| 901 | 1174 | 936 | 496 | 971 | 1204 |
| 902 | 803 | 937 | 903 | 972 | 1019 |
| 903 | 1491 | 938 | 1138 | 973 | 726 |
| 904 | 966 | 939 | 911 | 974 | 2039 |
| 905 | 2353 | 940 | 1744 | 975 | 698 |
| 906 | 877 | 941 | 281 | 976 | 513 |
| 907 | 685 | 942 | 757 | 977 | 1044 |
| 908 | 661 | 943 | 977 | 978 | 620 |
| 909 | 1575 | 944 | 1311 | 979 | 1203 |
| 910 | 506 | 945 | 1105 | 980 | 943 |
| 911 | 913 | 946 | 687 | 981 | 593 |
| 912 | 1114 | 947 | 925 | 982 | 512 |
| 913 | 739 | 948 | 884 | 983 | 635 |
| 914 | 1349 | 949 | 1209 | 984 | 1476 |
| 915 | 575 | 950 | 389 | 985 | 355 |
| 916 | 1514 | 951 | 354 | 986 | 682 |
| 917 | 715 | 952 | 662 | 987 | 807 |
| 918 | 551 | 953 | 811 | 988 | 868 |
| 919 | 1289 | 954 | 1212 | 989 | 798 |
| 920 | 1466 | 955 | 1351 | 990 | 309 |
| 921 | 616 | 956 | 366 | 991 | 570 |
| 922 | 1225 | 957 | 1981 | 992 | 1857 |
| 923 | 1075 | 958 | 426 | 993 | 1460 |
| 924 | 839 | 959 | 894 | 994 | 789 |
| 925 | 814 | 960 | 506 | 995 | 1018 |
| 926 | 1310 | 961 | 988 | 996 | 1545 |
| 997 | 999 | 1032 | 1447 | 1067 | 876 |
| 998 | 1909 | 1033 | 695 | 1068 | 1060 |
| 999 | 1057 | 1034 | 923 | 1069 | 1147 |
| 1000 | 295 | 1035 | 1028 | 1070 | 2155 |
| 1001 | 1231 | 1036 | 1075 | 1071 | 489 |
| 1002 | 1628 | 1037 | 834 | 1072 | 517 |
| 1003 | 780 | 1038 | 780 | 1073 | 707 |
| 1004 | 1477 | 1039 | 1118 | 1074 | 1067 |
| 1005 | 945 | 1040 | 1982 | 1075 | 1887 |
| 1006 | 762 | 1041 | 546 | 1076 | 870 |
| 1007 | 843 | 1042 | 1777 | 1077 | 1418 |
| 1008 | 1291 | 1043 | 792 | 1078 | 527 |
| 1009 | 992 | 1044 | 651 | 1079 | 347 |
| 1010 | 1035 | 1045 | 243 | 1080 | 895 |
| 1011 | 680 | 1046 | 1349 | 1081 | 2105 |
| 1012 | 975 | 1047 | 562 | 1082 | 600 |
| 1013 | 1102 | 1048 | 144 | 1083 | 1094 |
| 1014 | 617 | 1049 | 1204 | 1084 | 610 |
| 1015 | 766 | 1050 | 630 | 1085 | 932 |
| 1016 | 1449 | 1051 | 914 | 1086 | 260 |
| 1017 | 512 | 1052 | 574 | 1087 | 1912 |
| 1018 | 903 | 1053 | 2149 | 1088 | 220 |
| 1019 | 605 | 1054 | 553 | 1089 | 406 |
| 1020 | 818 | 1055 | 812 | 1090 | 385 |
| 1021 | 1266 | 1056 | 950 | 1091 | 1156 |
| 1022 | 1426 | 1057 | 696 | 1092 | 721 |
| 1023 | 689 | 1058 | 1261 | 1093 | 317 |
| 1024 | 1830 | 1059 | 1061 | 1094 | 552 |
| 1025 | 742 | 1060 | 999 | 1095 | 1088 |
| 1026 | 768 | 1061 | 668 | 1096 | 1182 |
| 1027 | 720 | 1062 | 718 | 1097 | 877 |
| 1028 | 498 | 1063 | 463 | 1098 | 421 |
| 1029 | 1129 | 1064 | 1399 | 1099 | 1038 |
| 1030 | 689 | 1065 | 1282 | 1100 | 1943 |
| 1031 | 1813 | 1066 | 1752 | 1101 | 1154 |
| 1102 | 926 | 1137 | 1101 | 1172 | 1377 |
| 1103 | 1067 | 1138 | 1152 | 1173 | 1202 |
| 1104 | 880 | 1139 | 565 | 1174 | 2056 |
| 1105 | 945 | 1140 | 1302 | 1175 | 542 |
| 1106 | 1103 | 1141 | 1245 | 1176 | 1375 |
| 1107 | 950 | 1142 | 416 | 1177 | 543 |
| 1108 | 1452 | 1143 | 804 | 1178 | 2342 |
| 1109 | 628 | 1144 | 610 | 1179 | 291 |
| 1110 | 495 | 1145 | 1286 | 1180 | 675 |
| 1111 | 869 | 1146 | 582 | 1181 | 851 |
| 1112 | 1291 | 1147 | 779 | 1182 | 567 |
| 1113 | 1013 | 1148 | 1106 | 1183 | 1620 |
| 1114 | 798 | 1149 | 340 | 1184 | 1260 |
| 1115 | 1539 | 1150 | 1996 | 1185 | 528 |
| 1116 | 1172 | 1151 | 457 | 1186 | 1432 |
| 1117 | 989 | 1152 | 775 | 1187 | 1532 |
| 1118 | 1098 | 1153 | 1297 | 1188 | 879 |
| 1119 | 1378 | 1154 | 405 | 1189 | 1075 |
| 1120 | 1274 | 1155 | 1038 | 1190 | 1113 |
| 1121 | 1478 | 1156 | 1303 | 1191 | 659 |
| 1122 | 1455 | 1157 | 636 | 1192 | 1100 |
| 1123 | 533 | 1158 | 515 | 1193 | 1020 |
| 1124 | 171 | 1159 | 1090 | 1194 | 1082 |
| 1125 | 764 | 1160 | 782 | 1195 | 923 |
| 1126 | 895 | 1161 | 710 | 1196 | 1781 |
| 1127 | 1134 | 1162 | 1144 | 1197 | 436 |
| 1128 | 1998 | 1163 | 471 | 1198 | 547 |
| 1129 | 519 | 1164 | 719 | 1199 | 691 |
| 1130 | 660 | 1165 | 446 | 1200 | 1249 |
| 1131 | 1052 | 1166 | 937 | 1201 | 584 |
| 1132 | 846 | 1167 | 1470 | 1202 | 561 |
| 1133 | 1400 | 1168 | 1488 | 1203 | 852 |
| 1134 | 2904 | 1169 | 298 | 1204 | 666 |
| 1135 | 632 | 1170 | 1567 | 1205 | 798 |
| 1136 | 331 | 1171 | 1288 | 1206 | 1052 |
| 1207 | 211 | 1242 | 613 | 1277 | 1509 |
| 1208 | 834 | 1243 | 689 | 1278 | 670 |
| 1209 | 1019 | 1244 | 1472 | 1279 | 502 |
| 1210 | 1141 | 1245 | 1394 | 1280 | 940 |
| 1211 | 518 | 1246 | 555 | 1281 | 972 |
| 1212 | 1240 | 1247 | 732 | 1282 | 652 |
| 1213 | 1253 | 1248 | 564 | 1283 | 771 |
| 1214 | 731 | 1249 | 1683 | 1284 | 504 |
| 1215 | 414 | 1250 | 620 | 1285 | 789 |
| 1216 | 720 | 1251 | 492 | 1286 | 1345 |
| 1217 | 541 | 1252 | 882 | 1287 | 720 |
| 1218 | 1274 | 1253 | 1248 | 1288 | 709 |
| 1219 | 1437 | 1254 | 671 | 1289 | 756 |
| 1220 | 1286 | 1255 | 1093 | 1290 | 2055 |
| 1221 | 938 | 1256 | 1180 | 1291 | 1567 |
| 1222 | 1178 | 1257 | 418 | 1292 | 1202 |
| 1223 | 164 | 1258 | 854 | 1293 | 1135 |
| 1224 | 1938 | 1259 | 678 | 1294 | 462 |
| 1225 | 1414 | 1260 | 757 | 1295 | 2181 |
| 1226 | 1441 | 1261 | 1245 | 1296 | 455 |
| 1227 | 221 | 1262 | 613 | 1297 | 658 |
| 1228 | 1144 | 1263 | 510 | 1298 | 1397 |
| 1229 | 728 | 1264 | 776 | 1299 | 1398 |
| 1230 | 1418 | 1265 | 1332 | 1300 | 920 |
| 1231 | 521 | 1266 | 806 | 1301 | 1346 |
| 1232 | 1245 | 1267 | 802 | 1302 | 1034 |
| 1233 | 931 | 1268 | 445 | 1303 | 470 |
| 1234 | 986 | 1269 | 960 | 1304 | 625 |
| 1235 | 1354 | 1270 | 730 | 1305 | 511 |
| 1236 | 721 | 1271 | 581 | 1306 | 1913 |
| 1237 | 1171 | 1272 | 1021 | 1307 | 1257 |
| 1238 | 1038 | 1273 | 1678 | 1308 | 1444 |
| 1239 | 1670 | 1274 | 1289 | 1309 | 1457 |
| 1240 | 1396 | 1275 | 1153 | 1310 | 1363 |
| 1241 | 2100 | 1276 | 803 | 1311 | 232 |
| 1312 | 548 | 1347 | 440 | 1382 | 443 |
| 1313 | 1038 | 1348 | 565 | 1383 | 1037 |
| 1314 | 1218 | 1349 | 830 | 1384 | 933 |
| 1315 | 419 | 1350 | 1131 | 1385 | 401 |
| 1316 | 853 | 1351 | 762 | 1386 | 1090 |
| 1317 | 1135 | 1352 | 683 | 1387 | 2420 |
| 1318 | 563 | 1353 | 1186 | 1388 | 755 |
| 1319 | 1286 | 1354 | 576 | 1389 | 358 |
| 1320 | 1284 | 1355 | 1299 | 1390 | 1019 |
| 1321 | 1215 | 1356 | 502 | 1391 | 1147 |
| 1322 | 697 | 1357 | 833 | 1392 | 693 |
| 1323 | 909 | 1358 | 1199 | 1393 | 1160 |
| 1324 | 463 | 1359 | 449 | 1394 | 1698 |
| 1325 | 1025 | 1360 | 803 | 1395 | 168 |
| 1326 | 562 | 1361 | 914 | 1396 | 1122 |
| 1327 | 1546 | 1362 | 1136 | 1397 | 1363 |
| 1328 | 2066 | 1363 | 1850 | 1398 | 929 |
| 1329 | 902 | 1364 | 526 | 1399 | 1120 |
| 1330 | 915 | 1365 | 1192 | 1400 | 1634 |
| 1331 | 1024 | 1366 | 1129 | 1401 | 513 |
| 1332 | 1141 | 1367 | 1300 | 1402 | 381 |
| 1333 | 854 | 1368 | 1505 | 1403 | 1426 |
| 1334 | 1002 | 1369 | 841 | 1404 | 1103 |
| 1335 | 762 | 1370 | 1246 | 1405 | 998 |
| 1336 | 440 | 1371 | 2278 | 1406 | 1529 |
| 1337 | 654 | 1372 | 277 | 1407 | 325 |
| 1338 | 805 | 1373 | 612 | 1408 | 1517 |
| 1339 | 964 | 1374 | 726 | 1409 | 553 |
| 1340 | 1158 | 1375 | 978 | 1410 | 816 |
| 1341 | 1440 | 1376 | 1005 | 1411 | 979 |
| 1342 | 444 | 1377 | 945 | 1412 | 168 |
| 1343 | 1765 | 1378 | 886 | 1413 | 1178 |
| 1344 | 1132 | 1379 | 546 | 1414 | 912 |
| 1345 | 1656 | 1380 | 1025 | 1415 | 873 |
| 1346 | 1336 | 1381 | 1182 | 1416 | 1115 |
| 1417 | 1432 | 1452 | 1165 | 1487 | 1114 |
| 1418 | 1693 | 1453 | 613 | 1488 | 1143 |
| 1419 | 970 | 1454 | 345 | 1489 | 560 |
| 1420 | 835 | 1455 | 507 | 1490 | 711 |
| 1421 | 392 | 1456 | 895 | 1491 | 494 |
| 1422 | 1464 | 1457 | 1850 | 1492 | 946 |
| 1423 | 976 | 1458 | 1120 | 1493 | 1988 |
| 1424 | 969 | 1459 | 904 | 1494 | 774 |
| 1425 | 1283 | 1460 | 1051 | 1495 | 1250 |
| 1426 | 580 | 1461 | 921 | 1496 | 795 |
| 1427 | 490 | 1462 | 1721 | 1497 | 1428 |
| 1428 | 1564 | 1463 | 637 | 1498 | 1543 |
| 1429 | 2022 | 1464 | 809 | 1499 | 794 |
| 1430 | 557 | 1465 | 672 | 1500 | 676 |
| 1431 | 311 | 1466 | 1221 | 1501 | 690 |
| 1432 | 763 | 1467 | 1860 | 1502 | 856 |
| 1433 | 548 | 1468 | 1052 | 1503 | 1144 |
| 1434 | 932 | 1469 | 688 | 1504 | 1585 |
| 1435 | 1084 | 1470 | 1142 | 1505 | 688 |
| 1436 | 780 | 1471 | 576 | 1506 | 666 |
| 1437 | 1116 | 1472 | 732 | 1507 | 1272 |
| 1438 | 779 | 1473 | 726 | 1508 | 1640 |
| 1439 | 1646 | 1474 | 615 | 1509 | 378 |
| 1440 | 342 | 1475 | 616 | 1510 | 826 |
| 1441 | 1492 | 1476 | 1553 | 1511 | 1264 |
| 1442 | 513 | 1477 | 1678 | 1512 | 1331 |
| 1443 | 1085 | 1478 | 757 | 1513 | 1228 |
| 1444 | 1082 | 1479 | 1021 | 1514 | 628 |
| 1445 | 461 | 1480 | 1335 | 1515 | 684 |
| 1446 | 1410 | 1481 | 1113 | 1516 | 1140 |
| 1447 | 485 | 1482 | 890 | 1517 | 1154 |
| 1448 | 1175 | 1483 | 823 | 1518 | 285 |
| 1449 | 856 | 1484 | 888 | 1519 | 1147 |
| 1450 | 1270 | 1485 | 2267 | 1520 | 1254 |
| 1451 | 411 | 1486 | 447 | 1521 | 528 |
| 1522 | 644 | 1557 | 193 | 1592 | 1008 |
| 1523 | 470 | 1558 | 1462 | 1593 | 590 |
| 1524 | 1075 | 1559 | 1040 | 1594 | 1011 |
| 1525 | 1102 | 1560 | 772 | 1595 | 618 |
| 1526 | 1121 | 1561 | 1329 | 1596 | 1252 |
| 1527 | 764 | 1562 | 1183 | 1597 | 452 |
| 1528 | 563 | 1563 | 794 | 1598 | 600 |
| 1529 | 2163 | 1564 | 761 | 1599 | 655 |
| 1530 | 1137 | 1565 | 1223 | 1600 | 2007 |
| 1531 | 1009 | 1566 | 491 | 1601 | 914 |
| 1532 | 1783 | 1567 | 821 | 1602 | 1469 |
| 1533 | 656 | 1568 | 1356 | 1603 | 1748 |
| 1534 | 1001 | 1569 | 489 | 1604 | 581 |
| 1535 | 726 | 1570 | 708 | 1605 | 1622 |
| 1536 | 763 | 1571 | 921 | 1606 | 1787 |
| 1537 | 1077 | 1572 | 1216 | 1607 | 624 |
| 1538 | 816 | 1573 | 857 | 1608 | 750 |
| 1539 | 972 | 1574 | 492 | 1609 | 1346 |
| 1540 | 1005 | 1575 | 760 | 1610 | 444 |
| 1541 | 1655 | 1576 | 1120 | 1611 | 635 |
| 1542 | 458 | 1577 | 1224 | 1612 | 433 |
| 1543 | 1061 | 1578 | 1204 | 1613 | 2246 |
| 1544 | 616 | 1579 | 1871 | 1614 | 521 |
| 1545 | 1132 | 1580 | 1362 | 1615 | 1694 |
| 1546 | 514 | 1581 | 691 | 1616 | 890 |
| 1547 | 888 | 1582 | 1542 | 1617 | 1199 |
| 1548 | 929 | 1583 | 816 | 1618 | 963 |
| 1549 | 699 | 1584 | 1273 | 1619 | 653 |
| 1550 | 981 | 1585 | 807 | 1620 | 583 |
| 1551 | 1019 | 1586 | 980 | 1621 | 480 |
| 1552 | 514 | 1587 | 299 | 1622 | 597 |
| 1553 | 1845 | 1588 | 821 | 1623 | 1017 |
| 1554 | 1540 | 1589 | 1018 | 1624 | 1251 |
| 1555 | 859 | 1590 | 1324 | 1625 | 939 |
| 1556 | 995 | 1591 | 1065 | 1626 | 1391 |
| 1627 | 1164 | 1662 | 1463 | 1697 | 1268 |
| 1628 | 883 | 1663 | 620 | 1698 | 1276 |
| 1629 | 916 | 1664 | 524 | 1699 | 1254 |
| 1630 | 821 | 1665 | 1767 | 1700 | 512 |
| 1631 | 911 | 1666 | 478 | 1701 | 1006 |
| 1632 | 553 | 1667 | 633 | 1702 | 1018 |
| 1633 | 524 | 1668 | 807 | 1703 | 826 |
| 1634 | 1043 | 1669 | 894 | 1704 | 732 |
| 1635 | 2063 | 1670 | 470 | 1705 | 1589 |
| 1636 | 1412 | 1671 | 1409 | 1706 | 1009 |
| 1637 | 637 | 1672 | 958 | 1707 | 852 |
| 1638 | 385 | 1673 | 1025 | 1708 | 391 |
| 1639 | 1064 | 1674 | 838 | 1709 | 651 |
| 1640 | 840 | 1675 | 774 | 1710 | 1249 |
| 1641 | 807 | 1676 | 578 | 1711 | 809 |
| 1642 | 961 | 1677 | 1014 | 1712 | 1283 |
| 1643 | 344 | 1678 | 1465 | 1713 | 1251 |
| 1644 | 865 | 1679 | 973 | 1714 | 302 |
| 1645 | 752 | 1680 | 1698 | 1715 | 454 |
| 1646 | 1426 | 1681 | 1103 | 1716 | 1682 |
| 1647 | 1159 | 1682 | 1065 | 1717 | 1379 |
| 1648 | 1038 | 1683 | 1445 | 1718 | 600 |
| 1649 | 2057 | 1684 | 1170 | 1719 | 1328 |
| 1650 | 1520 | 1685 | 1165 | 1720 | 1049 |
| 1651 | 1383 | 1686 | 929 | 1721 | 2131 |
| 1652 | 256 | 1687 | 830 | 1722 | 348 |
| 1653 | 1467 | 1688 | 961 | 1723 | 2292 |
| 1654 | 674 | 1689 | 405 | 1724 | 1397 |
| 1655 | 1800 | 1690 | 555 | 1725 | 792 |
| 1656 | 716 | 1691 | 863 | 1726 | 1016 |
| 1657 | 920 | 1692 | 1254 | 1727 | 736 |
| 1658 | 935 | 1693 | 406 | 1728 | 243 |
| 1659 | 274 | 1694 | 1072 | 1729 | 1659 |
| 1660 | 463 | 1695 | 1272 | 1730 | 486 |
| 1661 | 1273 | 1696 | 824 | 1731 | 1050 |
| 1732 | 740 | 1767 | 732 | 1802 | 537 |
| 1733 | 492 | 1768 | 1297 | 1803 | 1563 |
| 1734 | 889 | 1769 | 1026 | 1804 | 1495 |
| 1735 | 1008 | 1770 | 976 | 1805 | 479 |
| 1736 | 2493 | 1771 | 1311 | 1806 | 844 |
| 1737 | 756 | 1772 | 819 | 1807 | 887 |
| 1738 | 119 | 1773 | 1002 | 1808 | 1016 |
| 1739 | 485 | 1774 | 764 | 1809 | 890 |
| 1740 | 1340 | 1775 | 1061 | 1810 | 1031 |
| 1741 | 1067 | 1776 | 1121 | 1811 | 840 |
| 1742 | 1199 | 1777 | 902 | 1812 | 1145 |
| 1743 | 797 | 1778 | 291 | 1813 | 715 |
| 1744 | 960 | 1779 | 735 | 1814 | 1208 |
| 1745 | 1474 | 1780 | 1548 | 1815 | 704 |
| 1746 | 571 | 1781 | 421 | 1816 | 475 |
| 1747 | 853 | 1782 | 721 | 1817 | 1408 |
| 1748 | 545 | 1783 | 1092 | 1818 | 504 |
| 1749 | 885 | 1784 | 544 | 1819 | 710 |
| 1750 | 961 | 1785 | 1202 | 1820 | 669 |
| 1751 | 1101 | 1786 | 1651 | 1821 | 2008 |
| 1752 | 830 | 1787 | 843 | 1822 | 994 |
| 1753 | 1226 | 1788 | 866 | 1823 | 827 |
| 1754 | 1011 | 1789 | 798 | 1824 | 710 |
| 1755 | 960 | 1790 | 1314 | 1825 | 1825 |
| 1756 | 918 | 1791 | 895 | 1826 | 358 |
| 1757 | 1506 | 1792 | 1118 | 1827 | 812 |
| 1758 | 641 | 1793 | 1512 | 1828 | 625 |
| 1759 | 563 | 1794 | 399 | 1829 | 1670 |
| 1760 | 628 | 1795 | 1704 | 1830 | 549 |
| 1761 | 1987 | 1796 | 1899 | 1831 | 585 |
| 1762 | 756 | 1797 | 414 | 1832 | 1132 |
| 1763 | 723 | 1798 | 1876 | 1833 | 1574 |
| 1764 | 744 | 1799 | 435 | 1834 | 1076 |
| 1765 | 553 | 1800 | 1608 | 1835 | 702 |
| 1766 | 1446 | 1801 | 730 | 1836 | 1852 |
| 1837 | 323 | 1872 | 913 | 1907 | 1527 |
| 1838 | 1215 | 1873 | 1245 | 1908 | 815 |
| 1839 | 660 | 1874 | 945 | 1909 | 1510 |
| 1840 | 1247 | 1875 | 1466 | 1910 | 954 |
| 1841 | 944 | 1876 | 661 | 1911 | 390 |
| 1842 | 789 | 1877 | 792 | 1912 | 1266 |
| 1843 | 1047 | 1878 | 758 | 1913 | 922 |
| 1844 | 734 | 1879 | 926 | 1914 | 424 |
| 1845 | 501 | 1880 | 477 | 1915 | 1185 |
| 1846 | 1742 | 1881 | 773 | 1916 | 884 |
| 1847 | 1195 | 1882 | 891 | 1917 | 1301 |
| 1848 | 755 | 1883 | 1481 | 1918 | 930 |
| 1849 | 107 | 1884 | 1388 | 1919 | 970 |
| 1850 | 1112 | 1885 | 1204 | 1920 | 824 |
| 1851 | 776 | 1886 | 1166 | 1921 | 1316 |
| 1852 | 809 | 1887 | 572 | 1922 | 1510 |
| 1853 | 895 | 1888 | 1695 | 1923 | 869 |
| 1854 | 1791 | 1889 | 1094 | 1924 | 1408 |
| 1855 | 1496 | 1890 | 329 | 1925 | 535 |
| 1856 | 352 | 1891 | 1357 | 1926 | 475 |
| 1857 | 912 | 1892 | 774 | 1927 | 498 |
| 1858 | 1174 | 1893 | 942 | 1928 | 1395 |
| 1859 | 950 | 1894 | 363 | 1929 | 1573 |
| 1860 | 1341 | 1895 | 1450 | 1930 | 877 |
| 1861 | 746 | 1896 | 992 | 1931 | 691 |
| 1862 | 1520 | 1897 | 643 | 1932 | 782 |
| 1863 | 855 | 1898 | 817 | 1933 | 599 |
| 1864 | 537 | 1899 | 1484 | 1934 | 977 |
| 1865 | 786 | 1900 | 1173 | 1935 | 610 |
| 1866 | 1640 | 1901 | 1444 | 1936 | 1034 |
| 1867 | 949 | 1902 | 339 | 1937 | 782 |
| 1868 | 1335 | 1903 | 844 | 1938 | 1194 |
| 1869 | 579 | 1904 | 700 | 1939 | 970 |
| 1870 | 1435 | 1905 | 1370 | 1940 | 869 |
| 1871 | 800 | 1906 | 927 | 1941 | 719 |
| 1942 | 837 | 1977 | 741 |  |  |
| 1943 | 834 | 1978 | 1288 |  |  |
| 1944 | 956 | 1979 | 604 |  |  |
| 1945 | 735 | 1980 | 678 |  |  |
| 1946 | 1929 |  |  |  |  |
| 1947 | 784 |  |  |  |  |
| 1948 | 1109 |  |  |  |  |
| 1949 | 1468 |  |  |  |  |
| 1950 | 806 |  |  |  |  |
| 1951 | 726 |  |  |  |  |
| 1952 | 922 |  |  |  |  |
| 1953 | 830 |  |  |  |  |
| 1954 | 681 |  |  |  |  |
| 1955 | 402 |  |  |  |  |
| 1956 | 1263 |  |  |  |  |
| 1957 | 1584 |  |  |  |  |
| 1958 | 1340 |  |  |  |  |
| 1959 | 1149 |  |  |  |  |
| 1960 | 920 |  |  |  |  |
| 1961 | 1273 |  |  |  |  |
| 1962 | 1249 |  |  |  |  |
| 1963 | 923 |  |  |  |  |
| 1964 | 932 |  |  |  |  |
| 1965 | 1504 |  |  |  |  |
| 1966 | 668 |  |  |  |  |
| 1967 | 880 |  |  |  |  |
| 1968 | 879 |  |  |  |  |
| 1969 | 1339 |  |  |  |  |
| 1970 | 585 |  |  |  |  |
| 1971 | 796 |  |  |  |  |
| 1972 | 1291 |  |  |  |  |
| 1973 | 1293 |  |  |  |  |
| 1974 | 1040 |  |  |  |  |
| 1975 | 928 |  |  |  |  |
| 1976 | 1235 |  |  |  |  |

### Grandfather Mountain Residuals

| Year | Grandfather Mountain Residual |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1563 | 948 | 1598 | 802 | 1633 | 1098 |
| 1564 | 296 | 1599 | 843 | 1634 | 984 |
| 1565 | 1308 | 1600 | 1108 | 1635 | 712 |
| 1566 | 747 | 1601 | 864 | 1636 | 980 |
| 1567 | 1273 | 1602 | 875 | 1637 | 1150 |
| 1568 | 1286 | 1603 | 1314 | 1638 | 640 |
| 1569 | 649 | 1604 | 1297 | 1639 | 1422 |
| 1570 | 1358 | 1605 | 1250 | 1640 | 892 |
| 1571 | 594 | 1606 | 838 | 1641 | 1283 |
| 1572 | 1226 | 1607 | 1141 | 1642 | 887 |
| 1573 | 1608 | 1608 | 496 | 1643 | 723 |
| 1574 | 519 | 1609 | 859 | 1644 | 1056 |
| 1575 | 810 | 1610 | 1083 | 1645 | 958 |
| 1576 | 1022 | 1611 | 1028 | 1646 | 1253 |
| 1577 | 855 | 1612 | 824 | 1647 | 973 |
| 1578 | 981 | 1613 | 769 | 1648 | 755 |
| 1579 | 682 | 1614 | 1074 | 1649 | 775 |
| 1580 | 652 | 1615 | 750 | 1650 | 1048 |
| 1581 | 871 | 1616 | 851 | 1651 | 908 |
| 1582 | 982 | 1617 | 978 | 1652 | 1008 |
| 1583 | 903 | 1618 | 945 | 1653 | 1063 |
| 1584 | 796 | 1619 | 1169 | 1654 | 987 |
| 1585 | 946 | 1620 | 1252 | 1655 | 686 |
| 1586 | 767 | 1621 | 988 | 1656 | 1307 |
| 1587 | 831 | 1622 | 874 | 1657 | 1255 |
| 1588 | 937 | 1623 | 1177 | 1658 | 1645 |
| 1589 | 1012 | 1624 | 1147 | 1659 | 975 |
| 1590 | 1500 | 1625 | 1063 | 1660 | 976 |
| 1591 | 1263 | 1626 | 1172 | 1661 | 914 |
| 1592 | 1444 | 1627 | 1003 | 1662 | 1157 |
| 1593 | 1539 | 1628 | 1200 | 1663 | 678 |
| 1594 | 435 | 1629 | 941 | 1664 | 991 |
| 1595 | 717 | 1630 | 1155 | 1665 | 808 |
| 1596 | 878 | 1631 | 1402 | 1666 | 1222 |
| 1597 | 894 | 1632 | 938 | 1667 | 931 |
| 1668 | 1010 | 1703 | 1163 | 1738 | 942 |
| 1669 | 886 | 1704 | 940 | 1739 | 1079 |
| 1670 | 1185 | 1705 | 1219 | 1740 | 935 |
| 1671 | 785 | 1706 | 1159 | 1741 | 866 |
| 1672 | 1104 | 1707 | 775 | 1742 | 1086 |
| 1673 | 1131 | 1708 | 773 | 1743 | 795 |
| 1674 | 845 | 1709 | 928 | 1744 | 977 |
| 1675 | 948 | 1710 | 1022 | 1745 | 1089 |
| 1676 | 1103 | 1711 | 1058 | 1746 | 881 |
| 1677 | 987 | 1712 | 934 | 1747 | 953 |
| 1678 | 808 | 1713 | 984 | 1748 | 749 |
| 1679 | 968 | 1714 | 818 | 1749 | 1134 |
| 1680 | 1095 | 1715 | 836 | 1750 | 987 |
| 1681 | 1074 | 1716 | 1133 | 1751 | 995 |
| 1682 | 1054 | 1717 | 1137 | 1752 | 1071 |
| 1683 | 1214 | 1718 | 1097 | 1753 | 966 |
| 1684 | 874 | 1719 | 1101 | 1754 | 1035 |
| 1685 | 699 | 1720 | 848 | 1755 | 1070 |
| 1686 | 1205 | 1721 | 913 | 1756 | 1039 |
| 1687 | 824 | 1722 | 1206 | 1757 | 882 |
| 1688 | 878 | 1723 | 999 | 1758 | 947 |
| 1689 | 1194 | 1724 | 869 | 1759 | 1151 |
| 1690 | 829 | 1725 | 1029 | 1760 | 773 |
| 1691 | 915 | 1726 | 1012 | 1761 | 878 |
| 1692 | 789 | 1727 | 1156 | 1762 | 968 |
| 1693 | 967 | 1728 | 1029 | 1763 | 1095 |
| 1694 | 931 | 1729 | 1206 | 1764 | 1050 |
| 1695 | 922 | 1730 | 866 | 1765 | 966 |
| 1696 | 927 | 1731 | 884 | 1766 | 1104 |
| 1697 | 1107 | 1732 | 923 | 1767 | 1045 |
| 1698 | 903 | 1733 | 1202 | 1768 | 1261 |
| 1699 | 929 | 1734 | 1054 | 1769 | 844 |
| 1700 | 979 | 1735 | 970 | 1770 | 931 |
| 1701 | 957 | 1736 | 1021 | 1771 | 900 |
| 1702 | 1063 | 1737 | 1049 | 1772 | 985 |
| 1773 | 936 | 1808 | 1037 | 1843 | 1162 |
| 1774 | 868 | 1809 | 892 | 1844 | 951 |
| 1775 | 780 | 1810 | 1120 | 1845 | 1042 |
| 1776 | 1080 | 1811 | 1137 | 1846 | 846 |
| 1777 | 1349 | 1812 | 1000 | 1847 | 997 |
| 1778 | 1065 | 1813 | 971 | 1848 | 1118 |
| 1779 | 578 | 1814 | 1154 | 1849 | 956 |
| 1780 | 954 | 1815 | 1176 | 1850 | 1087 |
| 1781 | 1002 | 1816 | 1126 | 1851 | 1221 |
| 1782 | 1071 | 1817 | 1051 | 1852 | 1034 |
| 1783 | 1031 | 1818 | 799 | 1853 | 934 |
| 1784 | 820 | 1819 | 944 | 1854 | 884 |
| 1785 | 979 | 1820 | 907 | 1855 | 956 |
| 1786 | 1099 | 1821 | 833 | 1856 | 893 |
| 1787 | 861 | 1822 | 1015 | 1857 | 1092 |
| 1788 | 921 | 1823 | 1348 | 1858 | 894 |
| 1789 | 1039 | 1824 | 1105 | 1859 | 961 |
| 1790 | 1031 | 1825 | 1091 | 1860 | 831 |
| 1791 | 1045 | 1826 | 818 | 1861 | 945 |
| 1792 | 1124 | 1827 | 1200 | 1862 | 1053 |
| 1793 | 932 | 1828 | 971 | 1863 | 1129 |
| 1794 | 960 | 1829 | 994 | 1864 | 902 |
| 1795 | 858 | 1830 | 987 | 1865 | 1054 |
| 1796 | 1213 | 1831 | 952 | 1866 | 933 |
| 1797 | 706 | 1832 | 953 | 1867 | 1168 |
| 1798 | 1064 | 1833 | 963 | 1868 | 920 |
| 1799 | 825 | 1834 | 909 | 1869 | 920 |
| 1800 | 1104 | 1835 | 973 | 1870 | 951 |
| 1801 | 1117 | 1836 | 960 | 1871 | 972 |
| 1802 | 1054 | 1837 | 921 | 1872 | 1101 |
| 1803 | 918 | 1838 | 1060 | 1873 | 849 |
| 1804 | 1077 | 1839 | 908 | 1874 | 987 |
| 1805 | 904 | 1840 | 962 | 1875 | 1149 |
| 1806 | 977 | 1841 | 878 | 1876 | 960 |
| 1807 | 841 | 1842 | 1032 | 1877 | 973 |
| 1878 | 1125 | 1913 | 1045 | 1948 | 951 |
| 1879 | 936 | 1914 | 813 | 1949 | 1043 |
| 1880 | 979 | 1915 | 750 | 1950 | 773 |
| 1881 | 1090 | 1916 | 1024 | 1951 | 958 |
| 1882 | 1050 | 1917 | 1072 | 1952 | 764 |
| 1883 | 969 | 1918 | 803 | 1953 | 867 |
| 1884 | 948 | 1919 | 994 | 1954 | 957 |
| 1885 | 990 | 1920 | 1172 | 1955 | 1068 |
| 1886 | 976 | 1921 | 830 | 1956 | 995 |
| 1887 | 1174 | 1922 | 1076 | 1957 | 1047 |
| 1888 | 1072 | 1923 | 1078 | 1958 | 1014 |
| 1889 | 1155 | 1924 | 936 | 1959 | 1140 |
| 1890 | 905 | 1925 | 816 | 1960 | 1067 |
| 1891 | 781 | 1926 | 795 | 1961 | 1047 |
| 1892 | 1125 | 1927 | 1024 | 1962 | 824 |
| 1893 | 1022 | 1928 | 1154 | 1963 | 1272 |
| 1894 | 960 | 1929 | 1281 | 1964 | 927 |
| 1895 | 928 | 1930 | 1063 | 1965 | 940 |
| 1896 | 780 | 1931 | 1254 | 1966 | 1030 |
| 1897 | 1280 | 1932 | 708 | 1967 | 957 |
| 1898 | 955 | 1933 | 983 | 1968 | 1019 |
| 1899 | 996 | 1934 | 904 | 1969 | 927 |
| 1900 | 1047 | 1935 | 974 | 1970 | 1040 |
| 1901 | 820 | 1936 | 971 | 1971 | 1031 |
| 1902 | 969 | 1937 | 899 | 1972 | 1079 |
| 1903 | 1093 | 1938 | 1006 | 1973 | 1026 |
| 1904 | 1031 | 1939 | 1038 | 1974 | 909 |
| 1905 | 1079 | 1940 | 948 | 1975 | 944 |
| 1906 | 1155 | 1941 | 979 | 1976 | 1254 |
| 1907 | 928 | 1942 | 976 | 1977 | 990 |
| 1908 | 1235 | 1943 | 847 | 1978 | 839 |
| 1909 | 1015 | 1944 | 1045 | 1979 | 978 |
| 1910 | 1178 | 1945 | 911 | 1980 | 1164 |
| 1911 | 687 | 1946 | 890 |  |  |
| 1912 | 909 | 1947 | 1164 |  |  |

### Scotts Gap Residuals

| Year | Scotts Gap Residual |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1686 | 1578 | 1721 | 1031 | 1756 | 919 |
| 1687 | 616 | 1722 | 972 | 1757 | 1061 |
| 1688 | 509 | 1723 | 863 | 1758 | 958 |
| 1689 | 636 | 1724 | 1588 | 1759 | 968 |
| 1690 | 930 | 1725 | 1054 | 1760 | 786 |
| 1691 | 989 | 1726 | 1075 | 1761 | 1112 |
| 1692 | 685 | 1727 | 1502 | 1762 | 742 |
| 1693 | 1389 | 1728 | 762 | 1763 | 1120 |
| 1694 | 1028 | 1729 | 908 | 1764 | 851 |
| 1695 | 799 | 1730 | 1443 | 1765 | 923 |
| 1696 | 740 | 1731 | 559 | 1766 | 683 |
| 1697 | 826 | 1732 | 536 | 1767 | 931 |
| 1698 | 663 | 1733 | 1088 | 1768 | 1121 |
| 1699 | 998 | 1734 | 1174 | 1769 | 800 |
| 1700 | 1226 | 1735 | 706 | 1770 | 886 |
| 1701 | 859 | 1736 | 868 | 1771 | 1134 |
| 1702 | 532 | 1737 | 1181 | 1772 | 801 |
| 1703 | 620 | 1738 | 1108 | 1773 | 946 |
| 1704 | 1129 | 1739 | 1559 | 1774 | 298 |
| 1705 | 868 | 1740 | 527 | 1775 | 948 |
| 1706 | 837 | 1741 | 810 | 1776 | 946 |
| 1707 | 1330 | 1742 | 1233 | 1777 | 924 |
| 1708 | 518 | 1743 | 680 | 1778 | 757 |
| 1709 | 747 | 1744 | 1234 | 1779 | 454 |
| 1710 | 1439 | 1745 | 1153 | 1780 | 1245 |
| 1711 | 1100 | 1746 | 1011 | 1781 | 1110 |
| 1712 | 837 | 1747 | 1111 | 1782 | 1419 |
| 1713 | 905 | 1748 | 1156 | 1783 | 1419 |
| 1714 | 1249 | 1749 | 975 | 1784 | 337 |
| 1715 | 954 | 1750 | 1218 | 1785 | 1380 |
| 1716 | 940 | 1751 | 813 | 1786 | 1033 |
| 1717 | 1346 | 1752 | 592 | 1787 | 1311 |
| 1718 | 446 | 1753 | 924 | 1788 | 954 |
| 1719 | 1328 | 1754 | 1312 | 1789 | 979 |
| 1720 | 907 | 1755 | 829 | 1790 | 621 |
| 1791 | 968 | 1826 | 845 | 1861 | 551 |
| 1792 | 947 | 1827 | 1094 | 1862 | 1119 |
| 1793 | 998 | 1828 | 859 | 1863 | 1141 |
| 1794 | 970 | 1829 | 874 | 1864 | 819 |
| 1795 | 881 | 1830 | 1676 | 1865 | 866 |
| 1796 | 1428 | 1831 | 1147 | 1866 | 1124 |
| 1797 | 987 | 1832 | 1156 | 1867 | 1029 |
| 1798 | 1039 | 1833 | 802 | 1868 | 898 |
| 1799 | 1024 | 1834 | 850 | 1869 | 1218 |
| 1800 | 1175 | 1835 | 835 | 1870 | 821 |
| 1801 | 923 | 1836 | 842 | 1871 | 916 |
| 1802 | 1414 | 1837 | 860 | 1872 | 712 |
| 1803 | 1239 | 1838 | 729 | 1873 | 854 |
| 1804 | 1174 | 1839 | 664 | 1874 | 450 |
| 1805 | 1508 | 1840 | 1010 | 1875 | 1387 |
| 1806 | 1375 | 1841 | 1043 | 1876 | 1092 |
| 1807 | 801 | 1842 | 983 | 1877 | 677 |
| 1808 | 665 | 1843 | 1186 | 1878 | 1086 |
| 1809 | 955 | 1844 | 933 | 1879 | 449 |
| 1810 | 1112 | 1845 | 1177 | 1880 | 834 |
| 1811 | 612 | 1846 | 1007 | 1881 | 1073 |
| 1812 | 663 | 1847 | 1120 | 1882 | 1024 |
| 1813 | 802 | 1848 | 1281 | 1883 | 1096 |
| 1814 | 1120 | 1849 | 1051 | 1884 | 955 |
| 1815 | 845 | 1850 | 706 | 1885 | 957 |
| 1816 | 1202 | 1851 | 978 | 1886 | 969 |
| 1817 | 996 | 1852 | 1243 | 1887 | 1131 |
| 1818 | 974 | 1853 | 1240 | 1888 | 765 |
| 1819 | 451 | 1854 | 183 | 1889 | 1313 |
| 1820 | 1182 | 1855 | 981 | 1890 | 1255 |
| 1821 | 750 | 1856 | 1067 | 1891 | 921 |
| 1822 | 866 | 1857 | 1075 | 1892 | 1413 |
| 1823 | 1169 | 1858 | 1191 | 1893 | 1102 |
| 1824 | 1288 | 1859 | 871 | 1894 | 944 |
| 1825 | 1015 | 1860 | 1232 | 1895 | 744 |
| 1896 | 1017 | 1931 | 917 | 1966 | 910 |
| 1897 | 1187 | 1932 | 895 | 1967 | 920 |
| 1898 | 521 | 1933 | 626 | 1968 | 1288 |
| 1899 | 930 | 1934 | 914 | 1969 | 910 |
| 1900 | 1108 | 1935 | 1096 | 1970 | 779 |
| 1901 | 953 | 1936 | 397 | 1971 | 1150 |
| 1902 | 931 | 1937 | 1061 | 1972 | 1394 |
| 1903 | 1209 | 1938 | 1093 | 1973 | 1273 |
| 1904 | 1038 | 1939 | 1051 | 1974 | 1309 |
| 1905 | 952 | 1940 | 912 | 1975 | 1074 |
| 1906 | 1183 | 1941 | 910 | 1976 | 977 |
| 1907 | 1007 | 1942 | 1102 | 1977 | 783 |
| 1908 | 882 | 1943 | 1086 | 1978 | 1041 |
| 1909 | 1298 | 1944 | 421 | 1979 | 896 |
| 1910 | 1171 | 1945 | 1115 | 1980 | 943 |
| 1911 | 609 | 1946 | 1133 |  |  |
| 1912 | 1451 | 1947 | 894 |  |  |
| 1913 | 715 | 1948 | 826 |  |  |
| 1914 | 552 | 1949 | 1197 |  |  |
| 1915 | 1307 | 1950 | 1114 |  |  |
| 1916 | 1704 | 1951 | 1064 |  |  |
| 1917 | 1089 | 1952 | 855 |  |  |
| 1918 | 955 | 1953 | 772 |  |  |
| 1919 | 788 | 1954 | 856 |  |  |
| 1920 | 1014 | 1955 | 998 |  |  |
| 1921 | 685 | 1956 | 845 |  |  |
| 1922 | 1613 | 1957 | 998 |  |  |
| 1923 | 1244 | 1958 | 1073 |  |  |
| 1924 | 915 | 1959 | 858 |  |  |
| 1925 | 381 | 1960 | 1142 |  |  |
| 1926 | 912 | 1961 | 1088 |  |  |
| 1927 | 891 | 1962 | 979 |  |  |
| 1928 | 1401 | 1963 | 880 |  |  |
| 1929 | 1074 | 1964 | 838 |  |  |
| 1930 | 594 | 1965 | 882 |  |  |

## Instrumental Based

### Valley River

|  | Monthly mean in cfs (Calculation Period: 1904-07-01 -> 2009-09-30) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| YEAR | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| 1919 | 441.4 | 355.4 | 416.9 | 247.3 | 230.9 | 162.5 | 142.8 | 90.5 | 57.3 | 135.5 | 104.1 | 307.5 |
| 1920 | 486.5 | 354.1 | 557.2 | 831.9 | 287.6 | 163.4 | 226.2 | 563 | 321.7 | 119.2 | 139 | 470 |
| 1921 | 373.5 | 634.6 | 256 | 331.3 | 302.8 | 146.3 | 160.5 | 228.4 | 119.8 | 94.1 | 350.6 | 405.7 |
| 1922 | 785.7 | 523.1 | 806.4 | 509.9 | 442.3 | 275.4 | 183.5 | 92.1 | 82.3 | 69.4 | 67.3 | 466.6 |
| 1923 | 435.4 | 687.7 | 591 | 417.3 | 395.6 | 312.3 | 192.1 | 149.7 | 84 | 56.2 | 71 | 174.6 |
| 1924 | 451.4 | 281 | 353 | 357.1 | 333.8 | 189.3 | 161 | 82.3 | 109 | 63.4 | 51.6 | 207.2 |
| 1925 | 497 | 236.3 | 226.2 | 166.2 | 156 | 90.9 | 51.3 | 24.6 | 21.3 | 67.8 | 155.7 | 139.6 |
| 1926 | 338 | 296.4 | 313.8 | 270.9 | 141.7 | 116.4 | 72.8 | 182.7 | 74 | 67.8 | 208.9 | 654.5 |
| 1927 | 266.4 | 471.4 | 540.7 | 288.2 | 175.6 | 294.4 | 159.4 | 229.7 | 101.9 | 82.1 | 120.9 | 542.8 |
| 1928 | 308.1 | 295.9 | 512.1 | 440.6 | 412.8 | 444.9 | 369.5 | 241.7 | 434.3 | 156.2 | 165.1 | 157.6 |
| 1929 | 481.6 | 447 | 740.8 | 493.5 | 755 | 306 | 176.5 | 105.4 | 228.2 | 137.6 | 684.6 | 263.6 |
| 1930 | 291.9 | 287.8 | 462.1 | 243 | 302.7 | 122.3 | 91 | 37.5 | 56.6 | 46.3 | 80.5 | 121.8 |
| 1931 | 160.1 | 202.3 | 170.6 | 419.1 | 180.2 | 82.5 | 78.5 | 59.2 | 44.5 | 33.3 | 44.6 | 439.1 |
| 1932 | 418.4 | 711.4 | 310.1 | 312.3 | 284 | 173.9 | 317.7 | 98.5 | 64.9 | 139.3 | 270.7 | 1,045 |
| 1933 | 466.7 | 716 | 375.4 | 278.8 | 378.9 | 117.7 | 92.1 | 81.7 | 61.1 | 43.2 | 38.6 | 57.4 |
| 1934 | 198.3 | 136.6 | 554.4 | 228.7 | 153.5 | 182.5 | 118.7 | 196.9 | 97.6 | 111.7 | 96.8 | 143.2 |
| 1935 | 248.7 | 290.6 | 480.3 | 493.8 | 226.5 | 109.8 | 81.3 | 70 | 39 | 37.5 | 102.1 | 76.7 |
| 1936 | 753.6 | 669.4 | 524 | 834.8 | 157.7 | 78.9 | 59.1 | 54 | 48 | 70.7 | 72.9 | 221.5 |
| 1937 | 704.4 | 483.6 | 262.3 | 284.7 | 250.3 | 112.8 | 88.1 | 112.1 | 72.9 | 87.7 | 69.5 | 125.4 |
| 1938 | 222.9 | 174.5 | 382.4 | 556.5 | 272.1 | 322 | 415.2 | 391.8 | 132.4 | 71.2 | 95.2 | 106.5 |
| 1939 | 267.4 | 720.6 | 529.1 | 349.5 | 194.7 | 142.7 | 117.1 | 79 | 45.6 | 36.3 | 39.4 | 63.7 |
| 1940 | 93 | 221.1 | 287.4 | 325.8 | 165 | 174.2 | 108.4 | 89.1 | 52.2 | 44.2 | 74 | 88.4 |
| 1941 | 157.6 | 92.7 | 218.7 | 176 | 88.9 | 61.3 | 215.3 | 99.9 | 61.8 | 43.7 | 80.6 | 131.1 |
| 1942 | 181.4 | 373.1 | 458.5 | 168.4 | 214.6 | 181.3 | 197.5 | 220.7 | 134.4 | 119.5 | 111 | 624.1 |
| 1943 | 488.9 | 477.4 | 465 | 309.9 | 223 | 127.1 | 301.5 | 147.1 | 93.8 | 66 | 74.1 | 100.7 |
| 1944 | 189.2 | 661.9 | 645 | 458.2 | 232.6 | 115.1 | 73.1 | 69.9 | 77 | 73.5 | 82.5 | 237.1 |
| 1945 | 268.2 | 489 | 313.5 | 301.4 | 300.2 | 126.8 | 108.8 | 83.3 | 72.7 | 83.8 | 145.2 | 276.6 |
| 1946 | 579 | 696.4 | 396.5 | 333.1 | 360.8 | 162.1 | 129.4 | 64.6 | 61.7 | 64.7 | 83.6 | 144 |
| 1947 | 882.8 | 271.4 | 308.3 | 257.3 | 190.6 | 129.7 | 104.7 | 76.3 | 48.1 | 45.6 | 79.3 | 115.7 |
| 1948 | 133.1 | 529.9 | 427.6 | 444.6 | 154.3 | 105.5 | 121.6 | 144.2 | 53.9 | 42.7 | 400.7 | 460.3 |
| 1949 | 658.9 | 588.5 | 361.3 | 404.6 | 279 | 207.3 | 442.9 | 177.3 | 182.5 | 232.4 | 237.4 | 339.8 |
| 1950 | 593.6 | 556.5 | 684.5 | 300.3 | 225.8 | 285.4 | 320.3 | 151.4 | 132.1 | 81.7 | 87.7 | 149.8 |
| 1951 | 194 | 266 | 578.4 | 428.2 | 180.4 | 196.7 | 151 | 78.2 | 74.7 | 56.9 | 142.8 | 451.3 |
| 1952 | 499.8 | 304.8 | 556.8 | 217.6 | 157.5 | 180.9 | 70.7 | 142 | 86.6 | 52.5 | 80.9 | 174.5 |
| 1953 | 395.3 | 615 | 334.8 | 238.8 | 332 | 104.7 | 79.5 | 46.6 | 52.9 | 36.5 | 46.1 | 162.9 |
| 1954 | 732.7 | 205.7 | 470.2 | 308.7 | 192.5 | 180.5 | 69.5 | 59.8 | 29.5 | 25.2 | 44.4 | 140.7 |
| 1955 | 121.4 | 447.6 | 379.4 | 405.4 | 185.6 | 119.2 | 159.6 | 143.7 | 60.6 | 97.3 | 127.8 | 193.7 |
| 1956 | 133 | 732.5 | 530 | 428.7 | 262.5 | 124 | 129.9 | 60.5 | 48.9 | 49.5 | 55.2 | 294.6 |
| 1957 | 474.8 | 1,022 | 343.7 | 492 | 184 | 263.1 | 130.9 | 83.5 | 69.9 | 116.9 | 374 | 452.1 |
| 1958 | 270.2 | 510.6 | 330.7 | 380.9 | 428.1 | 128.7 | 234.8 | 111.5 | 67.7 | 71 | 79.3 | 97.7 |
| 1959 | 270.7 | 306.5 | 269.7 | 408 | 205.5 | 257.7 | 124.5 | 73.9 | 69.8 | 118.4 | 281.8 | 305.9 |
| 1960 | 344.4 | 406.4 | 430.3 | 324.3 | 153.5 | 116.6 | 81.1 | 126.2 | 79.6 | 89.4 | 105.2 | 168.5 |
| 1961 | 211.9 | 636.5 | 527.9 | 351.4 | 227.6 | 284.7 | 278.8 | 131.2 | 89.7 | 67.1 | 85.2 | 679.4 |
| 1962 | 663.4 | 687.4 | 553.2 | 576.3 | 195.9 | 147.7 | 103.3 | 82.7 | 88.5 | 82.9 | 202.4 | 161.5 |
| 1963 | 299 | 381.6 | 948.2 | 300 | 285.6 | 198 | 256.8 | 142.3 | 88.4 | 60.5 | 93.2 | 171.8 |
| 1964 | 455.9 | 402.2 | 807.2 | 668.4 | 268.6 | 118 | 129 | 214.9 | 84.1 | 223.5 | 218.5 | 322.9 |
| 1965 | 377.8 | 487.9 | 649.7 | 430.7 | 217.3 | 160.2 | 107.9 | 82.4 | 94.5 | 109.9 | 97.2 | 81.3 |
| 1966 | 147.2 | 559.5 | 387.3 | 302.7 | 334.5 | 133.3 | 113.9 | 127.3 | 89.2 | 204.6 | 275.7 | 251.4 |
| 1967 | 355.1 | 451.9 | 334.2 | 205.9 | 405 | 236.6 | 360.7 | 276.7 | 169.2 | 103.9 | 247.1 | 580 |
| 1968 | 561.1 | 219.7 | 269.6 | 321.7 | 243.1 | 132 | 108.9 | 119.8 | 68.9 | 79 | 112 | 224.1 |
| 1969 | 256.6 | 688.4 | 288.1 | 355.2 | 178.2 | 162.1 | 101.6 | 116.1 | 89.8 | 67.4 | 114.1 | 169.4 |
| 1970 | 227.5 | 266.5 | 233.1 | 306.7 | 157.9 | 214 | 104 | 95.6 | 60.4 | 80.5 | 109.9 | 188.9 |
| 1971 | 463 | 595.6 | 476.5 | 324.1 | 199 | 148.8 | 333.2 | 358.4 | 106 | 78.5 | 83.1 | 232.3 |
| 1972 | 636.3 | 370.4 | 395.3 | 430 | 262.8 | 145 | 223.5 | 203.8 | 105.7 | 214.4 | 270.6 | 646.8 |
| 1973 | 337.2 | 437.6 | 518.6 | 453.3 | 474.7 | 296.6 | 214.4 | 176.4 | 114.5 | 84.1 | 158.8 | 391.7 |
| 1974 | 936.4 | 771.1 | 342.7 | 609.9 | 298.6 | 256.6 | 147.7 | 130.2 | 90.8 | 71.3 | 125 | 304.7 |
| 1975 | 526.2 | 837 | 869.7 | 406.1 | 195.9 | 132.4 | 85.5 | 93.5 | 69.1 | 96.9 | 147.2 | 212.6 |
| 1976 | 462.4 | 268.4 | 459.8 | 321.4 | 451.5 | 298.9 | 281.1 | 108.4 | 80.3 | 146.7 | 150.8 | 328.5 |
| 1977 | 320.3 | 211.4 | 452.3 | 649.2 | 203.9 | 156.2 | 91.5 | 128 | 168.6 | 209 | 333.1 | 351.5 |
| 1978 | 542.1 | 251.4 | 384 | 209.9 | 233.1 | 162 | 88.8 | 121.4 | 53.8 | 38.3 | 57.2 | 228.4 |
| 1979 | 633.7 | 418.6 | 660.2 | 633.5 | 238.6 | 236.8 | 322.4 | 217.8 | 208.7 | 163.1 | 373.9 | 221.5 |
| 1980 | 442.8 | 336.4 | 1,071 | 552.3 | 308.9 | 202.7 | 107.7 | 63.2 | 67.4 | 67.5 | 90 | 92.5 |
| 1981 | 69.9 | 316.5 | 190 | 272.9 | 201.4 | 271.2 | 82.1 | 54.5 | 60.5 | 53.8 | 78.3 | 228.6 |
| 1982 | 550.1 | 681.3 | 430.5 | 303.9 | 186.3 | 133.3 | 108.8 | 193.5 | 286.8 | 152.3 | 231.7 | 548.9 |
| 1983 | 243 | 327.2 | 232.3 | 472.9 | 440.8 | 204.7 | 102.2 | 69.1 | 64.4 | 60.9 | 138.5 | 420.3 |
| 1984 | 256.9 | 362.1 | 420.1 | 400.4 | 699.5 | 147.1 | 231.9 | 157.3 | 72.6 | 71.4 | 101.6 | 168.1 |
| 1985 | 169.2 | 481 | 193.2 | 166.1 | 111.5 | 89.6 | 101.5 | 184.1 | 73.6 | 72.2 | 129.3 | 178.3 |
| 1986 | 118.7 | 233.6 | 267.4 | 135.3 | 95.2 | 65.7 | 52 | 55.4 | 49 | 105.5 | 309.1 | 323.7 |
| 1987 | 303.6 | 407.4 | 365.8 | 290.3 | 191.7 | 234.6 | 169.3 | 54.7 | 53.9 | 49.1 | 55.1 | 91.3 |
| 1988 | 285.6 | 186.6 | 154.9 | 220.1 | 92.9 | 44.8 | 42.4 | 48.6 | 63.1 | 63.3 | 158.8 | 131.5 |
| 1989 | 404.1 | 518.6 | 588.9 | 401 | 402.8 | 607.2 | 367.2 | 153.9 | 261.8 | 338.1 | 411.9 | 355.6 |
| 1990 | 671.8 | 1,008 | 745 | 263.7 | 270 | 157 | 126.4 | 86.7 | 68.6 | 139.8 | 100.5 | 460.4 |
| 1991 | 369.2 | 560.6 | 506.7 | 428.6 | 291.5 | 185 | 139 | 298.9 | 177.1 | 85.9 | 157.7 | 440.8 |
| 1992 | 356.2 | 400.7 | 445.7 | 324 | 204.7 | 168.4 | 133 | 139.3 | 142.7 | 123.4 | 340.1 | 651.2 |
| 1993 | 518.5 | 291.5 | 511.5 | 336.7 | 174.7 | 111.2 | 63.2 | 67.6 | 72.5 | 50.3 | 160.3 | 290.3 |
| 1994 | 452.9 | 494.2 | 872 | 736.5 | 238.4 | 394.5 | 364.3 | 319 | 181.1 | 154.6 | 168.8 | 259.3 |
| 1995 | 469.9 | 853.2 | 494.2 | 204.2 | 207.4 | 153.2 | 76.2 | 96.6 | 76.2 | 174.6 | 343.2 | 216.6 |
| 1996 | 625 | 532.3 | 565.1 | 421.1 | 344 | 226.9 | 119.8 | 152.9 | 149.6 | 143.7 | 245.2 | 463 |
| 1997 | 460 | 439.2 | 698.3 | 399.7 | 375 | 381.6 | 181.7 | 86.3 | 99.4 | 121.5 | 144.2 | 196.3 |
| 1998 | 525.2 | 368 | 365.2 | 547.8 | 317.5 | 336.7 | 120.8 | 69.9 | 45.8 | 43.7 | 71.7 | 155.8 |
| 1999 | 367.5 | 432 | 367.5 | 210.7 | 366.4 | 288.3 | 288.4 | 82.7 | 50.4 | 64.2 | 81.7 | 95.9 |
| 2000 | 224.4 | 219 | 223.5 | 401.5 | 174 | 130.4 | 78.7 | 75.9 | 65.3 | 36.1 | 100.5 | 109.9 |
| 2001 | 218 | 327.9 | 267.4 | 182.2 | 128.9 | 178.7 | 129.8 | 149.7 | 126.5 | 71.4 | 70.9 | 164.5 |
| 2002 | 383.6 | 230.1 | 289.6 | 280.2 | 314.6 | 124.7 | 121 | 76.9 | 256.9 | 195.8 | 453.1 | 461 |
| 2003 | 289.1 | 566.2 | 389.5 | 326.6 | 721.2 | 249.2 | 409.8 | 202.5 | 142.5 | 96.4 | 202.5 | 296.7 |
| 2004 | 283.3 | 362.2 | 303.4 | 228.9 | 184.9 | 132.4 | 159.4 | 107.2 | 205.9 | 88.6 | 383.8 | 553.4 |
| 2005 | 237.1 | 367.3 | 391.7 | 381.5 | 242.8 | 222.3 | 230.6 | 173 | 95.1 | 64.8 | 85.6 | 216.6 |
| 2006 | 413.2 | 275.7 | 177.7 | 201.4 | 144 | 111.8 | 68.5 | 48.9 | 79 | 133.1 | 198.7 | 146.1 |
| 2007 | 320.6 | 152.4 | 229.8 | 148.8 | 93.7 | 60.8 | 70.8 | 39.5 | 42.2 | 31.4 | 52.2 | 98.5 |
| 2008 | 140.9 | 244.2 | 374.5 | 234.3 | 187.1 | 83.2 | 57.8 | 50.8 | 31.2 | 33.9 | 45.6 | 326.2 |

### Oconaluftee River

|  | Monthly mean in cfs (Calculation Period: 1945-08-01 -> 2009-09-30) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| 1948 |  |  |  |  |  |  | 284.9 | 389.9 | 190.8 | 139.4 | 771.5 | 824.1 |
| 1949 | 1,042 | 963.8 | 662.5 | 701 | 583.7 | 521.7 | 714.2 | 484.3 | 394.1 | 412.5 | 492.2 | 747 |
| 1950 | 1,068 | 1,086 | 1,014 | 516.8 | 431.4 | 506.8 | 457.1 | 282.8 | 291.3 | 192.1 | 214.7 | 448.4 |
| 1951 | 423 | 573.9 | 933.3 | 775.8 | 385.2 | 437.5 | 398.4 | 212 | 252.3 | 163.5 | 416.4 | 867.9 |
| 1952 | 941.6 | 552.7 | 992.2 | 464.8 | 312.2 | 274.8 | 168.6 | 172.3 | 148 | 100.4 | 203.5 | 298.9 |
| 1953 | 649.5 | 1,058 | 666.1 | 429.8 | 547.9 | 298.4 | 329 | 217.6 | 141.8 | 101.7 | 127.2 | 429.3 |
| 1954 | 1,161 | 425.2 | 805.4 | 633.3 | 369.1 | 270.2 | 202.5 | 199.5 | 121 | 94.5 | 145.5 | 410.7 |
| 1955 | 275.4 | 879.3 | 977.2 | 695.8 | 493.3 | 357 | 552.2 | 381.2 | 166.2 | 258.1 | 227.6 | 320.8 |
| 1956 | 233.4 | 1,166 | 947.6 | 940.6 | 504.8 | 311.4 | 460 | 202.7 | 203.1 | 173.4 | 207.9 | 593 |
| 1957 | 856.1 | 1,679 | 544.6 | 932.4 | 439.8 | 447.1 | 258.6 | 198.9 | 235.3 | 355.8 | 777.5 | 865 |
| 1958 | 414.3 | 659.1 | 628.9 | 737.5 | 775 | 314.8 | 517.9 | 290.3 | 184.2 | 172.5 | 189 | 244.1 |
| 1959 | 651.2 | 568.8 | 634.5 | 843.4 | 430.6 | 483 | 307.8 | 311.5 | 271.7 | 474.4 | 626.2 | 801.6 |
| 1960 | 676.4 | 744.3 | 679.4 | 895.1 | 346 | 291 | 261.4 | 535.4 | 260.8 | 252.1 | 234.7 | 319.1 |
| 1961 | 380.6 | 1,106 | 1,034 | 730.6 | 451.6 | 389.8 | 367.8 | 313.9 | 190.7 | 194.6 | 366.9 | 1,266 |
| 1962 | 976.9 | 1,253 | 1,136 | 935.6 | 343.2 | 338.4 | 358.7 | 323.9 | 236.4 | 203.6 | 382.9 | 307.9 |
| 1963 | 463.5 | 529 | 1,714 | 494.1 | 468.5 | 398.6 | 463.1 | 272.2 | 169.1 | 125.8 | 211 | 228.2 |
| 1964 | 743 | 504.1 | 1,611 | 1,265 | 603.9 | 281.6 | 340.9 | 438 | 244.3 | 507.8 | 425.1 | 617.8 |
| 1965 | 677.5 | 649.4 | 1,128 | 727.8 | 465.8 | 338.2 | 262.1 | 221.6 | 177.9 | 266 | 196.9 | 162.1 |
| 1966 | 210.8 | 1,044 | 610.8 | 412.9 | 487.7 | 216.4 | 294.2 | 486.8 | 309.7 | 521.7 | 686.3 | 606.5 |
| 1967 | 626.9 | 787.7 | 691.5 | 366 | 600.7 | 784.7 | 677.1 | 627.8 | 387.8 | 276.1 | 457.1 | 1,003 |
| 1968 | 833.9 | 405.9 | 612.5 | 625.1 | 413.6 | 411.8 | 283.3 | 295.8 | 175.5 | 244.5 | 319.8 | 475.2 |
| 1969 | 476.6 | 996.4 | 623.3 | 743.2 | 364.5 | 369 | 297.8 | 364 | 240.8 | 169.2 | 293 | 479.8 |
| 1970 | 434.8 | 455.4 | 504.1 | 742.1 | 400.5 | 538 | 257.1 | 253.6 | 178 | 239.9 | 287 | 413.1 |
| 1971 | 696.8 | 943.9 | 808.2 | 536.4 | 445.1 | 414.8 | 743.5 | 693.7 | 237.5 | 231.9 | 299.9 | 632.1 |
| 1972 | 1,057 | 658.4 | 773.9 | 742.4 | 571.1 | 344.1 | 363.3 | 304.8 | 270.4 | 591.2 | 614.3 | 1,249 |
| 1973 | 618.4 | 797.5 | 1,257 | 806.1 | 871.1 | 564.3 | 345.1 | 322.4 | 245.7 | 180.3 | 394.5 | 826.3 |
| 1974 | 1,428 | 1,070 | 697.7 | 1,124 | 816.8 | 493.8 | 374.7 | 402 | 275.1 | 225.5 | 382.4 | 558.4 |
| 1975 | 797 | 1,112 | 1,597 | 766.3 | 489.8 | 336.8 | 261.2 | 245.4 | 437.6 | 268.8 | 420.2 | 454.8 |
| 1976 | 908.6 | 665.4 | 753.8 | 583.3 | 758.9 | 503.9 | 350.1 | 197.1 | 187.2 | 450.2 | 307.2 | 516.3 |
| 1977 | 291.6 | 419 | 1,113 | 1,156 | 389 | 460.8 | 299 | 342.8 | 439.1 | 402.1 | 487.6 | 763.2 |
| 1978 | 977.4 | 392.4 | 902.9 | 462.2 | 692.9 | 458.5 | 260.6 | 455.1 | 181.6 | 132.5 | 200.1 | 629.6 |
| 1979 | 1,247 | 790.6 | 1,089 | 1,032 | 495.6 | 403.7 | 671.2 | 386.3 | 542.1 | 351.8 | 714 | 433 |
| 1980 | 760.7 | 460.6 | 1,386 | 890.4 | 555.1 | 336.1 | 223.3 | 161.9 | 174.3 | 212.8 | 294.9 | 232.7 |
| 1981 | 169.9 | 667.3 | 466.5 | 701 | 461.8 | 503.5 | 330.9 | 206.1 | 271.4 | 213.2 | 241.9 | 566.5 |
| 1982 | 1,202 | 1,183 | 790.9 | 512.6 | 461.8 | 440.5 | 333.1 | 351.1 | 343.8 | 305.1 | 568.3 | 932.8 |
| 1983 | 421.7 | 607.1 | 553.1 | 1,101 | 847.7 | 412.4 | 310.8 | 191.3 | 178.2 | 240.3 | 395 | 812.6 |
| 1984 | 485.1 | 808.2 | 935.6 | 821 | 1,202 | 523.3 | 902.2 | 475.7 | 200.6 | 183.2 | 279.9 | 335.9 |
| 1985 | 358.1 | 964.4 | 432.3 | 388.6 | 355.2 | 235.6 | 225.3 | 354 | 162.5 | 138.8 | 365.2 | 368.9 |
| 1986 | 187.3 | 440.9 | 531.3 | 277.1 | 239 | 189.9 | 188 | 173.4 | 199.2 | 248.5 | 535.5 | 595.2 |
| 1987 | 478.9 | 678.5 | 763.1 | 828.8 | 362 | 375.1 | 304.4 | 161.5 | 157.9 | 100.7 | 125 | 247 |
| 1988 | 591 | 422.7 | 329.8 | 514.4 | 314.7 | 175.1 | 206.9 | 220.9 | 249.2 | 176.7 | 405.3 | 307.8 |
| 1989 | 839.9 | 792.5 | 1,043 | 777 | 961.2 | 1,136 | 937.8 | 500.4 | 584.5 | 644.7 | 606.4 | 529.7 |
| 1990 | 1,001 | 1,700 | 1,397 | 476 | 613 | 359.4 | 349.7 | 215.5 | 199.6 | 410.7 | 222.3 | 824 |
| 1991 | 704.1 | 882 | 1,046 | 847.7 | 543.1 | 493.5 | 363.9 | 509.4 | 335.7 | 190.7 | 352.3 | 812.8 |
| 1992 | 516.9 | 619.1 | 736.8 | 478.8 | 504.4 | 535 | 274 | 415.7 | 383.3 | 298.9 | 684.3 | 998.8 |
| 1993 | 879.1 | 512.9 | 968.7 | 696.8 | 421.4 | 272.8 | 174.3 | 198.7 | 266.9 | 168.7 | 364.5 | 750.6 |
| 1994 | 1,050 | 1,245 | 1,456 | 1,315 | 462.3 | 661.5 | 701.7 | 733.2 | 380.4 | 288.3 | 265.1 | 407.5 |
| 1995 | 737.8 | 999.8 | 847.1 | 371.8 | 426 | 355.5 | 259.1 | 306.6 | 253.1 | 499 | 773.5 | 441.5 |
| 1996 | 1,239 | 1,025 | 839.7 | 672.8 | 689.1 | 618.8 | 384.9 | 494 | 388.1 | 299 | 543.3 | 986.9 |
| 1997 | 747.4 | 874.6 | 1,459 | 819 | 725.5 | 769.4 | 347.3 | 192 | 266 | 292.6 | 332.3 | 487.3 |
| 1998 | 1,180 | 802 | 866.5 | 1,234 | 662.8 | 512.7 | 261.4 | 176.2 | 123 | 116 | 172.9 | 435.1 |
| 1999 | 752.3 | 648.9 | 718 | 539.5 | 667.4 | 436.5 | 663.4 | 235.5 | 146.5 | 203.9 | 243.8 | 292.9 |
| 2000 | 417.3 | 611.7 | 500.5 | 791.7 | 539.6 | 359.9 | 315.9 | 300 | 219.6 | 160.9 | 278.9 | 323.3 |
| 2001 | 426.7 | 616.1 | 573.5 | 507.9 | 361.1 | 386.9 | 412.4 | 415.8 | 315.3 | 183.4 | 142.4 | 337.1 |
| 2002 | 646 | 398.8 | 577.2 | 523.2 | 567.4 | 258.8 | 238.4 | 151.9 | 283.3 | 292.3 | 613.2 | 722.7 |
| 2003 | 446.8 | 1,045 | 704.5 | 810.5 | 1,157 | 516.7 | 542.5 | 321.8 | 291.1 | 183.5 | 521.4 | 579.6 |
| 2004 | 553.4 | 629.8 | 637.5 | 556 | 433.2 | 433.8 | 402.7 | 322.1 | 967.7 | 327.4 | 689.5 | 959.7 |
| 2005 | 567.9 | 615.4 | 686.3 | 755.5 | 504.8 | 475 | 581.6 | 406.6 | 272.5 | 165.5 | 181.1 | 381.5 |
| 2006 | 913.7 | 573.3 | 443.7 | 573.9 | 501.7 | 329.7 | 354.1 | 264.8 | 392.5 | 440.2 | 546.6 | 399.3 |
| 2007 | 656.8 | 333.5 | 559.7 | 446.2 | 261.3 | 177.3 | 176.4 | 125.3 | 97.9 | 103.7 | 142.9 | 363.5 |
| 2008 | 420.8 | 680.7 | 827 | 464.2 | 382 | 195.6 | 198.5 | 181.7 | 102.1 | 93.5 | 137.8 | 727.3 |

### Nantahala River

|  | Monthly mean in cfs (Calculation Period: 1940-10-01 -> 2009-09-30) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| YEAR | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| 1940 |  |  |  |  |  |  |  |  |  | 86.4 | 120.8 | 149.2 |
| 1941 | 180.5 | 115.3 | 159.6 | 159.5 | 102.5 | 77.2 | 278.2 | 132.2 | 86.6 | 73.9 | 92.4 | 159.1 |
| 1942 | 176.7 | 265.5 | 346.5 | 173.1 | 200 | 154.8 | 128.2 | 166.5 | 137.5 | 113.1 | 109.1 | 355.9 |
| 1943 | 369.5 | 366.5 | 333 | 255.2 | 203.9 | 135.1 | 259 | 160.8 | 110.4 | 81.6 | 88 | 97.5 |
| 1944 | 163.5 | 376.4 | 432.6 | 338.8 | 191.2 | 110.2 | 81 | 80.7 | 75 | 69.8 | 88.4 | 169.9 |
| 1945 | 230.3 | 275.8 | 247.7 | 269 | 243.1 | 127.7 | 88.6 | 83 | 120.7 | 107.5 | 126.2 | 200.9 |
| 1946 | 431.6 | 443.2 | 384.7 | 253 | 314.1 | 162 | 151.7 | 80.3 | 75.3 | 98.2 | 113.2 | 125 |
| 1947 | 499.1 | 217.6 | 214.4 | 237.5 | 175.9 | 149.5 | 121.5 | 90.5 | 62.2 | 125.7 | 194 | 177.5 |
| 1948 | 166 | 418.7 | 357.8 | 299.1 | 139.3 | 93.5 | 136.3 | 206.3 | 95.8 | 66.1 | 355.5 | 360.2 |
| 1949 | 427.4 | 373.8 | 235.8 | 268.6 | 245 | 362.3 | 296.2 | 179 | 201.7 | 175.5 | 192.6 | 220.6 |
| 1950 | 318.6 | 367.4 | 388.7 | 212.1 | 151.7 | 188 | 191.6 | 144.8 | 374.4 | 151.9 | 127.2 | 201.4 |
| 1951 | 168.6 | 204 | 288.5 | 306.1 | 175 | 190.6 | 145.2 | 81.7 | 83.4 | 67.5 | 145.5 | 353.1 |
| 1952 | 339.1 | 299.3 | 484.5 | 229.3 | 155.3 | 123 | 70.4 | 73.8 | 65.8 | 45.6 | 79.7 | 154 |
| 1953 | 274 | 438.5 | 303.8 | 184.9 | 200.1 | 106.1 | 93.8 | 73.5 | 61.1 | 44.2 | 57.2 | 202.7 |
| 1954 | 502.2 | 240.7 | 341.5 | 297.8 | 180.6 | 150.5 | 98.4 | 83.9 | 46.8 | 42.2 | 56.6 | 129.5 |
| 1955 | 114.5 | 298.9 | 300 | 322.6 | 257.3 | 175.7 | 160.1 | 130.5 | 73.3 | 98.8 | 101.3 | 143 |
| 1956 | 96.5 | 377.2 | 360.2 | 321.3 | 221.7 | 118.9 | 172.5 | 87.5 | 71.6 | 67.6 | 74.4 | 231.9 |
| 1957 | 311.3 | 657.4 | 238.2 | 453.9 | 173.6 | 183.6 | 140.8 | 79.1 | 94.2 | 163.8 | 289.8 | 342.8 |
| 1958 | 219.1 | 306 | 212.6 | 263 | 265.9 | 107.7 | 156.8 | 102.9 | 63.2 | 64.7 | 67 | 77.2 |
| 1959 | 191.9 | 248.6 | 218.2 | 286.5 | 269.2 | 233.3 | 112.2 | 89.9 | 103 | 198.5 | 178.9 | 219.1 |
| 1960 | 260 | 316.7 | 263.7 | 307.3 | 142.4 | 112.2 | 89.6 | 150.6 | 103.8 | 143.2 | 118.2 | 129.7 |
| 1961 | 169.1 | 382.5 | 367.9 | 271.8 | 177 | 202.9 | 179.9 | 172.9 | 125.1 | 81.5 | 104.6 | 451.5 |
| 1962 | 391.2 | 347.5 | 321 | 367.3 | 147.5 | 130 | 82.9 | 63.5 | 54.9 | 71.8 | 137.2 | 116.7 |
| 1963 | 172.5 | 199.8 | 426.3 | 212.9 | 208.1 | 167.7 | 220.9 | 133 | 90.6 | 65 | 85.8 | 135.4 |
| 1964 | 302.4 | 267.2 | 554.2 | 487.3 | 228.1 | 102.3 | 83.7 | 112 | 121.6 | 414.5 | 183.9 | 254.7 |
| 1965 | 267.1 | 303.4 | 334.4 | 279.5 | 178.5 | 135.8 | 92.6 | 79.9 | 102.8 | 188.1 | 110.4 | 92.6 |
| 1966 | 109.5 | 508.9 | 298.5 | 224.9 | 283.2 | 147.7 | 86.7 | 85.1 | 96.1 | 157.3 | 244 | 228.8 |
| 1967 | 273.1 | 347.5 | 292.6 | 169 | 240.7 | 322.3 | 258 | 259.1 | 224.2 | 118.2 | 158.6 | 291 |
| 1968 | 343.8 | 154.8 | 230.5 | 260.7 | 170.9 | 137.5 | 108.4 | 104.8 | 86.8 | 86.6 | 125.6 | 212.1 |
| 1969 | 255.6 | 494.6 | 223.1 | 275.4 | 216 | 218.9 | 116.2 | 269.9 | 150.3 | 126.6 | 185.7 | 174.5 |
| 1970 | 193.8 | 222.4 | 215.6 | 220.8 | 150.7 | 313.2 | 127.2 | 103.3 | 74.9 | 122.2 | 155.7 | 150.5 |
| 1971 | 328.3 | 450.3 | 328.4 | 236.8 | 210.6 | 149 | 199.7 | 198.6 | 102.1 | 95.6 | 129.1 | 346.9 |
| 1972 | 455.5 | 250.6 | 273 | 250.7 | 211.6 | 164.4 | 179.6 | 170.6 | 90.8 | 123.2 | 229 | 370.6 |
| 1973 | 287.5 | 346.8 | 414.2 | 336.5 | 433.3 | 301.7 | 155.3 | 142 | 102.3 | 75.9 | 142.9 | 343.8 |
| 1974 | 568.5 | 481 | 247.4 | 425.4 | 303.8 | 266.9 | 131.5 | 180.5 | 93.9 | 76 | 131.5 | 214 |
| 1975 | 360 | 571 | 566.2 | 309.5 | 186.3 | 121.1 | 80.3 | 78.8 | 111.3 | 198 | 212.5 | 181.4 |
| 1976 | 376.3 | 267.2 | 315.8 | 285.6 | 491.4 | 271 | 159.9 | 96.2 | 81.6 | 190.7 | 146.4 | 263.1 |
| 1977 | 231.5 | 182 | 448.1 | 475.4 | 191.5 | 139.4 | 81.5 | 89.7 | 151.2 | 165.5 | 376.3 | 310.2 |
| 1978 | 399.8 | 206.1 | 258.3 | 159.2 | 187.3 | 124.6 | 79 | 154.9 | 65.2 | 48.4 | 71.8 | 187.8 |
| 1979 | 412 | 329.4 | 572.1 | 493 | 307.5 | 221.4 | 233 | 147.6 | 199.8 | 175.6 | 333.1 | 208.1 |
| 1980 | 259.5 | 207.7 | 551.3 | 390.2 | 254.4 | 166.9 | 90 | 66.5 | 71.4 | 75 | 105.4 | 107.9 |
| 1981 | 84.4 | 216.1 | 162.6 | 186.1 | 194.1 | 218.7 | 96.9 | 61.4 | 69.5 | 63 | 74.9 | 166.2 |
| 1982 | 341 | 413.4 | 229.1 | 187.7 | 165.7 | 122.9 | 124.7 | 121.9 | 141.6 | 125 | 184.9 | 452.7 |
| 1983 | 238.1 | 359.6 | 274.6 | 479.3 | 278.8 | 164.4 | 95.6 | 75.5 | 94.4 | 101.2 | 171.8 | 404.5 |
| 1984 | 217.1 | 335.5 | 333.8 | 320.9 | 395.3 | 134.8 | 176.6 | 157.9 | 75.5 | 82.3 | 125.4 | 189.6 |
| 1985 | 162.9 | 342.1 | 178.1 | 158.8 | 131 | 105.6 | 130.9 | 193.2 | 108.4 | 79.5 | 164.6 | 169.6 |
| 1986 | 110.5 | 162.2 | 187.8 | 118.5 | 96.8 | 67.1 | 59 | 49.5 | 41.8 | 83.1 | 219.6 | 325.1 |
| 1987 | 233.9 | 285.1 | 288.9 | 225.9 | 134 | 102.9 | 84.9 | 50.1 | 71.2 | 46.7 | 89.8 | 124.1 |
| 1988 | 258.5 | 191.7 | 137.6 | 209.4 | 125.1 | 78.6 | 79.2 | 71.7 | 98.1 | 86.6 | 139.4 | 128.3 |
| 1989 | 292.4 | 304.9 | 372.3 | 277.5 | 296.6 | 485.3 | 334.6 | 155.8 | 223.5 | 282.3 | 250.7 | 252.4 |
| 1990 | 375.6 | 612.1 | 503.9 | 201 | 232.6 | 116.7 | 139.4 | 90.5 | 75.1 | 152.5 | 101.3 | 303 |
| 1991 | 278.7 | 302.1 | 356.4 | 318.3 | 266.4 | 174.1 | 133.2 | 171.2 | 124.6 | 70.2 | 124.6 | 278 |
| 1992 | 226.2 | 224.9 | 319.1 | 237.4 | 187.2 | 227.1 | 111.5 | 161.6 | 166.4 | 154.5 | 372.1 | 474.5 |
| 1993 | 380.1 | 242 | 375.1 | 302.5 | 198.4 | 124.6 | 67.1 | 62.1 | 64.2 | 45.4 | 103.6 | 180.9 |
| 1994 | 242.7 | 298.1 | 392.8 | 481.9 | 168.7 | 296.2 | 256.2 | 327.5 | 143.5 | 191.9 | 144.9 | 216.8 |
| 1995 | 315.2 | 484.4 | 370.9 | 160.9 | 141.9 | 149.6 | 93.4 | 117.4 | 87.1 | 283.1 | 288.7 | 179.7 |
| 1996 | 409.3 | 398.9 | 311.1 | 260.3 | 237.9 | 203.6 | 142.9 | 196.2 | 240.5 | 184.1 | 224.8 | 345.5 |
| 1997 | 319.1 | 315.8 | 490.3 | 349.6 | 283.1 | 268.8 | 164.4 | 109 | 99.2 | 108.7 | 143.1 | 142.5 |
| 1998 | 441.3 | 473.2 | 327.5 | 350.5 | 239.1 | 220.1 | 98 | 61.6 | 45.8 | 38.4 | 73.5 | 116 |
| 1999 | 254.4 | 325.7 | 259.5 | 192.3 | 231.6 | 148.1 | 150 | 70.7 | 44 | 61.7 | 97 | 109.3 |
| 2000 | 146.9 | 165.1 | 202.7 | 303.4 | 135 | 98.5 | 66.1 | 58 | 49.4 | 35.6 | 84 | 93.9 |
| 2001 | 178.5 | 208.3 | 252.6 | 191.3 | 114.3 | 134 | 119.3 | 88 | 113.5 | 115.7 | 87.9 | 142.1 |
| 2002 | 237.3 | 198.3 | 202.8 | 210.8 | 233.1 | 112.6 | 85.9 | 62 | 162.3 | 148.6 | 261.1 | 269.7 |
| 2003 | 237.7 | 332.4 | 293.7 | 241.1 | 426.7 | 267.6 | 285.7 | 160.8 | 152.4 | 114 | 234 | 246.5 |
| 2004 | 197.5 | 242.4 | 210 | 188 | 183.5 | 165.4 | 149.4 | 104.2 | 594.4 | 190.3 | 321.1 | 409.9 |
| 2005 | 239 | 257.6 | 289.2 | 318.8 | 221.1 | 349.9 | 378.1 | 275 | 185.2 | 112.5 | 132.7 | 242.1 |
| 2006 | 316.8 | 238.2 | 163.6 | 163.7 | 121.8 | 95 | 82.5 | 62.5 | 104.3 | 114.8 | 181.9 | 183.8 |
| 2007 | 299.2 | 147.7 | 181.6 | 162.7 | 113.2 | 86.7 | 90.1 | 60.5 | 51.8 | 59.4 | 76.4 | 116.4 |
| 2008 | 157.5 | 202.9 | 317 | 179.9 | 165 | 95.3 | 61.2 | 88.5 | 59.4 | 50.6 | 62.2 | 254.9 |

### Little Tennessee River

|  | Monthly mean in cfs (Calculation Period: 1944-04-01 -> 2009-09-30) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| YEAR | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| 1944 |  |  |  | 700 | 370 | 235.1 | 162.5 | 153.4 | 147.1 | 132.2 | 161.2 | 223.3 |
| 1945 | 279.5 | 376 | 401.5 | 443 | 351.2 | 203.7 | 156 | 152.2 | 287.9 | 207.8 | 243.5 | 432.2 |
| 1946 | 1,008 | 907.6 | 892.7 | 593.5 | 664.7 | 342.6 | 244.4 | 159.4 | 156.2 | 182 | 188.6 | 206.5 |
| 1947 | 679.4 | 366.4 | 366.2 | 469.6 | 333.5 | 242.2 | 159.2 | 138.2 | 101.5 | 216.7 | 351.5 | 285.4 |
| 1948 | 285.3 | 683 | 716.9 | 591.7 | 300 | 208.6 | 352 | 372.3 | 202 | 135.3 | 751.4 | 632.9 |
| 1949 | 788.3 | 711.4 | 513.5 | 633.1 | 594.3 | 694.1 | 626.6 | 462.3 | 530.9 | 436.4 | 447.7 | 438 |
| 1950 | 541.9 | 566.5 | 716.7 | 428 | 309.9 | 373.4 | 285 | 249.7 | 671.3 | 333.8 | 246.5 | 411.1 |
| 1951 | 291.8 | 350.6 | 455.8 | 505.9 | 338.6 | 346 | 226.5 | 144.5 | 144.2 | 128 | 244 | 623.7 |
| 1952 | 493 | 553 | 1,199 | 599.3 | 364.9 | 313.1 | 152.7 | 152.4 | 131.1 | 105.6 | 160.3 | 227 |
| 1953 | 513.1 | 726.6 | 598.6 | 365.2 | 387.4 | 213.9 | 194.6 | 140.5 | 119.9 | 103.7 | 140.8 | 387.8 |
| 1954 | 734.9 | 442.5 | 570.1 | 563.9 | 351 | 245.4 | 168 | 131 | 80.2 | 70.5 | 100.6 | 238.9 |
| 1955 | 200.3 | 464.3 | 399.7 | 581.5 | 665.1 | 356.7 | 313.7 | 269.1 | 145.1 | 154.9 | 146.1 | 158.9 |
| 1956 | 131.5 | 598.2 | 499.7 | 694.3 | 370.4 | 214.2 | 252.5 | 149.9 | 132.5 | 135.7 | 140.7 | 360.9 |
| 1957 | 365.7 | 783 | 481.1 | 814.5 | 323.2 | 391.7 | 234.2 | 139.2 | 184.1 | 307.6 | 641 | 607.5 |
| 1958 | 493.7 | 564.7 | 511 | 636.6 | 556.5 | 257.1 | 406.8 | 243.8 | 156.5 | 138.2 | 128.4 | 155.5 |
| 1959 | 347.9 | 343.3 | 366.5 | 471.8 | 462.3 | 475 | 237.8 | 153.2 | 240.2 | 462.4 | 296.9 | 358.6 |
| 1960 | 439.5 | 779.2 | 621.2 | 711.3 | 336.2 | 284.4 | 240.9 | 376 | 256.2 | 333.4 | 229.5 | 236.1 |
| 1961 | 279.7 | 633.2 | 641.7 | 576.5 | 370.5 | 371.6 | 353.9 | 368.5 | 292.2 | 190 | 224.6 | 841 |
| 1962 | 665.6 | 654.1 | 642.8 | 785.1 | 371.3 | 367.9 | 202.8 | 157.5 | 151.3 | 181 | 207.1 | 202.5 |
| 1963 | 284.9 | 310.3 | 736.6 | 361.5 | 362.1 | 293.3 | 356.9 | 205.6 | 163.2 | 131.8 | 174.7 | 244.7 |
| 1964 | 549.5 | 490.5 | 950.2 | 1,014 | 500.2 | 259.5 | 191 | 214 | 316.5 | 1,078 | 369.4 | 549.5 |
| 1965 | 501.6 | 592.1 | 598 | 522.4 | 367.2 | 303 | 201.8 | 176 | 172.4 | 363.7 | 191 | 157.8 |
| 1966 | 243.7 | 843.1 | 587.7 | 406 | 568.3 | 318.3 | 210.5 | 189.6 | 227 | 275.9 | 450.6 | 357.7 |
| 1967 | 472.7 | 459.8 | 494.3 | 309 | 352.5 | 644 | 427.1 | 607.2 | 477.7 | 293 | 379.1 | 653.5 |
| 1968 | 664.8 | 348.5 | 544.5 | 496.2 | 355.8 | 260.3 | 190.2 | 173 | 154.6 | 152.6 | 218.1 | 308.8 |
| 1969 | 405.7 | 661.2 | 422.6 | 528.2 | 415.3 | 426.7 | 232.6 | 372.3 | 306.5 | 276.8 | 417.9 | 405 |
| 1970 | 405.8 | 405.6 | 401.2 | 422.1 | 273.1 | 415 | 192.5 | 196 | 139.7 | 232 | 305 | 230.4 |
| 1971 | 438.2 | 690.2 | 602.3 | 445.8 | 377.3 | 278.5 | 320.5 | 361.5 | 203.5 | 210.9 | 264.3 | 629.9 |
| 1972 | 693.1 | 437.3 | 414.7 | 361.2 | 423.2 | 342.8 | 291.5 | 287.8 | 180.2 | 254.5 | 431.9 | 635.9 |
| 1973 | 619.5 | 761.5 | 917.7 | 756.8 | 883.1 | 668.5 | 354.4 | 290.8 | 199.8 | 170.5 | 278.2 | 633.3 |
| 1974 | 928.8 | 829.3 | 507.1 | 889 | 648.6 | 534.7 | 369.5 | 695.1 | 324.8 | 229.9 | 266.7 | 381.6 |
| 1975 | 556.9 | 849.6 | 1,019 | 636 | 429.9 | 315.4 | 193.2 | 153.8 | 337.5 | 469.8 | 475.3 | 391.7 |
| 1976 | 688.1 | 507.6 | 576.1 | 639.1 | 999.4 | 668.8 | 361.2 | 222.9 | 181.9 | 360.7 | 265.8 | 426.6 |
| 1977 | 384.5 | 299.8 | 863.1 | 906.6 | 405.1 | 285.2 | 156.2 | 157.7 | 330 | 280.1 | 640.3 | 579.8 |
| 1978 | 810 | 481.3 | 496.1 | 311.8 | 348.3 | 229.9 | 144.1 | 419.8 | 163.6 | 121.4 | 120.9 | 287 |
| 1979 | 624 | 534 | 1,119 | 969.4 | 618.5 | 412.9 | 406.4 | 306.8 | 514.9 | 454.1 | 814.7 | 498.2 |
| 1980 | 518.2 | 411.4 | 983.4 | 926.9 | 602.5 | 386.1 | 212.6 | 155.8 | 156.8 | 162.1 | 181.8 | 153.6 |
| 1981 | 119.7 | 364.5 | 253.3 | 265.2 | 294.6 | 364.5 | 162.5 | 106.5 | 121.9 | 110.4 | 113.9 | 238.3 |
| 1982 | 517.9 | 767.2 | 458.2 | 381.4 | 296.6 | 211.2 | 189.7 | 177.5 | 165.2 | 182.8 | 233 | 679.2 |
| 1983 | 430.6 | 823.6 | 581.7 | 900.7 | 567.5 | 359.3 | 204.5 | 135.3 | 165.9 | 230.1 | 313 | 741.1 |
| 1984 | 451.5 | 664.1 | 640.7 | 718.2 | 759.8 | 349.8 | 313.4 | 291.2 | 142.8 | 164.9 | 221.6 | 255.5 |
| 1985 | 217.4 | 550.2 | 267.3 | 296.4 | 241.3 | 170 | 194 | 267.9 | 141.2 | 124.4 | 296.2 | 259.5 |
| 1986 | 177.4 | 221.7 | 282 | 172 | 156.8 | 138.2 | 94.8 | 78.3 | 81.8 | 138.8 | 318.5 | 501.7 |
| 1987 | 414.8 | 449.4 | 548.7 | 460.6 | 259 | 210.6 | 158.4 | 148.5 | 153.7 | 107 | 212.8 | 274.1 |
| 1988 | 489.9 | 310.3 | 243.9 | 372.3 | 188.5 | 109.6 | 120.2 | 112.5 | 122.3 | 131.1 | 172.6 | 154.3 |
| 1989 | 324.8 | 339.2 | 563.7 | 410.7 | 416.1 | 626.8 | 772.5 | 367.5 | 367.2 | 567.9 | 413.7 | 435.6 |
| 1990 | 598.2 | 1,252 | 1,079 | 508.5 | 428 | 233.4 | 242.6 | 166.8 | 178.3 | 325.3 | 228.7 | 432.9 |
| 1991 | 515.3 | 470.1 | 696.7 | 703.3 | 610.3 | 392.3 | 306.7 | 322.9 | 247.9 | 162.4 | 232.4 | 385.9 |
| 1992 | 354.5 | 419.1 | 553.4 | 395.8 | 333.9 | 573.3 | 287.9 | 399.6 | 439.8 | 383.9 | 811.2 | 777.4 |
| 1993 | 768.8 | 528.5 | 778.1 | 705.4 | 419.3 | 212.9 | 128.5 | 110.5 | 137.3 | 94.6 | 238.8 | 313.2 |
| 1994 | 409.5 | 512.1 | 741.9 | 833.4 | 362.2 | 342.4 | 368.2 | 609.2 | 322.8 | 463.1 | 334.1 | 427.6 |
| 1995 | 646.5 | 769.1 | 767.7 | 368.7 | 278.4 | 299.4 | 211.6 | 310.5 | 222.3 | 557.7 | 532.4 | 345.1 |
| 1996 | 800.9 | 707.3 | 590.8 | 457.6 | 394.5 | 491.2 | 225.5 | 224.1 | 306 | 269.6 | 281.8 | 552.1 |
| 1997 | 527.3 | 582.9 | 865 | 649.2 | 520.9 | 489.7 | 322.4 | 207.7 | 213.5 | 252.5 | 235.8 | 276.7 |
| 1998 | 825 | 939.5 | 754.3 | 722.2 | 518.6 | 340.2 | 176.7 | 136.3 | 92 | 103.9 | 147.1 | 193.9 |
| 1999 | 355.2 | 519.6 | 396.9 | 423.8 | 397.7 | 251.8 | 216.2 | 112.7 | 85.2 | 181.7 | 273.7 | 281.3 |
| 2000 | 275.4 | 293.9 | 379.8 | 494.5 | 267.1 | 166.4 | 105.5 | 80.5 | 88 | 70.7 | 155.3 | 152.8 |
| 2001 | 228.5 | 265.4 | 354.4 | 242 | 160.1 | 203.5 | 166.3 | 146.4 | 168.8 | 170.3 | 144.4 | 204.8 |
| 2002 | 336.2 | 282.1 | 347 | 348.6 | 336.9 | 195.5 | 142.9 | 107 | 299 | 268.9 | 378.8 | 475.5 |
| 2003 | 371.6 | 466.6 | 508.9 | 505.7 | 781.3 | 521.3 | 516.8 | 380.7 | 400.8 | 271.7 | 511.3 | 503.5 |
| 2004 | 363.9 | 509.2 | 319.5 | 296.1 | 256 | 244.7 | 228.2 | 180 | 1,030 | 357 | 497.9 | 695.8 |
| 2005 | 457.1 | 435.8 | 483.7 | 572.1 | 402.2 | 716.6 | 809.6 | 589.3 | 385.8 | 270.2 | 297.6 | 468.7 |
| 2006 | 506.6 | 414.5 | 291.5 | 275.9 | 198.2 | 184.6 | 134.2 | 105.4 | 183.4 | 201.3 | 309.5 | 326.7 |
| 2007 | 567.6 | 278.6 | 377.8 | 260.1 | 176.2 | 148 | 162.3 | 105.4 | 97.5 | 98.7 | 109 | 174 |
| 2008 | 249.7 | 360.3 | 584.7 | 304.5 | 222.4 | 137.4 | 99.1 | 127.2 | 84.4 | 81.5 | 97.1 | 303.2 |

### Watauga River

|  | Monthly mean in cfs (Calculation Period: 1940-04-01 -> 2009-09-30) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| YEAR | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| 1940 |  |  |  | 300 | 122.8 | 169 | 134.2 | 1,169 | 184.8 | 72.3 | 78.1 | 152.8 |
| 1941 | 115.9 | 67.5 | 140.4 | 168 | 67.5 | 52.2 | 376.8 | 74 | 36.5 | 29.4 | 45.2 | 111.2 |
| 1942 | 101.1 | 152.2 | 297.5 | 106.2 | 334.8 | 329.4 | 123.1 | 151.5 | 282.8 | 99.7 | 85.8 | 276.2 |
| 1943 | 271.1 | 264.3 | 204.9 | 255.8 | 202.8 | 132.3 | 186.3 | 59.5 | 39.7 | 28.5 | 48.6 | 51.1 |
| 1944 | 108.2 | 295.4 | 305.8 | 198 | 115.8 | 52 | 35 | 30 | 51.5 | 109.9 | 96 | 143.5 |
| 1945 | 181.7 | 227.9 | 202.5 | 249.8 | 314.3 | 120.4 | 108.4 | 64.2 | 343.1 | 111.9 | 118.3 | 224.6 |
| 1946 | 428.8 | 261.2 | 279.8 | 165.8 | 185.2 | 91 | 109.4 | 55.4 | 40.2 | 67.1 | 57.9 | 53.3 |
| 1947 | 239.4 | 122.9 | 195.4 | 192.3 | 81.5 | 119.5 | 81.6 | 96 | 98.9 | 300 | 386.4 | 117.1 |
| 1948 | 98.3 | 310.7 | 237.5 | 260.8 | 76.3 | 94 | 153.3 | 156.7 | 209.2 | 75.6 | 349.9 | 275.9 |
| 1949 | 216.4 | 232.3 | 223.1 | 329.5 | 229.8 | 218.5 | 300.3 | 416.2 | 150.1 | 145.7 | 239.6 | 171.7 |
| 1950 | 179.4 | 285.2 | 239 | 132.7 | 178.9 | 124 | 115.3 | 122.8 | 301.5 | 88.2 | 89.1 | 433.8 |
| 1951 | 145.9 | 193.9 | 286.8 | 241.5 | 105.3 | 77.3 | 49.3 | 38.5 | 27.5 | 24.3 | 61.4 | 186.2 |
| 1952 | 164.4 | 230.2 | 312 | 155.2 | 98 | 63 | 36.3 | 53.9 | 36.5 | 32.6 | 86.7 | 100.3 |
| 1953 | 273.5 | 297.3 | 359.4 | 141.4 | 170.5 | 106 | 65.5 | 32.5 | 29.5 | 23.4 | 41.9 | 92.2 |
| 1954 | 225.2 | 195.3 | 276 | 185.7 | 193.6 | 65.7 | 36.5 | 28.2 | 18.1 | 19.2 | 51.9 | 113.2 |
| 1955 | 70 | 225 | 378.6 | 359.9 | 117.1 | 76.8 | 146.9 | 111.6 | 45.6 | 40.9 | 46 | 52.7 |
| 1956 | 55.5 | 269.7 | 211.5 | 286.8 | 107 | 46.7 | 65.7 | 25.6 | 41.7 | 92.2 | 109.6 | 106.5 |
| 1957 | 214 | 452.6 | 191.5 | 556.6 | 106.8 | 230.6 | 94.5 | 39.2 | 102.1 | 110.3 | 333.8 | 363.5 |
| 1958 | 225 | 300.6 | 252.1 | 383.5 | 322.9 | 100.7 | 93.1 | 69.2 | 37.4 | 37 | 40.9 | 123.4 |
| 1959 | 184.3 | 119.2 | 177.2 | 300 | 129 | 124.3 | 78.9 | 91.5 | 256.4 | 232.9 | 113.7 | 224.4 |
| 1960 | 205.7 | 399.5 | 438 | 428.3 | 153.4 | 122.9 | 159.5 | 156.5 | 92.4 | 78.7 | 58.3 | 73.9 |
| 1961 | 122.7 | 324.2 | 243.9 | 327.4 | 216.2 | 132.9 | 77.8 | 287.6 | 95.8 | 67.8 | 167.6 | 343.9 |
| 1962 | 303.1 | 297.8 | 312.5 | 281.1 | 89.9 | 91.6 | 58.5 | 41.1 | 50.7 | 69 | 155.8 | 144.6 |
| 1963 | 127.9 | 90.9 | 557.2 | 104.6 | 126.6 | 83.2 | 57.4 | 36.2 | 28.9 | 21.8 | 53 | 45.6 |
| 1964 | 184.5 | 180.3 | 362.5 | 232.8 | 111 | 56.2 | 75.2 | 79.3 | 94.5 | 379.7 | 181.5 | 179 |
| 1965 | 186.7 | 278.9 | 380.9 | 306.4 | 149.4 | 108.1 | 77.2 | 55.9 | 60.2 | 155.4 | 65.9 | 50.2 |
| 1966 | 65.5 | 598.6 | 177.4 | 132.9 | 227.5 | 80.4 | 71.6 | 174.4 | 260.3 | 144.8 | 256.6 | 288.8 |
| 1967 | 186.8 | 190.4 | 267.9 | 94.5 | 148.8 | 147.3 | 132 | 173.7 | 111.9 | 128.1 | 131.9 | 220.6 |
| 1968 | 280.3 | 164.9 | 237.9 | 213.9 | 103.3 | 123.4 | 60.6 | 51.1 | 37.4 | 130.3 | 106.7 | 107.4 |
| 1969 | 136.1 | 243.5 | 299.9 | 228.2 | 139.1 | 155.3 | 105 | 82.8 | 98.3 | 138.2 | 237.5 | 255.2 |
| 1970 | 199.7 | 284.8 | 186.1 | 229.4 | 141.4 | 94.5 | 67.7 | 216.6 | 55.4 | 157.2 | 225.5 | 104.4 |
| 1971 | 184.7 | 403.1 | 244.2 | 149.8 | 254.6 | 142.4 | 188.4 | 183.6 | 79.8 | 205.8 | 197.8 | 183.7 |
| 1972 | 204.4 | 327 | 205.5 | 246.4 | 243 | 300.6 | 119.9 | 72.8 | 52.3 | 70.1 | 115.9 | 204.6 |
| 1973 | 207.3 | 314 | 512.7 | 297.9 | 411.4 | 152 | 83.5 | 76.9 | 62.6 | 67.8 | 106.7 | 332.7 |
| 1974 | 313.5 | 324.6 | 289.5 | 418 | 194.9 | 358.4 | 162.7 | 151.1 | 292.1 | 106.1 | 111.9 | 326.3 |
| 1975 | 274.8 | 382.9 | 557.9 | 180.2 | 263 | 261.1 | 104.8 | 94 | 262.6 | 299.8 | 178.5 | 149 |
| 1976 | 323.2 | 250.1 | 163.4 | 167.3 | 348.8 | 519.1 | 126.8 | 55.5 | 65.1 | 277 | 104.1 | 238.8 |
| 1977 | 117.6 | 168 | 499.2 | 371.5 | 163.4 | 147.9 | 95.7 | 72.3 | 173.8 | 205.2 | 661.7 | 216.8 |
| 1978 | 402.4 | 139.9 | 344.4 | 164.4 | 201.7 | 75.6 | 47.1 | 144.1 | 47.3 | 35.3 | 46.8 | 112.5 |
| 1979 | 285.6 | 360 | 857.6 | 368.2 | 252.7 | 192.4 | 256.4 | 116.6 | 690.6 | 223.1 | 579.5 | 177.6 |
| 1980 | 216.5 | 163.1 | 399.2 | 535.6 | 213.6 | 119.1 | 103 | 65.5 | 48 | 62.1 | 71 | 67 |
| 1981 | 56.1 | 286.6 | 180.2 | 190.8 | 278.4 | 212 | 67.6 | 32.3 | 35 | 35.2 | 34.6 | 72.5 |
| 1982 | 252.1 | 328.6 | 252.3 | 163.6 | 161.9 | 112.3 | 95.9 | 84 | 80.5 | 74 | 147.8 | 329.6 |
| 1983 | 224.3 | 551 | 570.3 | 567.4 | 249.2 | 168.4 | 94.5 | 46.9 | 44.1 | 118.8 | 154.8 | 403.4 |
| 1984 | 142.5 | 425 | 356.3 | 303.2 | 339.4 | 130.8 | 117.7 | 89.9 | 42 | 44.5 | 96.1 | 84.6 |
| 1985 | 103.9 | 333.8 | 124.2 | 136.7 | 86.9 | 74.5 | 60.6 | 129.1 | 48.2 | 49.2 | 367 | 168.2 |
| 1986 | 166.5 | 191.8 | 167.8 | 82.1 | 97 | 58.1 | 43.6 | 38.6 | 79.9 | 42.6 | 88.1 | 243 |
| 1987 | 164.3 | 211.3 | 389.5 | 688.6 | 145.6 | 111.8 | 64.9 | 31.1 | 68 | 29.6 | 105.6 | 117.4 |
| 1988 | 152.9 | 118 | 77 | 178.8 | 76.9 | 41.4 | 39.5 | 30.5 | 52.3 | 36.7 | 116 | 68.8 |
| 1989 | 130.9 | 154.7 | 229.2 | 155.8 | 287 | 227.9 | 461.2 | 141 | 359.5 | 273.5 | 197.6 | 199.2 |
| 1990 | 263.9 | 402.7 | 453.3 | 165.9 | 225 | 88.3 | 83 | 77.2 | 48.5 | 361.4 | 134.5 | 195.1 |
| 1991 | 230.1 | 242.2 | 467.4 | 354.7 | 172.9 | 127.5 | 95.6 | 66.7 | 55.6 | 32.5 | 107.4 | 150.6 |
| 1992 | 203.2 | 140.7 | 207.9 | 417.7 | 240.8 | 582.7 | 104.3 | 145.2 | 127.2 | 197.2 | 521 | 288.3 |
| 1993 | 270.8 | 241.8 | 552.8 | 338 | 291.2 | 80.6 | 43.1 | 34 | 33.6 | 26.9 | 50.8 | 121.5 |
| 1994 | 181.3 | 256.2 | 363 | 315 | 113.3 | 101.4 | 125.5 | 467.4 | 93.3 | 81 | 66.1 | 128.5 |
| 1995 | 816.8 | 273 | 372.3 | 97 | 142.3 | 211.3 | 109.2 | 98.5 | 65.7 | 355.4 | 169.6 | 102.1 |
| 1996 | 468.9 | 313.1 | 213 | 169 | 154.5 | 148 | 72.7 | 230.8 | 115.6 | 83.4 | 221.8 | 265.7 |
| 1997 | 177.5 | 211.9 | 414.7 | 211.4 | 158.4 | 198.4 | 120.1 | 58.7 | 64.4 | 51.8 | 54 | 70.5 |
| 1998 | 496.9 | 642.8 | 395.2 | 403.5 | 285.2 | 175.2 | 80.6 | 60 | 28.5 | 39.5 | 34.7 | 70.5 |
| 1999 | 228 | 170.7 | 214.6 | 143.3 | 163.1 | 105.3 | 117.5 | 41.6 | 34.4 | 40.8 | 187.3 | 109.6 |
| 2000 | 97.6 | 198.9 | 218.4 | 302.1 | 116.8 | 72.8 | 67.3 | 117 | 45.4 | 28 | 49.6 | 50.3 |
| 2001 | 70.8 | 84 | 208.3 | 139.1 | 68.3 | 104.2 | 212.2 | 146.1 | 69.9 | 59.1 | 56.1 | 92.1 |
| 2002 | 181.5 | 114.2 | 200 | 134.7 | 82.3 | 62.4 | 59.3 | 23.9 | 94.6 | 89.4 | 176.1 | 229.5 |
| 2003 | 170.6 | 332.1 | 284.5 | 427.8 | 187.6 | 254 | 191.1 | 213.1 | 112.5 | 71.2 | 385.1 | 255.3 |
| 2004 | 140 | 237.6 | 178.9 | 227.2 | 130.6 | 201.1 | 106.9 | 62.6 | 1,049 | 208.1 | 242.7 | 261.3 |
| 2005 | 238 | 175.6 | 280.7 | 302.4 | 155.3 | 194.9 | 186.9 | 116.7 | 65.2 | 89.1 | 197.9 | 176 |
| 2006 | 256.5 | 189 | 117.5 | 202.2 | 140.5 | 139.4 | 78.5 | 57.1 | 130.8 | 75.7 | 266.3 | 195 |
| 2007 | 255 | 104.8 | 192.2 | 119.9 | 68.1 | 59.1 | 80.6 | 38.7 | 31 | 71.1 | 42.9 | 72 |
| 2008 | 82.3 | 131.5 | 316.2 | 109 | 67 | 62.3 | 71.5 | 73.5 | 67.7 | 52.9 | 61 | 141.1 |

# Appendix 3:  Minitab Output

## Valley River

Stepwise Regression: Valley MJJ versus Altamaha Riv, Black River , ...

Alpha-to-Enter: 0.05  Alpha-to-Remove: 0.1

Response is Valley MJJ on 30 predictors, with N = 62

Step                                   1        2        3        4

Constant                          -4.523  -16.451  -18.399  -25.117

Piney Creek Residual              0.0197   0.0188   0.0168   0.0160

T-Value                             4.71     4.84     4.56     4.43

P-Value                            0.000    0.000    0.000    0.000

Ramseys Draft Recollection Resi            0.0129   0.0117   0.0105

T-Value                                      3.26     3.16     2.89

P-Value                                     0.002    0.003    0.006

Black River Residual                                0.0052   0.0050

T-Value                                               3.15     3.09

P-Value                                              0.003    0.003

Grandfather Mountain Residual                                0.0091

T-Value                                                        2.14

P-Value                                                       0.037

S                                   4.76     4.41     4.11     3.99

R-Sq                               26.98    38.14    47.19    51.10

R-Sq(adj)                          25.76    36.04    44.46    47.67

Mallows Cp                          22.8     12.5      4.4      2.1

PRESS                            1443.91  1269.92  1129.71  1089.94

R-Sq(pred)                         22.31    31.67    39.21    41.36

Regression Analysis: Valley MJJ versus Piney Creek , Ramseys Draf, ...

The regression equation is

Valley MJJ = - 25.1 + 0.0160 Piney Creek Residual

+ 0.0105 Ramseys Draft Recollection Resi

+ 0.00501 Black River Residual

+ 0.00906 Grandfather Mountain Residual

Predictor                            Coef   SE Coef      T      P    VIF

Constant                          -25.117     5.810  -4.32  0.000

Piney Creek Residual             0.015953  0.003599   4.43  0.000  1.049

Ramseys Draft Recollection Resi  0.010509  0.003642   2.89  0.006  1.040

Black River Residual             0.005010  0.001619   3.09  0.003  1.049

Grandfather Mountain Residual    0.009064  0.004243   2.14  0.037  1.051

S = 3.99285   R-Sq = 51.1%   R-Sq(adj) = 47.7%

PRESS = 1089.94   R-Sq(pred) = 41.36%

Analysis of Variance

Source          DF       SS      MS      F      P

Regression       4   949.79  237.45  14.89  0.000

Residual Error  57   908.74   15.94

Total           61  1858.53

Durbin-Watson statistic = 1.65878

Autocorrelation Function: RESI1

Lag        ACF      T   LBQ

1   0.167012   1.32  1.81

2  -0.069569  -0.53  2.13

3   0.145373   1.11  3.56

4   0.057998   0.43  3.79

5   0.107097   0.80  4.58

6   0.069020   0.51  4.92

7  -0.054875  -0.40  5.14

8  -0.097563  -0.72  5.84

9   0.120260   0.87  6.92

10   0.063087   0.45  7.23

11  -0.085157  -0.61  7.79

12   0.064517   0.46  8.12

13  -0.037980  -0.27  8.24

14  -0.027922  -0.20  8.30

15  -0.037327  -0.26  8.42

16  -0.113129  -0.80  9.52

Autocorrelation for RESI1

## Oconaluftee River

Stepwise Regression: Oconaluftee  versus Chickahominy, Hampton Hill, ...

Alpha-to-Enter: 0.05  Alpha-to-Remove: 0.1

Response is Oconaluftee MJJ on 7 predictors, with N = 32

Step                            1        2

Constant                   -7.921  -25.212

Piney Creek Residual       0.0412   0.0378

T-Value                      3.74     3.85

P-Value                     0.001    0.001

Kelsey Tract II Residual            0.0205

T-Value                               3.06

P-Value                              0.005

S                            6.31     5.58

R-Sq                        31.75    48.39

R-Sq(adj)                   29.47    44.83

Mallows Cp                    9.9      2.7

PRESS                     1336.95  1072.07

R-Sq(pred)                  23.63    38.76

Regression Analysis: Oconaluftee  versus Piney Creek , Kelsey Tract

The regression equation is

Oconaluftee MJJ = - 25.2 + 0.0378 Piney Creek Residual

+ 0.0205 Kelsey Tract II Residual

Predictor                     Coef   SE Coef      T      P    VIF

Constant                    -25.21     11.12  -2.27  0.031

Piney Creek Residual      0.037768  0.009820   3.85  0.001  1.013

Kelsey Tract II Residual  0.020549  0.006721   3.06  0.005  1.013

S = 5.58207   R-Sq = 48.4%   R-Sq(adj) = 44.8%

PRESS = 1072.07   R-Sq(pred) = 38.76%

Analysis of Variance

Source          DF       SS      MS      F      P

Regression       2   847.10  423.55  13.59  0.000

Residual Error  29   903.63   31.16

Total           31  1750.73

Durbin-Watson statistic = 2.08473

Autocorrelation Function: RESI1

Lag         ACF      T   LBQ

1  -0.0490227  -0.28  0.08

2  -0.0351592  -0.20  0.13

3   0.0329315   0.19  0.17

4  -0.0343890  -0.19  0.22

5   0.0070939   0.04  0.22

6   0.0203551   0.11  0.24

7   0.0417493   0.23  0.31

8  -0.0892418  -0.50  0.67

Autocorrelation for RESI1

## Nantahala River

Stepwise Regression: Nantahala MJ versus Grandfather , Hampton Hill, ...

Alpha-to-Enter: 0.05  Alpha-to-Remove: 0.1

Response is Nantahala MJJ on 10 predictors, with N = 40

Step                              1           2           3

Constant                  -0.002453  -11.852179  -13.085839

Kelsey Tract II Residual     0.0133      0.0128      0.0127

T-Value                        3.55        3.76        3.96

P-Value                       0.001       0.001       0.000

Piney Creek Residual                     0.0126      0.0104

T-Value                                    2.98        2.56

P-Value                                   0.005       0.015

Pearl River Residual                                 0.0037

T-Value                                                2.46

P-Value                                               0.019

S                              3.36        3.05        2.87

R-Sq                          24.87       39.45       48.17

R-Sq(adj)                     22.89       36.17       43.85

Mallows Cp                     17.2         8.9         4.7

PRESS                       481.587     415.668     366.551

R-Sq(pred)                    15.52       27.09       35.70

Regression Analysis: Nantahala MJ versus Kelsey Tract, Piney Creek , ...

The regression equation is

Nantahala MJJ = - 13.1 + 0.0127 Kelsey Tract II Residual

+ 0.0104 Piney Creek Residual + 0.00368 Pearl River Residual

Predictor                     Coef   SE Coef      T      P    VIF

Constant                   -13.086     4.967  -2.63  0.012

Kelsey Tract II Residual  0.012665  0.003196   3.96  0.000  1.002

Piney Creek Residual      0.010354  0.004050   2.56  0.015  1.054

Pearl River Residual      0.003677  0.001494   2.46  0.019  1.053

S = 2.86502   R-Sq = 48.2%   R-Sq(adj) = 43.8%

PRESS = 366.551   R-Sq(pred) = 35.70%

Analysis of Variance

Source          DF       SS      MS      F      P

Regression       3  274.581  91.527  11.15  0.000

Residual Error  36  295.500   8.208

Total           39  570.081

Durbin-Watson statistic = 2.25023

Autocorrelation Function: RESI1

Lag        ACF      T    LBQ

1  -0.135274  -0.86   0.79

2  -0.064525  -0.40   0.97

3   0.373239   2.31   7.30

4   0.024883   0.14   7.33

5  -0.284483  -1.56  11.21

6   0.280716   1.46  15.10

7   0.198597   0.98  17.11

8  -0.255469  -1.23  20.54

9   0.012982   0.06  20.55

10   0.197439   0.92  22.73

Autocorrelation for RESI1

## Little Tennessee River

Stepwise Regression: Little TN MJ versus Ebenezer Cre, Kelsey Tract, ...

Alpha-to-Enter: 0.05  Alpha-to-Remove: 0.1

Response is Little TN MJJ on 6 predictors, with N = 36

Step                            1        2

Constant                   -2.671  -13.894

Kelsey Tract II Residual   0.0293   0.0300

T-Value                      3.48     3.97

P-Value                     0.001    0.000

Pearl River Residual                0.0105

T-Value                               2.99

P-Value                              0.005

S                            7.37     6.64

R-Sq                        26.27    41.96

R-Sq(adj)                   24.10    38.44

Mallows Cp                   13.4      5.7

PRESS                     2132.17  1721.59

R-Sq(pred)                  14.93    31.31

Regression Analysis: Little TN MJ versus Kelsey Tract, Pearl River

The regression equation is

Little TN MJJ = - 13.9 + 0.0300 Kelsey Tract II Residual

+ 0.0105 Pearl River Residual

Predictor                     Coef   SE Coef      T      P    VIF

Constant                   -13.894     8.632  -1.61  0.117

Kelsey Tract II Residual  0.030048  0.007578   3.97  0.000  1.001

Pearl River Residual      0.010543  0.003529   2.99  0.005  1.001

S = 6.63927   R-Sq = 42.0%   R-Sq(adj) = 38.4%

PRESS = 1721.59   R-Sq(pred) = 31.31%

Analysis of Variance

Source          DF       SS      MS      F      P

Regression       2  1051.70  525.85  11.93  0.000

Residual Error  33  1454.64   44.08

Total           35  2506.34

Durbin-Watson statistic = 2.06196

Autocorrelation Function: RESI1

Lag        ACF      T    LBQ

1  -0.057414  -0.34   0.13

2   0.084715   0.51   0.42

3   0.261060   1.55   3.24

4   0.088982   0.50   3.58

5  -0.090073  -0.50   3.94

6   0.059486   0.33   4.10

7   0.157190   0.86   5.27

8  -0.287765  -1.55   9.31

9   0.212106   1.07  11.59

Autocorrelation for RESI1

## Watauga River

Stepwise Regression: Watauga MJJ versus Bluff Mounta, Boogerman Tr, ...

Alpha-to-Enter: 0.05  Alpha-to-Remove: 0.1

Response is Watauga MJJ on 8 predictors, with N = 40

Step                       1

Constant              -2.269

Scotts Gap Residual   0.0133

T-Value                 3.81

P-Value                0.000

S                       3.96

R-Sq                   27.68

R-Sq(adj)              25.77

Mallows Cp               2.3

PRESS                648.654

R-Sq(pred)             21.25

Regression Analysis: Watauga MJJ versus Scotts Gap Residual

The regression equation is

Watauga MJJ = - 2.27 + 0.0133 Scotts Gap Residual

Predictor                Coef   SE Coef      T      P    VIF

Constant               -2.269     3.516  -0.65  0.523

Scotts Gap Residual  0.013335  0.003497   3.81  0.000  1.000

S = 3.95934   R-Sq = 27.7%   R-Sq(adj) = 25.8%

PRESS = 648.654   R-Sq(pred) = 21.25%

Analysis of Variance

Source          DF      SS      MS      F      P

Regression       1  227.98  227.98  14.54  0.000

Residual Error  38  595.70   15.68

Total           39  823.68

Durbin-Watson statistic = 1.44154

Autocorrelation Function: RESI1

Lag        ACF      T   LBQ

1   0.273046   1.73  3.21

2   0.061700   0.36  3.38

3   0.232834   1.37  5.84

4   0.195730   1.10  7.63

5   0.151070   0.82  8.72

6   0.062843   0.34  8.92

7   0.006541   0.04  8.92

8  -0.077154  -0.41  9.23

9  -0.026882  -0.14  9.27

10  -0.008539  -0.05  9.28

Autocorrelation for RESI1

# Vita

Tate Geren was born in Knoxville, Tennessee.  He currently resides in Knoxville with his wife Kristin. He has four younger brothers: Wesley Geren, Cole Geren, Zac Geren, and Seth Bales.

Tate graduated from a local Knoxville high school, South-Doyle, and after graduation he attended Maryville College in Maryville, TN.  He has a Bachelor of Arts in Civil Engineering from Maryville College, and a Bachelor of Science in Civil Engineering from the University of Tennessee, Knoxville where he graduated Summa Cum Laude.

Currently, Tate is pursuing a Master of Science in Environmental Engineering with a concentration in Water Resources.  He is on track to graduate in December of 2010.